3GPP TSG-RAN WG1 Meeting #110bis-e	R1-xxxxxxx

e-Meeting, October 10 – 19, 2022

Agenda Item:	8.13

Source:	Moderator (Huawei)

Title:	[110bis-e-R17-DSS-01] draft Summary of further MR-DC/CA Enhancement

Document for:	Discussion and Decision

# Introduction

According to the contribution papers under agenda item 8.13 for WI further MR-DC/CA Enhancement, all identified issues are summarized in Section 2.

# Summary of issues

According to all of companies’ contribution documents, three issues are summarized below.

- Issue-1: CR for disabling EN-DC power split when SCG is deactivated. [1]

## Issue-1: CR for disabling EN-DC power split when SCG is deactivated.

In [1], clarification on UE procedure for power control when SCG is deactivated is provided. Details of the CR are as follows:

| Reason for change: | RAN1#109 agreed a CR in R1-2205683 capturing that the power control operation for NR-DC is applicable only when the SCG is activated in clause 7.6.2. by adding the following sentence in the beginning of 7.6.2: The UE procedures described in this clause are not applicable if the UE is provided scg-State [12, TS 38.331]. RAN1 #110 discussed briefly for extending the same to EN-DC, but at the time the need was questioned due to the mandatory dynamic power sharing support between MCG and SCG from Rel-16 onwards allowing the network to configure the full Tx power for each RAT, and the power sharing making sure that LTE can take all the power that is available to it. It should be noted that the EN-DC deployment may want to configure the LTE MCG with reduced max power in order to guarantee minimum available power for the NR SCG when the SCG is active so that the NR link is able to close the feedback loops and provide needed uplink feedback for the different protocol layers. This can be a desired system configuration even if the UE supported dynamic power sharing between the LTE MCG and the NR SCG as configuring full Tx power to the LTE MCG may lead to power starvation of the NR SCG. When the UL coverage is running out, the SCG deactivation should be able to free the power allocated ot the SCG for the MCG use the same way this is possible with NR-DC as per the RAN1#109 agreement, and this can be helpful even if the UE supports dynamic power sharing between the LTE MCG and the NR SCG. |
| --- | --- |
| Summary of change: | Condition the UE procedures for power control in clause 7.6.1 on the SCG being activated. |
| Consequences if not approved: | Incomplete support for NR-DC enhancements with EN-DC. |

Changes to TS 38.213:

*** Unchanged text is omitted ***

7.6	Dual connectivity

7.6.1	EN-DC

The UE procedures described in this clause are not applicable if the UE is provided scg-State [12, TS 38.331].

If a UE is configured with a MCG using E-UTRA radio access and with a SCG using NR radio access, the UE is configured a maximum power  for transmissions on the MCG by p-MaxEUTRA and a maximum power  for transmissions in FR1 on the SCG by p-NR-FR1.

The UE determines a transmission power for the MCG as described in [13, TS 36.213] using  as the maximum transmission power. The UE determines transmission power for the SCG in FR1 as described in clauses 7.1 through 7.5 using  as the maximum transmission power. The UE determines transmission power for the SCG in FR2 as described in clauses 7.1 through 7.5.

A UE does not expect to be configured for operation with shortened TTI and/or processing time [13, TS 36.213] on a cell that is included in an EN-DC configuration.

If a UE is configured with , where  is the linear value of ,  is the linear value of , and  is the linear value of a configured maximum transmission power for EN-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the SCG as follows.

*** Unchanged text is omitted ***

## Topics for discussion

The following topic for feMR-DC WI is discussed in the tdocs

Issue-1: CR for disabling EN-DC power split when SCG is deactivated.

The motivation to have this CR seems to disable the text of TS 38.213 on LTE max power below when NR SCG is deactivated.

“The UE determines a transmission power for the MCG as described in [13, TS 36.213] using  as the maximum transmission power.”

Regarding whether to discuss the above topic, companies are invited to provide comments.

| Company | View |
| --- | --- |

# Conclusions

TBD

# References

- R1-2210191	Disabling EN-DC power split when SCG is deactivated	Nokia, Nokia Shanghai Bell