3GPP TSG RAN WG1 #111		     R1-2210984

Toulouse, France, November 14th – 18th, 2022

Source:	vivo

Title:	Discussion on PUCCH resources for SPS PDSCHs with NACK only feedback mode

Agenda Item:	8.12

Document for:	Discussion and Decision

- Introduction

In this contribution, we share our view on PUCCH resources for SPS PDSCHs with NACK only feedback mode.

- PUCCH resources for SPS PDSCHs with NACK only feedback mode

In the last RAN1 meeting, FG#33-5-1j was agreed for NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission for SPS group-common PDSCH for multicast, which includes a component of support of separate SPS-PUCCH-AN-List from unicast [1].

| 33. NR_MBS | 33-5-1j | NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission for SPS group-commmon PDSCH for multicast | 1. Support NACK-only based HARQ-ACK feedback for SPS PDSCH for multicast, including: a) Multiple TBs with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of separate SPS-PUCCH-AN-List from unicast | 33-5-1f |
| --- | --- | --- | --- | --- |

That means, sps-PUCCH-AN-ListMulticast in pucch-ConfigurationListMulticast2 should be provided for SPS group-common PDSCH for multicast with NACK-only feedback when the UE is provided moreThanOneNackOnlyMode. The PUCCH for NACK-only based HARQ-ACK feedback for only SPS group-common PDSCH for multicast should be determined from sps-PUCCH-AN-ListMulticast configured in pucch-ConfigurationListMulticast2. However, this operation is not captured in the current specification since in the current specification, it only says that UE determines a PUCCH as described in Table 18-1 for a UE that is indicated the second HARQ-ACK reporting mode for only one G-RNTI. For the G-CS-RNTI case, no UE behavior is specified.

A UE that is indicated the second HARQ-ACK reporting mode for only one G-RNTI can be indicated by moreThanOneNackOnlyMode to provide associated HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or by selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1.

Proposal 1: When a UE provides only HARQ-ACK information bits for multicast SPS PDSCHs according to the second HARQ-ACK reporting mode and moreThanOneNackOnlyMode is provided, the UE determines a PUCCH to provide the HARQ-ACK information bit from sps-PUCCH-AN-ListMulticast configured in pucch-ConfigurationListMulticast2 should be captured in the specification of TS 38.213.

That means, when a UE provides only HARQ-ACK information bits for multicast SPS PDSCHs according to the second HARQ-ACK reporting mode and moreThanOneNackOnlyMode is provided, the UE determines a PUCCH to provide the HARQ-ACK information bit as described in Table 18-1 where resourceList is replaced by sps-PUCCH-AN-ListMulticast configured in pucch-ConfigurationListMulticast2.

In addition, note that sps-PUCCH-AN-ListMulticast can configured at most 4 PUCCH resources which can’t support the case with more than 2 HARQ-ACK information bits in a slot. Thus, whether/how to support of more than 2 TBs in a slot for only multicast SPS PDSCHs with the second HARQ-ACK reporting mode needs further consideration. One simple option is not to support the case to provide more than 2 HARQ-ACK information bits for multicast SPS PDSCHs with second HARQ-ACK reporting mode when moreThanOneNackOnlyMode is provided. Another option is if there are more than 2 TBs for multicast SPS PDSCHs with second HARQ-ACK reporting mode and moreThanOneNackOnlyMode is provided, UE provides the HARQ-ACK information according to the first reporting mode and determines a PUCCH resource to provide the HARQ-ACK information from sps-PUCCH-AN-ListMulticast configured in pucch-ConfigurationListMulticast1 or sps-PUCCH-AN-List configured in pucch-ConfigurationList. For example, UE determines the PUCCH resource to provide the HARQ-ACK information from sps-PUCCH-AN-ListMulticast configured in pucch-ConfigurationListMulticast1 if provided; otherwise, the UE determines the PUCCH from sps-PUCCH-AN-List configured in pucch-ConfigurationList.

Proposal 2: When a UE provides only HARQ-ACK information bits for multicast SPS PDSCHs according to the second HARQ-ACK reporting mode and moreThanOneNackOnlyMode is provided,

- When the number of HARQ-ACK information bits is no more than two, the UE provides HARQ-ACK information according to the second HARQ-ACK reporting mode and determines a PUCCH to provide the HARQ-ACK information bits from sps-PUCCH-AN-ListMulticast configured in pucch-ConfigurationListMulticast2.

- Option 1: When the number of HARQ-ACK information bits is more than two, the UE provides HARQ-ACK information according to the first HARQ-ACK reporting mode and determines a PUCCH to provide the HARQ-ACK information bits from sps-PUCCH-AN-ListMulticast configured in pucch-ConfigurationListMulticast1 or sps-PUCCH-AN-List configured in pucch-ConfigurationList.

- Option 2: UE does not expect to provide more than two HARQ-ACK information bits only for multicast SPS PDSCHs according to the second HARQ-ACK reporting mode.

- Conclusion

In this contribution, we discuss PUCCH resources for SPS PDSCHs with NACK only feedback mode, and the following proposals are made.

Proposal 1: When a UE provides only HARQ-ACK information bits for multicast SPS PDSCHs according to the second HARQ-ACK reporting mode and moreThanOneNackOnlyMode is provided, the UE determines a PUCCH to provide the HARQ-ACK information bit from sps-PUCCH-AN-ListMulticast configured in pucch-ConfigurationListMulticast2 should be captured in the specification of TS 38.213.

Proposal 2: When a UE provides only HARQ-ACK information bits for multicast SPS PDSCHs according to the second HARQ-ACK reporting mode and moreThanOneNackOnlyMode is provided,

- When the number of HARQ-ACK information bits is no more than two, the UE provides HARQ-ACK information according to the second HARQ-ACK reporting mode and determines a PUCCH to provide the HARQ-ACK information bits from sps-PUCCH-AN-ListMulticast configured in pucch-ConfigurationListMulticast2.

- Option 1: When the number of HARQ-ACK information bits is more than two, the UE provides HARQ-ACK information according to the first HARQ-ACK reporting mode and determines a PUCCH to provide the HARQ-ACK information bits from sps-PUCCH-AN-ListMulticast configured in pucch-ConfigurationListMulticast1 or sps-PUCCH-AN-List configured in pucch-ConfigurationList.

- Option 2: UE does not expect to provide more than two HARQ-ACK information bits only for multicast SPS PDSCHs according to the second HARQ-ACK reporting mode.

References

[1]	RAN WG1 #110bis e-meeting Chairman’s Notes