3GPP TSG RAN WG1 #111		R1-2211037

Toulouse, France, November 14th – 18th, 2022

Source:	vivo

Title:	Remaining issues on NR Dynamic spectrum sharing

Agenda Item:	9.17

Document for:	Discussion and Decision

# Introduction

In the previous meeting, the following agreements were made for PDCCH reception on CRS symbol. In the contribution, we provide our views on some remaining issues regarding this aspect.

Agreement

If the feature “Reception of NR PDCCH candidates that overlap with LTE CRS REs” is supported, it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation.

Agreement

Reception of NR PDCCH candidates that overlap with LTE CRS REs is supported by Rel18 UEs

PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15 from UE side.

Note: depends on UE capability

Following options can be used for PDCCH-DMRS channel estimation

- legacy CE assumption

  - RAN1 consider support this, if no RAN4 performance requirements are defined

- CE on clean symbol(s) only (this channel estimation option does not apply for 1 symbol CORESET)

Note: Restriction on the symbols and/or LTE CRS patterns applicable for above agreements can be considered during UE capability session.

# Discussion

## UE capability for CE/PDCCH decoding

As usual, UE capability is needed to help NW identify whether UE supports R18 DSS as well as the supported CE options for R18 DSS. The following proposal was discussed in the last meeting.

Proposal-2: Introduce UE capability (or capabilities) to indicate supported channel estimation for the reception of NR PDCCH candidates that overlap with LTE CRS REs - details to be discussed during UE capability discussions (e.g. CE on all PDCCH-DMRS symbols, CE on clean symbol(s) PDCCH-DMRS only)

As RAN4 has so far not verified any additional requirements for legacy CE, RAN1 can discuss whether separate capabilities or a single capability with multiple candidate CE option is required to indicate the following cases:

- option1. Support CE on all PDCCH-DMRS symbols

- option2. Support CE on clean symbol(s) PDCCH-DMRS only

- option3. option1 and option2

Alternatively, it was also proposed to report the supported necessary condition of channel estimation for the reception of NR PDCCH candidates that overlap with LTE CRS RE. In our understanding, either way works. If the necessary condition of channel estimation is to report, the condition should include whether the UE supports R18 DSS for 1-symbol CORESET and whether the UE requires at least one-clean symbol for PDCCH reception. For example, if the UE reports that it supports R18 DSS but does not support 1-symbol CORESET for R18 DSS, it means the UE supports CE on clean symbol(s) only.

Proposal 1. Regarding UE capability(ies), consider one of the following: 
-  Alt1. Indicate the supported CE mechanism (e.g., including support CE on all PDCCH-DMRS symbols, support CE on the clean symbol(s) PDCCH-DMRS only, support CE either on clean symbol(s) PDCCH-DMRS only or on all PDCCH-DMRS symbols); 
-  Alt2. Indicate the unsupported/supported CORESET configuration (e.g., including whether supports 1-symbol CORESET overlapping with CRS, whether requires multi-symbol CORESET or whether requires the presence of at least one clean symbol).

## RRC configuration

Alt-1: A new RRC parameter to enable reception of NR PDCCH candidates overlapped with LTE CRS REs Alt-2: Specify configuration of PDCCH candidates/aggregation levels of a search space that is to be monitored when overlapped with LTE CRS REs Alt-3: Specify that PDCCH aggregation levels higher than the aggregation levels employed in the CSS during the transition from RRC Idle to RRC connected state are assumed to be monitored

It was discussed in the last meeting whether a new RRC parameter is needed to enable the reception of NR PDCCH candidates overlapped with LTE CRS REs. Some companies commented that UE will automatically determine whether the R18 DSS feature is enabled based on CORESET configuration. However, this would lead to some issues. First, it is worth noting that RAN2 provides the following guideline in LS R2-2002378, which means gNB should always configure the UE explicitly by DL RRC signaling, respecting the reported capabilities.

5 Avoid defining functionality that has no RRC configuration but is dependent on capability bits. The specification should not be written so that the network determines what configuration it can use for a UE implicitly by the reported UE capabilities. Instead, the gNB should always configure the UE explicitly by DL RRC signalling, respecting the reported capabilities. A problematic case in Rel-15 was the UL/DL MIMO layers, which resulted in a late-stage introduction of explicit MIMO signalling support by RAN2 (maxLayersMIMO-Indication).

Second, starting from R15, NW is allowed to configure a CORESET with precoderGranularity = sameAsREG-bundle overlapping with CRS symbol. The UE drops the candidate overlapped with CRS and decodes the clean ones. Without the RRC indication proposed, the R18 UE has no information on whether the NW support R18 DSS or not, then it does not know whether to drop or decode the PDCCH candidate overlapped with CRS. If the R18 UE always buffers the overlapping part and performs PDCCH reception, it consumes unnecessary power in the case where NW is not deploying R18 DSS because NW may even never use the overlapping part for PDCCH transmission.

Furthermore, when the NW enables R18 DSS and the UE supports both CE mechanisms, which CE mechanism is used should be determined, considering the PDCCH decoding performance. According to the previous simulation results, the PDCCH performance of the different receivers is heavily dependent on the PDCCH decoding and CE mechanisms. When the PSD ratio of CRS and PDCCH is 1:1, the PDCCH performance of legacy receivers (i.e., legacy PDCCH decoding+legacy CE) deteriorates significantly compared with R17, and brings unacceptable performance losses especially when the AL is small, while receivers capable of PDCCH puncturing and CE based on clean symbol can achieve an attractive gain in most cases. If UE performs PDCCH puncturing but performs legacy CE, the PDCCH performance is worse than the case with PDCCH puncturing and advanced CE, but it still outperforms the legacy receiver. On the other hand, when the CRS PSD is relatively low, legacy receivers may have better performance. Thus, it is important to apply a suitable CE mechanism to achieve the best performance.

It was also discussed whether an RRC indication should be introduced to explicitly indicate which CE mechanism to use. Some companies commented that it should be up to UE implementation to use the CE mechanism with the best performance. However, it is questionable whether the UE can identify which CE mechanism has the best performance without any guidance from NW. Furthermore, leaving the selection of CE mechanism entirely to the UE also means that the gNB shall guarantee PDCCH decoding performance to meet the requirements in the worst case, which could be difficult or impossible from the network perspective. Additionally, from the NW side, it may not transmit PDCCH/DMRS on CRS REs to protect LTE RAT and in this case ‘CE on clean symbol(s) only’ is preferred. Therefore, RRC configuration should be introduced to suggest the preferred CE to be used. If the gNB indicates two CE mechanisms, the UE can select one of them. Moreover, if the RRC indication is introduced, the UE is not expected to be indicated with 'clean symbol based CE' by the gNB for a single symbol CORESET overlapped with a CRS symbol.

The RRC indicator to enable R18 DSS can be also used to indicate the preferred CE mechanism, thus we have the following proposal

Proposal 2. Introduce a RRC indicator to inform UE whether and how to apply R18 DSS CE mechanism:
-  if the RRC indicator is provided, R18 DSS is enabled;
-  if the RRC indicator is not provided, R18 DSS is disabled;
-  the RRC indicator may indicate one or more CE mechanisms to be used, and when the RRC indicator indicates two CE mechanisms, it is up to UE to choose which CE mechanism to use.

Proposal 3. For 1-symbol CORESET that is overlapped with CRS, UE is not expected to be indicated to perform 'clean symbol-based CE' by the RRC indicator.

## CORESET for PDCCH reception on CRS symbol

One issue that needs to be discussed is whether this R18 DSS feature can be used for any CORESETs. Common CORESETs are shared by idle/inactive UEs and connected R18 UEs, and should not overlap with CRS symbols. Otherwise, there would be a negative impact on the PDCCH reception performance of idle UEs.

Proposal 4. Reception of NR PDCCH candidates that overlap with LTE CRS REs is not supported for common CORESET that is configured at least for idle/inactive UE.

## Clarification on UE behavior when two CRS lists are configured

In R17, if a PDCCH candidate is overlapped with the CRS pattern indicated by either lte-CRS-ToMatchAround (included in ServingCellConfigCommon or ServingCellConfig) or lte-CRS-PatternList1/lte-CRS-PatternList2 (included in ServingCellConfig), UE is not expected to monitor the PDCCH candidate. Moreover, CORESET with precoderGranularity = allContiguousRBs should not be overlapped with CRS pattern indicated by lte-CRS-ToMatchAround or lte-CRS-PatternList1/lte-CRS-PatternList2.

TS 38.213 h.3.0 If at least one RE of a PDCCH candidate for a UE on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, or of LTE-CRS-PatternList, the UE is not required to monitor the PDCCH candidate. ==omitted== When precoderGranularity = allContiguousRBs, a UE does not expect - to be configured a set of resource blocks of a CORESET that includes more than four sub-sets of resource blocks that are not contiguous in frequency - any RE of a CORESET to overlap with any RE determined from lte-CRS-ToMatchAround, or from LTE-CRS-PatternList, or with any RE of a SS/PBCH block.

In R18 DSS, the UE may be configured to receive the PDCCH overlapping with one CRS pattern list, and at the same time may also be configured to receive PDSCH overlapping with two CRS pattern lists. If two CRS pattern lists are configured and both lists are configured with 4 CRS ports, there can be up to 8 CRS REs on one RB of PDCCH symbol overlapping with CRS, in which case UE can hardly decode any PDCCH properly. Therefore,  receiving a PDCCH overlapping with two CRS pattern lists is not supported in R18 DSS, which was the common understanding during the WID discussion. Nevertheless, when two CRS pattern lists are configured, it is still not clear whether a UE is required to receive a PDCCH overlapping with only one CRS pattern list, or not (i.e., the two features cannot be enabled simultaneously). Further, even if the UE is required to receive the PDCCH, it is not clear whether that PDCCH is expected to overlap with lte-CRS-PatternList1 only, lte-CRS-PatternList2 only, lte-CRS-PatternList3 only, lte-CRS-PatternList4 only or any one of them.

To address the aforementioned issue, the following alternatives were proposed by FL in the last meeting. In our understanding, alt-2 leads to serious performance loss when more than one CRS pattern are overlapped with NR PDCCH on the same set of RBs. In contrast, alt-1 is a much more realistic approach – for the overlapping RB between a LTE carrier and NR PDCCH, only one of the CRS pattern lists is expected to be overlapped with NR PDCCH if UE is required to monitor PDCCH candidates in a CORESET overlapping with CRS. Nevertheless, further clarification is needed that alt-1 is for a LTE frequency instead of all LTE frequencies.

- Alt-1: Reception of NR PDCCH candidates overlapped with LTE CRS REs from only one configured LTE CRS pattern is supported - Alt-2: Reception of NR PDCCH candidates overlapped with LTE CRS REs from one or multiple LTE CRS patterns of either one or two configured LTE CRS pattern lists is supported

Proposal 5. Reception of NR PDCCH candidates that overlap with LTE CRS REs of a LTE carrier can be supported when the PDCCH candidates are overlapped with only one LTE CRS REs from only one configured LTE CRS pattern from one CRS pattern list for that LTE carrier.

## Clarification on wideband CORESET

In R15, CORESET configured with wideband DMRS is not expected to overlap with CRS while CORESET with narrowband DMRS can be partially overlapped with CRS, this principle should be retained in R18 DSS. With the newly introduced CRS pattern list3/4, wideband DMRS CORESET is still not expected to collide with CRS of either CRS pattern list, while narrowband DMRS CORESET can be only overlapped with lte-CRS-PatternList3.

Proposal 6. In R18 DSS, CORESET with precoderGranularity = sameAsREG-bundle can be overlapped with RE determined from LTE-CRS-PatternList of up to one CRS pattern list (i.e., lte-CRS-PatternList1 or lte-CRS-PatternList3) per LTE carrier.

Proposal 7. CORESET with precoderGranularity = allContiguousRBs is not expected to overlap with CRS RE determined from any LTE-CRS-PatternList.

# Conclusion

In this contribution, we have the following proposals.

Proposal 1. Regarding UE capability(ies), consider one of the following: 
-  Alt1. Indicate the supported CE mechanism (e.g., including support CE on all PDCCH-DMRS symbols, support CE on the clean symbol(s) PDCCH-DMRS only, support CE either on clean symbol(s) PDCCH-DMRS only or on all PDCCH-DMRS symbols); 
-  Alt2. Indicate the unsupported/supported CORESET configuration (e.g., including whether supports 1-symbol CORESET overlapping with CRS, whether requires multi-symbol CORESET or whether requires the presence of at least one clean symbol).

Proposal 2. Introduce a RRC indicator to inform UE whether and how to apply R18 DSS CE mechanism:
-  if the RRC indicator is provided, R18 DSS is enabled;
-  if the RRC indicator is not provided, R18 DSS is disabled;
-  the RRC indicator may indicate one or more CE mechanisms to be used, and when the RRC indicator indicates two CE mechanisms, it is up to UE to choose which CE mechanism to use.

Proposal 3. For 1-symbol CORESET that is overlapped with CRS, UE is not expected to be indicated to perform 'clean symbol-based CE' by the RRC indicator.

Proposal 4. Reception of NR PDCCH candidates that overlap with LTE CRS REs is not supported for common CORESET that is configured at least for idle/inactive UE.

Proposal 5. Reception of NR PDCCH candidates that overlap with LTE CRS REs of a LTE carrier can be supported when the PDCCH candidates are overlapped with only one LTE CRS REs from only one configured LTE CRS pattern from one CRS pattern list for that LTE carrier.

Proposal 6. In R18 DSS, CORESET with precoderGranularity = sameAsREG-bundle can be overlapped with RE determined from LTE-CRS-PatternList of up to one CRS pattern list (i.e., lte-CRS-PatternList1 or lte-CRS-PatternList3) per LTE carrier.

Proposal 7. CORESET with precoderGranularity = allContiguousRBs is not expected to overlap with CRS RE determined from any LTE-CRS-PatternList.