3GPP TSG RAN WG1 #111		R1-2211226

Toulouse, France, November 14th – 18th, 2022

Agenda Item:     9.2.1

Source:	Spreadtrum Communications

Title:	             Discussion on general aspects of AI/ML framework

Document for:	Discussion and decision

Introduction

In RAN#94e meeting, one SID on AI/ML for Air interface is approved [1]. The objective of the study item is to study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact, and set up a common AI/ML framework, including functional requirements of AI/ML architecture, which could be used in future projects.

In this paper, we would share our opinions on the general aspects of AI/ML framework.

Discussion

Collaborative level

Regarding the boundary among level x, y and z, last meeting [2], after extensive discussion, we have the following agreement and working assumption.

Agreement Clarify Level x/y boundary as: - Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. - (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.) Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not. - Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z - Clarifying note: Level y includes cases without model delivery.

Regarding level y and level z, literally the difference lies in whether there exists model transfer from the perspective of signalling interactive between network side and UE side. During previous meetings, there were some discussions on the boundary between level y and level z. The controversial point is that proprietary model deliver is level y or level z. Some people think that even if for non-3gpp-based model delivery (e.g., via application layer from OAM or OTT sever), it also may belong to level z, since there still perhaps exist signalling interactive between NW and UE, e.g., model ID, the input/output of model, and so on. Some people think that the proprietary model deliver is level y, due to that the model representation format (MRF) is not needed to be defined in 3GPP.

For proprietary model delivery, in our mind, it can be level x, or level y, or level z. For example, for level x, UE or NW may also need to download the model from its private server while all AI/ML related operations are transparent to network/UE. For both level y and level z, the AI/ML operations are not transparent to specification. The boundary between y and z lies in that whether the model delivery procedure is transparent to spec. For example, even if for proprietary model delivery, if there is specific procedure to demonstrate model delivery, e.g., one dedicated PDU session or DRB, it can be level z. While for level y, proprietary model delivery perhaps would happen but it is transparent to 3GPP by utilizing the legacy 3GPP signalling. Thus, we think the working assumption could confirmed.

Proposal 1: Confirm the working assumption on the definition of Level y-z boundary.

Last meeting, we also discussed about the terminology of delivery transfer. There was one proposal proposed by FL [3] shown below:

| AI/ML model transfer | Delivery of an AI/ML model over the air interface with 3GPP standardized mechanism, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. FFS: Model being transferred may be in a proprietary format or a standardized format. |
| --- | --- |

As aforementioned, even if for proprietary model, it also can be delivered through 3GPP network and the model delivery signaling/procedure can be transparent or not to 3GPP. The Level y-z boundary lies in whether the model delivery signaling/procedure is transparent to 3GPP or not. The delivered model can be proprietary format, or a standardized format. Since model transfer is used to identify the boundary between level y and level z, corresponding to the the definition of Level y-z boundary, the terminology of AI/ML model transfer can be considered as below:

- Delivery of an AI/ML model over the air interface with non-transparent 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model.

Proposal 2: Suggest to consider the following terminology definition:

- AI/ML model transfer: Delivery of an AI/ML model over the air interface with non-transparent 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model.

Functional Framework of AI/ML

TR37.817 also provides one functional framework for AI/ML in RAN level, illustrated in below Figure 1.

Figure 1. Functional Framework for RAN Intelligence

The detailed explanations of all terminologies in the framework or Fig.1 can be found in TR37.817 [4]. In our opinion, the functional framework also could be referenced for AI/ML enabled in air interface, while some enhancement can be considered if needed.

Proposal 3: The general framework of TR37.817 (i.e., Section 4 of TR37.817) also can be as the starting point of AI functional framework for the study. Further enhancements also can be considered if needed.

Lifecycle management of AI/ML model

The lifecycle of AI/ML model typically includes these stages of model training, model validation, model test, model deployment, model inference, model monitoring, and model update. In general, model generation includes model training, model validation, and model test. Next, we would provide our views on these stages of AI/ML model.

- Model training

Last meeting, we discussed about the training type for one-sided model and two-sided model. The majority think that the part related to two-sided model can be left to AI CSI use case to discuss. Although we think it can be discussed in AI9.2.1 to provide general guidance for two-sided model other than CSI spatial-frequency compression use case, but we are fine to discuss one-sided model firstly in AI9.2.1.

Generally, for one-sided model, there are two training types to be considered. One type refers that both training and inference take place at the same node, while for another type training operation and inference operation are at different nodes. Thus, we have the following proposal for one-sided model training:

Proposal 4: For model training for one-sided model, the following model training types can be further discussed:

- Type 0: Training at a single side/entity without model transfer

- Type 1: Training at a single side/entity, and model transfer to another side/entity

In the following, the potential specification, pros and cons of the above two training types would be provided from our side.

Type 0:

For type 0, training and reference take place at the same node. It only applies for one-sided model.

- Potential spec impact:

  - The signaling and/or procedures for training dataset exchange if required.

- Pros:

  - Match with UE’s or NW’s hardware;

  - AI/ML model proprietary can be kept.

- Cons

  - Large overhead due to training dataset exchange if required.

Type 1:

For type 1, AI/ML model can be trained at UE side or gNB side, which applies for both one-sided model and two-sided model. For two-sided model, only part of model would be transferred to another side different from training side while all parts of model is needed to be delivered to another side for one-sided model.

- Potential spec impact:

- AI/ML model transfer

  - Model representation format (MRF) is needed to be defined in 3GPP to enable private implementation among different sides. From the perspective of specification, the procedure of AI/ML model transfer and the signaling about MRF of one model may be needed.

- Pros:

  - If the model is trained at UE side,

      - It can match with UE’s hardware.

  - If the model is trained at NW side,

      - The model can match with gNB’s hardware;

      - Optimized loss function can be considered for MU-MIMO, CJT, NCJT, etc.

- Cons

  - If the model is trained at UE side,

      - Multiple models are needed to be stored/maintained/executed by NW, to receive from multiple UEs;

      - Loss function may be not optimized for MU-MIMO, CJT, NCJT, and so on. It may not match with NW implementation.

  - If the model is trained at NW side,

      - Multiple models may be needed to be stored/maintained/executed by UE if different NW has different AI/ML model;

      - UE may not support the AI/ML model delivered by gNB, since gNB has the stronger processing capability. In addition, if the AI/ML model not well matching with UE’s implementation, it would result in low efficiency and high power consumption.

Observation 1: For AI/ML model training Type 1, AI/ML model possibly can not be executed, due to incompatibility issue between NW side and UE side.

Observation 2: AI/ML model proprietary can be kept for AI/ML model training Type 0

Based on the above, we have the following proposal:

Proposal 5: For one-sided AI/ML model, model training and model inference at the same side should be considered as a starting point, i.e.,

- Network-side training for Network-side model

- UE-side training for UE-side model

For online training of AI/ML model, additional complexity is needed, e.g., a large amount of data needed to be stored, and tackled. It can not be born at UE side, e.g, for smartphone. Thus, we suggest to focus on offline AI/ML model training at the first stage. Online training for advanced UE, e.g., laptop, can be considered in future.

Proposal 6: Offline AI/ML model training is the first priority.

- Model registration

Since collaboration level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement collaboration between network and UE, we focus on level y and level z where there is signaling/procedure related to AI/ML operation, we think model registration only can be applicable for collaboration level y and level z.

For UE-sided model, and two-sided model, in our understanding, it is necessary to do model registration at NW side, to make NW well understand UE’s capability. For example, if gNB can be aware that UE is capable of beam prediction in spatial domain, RS overhead reduction can be achieved. For NW-sided model, we have not seen clear motivation to do model registration since the management of AI/ML model can be up to NW.

Observation 3: At least for UE-sided model and two-sided model, model registration at NW side is necessary.

Last meeting [2], after extensive discussion, finally we have agreed to introduce model ID with associated information to demonstrate one AI/ML model for the study of LCM procedure.

Agreement Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models FFS: Detailed discussion of model ID with associated information and/or model functionality. FFS: usage of model ID with associated information and/or model functionality based LCM procedure FFS: whether support of model ID FFS: the detailed applicable AI/ML operations

From our side, model ID can be utilized for model registration to denote the identification of one AI/ML model. Other than model ID, the following information also can be considered to facilitate model registration for effective AI/ML model management at gNB side:

    - Model function

      - If gNB doesn’t know the function of AI/ML model, it is impossible for gNB to manage AI/ML model. Model function should be one of basic components of AI/ML model registration.

    - Model applicable condition

      - It can be discussed case by case. For example, when only one large model is supported by UE for CSI predication, there seems no necessary to register the applied condition. After all, AI/ML model monitoring procedure can be considered by gNB or UE, to carry out model activation/deactivation. When a family of AI/ML model are supported for the same function, each AI/ML model is applicable for different scenarios, e.g., Dense urban, Urban, Rural. It can help NW effectively management AI/ML model if the model applied condition also included.

    - Model complexity

      - Generally, the model complexity can be considered. For example, if UE supports multiple AI/ML models for different configurations, it doesn’t mean that UE must support them simultaneously. How to define/reflect the complexity in model registration can be further discussed.

    - Model input

      - It can be optionally considered, depending on specific use case. For example, for CSI prediction, the input totally can be up to UE’s implementation. However, for CSI compression and recovery sub use case, the input for CSI generation part may be needed to match with the output of CSI recovery part at gNB side.

    - Model output

      - Like model input, it can be optionally considered, depending on specific use case. For example, for CSI prediction, the AI/ML output can be up to UE’s implementation, e.g, the output can be predicted channel, or predicted PMI. However, for CSI compression and recovery sub use case, the output for CSI generation part may be needed to match with the input of CSI recovery part at gNB side.

For the inference latency, in our understanding, the latency for all AI/ML models should not be beyond the current computation latency requirement. As to real inference latency, it can be up to device’s implementation. Thus, there is no need to register the inference latency for this model.

Proposal 7: For AI/ML model registration, the following information can be considered:

    - Model ID;

    - Model function;

    - Model applicable condition;

    - Model complexity;

    - Model input;

    - Model ouput;

- AI/ML model selection/activation/deactivation/switching

Last meeting [2], after discussion, we have the following agreement for UE sided models and two-sided models.

Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models FFS: other mechanisms

In general, the decision on model selection/activation/deactivation/switching can be based on model monitoring performance either for decision by the Network or the UE, e.g., inference accuracy, system, data distribution, applicable condition and so on. For decision by the UE, other factors such power consumption, the availability of computation resource, also can be considered.

For network sided models, network also could do the decision based on the monitoring performance. Since the network is generally responsible for the system performance and it is the experiencer, the decision by network seems to be enough. Especially, if UE doesn’t be aware of AI/ML operation at network side, all of  AI/ML operations can be up to network’ s implementation.

Proposal 8: For model selection, activation, deactivation, switching, and fallback for network sided models, study the following mechanisms:

- Decision by the network

  - Network-initiated

Conclusion

In this contribution, we provide our opinions on general aspects of AI/ML:

Observation 1: For AI/ML model training Type 1, AI/ML model possibly can not be executed, due to incompatibility issue between NW side and UE side.

Observation 2: AI/ML model proprietary can be kept for AI/ML model training Type 0

Observation 3: At least for UE-sided model and two-sided model, model registration at NW side is necessary.

Proposal 1: Confirm the working assumption on the definition of Level y-z boundary.

Proposal 2: Suggest to consider the following terminology definition:

- AI/ML model transfer: Delivery of an AI/ML model over the air interface with non-transparent 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model.

Proposal 3: The general framework of TR37.817 (i.e., Section 4 of TR37.817) also can be as the starting point of AI functional framework for the study. Further enhancements also can be considered if needed.

Proposal 4: For model training for one-sided model, the following model training types can be further discussed:

- Type 0: Training at a single side/entity without model transfer

- Type 1: Training at a single side/entity, and model transfer to another side/entity

Proposal 5: For one-sided AI/ML model, model training and model inference at the same side should be considered as a starting point, i.e.,

- Network-side training for Network-side model

- UE-side training for UE-side model

Proposal 6: Offline AI/ML model training is the first priority.

Proposal 7: For AI/ML model registration, the following information can be considered:

    - Model ID;

    - Model function;

    - Model applicable condition;

    - Model complexity;

    - Model input;

    - Model ouput;

Proposal 8: For model selection, activation, deactivation, switching, and fallback for network sided models, study the following mechanisms:

- Decision by the network

  - Network-initiated

Reference

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- Draft Report of 3GPP TSG RAN WG1 #110b-e

- R1-2210661, “Summary#5_9.2.1_v000_Mod,” Moderator (Qualcomm)

- 3GPP TR 37.817: " Study on enhancement for Data Collection for NR and EN-DC "