3GPP TSG RAN WG1 #111		       	R1-2211355

Toulouse, France, November 14th – 18th, 2022

Agenda item:	9.2.2.1

Source:	            		Xiaomi

Title:	Discussion on evaluation on AI/ML for CSI feedback enhancement

Document for:	Decision

Introduction

In RAN1#110bis e-Meeting[1], the following agreements achieved on evaluation on AI/ML for CSI feedback enhancement:

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Traffic model’ in the baseline of EVM is captured as follows: Traffic model; At least, FTP model 1 with packet size 0.5 Mbytes is assumed Other options are not precluded.. Agreement In the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, if realistic DL channel estimation is considered, regarding how to calculate the intermediate KPI of CSI accuracy, - Use the target CSI from ideal channel and use output CSI from the realistic channel estimation - The target CSI from ideal channel equally applies to AI/ML based CSI feedback enhancement, and the baseline codebook Note: there is no restriction on model training Agreement In the evaluation of the AI/ML based CSI feedback enhancement, for “Baseline for performance evaluation” in the EVM table, Type I Codebook (if it outperforms Type II Codebook) can be optionally considered for comparing AI/ML schemes up to companies - Note: Type II Codebook is baseline as agreed Conclusion If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for the outdoor UEs, add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles. Conclusion If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, no explicit trajectory modeling is considered for evaluation Conclusion If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, and if the AI/ML model outputs multiple predicted instances, the intermediate KPI is calculated for each prediction instance Conclusion If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, both of the following types of AI/ML model input are considered for evaluations: - Raw channel matrixes - Eigenvector(s) Conclusion If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for the evaluation of CSI prediction: - Companies are encouraged to report the assumptions on the observation window, including number/time distance of historic CSI/channel measurements as the input of the AI/ML model, and - Companies to report the assumptions on the prediction window, including number/time distance of predicted CSI/channel as the output of the AI/ML model Conclusion If ideal DL channel estimation is considered (which is optional) for the evaluations of CSI feedback enhancement, there is no consensus on how to use the ideal channel estimation for dataset construction, or performance evaluation/inference. - It is up to companies to report whether/how ideal channel is used in the dataset construction as well as performance evaluation/inference. Conclusion For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), following procedure is considered as an example: - For each FP/BP loop, - Step 1: UE side generates the FP results (i.e., CSI feedback) based on the data sample(s), and sends the FP results to NW side - Step 2: NW side reconstructs the CSI based on FP results, trains the CSI reconstruction part, and generates the BP information (e.g., gradients), which are then sent to UE side - Step 3: UE side trains the CSI generation part based on the BP information from NW side - Note: the dataset between UE side and NW side is aligned. - Other Type 2 training approaches are not precluded and reported by companies Conclusion For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with NW side training (NW-first training): - Step1: NW side trains the NW side CSI generation part (which is not used for inference) and the NW side CSI reconstruction part jointly - Step2: After NW side training is finished, NW side shares UE side with a set of information (e.g., dataset) that is used by the UE side to be able to train the UE side CSI generation part - Step3: UE side trains the UE side CSI generation part based on the received set of information - Other Type 3 NW-first training approaches are not precluded and reported by companies Conclusion For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with UE side training (UE-first training): - Step1: UE side trains the UE side CSI generation part and the UE side CSI reconstruction part (which is not used for inference) jointly - Step2: After UE side training is finished, UE side shares NW side with a set of information (e.g., dataset) that is used by the NW side to be able to train the CSI reconstruction part - Step3: NW side trains the NW side CSI reconstruction part based on the received set of information - Other Type 3 UE-first training approaches are not precluded and reported by companies Working assumption In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors - Note: Eventual KPI can still be used to compare the performance Agreement For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, at least Method 3 is adopted, FFS whether additionally adopt a down-selected metric between Method 1 and Method 2. - Method 1: Average over all layers - Method 2: Weighted average over all layers SGCS=E1Ni=1NjKλijk=1Kλik¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦2 where wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the jth output vector of the output CSI of resource unit i. N is the total number of resource units. E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij. - Method 3: SGCS is separately calculated for each layer (e.g., for K layers, K SGCS values are derived respectively, and comparison is performed per layer) Agreement In CSI compression using two-sided model use case, evaluate and study quantization of CSI feedback, including at least the following aspects: - Quantization non-aware training - Quantization-aware training - Quantization methods including uniform vs non-uniform quantization, scalar versus vector quantization, and associated parameters, e.g., quantization resolution, etc. - How to use the quantization methods Agreement For evaluating the performance impact of ground-truth quantization in the CSI compression, study high resolution quantization methods for ground-truth CSI, e.g., including at least the following options - High resolution scalar quantization, e.g., Float32, Float16, etc. - FFS select one of the scalar quantization resolutions as baseline - High resolution codebook quantization, e.g., R16 Type II-like method with new parameters - FFS new parameters - Other quantization methods are not precluded Agreement For the evaluation of the potential performance benefits of model fine-tuning of CSI feedback enhancement which is optionally considered by companies, the following case is taken - The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B - Company to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance Agreement For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following cases are considered for evaluations: - Case 1 (baseline): Aligned AI/ML model structure between NW side and UE side - Case 2: Not aligned AI/ML model structures between NW side and UE side - Companies to report the AI/ML structures for the UE part model and the NW part model, e.g., different backbone (e.g., CNN, Transformer, etc.), or same backbone but different structure (e.g., number of layers) - FFS different sizes of datasets between NW side and UE side - FFS aligned/different quantization/dequantization methods between NW side and UE side - FFS: whether/how to evaluate the case where the input/output types and/or pre/post-processing are not aligned between NW part model and UE part model Agreement For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), the following evaluation cases are considered for multi-vendors, - Case 1 (baseline): Type 2 training between one NW part model to one UE part model - Case 2: Type 2 training between one NW part model and M>1 separate UE part models - Companies to report the AI/ML structures for the UE part model and the NW part model - FFS Companies to report the dataset used at UE part models, e.g., whether the same or different dataset(s) are used among M UE part models - Case 3: Type 2 training between one UE part model and N>1 separate NW part models - Companies to report the AI/ML structures for the UE part model and the NW part model - FFS Companies to report the dataset used at NW part models, e.g., whether the same or different dataset(s) are used among N NW part models - FFS N NW part models to M UE part models - FFS different quantization/dequantization methods between NW and UE - FFS: whether/how to evaluate the case where the input/output types and/or pre/post-processing are not aligned between NW part model and UE part model - FFS: companies to report the training order of UE-NW pair(s) in case of M UE part models and/or N NW part models - FFS: whether/how to report overhead Agreement For the evaluation of the AI/ML based CSI compression sub use cases, at least the following types of AI/ML model input (for CSI generation part)/output (for CSI reconstruction part) are considered for evaluations - Raw channel matrix, e.g., channel matrix with the dimensions of Tx, Rx, and frequency unit - Companies to report the raw channel is in frequency domain or delay domain - Precoding matrix - Companies to report the precoding matrix is a group of eigenvector(s) or an eType II-like reporting (i.e., eigenvectors with angular-delay domain representation) - Other input/output types are not precluded - Companies to report the combination of input (for CSI generation part) and output (for CSI reconstruction part), - Note: the input and output may be of different types Conclusion If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for SLS, spatial consistency procedure A with 50m decorrelation distance from 38.901 is used (if not used, company should state this in their simulation assumptions) - UE velocity vector is assumed as fixed over time in Procedure A modeling Agreement In the evaluation of the AI/ML based CSI feedback enhancement, for the calculation of intermediate KPI, the following is considered as the granularity of the frequency unit for averaging operation - For 15kHz SCS: For 10MHz bandwidth: 4 RBs; for 20MHz bandwidth: 8 RBs - For 30kHz SCS: For 10MHz bandwidth: 2 RBs; for 20MHz bandwidth: 4 RBs Note: Other frequency unit granularity is not precluded and reported by companies

In this contribution, we present our views on evaluation methodology and evaluation results on the generalization performance and performance results for AI based CSI compression.

Discussion

Metric of AI model

In last meeting [1], at least Method 3 is adopted for intermediate KPI calculation, down selection may needed between Method 1 and Method 2. For comparing the difference between Method 1 and Method 2, we set rank=2 in the evaluation, two separate AI models are trained by layer 1 eigenvector and layer 2 eigenvector respectively, in other words, layer-specific model is used in the simulation. The SGCS calculated by different methods are shown in Table 1 and as Table 1 shows, method 2 outperforms method 1 as layer 1 is given more weight than layer 2. As Method 3 can already provide sufficient information, method 1 and method 2 seems not necessary. As option 1 can be easily calculated by option 3, we suggest to adopt option 2 which consider the impact of eigenvalue, if down selection is needed.

Table 1, SGCS calculation results by different methods

| Method | Method 1 SGCS | Method 2 SGCS | Method 3 SGCS |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st layer SGCS | 2nd layer SGCS |
| AI 120bit | 0.8339 | 0.8507 | 0.8804 | 0.8036 |

On metric of AI/ML model complexity, we prefer to use number of AI/ML parameters. Considering that the model size is varies with deep learning frameworks, e.g., TensorFlow, PyTorch, while the AI/ML parameter number is framework-independent. A framework-independent metric is preferred for calibration, in that sense, number of model parameters is selected as metric of model complexity.

Proposal 1: Adopt Method 2 as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, if down-selection between Method 1 and Method 2 needed.

Proposal 2: Number of model parameters is selected between AI/ML model size and number of AI/ML parameters as metric of model complexity.

AI/ML model generalization verification

In RAN1#110 meeting [2], different configurations and scenarios were agreed for AI/ML model generalization verification. We present our preliminary evaluation results for different scenarios and different configurations such as various sizes of CSI feedback payloads, different subband numbers in this section, and different training dataset and testing dataset are considered in the evaluation.

A Transformer-based AI model is used in the evaluation for AI based CSI compression. As Fig.1 shows, the AI model including two parts as encoder part and decoder part. The input data is eigenvector wk based on SVD of channel matrix, after linear embedding and positional embedding, the input data are encoded in the encoder. The encoded data then input to the decoder to get recovered eigenvector wk'. In general, the encoding procedure is performed at UE side and UE feedback quantized data to gNB, then the decoding procedure is performed at gNB side.

The simulation assumption for intermediate results calculation is as Table 2 shows, and the AI model training parameters are as Table 3 shows.

Figure 1, The basic structure of Transformer model

Table 2, Simulation assumption for intermediate result calculation

| Parameter | Value |
| --- | --- |
| Scenario | Uma and/or Umi |
| Frequency Range | 2GHz |
| Bandwidth | 10MHz |
| Subcarrier spacing | 15KHz |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| AI content | Eigenvector |
| Channel estimation | Ideal |

Table 3, Parameter of AI model training

| AI training parameter | Value |
| --- | --- |
| Quantization | Scalar quantization |
| Loss function | SGCS |
| Learning rate | 0.001 |
| Optimizer | Adam |
| Epoch | 100 |
| Batchsize | 128 |
| Dataset construction | 210,000 200 drop*21 cell*50 UE*1 samples per UE |
| Sample of training set | 199,500 |
| Sample of validation set | 10000 |
| Sample of test set | 10500 |

Configuration generalization

Generalization of different rank number

We firstly conduct the evaluation for AI model generalization of different rank number. The evaluation result is obtained based on feedback payload equals to 120 bits, the baseline is layer-specific AI models which including two AI models trained by layer 1 eigenvector and layer 2 eigenvector separately. And the layer-common is the results of generalized AI model, which is a single AI model trained by both layer 1 and layer 2 eigenvectors. In last meeting, the disorder issue of unmatched input eigenvectors and output eigenvectors when calculating SGCS is proposed and companies need to ensure the correct calculation. In our simulation, there is no disorder issue as the SGCS is calculated per-layer for both layer-common and layer-specific AI models.

As Table 4 shows, the performance of the baseline and generalized AI model is very close, and the generalized AI model has even better performance than the baseline. It can be observed that the AI/ML model shows good generalization performance in various rank number.

Observation 1: AI/ML model shows good generalization performance in various rank number.

Table 4, SGCS results of rank generalization

|  | 1st layer SGCS | 2nd layer SGCS |
| --- | --- | --- |
| Layer-specific | 0.70 | 0.55 |
| Layer-common | 0.71 | 0.56 |

Generalization of various sizes of CSI feedback payloads

The generalization evaluation of different size of CSI feedback payloads is conducted by parallel training of one encoder part and three decoder parts. The basic AI model for feedback payloads generalization is as Fig.2 shows. Setting the maximum feedback bit number as 240 bit, different feedback payloads are obtained by cutting off the tail of the maximum 240 bit. The output of encoder is always equal to the maximum bit number, while the input of the decoder is the truncated bit of the output of encoder. The loss function is the average results of different decoder parts. In this two-sided model, the UE side only need to deploy one encoder part, and the output of encoder can be various based on gNB’s configuration and/or indication.

The evaluation result is shown in Table 5, where the baseline results is achieved by training one encoder part and one decoder part with different feedback payloads separately, the generalization is results of generalized AI model, which is a single AI model with one encoder part and three decoder parts. As Table 5 shows, the generalized AI model has worse performance than baseline, the less the feedback payloads, the lager the gap. In that sense, it is proposed to study the pre-processing mechanisms for input of decoder to improve the AI model generalization performance on various feedback payloads.

Observation 2: The generalized AI model does not work well on various CSI feedback payloads, especially for a small number of feedback payloads.

Proposal 3: RAN1 study pre-processing mechanisms for the input of decoder to  improve the AI model generalization performance on various feedback payloads.

Figure 2, The basic structure of AI model for feedback payloads generalization

Table 5, SGCS results of feedback payload generalization

|  | 120 bit | 180 bit | 240 bit |
| --- | --- | --- | --- |
| Baseline | 0.70 | 0.78 | 0.83 |
| Generalization | 0.60 | 0.72 | 0.81 |

Generalization of different subband number

We conduct the simulation by using different size of subband. Keeping totally 48 RB unchanged, two options are considered in the simulation. When training the AI model for generalization, 0 is padding to the end of vector for option 2.

- Option 1: 2 RB per subband

- Option 2: 4 RB per subband

The evaluation result is obtained based on feedback payload equals to 120 bits and shown in Table 6, where the baseline results is achieved by training AI model with different subband number separately. As Table 6 shows, the generalization model can achieve similar SGCS performance with the baseline. It can be observed that the AI/ML model shows good generalization performance in various subband number.

Table 6, SGCS results of subband number generalization

|  | 2RB per subband | 4RB per subband |
| --- | --- | --- |
| Baseline | 0.65 | 0.70 |
| Generalization | 0.63 | 0.69 |

Observation 3: AI/ML model shows good generalization performance in different sizes of subband and subband number.

Scenario generalization

In last meeting, different scenarios were agreed for AI/ML model generalization verification. We present our preliminary evaluation results for different scenarios in this section, and all three cases with different training dataset and testing dataset are considered in the evaluation.

Four kinds of training dataset as Uma only, Umi only and mixed Uma and Umi with different ratios are considered, and the AI/ML model is tested by Uma and Umi dataset separately. The evaluation result is obtained based on feedback payload equals to 120 bits and shown in Table 7. As table 7 shows, for Umi testing dataset, the best performance is achieved when the AI/ML model is trained by Umi dataset only, and the worst performance is achieved when the AI/ML model is trained by Uma dataset only. When the training dataset is mixed dataset, the dataset with more Umi data has higher SCGS value. While for Uma testing dataset, the SGCS performance are very close for different training datasets. It can be observed that the AI/ML model shows good generalization performance in various scenarios.

Observation 4: AI/ML model shows good generalization performance in different scenarios.

Table 7, SGCS results of scenarios generalization

|  | Testing dataset |  |
| --- | --- | --- |
| Training dataset | Uma | Umi |
| Uma | 0.70 | 0.66 |
| Umi | 0.69 | 0.71 |
| Uma : Umi=5 : 5 | 0.71 | 0.70 |
| Uma : Umi=8 : 2 | 0.70 | 0.68 |

There is minor complexity and memory storage difference between different evaluation cases. We provide FLOPs and number of AI/ML parameters separately for the CSI generation part and the CSI reconstruction part in Table 8.

Table 8, Complexity and memory storage of AI model

|  | CSI generation part | CSI reconstruction part |  |  |
| --- | --- | --- | --- | --- |
|  | 240 bit | 240 bit | 180 bit | 120 bit |
| FLOPs | 257.97*106 | 257.97*106 | 257.92*106 | 257.88*106 |
| Number of AI/ML model parameters | 10.79*106 | 10.79*106 | 10.77*106 | 10.74*106 |

Fine-tuning performance

In last meeting, the methodology of model fine-tuning was agreed and performance improvement is expected with model fine-tuning. We conduct the evaluation of model fine-tuning in this section. The AI/ML model is trained based on training dataset#1 from Uma, then the trained model is updated based on fine-tuning dataset #2 from Umi. After that, the AI/ML model is tested on dataset from Umi. The amount of data in dataset#2 is around fifteenth of dataset#1.

To make it more easier to compare, we copied the evaluation results from section 2.2.2 to Table 9. As Table 9 shows, the performance improved to 0.673 from 0.655. Setting 0.705 as the upper band of testing results of AI/ML model in Umi scenario, and 0.655 as the lower band of testing results of AI/ML model in Umi scenario. The fine-tuning procedure with much less data can make up half the gap between lower band and upper band.

Observation 5: The fine-tuning procedure with much less data can improve the performance significantly.

Table 9: SGCS performance of AI/ML fine-tuning

| Training dataset | Uma | Umi | Uma + Umi fine-tuning |
| --- | --- | --- | --- |
| Testing dataset | Umi | Umi | Umi |
| SGCS results | 0.655 | 0.705 | 0.673 |

Separate training results

In late meeting, there was big progress on the evaluation of different training types. We present our preliminary evaluation results on Type 3 separate training in this section. Based on the example of Type 3, the following procedure is conducted in our evaluation for the sequential training starting with NW side training:

- Step 1: NW trains a two-sided AI model with both encoder part and decoder part based on dataset#1 of original CSI, Vin.

- Step 2: NW shares a dataset#2 to UE for UE training encoder part, dataset#2 includes Vin and Vq, where Vq is the output of encoder at NW side. Note that Vin is not the output during the NW training, but the output results after NW side training is finished.

- Step 3: UE trains encoder based on dataset#2 with the input as Vin and the label as Vq.

- Step 4: Joint inference is performed by the decoder at NW side and encoder at UE side trained separately.

Two cases for Type 3 training was agreed in last meeting as following:

- Case 1 (baseline): Aligned AI/ML model structure between NW side and UE side

- Case 2: Not aligned AI/ML model structures between NW side and UE side

We firstly evaluated the baseline scheme and compared the baseline with Type 1 joint training. The backbone and structure of encoder of Type 3 is same with Type 1, the only difference is how the AI model parameters  were trained. As Table 10 shows, there is small degradation of Type 3 separate training compared with Type 1 joint training on SGCS.

Table 10：Evaluation results comparison of different training types

| Training type | SGCS |
| --- | --- |
| Type 1 Joint training | 0.692 |
| Type 3 separate training with NW-first training | 0.690 |

Different with Type 1 joint training, Type 3 separate training needs to transport dataset between UE and gNB, in our evaluation, the original CSI is represented by Float32 and the compressed CSI is using 2-bit quantization. The overhead of Vin is about 584MByte and Vq is about 3MByte.

Observation 6: Type 3 separate training can achieve similar performance with Type 1 joint training on SGCS.

Impact of quantization

In last meeting, it was agreed to study the quantization of CSI feedback including quantization non-aware training and quantization-aware training. We consider the following cases to study the impact of quantization in AI/ML model based CSI compression.

- Case 1: Training without quantization, testing without quantization

- Case 2: Training with quantization, testing with quantization

- Case 3: Training with quantization, testing with quantization

Wherein, case 1 is the ideal case without any impact of quantization, which can be assumed to the upper band of AI/ML with quantization. Case 2 and case 3 is more in line with the real system where the testing is with quantization. Note that 2-bit scale quantization is used in the evaluation. The SGCS results is shown in Table x. It can be observed that there is about 13% gap caused by quantization operation by comparing the SGCS results of case 1 and case 2, more suitable quantization method such as vector quantization can be considered to improve the performance. An obvious performance gap can be observed by comparing case 2 and case 3, which implies that quantization aware training is supposed for AI/ML model training for CSI compression.

Table 11: SGCS performance of different quantization cases

| Case | SGCS |
| --- | --- |
| Case 1, Training without quantization, testing without quantization | 0.794 |
| Case 2, Training with quantization, testing with quantization | 0.692 |
| Case 3, Training with quantization, testing with quantization | 0.389 |

Observation 7: There is about 13% performance loss caused by 2-bit scalar quantization.

Observation 8: There is an obvious performance gap between quantization–aware training and quantization non-aware training, and quantization–aware training outperforms quantization non-aware training.

Throughput results

We select Rel-16 TypeII codebook as the baseline, and comparing the throughput results of AI CSI with baseline with rank adaptation. The simulation results and assumption are shown in Table 12 and Table 13 respectively. As Table 12 shows, the AI based CSI enhancement outperforms eType2 codebook in both SGCS and average SE. Setting ideal CSI based on SVD as the upper band, AI based CSI enhancement has about 5% performance loss while eType2 codebook has about 15% performance loss. Setting eType2 codebook as benchmark and AI based CSI enhancement shows 10% performance gain on average SE.

Observation 9: AI based CSI enhancement shows 10% performance gain on average SE comparing with eType2 codebook.

Table 12, performance simulation results

| Case | SGCS | Average SE(BS) |
| --- | --- | --- |
| Ideal | 1 (100%) | 5.052 (100%) |
| AI per layer(120bit) | 0.8475 (-15%) | 4.802 (-5%) |
| eType2 (Param4, 130bit) | 0.5689 (-44%) | 4.315 (-15%) |

Table 13, performance simulation assumption

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | UMa |
| Frequency Range |  | 2GHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 41 dBm for 10MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation. |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | full buffer |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Channel estimation |  | ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | Rel-16 Type II codebook |

Conclusions

In this contribution, we present views on evaluation methodology, evaluation metrics and initial evaluation results on AI/ML for CSI feedback enhancement, we have the following observations and proposals:

Observation 1: AI/ML model shows good generalization performance in various rank number.

Observation 2: The generalized AI model does not work well on various CSI feedback payloads, especially for a small number of feedback payloads.

Observation 3: AI/ML model shows good generalization performance in different sizes of subband and subband number.

Observation 4: AI/ML model shows good generalization performance in different scenarios.

Observation 5: The fine-tuning procedure with much less data can improve the performance significantly.

Observation 6: Type 3 separate training can achieve similar performance with Type 1 joint training on SGCS.

Observation 7: There is about 13% performance loss caused by 2-bit scalar quantization.

Observation 8: There is an obvious performance gap between quantization–aware training and quantization non-aware training, and quantization–aware training outperforms quantization non-aware training.

Observation 9: AI based CSI enhancement shows 10% performance gain on average SE comparing with eType2 codebook.

Proposal 1: Adopt Method 2 as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, if down-selection between Method 1 and Method 2 needed.

Proposal 2: Number of model parameters is selected between AI/ML model size and number of AI/ML parameters as metric of model complexity.

Proposal 3: RAN1 study pre-processing mechanisms for the input of decoder to  improve the AI model generalization performance on various feedback payloads.

- References

- Draft Report of 3GPP TSG RAN WG1 #110bis-e

- Draft Report of 3GPP TSG RAN WG1 #110