3GPP TSG RAN WG1 #111		R1-2211490

Toulouse, France, November 14th – 18th, 2022

Source:	OPPO

Title:	Discussion on XR specific power saving techniques

Agenda Item:	9.10.1

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss XR specific power saving techniques, focusing on C-DRX enhancements and PDCCH monitoring enhancements.

# Discussion

## C-DRX enhancements

    - C-DRX enhancements to handle non-integer periodicity

As agreed in Rel-17 XR SI, the packet arrival rates of XR could be {30, 60, 90, 120} fps, so the corresponding traffic periodicities of XR are {33.33, 16.67, 11.11, 8.33}ms. In the current specification, the long DRX cycle can be configured as {10ms, 20ms, 32ms, 40ms, 60ms, 64ms…} while the candidate cycle values for short DRX cycle are {2ms, 3ms, 4ms,5ms, 6ms…}. Obviously, there exists mismatch between the traffic arrival interval and the DRX cycle. Even if the DRX cycle is configured with finer granularity and more candidate cycle values are provided, the mismatch problem cannot be completely solved since the arrival interval is not divisible by the time duration of radio resource allocation unit, e.g. symbol or slot. The mismatch would lead to larger latency for data scheduling and further reduce the XR/CG capacity. Figure 1 illustrates the mismatch issue between DL video with 60bps and 16ms DRX cycle with 2ms on-duration timer. It can be observed that with the following DRX configuration, the mismatch issue appears every three traffic arrival intervals. In general, for a video traffic frame rate equal to FPS milliseconds, the mismatch pattern repeats in every lcmFPS,100FPS radio frames, which corresponds to lcmFPS,100100 traffic arrival cycles.

Figure 1: Mismatch between DL video @60fps and 16ms DRX cycle with 2ms on duration timer

One solution is to configure the DRX in a new way to effectively allow two DRX cycle lengths in one DRX configuration. Take Figure 2 as an example, the traffic arrival interval and DRX configuration are the same as above, except that two DRX cycles are observed: 17ms and 16ms.

Figure 2: Two DRX cycles are applied in a semi-static pattern {17ms, 17ms, 16ms}

Proposal 1: The following DRX enhancement can be further studied to solve periodicity mismatch issue:

- To generate a DRX cycle pattern with two inter-DRX interval lengths, either by derivation from configured parameters or by configuration directly.

In the RAN1 #109e discussion, one method is to determine the starting slot of DRX-ON as drx-SlotOffset from the beginning of the subframe satisfying

floor{[(SFN × 10) + subframe number] modulo (drx-Cycle)} = drx-Offset                 (1)

Equation (1) has two issues:

- The starting slot is always the slot with a given fixed slot offset (drx-SlotOffset) from the beginning from a subframe. However, a single traffic stream with non-integer periodicity may have different burst occasions falling into different subframes with different slot offsets. For example, assume 60fps traffic on a 30kHz SCS carrier. In case a first traffic burst emerges at time=0 that aligns to the beginning of frame #0 corresponding to slot #0 of subframe #0 in frame #0, a second traffic burst occurs at time=16.667ms corresponding to slot #1 of subframe #6 in frame #1, and a third burst occurs at time=33.33ms corresponding to slot #0 of frame #3 in frame #3.  So in order to have DRX-ON to cover all traffic burst,  drx-SlotOffset has to be 0 which however causes one PDCCH monitoring occasion in slot #0 of subframe #6 in frame #1 where the traffic has not yet occur. In order to keep the power consumption minimum, the above DRX-ON formula should be modified from subframe level to slot level.

- The parameter drx-Offset that is placed outside floor() function may cause incorrect subframe determination. For example, assume 60fps traffic on a 15kHz SCS carrier, and the first traffic burst arrives at 0.5ms after the starting boundary of frame #0. As shown in Table 1,

  - If  drx-Offset is set to 0, the DRX-ON for the occasions {1,2} starts in the same subframe as traffic arrives in, which means the gNB may not get enough time to prepare the PDCCH for these two occasions in the corresponding subframes and therefore the UE may spend power to decode PDCCH for nothing;

  - If  drx-Offset is set to 1, the DRX-ON for the occasions {1,2} starts in the next subframe after the one in which traffic arrives, so the gNB can get enough time to prepare PDCCH; however, the occasion #3 has an issue of one more subframe delay.

Table 1 Example of mismatch between traffic arrival and DRX-ON

| Traffic arrival/DRX-ON occasion index | Traffic arrival time | Subframe in which the traffic arrival falls | Subframe in which DRX-ON starts |  |
| --- | --- | --- | --- | --- |
|  |  |  | drx-Offset=0 | drx-Offset=1 |
| 1 | 0.5 ms | Subframe #0 in frame #0 | Subframe #0 in frame #0 | Subframe #1 in frame #0 |
| 2 | 17.167 ms | Subframe #7 in frame #1 | Subframe #7 in frame #1 | Subframe #8 in frame #1 |
| 3 | 33.833 ms | Subframe #3 in frame #3 | Subframe #4 in frame #3 | Subframe #5 in frame #3 |

Assume the target of DRX-ON is to let UE starts to decode PDCCH in a slot immediately after the slot corresponding to the traffic arrival, i.e., the m-th DRX-ON starts at the slot ns,m=m∙TP+tref∙2-μ+e2-μ , where TP is the traffic periodicity in unit of millisecond, tref refers to a reference time in unit of slot and e represents the time gap (e.g., in unit of 1/32 millisecond) between a traffic burst arrival and the reference time. Then the final formulation is that, the m-th DRX-ON duration for m≥0 starts at the ns-th slot in SFN-th radio frame in the hyper-frame that contains the starting instance of the m-th DRX-ON duration, where SFN and ns satisfy

SFN∙10∙2μ+ns=tref+m∙TP+e∙2μ mod 10240∙2μ                          (2)

It can be mathematically proved that the interval between two adjacent DRX-ON starting moments based on (2) can only be either TP∙2μ slots or TP∙2μ slots.

Proposal 2: The two-cycle DRX pattern is configured to UE via {TP(traffic periodicity), tref (reference time in integer multiple of slot), e (traffic arrival offset in relative to referenced time)}. The m-th DRX-ON duration for m≥0 has the semi-static start at the ns-th slot in SFN-th radio frame in the hyper-frame that contains the starting instance of the m-th DRX-ON duration, where SFN and ns satisfy

SFN∙10∙2μ+ns=tref+m∙TP+e∙2μ mod 10240∙2μ

    - C-DRX enhancements to handle Jitter handling

There exists jitter characteristic for XR traffic arrival. According to the previous RAN1 agreements, the jitter can be modeled as truncated Gaussian distribution with varying range of [-4,4] ms (baseline) or [-5,5] ms (optional). If the mismatch between arrival interval and DRX cycle can be regarded as some kind of semi-static mismatch, the jitter impact would lead to dynamic mismatch between data arrival and UE on duration.

One way to solve the jitter problem is to configure the DRX on duration timer long enough to cover the jitter range. Then no matter when the packet is arrived, UE maintains awake for data reception. However, considering the jitter range is relatively large, the DRX on duration timer needs to be configured much longer, which is unfavorable for UE power saving. Then companies propose that layer-1 based dynamic PDCCH monitoring reduction schemes, i.e. the PDCCH skipping and search space group switching, can be applied to avoid unnecessary power consumption. However, with XR service character more and more diverse and scheduling congestion of gNB increasingly unpredictable, jitter handling by configuring long enough ODT and well matched PDCCH skipping duration becomes more and more difficult, which would finally lead to larger power consumption or traffic latency.

Another way to solve jitter problem is to configure DRX on duration timer with a small value. However, with smaller on duration timer, the packet would be possible to arrive too early or too late. If the packet arrives too early, the UE is still outside active time when packet arrives, then the packet cannot be scheduled until UE wakes up. This would bring additional latency for the packet and may result in packet loss within the required time budget. While if the packet arrives too late, UE may have completed its on duration time and no longer stay awake for PDCCH monitoring. In such a case, not only UE would perform unnecessary PDCCH monitoring during on duration timer, but also the late arrived packet cannot get timely scheduling and need to wait for the next on duration instance. This increases the end-to-end latency more significantly compared to early packet arrival because a reasonable DRX configuration is to configure the short DRX on duration timer to cover the most central area of the jitter range. This makes the delay between packet arrival time and next on duration timer caused by late arrival (delay 2 in Figure 3) much longer than that of early arrival (delay 1 in Figure 3), so late arrival issue results in more serious problem.

Regarding to the concern that “DCI-triggered extension of DRX active time can be realized by dummy grant in current spec”, we cannot agree with it since each dummy grant can only extend the DRX active time equal to the value of IAT. If gNB plans to schedule the packet long after the end of ODT, the mechanism exploiting the current network would need several DCIs, which would surely bring larger DCI overhead than the proposed solution, in addition, it would cause unnecessary PDCCH monitoring in between, leading to larger power consumption.

Figure 3 jitter issue when short DRX on duration timer is configured

Observation 1: There would exist early/late packet arrival issue since gNB may configure DRX on duration timer shorter than jitter range, and late packet arrival would lead to longer scheduling latency and additional power consumption.

One solution to cope with late packet arrival issue is to trigger an additional on duration time or extending the current on duration time dynamically. As shown in Figure 4, there is no XR data arriving in the original on duration time, gNB can send a dynamic signaling to UE to indicate another on duration time, in which the UE can perform PDCCH monitoring for the XR traffic. By the way, gNB can also indicate the proper SSSG to monitor.

Figure 4 Additional on duration time duration DRX off

Proposal 3: An additional DRX on-duration time can be dynamically indicated to UE to solve jitter issue.

    - Evaluation results for R15/16 C-DRX and enhanced C-DRX schemes

In this section, we provide our evaluation results for the above proposed enhanced C-DRX schemes. For the power saving evaluation, both satisfied UE rate and power saving gain should be considered jointly, which is inline with the methodology in TR 38.838. In the following simulation, DL single stream AR/VR video traffic with 60fps and 30Mbps is evaluated for FR1 Dense urban scenario. The power consumption results are counted for all UEs following the power consumption model defined in TR 38.840. The detailed simulation parameters and assumptions can be found in Appendix.

The following schemes are considered for power saving simulation:

- AlwaysOn: it is assumed as the baseline scheme;

- R15/16 C-DRX with parameter set {DRX cycle, On duration timer, inactivity timer} = {16ms, 14ms, 2ms} is evaluated for comparison with the enhanced C-DRX schemes. For the R15/16 C-DRX, the On duration timer is configured to 14ms, which is extremely long in order to cope with the periodicity mismatch issue and jitter impact.

- For the enhanced C-DRX scheme, two candidate schemes are considered in the evaluation:

- Semi-static C-DRX enhancements: Non-uniform CDRX cycle pattern with parameter set {DRX cycle, On duration timer, inactivity timer} = {{17ms,17ms,16ms}, 10ms, 2ms}

- Dynamic C-DRX enhancements: dynamic indication of another DRX On Duration time by DCI, with parameter set {DRX cycle, On duration timer, inactivity timer} = {{17ms,17ms,16ms}, 4ms, 2ms}, another on Duration time = 4ms

The power saving gain and satisfied UE rate of high load case (number of UEs per cell N= floor (Capacity)) are provided in Table 2.

Table 2: Power saving results in FR1 DL Dense Urban with 60fps and 30Mbps traffic model

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Another ODT (ms) | #UE /cell= floor (Capacity) | satisfied UE rate | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Always On | - | - | - | - | 5 | 90.18% | / |
| R15/16 CDRX | 16 | 14 | 2 | - | 5 | 88.77% | 5.11% |
| Non-uniform CDRX cycle pattern | {17,17,16} | 10 | 2 | - | 5 | 88.77% | 18.72% |
| Non-uniform CDRX cycle pattern with dynamic additional ODT | {17,17,16} | 4 | 2 | 4 | 5 | 90.18% | 30.86% |

From the evaluation results, it can be observed that with limited loss of satisfied UE rate (limited to 1%), R15/16 CDRX scheme can only achieve 5.11% power saving gain. This is because for R15/16 C-DRX, the On duration timer needs to be set to a very large value to cover the periodicity mismatch issue and jitter range, only with this the satisfied UE rate can be ensured. With the enhanced C-DRX scheme, the On duration timer can be configured to a much shorter value and align the periodicity by non-uniform CDRX cycle pattern and cover the jitter range by dynamically triggered additional On duration time. In such a case, higher power saving gain (18.72% for semi-static scheme and 30.86% for dynamic scheme) can be obtained.

Observation 2: In FR1 Dense Urban, AR/VR 30Mbps and 60fps, the following is observed:

- For R15/16 C-DRX, the power saving gain can be 5.11% with loss of satisfied UE rate less than 1%;

- For non-uniform CDRX cycle pattern, the power saving gain is 18.72% with loss of satisfied UE rate less than 1%;

- For Non-uniform CDRX cycle pattern with dynamic additional on duration timer, the power saving gain is 30.86% with similar satisfied UE rate.

## PDCCH monitoring enhancements

Similar with DRX, there also exists periodicity mismatch issue and jitter impact for search space set configuration. To be specific, the packet would be possible to arrive out of the duration of the search space set, then the same early arrival and late arrival may happen, leading to additional scheduling delay or unnecessary power consumption.

In addition, as discussed above, the XR packet size is not fixed. Taking AR/VR 30Mbps as example, maximum packet size is 93750 bytes but minimum packet size is 31250 bytes. Obviously, there exists large gap between maximum packet size and minimum packet size. So different XR packets may need different number of slots for transmission and hence different time length of PDCCH monitoring is needed. In such a case, a fixed “duration” value for the search space set cannot work well since it fails to match the various packet size.

One solution to cope with both jitter problem and various packet size issue is to change the value of monitoringSlotPeriodicityAndOffset and duration on the fly. Take Figure 5 as an example, search space set #1 is configured with 16-slots periodicity, 1-slot offset and 2-slot duration; search space set #2 is configured with 16 slots periodicity, 12-slot offset and 2-slot duration. If the packet size is very large and 2 slots are not enough for the scheduling, gNB can indicate UE to extend the duration range via dynamic signaling. If the packet arrives out of the duration time, gNB can also dynamically indicate UE to adjust the monitoring offset. Details can be further studied such as the DCI format used for dynamic indication, the effective time of the dynamic indication and so on.

Figure 5 dynamic change of monitoringSlotPeriodicityAndOffset and duration

Proposal 4: The value of monitoringSlotPeriodicityAndOffset and duration for a search space set can be changed dynamically by gNB to cope with jitter and packet size variation.

# Conclusions

In this contribution, we discuss XR specific power saving techniques and following observation and proposals are made:

Observation 1: There would exist early/late packet arrival issue since gNB may configure DRX on duration timer shorter than jitter range, and late packet arrival would lead to longer scheduling latency and additional power consumption.

Observation 2: In FR1 Dense Urban, AR/VR 30Mbps and 60fps, the following is observed:

- For R15/16 C-DRX, the power saving gain can be 5.11% with loss of satisfied UE rate less than 1%;

- For non-uniform CDRX cycle pattern, the power saving gain is 18.72% with loss of satisfied UE rate less than 1%;

- For Non-uniform CDRX cycle pattern with dynamic additional on duration timer, the power saving gain is 30.86% with similar satisfied UE rate.

Proposal 1: The following DRX enhancements can be further studied to solve periodicity mismatch issue:

- To generate a DRX cycle pattern with two inter-DRX interval lengths, either by derivation from configured parameters or by configuration directly.

Proposal 2: The two-cycle DRX pattern is configured to UE via {TP(traffic periodicity), tref (reference time in integer multiple of slot), e (traffic arrival offset in relative to referenced time)}. The m-th DRX-ON duration for m≥0 has the semi-static start at the ns-th slot in SFN-th radio frame in the hyper-frame that contains the starting instance of the m-th DRX-ON duration, where SFN and ns satisfy

SFN∙10∙2μ+ns=tref+m∙TP+e∙2μ mod 10240∙2μ

Proposal 3: An additional DRX-on duration time can be dynamically indicated to UE to solve jitter issue.

Proposal 4: The value of monitoringSlotPeriodicityAndOffset and duration for a search space set can be changed dynamically by gNB to cope with jitter and packet size variation.

# References

- TR 38.838 Study on XR (Extended Reality) Evaluations for NR (Release 17) v17.0.0

- TR 38.840 Study on User Equipment (UE) power saving in NR (Release 16) v16.0.0

# Appendix

Table A1 Simulation parameters for FR1 DL

| Parameters | Value |
| --- | --- |
| Scenario | Dense Urban hexagonal layout with (19, 3) Sectors |
| Inter-BS distance | 200m |
| Carrier frequency | 4GHz |
| Duplex Mode / Simulation bandwidth | 100MHz |
| SCS | 30KHz |
| TDD pattern | DDDSU |
| BS Antenna Configuration | 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) (dH, dV) = (0.5λ, 0.5λ) |
| UE Antenna Configuration | 4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| Transmit Power | 51dBm |
| Antenna Height | 3 m for BS and 1.5 m for UE |
| Receiver Noise Figure | 5 dB for BS and 9 dB for UE |
| UE speed | 3km/h |
| Scheduling Algorithm | SU-MIMO+PF |
| Channel estimation | Realist |
| BS receiver | MMSE-IRC |
| Data rate (Mbps) | 30 |
| Packet size distribution | Truncated Gaussian distribution |
| Packet delay budget (ms) | 10 |
| Jitter distribution | Truncated Gaussian distribution |
| Jitter range | [-4ms,4ms] |