3GPP TSG RAN WG1 #111	                                                         　R1-2212007

Toulouse, France, November 14th - 18th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on low power WUS receiver architectures

Agenda Item:	9.13.2

Document for: 	Discussion and decision

- Introduction

At the RAN1 #110b-e, the following was agreed.

Conclusion RAN1 does not intend to mandate the implementation of any specific type(s) of LP WUR architecture at the UE. - Note: this does not prevent RAN4 from defining requirements for LP WUR in the normative phase. Agreement Study at least the following three types of receiver architectures for LP-WUR: - Architecture with RF envelope detection - Heterodyne architecture with IF envelope detection - Homodyne/zero-IF architecture with baseband envelope detection - Note: The details of each type of receiver architecture are discussed separately. - Note: Above receiver architectures are considered suitable for OOK modulation. Some of the architectures can be applicable for other modulations such as FSK. Agreement Study the architecture with RF envelope detection based on at least the following diagram for LP-WUR. - The RF signal is converted into baseband signal directly via an RF envelope detector. - There is no Local Oscillator (LO) and no Phase-Locked Loop (PLL). - 1-bit or multi-bit ADC is applied. - Some component(s), e.g., RF LNA and/or BB AMP, can be optionally applied. - High-Q matching network and/or RF BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers. - FFS the support of band and/or carrier tuning Agreement Study the heterodyne architecture with IF envelope detection based on at least the following diagram for LP-WUR. - The RF signal is down converted into IF signal via an RF mixer with a LO. The IF signal is converted into baseband signal via an IF envelope detection. - There may be one or multiple IF stages depending on design. - The choice of the LO is one of the major factors that determines the power consumption. - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such increased frequency offset and phase noise should be taken into account in the design and evaluation. - FLL (frequency locked loop) may replace PLL for non-coherent detection. - 1-bit or multi-bit ADC is applied. - High-Q matching network and/or RF BPF and/or IF BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers. - Some component(s), e.g., RF LNA and/or IF AMP and/or BB AMP, can be optionally applied. - Image rejection filter or an image rejection mixer is required. - FFS the support of band and/or carrier tuning FFS the choice of IF frequency range Agreement Study the homodyne/zero-IF architecture with baseband envelope detection based on at least the following diagram for LP-WUR. - The RF signal is directly down converted into baseband signal via an RF mixer with a LO. - Baseband envelope detection can be done either in analog domain or in digital domain depending on design, which is not explicitly shown in the diagram. - The choice of the LO is one of the major factors that determines the power consumption. - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such increased frequency offset and phase noise should be taken into account in the design and evaluation. - FLL (frequency locked loop) may replace PLL for non-coherent detection. - 1-bit or multi-bit ADC is applied. - High-Q matching network and/or RF BPF and/or BB BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers. - No image rejection filter is required. - Some component(s), e.g., RF LNA and/or BB AMP, can be optionally applied. FFS the support of band and/or carrier tuning Agreement Further study the receiver architectures for FSK, with two examples shown below: - Example 1: parallel OOK receivers and a comparator circuit, e.g., - Each path can be implemented using either of [the architecture with RF envelope detection,] heterodyne architecture with IF envelope detection, or homodyne/zero-IF architecture with baseband envelope detection. - Example 2: using an FM-to-AM detector [or an FM detector] - Alt 1: Use an analog FM-to-AM detector with a similar architecture as for OOK (e.g. heterodyne or zero-IF architecture), except that the envelope detector is replaced by a FM-to-AM detector. - Analog FM-to-AM detector can be implemented at least in BB or low-IF. - Alt 2: Use a FM-to-AM detector [or an FM detector] implemented in digital domain after ADC, with a heterodyne or zero-IF architecture. - Digital FM-to-AM detector implementation can be considered as part of digital baseband processing. Here is an example of using zero-IF architecture: - The FM-AM detector can be implemented using a frequency discriminator, which converts frequency variations into amplitude changes. It can be implemented in either analog domain (as in Alt 1) or digital domain (as in Alt 2). - One example, as shown in the figure below, is a conventional quadrature FM discriminator. It multiplies received frequency modulated signal with a phase shifted version, followed by a low pass filter. The amplitude of the output signal is proportional to the frequency of the input signal. - Note: Other architectures are not precluded. Agreement For the analysis of a receiver architecture, companies are encouraged to provide at least the following (when applicable): - Details of the receiver - Receiver architecture type - Assumed modulation/waveform/coding - Presence of a RF LNA / IF AMP / BB AMP, and the corresponding gain, if any - Local oscillator - Type of oscillator and the corresponding frequency accuracy/drifting - Handling of time/frequency impairments - Presence of PLL or FLL - ADC: sampling rate, bit-width - Assumed signal bandwidth and guard band, and frequency location within a carrier (including whether it is fixed or can be flexible) - RF/IF/BB filter characteristics (e.g. type of filter, order, cut-off frequency/frequencies), if any - Baseband processing (e.g., sequence correlation detection / decoding, other signal processing, if any) - Assumed frequency band(s) and the support of band and/or carrier tuning - Duty cycle handling of WUS and other signals (if any) - Interference rejection capability (including both adjacent-channel interference and interference from adjacent subcarriers occupied by legacy NR signals or other LP WUS) - Handling of inter-cell interference - Whether there is any mobility support function, e.g. measurement capability - Performance metrics - Power consumption during active monitoring/reception and during off state (and breakdown if possible) - Noise figure - Sensitivity/coverage - Data rate - FFS: other performance metrics for, e.g., cost/complexity, interference rejection capability and inter-cell interference handling - Note: The performance and design of receiver architecture is expected to be dependent on WUS design. This list can be updated later when the discussion on WUS signal/procedure design (AI 9.13.3) starts.

In this contribution, we provide our views on low power WUS receiver.

- Discussion

  - Performance metrics for receiver architecture

As described in Section 1, performance metrics were summarized and agreed for further analysis, and several points are remained as FFS such as cost/complexity, interference rejection capability, and inter-cell interference handling.

During the last meeting, several types of receiver architecture (e.g. RF envelope detection, IF envelope detection, etc.) was agreed to study further, and which components in the receiver can be used for interference rejection/handling is also captured in their agreements. However, such interference rejection/handling are still FFS to be included in the performance metrics.

Since the scope of this WI includes eMBB use case, target of coverage performance may need to be the same as other regular NR signals/channels. Furthermore, to support a variety of possible use cases by NR devices, we consider that NR reduced capability devices (e.g. Rel-17 RedCap) need to support LP-WUS/WUR. In the case, due to the reduced number of Rx antenna, coverage performance may be degraded even for LP-WUS. In the sense, to meet such stringent requirements of LP-WUS coverage, coverage performance should be carefully studied.

Regarding cost/complexity, considering that LP-WUS use cases include IoT market, it is clear that cost/complexity is one of important metrics. However, how to estimate cost/complexity and define reasonable performance metric needs to be further studied for LP-WUR.

Proposal 1: Interference rejection capability and inter-cell interference handling should be added into the performance metrics of LP-WUR architecture.

Proposal 2: Cost/complexity should be further studied for performance metrics of LP-WUR architecture

- Conclusion

In this contribution, the low power WUS receiver was discussed. Based on the discussion, the following observations and proposals were made.

Proposal 1: Interference rejection capability and inter-cell interference handling should be added into the performance metrics of LP-WUR architecture.

Proposal 2: Cost/complexity should be further studied for performance metrics of LP-WUR architecture

References

- RAN1 Chair’s Notes, RAN1#110b-e, October 2022.