3GPP TSG RAN WG1 #111	R1-2212149

Toulouse, France, November 14th – 18th, 2022

Source:	Sharp

Title:	Discussion on subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for:	Discussion and Decision

# Background

Several agreements have been achieved in RAN1#110bis e-meeting [1], which could serve as basis for discussion.

Agreement For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline. - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation. Agreement For SBFD operation within a TDD carrier, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline. Agreement The maximum number of UL subbands for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier is one for the study in RAN1. - The UL subband can be located at one side of the carrier. - The UL subband can be located at the middle part of the carrier, subject to RAN4’s study and conclusion Note: RAN1 considers the above two possibilities unless RAN4 concludes that any one is infeasible. Note: Two UL subbands for SBFD operation in an SBFD symbol within a TDD carrier due to SBFD operation in legacy UL symbols is subject to further RAN1 discussions which is 2nd priority as per RAN guidance. Send an LS to RAN4 to inform the above agreement. If RAN4 has response, it will be taken into account but in the meanwhile, RAN1 work will continue based on the above. LS on maximum number of UL subbands for duplex evolution to RAN4 is endorsed. Final LS in R1-2210671.

# SBFD subband configuration

# SBFD subband definition

The following agreement has been made for SBFD subband definition at RAN1#110 meeting [2].

Agreement For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline.

Figure 1 shows an example SBFD configuration for baseline assumption. There is a UL subband whose time/frequency location is configured with semi-static manner. SBFD region is defined as a set of SBFD symbols where the SBFD symbols are symbols with subbands that gNB would use for SBFD operation, as agreed at RAN1#109 meeting [3].

Conclusion For discussion purpose only, SBFD symbols is defined as symbols with subbands that gNB would use for SBFD operation.

Here, it should be noted that the above agreement should be understood that subbands are not necessarily used for SBFD operation, but maybe used for SBFD operation.

Observation 1: As baseline for study, subbands are a set of resources defined by time/frequency region which is semi-statically configured by gNB. The subbands are not necessarily used for SBFD operation, but maybe used for SBFD operation.

Figure 1A: Legacy TDD pattern

The usage of UL subband which is semi-statically configured should be flexible. There are several motivations listed as follows:

- Support dynamic TDD within a legacy TDD carrier

- Cater for legacy UEs DL traffic

- Secure resources for cell-specific signalling (e.g., SS/PBCH blocks)

Support dynamic TDD within a legacy TDD carrier

One of the motivations for SBFD should be supporting dynamic TDD within a legacy TDD carrier. With operating dynamic TDD confined within middle of the carrier, operators can mitigate potential problem for cross-operator CLI.

Cater for legacy UEs DL traffic

Semi-static UL subband configuration would lead to shortage of legacy DL region. Although it is possible to schedule PDSCH for a legacy UE outside the UL subband in SBFD symbols, the availability is quite limited. For example, in SBFD symbols, distributed transmission for the legacy UE is not feasible, due to potential collision with UL subband if the UL subband cannot be used for DL. On the other hand, since the legacy UE is not recognizing the UL subband, there is no clear motivation prohibiting for gNB to perform DL transmission dedicated for legacy UEs in the UL subband. Therefore, at this stage, we can make the following proposal:

Proposal 1: From gNB perspective, UL subband can be used to perform DL transmission dedicated for legacy UEs as well as UL reception.

Secure resources for cell-specific signalling (e.g., SS/PBCH blocks)

As discussed above, semi-static UL subband configuration would lead to shortage of legacy DL region. This fact leads to potential shortage of resources for cell-specific signalling. Cell-specific signalling like SS/PBCH blocks should avoid being transmitted in symbols with simultaneous transmission/reception, to reduce potential UE-to-UE intra-cell inter-subband CLI. Therefore, UL subband should be available for such cell specific signalling at a time. Since the cell-specific signalling should be received by the SBFD capable UEs as well as legacy UEs, we make the following proposal:

Proposal 2: From SBFD capable UE perspective, UL subband can be used to perform DL reception for cell-specific signalling.

With above discussion, there are strong motivations to support flexible usage of downlink and uplink for UL subband. Therefore, our proposal is as follows:

Proposal 3: From SBFD capable UE perspective, UL subband can be used to perform DL reception as well as UL transmission.

# SBFD subband indication

The following agreement has been made for SBFD subband indication at RAN1#110bis e-meeting [1].

Agreement For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

For semi-static configuration of subband time locations for SBFD operation, it has been agreed that explicit configuration within a period has been the baseline. What’s unclear in the agreement is the definition of the period.

Intuitively, the period could be the same as the period of TDD configuration. On the other hand, the TDD configuration provided by TDD-UL-DL-ConfigCommon is optional field in SIB1 for TDD cells, which implies the network may or may not provide the TDD configuration. Therefore, at least explicit/implicit configuration of length of the period for SBFD subband time location indication should be considered as a baseline.

Proposal 4: Explicit/implicit configuration of length of the period for SBFD subband time location indication should be considered as a baseline.

If dynamic indication is to be supported, two types of dynamic indicator should be provided to the UE. One is the legacy dynamic SFI provided by DCI format 2_0, and the other is new dynamic signalling for UL subband usage indication. The former would be used to indicate slot formats applicable to the outside of the UL subband, and the new signalling could be used to specify which direction of DL/UL is applied to the UL subband.

Proposal 5: New dynamic signalling should be studied to specify which direction of DL/UL is applied to the UL subband.

# Potential enhancements to procedure for physical channels and signals

# SS/PBCH blocks in SBFD symbols

The following agreement has been made for SS/PBCH block reception procedure at RAN1#110bis e-meeting [1].

Agreement Study whether SBFD operation in SSB symbols is supported or not.

To preserve the reception quality (e.g., UE cell (re)collection, RLM/RRM, beam training, etc.) of SS/PBCH blocks, it is safer to confine the SS/PBCH blocks in the legacy DL region such that no inter-subband CLI affects the reception quality. On the other hand, it is difficult especially in a case that the network is using several (e.g., 8) SS/PBCH blocks, which spans in a half frame.

Figure 2 shows two cases for SS/PBCH blocks over SBFD symbols. Figure 2A shows an example of SS/PBCH block overlapping with UL subband in time and frequency resource. In this case, the gNB cannot operate with SBFD in symbols with SS/PBCH blocks. Thus, the UE should not expect UL transmission in the resource and the UE should be able to receive the SS/PBCH block.

Proposal 6: When a SS/PBCH block overlaps with the UL subband in time and frequency resource, the UE should not expect UL transmission in the resource and the UE should be able to receive the SS/PBCH block.

Figure 2B shows an example of SS/PBCH block overlapping with UL subband only in time domain. There are two subcases as follows:

- The gNB performs simultaneous transmission/reception on symbols with the SS/PBCH block

- The gNB only performs transmission on symbols with the SS/PBCH block

If simultaneous transmission/reception is allowed on the symbols with the SS/PBCH block, the more uplink resource is available, and thus it leads to more uplink coverage and reduced uplink latency. On the other hand, there is a risk of increased interference over SS/PBCH block reception due to UE-to-UE intra-cell inter-subband CLI. Further, the UE scheduled with uplink transmission in the symbols cannot measure the quality of the SS/PBCH block at that time. It may affect RLM, time/frequency tracking, beam tracking.

On the other hand, if only transmission is allowed on the symbols with the SS/PBCH block, it implies that the UL subband on the symbols could be only usable for DL. At this stage, both options can be discussed for further study.

Proposal 7: When a SS/PBCH block overlaps with the UL subband in time resource whereas the SS/PBCH block doesn’t overlap with the UL subband in frequency domain, the following two options should be further studied:

- The UE shouldn’t expect UL transmission in the resource and the UE should be able to receive the SS/PBCH block

- The UE should expect UL transmission in the resource and the UE is not required to receive the SS/PBCH block

| Figure 2A: SS/PBCH block overlapping with UL subband in time/frequency | Figure 2B: SS/PBCH block overlapping with UL subband only in time |
| --- | --- |

# CSI-RS resource allocation

The following agreement has been made for CSI-RS at RAN1#110bis e-meeting [1].

Agreement Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands.

Figure 3 shows two examples of CSI-RS allocation. Figure 3A shows an example of CSI-RS allocation where the CSI-RS is overlapping with the UL subband. In this option, the UE is supposed to assume that CSI-RS REs inside the UL subband is not available. No change in RRC information elements would be necessary for this example.

On the other hand, In Figure 3B, single CSI-RS is configured with two partitions. In this option, RRC information elements may need to change such that a CSI-RS resource configuration includes two partitioned resources.

Proposal 8: The following two options can be further studied:

- CSI-RS allocation overlapping with the UL subband

- CSI-RS configured with two partitions

| Figure 3A: CSI-RS allocation overlapping with the UL subband | Figure 3B: CSI-RS allocation with two partitions |
| --- | --- |

# PDSCH resource allocation

We had extensive discussion on whether to study resource allocation for PDSCH. In our view, type-1 RA with DCI format 1_0 in USS is of the highest priority. The reason is that DCI format 1_0 in USS only supports type-1 RA, and it has possibility of wideband allocation (over 100 MHz). On the other hand, maximum bandwidth for type-1 RA with DCI format 1_0 in CSS is limited to the size of CORESET 0 as long as the CORESET 0 is configured in the cell. Maximum bandwidth of CORESET 0 is less than 20 MHz for FR1. Therefore, the maximum bandwidth for type-1 RA can be confined in any subband. On the other hand, type-0 RA is a strong tool for frequency selective scheduling. Therefore, type-0 RA should also be studied for potential enhancements. Therefore, we propose:

Proposal 9: For PDSCH resource allocation, type-1 RA with DCI format 1_0 in USS and type-0 RA is priority.

# Potential enhancements to physical layer procedures

# Time domain conflict of UE’s UL and DL operation

The following agreement has been made for time domain conflict of UE’s UL and DL operation at RAN1#110bis e-meeting [1].

Agreement Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE - If there are, whether/how to avoid/handle such collision cases (as second step)

Most of time domain conflict of UE’s UL and DL operation in flexible symbols has been specified in the current specification. Due to the half duplex limitation, in our view, there are not so many cases to be revisited for SBFD symbols. What we need to consider for SBFD operation could be as follows:

- Collision between SS/PBCH blocks and any UL transmission

- Collision between higher layer configured DL and UL

Collision between SS/PBCH blocks and any UL transmission

SS/PBCH blocks has been regarded as the highest priority in NR from Rel-15. On the other hand, If SBFD operation with SS/PBCH blocks would be agreed, this principle needs to be removed in some cases. For example, in SBFD symbols, if the SS/PBCH block is overlapping with at least partially with the UL transmission, but the SS/PBCH block is outside of the UL subband in frequency domain (as described in Figure 2B), the UE should be allowed to prioritize the UL transmission over SS/PBCH reception. Therefore, we need to discuss whether simultaneous SS/PBCH block transmission and UL reception in SBFD symbols would be supported or not, before discussing the collision between SS/PBCH blocks and any UL transmission.

Proposal 10: RAN1 should discuss whether simultaneous SS/PBCH block transmission and UL reception in SBFD symbols would be supported or not, before discussing the collision between SS/PBCH blocks and any UL transmission.

Collision between higher layer configured DL and UL

In the existing specification, the UE is not required to handle the case of collision between higher layer configured DL and UL. The reason is that proper network configuration would avoid such a problematic situation. This principle still holds in duplex evolution. Therefore, we don’t think the necessity of revisiting this matter again.

Proposal 11: No further study on handling cases of collision between higher layer configured DL and UL.

# Repetition procedures

UE-specific uplink channels

Figure 4 to 6 show examples of 8 repetitions for an uplink channel assuming available slot counting. Figure 4 shows an example in which the repetitions are transmitted in both the SBFD region and the legacy UL region. Repetitions are mapped continuously in the SBFD region and the legacy UL region, and they are not mapped in the legacy DL region. In this example, Rep#4 and Rep#5 leads to uplink resource fragmentation which has been discussed and addressed in Rel-17 RedCap. To reduce fragmentation, different frequency resource allocation for each region like Figure 5 may be considered further. In Figure 5, different starting RBs and different hopping offsets (i.e., Offset#0 and Offset#1) are provided for each region such that the repetitions are placed on the edge of each available uplink resource.

Figure 6 is another example to reduce fragmentation in the legacy region, where the repetitions are mapped only in the SBFD region. In this example, we can keep the simplicity of transmission/reception procedure for repetition due to the aligned frequency resource.

At this stage, rather than trying to agree on the details on repetition procedure, we suggest making a general agreement like following:

Proposal 12: RAN1 further study the impact on procedures for UE-specific uplink repetition.

Figure 4: Repetitions transmitted in both the SBFD region and the legacy UL region

Figure 5: Repetitions transmitted in both the SBFD region and the legacy UL region with different frequency resource for the SBFD region and the legacy UL region

Figure 6: Repetitions transmitted only in SBFD region

Msg3 PUSCH repetition

Msg3 PUSCH repetition has been introduced in Rel-17 for coverage compensation. Therefore, the msg3 PUSCH repetition should be in the scope of this SI. There is no justifiable reason to exclude the msg3 PUSCH repetition from this study.

Proposal 13: Msg3 PUSCH repetition should be studied for SBFD.

# Conclusion

In this contribution, we have the following observations:

Observation 1: As baseline for study, subbands are a set of resources defined by time/frequency region which is semi-statically configured by gNB. The subbands are not necessarily used for SBFD operation, but maybe used for SBFD operation.

In this contribution, we have the following proposals:

Proposal 1: From gNB perspective, UL subband can be used to perform DL transmission dedicated for legacy UEs as well as UL reception.

Proposal 2: From SBFD capable UE perspective, UL subband can be used to perform DL reception for cell-specific signalling.

Proposal 3: From SBFD capable UE perspective, UL subband can be used to perform DL reception as well as UL transmission.

Proposal 4: Explicit/implicit configuration of length of the period for SBFD subband time location indication should be considered as a baseline.

Proposal 5: New dynamic signalling should be studied to specify which direction of DL/UL is applied to the UL subband.

Proposal 6: When a SS/PBCH block overlaps with the UL subband in time and frequency resource, the UE should not expect UL transmission in the resource and the UE should be able to receive the SS/PBCH block.

Proposal 7: When a SS/PBCH block overlaps with the UL subband in time resource whereas the SS/PBCH block doesn’t overlap with the UL subband in frequency domain, the following two options should be further studied:

- The UE shouldn’t expect UL transmission in the resource and the UE should be able to receive the SS/PBCH block

- The UE should expect UL transmission in the resource and the UE is not required to receive the SS/PBCH block

Proposal 8: The following two options can be further studied:

- CSI-RS allocation overlapping with the UL subband

- CSI-RS configured with two partitions

Proposal 9: For PDSCH resource allocation, type-1 RA with DCI format 1_0 in USS and type-0 RA is priority.

Proposal 10: RAN1 should discuss whether simultaneous SS/PBCH block transmission and UL reception in SBFD symbols would be supported or not, before discussing the collision between SS/PBCH blocks and any UL transmission.

Proposal 11: No further study on handling cases of collision between higher layer configured DL and UL.

Proposal 12: RAN1 further study the impact on procedures for UE-specific uplink repetition.

Proposal 13: Msg3 PUSCH repetition should be studied for SBFD.

# References

- [1]	RAN1 chairman’s note at RAN1#110bis e-meeting, October 2022

- [2] 	RAN1 chairman’s note at RAN1#110 meeting, August 2022

- [3] 	RAN1 chairman’s note at RAN1#109 e-meeting, May 2022