3GPP TSG WG RAN1 #111	R1-2212191

eMeeting, November 14th – 18th, 2022

# Title:	[DRAFT] Reply LS on further questions on feMIMO RRC parameters

# Response to:	R1-2210808 LS on further further questions on feMIMO RRC parameters

# Release:	Rel-17

# Work Item:	NR_feMIMO-Core

Source:	Ericsson [RAN1]

To:	RAN2

Cc:

Contact Person:

Name:	Siva Muruganathan

E-mail Address:	siva.muruganathan@ericsson.com

1. Overall Description:

RAN1 thanks RAN2 for the LS with further questions on feMIMO RRC parameters and would like to provide the following response.

RAN2 specifically asked the three questions.

Question 1: According to above RAN2 understandings, RAN2 would like to ask RAN1 to explain what the association of BWP and cell ID for srs-TCIState is?

Answer 1: The UL-TCI-State is associated with the BWP and serving cell where the SRS-Config is configured. The joint TCI state is associated with the BWP and serving cell where the SRS-Config is configured.

Question 2

RAN2 would like to ask RAN1

- Is there a need to add the parameter codebookMode which selects mode1 or mode2 in the IE CodebookConfig-r17?

- If a) is needed, which types of codebook the parameter is needed, for example:

  - typeI-SinglePanel-Group1-r17

  - typeI-SinglePanel-Group2-r17

  - typeII-PortSelection-r17

- Additionally, if a) is needed, for each type of codebook that needs a parameter codebookMode, please confirm if a separate parameter is needed or if they share the same configuration of codebookMode.

Answer 2:

[Answer for 2a]  The selection between mode1 and mode2 is only applicable when the codebookType is set to ‘type1’.  One possibility is to add codebookMode in the IE CodebookConfig-r17.  Alternatively, it may be ok to assume mode1 when the codebookType is ‘type1’ in CodebookConfig-r17.  Given that the ASN.1 is frozen, it is sufficient to assume mode1 when the codebookType is ‘type1’ in CodebookConfig-r17.  This assumption should be clarified in the field descriptions.

[Answer for 2b]  If codebookMode is added in the IE CodebookConfig-r17, then it is applicable to only both typeI-SinglePanel-Group1-r17 and typeI-SinglePanel-Group1-r17.  codebookMode is not applicable to typeII-PortSelection-r17.

[Answer for 2c]  If codebookMode is added in the IE CodebookConfig-r17, then the same configuration of codebookMode applies to both typeI-SinglePanel-Group1-r17 and typeI-SinglePanel-Group1-r17.

Question 3

RAN2 assumes RAN1 informs RAN2 in case the above described pathloss reference signal configuration for unified TCI state is not what RAN1 has intended.

Answer 3: The above description is fine.

2. Actions:

To RAN2 group:

ACTION:

RAN1 respectfully asks RAN2 to take the above answers into consideration.

3. Date of Next RAN1 Meetings:

TSG WG RAN1 #112		27th February – 3rd March 2023	Athens, Greece

TSG WG RAN1 # 112-bis-e	17th April – 26th April 2023	e-meeting