3GPP TSG RAN WG1 #111	 R1-2212397

Toulouse (France), 14 – 18 November, 2022

Agenda item:		9.16.1

Source:	Nokia, Nokia Shanghai Bell

Title:	NR support for below 5 MHz BW

Document for:		Discussion and Decision

# Introduction

The work item on NR support for dedicated spectrum less than 5MHz for FR1 was approved at RAN#94-e, with the latest revision agreed at RAN#95-e [1]. This relates to so called specialized networks, which are used to provide mission critical communications for industry verticals such as smart energy and infrastructure, public safety, and railway communications. These networks would benefit not only from the high spectral efficiency of 5G NR, but also from its ultra-reliability and low latency.

This contribution deals with the following objectives of the WID:

The following objectives shall be included for dedicated FDD spectrum in FR1:

- Identify and specify necessary changes to NR physical layer with minimum specification impact to operate in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN1]:

  - Restrict to subcarrier spacing of 15kHz and the use of normal cyclic prefix.

  - For SSB:

    - Reuse PSS/SSS specification without puncturing.

    - PBCH based on current design

  - Identify and specify necessary minimum changes to PDCCH, CSI-RS/TRS, PUCCH, and PRACH for functional support based on existing design, without optimization.

- Specify necessary RAN4 requirements to support deploying NR in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN4], including in bands n100, n8, n26 and n28:

  - Specify system parameters (including channel and sync rasters) for the associated dedicated spectrum.

  - Minimize impact on RF requirements:

    - Reuse 5 MHz channel bandwidth at least for FRMCS use case (assuming co-located NR and GSM-R with same operator).

    - Specify the required RF requirements for optional 3 MHz channel bandwidth in bands n100, n8, n26 and n28.

  - Specify RRM requirements while minimizing specification impact to support operation in dedicated spectrum allocations from approximately 3 MHz up to below 5 MHz.

In the following, we discuss the changes that are necessary to support the new optional 3 MHz channel bandwidth and other bandwidths below 5 MHz. We consider synchronization raster and PBCH in Section 4, PDCCH and especially CORESET#0 in Section 5, and PRACH, PUCCH and CSI-RS in Section 6. In Section 2, we discuss what new signal bandwidths should be introduced to NR to successfully support the use cases targeted in the WID.

# Necessary new transmission bandwidths for physical channels and signals

While NR transmission bandwidths can be flexibly configured for most of the physical channels and signals, only limited transmission bandwidths are supported for PDCCH CORESET#0. Furthermore, SSB has a single Tx BW for each SCS. Hence, there is a need to consider what new transmission bandwidths are introduced in the sub-5 MHz WI, especially for SSB and PDCCH.

RAN4 will introduce RF requirements for 3 MHz channel bandwidth according to the WID. LTE supports 15 RB transmission bandwidth on the 3 MHz channel bandwidth, with 0.9 spectrum utilization. As the NR supports up to 25 RBs with 15 kHz SCS for 5 MHz channel bandwidth, corresponding to the same 0.9 spectrum utilization, it is natural to assume that NR will support up to 15 RBs on the 3 MHz channel bandwidth. Hence, we propose that RAN1 will assume maximum Tx BW of 15 RBs for the 3 MHz channel BW until RAN4 achieves agreement on that.

Proposal 1: RAN1 assumes 15 RB max Tx BW for the 3 MHz CBW until RAN4 achieves agreement on that. Request RAN4 feedback through an outgoing LS from RAN1#111.

3 MHz channel bandwidth and maximum Tx BW of 15 RBs is sufficient assumption for the use cases relevant for bands n8, n26 and n28. For these bands, there is no motivation to consider narrower max Tx BWs than 15 RBs; quite the contrary, due to coverage drawbacks of narrower BWs. However, band n100 of 5.6 MHz, allocated to railway mobile radio, requires further considerations as there the sub-5 MHz BW is needed during the migration from GSM-R to FRMCS (Future Railway Mobile Communication System that will use NR). One can anticipate numerous different kinds of migration scenarios depending e.g. on the operation and characteristics of the related railway and GSM-R network deployments. This means that a variety of BWs may be available for FRMCS.

When the the available BW is between 4 MHz and 5 MHz (i.e. from 20 to 25 RBs), the NR design with e.g. 20 RB SSBs works as such. That is, the system will just occypy a fraction of the 5 MHz channel BW.

On the other hand, when the available BW is between 3 MHz and less than 4 MHz (i.e. from 15 to 19 RBs), the UE may assume 3 MHz / 15 RB BW prior to SIB1 acquisition. In other words, 15-RB SSB and CORESET#0 options would be used in such cases.

In [2], [3] UIC states that roughly 10-14 GSM-R carriers are required for safe railway communications on band n100. 10-14 GSM-R carriers occupy 2–2.8 MHz leaving 3.6–2.8 MHz for the NR based FRMCS and necessary guard bands. While 15 RB BW is narrow enough to leave sufficient space for 10 GSM-R carriers, it is too wide to facilitate coexistence with 14 GSM-R carriers. Hence it makes sense to consider an optional second narrower BW for SSB and PDCCH in addition to the 15 RB Tx BW discussed above. 12 or 13 RB BW could be considered as the optional second narrow BW, leaving a total of 640 kHz for the guard bands at the n100 band lower edge as well as between GSM-R and NR.

Observation 1: For FRMCS gradual migration, 3 MHz and 5 MHz channel bandwidths are sufficient from RF viewpoint, while a larger set of L1 transmission bandwidths is needed to facilitate co-existence with up to 14 GSM-R carriers on the n100 band.

Proposal 2: Optional 12 or 13 RB Tx BW is considered for SSB and CORESET#0 in addition to 15 RB Tx BW for FRMCS migragation.

# Initial access

Synchronization signal/PBCH block (SSB) is a core building block of the NR system (Figure 1). Two challenges related to SSB can be identified: the PBCH occupies bandwidth of 3.6 MHz, or 20 RBs, and the existing set of possible frequency positions for SSB is rather coarse for bandwidths below 5 MHz. The PBCH needs to be narrowed down to the desired transmission bandwidth, preferably by means of puncturing, and this is discussed in the following subsection. Additionally, the set of possible SSB frequency positions needs to be redesigned to support NR bandwidths below 5 MHz. This is considered in the subsection 3.2.

Figure 1. SSB structure at 15 kHz subcarrier spacing.

## SSB transmission

In the cell search procedure, the UE acquires time and frequency synchronization to a cell, and determines the physical layer cell ID. The UE does this by searching for the PSS and SSS and decoding the PBCH. The 3GPP work item assumes that the current PSS/SSS design is reused without puncturing, which is possible as its bandwidth, 1.905 MHz, is narrower than the transmission bandwidths, down to around 2.16 MHz, that need to be considered.

Puncturing of the transmitted signal is considered as a solution with minimum change to narrow the transmission bandwidth. In the puncturing operation, the NR base station just blanks the signal mapped on certain predefined RBs that fall outside the desired transmission bandwidth (i.e., it does not transmit them), but otherwise the NR base stations encoding and transmit processing can be kept unchanged. When the UE receives the transmission with punctured RBs, it may just null the punctured RBs at the receiver (e.g. setting the log-likelihood ratios (LLRs) to zero in the channel decoder). Otherwise, the UE’s receiver processing can be kept unchanged. To limit the impact to other UE procedures such as channel estimation for the PBCH demodulation, it is seen preferable to perform puncturing with RB granularity and avoid puncturing of RB fractions. From performance viewpoint, straightforward puncturing is not strictly the optimal solution. For example, PBCH in SSB symbol 3 simply repeats (except for 8 bits) the PBCH bits transmitted in SSB symbol 1 almost on the same subcarriers (with subcarrier offset of 5). As results of puncturing, some codeword bits are not transmitted at all while others are repeated. With an interleaver, the number of different codewords bits actually transmitted on a punctured PBCH could be maximised, improving detection. However, the simplicity and minimised implementation impact of straightforward puncturing outweigths any gains achievable with performance optimisation.

Proposal 3: Perform puncturing of SSB (PBCH) with RB granularity for transmission bandwidths below 20 RBs.

Adding a 3 MHz spectrum allocation to NR would result in a 15 RB channel bandwidth with 15 kHz sub-carrier spacing when 90 % spectrum utilization is assumed. Transmitting PSS and SSS without puncturing means that SSB transmission bandwidth in 3 MHz allocation with 90 % spectrum utilization can be 12, 13, 14 or 15 RBs. For the PBCH, this would mean that 8, 7, 6 or 5 RBs need to be punctured. To avoid affecting the PSS/SSS, at most 4 RBs of the PBCH can be punctured on either side of the SSB, requiring both sides of the PBCH to be punctured to a certain extent.

Observation 2: To avoid affecting the PSS/SSS, less than 5 RBs of the PBCH can be punctured on either side of the SSB, requiring that both sides of the PBCH are punctured to a certain extent.

When considering puncturing of the PBCH to limit the bandwidth, it is evident that the detection performance will degrade compared to the non-punctured case. However, the performance impact of the puncturing can be mitigated if the UE can determine which RBs are punctured before trying to decode the PBCH. In this way, the UE can avoid the noise and potential interference from punctured RBs. Next we analyse the PBCH demodulation performance with different amount of puncturing applied to the SSB. For the reception the UE has basically two options:

- Option 1: the UE is aware of the transmission bandwidth and puncturing pattern of the SSB

- Option 2: the UE is not aware of the transmission bandwidth or puncturing pattern of SSB. Instead, UE detects SSB with current design of 20 RBs.

Table 1 compares the simulated SNR that is required for 1% PBCH BLER for both Options 1 and 2 with and without interference. Interference was modelled as AWGN with 1 RB width and 10 dB higher PSD than NR. The simulation parameters are given in the appendix. In the table, the required SNR is shown when transmission bandwidth of PBCH is 12, 14, 15, 18 or 20 RBs. It can be noted that in the case of 15 RB PBCH, for example, there is 1.1 dB difference in detection performance depending whether the UE is aware of the RBs punctured or not, even when there is no interference present. This difference is dramatically larger with the presence of interference, and the decoding of 15 RB PBCH, for example, does not reach the adequate 1% BLER within reasonable SNR range. Hence, it is crucial for PBCH performance that UE has knowledge of punctured PBCH RBs prior to the PBCH decoding. One possible way for the UE to ascertain which RBs are punctured is to define a relationship between the synchronization raster position and the puncturing. There can be several ways to define the relationship, and the most reasonable way can depend e.g. on the number of supported puncturing patterns.

Observation 3: It is crucial for PBCH performance that UE has knowledge of punctured PBCH RBs prior to the PBCH decoding.

Proposal 4: Study how UE could determine whether or not puncturing is applied to PBCH and what is the applied puncturing pattern.

Table 1 shows also considerable increase in the required SNR with increasing amount of puncturing. However, when the transmission bandwidth reduces, gNB can increase PSD to maintain the same overall PBCH Tx power. This can be taken into account by considering the loss in MCL instead of SNR degradation alone. In Table 2, performance for both PBCH decoding and PBCH DMRS detection is compared for different Tx bandwidths in terms of both required SNR and MCL loss (calculated for constant PBCH Tx power). It can be noted that PBCH decoding is a more limiting factor than the PBCH DMRS detection. The results also show that PBCH with 15 RBs outperforms the 14-RB case by 0.5 dB in MCL when the UE is aware of the puncturing. This is quite remarkable difference given that the MCL is degraded from the non-punctured case by roughly 1 dB with 15 RB Tx bandwidth. Thus, we consider that punctured PBCH should be transmitted using 15 RBs when 15 RB spectrum is available. The performance loss increases further with decreasing PBCH Tx BW, but also other transmission bandwidths below 15 RBs should be considered as required for FRMCS co-existence.

Proposal 5: For punctured PBCH, support at least transmission bandwidth of 15 RBs. FFS other, optional, transmission bandwidths for the punctured PBCH.

Table 1. Required SNR for PBCH detection with 1 % BLER for different puncturing options with and without UE awareness of the transmission bandwidth

| Receiver BW | Interference vs. PBCH PSD [dB] | SNR @ 1% BLER |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 12 RBs TX BW | 14 RBs TX BW | 15 RBs TX BW | 18 RBs TX BW | 20 RBs TX BW |
| Option 1: same as Tx BW | No interf. | -0.9 | -3.1 | -4.0 | -5.4 | -6.3 |
|  | +10 | -0.8 | -2.9 | -3.9 | -5.4 | -6.3 |
| Option 2: 20 RBs | No interf. | 1.2 | -1.8 | -2.9 | -5.0 | -6.3 |
|  | +10 | >> 10 | >> 10 | >> 10 | -4.7 | -6.3 |

Table 2. Required SNR for PBCH detection with 1 % BLER for different puncturing options with UE awareness of the transmission bandwidth

| Detected channel/signal | Metric | Transmission bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 12 RBs | 14 RBs | 15 RBs | 18 RBs | 20 RBs |
| PBCH | SNR @ 1% BLER | -0.9 | -3.1 | -4.0 | -5.4 | -6.3 |
| PBCH DMRS | 1% missed detection | -6.8 | -7.9 | -8.1 | -9.1 | -9.5 |
| PBCH | MCL loss | 3.2 | 1.6 | 1.1 | 0.4 | n/a |
| PBCH DMRS | MCL loss | 0.5 | 0.1 | 0.2 | 0.0 | n/a |

## Channel and synchronization raster

Channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel maps to a resource element on the carrier. The channel raster for n8, n26 and n28 bands is 100 kHz. [4] The synchronization raster indicates the frequency positions of the synchronization block that can be used by the UE for system acquisition when explicit signalling of the synchronization block position is not present. In order to expedite cell search, the synchronization raster is much sparser than the channel raster. In the bands of interest, the channel raster typically has a 100 kHz spacing, but the synchronization raster points are defined in clusters of three points where the points in the cluster are separated by 100 kHz (the raster offsets within a cluster are 50, 150 and 250 kHz) and the clusters are separated by 1200 kHz from each others. The principle is depicted in Figure 2.

Figure 2. Principle of synchronization raster points at frequency range 0-3000 MHz.

Now, for an allowed bandwidth of 3 MHz, and with the principle of not modifying PSS and SSS, the clusters of synchronization raster points need to be separated by less than 1.2 MHz to facilitate two synchronization clusters, as shown in Figure 3. In this figure, two channels of 3 MHz bandwidth separated by 100 kHz are shown. The synchronization raster will therefore need redesigning for narrowband NR operation.

Figure 3. Synchronization raster consideration for 3 MHz bandwidth.

Observation 4: For a bandwidth of 3 MHz, and with the principle of not modifying PSS and SSS, the clusters of synchronization raster points need to be separated by less than 1.2 MHz in order to have at least one valid synchronization raster point for each 3 MHz channel when 100 kHz channel raster is applied.

For the analysis of band n100, two guard band assumptions are considered: 142.5 kHz assuming 3 MHz bandwidth allocation and 242.5 kHz assuming 5 MHz bandwidth allocation, both with 90 % spectrum utilization (SU). Valid sync raster points (so that PSS/SSS can be allocated in the band) for n100 based on existing sync raster design are [4]:

| N | M | SS_REF [Hz] | GSCN |
| --- | --- | --- | --- |
| 768 | 1 | 921650000 | 2303 |
| 768 | 3 | 921750000 | 2304 |
| 768 | 5 | 921850000 | 2305 |
| 769 | 1 | 922850000 | 2306 |
| 769 | 3 | 922950000 | 2307 |

One can observe that current sync raster design in FR1 does not provide sync raster points in n100 to support narrow channel bandwidth allocation in either of the edges of the band when assuming guard bands of 5 MHz NR allocation. In addition, when assuming guard bands necessary for 3 MHz NR allocation (e.g., 142.5 kHz guard) PSS/SSS can be allocated only in the lower part of the n100 band. Thus, a new design would be needed for n100 sync raster in order to support narrowband NR allocation in both ends of the band.

Observation 5: A new sync raster design is needed for band n100 to support narrowband NR allocation in both ends of the band.

As discussed above, the sync raster points (or clusters) should be placed more closely than every 1.2 MHz for 3 MHz NR operation. One tempting design would be to define sync raster points with 100 kHz raster, i.e. same as channel raster. With this approach, only a single SSB puncturing pattern for a given SSB Tx BW would need to be defined.

Observation 6: A possible design would be to define sync raster points with 100 kHz raster, i.e. same as channel raster. With this approach, only a single SSB puncturing pattern for a given SSB Tx BW would need to be defined.

As an illustration in high level, the synchronization raster points for the bands of interest would be as in Figure 4.

Figure 4. Synchronization raster points for punctured and non-punctured SSBs in the bands of interest.

SSB using full transmission bandwidth, e.g. 15 entire RBs within a 15 RB channel bandwidth, means that subcarrier offset between the first subcarrier of the SSB and the first subcarrier of the RB of the common RB grid should be zero.

Observation 7: Subcarrier offset between the first subcarrier of the SSB and the first subcarrier of the RB of the common RB grid should be zero in order to have 15 RB transmission bandwidth for the SSB within a 15 RB channel bandwidth.

For the 15 RB SSB transmission bandwidth in a 15 RB CBW, there are four different options for defining the synchronization raster point as well as the SSB puncturing pattern (‘x+y’) as illustrated in Figure 5.

Figure 5. Synchronization raster and puncturing pattern options for the SSB for the 15 RB allocation.

In order to limit the complexity, for 15 RB channel bandwidth it would be feasible to select one puncturing pattern and thus also one synchronization raster point per channel raster point. Given the channel raster is 100 kHz, also the synchronization raster should be 100 kHz.

Proposal 6: Support one puncturing pattern for the 15 RB punctured SSB for the 15 RB channel bandwidth.

Proposal 7: Conclude that from RAN1 point of view support of 100 kHz synchronization raster for the punctured SSB transmissions is beneficial. Request RAN4 feedback on the feasibility of the support through an outgoing LS from RAN1#111.

Existing synchronization raster points in n8, n26, n28 and n100 bands are in 50, 150 or 250 kHz offset from the channel raster. The new synchronization raster points for the punctured transmissions in the four options illustrated in Figure 5 would be in either +90, -90, -270 or +270 kHz offset to the channel raster meaning that the new synchronization raster points would have 40 kHz (option 1 and option 2) or 20 kHz (option 3 and option 4) minimum offset to the existing sync raster points. Thus, it would make sense to select either option 1 or option 2 so that the minimum frequency difference between new synchronization raster points and existing raster points is maximized. The PBCH detection performance is assumed to be similar for options 1 and 2.

Proposal 8: New 100 kHz synchronization raster for the punctured SSB transmissions is either in +90 or -90 kHz offset to the channel raster.

Proposal 9: Support one of the following puncturing patterns for the SSB for 15 RB SSB bandwidth:

- 2 RBs are punctured from the lower edge of the SSB and 3 RBs from the higher edge of the SSB in frequency when the offset between synch and channel raster is 90 kHz

- 3 RBs are punctured from the lower edge of the SSB and 2 RBs from the higher edge of the SSB in frequency when the offset between synch and channel raster is -90 kHz

Then, as discussed in the previous section we consider also optionally the 12 or 13 RB transmission bandwidth at the n100 band. As shown in Figure 6 by following the same principle as above regarding the offset between channel raster and synch raster in the 15 RB case, we can allocate 13 RB SSB into 13 RB transmission bandwidth. In the figure, we have +90 kHz offset between channel raster and corresponding synch raster (also -90 kHz offset could be used).

Figure 6. Allocating 13 RB SSB on 13 RB channel bandwidth.

Observation 8: 13 RB SSB allocation on 13 RB transmission bandwidth can be facilitated with the following assumptions:

- 100 kHz synchronization raster for the punctured SSB transmissions is either in +90 or -90 kHz offset to the channel raster, as for 15 RB SSB allocation

- Puncturing patterns that are possible for 13 RB SSB bandwidth

  - 3 RBs are punctured from the lower edge of the SSB and 4 RBs from the higher edge of the SSB in frequency when the offset between synch and channel raster is 90 kHz

  - 4 RBs are punctured from the lower edge of the SSB and 3 RBs from the higher edge of the SSB in frequency when the offset between synch and channel raster is -90 kHz

# PDCCH

## Discussion

It is stated in [1] that RAN1 should “identify and specify necessary minimum changes to PDCCH […] for functional support based on existing design, without optimization”. During system information acquisition, UE monitors PDCCH on resources spanning at least 4.32 MHz (i.e. 24 RBs), i.e., exceeding the targeted transmission bandwidths. The PDCCH changes that are necessary to support NR in narrow spectrum allocations should therefore be focused on these PDCCH resources. Another PDCCH aspect requiring attention is the PDCCH frequency domain location with respect to SSB.

The PDCCH is mapped to sets of physical resources known as CORESETs (Control Resource Sets), which in turn is comprised of control channel elements (CCEs). CORESETs can be flexibly configured to the UE after the initial access. However, there are limited configuration options available for the CORESET#0 that are used e.g. for the PDCCH that schedules the transmission of System Information Block 1 (SIB1), known as Type0-PDCCH.

Observation 9: PDCCH for SIB1 requires special consideration.

First, we consider the CORESET#0 frequency domain location with respect to SSB. After the UE has detected PSS and SSS and demodulated the PBCH, the UE has acquired the Master Information Block (MIB) on the PBCH. Next the UE needs to acquire the remaining minimum system information, carried by the SIB1. The UE reads the CORESET#0 configuration index from the MIB on the PBCH, which indicates time and frequency resource allocation parameters for the CORESET. One of the parameters defines the frequency domain offset between the first RB in which the SSB is located and the first RB of CORESET#0. This procedure is illustrated in Figure 7.

Figure 7. CORESET#0 frequency domain resource allocation signalling.

Similar to the PBCH transmission, puncturing may be needed to constrain CORESET#0 within the available spectrum. This leads to situations where the constrained CORESET#0 would contain some partial CCEs. For example when the available spectrum is not an integer multiple of CCE size and is less than 24 RBs, there would be at least one partial CCE. However, if the puncturing of PBCH is not taken into account on the CORESET#0 frequency domain allocation, there would be unnecessarily two CCEs that are punctured. As a CCE is used as the basic resource unit in channel estimation, partial CCEs can degrade channel estimation. Although the use of partial CCEs is unavoidable, it makes sense to minimize the number of partial CCEs. Thus, the CORESET#0 frequency domain allocation may require re-consideration to keep CORESET#0 aligned at the CCE level with the non-punctured RBs of the SSB.

Proposal 10: Keep CORESET#0 aligned at the CCE level with the non-punctured RBs of the SSB

Next, we consider how to fit Type0-PDCCH to a narrower bandwidth than earlier. CORESET#0 spans at least 24 RBs (i.e., 4.32 MHz) with 15 kHz subcarrier spacing. Hence, the CORESET#0 needs to be reduced, for example, by 9 RBs or 12 RBs to reach 15 RB or 12 RB Tx bandwidth, correspondingly. Figure 8 shows the normal CCE indices for 2 and 3 OFDM symbol durations. Two CCE mappings are defined in NR: interleaved and non-interleaved CCE. According to the current NR specification, CORESET#0 always utilizes the interleaved CCE mapping, while other CORESETs that are read later by the UE can use either interleaved or non-interleaved CCE mapping. Hence, we discuss in the following two alternatives for the CORESET#0 changes: use of non-interleaved PDCCH with minor puncturing or solely relying on PDCCH puncturing with interleaved CCE mapping.

It can be observed from Figure 8 the limited number of CCEs available for PDCCH transmission in a narrow bandwidth scenario. For example, for a 3 MHz carrier with only 15 available RBs, the maximum number of CCEs that can be used for PDCCH in the non-interleaved mapping with a 2-symbol CORESET duration is 5. If the PDCCH coverage requirement is such that more CCEs are needed, then the CORESET would have to be extended to a third symbol, allowing up to 7 full CCEs. This will improve the PDCCH coverage.

With the interleaved mapping, the maximum number of full CCEs would be 5 assuming PDCCH punctured from aggregation level 8. On other hand, with non-interleaved CCE mapping and 3-symbol CORESET, only few CCEs (or just one RB) need to be punctured with large aggregation levels. The differences in the amount of required puncturing are shown in Table 3. In the table, we show the maximum number of non-punctured CCEs usable with different ALs for transmission of single PDCCH for both interleaved and non-interleaved mappings. The values are shown for both 15 RB or 12 RB Tx BW. It can be seen that considerably more CCEs can be used for PDCCH transmission with non-interleaved puncturing.

Figure 8. 2-symbol and 3-symbol CORESETs with interleaved and non-interleaved CCEs. PDCCH candidates with AL4 (2-symbol CORESET) and AL8 (3-symbol CORESET) highlighted.

Further, non-interleaved CCE mapping limits the number of PDCCHs that can be multiplexed on the CORESET#0 without extensive puncturing. Multiplexing is important aspect as CORESET#0 is used also for other scheduling than just SIB1. The differences on the multiplexing capacity are illustrated in Table 4 showing the maximum number of AL2 PDCCHs that can be multiplexed without any puncturing for 2-symbol and 3-symbol CORESETs with 15 RB and 12 RB Tx BWs. For example, there exists only two AL2 PDCCH candidates for the interleaved 15 RB CORESET (aggregation of CCE indeces #0 & #1 or CCE indeces #1 & #2). Otherwise to reach 2 CCE transmission, the PDCCH should be transmitted with AL4, from which two CCEs are punctured (e.g. aggregation of CCE indeces #2, #3, #4, #5, from which CCEs #3 and #5 would be punctured).

Hence, the simplest, and preferred, way to maximize the PDCCH coverage and multiplexing capability for Type0-PDCCH in transmission bandwidths of 15 RBs or less is to introduce non-interleaved CCE mapping to CORESET#0 with the necessary minor puncturing. The drawback is reduced PDSCH capacity due to use of 3-symbol CORESET and of course the total PDCCH capacity is reduced compared to the 5 MHz case. To further mitigate the loss from the puncturing, we see that the puncturing should be done with RB resolution, despite of the potential degradation of channel estimation for partial CCEs.

Observation 10: Interleaved CCE mapping limits the number of CCEs available for a single PDCCH as well as the number of PDCCHs that can be multiplexed on the CORESET#0 without extensive puncturing.

Observation 11: PDCCH puncturing is unavoidable with 3MHz channel bandwidth.

Proposal 11: Support non-interleaved CCE mapping also for CORESET#0 comprising 15 RBs or less.

Proposal 12: Support PDCCH puncturing with RB resolution for CORESET#0 comprising 15 RBs or less.

Table 3. Maximum number of full CCEs that can aggregated for single PDCCH with different PDCCH aggregation levels and for 2-symbol and 3-symbol CORESETs having 15 RB or 12 RB Tx BW

| Transmission BW | CCE mapping | Max number of transmitted full CCEs |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2-symbol CORESET |  | 3-symbol CORESET |  |
|  |  | AL8 | AL4 | AL8 | AL4 |
| 15 PRBs | Interleaved CCE | 5 | 3 | 5 | 3 |
|  | Non-interleaved | 5 | 4 | 7 | 4 |
| 12 PRBs | Interleaved | 4 | 2 | 4 | 2 |
|  | Non-interleaved | 4 | 4 | 6 | 4 |

Table 4. Maximum number of AL2 PDCCHs that can be multiplexed on 2-symbol and 3-symbol CORESETs without puncturing for 15 RB and 12 RB Tx BWs.

| Transmission BW | CCE mapping | Max # of AL2 PDCCHs multiplexed |  |
| --- | --- | --- | --- |
|  |  | 2-symbol CORESET | 3-symbol CORESET |
| 15 PRBs | Interleaved CCE | 1 | 1 |
|  | Non-interleaved | 2 | 3 |
| 12 PRBs | Interleaved | n/a | n/a |
|  | Non-interleaved | 2 | 3 |

## PDCCH performance

In here, we consider the PDCCH link performance with CCE puncturing. As mentioned, the need for puncturing varies according the bandwidth available, interleaving configuration and the number OFDM symbols in the CORESET. To see the performance limits for CCE puncturing we consider the scenario relying solely on CCE puncturing with interleaved CCE mapping.

Table 5 compares the simulated PDCCH link performance for Type0-PDCCH with different transmission bandwidths, for a two-symbol CORESET with interleaved mapping. The simulation parameters are given in the appendix. In all cases the Type0-PDCCH is rate matched for AL8 PDCCH (within 24-RB CORESET) but some of the RBs are punctured, resulting in PDCCH transmission bandwidths of 6 CCEs (18 RBs), 5 CCEs (15 RBs) and 4 CCEs (12 RBs). In the table, a change in MCL is shown when punctured PDCCH performance is compared against the unmodified AL8 PDCCH on 24 RB CORESET.

The results show that puncturing an 8-CCE PDCCH down to 5 CCEs (i.e., 15 RBs) and to 4 CCEs (12 RBs) will cause a 0.9 dB loss and 2.0 dB loss, respectively, in MCL when compared to the non-punctured case. In the simulations, UE had prior knowledge of the punctured RBs, which is important for keeping the PDCCH performance degradation at a reasonable level. For RRC connected UEs, the knowledge can be made available for UE via RRC configuration, while for UEs without RRC configuration, the puncturing information can be derived from PBCH/MIB.

Observation 12: Puncturing an 8-CCE PDCCH down to 5 CCEs (i.e., 15 RBs) and to 4 CCEs (i.e. 12 RBs) will cause a 0.9 dB loss and 2.0 dB loss, respectively, in MCL when compared to the non-punctured case in case of 2-symbol CORESET

Proposal 13: To minimize the loss due to PDCCH puncturing, UE should know the punctured RBs in advance.

Table 5. PDCCH detection loss due to puncturing [dB] @ 1% BLER (compared to aggregation of 8 CCEs) with interleaved CCE mapping.

|  | CORESET#0 size in frequency |  |  |
| --- | --- | --- | --- |
|  | 12 RBs | 15 RBs | 18 RBs |
| MCL loss [dB] | 2.0 | 0.9 | 0.7 |

# Other channels

## PRACH

When initiating the random access procedure, UE has received the SIB1. From the system information the UE determines the initial UL BWP configuration used in the cell. Since the network can configure initial UL BWP and can also allocate Random Access Occasions (RO(s)) in frequency within the initial UL BWP in flexible manner the current RACH configuration allows sufficient flexibility also for narrowband NR operation and thus it can be used as is.

NR defines four long and nine short PRACH preamble formats as tabulated on Table 10 in Appendix 2. Long preambles use sequences of length L=839 and subcarrier spacing of either 1.25 kHz or 5 kHz. Short preambles have a sequence length L=139 and the subcarrier spacing is the same as for other NR uplink channels, which is agreed in WID to be 15 kHz for the considered bands. It can be noted from the table that all the PRACH preamble formats except one long format (Format 3 using 5 kHz subcarrier spacing and designed for high speed users) fit into 12 RB bandwidth.

It is further noted that, for instance, Format 0 (long preamble format with 1.25 kHz subcarrier spacing) can support high UE speeds at 1 GHz carrier frequency with only about 1.7 dB loss in sensitivity compared to Format 3 as shown in Appendix 2. Format 0 can support also the same cell ranges as Format 3. Thus, no changes are needed for random access or PRACH formats other than excluding the use of Format 3 in spectrum allocations below 5 MHz.

Observation 13: No changes are required to PRACH formats to support below 5 MHz NR bandwidths.

## PUCCH

Before UE receives dedicated PUCCH resource configuration, e.g. during the initial access to the cell, UE uses PUCCH resources from a PUCCH resource set determined in Table 9.2.1-1, TS38.213. The PUCCH resource frequency domain allocation is determined relative to the BWP edges of initial UL BWP, which in turn can be flexibly configured in the SIB1 as discussed earlier. Also the UE specific dedicated PUCCH resource configuration can be made according to the available bandwidth. Hence, NR already provides sufficient PUCCH configuration flexibility for spectrum allocations below 5 MHz.

However, narrower bandwidths inevitably degrade the PUCCH performance as the frequency separation between the hops in the intra-slot hopping is reduced, decreasing also the achievable the PUCCH frequency diversity. In the case of the predefined PUCCH resource sets, the hopping distance is unequal among the predefined PUCCH resources:

- PUCCH resource sets comprise resources on both edges of initial UL BWP. The distance from the BWP edge is given with resource set specific RB offset, ranging from 0 to 4 RBs

- Within each PUCCH resource set, the PUCCH resources are allocated starting from the most distant RBs of the PUCCH resource set, and progressing towards inner RBs of the PUCCH resource set with increasing PUCCH resource index.

With normal NR BWPs, the PUCCH hopping distance remains clearly wider than channel coherence bandwidth in most cases and do not cause performance difference between the resources. However, with narrow BWPs, it is worth to have a closer look on the hopping distances.

Table 6 compares the simulated PUCCH detection performance for 14 symbol PUCCH Format 1 with reducing frequency separation between the hops. The simulation parameters are given in the appendix. The SNR required for 1% BER was compared against the PUCCH having a frequency hop of 24 RBs, and the increase in the required SNR is reported on the table. It can be noted that there is no drastic sensitivity loss when frequency hopping is 12 RBs or more, but the sensitivity loss becomes considerable when the frequency hopping is less than 8 RBs.

Table 6. PUCCH Format 1 sensitivity loss @ 1% BER due to reduced frequency diversity

| Frequency hopping distance [RB] | Sensitivity loss [dB] @ 1% BER |  |
| --- | --- | --- |
|  | 3 km/h​ | 120 km/h​ |
| 15​ | 0.0 | 0.6 |
| 12​ | 0.1​ | 0.7 |
| 8​ | 0.8​ | 1.2 |
| 4​ | 1.6​ | 2.0 |
| 2​ | 2.0​ | 2.8 |
| 0​ | 2.1​ | 2.7 |

Considering the PUCCH resource sets, it can be noted that

- For a BWP of 15 RBs, there are 11 PUCCH resource sets where all PUCCH resources have a frequency hop of at least 8 RBs. In additional 2 resource sets, at least some of the PUCCH resources have a frequency hop of at least 8 RBs. On the other side, there are 3 resource sets with frequency hop less than 8 RBs.

- Even in the case of a BWP of 12 RBs, there are 3 resource sets where all PUCCH resources have a frequency hop of more than 8 RBs. There are also additional 5 resource sets where at least some of the PUCCH resources have frequency hop of at least 11 RBs, which is the maximum for the considered BWP size. On the other side, there are 8 resource sets with frequency hop less than 8 RBs.

Hence, there are several PUCCH resource sets that have sufficient frequency hop even with narrow BWPs. There are also further PUCCH resource sets, where at least some PUCCH resources have maximum frequency hop for the given BWP size. Network can use these resources and avoid the use of PUCCH resource sets or PUCCH resources with the largest reduction in the hopping distance. As discussed above, there are also some PUCCH resources and even PUCCH resource sets that suffer significantly from the reduced hopping distance. These resources could benefit somewhat e.g. from modified frequency hopping ensuring the same hopping distance among all the resources within a set. However, the change would not be pivotal e.g. for resource sets with maximum hopping distance below 8 RBs as the hopping distance would anyway remain below 8 RBs. Hence, we do not see need to modify PUCCH and PUCCH can be used as it is.

Observation 14: No changes are required to PUCCH to support below 5 MHz NR bandwidths.

- Even with narrow BWPs, there are several PUCCH resource sets and even more PUCCH resources that have sufficient frequency separation between the frequency hops.

## CSI-RS

CSI-RS is configured with granularity of 4 RBs in frequency domain. This is rather coarse for a bandwidth of, e.g., 15 RBs for which the widest integer multiple of 4 RBs is 12 RBs. However, the CSI-RS can be allocated to cover entirely bandwidth parts that are not integer multiple of 4 RBs. That can be provided by configuring CSI-RS allocation that is wider in frequency domain than the corresponding bandwidth part. Thus, CSI-RS can support narrowband NR operation where the configured bandwidth part has bandwidth of 15 RBs or 18 RBs for instance.

Observation 15: No changes are required to CSI-RS to support below 5 MHz NR bandwidths.

# Conclusion

In this contribution we have discussed the required changes to support NR operation with below 5 MHz bandwidth. Based on the discussion we make the following observations and proposals:

## Bandwidth

Proposal 1: RAN1 assumes 15 RB max Tx BW for the 3 MHz CBW until RAN4 achieves agreement on that. Request RAN4 feedback through an outgoing LS from RAN1#111.

Observation 1: For FRMCS gradual migration, 3 MHz and 5 MHz channel bandwidths are sufficient from RF viewpoint, while a larger set of L1 transmission bandwidths is needed to facilitate co-existence with up to 14 GSM-R carriers on the n100 band.

Proposal 2: Optional 12 or 13 RB Tx BW is considered for SSB and CORESET#0 in addition to 15 RB Tx BW for FRMCS migragation.

## Initial access

Proposal 3: Perform puncturing of SSB (PBCH) with RB granularity for transmission bandwidths below 20 RBs.

Observation 2: To avoid affecting the PSS/SSS, less than 5 RBs of the PBCH can be punctured on either side of the SSB, requiring that both sides of the PBCH are punctured to a certain extent

Observation 3: It is crucial for PBCH performance that UE has knowledge of punctured PBCH RBs prior to the PBCH decoding.

Proposal 4: Study how UE could determine whether or not puncturing is applied to PBCH and what is the applied puncturing pattern.

Proposal 5: For punctured PBCH, support at least transmission bandwidth of 15 RBs. FFS other, optional, transmission bandwidths for the punctured PBCH.

Observation 4: For a bandwidth of 3 MHz, and with the principle of not modifying PSS and SSS, the clusters of synchronization raster points need to be separated by less than 1.2 MHz in order to have at least one valid synchronization raster point for each 3 MHz channel when 100 kHz channel raster is applied.

Observation 5: A new sync raster design is needed for band n100 to support narrowband NR allocation in both ends of the band.

Observation 6: A possible design would be to define sync raster points with 100 kHz raster, i.e. same as channel raster. With this approach, only a single SSB puncturing pattern for a given SSB Tx BW would need to be defined.

Observation 7: Subcarrier offset between the first subcarrier of the SSB and the first subcarrier of the RB of the common RB grid should be zero in order to have 15 RB transmission bandwidth for the SSB within a 15 RB channel bandwidth.

Proposal 6: Support one puncturing pattern for the 15 RB punctured SSB for the 15 RB channel bandwidth.

Proposal 7: Conclude that from RAN1 point of view support of 100 kHz synchronization raster for the punctured SSB transmissions is beneficial. Request RAN4 feedback on the feasibility of the support through an outgoing LS from RAN1#111.

Proposal 8: New 100 kHz synchronization raster for the punctured SSB transmissions is either in +90 or -90 kHz offset to the channel raster.

Proposal 9: Support one of the following puncturing patterns for the SSB for 15 RB SSB bandwidth:

- 2 RBs are punctured from the lower edge of the SSB and 3 RBs from the higher edge of the SSB in frequency when the offset between synch and channel raster is 90 kHz

- 3 RBs are punctured from the lower edge of the SSB and 2 RBs from the higher edge of the SSB in frequency when the offset between synch and channel raster is -90 kHz

Observation 8: 13 RB SSB allocation on 13 RB transmission bandwidth can be facilitated with the following assumptions:

- 100 kHz synchronization raster for the punctured SSB transmissions is either in +90 or -90 kHz offset to the channel raster, as for 15 RB SSB allocation

- Puncturing patterns that are possible for 13 RB SSB bandwidth

  - 3 RBs are punctured from the lower edge of the SSB and 4 RBs from the higher edge of the SSB in frequency when the offset between synch and channel raster is 90 kHz

  - 4 RBs are punctured from the lower edge of the SSB and 3 RBs from the higher edge of the SSB in frequency when the offset between synch and channel raster is -90 kHz

## PDCCH

Observation 9: PDCCH for SIB1 requires special consideration

Proposal 10: Keep CORESET#0 aligned at the CCE level with the non-punctured RBs of the SSB

Observation 10: Interleaved CCE mapping limits the number of CCEs available for a single PDCCH as well as the number of PDCCHs that can be multiplexed on the CORESET#0 without extensive puncturing.

Observation 11: PDCCH puncturing is unavoidable with 3MHz channel bandwidth.

Proposal 11: Support non-interleaved CCE mapping also for CORESET#0 comprising 15 RBs or less.

Proposal 12: Support PDCCH puncturing with RB resolution for CORESET#0 comprising 15 RBs or less.

Observation 12: Puncturing an 8-CCE PDCCH down to 5 CCEs (i.e., 15 RBs) and to 4 CCEs (i.e. 12 RBs) will cause a 0.9 dB loss and 2.0 dB loss, respectively, in MCL when compared to the non-punctured case in case of 2-symbol CORESET

Proposal 13: To minimize the loss due to PDCCH puncturing, UE should know the punctured RBs in advance.

## Other channels and signals

Observation 13: No changes are required to PRACH formats to support below 5 MHz NR bandwidths.

Observation 14: No changes are required to PUCCH to support below 5 MHz NR bandwidths.

- Even with narrow BWPs, there are several PUCCH resource sets and even more PUCCH resources that have sufficient frequency separation between the frequency hops.

Observation 15: No changes are required to CSI-RS to support below 5 MHz NR bandwidths.

# References

- RP-222645 , “Revised WID: NR support for dedicated spectrum less than 5MHz for FR1”, Nokia, Nokia Shanghai Bell

- RWS-210280, “NR Narrowband Channel Bandwidth (<5MHz) Railway Use Case”, UIC

- RWS-210601, “Email discussion summary for [RAN-R18-WS-non-eMBB-UIC] - Version 0.0.6 RAN”, UIC

- 3GPP TS 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 17)”

# Appendix 1 Link simulation assumptions

In the calculation of MCL, PBCH Tx power remains constant for all Tx BWs, i.e., PSD increases with reducing Tx BW

Table 7. Simulation assumptions for PBCH

| Carrier frequency | 900 MHz |
| --- | --- |
| Antenna setup | 1x2 |
| Channel model | TDL-C 300ns 3km/h |
| Subcarrier spacing | 15 kHz |
| Resource allocation | Resource allocations of 12,14,15,18 and 20 RBs for PBCH is studied PBCH puncturing patterns: 18 RBs – 1RB from lower end, 1 RB from upper end (L1/U1); 15 RB - L3/U2; 14 RB – L3/U3; 12 RB – L4/U4 |
| GSM interference outside transmission BW | 1 RB allocation with 1 empty RB between interference and PBCH +10dB PSD offset to NR PSD Interference modeled as AWGN |
| Receiver assumptions on bandwidth | Receiver detects whole 20 RB BW with GSM interference (Option 2) or uses the same BW as in TX side (Option 1) |

Table 8. Simulation assumptions for PDCCH

| Carrier frequency | 900 MHz |
| --- | --- |
| Antenna setup | 1x2 |
| Channel model | TDL-C 300ns 3km/h |
| Subcarrier spacing | 15 kHz |
| CORESET allocation | 2 and 3 symbols 24 RB CORESET limited to 12, 15 and 18 RBs |
| Receiver assumptions on Tx bandwidth | Receiver detects using same BW as in TX side |
| Payload | 40 bits + 24 bit CRC |

Table 9. Simulation assumptions for PUCCH

| Carrier frequency | 900 MHz |
| --- | --- |
| Antenna setup | 1x2 |
| Channel model | TDL-C 300ns 3 km/h. 120 km/h |
| Subcarrier spacing | 15 kHz |
| PUCCH | Format 1 14 symbols |

# Appendix 2 Link budget analysis for PRACH formats

PRACH formats in NR are provided in Table 10.

Table 10. NR PRACH formats

Link budget analysis comparing Format 0 and Format 3 is provided in Table 11.

Table 11. Link budget analysis between Format 0 and Format 3