3GPP TSG-RAN WG1 Meeting #111	R1-2212409

Toulouse, France, 14th – 18th November 2022

Agenda Item:	8.6

Title:	Msg1/MsgA retransmission timeline for RedCap UEs

Source:	Ericsson

Document for:	Discussion

# 1	Introduction

This contribution discusses the potential clarifications of the Msg1/Msg3 retransmission timeline for Rel-17 RedCap UEs [1, 2] which has been brought up in earlier meetings [3, 4].

# 2	Discussion

The Msg1 retransmission timeline is specified in the following way in 38.213 [5] clause 8.2:

If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window and LSBs of a SFN field in the DCI format 1_0, if included and applicable, are not same as corresponding LSBs of the SFN where the UE transmitted PRACH, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit a PRACH. If requested by higher layers, the UE shall be ready to transmit a PRACH no later than NT,1+0.75 msec after the last symbol of the window, or the last symbol of the PDSCH reception, where NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 assuming μ corresponds to the smallest SCS configuration among the SCS configurations for the PDCCH carrying the DCI format 1_0, the corresponding PDSCH when additional PDSCH DM-RS is configured, and the corresponding PRACH. For μ=0, the UE assumes N1,0=14 [6, TS 38.214]. For a PRACH transmission using 1.25 kHz or 5 kHz SCS, the UE determines N1 assuming SCS configuration μ=0.

The MsgA retransmission timeline is specified in a similar way in 38.213 [5] clause 8.2A.

RAN1 has received an LS from RAN2 [6] with the following content:

1. Overall Description: Regarding the following scenario mentioned by RAN1 in the LS provided in R1-2112802: - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. RAN2 has discussed this scenario and how a RedCap UE performs RSRP measurements before Msg1 or MsgA retransmission on separate initial UL BWP and agreed on the following." - “From RAN2 perspective, if a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, it is up to UE implementation to perform new RSRP measurement in a DL BWP associated with CD-SSB before Msg1/A retransmission.” RAN2 respectfully asks RAN4 and RAN1 to take the agreement above into consideration and check if any update is required in their specifications. 2. Actions: To RAN4 and RAN1 ACTION: RAN2 kindly asks RAN4 and RAN1 to take the agreement above into consideration and check, as it is up to RAN4 and RAN1, if any update is required in their specifications.

Note that the LS concerns the case when a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH. It is important that any potential clarification or relaxation does not impact other cases such as the non-RedCap UE case or the RedCap UE case when the BWP includes SSB. It is also important that the potential clarification of the RAN1 specification does not break the relation to the MAC procedure in 38.321 [6] clause 5.1 (Random access procedure).

If the above can be ensured, we are open to consider a clarification or relaxation of the Msg1/MsgA retransmission timeline for the mentioned case.

# References

| [1] | RP-220966 | Revised WID on support of reduced capability NR devices | Ericsson |
| --- | --- | --- | --- |
| [2] | R1-2210636 | RAN1 agreements for Rel-17 NR RedCap | Rapporteur (Ericsson) |
| [3] | R1-2207729 | FL summary #3 for Rel-17 RedCap maintenance | Moderator (Ericsson) |
| [4] | R1-2210247 | FL summary #3 on Rel-17 RedCap maintenance | Moderator (Ericsson) |
| [5] | 38.213 V17.3.0 | NR; Physical layer procedures for control (Release 17) | 3GPP TSG RAN |
| [6] | R1-2200877 | LS on RSRP measurement before Msg1 or MsgA retransmission | RAN2, Ericsson |
| [7] | 38.321 V17.2.0 | NR; Medium Access Control (MAC) protocol specification (Release 17) | 3GPP TSG RAN |