3GPP TSG RAN WG1#111		R1-2212434

Toulouse, France, November 14th – 18th, 2022

Agenda Item:	8.2

Source:	Moderator (Lenovo)

Title:	FL Summary for B52.6 GHz PDCCH monitoring enhancements

Document for:	Discussion, Decision

## Issues for PDCCH monitoring enhancements

Table 1 – Identified issues for PDCCH monitoring enhancements

| Issue# | Issue | References | FL initial assessment | Comments / Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 8.2.2-1 | PDCCH monitoring occasion for DCI format 2_1 | [1]+[2] | H | FL: This issue appears to be on the face similar to RAN1#110 and RAN1#110bis_e contributions, however CATT has provided an updated motivation, explaining that without the proposed correction the legacy behaviour for URLLC pre-emption is changed for multi-slot monitoring. |
| 8.2.2-2 | Definition of configured DL-CCs number for BD/CCE budge | [3]+[4] | H | FL: The suggested change "with associated PDCCH candidates" may be vague, this could be addressed during the RAN1#111 discussion. |
| 8.2.2-3 | Clarification of multi-slot monitoring in groups of slots | [5]+[6] | H |  |

## Discussion/Comments

Issue 8.2.2-1: PDCCH monitoring occasion for DCI format 2_1

In [2] CATT has suggested a draft CR to introduce relaxations to monitoring requirements for DCI format 2-1.

Moderator suggests to consider the following possible agreements/conclusions:

Option 8.2.2-1.1: Adopt CR in [2], i.e. specify that the UE does not expect to be configured with more than one PDCCH monitoring occasion for DCI format 2_1 in a slot group of Xs slots when configured with monitoringSlotsWithinSlotGroup

Option 8.2.2-1.2: Conclude that PDCCH monitoring relaxations are not supported for DCI format 2_1 for a UE configured with monitoringSlotsWithinSlotGroup with 480/960 kHz.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We prefer Option 8.2.2-1.2 |
| vivo | We are fine with either option |
| CATT | We prefer option 8.2.2-1.1. |
| LG Electronics | Fine with Option 8.2.2-1.2 but still not sure this conclusion is needed. What's the difference between having and not having this conclusion? |
| Ericsson | We prefer Option 8.2.2-1.2. We do not support Option 8.2.2-1.1 since the “default” UE capability is (Xs,Ys) = (4,1) for 480 kHz and (8,1) for 960 kHz, so the UE will not monitor more than once anyway. For other values of (Xs,Ys), those are optional anyway, and we prefer not to introduce new UE behavior at this point. |

Proposal #1:

Adopt the following as conclusion in the chairman notes:

PDCCH monitoring relaxations are not supported for DCI format 2_1 for a UE configured with monitoringSlotsWithinSlotGroup with 480/960 kHz.

Issue 8.2.2-2: Definition of configured DL-CCs number for BD/CCE budget

In [4] CATT has suggested a draft CR to change the definition of Ncells,r17DL,μ to “with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ”. During Tuesday online session it was questioned whether the word "associated" is needed in the changed text, this should be further discussed.

| Company | Comment |
| --- | --- |
| Moderator | Upon closer inspection, the latest version of 38.213 uses the suggested wording in similar context, e.g.: If a UE is configured with Ncells,r16DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, where μ=01Ncells,r16DL,μ≤Ncellscap-r16, the UE is not required to monitor, on the active DL BWP of the scheduling cell, more than MPDCCHtotal,(X,Y),μ=MPDCCHmax,(X,Y),μ PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=CPDCCHmax,(X,Y),μ non-overlapped CCEs per span for each scheduled cell when the scheduling cell is from the Ncells,r16DL,(X,Y),μ downlink cells. Therefore the suggested wording in [4] should be fine. |
| Huawei, HiSilicon | Agree with Moderator’s comment |
| vivo | Agree with Moderator’s comments |
| LG Electronics | Agree with Moderator’s comments |
| Ericsson | Agree with moderators suggestion to use wording in [4] |

Proposal #2:

Draft CR R1-2211159 is endorsed.

Issue 8.2.2-3: Clarification of multi-slot monitoring in groups of slots

The draft CR in [6] is endorsed in principle, however some modification is necessary to avoid ambiguity within 38.213 due to other uses of symbol L. It may be simplest to add a subscript to disambiguate, FL suggests to add subscript s which aligns with symbol Ts.

TP#1:

A UE determines a PDCCH monitoring occasion on an active DL BWP from the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern within a slot. If monitoringSlotsWithinSlotGroup is not provided, the UE determines that PDCCH monitoring occasions exist in a slot with number ns,fμ [4, TS 38.211] in a frame with number nf if (nf Nslot frame, μ+ ns,f μ- os) modks=0nfNslotframe,μ+ns,fμ-op,smod kp,s=0. The UE monitors PDCCH candidates for search space set s for Ts consecutive slots, starting from slot ns,fμ, and does not monitor PDCCH candidates for search space set s for the next ks-Ts consecutive slots. If monitoringSlotsWithinSlotGroup is provided, for search space set s, the UE determines that the slot with number ns,fμ [4, TS 38.211] in a frame with number nf satisfying (nf Nslot frame, μ+ ns,f μ- os) modks=0 is the first slot in a first group of Ls slots and that PDCCH monitoring occasions exist in Ts/Ls consecutive groups of slots starting from the first group, where Ls is the size of monitoringSlotsWithinSlotGroup. The UE monitors PDCCH candidates for search space set s within Tseach of the Ts/Ls consecutive groups of slots according to monitoringSlotsWithinSlotGroup, starting from slot ns,fμ, and does not monitor PDCCH candidates for search space set s for the next ks-Ts consecutive slots.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | OK with FL suggestion to add subscript s to L |
| vivo | We support the draft CR and moderator’s suggestion |
| LG Electronics | OK with FL suggestion |
| Ericsson | Okay to add subscript s to L in order to avoid an ambiguity with the variable L that is already used in the same section of 38.213 for PDCCH aggregation level. |

Proposal #3:

Draft CR R1-2211948 is endorsed with changing symbol L to Ls.

## References

Contributions related to PDCCH monitoring enhancements

[1]	R1-2211156	Discussion on the correction for PDCCH monitoring occasion for DCI format 2_1 for the features extending NR operation to 71 GHz	CATT

[2]	R1-2211157	Correction for PDCCH monitoring occasion for DCI format 2_1 for the features extending NR operation to 71 GHz	CATT

[3]	R1-2211158	Discussion on the corrections of the definition of configured DL-CCs number for BD/CCE budge of  scheduling cell(s)  for the features extending NR operation to 71 GHz	CATT

[4]	R1-2211159	corrections of the definition of configured DL-CCs number for BD/CCE budge of  scheduling cell(s)  for the features extending NR operation to 71 GHz	CATT

[5]	R1-2211947	Discussion on per-slot group monitoring within a duration	Ericsson

[6]	R1-2211948	Draft CR on per-slot group monitoring within a duration	Ericsson