3GPP TSG-RAN WG1 #111			R1-2212460

Toulouse, France, November 14 – 18, 2022

Title:	Draft LS to RAN2 on correcting interpretation of moreThanOneNackOnlyMode

Response to:

Release:	Rel-17

Work Item:	NR_MBS-Core

Source:	Huawei [to be TSG-RAN WG1]

To:	TSG-RAN WG2

Cc:

Contact Person:

#### Name:	Jinhuan XIA

Tel. Number:

E-mail Address:	jinhuan.xia@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	No

- Overall Description:

Background:

During RAN1 discussion for Rel-17 NR MBS maintenance, it is discussed that whether the higher layer parameter moreThanOneNackOnlyMode includes the case where HARQ-ACK is for a single TB.

After the discussion in RAN1, it is clarified that, firstly, in either mode for NACK-only indicated by moreThanOneNackOnlyMode, gNB can still schedule a single TB with NACK-only feedback in a PUCCH. Secondly, for the case of HARQ-ACK bit for a single TB, the PUCCH resource determination approaches for the two NACK-only modes are different, although the PUCCH format and the PUCCH transmission is the same as specified in TS38.213.

With the above clarification, RAN1 suggests the following text proposal to the RRC parameter name moreThanOneNackOnlyMode and to the field description in TS38.331 v17.2.0 to avoid confusion whether UE can be scheduled to feedback a single HARQ-ACK bit in a PUCCH transmission in either mode:

moreThanOneNackOnlyMode Indicates the mode of supporting more than one NACK-only feedback in the same PUCCH transmission when the number of HARQ-ACK bit is more than one. Mode 1 means UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits for more than one HARQ-ACK bit. Mode 2 means UE transmitting a specific sequence or a PUCCH transmission corresponding to the combination of more than one NACK-only HARQ feedback according to the mapping of values of HARQ-ACK information bits to PUCCH resources as specified in TS 38.213 [13]. In either mode, the PUCCH transmission for a single HARQ-ACK bit is specified in TS38.213 [13]. If multicast CFR is not configured, this field is not included. Otherwise, if the field is absent, UE uses mode 1 for multicast CFR.

RAN1 would like RAN2 to take into account the above text proposal suggested by RAN1 and check whether other RAN2 specification is affected and request RAN2 to update the RAN2 specifications where needed.

2. Actions:

To TSG-RAN WG2

ACTION: 	RAN1 respectfully requests RAN2 to take into account the text proposal suggested by RAN1 and check whether other RAN2 specification is affected and request RAN2 to update the RAN2 specifications where needed.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #112                              27 February – 3 March 2023	  	 Athens, Greece

TSG-RAN WG1 Meeting #112bis-e                      17 – 26 April 2023	  	                Online