3GPP TSG-RAN WG1 Meeting #112	R1-2300103

Athens, Greece, 27 February – 3 March 2023

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on RAN2 Reply LS to RAN1 on default CBR configuration

Document for:	Discussion and Decision

- Introduction

A RAN2 LS was sent to RAN1 [1], where the following questions was asked:

When UE performs full sensing in R16/17 normal resource pool and has full sensing result available, is it a valid case that UE has no CBR measurement results and makes use of R16 default CBR for congestion control?

RAN1 was requested to provide answer to Q1 above

- Discussions

Given the CBR and SL RSSI definition as per TS38.215:

SL Channel Busy Ratio (SL CBR) measured in slot n is defined as the portion of sub-channels in the resource pool whose SL RSSI measured by the UE exceed a (pre-)configured threshold sensed over a CBR measurement window [n-a, n-1], wherein a is equal to 100 or 100·2µ slots, according to higher layer parameter sl-TimeWindowSizeCBR.

Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH, starting from the 2nd OFDM symbol.

SL CBR is determined based on SL RSSI measurement results. For a UE performing full sensing in R16/17 normal resource pool and having full sensing result available, it means the UE has already monitored SCI and performed RSRP measurement over PSCCH or PSSCH DMRS in the sensing window as per TS38.214. Therefore, in this case it does not seem reasonable to assume the UE cannot measure the total received power (RSSI).

On the other hand, sidelink congestion control (15-5) is the basic FG for NR sidelink, where a UE has to measure CBR to determine CRLimitk.  In mode 2 resource allocation, the UE shall ensure i≥kCRi≤CRLimitk as per TS38.214 for congestion control. Therefore, for a UE performs full sensing in a normal resource pool, it shall also measure the CBR. However, there may be a few special cases:

- When all slots of the CBR measurement window [n-a, n-1] (note these slots are physical slots as per TS38.215) does not belong to the resource pool, the portion of sub-channels whose SL RSSI measured by the UE exceed a (pre-)configured threshold in the resource pool cannot be determined, and thus CBR result in slot n does not exist. In this case (only), the default CBR may be used.

- When there are a number of slots within CBR measurement window [n-a, n-1] belonging to the resource pool, however, the UE does not monitor all these slots due to half-duplex (i.e. step 5 of resource exclusion procedure), there does not exist CBR result.

It should be noted that, the above special cases are rare in practical deployment, i.e. the entire slots of CBR measurement window (length is 100 slots or 100ms) are missed either due to HD or completely excluded from the resource pool.

Proposal 1:  Reply to RAN2 on Q1 as follows:

- When the UE performs full sensing in R16/17 normal resource pool and has full sensing result available, the UE shall measure and obtain CBR result except some special cases:

  - If CBR measurement window [n-a, n-1] does not belong to the resource pool, there is no CBR result for the UE in slot n.

  - The UE does not monitor all the slots which belongs to the resource pool due to step 5 (half duplex) within CBR measurement window [n-a, n-1].

  - In the above cases, default CBR result can be used.

- Conclusions

Following proposal was made to provide an answer to RAN2:

Proposal 1:  Reply to RAN2 on Q1 as follows:

- When the UE performs full sensing in R16/17 normal resource pool and has full sensing result available, the UE shall measure and obtain CBR result except some special cases:

  - If CBR measurement window [n-a, n-1] does not belong to the resource pool, there is no CBR result for the UE in slot n.

  - The UE does not monitor all the slots which belongs to the resource pool due to step 5 (half duplex) within CBR measurement window [n-a, n-1].

  - In the above cases, default CBR result can be used.

# References

- R1-2300016 “Reply LS to RAN1 on default CBR configuration”, RAN2, February, 2023