3GPP TSG RAN WG1 #112	 	                                               R1-2300282

Athens, Greece, February 27th – March 3rd, 2023

Source:	OPPO

Title:	Evaluation methodology and results on AI/ML for beam management

Agenda Item:	9.2.3.1

Document for:	Discussion

Introduction

The SID [1] of artificial intelligent (AI) and machine learning (ML) for NR air interface was agreed in RAN#94e meeting. The initial set of use cases including beam management was selected as followings

- CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

- Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

- Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

In this contribution, we present our evaluations on spatial and/or temporal domain beam prediction.

Evaluations and discussions

Use case of beam management

According to the SID, the AI/ML beam management can be categorized into the following sub use cases, but not limited to

- Beam prediction in spatial domain for overhead/latency reduction

- Beam prediction in time domain for overhead/latency reduction

- Beam selection accuracy improvement

In this contribution, we evaluate the 1st two sub use cases, e.g. BM-Case1 and BM-Case2 as defined in RAN1#109e, with Top-K Tx-Rx beam pair(s) prediction. Moreover, the dense urban scenario with SLS has been agreed as the baseline in the agreements below. Additionally, we evaluate BM-Case1 for the indoor hotspot (InH) scenario as supplementary results in Appendix II.

Agreement

- For dataset construction and performance evaluation (if applicable) for the AI/ML in beam management, system level simulation approach is adopted as baseline

  - Link level simulation is optionally adopted

Agreement

- At least for temporal beam prediction, Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation.

  - Other scenarios are not precluded.

- For spatial-domain beam prediction, Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation.

  - Other scenarios are not precluded.

Dense urban scenario for BM-Case1

Conventionally, NW broadcasts up to 64 SSBs periodically at FR2 for DL beam sweeping. Deployed with either single panel or multiple panels, UEs sweep all its Rx beams (typically 4 Rx beams) per panel to search the best DL beam pair(s). To fulfill the P1 procedure specified in Rel.15, large DL overhead and latency would be expected.

Intuitively, both NW and UE apply DL Tx and Rx beams respectively with certain pattern. For instance, NW carries out analog beamforming in both vertical and horizontal direction, e.g. 8 beams in horizontal domain and 4 beams in vertical domain (depicted in Figure 2). The combinations of vertical beam and horizontal beam can be seen by UE as total 32 DL Tx beams. Then the number of DL beam pairs in Set A is 128 (32 Tx beams and 4 Rx beams) as illustrated in Figure 1.

Assuming the beamforming pattern can be somehow learnt by neural network (NN), then only a limited number of DL measurement would be enough for NN to infer the quality of all DL beam pair links. The benefit comes from that a large portion of DL measurement of beam pairs can be replaced by beam prediction in spatial domain. This can be called as super-resolution problem.

One may wonder how the Tx beams for UE to measure are selected, i.e. part of Set B selection. We show our selection on Tx beams in Figure 2 where the highlighted circles in both vertical and horizontal domain. They are the most “welcomed” Tx beams among UEs within a cell. The selected Tx beams represent a spatial pattern of beamforming which are widely adopted by UEs. Surely, other Tx beams selection can be applied as well with certain rules and probably with even better performance. We didn’t try all the possibilities, and the intention here is just to show that this typical selection is workable.

- : DL beam sweeping procedure where for traditional beam sweeping 32 Tx beams and 4 Rx beams per panel and for AI/ML beam prediction 8 Tx beams and 4 Rx beams per panel

- : Only a subset of DL beams measured among all DL beams

Dense urban scenario for BM-Case2

For temporal domain beam prediction, the basic assumption can be found in Figure 3, where UE measures DL beam pairs in K time instances and predict beam pairs for next F continuous instances. For example, K = 4 and F = 4. Different from BM-Case1, the time domain beam prediction can be applied when UE is with low/medium mobility, e.g. 30km/h as agreed as baseline. To increase the prediction accuracy for moving UE, spatial consistency should be modeled as well.

- : Temporal domain beam prediction where K = 4 measurements instance and F = 4 prediction instance

AI/ML model training and inference

Data set generation

In our evaluation, the data set are generated by SLS as two parts. The 1st part is a subset of measured beam pairs (highlighted yellow in Figure 9) and the corresponding L1-RSRP. This can be used as input of the NN for training purpose. The 2nd part are the best (with highest L1-RSRP) beam pair index(es) among all the beam pairs and their L1-RSRP, which can be seen as labels to the NN model.

With the labelled data, NN can see which beam pair(s) is/are the best one(s) and then learn the relation or pattern between the limited measurements (as input) and the best one(s) (as output). The limited measurement refers to Set B for both BM-Case1 and BM-Case2. In other words, the Tx and Rx beam pattern in time or space domain can be somehow educated to the NN. Then even with a small portion of actual measurement in time and/or space domain, NN is able to infer which beam pair(s) are most likely to be optimal with the highest L1-RSRP.

Neural network models

For BM-Case1, we adopt two separate DNN (Deep Neural Network) models which are depicted in Figure 4. The inputs to both AI/ML models are the subset of DL measurements, i.e. L1-RSRP values of measured beam pairs in an order. Since the beam pair indexes of Set B is sequentially coupled with the model as inputs, the beam pair indexes can be implicitly captured by the NN models.

Specifically, the AI/ML model 1 is to predict the Top-K beam pair index(es) and the DNN structure with 3 hidden layers is depicted in Figure 5. The AI/ML model 2 is to predict the best L1-RSRP(s). These two AI/ML models are trained separately and the prediction of both models may not be perfectly aligned. Note that the best L1-RSRP predicted by AI/ML model 2 may not always correspond to the Top-K beam pair(s) predicted by AI/ML model 1. There could be a case that the predicted beam pair index is not correct, but the predicted L1-RSRP is quite close to the actual L1-RSRP of the genie-aided beam pair.

- : NN models to predict Top-K beam pair index(es) and corresponding K L1-RSRP(s)

- : Beam pair prediction (model 1) implementation by using DNN model

For BM-Case2, we adopt the concatenation of LSTM (Long-Short Term Memory) + DNN models to predict Top-K beam pair(s) in temporal domain. As one may know, the LSTM model is good at handling the inference in time domain, i.e. remembering key information and forgetting non-essential information. Since both LSTM + DNN are well known in ML field and to save space, we omit the details of LSTM + DNN implementations in this contribution. For the combination of BM-Case1 and BM-Case2, i.e. spatial and temporal beam prediction, we also apply the LSTM + DNN model.

Beam pattern of Set B

In RAN1#110, the following agreement on Set B was achieved. Option 2 was further refined with different patterns in RAN1#110bis-e. In RAN1#111, it was agreed for companies to report their implementations of Set B.

Agreement @ RAN1#110

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

    - FFS on the beams of Set B

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each report/measurement during training and/or inference)

    - FFS on fixed or variable number of beams (pairs)

    - FFS on the details

  - Other options are not precluded.

  - FFS on the number of beams (pairs) in Set B

  - Note: This does not preclude the alternative that Set B is different from Set A.

Agreement @ RAN1#110bis-e

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS:

    - Opt A: Set B is changed following a set of pre-configured patterns

    - Opt B: Set B is randomly changed among pre-configured patterns

    - Opt C: Set B is randomly changed among Set A beams (pairs)

    - The number of beams(pairs) in Set B can be fixed or variable

    - Note: BM-Case1 and BM-Case2 may be considered for different option.

  - Other options are not precluded.

Agreement @ RAN1#111

- Companies report the pattern of Set B.

- Further study the performance with different patterns of set B(s) for fixed Set B (Option 1) and different pre-configured/pre-known patterns of Set B(s) (Option 2A and 2B).

In this contribution, to simplify training and inference phase, we adopt Option 1 where Set B is fixed across training and inference phases. Compared with Option 2, fixed Set B may provide more stable performance on spatial and/or temporal beam prediction. Dynamically changing the input of model would require larger scale of training to adapt to variants of Set B. Moreover, there seems evaluation results demonstrating that variable Set (Option 2) can be outperformed by fixed Set B (Option 1). The rationale behind this observation may lie in the fact that inputting variable Set B to model implies insufficient knowledge for inference. For more detailed information on Set B implementation, one may see Figure 2 and corresponding explanation.

Hence, we would like to share our observation on the selection of Set B.

- fixed Set B across training and inference phases has the potential to provide more stable and accurate performance for beam prediction when compared with variable Set B.

KPIs of AI/ML models

In legacy (prior to Rel.18) beam management, L1-RSRP has been widely-used as beam selection metric. With its features of simplicity and computation-friendly, it would be proper to serve as the metric for AI/ML-based beam prediction as well. Beam prediction accuracy based on L1-RSRP was established as a metric to be compared with baseline scheme. In system level, CDF of throughput can be evaluated as a final check on performance. Moreover, overhead/latency reduction (main purpose of beam prediction) can be straightforwardly calculated.

Beam prediction accuracy

However, there are different options on how to measure the beam prediction accuracy. In RAN1#110bis-e, the following agreement was made to list three different metrics.

Agreement

- The options to evaluate beam prediction accuracy (%):

  - Top-1 (%): the percentage of “the Top-1 genie-aided beam is Top-1 predicted beam”

  - Top-K/1 (%): the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

  - Top-1/K (%) (Optional): the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams”

  - Where K >1 and values can be reported by companies.

In our evaluation, Top-K beam pair(s) can be predicted which may or may not include the genie-aided best beam(s). If the genie-aided best beam is within the predicted Top-K beam(s), then a simple beam sweeping (among Top-K predicted beams) procedure can be triggered to find the best actual beam. In addition, for Top-K/1 (%) if K = 1, then Top-K/1 (%) can be degraded as to Top-1 (%) where the Top one predicted beam is the genie-aided one. As for the 3rd metric, i.e. Top-1/K (%), only the Top-1 predicted beam is evaluated, leaving the other (K-1) predicted beam(s) unevaluated. We think the main target of collecting beam prediction accuracy is to evaluate whether the predicted Top-K beam(s) as a whole are accurate or not.

Hence, we tend to apply Top-K/1 (%) in above agreement to evaluate the beam prediction accuracy.

- For beam prediction accuracy, adopt the metric of Top-K/1 (%).

In RAN1#110bis-e, the following two agreements were achieved to define genie-aided Tx beam and Tx-Rx beam pair respectively. Since both genie-aided Tx beam and genie-aided Tx-Rx beam pair matters in the calculation of beam prediction accuracy (%), hopefully these Option A or Option B can be settled down for each type of beam prediction.

Agreement

- For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam considers the following options

  - Option A, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams

  - Option B, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s)

    - FFS on specific Rx beam(s)

    - Note: specific Rx beams are subset of all Rx beams

Agreement

- For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair considers the following options:

  - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams

  - Option B: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s)

    - FFS on specific Rx beam(s)

    - Note: specific Rx beams are subset of all Rx beams

For DL Tx beam prediction, specific Rx beam(s) are assumed corresponding to the predicted Top-K Tx beam(s). If the Top-1 genie-aided Tx beam is selected among all Tx and Rx beam(s), then the genie-aided Rx beam would be highly likely not the specific Rx beam(s). Then the corresponding genie-aided Tx beam will be different from the predicted Tx beam with a higher chance. If that’s the case, then the beam prediction accuracy (%) would not make too much sense, due to different Rx beam(s) used for genie-aided Tx beam and predicted Tx beam.

- For DL Tx beam prediction, adopt Option B in which Top-1 genie-aided Tx beam is selected over all Tx beams with specific Rx beam(s).

For DL Tx-Rx beam pair prediction, the predicted Top-K beam pair(s) are selected among all Tx-Rx beam pairs, i.e. Set A. Hence, it is reasonable to adopt the genie-aided Tx-Rx beam pair from the same scope of Tx-Rx beam pairs, otherwise (if down-scoping the genie-aided Rx beams to specific Rx beam(s)) the beam predication accuracy would be negatively impacted as well.

- For DL Tx-Rx beam prediction, adopt Option A in which Top-1 genie-aided Tx-Rx beam pair is selected over all Tx and Rx beams.

Regarding L1-RSRP difference, it was defined in above agreement as the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam. But some AI/ML model implementation can predict the corresponding L1-RSRP of the Top-K predicted beam pair(s). If the inference is carried out by NW, then it’s necessary for NW to know the predicted L1-RSRP. Hence, we think there should be another L1-RSRP difference as KPI.

- Support another definition of L1-RSRP difference

- The difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam

Overhead and latency reduction

In RAN1#111, the progress of down selection on RS overhead reduction [%] for BM-Case1 was achieved as in the following agreement.

Agreement

- For the evaluation of the overhead for BM-Case1, adoption the following metrics:

  - RS overhead reduction,

    - Option 1: RS OH reduction%=1-NM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML

      - where M is the total number of beams (pairs) to be predicted

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Companies report the assumption on additional measurements

For RS overhead reduction, the percentage of reduction should be evaluated with proper calculation. The basic rule to calculate the overhead reduction at least for BM-Case1 in above agreement is as 1-N/M, where N is the number of beams (with reference signal (SSB and/or CSI-RS)) required for measurement, i.e. the size of Set B; and M is the total number of beams to be predicted, i.e. the size of Set A.

Apparently, the formula 1-N/M can somehow capture a rough evaluation on how much overhead can be saved by beam prediction in spatial domain. Considering the 1/4 or 1/8 overhead reduction ratio, that seems very likely that the predicted Top-K beam pair(s) is (are) not within Set B (the measurement set), but within Set A (the prediction set). This cannot be pre-determined before beam prediction by AI/ML model. Assume K = 4 (maximal reported beam numbers per beam reporting instance), the size of Set B and Set A (32 and 128), additional beam sweeping for predicted Top-K beams may only contribute 4/128 = 3.125% more effort.

In our understanding, the additional effort seems marginal when compared with the benefits introduced by spatial domain beam prediction.

- For RS overhead reduction [%] of BM-Case1, adopt 1-N/M (Option 1) to briefly reflect the overhead reduction.

As for BM-Case2, the temporal domain factor should be added based on that of spatial domain beam prediction. According the following agreement made in RAN1#109e, the overhead reduction should consider the T1 duration (measurement on Set B) and T2 duration (prediction among Set A). In RAN1#111, the following agreement was made to reflect the overhead reduction in time domain.

Agreement @ RAN1#109e

- For temporal beam prediction, further study the following options as baseline performance

  - Option 1a: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources or all possible beams from Set A of beams at the time instants within T2

  - Option 2: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources from Set B of beams at the time instants within T1

    - Companies explain the detail on how to select the best beam for T2 from Set A based on the measurements in T1

  - Where T2 is the time duration for the best beam selection, and T1 is a time duration to obtain the measurements of all the RS resource from Set B of beams.

    - T1 and T2 are aligned with those for AI/ML based methods

  - Whether Set A and Set B are the same or different depend on the sub-use case

  - Other options are not precluded.

Agreement @ RAN1#111

- For the evaluation of the overhead for BM-Case2, adoption the following metrics:

  - RS overhead reduction,

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme

      - Companies report the assumption on additional measurements

    - FFS: Option 3: RS OH reduction%=1-NLM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance

      - where M is the total number of beams (pairs) to be predicted for each time instance

      - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction

    - Companies report the assumption on T1 and T2 patterns

    - Other options are not precluded and can be reported by companies.

On Option 3 under FFS, we somehow did not fully understand its meaning by going through the definition of the ratio L. It should be further clarified at least on the interpretation of the so-called “long” and “short” periods to correctly calculate the overhead reduction [%].

- For BM-Case2, clarify Option 3 on the definition of L to correctly calculate the overhead reduction [%], if needed.

If we could put the potential 2nd stage beam sweeping procedure aside, then the basic rule to calculate overhead reduction can be 1 - (T1*N)/((T1+T2)*M), where T1 and T1+T2 represent the duration of measurement and duration of measurement plus prediction respectively. If the size of Set B equals to that of Set A, i.e. N = M, then this rule can be further simplified as 1 – T1/(T1+T2). Note that for simplicity, we assume for each instance of measurement and prediction, the size of Set B and Set A remain the same.

- For BM-Case2, support the formula 1 - (T1*N)/((T1+T2)*M) to reflect the overhead reduction [%].

Baseline schemes

For the selection of baseline schemes, in RAN1#109e, the following agreements were reached for both spatial domain and temporal domain beam prediction. In our understanding, one of the key benefits of beam prediction is to significantly reduce overhead/latency. Hence, we think it’s fine to choose the baseline with best performance in the sense of beam selection.

Agreement

- For spatial-domain beam prediction, further study the following options as baseline performance

  - Option 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping)

    - FFS CSI-RS/SSB as the RS resources

  - Option 2: Select the best beam within Set A of beams based on the measurement of RS resources from Set B of beams

    - FFS: Set B is a subset of Set A and/or Set A consists of narrow beams and Set B consists of wide beams

    - FFS: how conventional scheme to obtain performance KPIs

    - FFS: how to determine the subset of RS resources is reported by companies

  - Other options are not precluded.

Agreement

- For temporal beam prediction, further study the following options as baseline performance

  - Option 1a: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources or all possible beams from Set A of beams at the time instants within T2

  - Option 2: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources from Set B of beams at the time instants within T1

    - Companies explain the detail on how to select the best beam for T2 from Set A based on the measurements in T1

  - Where T2 is the time duration for the best beam selection, and T1 is a time duration to obtain the measurements of all the RS resource from Set B of beams.

    - T1 and T2 are aligned with those for AI/ML based methods

  - Whether Set A and Set B are the same or different depend on the sub-use case

  - Other options are not precluded.

- For spatial domain beam prediction, select the best beam within Set A via exhaustive beam sweeping (Option 1) as baseline.

- For temporal domain beam prediction, select the best beam for T2 within Set A via exhaustive beam sweeping (Option 1a) as baseline.

Evaluation results

Company results

In RAN1#110bis-e, the table for collecting evaluation results from companies was forged as working assumption. In this contribution, firstly we would like to input our evaluation conditions and results to the table as listed below. It can also be viewed as a summary of our evaluation effort. For more evaluation details, one may find in the following sections.

- Evaluation results without model generalization for Tx-Rx beam pair prediction

|  |  |  | BM-Case1: beam pair | BM-Case1: Tx beam | BM-Case2: beam pair | BM-Case2: Tx beam |
| --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of [beams/beam pairs] in Set A | 128 Tx-Rx beam pairs | 32 Tx beams | 128 Tx-Rx beam pairs | 32 Tx beams |
|  |  | Number of [beams/beam pairs] in Set B | 32 Tx-Rx beam pairs | 8 Tx beams | 128 Tx-Rx beam pairs | 32 Tx beams |
|  |  | Baseline scheme | Best beam pair(s) within Set A via exhaustive beam pair search (Option 1) | Best Tx beam(s) within Set A via exhaustive beam search (Option 1) assuming best Rx beam | Best beam pair (s) within Set A via exhaustive beam pair search for T2 period (Option 1a) | Best Tx beam(s) within Set A via exhaustive Tx beam search for T2 period (Option 1a) assuming best Rx beam |
| AI/ML model input/output |  | Model input | L1-RSRPs with proper tensor order | L1-RSRPs with proper tensor order | L1-RSRPs with proper tensor order of T1 period | L1-RSRPs with proper tensor order of T1 period |
|  |  | Model output | Top-K Tx-Rx beam pair(s) and corresponding K L1-RSRP(s) | Top-K Tx beam(s) and corresponding K L1-RSRP(s) | Top-K Tx-Rx beam pair(s) and corresponding K L1-RSRP(s) of T2 period | Top-K Tx beam(s) and corresponding K L1-RSRP(s) of T2 period |
| Data Size |  | Training | 40k | 40k | 100k | 100k |
|  |  | Testing | 2k | 2k | 5k | 5k |
| AI/ML model |  | [Short model description] | DNN | DNN | LSTM + DNN | LSTM + DNN |
|  |  | Model complexity | 0.07M trainable parameters | 0.05M trainable parameters | 1.74M trainable parameters | 1.62M trainable parameters |
|  |  | Computational complexity | 0.22M FLOPs | 0.10 FLOPs | 5.05M FLOPs | 4.80M FLOPs |
| Evaluation results [With AI/ML / baseline] | [Beam prediction accuracy (%)] | [KPI A] | KPI: Top-K/1 79.15% Top-1/1 92.45% Top-2/1 95.90% Top-3/1 97.95% Top-4/1 | KPI: Top-K/1 92.6% Top-1/1 98.6% Top-2/1 99.6% Top-3/1 99.8% Top-4/1 | KPI: Top-K/1 4 measurements in T1 period and 4 prediction in T2 period 81.0% Top-1/1 95.0% Top-2/1 98.0% Top-3/1 98.9% Top-4/1 | KPI: Top-K/1 4 measurements in T1 period and 4 prediction in T2 period 77.1% Top-1/1 93.7% Top-2/1 97.3% Top-3/1 98.8% Top-4/1 |
|  |  | [KPI B] … |  |  |  |  |
|  | [L1-RSRP Diff] | [Average L1-RSRP diff] … | KPI: Top-K/1 0.41dB Top-1/1 0.15dB Top-2/1 0.07dB Top-3/1 0.04dB Top-4/1 | KPI: Top-K/1 0.14dB Top-1/1 0.05dB Top-2/1 0.02dB Top-3/1 0.01dB Top-4/1 | KPI: Top-K/1 0.49dB Top-1/1 0.13dB Top-2/1 0.03dB Top-3/1 0.02dB Top-4/1 | KPI: Top-K/1 0.25dB Top-1/1 0.04dB Top-2/1 0.02dB Top-3/1 0.01dB Top-4/1 |
|  | [System performance] | [RS overhead Reduction (%)/ RS overhead] | KPI: 1-N/M 75% = 1-32/128 | KPI: 1-N/M 75% = 1-8/32 | KPI: 1-(T1*N) / ((T1+T2)*M) 50% = 1-(4*128)/((4+4)*128) | KPI: 1-(T1*N) / ((T1+T2)*M) 50% = 1-(4*32)/((4+4)*32) |
|  |  | [UCI report] |  |  |  |  |
|  |  | [UPT] … | Spectrum efficiency Baseline: 1.534bits/s/Hz Bandwidth: 80MHz 1.500bits/s/Hz Top-1/1 1.519bits/s/Hz Top-2/1 1.529bits/s/Hz Top-3/1 1.534bits/s/Hz Top-4/1 |  |  |  |

BM-Case1: Spatial domain beam pair prediction

In Table 2, we summarize the evaluation results from Top-1 to Top-4 spatial domain beam pair prediction. It is obvious that the more beam pair predicted, the higher probability that Top-K predicted beam pairs include the genie-aided best beam pair. For this evaluation, from Top-1 to Top-4, the beam pair prediction accuracy increases from 79.15% to 97.95%. In addition, the gap between ideal L1-RSRP of predicted beam pair and that of genie-aided best beam pair is quite narrow, i.e. smaller than 0.5dB. In terms of SE, the SE of UE with Top-1 predicted beam pair is very close to the baseline scheme which is based exhaustive search to find the best beam pair.

Since there are total 128 beam pairs and UEs only measure 32 beam pairs for spatial domain beam prediction, the rough overhead reduction or latency reduction can be up to (1 – 32 beams / 128 beams) = 75%.

- BM-Case1 beam pair prediction performance

|  | Baseline | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- | --- |
| Beam pair prediction accuracy |  | 79.15% | 92.45% | 95.90% | 97.95% |
| Avg. L1-RSRP estimate error |  | 0.41dB | 0.15dB | 0.07dB | 0.04dB |
| Spectrum efficiency (bits/s/Hz) | 1.534 | 1.500 | 1.519 | 1.529 | 1.534 |

- Spatial domain beam pair prediction can yield beam prediction accuracy (at least 80%) while overhead/latency reduction rate is 75%.

- The system level metric, i.e. spectrum efficiency or throughput, is not sensitive to the L1-RSRP difference introduced by spatial domain beam pair prediction.

Next, one may see the CDF of L1-RSRP gap of the Top-1 predicted beam as in Figure 6. To have a clear view on the CDF, we only count the L1-RSRP with wrong beam pair prediction.

- : CDF of the L1-RSRP gap for the Top-1 predicted beam pair

- For 80% of the incorrect spatial domain beam pair prediction cases, the L1-RSRP difference can be kept within 2dB.

Finally, let’s provide the CDF of spectrum efficiency (S.E. in bits/s/Hz) as system level performance for BM-Case1. In Figure 7, the spectrum efficiency of baseline scheme and Top-1 to Top-4 beam pair prediction are shown and one may find the performance are quite close.

We also note that as for scenario of dense urban (macro-layer only) with 200m ISD @FR2, a portion of UEs located close to cell edge (around 35%) are not well served due to high pathloss, and their S.E. are nearly zero.

- : S.E. CDF of baseline (upper bound) and Top-1 to Top-4

Moreover, since the beam pair predication accuracy is promising as shown in Table 2 (at least 80%) and L1-RSRP error is very marginal (lower than 1dB), the performance on S.E. of Top-1 to Top-4 are very closed to that of the upper bound baseline in Figure 7. We find the hint that if both beam pair prediction accuracy and L1-RSRP error are good, the comparison on system-level results, i.e. S.E. or throughput may only provide neglectable insight. So, it is reasonable for us to focus on the KPI directly related to L1-RSRP.

- When beam pair prediction accuracy is high (at least 80%) and L1-RSRP difference is small (within 1 dB), the system-level performance, i.e. spectrum efficiency or throughput, may only provide non-essential insight, therefore focusing on L1-RSRP difference and beam prediction accuracy would be enough.

BM-Case1: Spatial domain Tx beam prediction

Along with beam pair prediction, we also evaluate the performance of Tx beam prediction when the corresponding best Rx beam is selected. In RAN1#111, the following agreement was made to determine Rx beam for Tx beam prediction. In our evaluations, we pick the “best” Rx beam for each Tx beam. Since each Rx panel implements 4 Rx beams, and 2 panels deployed per UE, the exhaustive beam sweeping seems acceptable during the phase of data collection. The data set can be easily generated from that of beam pair training and prediction.

Agreement

At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable

- Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample

- Option 2: Measurements of specific Rx beam(s)

  - Option 2a: Measurements of specific Rx beam(s) per model input sample

  - Option 2b: Measurements of specific Rx beam(s) for all model input sample

  - FFS how to select the specific Rx beam(s)

- Option 3: Measurements of random Rx beam(s) per model input sample

- Other options are not precluded and can be reported by companies.

- For DL Tx beam prediction, adopt the “best” Rx beam (Option 1) with exhaustive beam sweeping for each model input sample.

In Table 3, one may see that the Tx beam prediction accuracy is as high as 92.6% for Top-1 predicted Tx beam. In addition, its L1-RSRP difference gap is quite small, i.e. 0.136dB for the same case. When we compare the Tx beam prediction with beam pair prediction, it is clear that Tx beam prediction achieves even higher beam prediction accuracy. It may come from the fact that beam pair prediction is classic classification problem with 128 Tx-Rx beam pair in total, whereas Tx beam prediction only has 32 Tx beams to select. Apparently, the latter is easier classification problem than the former.

- BM-Case1 Tx beam prediction performance

|  | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- |
| Tx beam prediction accuracy | 92.6% | 98.6% | 99.6% | 99.8% |
| Avg. L1-RSRP estimate error | 0.136dB | 0.054dB | 0.021dB | 0.011dB |

- The accuracy of Tx beam prediction is as high as 92.6% and the L1-RSRP difference of it is as small as 0.13dB. Tx beam prediction can achieve even better prediction performance than that of beam pair prediction.

BM-Case2: Temporal domain beam pair prediction

As for BM-Case2, the evaluation assumption can be found in appendix Table 18. The UE measures K = 4 continuous time instances from Set B which is the same as Set A, and predicts Top-K (from Top-1 to Top-4) in the upcoming F = 1/2/4 time instances. Apparently, from Table 4, one may find that given Top-K predicted beam pair(s), the beam pair prediction accuracy decreases from F = 1 to F = 4. The reason lies in the fact that the higher the time gap between measurement and prediction, the higher probability that the prediction is not accurate. Similar observation can be found in Table 5 for L1-RSRP prediction error. But anyway, the performance seems promising (higher than 90.0%) from Top-2 to Top-4 prediction.

- Beam pair prediction accuracy for BM-Case2

| Beam pair prediction accuracy | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- |
| F = 1 | 89.1% | 97.8% | 99.0% | 99.4% |
| F = 2 | 86.6% | 97.1% | 98.6% | 99.2% |
| F = 4 | 81.0% | 95.0% | 98.0% | 98.9% |

- L1-RSRP prediction error for BM-Case2

| Avg. RSRP prediction error (dB) | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- |
| F = 1 | 0.168 | 0.055 | 0.028 | 0.021 |
| F = 2 | 0.273 | 0.078 | 0.033 | 0.024 |
| F = 4 | 0.490 | 0.130 | 0.037 | 0.026 |

- Temporal domain beam pair prediction can provide prediction accuracy (e.g. 81%) while overhead/latency reduction is as large as 50% (for the case of K = 4 and F = 4).

- Beam pair predication accuracy slightly decreases from 89.1% to 81% (the case of Top-1) when F increases from 1 to 4, but strongly increases from 81% to 98.9% (the case of F = 4) when the number of predicted beam pair increases from Top-1 to Top-4.

In Figure 8, the CDF of L1-RSRP difference is presented. Even for the worst case (Top-1, F = 4), 80% of L1-RSRP prediction error is lower than 3.5dB. Note here we only calculate the L1-RSRP for the case when beam pair prediction in time domain is incorrect.

- : CDF of L1-RSRP gap of Top-1 predicted beam in 1/2/4 time instance(s)

- For 80% of the incorrect temporal domain beam pair prediction cases, the L1-RSRP difference is lower than 3.5dB.

BM-Case2: Temporal domain Tx beam prediction

In Table 6 and Table 7, the Tx beam prediction and its L1-RSRP difference are provided respectively. Once again, we compare its performance in the sense of prediction accuracy with that of beam pair prediction in temporal domain. Obviously, Tx beam prediction can achieve slightly better performance in prediction accuracy and L1-RSRP difference. For instance, for the case of F = 4 and Top-1, the prediction accuracy is 88.3% in Table 6 and 81.0% in Table 4 for the same reason as we mentioned in Section 2.4.3.

- Tx beam prediction accuracy for BM-Case2

| Tx beam prediction accuracy | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- |
| F=1 | 93.9% | 99.4% | 99.8% | 99.9% |
| F=2 | 92.0% | 99.1% | 99.8% | 99.9% |
| F=4 | 88.3% | 98.1% | 99.6% | 99.8% |

- L1-RSRP difference for BM-Case2 Tx beam prediction

| L1-RSRP difference | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- |
| F = 1 | 0.075 | 0.025 | 0.013 | 0.006 |
| F = 2 | 0.126 | 0.038 | 0.016 | 0.011 |
| F = 4 | 0.253 | 0.049 | 0.022 | 0.010 |

- For BM-Case2, the case of Tx beam prediction can slightly outperform that of beam pair prediction in terms of prediction accuracy and L1-RSRP difference.

Combination of spatial and temporal domain beam prediction

In this section, we evaluation the use case of spatial and temporal domain beam prediction by taking the model of BM-Case2 as basis. Accordingly, we shrank the size of Set B (previously equal to that of Set A in BM-Case2, i.e. 128 beam pairs) to that of BM-Case1 (i.e. 32 beam pairs) to obtain the beam prediction functions in both spatial and temporal domain.

For K = 4 measurement instances, the beam prediction accuracy and L1-RSRP prediction error can be found in Table 8 and Table 9, respectively. The overhead reduction for the case of K = 4 and F = 4 can be calculated as 1 – (32 beam pairs * 4 measurement instances) / (128 beam pairs * (4 measurement instances + 4 prediction instances)) = 87.5%.

- Beam prediction accuracy for combined BM-Case1 and BM-Case2 (when K = 4 measurement instances)

| Beam prediction accuracy | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- |
| F = 1 | 85.7% | 97.0% | 98.4% | 99.5% |
| F = 2 | 82.8% | 95.7% | 98.1% | 99.3% |
| F = 4 | 74.4% | 93.1% | 97.0% | 98.8% |

- L1-RSRP prediction error for combined BM-Case1 and BM-Case2 (when K = 4 measurement instances)

| Avg. RSRP prediction error in dB | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- |
| F = 1 | 0.200 | 0.057 | 0.030 | 0.021 |
| F = 2 | 0.316 | 0.077 | 0.042 | 0.023 |
| F = 4 | 0.731 | 0.15 | 0.062 | 0.030 |

- Spatial and temporal domain beam prediction can provide beam prediction accuracy (at least 74.4%) while overhead/latency reduction can be up to 87.5% (for the case of K = 4, F = 4 and Set B = 32 beam pairs, Set A = 128 beam pairs).

Moreover, when compared with that of BM-Case2 with same setting but different Set B, the beam prediction accuracy and avg. L1-RSRP error of combined spatial and temporal domain beam prediction only slightly decrease.

For K = 8 measurement instances, the beam prediction accuracy and L1-RSRP prediction error can be found in Table 10 and Table 11, respectively. Similarly, the overhead reduction for the case of K = 8 and F = 8 can be calculated as 1 – (32 beam pairs * 8 measurement instances) / (128 beam pairs * (8 measurement instances + 8 prediction instances)) = 87.5% which is the same as the case of K = 4 and F = 4.

- Beam prediction accuracy for combined BM-Case1 and BM-Case2 (when K = 8 measurement instances)

| Beam prediction accuracy | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- |
| F = 1 | 84.4% | 96.7% | 98.8% | 99.7% |
| F = 2 | 80.6% | 95.8% | 98.3% | 99.3% |
| F = 4 | 74.4% | 92.3% | 96.0% | 98.4% |
| F = 8 | 64.5% | 84.4% | 92.9% | 96.2% |

- L1-RSRP prediction error for combined BM-Case1 and BM-Case2 (when K = 8 measurement instances)

| Avg. RSRP prediction error in dB | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- |
| F = 1 | 0.192 | 0.036 | 0.011 | 0.006 |
| F = 2 | 0.353 | 0.067 | 0.024 | 0.014 |
| F = 4 | 0.675 | 0.174 | 0.081 | 0.034 |
| F = 8 | 1.697 | 0.512 | 0.145 | 0.078 |

- Spatial and temporal domain beam prediction can provide beam prediction accuracy (at least 64.5%) while overhead/latency reduction can be up to 87.5% (for the case of K = 8, F = 8 and Set B = 32 beam pairs, Set A = 128 beam pairs).

- For spatial and temporal domain beam prediction, the longer beam prediction period (e.g. F = 8 prediction instances), the deeper performance loss can be observed given the same measurement period (e.g. K = 8 measurement instances).

Generalization performance

In RAN1#111, the following agreement on the evaluation of generalization was updated. In addition, various Set B of beam (pairs) was considered as one factor which may impact the generalization performance.

Agreement

- For generalization performance verification, consider the following

  - Scenarios

    - Various deployment scenarios,

      - e.g., UMa, UMi and others,

      - e.g., 200m ISD or 500m ISD and others

      - e.g., same deployment, different cells with different configuration/assumption

      - e.g., gNB height and UE height

      - FFS: e.g., Carrier frequencies

    - Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others

    - Various UE mobility,

      - e.g., 3km/h, 30km/h, 60km/h and others

  - Configurations (parameters and settings)

    - Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions)

    - Various gNB settings, e.g., DL Tx beam codebook (including various Set A of beam(pairs) and gNB antenna array dimensions)

    - Various Set B of beam (pairs)

    - T1 for measurement /T2 for prediction for BM-Case2

  - Other scenarios/configurations(parameters and settings) are not precluded and can be reported by companies.

Agreement

For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, additionally considering

- Various Set B of beam(pairs)

Spatial domain beam pair prediction (BM-Case1)

In this section, we evaluate the property of model generalization for BM-Case1. Specifically, under UMa scenario, we assume the beam prediction model is trained with Set B (32 beam pairs (8T4R)) and Set A (128 beam pairs (32T4R)). Next, the inference was carried for 3 different testing sets. We evaluate the generalization performance of 4 different cases. Note that each case of these 4 cases represent a different combination of scenario and configuration.

For Case 1, we use it as a baseline scheme where the training set and testing set are the same type, i.e. both are UMa scenario with 128 beam pairs (32T4R) as Set A and 32 beam pairs (8T4R) as Set B;  For Case 2, we only change the scenario of testing set from UMa to UMi while keeping other factors the same; For Case 3, Set A of testing set is modified with half number of Rx beams (i.e. from 4 to 2); For Case 4, Set A of testing set is changed with a quarter number of that of training set (i.e. 32T4R to 16T2R). We have to note that only for the last 2 cases, where Set A of testing set is with 32T2R or 16T2R, the corresponding Set B (subset of Set A in our evaluations) should be cut accordingly. In Case 3 and Case 4, Set B of testing set is composed of 16 beam pairs (i.e. 8T2R).

In Table 12 and Table 13, from Case 1 through Case 4, the beam prediction accuracy and avg. L1-RSRP difference are shown, respectively. One may find that by comparing Case 1 and Case 2, changing environment (training from UMa and inferencing for UMi) would not deteriorate the performance of beam prediction. One of the explanations would be that the model trained under UMa scenario is more adaptable to UMi scenario. For Case 3 (a half number of Rx beams) and Case 4 (a half number of Tx beams and a half number of Rx beams), we observe slight performance loss due to the generalization issue. The more dramatic change from training set to testing set, the more performance loss can be observed. But when the number of Top-K predicted beams increases to 4, the beam prediction accuracy can be higher than 95% associated with neglectable RSRP difference.

In addition, we would like to mention that to make the AI/ML model workable between training phase and inference phase, simple pre-processing and post-processing techniques could be applied. For example, at the input end of the model, if the size of Set B is changed from 8T4R (training phase) to 8T2R (inference phase), then zero-padding or 1-on-1 replica can be added on 8T2R to fill the input of 8T4R. That’s the pre-processing part. At the output end of the model, if the size of Set A is changed from 32T4R (training phase) to 16T2R (inference phase), post-processing, such as grouping, should be applied to determine rough beam pair prediction (16T2R) from the inference of fine beam pair(s) (32T4R).

- Beam prediction accuracy where the Set B/Set A of training set are UMa_8T4R/UMa_32T4R

| Beam prediction accuracy | Testing Set (Set B) | Testing Set (Set A) | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | UMa_8T4R | UMa_32T4R | 79.2% | 92.5% | 95.9% | 98.0% |
| Case 2 | UMa_8T4R | UMi_32T4R | 80.7% | 94.5% | 97.4% | 98.6% |
| Case 3 | UMa_8T2R | UMa_32T2R | 76.9% | 91.8% | 96.2% | 97.9% |
| Case 4 | UMa_8T2R | UMa_16T2R | 64.8% | 85.1% | 91.9% | 95.1% |

- Avg. L1-RSRP difference in dB where the Set B/Set A of training set are UMa_8T4R/UMa_32T4R

| Avg. L1-RSRP difference in dB | Testing Set (Set B) | Testing Set (Set A) | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | UMa_8T4R | UMa_32T4R | 0.411 | 0.155 | 0.071 | 0.041 |
| Case 2 | UMa_8T4R | UMi_32T4R | 0.236 | 0.055 | 0.022 | 0.016 |
| Case 3 | UMa_8T2R | UMa_32T2R | 0.608 | 0.201 | 0.079 | 0.043 |
| Case 4 | UMa_8T2R | UMa_16T2R | 2.014 | 0.706 | 0.308 | 0.184 |

- Thanks to generalization capability of well-trained AI/ML model, changing scenario from UMa to UMi may not necessarily deteriorate the beam prediction performance.

- Changing beam pair configuration on Set B and Set A from training phase to inference phase would slightly lower the beam prediction performance.

- When more predicted beam pairs are provided by AI/ML model, e.g. Top-K = 4, the beam selection accuracy can be up to 95% and avg. L1-RSRP difference can be lower than 1dB.

- Study the techniques of pre-processing at model input and post-processing at model output to enable the generalization capability of AI/ML model.

Temporal domain beam prediction (BM-Case2)

We evaluate the inference performance with training at UE speed 30km/h and inference at different UE speed, ranging from 30km/h through 120km/h. Other evaluation aspects are the same as we provide in our company results.

In Table 14 and Table 15, the model is trained with UE speed at 30km/h. To see the generalization performance, the inference/testing set ranges from 30km/h (as a baseline performance), 60km/h, 90km/h and 120km/h. A clear observation is that when UE speeds up higher than 60km/h, the beam prediction accuracy drops as we expected. Similar observation also holds for the avg. L1-RSRP difference.

- Beam prediction accuracy where UE speed increases from 30km/h to 120km/h

| Training set with UE speed 30km/h, testing set with 30km/h |  |  |  |  |
| --- | --- | --- | --- | --- |
| Beam prediction accuracy | Top-1 | Top-2 | Top-3 | Top-4 |
| F=1 | 89.1% | 97.8% | 99.0% | 99.4% |
| F=2 | 86.6% | 97.1% | 98.6% | 99.2% |
| F=4 | 81.0% | 95.0% | 98.0% | 98.9% |
| Training set with UE speed 30km/h, testing set with 60km/h |  |  |  |  |
| F=1 | 68.2% | 78.2% | 80.6% | 81.6% |
| F=2 | 63.4% | 75.7% | 79.0% | 80.6% |
| F=4 | 57.0% | 71.3% | 76.4% | 79.0% |
| Training set with UE speed 30km/h, testing set with 90km/h |  |  |  |  |
| F=1 | 58.9% | 69.7% | 72.3% | 73.6% |
| F=2 | 54.7% | 66.4% | 69.8% | 71.6% |
| F=4 | 46.8% | 60.7% | 65.7% | 68.7% |
| Training set with UE speed 30km/h, testing set with 120km/h |  |  |  |  |
| F=1 | 50.5% | 61.3% | 64.5% | 66.2% |
| F=2 | 46.6% | 57.2% | 61.0% | 63.5% |
| F=4 | 37.9% | 50.2% | 55.7% | 59.2% |

- Avg. L1-RSRP difference in dB where UE speed increases from 30km/h to 120km/h

| Training set with UE speed 30km/h, testing set with 30km/h |  |  |  |  |
| --- | --- | --- | --- | --- |
| Avg. L1-RSRP difference | Top-1 | Top-2 | Top-3 | Top-4 |
| F=1 | 0.168 | 0.055 | 0.028 | 0.021 |
| F=2 | 0.273 | 0.078 | 0.033 | 0.024 |
| F=4 | 0.490 | 0.130 | 0.037 | 0.026 |
| Training set with UE speed 30km/h, testing set with 60km/h |  |  |  |  |
| F=1 | 2.721 | 2.066 | 1.812 | 1.708 |
| F=2 | 3.142 | 2.453 | 2.193 | 2.048 |
| F=4 | 3.519 | 2.601 | 2.252 | 2.080 |
| Training set with UE speed 30km/h, testing set with 90km/h |  |  |  |  |
| F=1 | 3.862 | 2.872 | 2.569 | 2.441 |
| F=2 | 4.323 | 3.449 | 3.101 | 2.900 |
| F=4 | 5.105 | 3.828 | 3.283 | 2.955 |
| Training set with UE speed 30km/h, testing set with 120km/h |  |  |  |  |
| F=1 | 5.276 | 3.949 | 3.576 | 3.395 |
| F=2 | 6.063 | 4.739 | 4.204 | 3.969 |
| F=4 | 6.942 | 5.361 | 4.631 | 4.204 |

- For AI/ML model trained with UE speed 30km/h and tested with UE speed higher than 60km/h, the generalization performance on beam prediction accuracy and avg. L1-RSRP difference drops accordingly.

To evaluate the generalization performance (Case 3), we have generated a mix the data set with ¼ data from 30km/h data, ¼ data from 60km/h data and so on. Then we trained the AI/ML model and evaluate its performance of beam pair prediction accuracy and L1-RSRP difference as in Table 16 and Table 17 respectively. From the aspects of prediction accuracy, the generalized performance on prediction accuracy can be kept between 60% and 90% which is obviously better than the case trained with single UE speed.

- Beam pair prediction accuracy where UE speed increases from 30km/h to 120km/h

| Training set with mix speed, testing set with mix speed |  |  |  |  |
| --- | --- | --- | --- | --- |
| Beam pair prediction accuracy | Top-1 | Top-2 | Top-3 | Top-4 |
| F=1 | 87.1% | 97.5% | 99.1% | 99.5% |
| F=2 | 82.5% | 96.1% | 98.4% | 99.1% |
| F=4 | 74.8% | 91.2% | 96.0% | 97.9% |
| Training set with mix speed, testing set with 30km/h |  |  |  |  |
| F=1 | 85.7% | 96.8% | 98.4% | 99.0% |
| F=2 | 81.9% | 95.2% | 97.7% | 98.6% |
| F=4 | 74.1% | 90.8% | 95.6% | 97.5% |
| Training set with mix speed, testing set with 60km/h |  |  |  |  |
| F=1 | 81.3% | 94.4% | 97.1% | 98.1% |
| F=2 | 76.3% | 91.0% | 95.1% | 96.8% |
| F=4 | 67.9% | 85.0% | 91.2% | 94.4% |
| Training set with mix speed, testing set with 90km/h |  |  |  |  |
| F=1 | 80.2% | 94.7% | 97.5% | 98.6% |
| F=2 | 75.6% | 91.6% | 95.8% | 97.5% |
| F=4 | 65.2% | 83.7% | 91.2% | 94.7% |
| Training set with mix speed, testing set with 120km/h |  |  |  |  |
| F=1 | 78.6% | 92.9% | 96.5% | 97.8% |
| F=2 | 73.6% | 89.7% | 94.5% | 96.4% |
| F=4 | 62.2% | 80.9% | 88.8% | 92.9% |

- Avg. L1-RSRP difference in dB where UE speed increases from 30km/h to 120km/h

| - Training set with mix speed, testing set with mix speed |  |  |  |  |
| --- | --- | --- | --- | --- |
| L1-RSRP difference (dB) | Top-1 | Top-2 | Top-3 | Top-4 |
| F=1 | 0.202 | 0.052 | 0.021 | 0.015 |
| F=2 | 0.455 | 0.098 | 0.042 | 0.020 |
| F=4 | 1.107 | 0.317 | 0.133 | 0.060 |
| Training set with mix speed, testing set with 30km/h |  |  |  |  |
| F=1 | 0.299 | 0.121 | 0.094 | 0.084 |
| F=2 | 0.429 | 0.164 | 0.118 | 0.098 |
| F=4 | 0.906 | 0.275 | 0.148 | 0.106 |
| Training set with mix speed, testing set with 60km/h |  |  |  |  |
| F=1 | 0.327 | 0.106 | 0.061 | 0.047 |
| F=2 | 0.562 | 0.188 | 0.093 | 0.059 |
| F=4 | 1.216 | 0.418 | 0.215 | 0.127 |
| Training set with mix speed, testing set with 90km/h |  |  |  |  |
| F=1 | 0.333 | 0.089 | 0.049 | 0.029 |
| F=2 | 0.680 | 0.206 | 0.087 | 0.052 |
| F=4 | 1.574 | 0.570 | 0.238 | 0.132 |
| Training set with mix speed, testing set with 120km/h |  |  |  |  |
| F=1 | 0.468 | 0.140 | 0.074 | 0.050 |
| F=1 | 0.817 | 0.270 | 0.135 | 0.085 |
| F=4 | 1.953 | 0.724 | 0.339 | 0.204 |

- For AI/ML model trained with mixed UE speed (e.g. from 30km/h to 120km/h) and tested with different UE speed, the generalization performance on beam prediction accuracy and avg. L1-RSRP difference outperform the one trained with single UE speed.

Conclusion

In this section, allow us to repeat our observations and proposals

- fixed Set B across training and inference phases has the potential to provide more stable and accurate performance for beam prediction when compared with variable Set B.

- Spatial domain beam pair prediction can yield beam prediction accuracy (at least 80%) while overhead/latency reduction rate is 75%.

- The system level metric, i.e. spectrum efficiency or throughput, is not sensitive to the L1-RSRP difference introduced by spatial domain beam pair prediction.

- For 80% of the incorrect spatial domain beam pair prediction cases, the L1-RSRP difference can be kept within 2dB.

- When beam pair prediction accuracy is high (at least 80%) and L1-RSRP difference is small (within 1 dB), the system-level performance, i.e. spectrum efficiency or throughput, may only provide non-essential insight, therefore focusing on L1-RSRP difference and beam prediction accuracy would be enough.

- The accuracy of Tx beam prediction is as high as 92.6% and the L1-RSRP difference of it is as small as 0.13dB. Tx beam prediction can achieve even better prediction performance than that of beam pair prediction.

- Temporal domain beam pair prediction can provide prediction accuracy (e.g. 81%) while overhead/latency reduction is as large as 50% (for the case of K = 4 and F = 4).

- Beam pair predication accuracy slightly decreases from 89.1% to 81% (the case of Top-1) when F increases from 1 to 4, but strongly increases from 81% to 98.9% (the case of F = 4) when the number of predicted beam pair increases from Top-1 to Top-4.

- For 80% of the incorrect temporal domain beam pair prediction cases, the L1-RSRP difference is lower than 3.5dB.

- For BM-Case2, the case of Tx beam prediction can slightly outperform that of beam pair prediction in terms of prediction accuracy and L1-RSRP difference.

- Spatial and temporal domain beam prediction can provide beam prediction accuracy (at least 74.4%) while overhead/latency reduction can be up to 87.5% (for the case of K = 4, F = 4 and Set B = 32 beam pairs, Set A = 128 beam pairs).

- Spatial and temporal domain beam prediction can provide beam prediction accuracy (at least 64.5%) while overhead/latency reduction can be up to 87.5% (for the case of K = 8, F = 8 and Set B = 32 beam pairs, Set A = 128 beam pairs).

- For spatial and temporal domain beam prediction, the longer beam prediction period (e.g. F = 8 prediction instances), the deeper performance loss can be observed given the same measurement period (e.g. K = 8 measurement instances).

- Thanks to generalization capability of well-trained AI/ML model, changing scenario from UMa to UMi may not necessarily deteriorate the beam prediction performance.

- Changing beam pair configuration on Set B and Set A from training phase to inference phase would slightly lower the beam prediction performance.

- When more predicted beam pairs are provided by AI/ML model, e.g. Top-K = 4, the beam selection accuracy can be up to 95% and avg. L1-RSRP difference can be lower than 1dB.

- For AI/ML model trained with UE speed 30km/h and tested with UE speed higher than 60km/h, the generalization performance on beam prediction accuracy and avg. L1-RSRP difference drops accordingly.

- For AI/ML model trained with mixed UE speed (e.g. from 30km/h to 120km/h) and tested with different UE speed, the generalization performance on beam prediction accuracy and avg. L1-RSRP difference outperform the one trained with single UE speed.

- For beam prediction accuracy, adopt the metric of Top-K/1 (%).

- For DL Tx beam prediction, adopt Option B in which Top-1 genie-aided Tx beam is selected over all Tx beams with specific Rx beam(s).

- For DL Tx-Rx beam prediction, adopt Option A in which Top-1 genie-aided Tx-Rx beam pair is selected over all Tx and Rx beams.

- Support another definition of L1-RSRP difference

- The difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam

- For RS overhead reduction [%] of BM-Case1, adopt 1-N/M (Option 1) to briefly reflect the overhead reduction.

- For BM-Case2, clarify Option 3 on the definition of L to correctly calculate the overhead reduction [%], if needed.

- For BM-Case2, support the formula 1 - (T1*N)/((T1+T2)*M) to reflect the overhead reduction [%].

- For spatial domain beam prediction, select the best beam within Set A via exhaustive beam sweeping (Option 1) as baseline.

- For temporal domain beam prediction, select the best beam for T2 within Set A via exhaustive beam sweeping (Option 1a) as baseline.

- For DL Tx beam prediction, adopt the “best” Rx beam (Option 1) with exhaustive beam sweeping for each model input sample.

- Study the techniques of pre-processing at model input and post-processing at model output to enable the generalization capability of AI/ML model.

References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#94e, Dec. 2021

- TS 38.215 v17.1.0 Physical layer measurements

- TS 38.214 v17.1.0 Physical layer procedures for data

- Appendix I: evaluation assumptions

- Evaluation assumptions for dense urban (both BM-Case1 and BM-Case2)

| Parameter | Value |
| --- | --- |
| Scenario | Dense urban (macro layer only), 21 cells |
| Frequency setting | 30GHz carrier & 120kHz SCS & 80MHz BW |
| BS antenna config | [Mg Ng M N P] = [1 1 4 8 2] baseline |
| UE antenna config | [Mg Ng M N P] = [1 2 1 4 2] baseline |
| BS beamforming | 32 Tx beams (8 horizontal & 4 vertical) |
| UE beamforming | 4 Rx beams (4 horizontal & 1 vertical) |
| UE rotation | OFF |
| Beam selection metric | L1-RSRP |
| Traffic model | Full buffer |
| Performance metric |  |
| L1-RSRP | Top-K beam pairs with maximum L1-RSRPs |
| Spectrum efficiency | Final output of SLS |

- Evaluation assumptions for BM-Case1 (dense urban)

| Model parameter | Values |
| --- | --- |
| Model selection | DNN with 3 hidden layers |
| Input | 32 beam pairs (8 Tx beams and 4 Rx beams) |
| Output | Predicted Top-K beam pairs (K = 1, 2, 3, 4) |
| Training set and mode | 40000, offline |
| Spatial domain specific | Values |
| UE speed | 3km/h |
| UE mobility | OFF |
| Spatial consistency | OFF |

- Evaluation assumptions for BM-Case2 (dense urban)

| Model parameter | Values |
| --- | --- |
| Model selection | LSTM + full connection |
| Input | 128 beam pairs (32 Tx beams and 4 Rx beams) K = 4 measurement instance |
| Output | Predicted Top-K beam pairs (K = 1, 2, 3, 4) in F time instances (F = 1, 2, 4). Each time instance is with granularity 100ms. |
| Training set and mode | 20000 (in case of K=4 and F=1), offline |
| Time domain specific | Values |
| UE speed | 30km/h |
| UE trajectory | Option 2: linear trajectory with random direction change, and bounce back when hitting cell boundary |
| Spatial consistency | Procedure A in TR 38.901 |

- Appendix II: evaluation for indoor hotspot scenario

- Assumption for BM-Case1

In our initial evaluation (before RAN1#109e), we focused on the sub use case of beam prediction in spatial domain for overhead and latency reduction.

The illustration on traditional beam selection and AI/ML beam prediction in spatial domain can be found in Figure 9. For traditional beam sweeping, there are 64 Tx beams, 4 Rx beams, and then the total number of beam pairs, i.e. Set A, to measure can be up to 256. For AI/ML beam prediction, only a subset of all beam pairs (e.g. 8 Tx beams highlighted as yellow) are actually measured by UE with all Rx beams as Set B. Then the measurement number can be calculated as 32 beam pairs (8 Tx beam * 4 Rx beam). Correspondingly, the overhead and latency can be significantly compressed as 1/8 (32 beam pairs / 256 beam pairs).

- : DL beam sweeping procedure where for traditional beam sweeping 64 Tx beams at NW and 4 Rx beams per panel and for AI/ML beam prediction 8 Tx beams and 4 Rx beams per panel

- : Only a subset of DL beams measured among all DL beams

- Indoor hotspot scenario

| Parameter | Value |
| --- | --- |
| Scenario | TR38.901: Indoor hotspot, 12 sites, 3 cells per site |
| SCS | 120kHz |
| Bandwidth | 40MHz |
| Carrier Frequency | 30GHz |
| Tx power | 20dBm |
| BS antenna config | [Mg Ng M N P] = [1 1 4 16 2] |
| UE antenna config | [Mg Ng M N P] = [1 1 1 4 2] |
| BS Tx beam pattern | 64 Tx = 16 horizontal * 4 vertical Azimuth angle = [-16*pi/32, -14*pi/32, …, 0, …, 12*pi/32, 14*pi/32] Zenith angle = [2*pi/8, 3*pi/8, 4*pi/8, 5*pi/8] (azimuth, zenith) = (0, pi/2) is the direction perpendicular to the array |
| BS mech. tilting | 20 degree |
| BS height | 3m |
| UE Rx beam pattern | 4 Rx per panel = 4 horizontal * 1 vertical UE panels deployment [-90, 0, 90], assumed the best panel is selected Azimuth angle = [-3*pi/8, 1*pi/8, pi/8, 3*pi/8] Zenith angle = [pi/2] |
| UE velocity | 3km/h |
| UE height | 1.5m |
| Beam selection method | L1-RSRP |
| Training set | 39600 samples = 12 sites * 3 cells per site * 100 UE per cell * 11 drops |

- Evaluations for BM-Case1

Given the previously introduced data set and NN model setting, we would like to share our initial evaluation results in Table 20. In this table, we compare the AI/ML based prediction with traditional beam selection, where the latter applies no interpolation, but just select the best measured beam pair with highest L1-RSRP among the actual measurements.

In the 2nd column of Table 20, we compare the best beam selection/prediction rate for both AI/ML beam prediction and traditional beam selection. One may see that the correct best beam pair prediction rate of AI/ML could be around 80% and the traditional beam selection has a smaller chance, i.e. round 24% to find the best beam pair.

In the 3rd and 4th column of Table 20, two performance metrics on L1-RSRP error are calculated. L1-RSRP prediction error is the difference between actual (real) L1-RSRP of the best beam pair and the predicted L1-RSRP. L1-RSRP selection error is the difference between actual (real) L1-RSRP of the best beam pair and the actual (real) RSRP of the selected beam pair.

When the best beam pair is predicted right by the NN model, its L1-RSRP prediction error is 1.02dB. That is very close to the quantization step for L1-RSRP beam reporting (e.g. 1dB step size) in TS 38.214 [3]. Note that when the best beam pair is correctly selected, then the L1-RSRP selection error (between the actual L1-RSRP of the best beam pair and the actual R1-RSRP of selected best beam) is 0dB, since in this case, they are of the same beam pair.

The false beam pair selection rate of AI/ML based prediction is round 20%, and the L1-RSRP prediction error is around 2.5dB. Intuitively, if the NN cannot predict the right beam pair, then it would be harder to predict the best L1-RSRP within very small error. Moreover, in the cases of false beam prediction, the L1-RSRP selection error is more than 5dB, which reflects the difference between the actual (real) L1-RSRP of best beam pair and actual (real) L1-RSRP of the predicted beam pair.

As for traditional beam selection, since there is no interpolation/prediction, the L1-RSRP prediction error is not applicable. Hence, we only average the L1-RSRP selection error of traditional beam selection which could be up to 11dB. The reason of such huge gap lies in the fact that UE can only pick up the best beam within 1/8 beam pair of all candidate beam pairs.

- Performance based on L1-RSRP

|  | Correct beam prediction rate | Mean of L1-RSRP prediction error | Mean of L1-RSRP selection error |
| --- | --- | --- | --- |
| AI/ML-Right selection | 80.35% | 1.02dB | 0dB |
| AI/ML-False selection | 19.65% | 2.47dB | 5.49dB |
| Traditional-Right selection | 23.94% | / | 0dB |
| Traditional-False selection | 76.06% | / | 11.16dB |

With above analysis on performance, we would like to share our observations.

- For InH scenario, AI/ML beam prediction in spatial domain can yield relatively high correct prediction rate (around 80%) while using only a small portion (1/8) of DL measurement.

- For InH scenario, when AI/ML beam prediction is correct, the L1-RSRP prediction error is acceptably small (around 1dB); otherwise (AI/ML beam prediction incorrect), the L1-RSRP prediction error would increase slightly (up to 2.5dB).

In Figure 11, we show the CDF of both predicted L1-RSRP and actual (real) L1-RSRP when the AI/ML beam prediction is correct. Statistically speaking, thanks to the technique of AI/ML, these CDFs are closed to each other. It aligns well with average L1-RSRP prediction error (1.02dB as shown in Table 20).

In Figure 12, we show the CDFs when false beam pair selection/prediction happens. When AI/ML model 1 (introduced in Figure 4) predicts an incorrect best beam pair, the CDF of its predicted L1-RSRP (generated by AI/ML model 2 separately) stays not far away from that of actual (real) L1-RSRP of the best beam pair. This curve is marked as “AI-predicted L1-RSRP”.

As for the curve marked as “AI-selected L1-RSRP”, it refers to the actual (real) L1-RSRP of the wrongly-predicted best beam pair. This gap between actual L1-RSRP of AI/ML prediction and actual L1-RSRP of the best beam pair is around 5dB. This performance also matters, since it reflects actual L1-RSRP performance gap of the best beam predicted wrongly by NN.

As we explained previously, the traditional sub-set beam selection scheme has a large gap when compared with the best beam. The reason lies in the fact that the traditional beam selection only has partial beam pair candidates. Note that in this contribution, we only select the very basic traditional beam selection scheme. To make more meaningful comparison, we may need to study more advanced traditional scheme, e.g. beam selection with linear interpolation.

- : CDF of actual (real) L1-RSRP and predicted L1-RSRP when the beam prediction is correct

- : CDF of actual (real)/selected/predicted L1-RSRP when the beam prediction is incorrect