3GPP TSG RAN WG1 #112		R1-2300299

Athens, Greece, February 27th – March 3rd, 2023

Source:	OPPO

Title:	Discussion on dynamic resource sharing in co-channel coexistence of LTE and NR SL

Agenda Item:	9.4.2

Document for:	Discussion and Decision

Introduction

The WID on NR SL evolution was revised in RAN#97-e with the following objective[1]:

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4]

- Reuse the in-device coexistence framework defined in Rel-16 as much as possible

- Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A

According to the above RAN guidance in yellow highlight on co-channel coexistence, we continue to discuss the details of dynamic resource pool sharing in this paper.

Discussion

The main motivation to study the dynamic resource pool sharing is full utilization of all available radio resources allocated to either LTE or NR-based SL communications while transmission collisions can be largely avoided. In the last few meetings, it was already agreed that the feasibility of dynamic resource sharing as possible solutions for co-channel coexistence is to be studied. Therefore, we propose to support the dynamic resource pool sharing in this release considering its benefits and performance gains.

Proposal 1: Dynamic resource pool sharing between LTE SL and NR SL should be supported.

Based on the updated objective of co-channel coexistence, type A devices with dual modules ought to be considered with higher priority in dynamic resource sharing mechanism. For type A devices, LTE module will share sensing and resource reservation information to NR module. In RAN1#110bis-e, the following agreements for the shared information from LTE module to NR module was achieved. During the last meeting (RAN1#111), further down-selection for possible parameters in the shared information has been discussed while there was no consensus.

Agreement For dynamic resource pool sharing, the candidate information shared by the LTE SL module to the NR SL module may include one or more of the following parameters, to be down-selected: - Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs - SL RSRP measurement results - Resource reservation periods based on decoded SCI and for own LTE SL transmissions - Priority based on decoded SCI and for own LTE SL transmissions - Time and frequency location of resources used for own LTE SL transmissions - Candidate resource set SA or SB - SL RSSI measurements - LTE logical subframe related information - Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE

In our view, there is no doubt that the time and frequency location of reserved resources, priority and resource reservation period based on decoded SCI of LTE SL as well as the associated RSRP measurement result belong to the information shared by LTE module. Correspondingly, NR module utilizes the shared information for resource exclusion in PHY layer during the Mode 2 resource selection procedure.

In mode 4 RA of LTE SL, the resource reservation period is converted to a number of logical subframes based on  and then applied within the set of logical subframes (i.e.,  ).  denotes the subframes that may belong to LTE SL resource pool in TS 36.213. Hence, LTE logical subframe related information also needs to be shared to NR module such that NR module is able to determine the periodic resources reserved by LTE SCI. More specifically, the LTE logical subframe related information refers to the set of  or the parameters used to determine , such as the subframes corresponding to LTE SLSS and the reserved subframes of LTE SL. During the last meeting, some companies observed that LTE logical subframe related information is needed in NR SL module. However, they think it is a kind of semi-static information so it should not be shared in a dynamic manner. After careful deliberation, we understand the concern from other companies and also agree to avoid too much redundant information exchanged between LTE and NR SL module. For making further progress, a possible way forward is to add a note that NR SL module is aware of the logical subframe related information, which has been proposed at the end of last meeting.

The in-device coexistence between LTE SL and NR SL in different frequency channels was discussed and specified in Rel-16. And it was mainly handled by dropping the transmission of LTE or NR SL based on priority comparison, which may cause a performance loss to the RAT with lower priority. In this co-channel coexistence work, the preferred approach is to avoid in-device collision instead of dropping transmissions. For instance, LTE module forwards the selected SL grant(s) to NR module such that NR module can exclude candidate resources overlapping with the SL grant(s) in time domain. Therefore, we suggest that the time and frequency locations of resources used for own LTE SL transmissions together with the corresponding priority and resource reservation period should be shared by LTE module to NR module as well.

As for the resources corresponding to half-duplex subframes which are not monitored by LTE SL module, we think it can be a candidate information shared to NR module and NR module may use it during resource exclusion just like current step 5 in mode 2 RA.

Proposal 2: For dynamic resource pool sharing, the sensing and resource reservation information shared by LTE SL module to NR SL module includes at least the following parameters:

- Time and frequency location of reserved resources by other LTE SL UEs, determined based on decoded SCIs

- SL RSRP measurement results

- Resource reservation periods based on decoded SCIs and for own LTE SL transmissions

- Priority based on decoded SCIs and for own LTE SL transmissions

- Time and frequency location of resources used for own LTE SL transmissions

- Resources corresponding to half-duplex subframes which are not monitored by LTE SL module

It is RAN1’s understanding that NR SL module is aware of the LTE SL logical subframes (i.e., t0SL,t1SL,...,tTmaxSL in TS 36.213).

Another open issue is how the NR module utilizes the sensing and resource reservation information from LTE module. During the last meeting, the following agreement was approved:

Agreement For dynamic resource pool sharing, the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, where - The NR SL module excludes resources based on the shared information from its own candidate resource set when performing the resource (re)selection procedure in the PHY layer. - FFS how to exclude resources at least based on the time and frequency locations of LTE SL transmissions that have been indicated in the shared candidate information. - FFS how the exclusion is performed according to clause 8.1.4 of TS 38.214. - FFS: whether/how NR SL module excludes resources not belonging to the generated LTE SL’s candidate resource set SB from its own candidate resource set.

As can be observed in this agreement, the shared information is used during the procedure of resource exclusion in physical layer of NR SL module. In our understanding, the behavior of resource exclusion is related to the elements/parameters of the shared information. In other words, different parameters will lead to different UE behaviors for resource exclusion. Generally, we can divide all possible parameters in the shared information into three branches:

- Parameters associated with own LTE SL transmissions.

In this case, the parameters include the time and frequency location of resources, the resource reservation period and the priority used for own LTE SL transmissions. First, NR SL module may use them to identify a set of periodic resources. For example, the time and frequency location of resources are repeated periodically according to the resource reservation period. More specifically, the resource reservation period should be converted to a number of logical subframes and the set of periodic resources is determined within LTE SL logical subframes as well. When the candidate resource overlaps with the identified periodic resources in time domain, NR SL module excludes the candidate resource from the candidate resource set. Optionally, the candidate resource is excluded only when the priority of LTE SL transmission is higher than the priority of NR SL transmission.

- Parameters related to half-duplex subframes.

When LTE SL module shares the resources corresponding to half-duplex subframes, NR SL module can exclude the candidate resource directly when the candidate resource or the periodic resources corresponding to the candidate resource overlaps with the shared resources. This step is similar to current step 5 of mode 2 RA. Therefore, it is also important for NR SL module to address the issue of excessive exclusion/infinite loop due to the non-monitored subframes. For example, similar method like current step 5a) in section 8.1.4 of TS 38.214 can be considered.

- Parameters based on the decoded SCIs of LTE SL.

The parameters here refer to the time and frequency location of resources, the resource reservation period and the priority of decoded LTE SCI and also include the RSRP measurement result. In the same way as the first branch, the time and frequency location of resources and the resource reservation period of decoded LTE SCI are used to identify a set of periodic resources. It should be noted that the number of periodicities (i.e., the value Q) is calculated by following the formula in TS 36.213. On the basis of current step 6 in mode 2 RA, NR SL module excludes the candidate resource when the candidate resource or the periodic resources corresponding to the candidate resource overlaps with the identified periodic resources and the SL RSRP delivered by LTE SL module is higher than a (pre-)configured RSRP threshold. As for the RSRP threshold, we think a separate table of RSRP threshold can be (pre-)configured and the RSRP threshold is determined based on the priority of decoded LTE SCI and the priority of NR SL transmissions.

Proposal 3: NR SL module excludes the candidate resource when the candidate resource overlaps with a set of periodic resources in time domain.

- The set of periodic resources is identified according to the parameters associated with own LTE SL transmissions.

- Optionally, the candidate resource is excluded only when the priority of LTE SL transmission is higher than the priority of NR SL transmission.

Proposal 4: NR SL module excludes the candidate resource when the candidate resource or the periodic resources corresponding to the candidate resource overlaps with the shared resources.

- The shared resources correspond to half-duplex subframes.

- Similar method like current step 5a) in section 8.1.4 of TS 38.214 can be considered for the issue of infinite loop.

Proposal 5: NR SL module excludes the candidate resource when it meets all the following conditions:

- The candidate resource or the periodic resources corresponding to the candidate resource overlaps with a set of periodic resources.

  - The set of periodic resources is identified according to the parameters based on decoded SCIs of LTE SL.

- The SL RSRP delivered by LTE SL module is higher than a (pre-)configured RSRP threshold

  - A separate table of SL RSRP threshold can be (pre-)configured.

  - The RSRP threshold is determined based on the priority of decoded LTE SCI and the priority of NR SL transmissions.

In the meanwhile, some companies proposed that LTE SL module generates the candidate resource set (i.e., set B) according to current specification of LTE SL with some inputs from NR SL module and then forwards the set B to NR module. NR SL module excludes the candidate resource from its own candidate resource set when the candidate resource doesn’t belong to the set B. From our perspective, we are doubtful about the feasibility of this method.

- First, the resource reservation period in NR SL can be any value from 1ms to 99ms which is not supported in LTE SL. Also, LTE module always assumes periodic transmissions, hence, the supported reservation periodicity starts from 20ms. However, aperiodic transmissions are supported in NR V2X.

- In addition, as we mentioned in the above, the resource reservation period ought to be converted to the number of logical subframes/slots and then applied within the set of logical subframes/slots in both LTE and NR SL according to current specification. Therefore, even for the periodicity like 100ms which is supported by LTE SL, it is still impossible for LTE module to determine the periodic resources used for the own transmissions of NR module due to different formulas are used during the conversion from physical time to logical subframes/slots and the different definitions of logical subframes/slots in LTE and NR SL. For example, 100ms is equal to A logical subframes based on the formula in TS 36.213 but it is equal to B logical slots according to the formula in TS 38.214. As a result, LTE module and NR module will have different understanding about the time location of periodic transmission resources for NR SL module. Obviously, the generated set A or set B by LTE SL module is not suitable for NR SL module in this case.

- Furthermore, the supported sizes of LsubCH used in NR SL are also different to LTE SL. And the minimum and maximum number of sub-channels which may be used for transmissions on PSSCH ranges from 1...27 in NR SL. In LTE SL, the supported number of sub-channels ranges only from 1…20.

Observation 1: The set B generated by LTE SL module is not suitable for NR SL module.

From the perspective of NR module, it may use the shared information from LTE module during initial (re-)selection, re-evaluation and pre-emption checking. In current specification, the resource exclusion procedure in physical layer is common to initial selection, re-evaluation and pre-emption checking. Hence, when the shared information from LTE module is applied for resource exclusion in the physical layer of NR module, it is natural that it should be reused for re-evaluation and pre-emption as well. For example, the shared information is utilized to perform re-evaluation and/or pre-emption checking by NR module and trigger resource re-selection for corresponding resources.

Proposal 6: NR module ought to use the shared sensing and resource reservation information from LTE module for initial selection, re-evaluation and pre-emption checking.

As can be observed in the yellow highlights of the agreement below, two alternatives were proposed and extensively discussed during the last few meetings in order to address the AGC issue caused by PSFCH transmission of NR SL.

Agreement For co-channel coexistence in Rel-18, dynamic resource pool sharing is studied, with the following constraints: - NR SL resource pool is configured with 15 kHz SCS. - FFS support of NR SL resource pool configured with higher SCS, including other solutions to overcome the AGC issue caused by the differing SCSs between the NR SL and LTE SL resource pools - For NR PSFCH (if configured), at least the following alternatives are studied: - Alt 1: Avoid PSFCH transmission in time slots that overlap with subframes used for LTE SL transmissions. - FFS: Avoiding PSFCH transmissions can be performed by the UE transmitting PSFCH and/or the UE transmitting PSSCH. - Alt 2: NR SL UEs use a periodically repeating set of PSFCH slots. - FFS: periodicities of the set.

In alternative 1, NR SL UE tries to avoid performing PSFCH transmission in the slots which may be occupied by LTE SL UE. More specifically, NR SL UE may drop PSFCH transmission or exclude PSSCH resource during resource exclusion when the corresponding PSFCH overlaps with the resources reserved by LTE SL UE in time domain. In our understanding, alternative 1 has a large impact on the HARQ procedure in NR SL, which will lead to the degradation of reliability. For HARQ feedback option 1 (i.e., NACK-Only), if RX UE drops PSFCH transmission due to the collision with LTE SL, TX UE will think the data transmission has been decoded successfully and then doesn’t perform retransmissions anymore. For HARQ feedback option 2 (i.e., ACK-or-NACK), the PSFCH dropping will cause more retransmissions and congestion level increases in this way. Furthermore, even if no LTE transmission / reservation is detected by the LTE module in a subframe that overlaps with NR PSFCH slot, it does not guarantee that the transmission of PSFCH by the NR module will not cause interference / collision to LTE transmissions due to both hidden node issue and initial transmissions of LTE SL which may not be reserved/indicated in advanced.

Observation 2: For the AGC issue due to PSFCH transmissions of NR SL, alternative 1 will lead to a significant impact on HARQ feedbacks procedure, especially for groupcast option 1 (i.e., NACK-only based feedback).

Observation 3: For the AGC issue due to PSFCH transmissions of NR SL, alternative 1 doesn’t guarantee that the transmission of PSFCH will not cause interference to LTE SL transmission.

The original intention of alternative 2 is that NR SL UE uses a set of periodic PSFCH resources and also performs PSSCH/PSCCH transmissions in the slots with PSFCH such that LTE SL UEs will avoid using time overlapping resources based on RSSI detection. In LTE mode 4, RSSI is detected within the subframes that may belong to LTE SL resource pool (i.e.,  in TS 36.213). However, PSFCH period is configured in NR SL resource pool. That is, PSFCH resources are located within the slots that belong to NR SL resource pool (i.e., t'0SL, t'1SL, t'2SL,… in TS 38.214). Therefore, although RSSI detection period could be the multiples of PSFCH period, they work periodically in different sets of logical subframes/slots. In our view, the periodic relationship between PSFCH resources and RSSI detection must be guaranteed in alternative 2, otherwise it cannot work. One potential solution is to align the subframes that may belong to LTE SL resource pool and the slots of NR SL resource pool. Another way is to define the periodic PSFCH resources of NR SL within the set of logical subframes for LTE SL.

Observation 4: RSSI is detected within the subframes that may belong to LTE SL resource pool but PSFCH resources are configured within the slots of NR SL resource pool.

In the last meeting (RAN1#111), some companies proposed to support both alternative 1 and alternative 2 by (pre-)configuration. We do not think it is a feasible solution because it will cause more efforts on standardization work. In our view, there are lots of details and issues need to be addressed in either of these two alternatives. If we support both of them, it is really hard for the WG to finish this agenda or even the whole WI in time. Therefore, we still suggest to down-select between alternative 1 and alternative 2. If RAN1 cannot find a possible way forward, there could be no further enhancement for this issue.

Proposal 7: Disagree with (pre-)configuration between alternative 1 and alternative 2, try to down-select alternative 1 or alternative 2.

Simultaneously, the AGC issue caused by higher SCSs was discussed in the last few meetings as well. However, there was no appropriate method to address this issue. According to the guidance from RAN plenary, RAN1 continues the work on dynamic resource pool sharing based on existing agreements. Since higher SCSs have not been agreed yet, we think we should continue to study dynamic resource sharing with the SCS of 15kHz only.

Proposal 8: RAN1 continues to study dynamic resource pool sharing with the SCS of 15kHz only.

From the discussion and analysis in the above, the different definitions of logical subframes/slots in LTE and NR SL are mentioned for several times and it is related to the shared information by LTE SL module, resource exclusion of NR SL module and the solution to address the AGC issue. It should be noted that all of UE behaviors for LTE SL mode 4 operate in the set of logical subframes rather than physical time and it is the same for NR SL mode 2. If we do not attach importance to the issue of logical subframes/slots during the design phase, the selected solution for dynamic resource pool sharing may not be feasible in the end from the perspective of current specification even for the SCS of 15kHz.

Observation 5: The definition for the set of logical subframes in LTE SL mode 4 is different from the definition for the set of logical slots in NR SL mode 2.

Conclusion

In this contribution, we discussed the mechanism of dynamic resource pool sharing in co-channel coexistence, we have following observations and proposals:

Observation 1: The set B generated by LTE SL module is not suitable for NR SL module.

Observation 2: For the AGC issue due to PSFCH transmissions of NR SL, alternative 1 will lead to a significant impact on HARQ feedbacks procedure, especially for groupcast option 1 (i.e., NACK-only based feedback).

Observation 3: For the AGC issue due to PSFCH transmissions of NR SL, alternative 1 doesn’t guarantee that the transmission of PSFCH will not cause interference to LTE SL transmission.

Observation 4: RSSI is detected within the subframes that may belong to LTE SL resource pool but PSFCH resources are configured within the slots of NR SL resource pool.

Observation 5: The definition for the set of logical subframes in LTE SL mode 4 is different from the definition for the set of logical slots in NR SL mode 2.

Proposal 1: Dynamic resource pool sharing between LTE SL and NR SL should be supported.

Proposal 2: For dynamic resource pool sharing, the sensing and resource reservation information shared by LTE SL module to NR SL module includes at least the following parameters:

- Time and frequency location of reserved resources by other LTE SL UEs, determined based on decoded SCIs

- SL RSRP measurement results

- Resource reservation periods based on decoded SCIs and for own LTE SL transmissions

- Priority based on decoded SCIs and for own LTE SL transmissions

- Time and frequency location of resources used for own LTE SL transmissions

- Resources corresponding to half-duplex subframes which are not monitored by LTE SL module

It is RAN1’s understanding that NR SL module is aware of the LTE SL logical subframes (i.e., t0SL,t1SL,...,tTmaxSL in TS 36.213).

Proposal 3: NR SL module excludes the candidate resource when the candidate resource overlaps with a set of periodic resources in time domain.

- The set of periodic resources is identified according to the parameters associated with own LTE SL transmissions.

- Optionally, the candidate resource is excluded only when the priority of LTE SL transmission is higher than the priority of NR SL transmission.

Proposal 4: NR SL module excludes the candidate resource when the candidate resource or the periodic resources corresponding to the candidate resource overlaps with the shared resources.

- The shared resources correspond to half-duplex subframes.

- Similar method like current step 5a) in section 8.1.4 of TS 38.214 can be considered for the issue of infinite loop.

Proposal 5: NR SL module excludes the candidate resource when it meets all the following conditions:

- The candidate resource or the periodic resources corresponding to the candidate resource overlaps with a set of periodic resources.

  - The set of periodic resources is identified according to the parameters based on decoded SCIs of LTE SL.

- The SL RSRP delivered by LTE SL module is higher than a (pre-)configured RSRP threshold

  - A separate table of SL RSRP threshold can be (pre-)configured.

  - The RSRP threshold is determined based on the priority of decoded LTE SCI and the priority of NR SL transmissions.

Proposal 6: NR module ought to use the shared sensing and resource reservation information from LTE module for initial selection, re-evaluation and pre-emption checking.

Proposal 7: Disagree with (pre-)configuration between alternative 1 and alternative 2, try to down-select alternative 1 or alternative 2.

Proposal 8: RAN1 continues to study dynamic resource pool sharing with the SCS of 15kHz only.

References

- RP-222674, “WID revision: NR sidelink evolution”, RAN #97e

- 3GPP RAN1 #110bis Chairman’s Notes, October 10th – 19th, 2022

- 3GPP RAN1 #111 Chairman’s Notes, November 14th – 18th, 2022