3GPP TSG-RAN WG1 Meeting #112	Tdoc R1-2300335

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.12.1

Source:	Ericsson

Title:	L1 enhancements to inter-cell beam management

Document for:	Discussion

# Introduction

L1/L2 mobility is the main part of the Rel-18 work item [1]. In the Rel-18 mobility WI, the serving cells of will be updated based on an indication provided on L1 or L2. In the LS [4], RAN2 states that the term LTM (L1/L2-triggered mobility) will be used, and we will adopt this terminology in our contributions.

There has been reasonable progress in the first two RAN1 meetings on LTM. Initial agreements have been made on beam indication and beam reporting (which we will call LTM reporting to avoid confusion).

The next level of details to be agreed for beam indication and LTM reporting are:

The format of the beam indication, and the support for the Rel-15 TCI framework. With an agreement on these issues, the beam indication functionality in LTM would be done.

The applicable measurement RSs. So far, SSB has been agreed. It would be good to finalize which measurement RSs are supported – alternatively conclude there is no consensus to support additional measurement RSs.

The reporting formats. So far, UCI over PUSCH has been agreed. Additional reporting channels/containers should be agreed or discarded. This would make it possible to discuss the next level of details of the reporting format.

In this contribution, we discuss LTM, with focus on the above issues.

# 2	Discussion

The main motivation of LTM is to reduce the interruption, although there may be benefits also related to robustness. To understand how LTM would reduce the interruption, it is important to investigate what causes the interrupt for a handover.

Our understanding of the overall LTM procedure is illustrated in Figure 1.

Figure 1: Illustration of the overall LTM procedure. The intra-RAN signalling is simplified: for example, it is not possible to directly send information between DUs – the information needs to be relayed over the CU. Also, the serving DU will have to be configured by the CU to be able to perform its part.

The steps of the LTM procedure are described below:

Step1: The UE sends a L3 measurement report to the network.

Step 2: The network configures the UE with a set of candidates, and associated L1 measurement configurations

Step 3: The UE measures and reports a set of candidates. At some point in time, the network decides to perform a cell-switch to one of the candidates.

Step 4: The network sends a PDCCH order to the UE to send a PRACH preamble towards a selected target.

Step 5: The UE transmits the PRACH towards the selected target

Step 6: The target estimates the TA and forwards it to the serving.

Step 7: The serving cell sends the cell-switch command to the UE. The cell-switch command includes the TA to use in the new cell.

There are variants of the procedure. We could for example assume that the network sends the PDCCH order to estimate the TA for several candidates. This would mean that the cell-switch command could be performed quicker, once the decision to switch cell has been made. However, this would consume more resources. It may also be so that the UE can only maintain pre-synchronization to one candidate. In Figure 1, we also assume that no RAR is sent in response to the received PRACH.

LTM shall work for intra-DU and inter-DU. The internal RAN architecture was developed to allow the DUs to operate independently. One DU knows the configuration of its own cells, but not the configuration of cells controlled by other DUs. During a legacy handover between two cells, the handover command (including the configuration of the target) is prepared by the target cell, and transmitted to the UE by the source cell, without the source cell being aware of the configuration. This effect of the RAN architecture must be taken into account also during the RAN1 work. This was highlighted in the two reply LSs [8] and [9], by RAN2 and RAN3 respectively. In their response, RAN3 states that

Regarding Q3 about L1 measurement and TCI state configurations in the LS, RAN3 agreed that based on the current specification, the serving DU cannot know the measurement RS configuration and TCI state configuration of cells served by another DU.

Any possible consideration for Rel-18 on the coordination over F1 among serving DU, target DU, and CU would need clearly identified requirements from other groups.

In their response, RAN2 states that

RAN2 assumes that LTM (intra DU and inter DU) is network-controlled mobility where the control is from the source, i.e., measurements (L1 measurements) are configured in the UE from the source Cell, and the decision to switch cell is by the source cell, and enhancements considered for LTM before cell switch, e.g. pre-synchronization, TA handling, target beam management (to the extent it is supported) may be made by the source cell. RAN2 understands that this may require cooperation among source DU, CU, target DU, and/or OAM. RAN2 don’t see any blocking issue to share information between DUs but the support of this is in RAN3 domain. RAN2 see no necessity for a direct inter-DU-interface to support this.

From these responses, it is clear that the decisions RAN1 make may lead to large specification efforts for RAN2 and RAN3, efforts that could have been reduced if RAN1 had only been more careful. In particular, RAN1 should not assume that any configuration that is available at the UE is also available at the serving DU:

RAN1 should not assume that any information that is available at the UE is also available in the serving DU.

## 2.1	Beam indication

In the LS [2], RAN2 talks about the cell switch command, which is the L1/L2 command that triggers the cell switch.

The term “beam indication” is used in beam management and represents the signaling where the UE obtains a new QCL indication for reception of DL signals, most notably the PDCCH/PDSCH. In the context of LTM, once the new QCL indication takes effect, i.e., after the beam application time, the UE would receive the DL from the new TRP.

Conceptually, the LTM mobility trigger information has two parts:

the beam indication – this is RAN1’s responsibility.

the cell switch command – this is RAN2’s responsibility. For the purpose of the discussion in this paper, all content specified by RAN2 will be lumped into the term “cell switch command”.

In this section, we will discuss the properties of the beam indication.

In RAN1#110bis-e and RAN1#111, the following agreements were reached:

Agreement (RAN1#110bis-e)

- From RAN1 perspective, the following scenarios can be considered for Rel-18 L1/L2 mobility for beam indication timing. This will be updated depending on further RAN1 assessment and RAN2 decision on the time chart

  - Scenario 1: Beam indication before cell switch command

  - Scenario 2: Beam indication together with cell switch command

  - Scenario 3: Beam indication after cell switch command

Agreement (RAN1#110bis-e)

- For beam indication timing for Rel-18 LTM,

  - Support Scenario 2: Beam indication together with cell switch command,

    - For Rel-17 unified TCI framework,

      - Beam indication indicates TCI state for each target serving cell

  - FFS: Scenario 1: Beam indication before cell switch command

  - FFS: Scenario 3: Beam indication after cell switch command

- FFS: Activation of TCI state(s) of target serving and/or candidate cell(s).

Scenario 1 would imply that the UE uses the configuration of the source cell for the initial communication with the target cell. This means that the target cell must support the configuration of the source cell, which is true in many cases, and is also the foundation of ICBM. Since the beam indication can be transmitted earlier, the network could delay the cell-switch command when measurement reports validate that the UE is under the coverage of the target cell. Therefore, we propose

Support also scenario 1 with LTM: the beam indication is transmitted before the cell switch command.

Note that we should support scenario 1 without having to rely on the Rel-17 ICBM.

Our interpretation of scenario 1 is that the UE may not automatically execute the cell switch at the reception of the LTM cell-switch command. The question then naturally arises: when will the UE execute the cell switch? As explained, it would be beneficial if the network could choose to perform the cell switch later, when it is likely that the UE would not switch back to the source cell. This would mean that we would also need to support a variant of the LTM cell-switch command that triggers the cell switch, but leaves the beam indication, i.e., the indicated TCI state, unaffected:

Support that an LTM cell-switch command triggers a cell-switch, but not an update of the indicated TCI state

To us, the need for the functionality in Proposal 2 becomes clear if Proposal 1 is agreed.

In RAN1#111, RAN1 made the following agreement:

Agreement

- The beam indication of candidate cell(s) for Rel-18 LTM should be designed based on the following:

  - Beam indication for Rel-18 LTM is designed based on Rel-17 unified TCI framework, if both serving cell and candidate cell support Rel-17 unified TCI framework

  - FFS: whether/how to design mechanism for Beam indication for Rel-18 LTM when at least one from serving cell and candidate cell supports only Rel-15 TCI framework.

  - Note: How and whether to indicate the new serving cell(s) and timing for beam indication are separately discussed

The above agreement states that the beam indication will be designed based on the Rel-17 unified TCI framework. To us this would imply that the UE must be able to derive a Rel-17 TCI state at the reception of the beam indication command:

At the reception of the beam indication command, the UE must be able to derive a Rel-17 TCI state.

In the two reply LSs [8] and [9], RAN2 and RAN3 provided their view on what the serving DU would know about the configurations of another DU. In their response, RAN3 states that

Regarding Q3 about L1 measurement and TCI state configurations in the LS, RAN3 agreed that based on the current specification, the serving DU cannot know the measurement RS configuration and TCI state configuration of cells served by another DU.

Any possible consideration for Rel-18 on the coordination over F1 among serving DU, target DU, and CU would need clearly identified requirements from other groups.

In their response, RAN2 states that

RAN2 assumes that LTM (intra DU and inter DU) is network-controlled mobility where the control is from the source, i.e., measurements (L1 measurements) are configured in the UE from the source Cell, and the decision to switch cell is by the source cell, and enhancements considered for LTM before cell switch, e.g. pre-synchronization, TA handling, target beam management (to the extent it is supported) may be made by the source cell. RAN2 understands that this may require cooperation among source DU, CU, target DU, and/or OAM. RAN2 don’t see any blocking issue to share information between DUs but the support of this is in RAN3 domain. RAN2 see no necessity for a direct inter-DU-interface to support this.

Based on these responses, it is clear that specification enhancements are needed to ensure that knowledge of TCI states (and measurement configurations) is available in the serving DU:

In the current specification, the serving DU does not know the TCI states of cells served by another DU. Enhancements to F1 are needed to support this.

The LS responses indicate that knowledge of parameters of cells served by another DU cannot be automatically assumed: whatever RAN1 decides will have to be supported by an additional specification effort in RAN3. In order not to complicate this RAN3 work, RAN1 should avoid requiring the presence of specific RRC IEs. Instead, RAN1 should be very precise when the requirements are defined.

As previously noted, the beam indication will be used by the UE to derive a TCI state. The UE will subsequently use this TCI state to support its reception of DL signals, and its transmission of UL signals. Clearly, the beam indication could be a TCI state: this is how it is done in beam management. Alternatively, the beam indication could be an RS identity, which the UE would use to derive the TCI state. The two approaches are depicted in Figure 2.

Figure 2: Two approaches to derive the TCI state.

Looking at Figure 2, the two approaches are very similar. The procedure that needs to be performed is the same: the only difference is which node performs the mapping: the serving DU or the UE. Interestingly enough, the UE knows the mapping between the RS and the TCI state – the UE must have this information to be able to operate in the target cell. However, the serving DU does not necessarily have this information – this is in essence what RAN3 replied:

The serving DU does not automatically know the mapping between an RS and the corresponding TCI state – but the UE does.

Of course, it would be possible to convey this information separately, by introducing additional signaling. This would seem unnecessary, since the serving DU does not need to know the TCI state: the subsequent transmissions and receptions take place with the target DU. Based on this, we propose

The beam indication contains an explicit identifier of a reference signal.

The UE maps the RS identifier to a TCI state in the correct candidate configuration.

Another option could be to leave the decision to RAN3: if RAN1 explains the options in Figure 2, RAN3 could make the decision on which option to support.

We also note that with Proposal 3 and Proposal 4, it becomes straightforward to also support LTM with the Rel-15 TCI framework: if the beam indication is provided as an RS identifier, the UE can map the RS to either a Rel-15 or a Rel-17 TCI state, depending on what the target DU supports:

If the beam indication contains an explicit RS identifier, LTM can be supported both with the Rel-15 and Rel-17 TCI frameworks.

As mentioned in Proposal 3, the beam indication would be provided by a reference signal, i.e., an SSB of the target cell. By providing an SSB as a QCL source, the communication in the target cell can be kickstarted. This SSB would also be used as pathloss reference signal, as a DL timing reference, and to determine the spatial Tx filter in FR2.

In [4], RAN2 provides the agreements on LTM made in RAN2#119bis-e. One of the agreements is related to the dynamic cell-switch:

RAN2 assumes L1/2 mobility trigger information is conveyed in a MAC CE, FFS if the MAC CE or a DCI is used for the actual triggering.

In RAN2#120, RAN2 made the following additional agreement:

The MAC CE agreed to carry LTM related information for cell switch is used for LTM triggering of the cell switch.

Hence, we conclude that RAN2 agreed that the MAC CE that carries the LTM related information also triggers the execution of the actions related to the MAC CE. For completeness, we propose to acknowledge these RAN2 agreements:

The LTM mobility trigger information is conveyed in a MAC CE, and the same MAC CE is used for the LTM triggering.

Carrier aggregation (CA) will be supported with LTM. This means that the UE can be configured with multiple serving cells after the cell-switch. The relation between the beam indication and these multiple serving cells is not clear: for instance, the RS identity proposed in Proposal 3, to which serving cell does that refer? To clarify this, we propose

The explicit RS identity refers to the SpCell of the indicated candidate.

Again, we note that the UE has access to this information, whereas it is not readily available at the serving DU.

Additionally, it could be considered to deliver one such RS identity for each serving cell, to kick-start the communication in each serving cell. Here we note that legacy functionality can be used: the RRC parameters simultaneousTCI-UpdateList1 and simultaneousTCI-UpdateList2 can be configured in the target, and when the UE derives the TCI state for the target SpCell, it can also apply new TCI states according to simultaneousTCI-UpdateList1 and simultaneousTCI-UpdateList2.

We now turn our attention to the additional contents of the beam indication, i.e., additional contents of the LTM MAC CE that are the responsibility of RAN1.

As discussed in section 2, the UL synchronization time must be reduced. In other words, the time it takes to provide the UE with a valid TA must be shortened, and this is discussed in more detail in [3]. Many of the methods in [3] imply that the network knows the correct TA before the beam indication is sent, and includes that in the beam indication:

The beam indication contains a TA value for the target cell.

How the network determines the TA value is a separate issue, which is discussed in detail in [3].

Finally, it may be beneficial to also include an explicit BWP ID in the beam indication. The main use case for BWPs is UE power saving, where the network configures the UE with multiple BWPs, and switches the UE between the normal BWP and power-saving BWP. It would be beneficial if the UE could continue to use a BWP with the same properties before and after the LTM event:

The beam indication contains ID of the active DL and UL BWPs for the target SpCell.

For the SCells, it is assumed that the active DL and UL BWPs are configured using RRC, i.e., using the fields firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id. Hence, only for the SpCell, the values for these values can be overridden.

The content of the LTM mobility trigger information command is summarized in Table 1.

| Information | Number of bits |
| --- | --- |
| Cell-switch command | 10? |
| RS identifier(s) | 6 |
| TA | 12 |
| BWP IDs | 2+2 |

Table 1: Contents of the beam indication.

## 2.2	LTM measurements and reporting

In RAN1#111, RAN1 made the following agreement:

Agreement

- For gNB scheduled L1 measurement report for Rel-18 LTM, report as UCI is supported

  - Semi-persistent report on PUSCH, and aperiodic report on PUSCH are supported

    - FFS: periodic and semi-persistent PUCCH

  - In a single report instance, report for serving cell and candidate cell(s) for intra-frequency and/or inter-frequency can be included.

With this agreement, RAN1 decided to follow the UCI reporting paradigm from the Rel-15 beam reporting. We think this is unfortunate, since such UCI reporting has only limited flexibility to deal with varying report content. The report size would also be somewhat limited. Finally, the lack of retransmissions leads to less reliable reports. Nevertheless, once that decision has been made, RAN1 should make the best of the situation.

As always, RAN1 should avoid artificial restrictions. Today, UCI can be reported over PUCCH, and there seems to be little benefit of forbidding that for LTM. Hence, we propose

For gNB scheduled L1 measurement report for Rel-18 LTM, UCI can also be reported over periodic and semi-persistent PUCCH.

The agreement above states that the serving cell can be included in the LTM measurement report. We would like to strengthen this, and proposes:

In every LTM measurement report, one measurement from a serving cell is included.

The reason to always include measurements from the serving cell is that the LTM measurement report is used to determine the compare the serving cell with the candidates, and a measurement report that only contains measurement for a candidate is not useful. We note that L3 measurement reports always include measurement values related to the serving cell.

In addition to the serving cell measurement, the LTM measurement report must include measurement on candidates:

In every LTM measurement report, a measurement value from one primary candidate cell is included.

The UE would report the most suitable cell as the primary candidate cell. As for the intra-cell beam reporting, it is assumed that the most suitable cell (beam) is the cell (beam) with the highest measurement value, although this is not specified.

Based on configuration, measurements from additional secondary candidates can also be included:

In an LTM measurement report, measurements on additional secondary candidates can be included.

The high-level content of the LTM report is depicted in Figure 3.

Figure 3: High-level content of the LTM measurement report.

Next, we discuss the content of the individual measurement. Each measurement must contain a measurement value, and an identifier, which identifies the reference signal on which the measurement was made. So far, RAN1 has agreed to use L1-RSRP based on SSB as a measurement quantity. From a measurement report format point of view, adding L1-SINR would be straightforward. We note that L1-RSRQ could also be relevant for inter-frequency measurements.

The network (serving DU) would use the measurement report to select the target cell, i.e., to select which candidate configuration to indicate in the cell-switch command. In other words: based on LTM measurement report, the network would send one of the candidate configuration indices. Using the candidate configuration indices as an identifier in the report would then seem like a good option:

Use the candidate configuration index as an identifier for the primary and secondary candidate cell measurement.

Additionally, the network (serving DU) will need information to select the beam indication: a candidate cell can send multiple target RSs. The candidate cell measurement must thus also contain an identifier which RS was measured. For the SSB, the SSB index would be that identifier:

An SSB index is included in the primary and secondary candidate cell measurement.

The contents of the primary and secondary cell measurement are depicted in Figure 4.

Figure 4: The content of a candidate cell measurement.

Assuming that the primary candidate cell measurement is the cell with the highest measurement value, the measurement value for the secondary candidate cell measurements can be differentially encoded, just as for the beam reports.

We note that for the serving cell measurement, there is no corresponding candidate configuration index, and there is no need to include an SSB index. The content of the serving cell measurement is thus only a measurement value.

One consequence of the RAN1 agreement to rely on UCI over PUSCH for the LTM reporting is that the protection of the LTM measurement needs extra attention. In contrast to reporting over MAC CE, there is no retransmission mechanism available for UCI: an incorrect report can only be dropped. Even worse: for some UCI payloads, there is not even a CRC attached to the UCI. This is highly undesirable for an LTM measurement report: it must be possible to detect that an LTM measurement report was incorrectly decoded. The simplest way to ensure this is that the payload is always larger than 11 bits:

All LTM reports are zero-padded to ensure that the payload is always 12 bits or larger.

If we define a part 1/part 2 structure for the LTM reports, each part must be zero-padded to ensure that each part is larger than 11 bits.

With the lower flexibility of the UCI reporting, a higher degree of pre-configuration of measurement targets becomes necessary. The ICBM measurements specified in Rel-17 rely on very specific configuration: for SSB measurements, both PCI and SSB indices must be specified. This made the extension from the intra-cell beam reporting straightforward but was not really suitable for the extension into many more measurement targets. Furthermore, the choice to configure these measurements as part of the serving cell configuration was somewhat awkward. These pitfalls should be avoided for LTM reporting.

Here we again refer to the RAN3 LS response: the serving DU does not automatically have access to the candidate configurations: they are intended for the UE, and it is enough that the target DU knows them. Still, the serving DU needs some information to trigger, receive and interpret measurement reports, and that configuration is (most likely) provided to the serving DU as a separate configuration.

It would be beneficial to keep this measurement configuration small, and to keep it separate from the candidate configurations. However, the configuration details should be specified by RAN2/3, whereas RAN1 should focus on the required information. Since RAN2/3 indicated that the candidate configurations are not necessarily known by the serving DU, RAN1 should specify the required information explicitly:

RAN1 defines what information should be included in the measurement configuration, both for the UE and the serving DU, while leaving the RRC configuration details to RAN2/3.

In RAN1#111, the following agreement was made

Agreement

- For candidate cell measurement for Rel-18 LTM,

  - SSB based L1-RSRP is supported for intra-frequency measurement

  - SSB based L1-RSRP is supported for inter-frequency measurement from RAN1 point of view

  - FFS: L1-SINR, CSI-RS based L1-RSRP

Hence, RAN1 has so far only agreed reporting of L1-RSRP for SSB, for intra-frequency and inter-frequency. The configuration of these measurements is manageable: for each target, essentially only the PCI, ARFCN and SCS of the SSB needs to be configured. Hence, we propose

The configuration of a measurement target contains PCI, ARFCN and the SSB SCS.

In our view, introducing a list of SSBs is unnecessary: the UE should try to detect any SSB it finds for the PCI. If additional configuration details are needed, the information in the MeasObjectNR would be a good starting point. For example, the SMTC window or ssb-ToMeasure could be considered to assist the UE. However, those quantities would not be individually configured for each measurement target.

If additional measurement possibilities are introduced, the configuration overhead grows. For example, the CSI-RS configuration is quite flexible, resulting in a much larger overhead. Based on this, our preference is not to support CSI-RS as a measurement target for LTM:

Do not support CSI-RS as a measurement target for LTM.

Event-driven reporting was discussed during RAN1#111. It is often claimed that event-driven measurements have lower latency than network-triggered measurements. This is incorrect: event-driven reporting does not reduce latency, since the latency is determined by the UL transmission time, which is typically larger for event-driven reporting than for network-initiated reporting:

The latency of event-driven reporting is typically larger than the latency of network-initiated reporting.

Instead, the benefits of event-driven reporting lie in lower reporting overhead. The report is only sent in the UL when certain conditions are fulfilled, i.e., when the event is triggered:

Provided that the event is properly designed, the reporting overhead of event-driven reporting can be lower than network-initiated reporting.

Reducing reporting overhead can be important: if only periodic reporting is used, it is likely that the network would need one report from each UE every 40ms, and it is not clear that the network can handle that load.

With event-driven reporting, the UE sends a measurement report to the network once a certain condition is fulfilled. By its nature, an event-driven report is one-shot: it is transmitted only once. Therefore, it is imperative that the report reaches the network, which requires that it is possible for the network to request that the report is retransmitted. This would require that RAN1 defines an LTM report also for MAC CE.

Event-driven reporting would require that RAN1 defines events, which RAN1 has never done (apart from the RLM out-of-sync indications). Definitions of events would be further complicated by the fact that RAN1 specifications are state-less: RLM indications are counted in RRC, and beam failure indications in MAC. The mere introduction of events is thus quite complicated.

Based on the complexity of defining events, and the need to specify also a MAC CE reporting format, we propose

Event-driven LTM reporting is not specified in Rel-18.

As remarked above, there are benefits of event-driven reporting, and it would be relevant to consider such improvements for Rel-19.

## 2.3	Operations on candidate cells before the cell switch

Before accessing the target, the UE must have acquired the DL timing of the target. In legacy, the UE does this after processing the handover command. In other words, the UE waits until for the next SSB arrives. The same type of requirement applies also for TCI state activation. To eliminate or significantly reduce the interrupt due to DL synchronization, the UE should acquire the DL synchronization before processing the handover command.

RAN1 made the following agreement in RAN1#111:

Agreement

- Regarding the potential RAN1 enhancements to reduce the handover delay / interruption for Rel-18 LTM

  - Support at least DL synchronization for candidate cell(s) based on at least SSB before cell switch command

    - Further study the necessary mechanism, e.g. signaling and UE capability

This is very important agreement. Performing DL synchronization to the target takes some 20ms, during which the UE cannot receive from source, and in legacy, this is a large fraction of the interrupt. We note the agreement states that we should further study signaling and UE capability.

As this functionality is critical for the LTM performance, we propose

A UE that supports LTM must support DL synchronization for at least one candidate cell based on SSB.

Hence, DL pre-synchronization to one target is a basic feature.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN1 should not assume that any information that is available at the UE is also available in the serving DU.

Observation 2	At the reception of the beam indication command, the UE must be able to derive a Rel-17 TCI state.

Observation 3	In the current specification, the serving DU does not know the TCI states of cells served by another DU. Enhancements to F1 are needed to support this.

Observation 4	The serving DU does not automatically know the mapping between an RS and the corresponding TCI state – but the UE does.

Observation 5	If the beam indication contains an explicit RS identifier, LTM can be supported both with the Rel-15 and Rel-17 TCI frameworks.

Observation 6	The latency of event-driven reporting is typically larger than the latency of network-initiated reporting.

Observation 7	Provided that the event is properly designed, the reporting overhead of event-driven reporting can be lower than network-initiated reporting.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Support also scenario 1 with LTM: the beam indication is transmitted before the cell switch command.

Proposal 2	Support that an LTM cell-switch command triggers a cell-switch, but not an update of the indicated TCI state

Proposal 3	The beam indication contains an explicit identifier of a reference signal.

Proposal 4	The UE maps the RS identifier to a TCI state in the correct candidate configuration.

Proposal 5	The LTM mobility trigger information is conveyed in a MAC CE, and the same MAC CE is used for the LTM triggering.

Proposal 6	The explicit RS identity refers to the SpCell of the indicated candidate.

Proposal 7	The beam indication contains a TA value for the target cell.

Proposal 8	The beam indication contains ID of the active DL and UL BWPs for the target SpCell.

Proposal 9	For gNB scheduled L1 measurement report for Rel-18 LTM, UCI can also be reported over periodic and semi-persistent PUCCH.

Proposal 10	In every LTM measurement report, one measurement from a serving cell is included.

Proposal 11	In every LTM measurement report, a measurement value from one primary candidate cell is included.

Proposal 12	In an LTM measurement report, measurements on additional secondary candidates can be included.

Proposal 13	Use the candidate configuration index as an identifier for the primary and secondary candidate cell measurement.

Proposal 14	An SSB index is included in the primary and secondary candidate cell measurement.

Proposal 15	All LTM reports are zero-padded to ensure that the payload is always 12 bits or larger.

Proposal 16	RAN1 defines what information should be included in the measurement configuration, both for the UE and the serving DU, while leaving the RRC configuration details to RAN2/3.

Proposal 17	The configuration of a measurement target contains PCI, ARFCN and the SSB SCS.

Proposal 18	Do not support CSI-RS as a measurement target for LTM.

Proposal 19	Event-driven LTM reporting is not specified in Rel-18.

Proposal 20	A UE that supports LTM must support DL synchronization for at least one candidate cell based on SSB.

# References

RP-213565, New WID on Further NR mobility enhancements, MediaTek, RAN#94e, December 2021

R1-2208331, LS on L1/L2-based inter-cell mobility, RAN2, RAN2#119-e, August 2022

R1-2301095, Timing advance management to reduce latency, Ericsson, RAN1#112, Athens, February 2023

R2-2211061, LS on RAN2 agreements about L1/L2-triggered mobility (LTM), RAN2, RAN2#119bis-e, October 2022

R1-1716375, Details on TRS design, Ericsson, 3GPP TSG-RAN WG1 Meeting #NR3, September 2017

TS 38.215, NR; Physical layer measurements, RAN#97-e, v 17.2.0, September 2022

TS 38.133, NR; Requirements for support of radio resource management, RAN#96, V 17.6.0, June 2022

R1-2300007, Reply LS on L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility, RAN3, RAN3#118, August 2022

R1-2300014, Reply LS on L1 measurement and configurations for LTM, RAN2, RAN2#120, August 2022