3GPP TSG-RAN WG1 Meeting #112		R1-2300407

Athens, Greece, February 27th – March 3rd, 2023

Source:	vivo

Title:	Discussion on SL LBT failure indication and consistent SL LBT failure

Agenda Item:	5

Document for:	Discussion and Decision

# Introduction

RAN2 sent the following LS [1] to RAN1, informing the background of the discussion regarding SL LBT failure indication and consistent SL LBT failure in RAN2. Moreover, RAN2 would like to know the granularity (i.e., per SL BWP, per SL resource pool, per RB set, etc.) of this LBT failure indication of each SL transmission that the PHY layer can identify and report.

# 1 Overall description In RAN2 #119bis-e, RAN2 discussed consistent SL LBT failure detection and recovery procedure for SL-U and made the following agreements: Agreement on consistent LBT failure: 1: SL-specific LBT failure indication from PHY is needed for SL-specific consistent LBT failure detection in the MAC. How/whether it is used for other purposes can be further discussed. 2: Support SL-specific consistent LBT failure detection and recovery procedure in the MAC for SL-U. Details of recovery to be further worked on granularity of (consistent) LBT failure. 3: Send LS to RAN1 asking “When an SL-specific LBT failure indication is notified for an SL transmission by the PHY, in which resource granularity the SL-specific LBT failure can be considered as being detected (e.g. per Resource Pool, per RB set, per SL BWP, etc.)? - Detailed wording can be discussed during the email discussion. Some background information (e.g. why/what we (actually) ask) can be also provided. 4: As the general principle, reuse the consistent LBT failure detection procedure in NR-U as the baseline for SL-specific consistent LBT failure detection in SL-U. 5: As in NR-U, introduce the following parameters and variables for the SL-specific consistent LBT failure detection in SL-U as the baseline: - An SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER); - An SL-specific maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount); - An SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer). 6: Reuse the following MAC behaviors on TIMER/COUNTER handling in NR-U for SL-specific consistent LBT failure detection procedure in SL-U as the baseline: - As in NR-U, if an SL-specific LBT failure indication is received from the lower layer, the SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is incremented by one. - As in NR-U, if an SL-specific LBT failure indication is received from the lower layer, start or restart the SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer) - As in NR-U, if the SL-specific LBT failure indication counter value is equal to or larger than the SL-specific maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount), consistent LBT failure is triggered/declared by the MAC entity. - As in NR-U, if the SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer) expires, the SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is reset to 0. - As in NR-U, if the maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount) or SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer) is reconfigured, SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is reset to 0. 7: Support the mechanism that a mode-1 UE can indicate the SL-specific consistent LBT failure to the gNB. FFS on a mode-2 UE in RRC_CONNECTED. To support consistent SL LBT failure detection procedure in SL-U, RAN2 agreed to reuse the consistent LBT failure detection procedure in NR-U as the baseline. RAN2 found that for SL-U, how consistent SL LBT failure detection should be performed depends on in which granularity an SL LBT failure instance is indicated to MAC, when the SL LBT failure is notified by PHY. For example, in NR-U when LBT failure is notified due to an intended UL transmission by PHY, MAC considers the LBT failure as an LBT failure instance indicated for the UL BWP where the LBT failure has happened, so that “Consistent LBT failure is detected per UL BWP by counting LBT failure indications, for all UL transmissions, from the lower layers to the MAC entity” as specified in TS 38.321. By contrast, for SL-U RAN1 has already agreed to support only one SL BWP on a SL-U carrier (as in legacy R16/17 NR SL), which is essentially different from NR-U from resource configuration perspective. Thus, it is unclear to RAN2, when SL LBT failure is notified by PHY due to an intended SL transmission, whether the SL LBT failure can still be considered as an LBT failure instance indicated for the SL BWP where the SL LBT failure has happened, or alternatively it needs to be considered as an SL LBT failure instance indicated in other resource granularity (e.g. indicated for an SL resource pool, for an SL RB set, etc). This will affect RAN2’s decision on whether consistent SL LBT failure detection can be (or needs to be) performed in other granularity (e.g. per resource pool, per RB set, etc.) than the per BWP manner as in NR-U. Therefore, RAN2 respectfully request RAN1 to provide the guideline on the following question related to the SL LBT failure indication. - Question: When SL LBT failure is notified by PHY due to an intended SL transmission, what is the granularity in which MAC can consider that the SL LBT failure has been detected (e.g. whether MAC can consider that the SL LBT failure has been detected per SL BWP, per SL resource pool, per RB set, etc.). # 2 Actions To RAN1 ACTION: RAN2 respectfully request RAN1 to provide the feedback on the above Question regarding the granularity of SL LBT failure indication.

RAN1 discussed this issue in the RAN1#111 meeting but could not achieve a consensus. In this contribution, we provide our view on this issue.

# Discussion

RAN1 has already agreed that in SL-U, a SL BWP can be (pre-)configured to include one or multiple resource pools, and one resource pool can be (pre-)configured to include an integer number of RB sets. Therefore, when a SL LBT failure is notified for a SL PSCCH/PSSCH transmission, it is feasible to indicate on which SL BWP and SL resource pool, as well as the RB sets (pre-)configured to the resource pool, the failure has been detected.

Further, RAN1 has also agreed that for dynamic channel access mode with multi-channel case in SL-U, NR-U UL channel access procedure is considered as the baseline for PSSCH transmission on multiple channels. According to the TS 37.213 specification (cited below) for UL transmission, when the UE intends to transmit on a set of channels, the UE performs channel access procedure for each of the channels separately before transmission, and detects the LBT failure for the transmission if LBT fails on any of the channel. Therefore, for a SL LBT failure, from a technical perspective, the UE can indicate on which RB set(s) the failure has been detected.

If a UE - is scheduled to transmit on a set of channels C, and if the UL transmissions are scheduled to start transmissions at the same time on all channels in the set of channels C, or - intends to perform an uplink transmission on configured resources on the set of channels C, and if UL transmissions are configured to start transmissions at the same time on all channels in the set of channels C, the following is applicable: - if Type 1 channel access procedure is indicated or intended for the scheduled or configured UL transmissions, respectively, to be transmitted on the set of channels C, - the UE may transmit on channel ci∈C using Type 2 channel access procedure as described in clause 4.2.1.2, - if the channel frequencies of the set of channels C is a subset of the sets of channel frequencies defined in clause 5.7.4 in [2], and - if Type 2 channel access procedure is performed on channel ci immediately before the UE transmission on channel cj∈C, i≠j, and - if the UE has accessed channel cj using Type 1 channel access procedure as described in clause 4.2.1.1, - where channel cj is selected by the UE uniformly randomly from the set of channels C before performing Type 1 channel access procedure on any channel in the set of channels C. - the UE may transmit on channel ci∈C using Type 1 channel access procedure as described in clause 4.2.1.1 - the UE may not transmit on channel ci∈C within the bandwidth of a carrier, if the UE fails to access any of the channels, of the carrier bandwidth, on which the UE is scheduled or configured with UL resources. - the UE may not transmit on a channel within the bandwidth of a carrier if the UE is configured without intra-cell guard band(s) on an UL bandwidth part as described in clause 7 of [8], and the UE fails to access any of the channels of the UL bandwidth part.

On the other hand, for dynamic channel access mode with multi-channel case for PSFCH, RAN1 has agreed to use the NR-U DL multi-channel access procedure as the baseline for transmission on multiple channels. Similar to the PSSCH transmission, from a technical perspective, the UE can indicate on which RB set(s) the SL LBT failure has been detected for a PSFCH transmission.

Regarding the S-SSB, although RAN1 has not agreed yet on the channel access procedure for dynamic channel access mode with multi-channel case in SL-U, from the channel access perspective, there is no technical difference between the PSCCH/PSSCH/PSFCH and the S-SSB. However, if the S-SSB is transmitted outside of any SL resource pool (at least for the R16 S-SSB), it may be infeasible to indicate the SL resource pool that associates with the SL LBT failure.

The following table summarizes the feasibility of each reporting granularity for each channel.

Table 1

|  | PSCCH/PSSCH | PSFCH | S-SSB |
| --- | --- | --- | --- |
| Per RB set | Feasible | Feasible | Feasible |
| Per resource pool | Feasible | Feasible | Not possible |
| Per BWP | Feasible | Feasible | Feasible |

Observation 1: When a SL LBT failure is notified by PHY for a SL PSCCH/PSSCH/PSFCH transmission, it is feasible to indicate on which SL BWP, which SL resource pool and which the RB set(s) the failure has been detected.

Observation 2: When a SL LBT failure is notified by PHY for a SL S-SSB transmission, it is feasible to indicate on which SL BWP and which the RB sets the failure has been detected.

The decision of reporting granularity is highly dependent on how the consistent LBT failure recovery is designed. Given that the consistent LBT feature and recovery feature is introduced and designed by RAN2, it seems not reasonable to design the recovery mechanism in RAN1. Noted that in the LS, RAN2 clearly states that this is the RAN2’s decision:

Thus, it is unclear to RAN2, when SL LBT failure is notified by PHY due to an intended SL transmission, whether the SL LBT failure can still be considered as an LBT failure instance indicated for the SL BWP where the SL LBT failure has happened, or alternatively it needs to be considered as an SL LBT failure instance indicated in other resource granularity (e.g. indicated for an SL resource pool, for an SL RB set, etc). This will affect RAN2’s decision on whether consistent SL LBT failure detection can be (or needs to be) performed in other granularity (e.g. per resource pool, per RB set, etc.) than the per BWP manner as in NR-U.

Thus, it is more reasonable for RAN1 to provide the analysis of feasibility, and to leave the decision to RAN2. On the other hand, it would also be helpful to RAN2 if RAN1 can provide the guideline or recommendation on the granularity of LBT failure indication.

- The “per BWP” reporting granularity is not useful in SL-U.

  - As noted also by RAN2, for SL UE only one BWP is configured, thus such a reporting mechanism is useless for recovery – it is impossible for the SL UE to switch to another SL BWP.

- The “per resource pool” reporting granularity is not ideal.

  - Firstly, it is not applicable to S-SSB, which means that two different reporting granularities should be supported for different channels.

  - Secondly, such a reporting granularity requires at least two resource pools are configured to a UE. If these two pools are configured in FDM manner (e.g., one pool spans on one RB set, while the other pool spans on another RB set), it is essentially similar to “per RB set” granularity, but with additional restriction that the maximum TB size and peak data rate are reduced. If these two pools are configured in TDM manner (e.g., the pools span the same RB sets), when a consistent LBT failure happens in one RB set, such failure occurs in both pools including this RB set, thus this reporting granularity is useful.

- The “per RB set” reporting granularity seems to be reasonable.

  - Firstly, the “per RB set” reporting granularity is aligned with the granularity of UE performing channel access.

  - Secondly, it is applicable to all the physical channels, and does not require more than one resource pools are configured.

  - Additionally, it should be noted that the resource allocation mechanism is different between NR-U and SL-U. In NR-U, the gNB can configure the UE to report the RSSI measurement result for each RB set, and schedule the UL transmission in the in the suitable RB set (e.g., with lowest interference or potentially less collision). In other words, the gNB is responsible to schedule the UL transmission in the suitable RB set to avoid consistent LBT failure. Therefore, if consistent LBT failure still happens, it is likely that no suitable RB set is presented in the UL BWP. Consequently, triggering the BWP switching would be the only fallback solution. While in SL-U, the resource is allocated in a distributed manner (at least for mode-2), and the resource is selected randomly (in the MAC layer). Thus, if consistent LBT failure is detected in one RB set, it is still possible that the other RB sets are good enough for subsequent transmission.

Observation 3: The “per resource pool” reporting granularity is too restrictive and less useful for consistent LBT failure recovery, while the “per RB set” reporting granularity is applicable to all the channels and beneficial to the recovery mechanism.

In summary, if RAN1’ recommendation is desirable, “per RB set” reporting granularity is preferred.

Proposal 1: When a SL LBT failure is notified by PHY, the PHY layer reports to the MAC layer on which RB set(s) the LBT failure has been detected for a S-SSB, PSFCH or PSCCH/PSSCH transmission.

Alternatively, if RAN1 cannot achieve a consensus on the reporting granularity, we are OK to leave the decision to RAN2. A draft LS [2] is provided in this case.

Proposal 2: If RAN1 cannot achieve the consensus on the reporting granularity, it is up to RAN2 to determine the granularity of SL LBT failure indication.

# Conclusion

In the contribution, we provide our view on the LBT failure indication granularity, with the following observations and proposals.

Observation 1: When a SL LBT failure is notified by PHY for a SL PSCCH/PSSCH/PSFCH transmission, it is feasible to indicate on which SL BWP, which SL resource pool and which the RB set(s) the failure has been detected.

Observation 2: When a SL LBT failure is notified by PHY for a SL S-SSB transmission, it is feasible to indicate on which SL BWP and which the RB sets the failure has been detected.

Observation 3: The “per resource pool” reporting granularity is too restrictive and less useful for consistent LBT failure recovery, while the “per RB set” reporting granularity is applicable to all the channels and beneficial to the recovery mechanism.

Proposal 1: When a SL LBT failure is notified by PHY, the PHY layer reports to the MAC layer on which RB set(s) the LBT failure has been detected for a S-SSB, PSFCH or PSCCH/PSSCH transmission.

Proposal 2: If RAN1 cannot achieve the consensus on the reporting granularity, it is up to RAN2 to determine the granularity of SL LBT failure indication.

# Reference

- R1-2210805, “LS on SL LBT failure indication and consistent SL LBT failure”, RAN2, vivo

- R1-2300406, “Draft reply LS on SL LBT failure indication and consistent SL LBT failure”, vivo