3GPP TSG RAN WG1 #112                                                                                   R1-2300477

Athens, Greece, February 27th – March 3rd, 2023

Source:	vivo

Title:	Discussion on low power wake-up receiver architecture

Agenda Item:	9.13.2

Document for:	Discussion and Decision

- Introduction

A new SID on studying low-power wake-up signal (WUS) and receiver (WUR) for NR was approved in RAN P #94 e-meeting[1]. In previous RAN1 meeting[2], for the analysis of a receiver architecture,  it has been agreed that a list of details of the receiver and further analysis are encouraged to be provided by companies. In this contribution, we firstly introduce how the information exchanged between low-power WUR and main radio. Then we discuss the performance metrics as well as the potential LP-WUR receiver architectures with pros and cons as well as the achieved performance from existing literatures. At last, we provide the design on key modules of the WUR.

- Low-power WUR receiver and main radio

Upon reception of low-power WUS, low-power WUR triggers main radio to switch on, otherwise, the main radio is OFF or keep in an extreme deep sleep mode (this is for IDLE/INACTIVE mode) or light sleep/microsleep (this is for CONNECTED mode). Of course, the main radio and  low-power WUR will exchange information between each other which is illustrated in Figure 1 Information exchanged between low-power WUS and WURFigure 2 Low-power receiver architecture with RF envelope detection, such as

- Low-power WUR can get initial configurations before work from the main radio

- Low-power WUR can indicate ‘wake-up’ to the main radio

- Low-power WUR can pass additional messages to the main radio, and the main radio can further process/parse the messages so that it can be understandable for the main radio legacy system. However, these additional messages should be agnostics to the low-power WUR.

Proposal 1: The main radio and low-power WUR exchange information between each other, such as

- Low-power WUR gets initial configurations from the main radio (received from gNB configuration)

- Low-power WUR can indicate ‘wake-up’ to the main radio

- Low-power WUR can pass additional decoded messages to the main radio, these messages are processed and parsed in the main radio but agnostics to the low-power WUR

Figure 1 Information exchanged between low-power WUS and WUR

- Low-power WUR receiver architecture

  - Performance metrics of low-power WUR

Power consumption, sensitivity, and data rate

Along with expected ultra low power consumption of low-power WUR, the achievable sensitivity is critical to guarantee the communication range of the low-power WUS. Furthermore, the allowed data rate which impacts the wake-up efficiency should also be considered. Generally, the achievable receiver sensitivity can be increased by exploiting sophisticated hardware, i.e., using LNA at RF, high accuracy LO, and high-Q filter of narrow bandwidth, and etc., which may largely increase the power consumption. On the other hand, the sensitivity can be also increased by exploiting coding or spreading/repetition scheme by reducing the data rate. Further, the reduced data rate can also reduce power consumption as a lower sampling rate can be used for ADC and digital processing. According to the survey[3], the trade-off of power consumption, sensitivity and data rate can be represented by power consumption vs. sensitivity nomalized by data rate, and it is observed that a 20 dB increase of nomalized sensitivity requires 10 times power consumption increasing.

For given receiver architecture, the sensitivity varies as data rate changes, and thus, it’s better to investigate the achivable sensitivity of low-power WUR along with the supported data rate. In[4], it provides the sensitivity metric normalized to the data rate as given below:

(1)

(2)

where the sensitivity is normalized to the data rate with  for designs with a non-linear squaring function for envelop detection in (1), and the sensitivity is normalized to the data rate with  for designs with a linear operation to demodulate the signal or designs using a non-linear squaring function for envelop detection after high active pre-ED gain with sharp filtering.

Observation 1  Design on low-power WUR architecture is a trade-off of power consumption, sensitivity and data rate.

Observation 2 Achievable sensitivity of low-power WUR should be investigated along with the supported data rate.

Proposal 2:  Study the metric for representing the sensitivity at certain data rate for low-power WUR, e.g., the sensitivity normalized to data rate.

  - Potential receiver architectures for OOK

    - Architecture with RF envelope detection

As shown in Figure 2, the input RF signal passes through the matching network, optionally amplified by a low noise amplifier(LNA), filtered by a bandpass filter (BPF) at RF, then it is converted to baseband via RF envelop detector.

Due to the nonlinear nature of envelope detector, it simply detects the amplitude of the RF signal and discards frequency and phase information, and thus, interferences and target signal are superposed after envelope detection, which cannot be filtered out by low pass filter (LPF) at BB band. Therefore, the interference rejection, especially for adjacent channel interference and adjacent subcarrier interference, highly relies on a RF BPF, and it desires a high-Q RF BPF with bandwidth approximates the signal bandwidth of low-power WUS as much as possible. Considering high-Q RF BPF need larger area, higher power consumption and cost, it would be not effective to set an exclusive high-Q BPF to pick up the low-power WUS in each band when low-power WUR is used to receive low-power WUS in multiple bands. Therefore, the such architecture is more appropriate for low-power WUS reception in single band.

Observation 3  Due to demanding a band specific high-Q RF BPF, the receiver architecture with amplitude detection at RF is more suitable for devices supporting single band.

Figure 2 Low-power receiver architecture with RF envelope detection

Furthermore, as there is no local oscillator (LO) and mixer adopted, the power consumption can be very low, e.g., several uw or less than 1 uw[5] , which enables a long battery life for IoT devices. However, the sensitivity may be limited, e.g., -56.5dBm~-75dBm[5][6]. The sensitivity can be improved by adopting LNA but causing a increased power consumption. Therefore, such type of receiver architecture is more appropriate for the devices pursuing extreme low power consumption with limited sensitivity requirement.

Observation 4  The reported sensitivity for receiver architecture with amplitude detection at RF in the literatures[5][6]is -56.5dBm~-75dBm with data rate serval kbps to hundred kbps under power consumption less than 1uw to tens of uw.

| reference | Carrier frequency | sensitivity | Power consumtion | Data rate | Interference rejection | oscillator | tech | year |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [5] | 2.4GHz | -56.5dBm | 236nw | 8.192kbps | N/A | none | CMOS 65nm | 2016 |
| [6] | 2.4 GHz/ 915MHz | 915MHz: -75dBm at 100kb/s, -80dBm at 10kb/s 2.4GHz:-64dBm at 100kb/s, -69dBm at 10kbs | 51 uW | 10 kbs; 100kbs | N/A | none | CMOS 90nm | 2010 |

    - Heterodyne architecture with IF envelope detection

As shown in Figure 3, the input RF signal passes through the matching network, filtered by a BPF at RF. Then the RF signal is down-converted to low IF via LO and RF mixer to ease implemenatation of blocks of amplifier and filter at RF. The amplifier and filter at IF is more power efficient. As interference rejection can be performed by a high-Q IF BPF instead of a high-Q RF BPF, such receiver architecure can support low-power WUS reception in multiple bands flexibly.

After IF filtering,  the output IF signal is converted to BB by a IF envelop detector, and subsequently ditigalized for digital signal processing by a compartor or multi-bit ADC. By IF operation, such receiver architecure can avoid the influence of DC offset and flicker noise, however, it suffers from image interference which should be overcome by proper filter design at RF or IF. The power consumption of mixer-first receiver architecture is limited by the LO, i.e., LC oscillator,  the higher accuracy and stability causes larger power consumption, in order to reduce the power devoted to the LO,  duty-cycle mechanism can be considered either on LO block or the receiver, and thus, the power consumption can be redudced at the cost of certain latency. On the other hand,  power consumption can be also reduced by relaxing the accuracy and stability of the LO, e.g., replacing the LC oscillator with a ring oscillator which can reduce the power consumption ten times or more. Due to the frequency inaccuracy of the ring oscillator, the RF signal is down converted to the IF band with certain frequency offset, and thus, such frequency offset should be taken into account when designing the bandwidth of the IF BPF to accomandate the signal properly. Furthermore, with power budget given , the minimum frequency offset of the oscillator should also be studied and identified.

Figure 3 Heterodyne architecture with IF envelope detection

| reference | Carrier frequency | sensitivity | Power consumtion | Data rate | Interference rejection | oscillator | tech | year |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [7] | 2.4GHz | −83dBm | Active power: 227uw (LNA achieves 27 dB voltage gain dissipating 55uw) | 1 Mb/s | Image interference rejection through complex filter | LC-VCO phase noise of -132.85dBc/Hz at a 1MHz offset dissipating 78uw； FLL reduce the frequeny error from 1.067MHz to 100kHz | CMOS 65nm | 2015 |
| [8] | 2.4 GHz | −97 dBm | Active power: 99 uw | 10 kb/s @-97dBm; 50kb/s @-92dBm | a carrier-to-interferer ratio better than −27 dB at 5 MHz offset, for a data rate of 10 kb/s at a 10−3 bit error rate | LC-VCO: frequency error: 28.6ppm dissipating 44uw, -101dBc/Hz phase noise at 1MHz offset | CMOS 65nm | 2016 |

Observation 5  For heterodyne architecture with IF envelope detection, the power consumption can be reduced by replacing  a high accuracy LO with a medium accuracy LO, and the frequency offset caused by the frequency error of the LO can be further studied.

Observation 6  The reported sensitivity for heterodyne architecture with IF envelope detection in the literatures[7][8] is -83dBm~-97dBm with data rate tens of kbps to several Mbps under power consumption hundreds of uw.

    - Homodyne/zero-IF architecture with BB envelope detection

As shown in Figure 4, the input RF signal passes through the matching network, filtered by a BPF at RF. Then the RF signal is directly down-converted to BB via LO and RF mixer, and thus, a high accuracy LO, i.e., LC oscillator is required which consumes around hundreds of micro watts. Then the output BB signal is ditigalized for didital signal processing by a compartor or multi-bit ADC. Due to direct down coversion of the RF signal to DC, it avoids image interference existed in the 2nd type of receiver, however, the flicker noise and DC offset issue should be carefully resolved, otherwise, the signal will be drowned. Furthermore, a high-Q BB LPF can be used for interference rejection instead of a high-Q RF BPF, such receiver architecture can aslo support low-power WUS reception in multiple bands flexibly.

Observation 7    For homodyne/zero-IF architecture with BB envelope detection,  low-power solution on flicker noise and DC offset issue should be studied.

Figure 4 Homodyne/zero-IF architecture with BB envelope detection

Observation 8 The reported sensitivity for homodyne/zero-IF architecture with baseband envelope detection in the literature[9] is −92.6dBm with data rate tens of kbps to hundreds of kbps under power consumption hundreds of uw.

| reference | Carrier frequency | sensitivity | Power consumtion | Data rate | Interference rejection | oscillator | tech | year |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [9] | 2.4GHz | −92.6dBm | Active power: 667uw/495uw | 62.5kbps;250kbps | Tolerate -40.5 dBm wifi adjacent channel blocker with 3 dB receiver desensitizatio | LC-VCO phase noise of -128dBc/Hz at a 20MHz offset dissipating 200uw | CMOS 28nm | 2019 |

    - Summary on the receiver architectures for OOK detection

The summary on the receiver architectures for OOK detection is given in Table 1 including hardware, relative active power consumption, sensitivity per date rates in literatures, interference rejection, guardband requirements, potential noise figure range as well as multi-band capability.

Observation 9  The relative levels of active power consumtion of receive architectures for OOK detection are 0.01x~0.1x unit for RF envelope detection,  0.1x~1x unit for heterodyne architecture with IF envelope detection, and 0.1x~1x unit for homodyne/zero-IF architecture with BB envelope detection.

Table 1. Summary on the receive architectures for OOK detection

|  | RF envelope detection | Heterodyne architecture with IF envelope detection | Homodyne/zero-IF architecture with BB envelope detection |
| --- | --- | --- | --- |
| Hardware (including potential sharing components of MR) | Envelope detector first; possible to share RF BPF of MR | Mixer first and envelope detection at IF; possible to share RF BPF of MR | Mixer first with envelop detection in analog or digital BB; possible to share RF BPF of MR |
| Active power consumption (relative) | 0.01x~0.1x unit | 0.1x～1x unit | 0.1x~1x unit |
| Sensitivity@data rates in literatures | 915MHz: -75dBm @ 100kb/s, -80dBm @10kb/s 2.4GHz: -109dBm | 2.4GHz: -97dBm @10 kb/s, -92dBm @50kb/s, −83dBm @1Mb/s | 2.4GHz: −92.6dBm@62.5kb/s |
| Adjacent channel interference rejection & adjacent subcarrier interference rejection | Matching network, RF BPF, guardband | Matching network, IF BPF, guardband | Matching network, BB BPF, guardband |
| Co-channel interference rejection | Multi-bit ADC, spreading | Multi-bit ADC, spreading | Multi-bit ADC, spreading |
| guardband | guardband between WUS and other signals in the same channel, guardband for adjacent channel | guardband between WUS and other signals in the same channel, guardband for adjacent channel | guardband between WUS and other signals in the same channel, guardband for adjacent channel |
| Potential nosie figure | [17-20] dB | [13-17] dB | [14-18] dB |
| Multi-band capability | Yes, if multiple high-Q RF BPFs are available | Yes, via LO tuning | Yes, via LO tuning |

  - Potential receiver architectures for FSK

    - Single-subcarrier FSK and multi-subcarrier FSK

- Single-subcarrier FSK: 1 bit

- Single-subcarrier FSK: 2 bits

- Multi-subcarrier FSK: 1 bit

- Multi-subcarrier FSK: 2 bits

Figure 5 Single-subcarrier FSK and multi-subcarrier FSK: 1 bit & 2bits

Observation 10   Compared to single-subcarrier FSK, multi-subcarrier FSK is more robust to channel fluctuation.

Observation 11  2M frequencies or frequency sets are required to carry M bits for single-subcarrier FSK and multi-subcarrier FSK, respectively.

Observation 12  Frequency gaps are required among multiple frequencies or frequency sets for frequency differentiation .

    - Parallel OOK receivers based FSK detection

Regarding FSK detection based on parallel OOK receivers,  there are multiple parallel chains selecting one of the multiple frequencies or frequency sets and extracting the amplitude of the signal by envelope detection. As 2M frequencies or frequency sets are required to carry M bits for single-subcarrier FSK and multi-subcarrier FSK, respectively, a total of 2M analog bandpass/lowpass filters and evelep detectors are needed in this case.

Observation 13 For FSK receiver based on parallel OOK receivers, 2M analog bandpass filters and envelope detectors are needed for reception of FSK carrying M bits.

For the frequency gaps among multiple frequencies or frequency sets, a small one provides deployment flexibility but requires stringent design on BPFs with sharp transition band for frequency differenciation; a large one relax the requirements on BPFs but may cause resource waste as well as the deployment inflexibility.

Further, when each chain applies heterodyne architecture with IF envelope detection or homodyne/zero-IF architecture with baseband envelope detection, the frequency error of the oscillator will bring frequency offset  which brings further constraint on the frequency gaps among multiple frequencies or frequency sets. An illustrative example for single-subcarrier 2FSK detection by homodyne/zero-IF architecture is shown in Figure 6 Single-subcarrier 2FSK with frequency error. When downconversion to zero IF is performed, with a maximum frequency offset of the oscillator caused by the frequency error, the downconverted frequencies will be in the range of centered by the target one  e.g., for , after mixed with central frequency , it will fall into the range of  , and for , it will fall into the range of  . Therefore, the passband of RF BPF shall cover the frequency range caused by the frequency error, otherwise, the desired signal maybe filtered out. Accordingly, the frequency gap between two adjacent frequencies or two adjacent frequency sets should be not smaller than two times of the max frequency offset.

Figure 6 Single-subcarrier 2FSK with frequency error

Observation 14 For FSK receiver based on parallel OOK receivers with a heterodyne or zero-IF architecture, the frequency gap between two adjacent frequencies or two adjacent frequency sets should be not smaller than two times of the max frequency offsets.

Regarding the frequency gap, whether it can be used by other DL signals/channels or other WUS signals is another issue to be studied. As discussed above, the frequency offset to the target frequency caused by the frequency error shall be included in the passband of the BPF, and thus, at least an amount of  the max frequency offset at each end of the frequency gap shall not be used by other DL signals/channels or other WUS signals.

Observation 15 For FSK receiver based on parallel OOK receivers with a heterodyne or zero-IF architecture, at least two times of the max frequency offsets within the frequency gap shall not be used by other DL signals/channels or other WUS signals.

With the parallel OOK receivers, FSK detection can reduce interference effects by comparing the outputs of multiple branches and extracts interfefrences before sampling decisions,  if the interferences across 2M frequencies or frequency sets are coherent. However, if the interferences are independent, it may not work.

Observation 16 For FSK receiver based on parallel OOK receivers, interference rejection performance highly depends on  whether the interferences across 2M frequencies or frequency sets are coherent or not.

    - Frequency to amplitude conversion based FSK detection: quadrature frequency discriminator

A quadrature frequency discriminator in analog domain comprises of a phase shift network, an anolog multiplier and a low-pass filter. The phase shift network shifts the phase of incoming FM signal by an amount which is proportional to the instantaneous frequency.  One way to the multiplier is the input FM signal, and the other way to the multiplier is the input FM signal after being phase-shifted by the phase shift network. The two input signals are multiplied with the multiplication gain factor of the multiplier and thus, the instantaneous frequency is transformed into shifting of the phase. Finally after the amplitude detector, the shifting of the phase is reflected to amplitude difference. Consequently, a high precise phase shifting network is necessary to discriminate frequency deviation.

In practice, the center frequency of the phase shift network may not be precisely aligned with the carrier frequency of the input FM signal, and thus, a DC offset will be generated which may cause blocking and increase the distortion level of the signal variation. The DC offset is a critical issue of this type of detector, which should be minimized.

In general, the quadrature frequency discriminator is able to operate with relatively low input levels, typically down to levels of around 100 microvolts, corresponding to a sensitivity around -70dBm when input impedance matches to 50 ohm.  In [10], a sensitivity of -72dBm is achieved at data rate 3.2kHz with power consumption of 2.3mW.

Figure 7 analog quadrature frequency discriminator

Observation 17  A high precise phase shifting network is necessary to discriminate frequency deviation.

Observation 18 The DC offset in analog quadrature FM discriminator deteriorates the detection performance.

Observation 19  A sensitivity level of around -70dBm with data rate several kbps under power consumption several milli watts is achieved by analog quadrature FM discriminator.

Through quadrature frequency discriminator,  it is understood that the instantaneous frequency is transformed into shifting of the phase and then reflected by amplitude difference, and thus, it does not work well to transform multiple subcarriers to a unified phase to get a amplitude.

Observation 20  For FSK receiver based on frequency to amplitude conversion, it does not work for multi-subcarrier FSK detection.

At last, considering WUS signal may include sequence part, correlation-based sequence detection can be considered. Compared to analog sequence detection, it is easier to implement sequence detection in digital domain without addional hardware elements needed. In digital sequence detection,  the output soft information of ADC is expected to provide better detection performance than hard decision of 0, 1. However, for FSK carrying M bits, the output soft information from one of 2M codepoint cannot be directly applied to correlation-based detection, as the soft information of one codepoint cannot be used as the soft information of  the M information bits. Therefore, hard decision should be made before performing correlation-based sequence detection for FSK carrying M bit.

Observation 21  For FSK receiver, it is difficult to utilize soft information in correlation-based sequence detection for FSK carrying M bits.

    - Summary on the receiver architectures for OOK detection

The summary on the receiver architectures for FSK detection is given in Table 2 including hardware, relative active power consumption, sensitivity per date rates in literatures, interference rejection, guardband requirements, as well as multi-band capability.

Table 2. Summary on the receive architectures for FSK detection

|  | Based on parallel RF/IF/BB envelop detection | Based on frequency to amplitude conversion (via phase shifting network) |
| --- | --- | --- |
| Whether support single-tone FSK or multi-tone FSK | both | Single-subcarrier FSK only |
| Hardware (including potential sharing components of MR) | 2M parallel paths of RF/IF/BB envelop detection for M-bit FSK; possible to share RF BPF of MR | Phase shifting network is needed; possible to share RF BPF of MR |
| Active power consumption (relative) | 0.01x~0.1x unit | 1x unit |
| Sensitivity@data rates in literatures | Not available | 455kHz: -72dBm @3.2 kb/s, |
| Adjacent channel interference rejection & adjacent subcarrier interference rejection | Matching network, RF /IF/BB BPF, guardband | Matching network, IF/BB BPF, guardband |
| Co-channel interference rejection | Multi-bit ADC, spreading | Phase shifting network |
| Frequency gaps among multiple frequencies or frequency sets | Frequency gaps can be used for other DL signals/channels transmission | Frequency gaps shall not be used |
| guardband | guardband between WUS and other signals in the same channel, guardband for adjacent channel; guardband between multiple frequencies or frequency sets | guardband between WUS and other signals in the same channel, guardband for adjacent channel; guardband between multiple frequencies or frequency sets |
| Multi-band capability | Yes, via multiple high-Q RF BPFs or LO tuning | Yes, via LO tuning |

Observation 22  The relative levels of active power consumtion of receive architectures for FSK detection are 0.01x~0.1x unit for parallel RF/IF/BB envelop detection based receiver and 1x unit for frequency to amplitude conversion based receiver.

  - Receiver architectures for OFDM rceiver

OFDM receiver for main radio

For typical OFDM receiver used for main radio, the active power consumption level is hundreds of milliwatts and beyond, which definitely can’t provide the demanding substantial power saving gain as stated in SID, and thus, OFDM receiver architecture for main radio is not considered for LP WUR.

Proposal 3: OFDM receiver architecture for main radio is not considered for LP WUR due to high power consumption.

OFDM receiver with relaxed hardware requirements

In literatures, there exist some designs to reduce power consumption of OFDM receiver by relaxing hardware requirements, e.g., phase noise performance can be relaxed if such receiver performs sequence detection only, and thus, the power consumption of oscillator and PLL can be reduced. With certain relaxed hardware requirements, the hardware components are typically different from the ones used in normal OFDM receiver.

Furthermore, the detection performance of OFDM signal highly depends on the relaxed hardware requirements. Less power consumption can be achieved by looser hardware requirements, but causing worse detection performance.

Thus, when the reduced power consumption is considerd for a OFDM receiver, the receiver architecture with functional blocks as well as the relaxed hardware requirements, e.g., frequency error, phase noise,  noise figure, I/Q mismatch, etc should be provided.

The reported performance of OFDM receiver with low power consumption in the literatures are summarized in Table 3, it is observed that the reported values mainly fall into level of  tens of milliwatts.

Observation 23  The hardware components of OFDM receiver with relaxed hardware requirements are typically different from those of OFDM receiver for main radio.

Observation 24  The reported values of OFDM receiver with low power consumption mainly fall into level of  tens of milliwatts.

Proposal 4: When considering the reduced power consumption for a OFDM receiver, the receiver architecture with detailed functional blocks as well as the relaxed hardware requirements, e.g.,  frequency error, phase noise,  noise figure, I/Q mismatch, and etc.,  should be provided.

Table 3. Summary on OFDM receiver with relaxed hardware requirements in literatures

| Reference | Carrier frequency | Sensitivity | Power consumtion (mw) | Demo /commercial | Year |
| --- | --- | --- | --- | --- | --- |
| [11] | 2.4GHz | -76dBm | 42 | Demo | 2014 |
| [12] | 930MHz | -92dBm | 18.9 | Demo | 2015 |
| [13] | 750-960MHz | N/A | 25 | Demo | 2017 |
| [14] | 450-2220MHz | -140dBm | 11.8 | Demo | 2019 |
| [15] | 2.2GHz | -111.9dBm | 56 | Demo | 2020 |
| [16] | 2.2GHz | -125dBm | 50 | Demo | 2020 |
| [17] | LTE | N/A | 57(WUR ON) 16(WUR OFF) | Demo | 2021 |
| [18] | 750-960MHz | -109dBm | 2.1 | Demo | 2021 |
| [19] | 700-2200MHz | -114dBm | 43.9 | commercial | Nrf9160 rev2 |

- Design on key modules of LP-WUR receiver

  - Analog RF/IF/BB filter

For low-power WUR architecture with mixer-first followed by amplitude detection, RF signal is down converted to low IF or BB, and thus, adjacent channel interference or adjacent subcarrier interferference can be filtered out at low IF or BB instead of RF. The RF BPF can be served for out-of-band interference rejection, and when low-power WUR shares the same bands as the main radio, the RF BPF can be also shared between low-power WUR and main radio.

For IF BPF and BB LPF, it is a trade-off between interference rejection performance and power consumption. For given low-power WUS signal bandwidth and receiver requirements on adjacent channel selectivity or adjacent subcarrier selectivity, the power consumption depends on the filter order, smaller filter order keeps lower power consumption but may require a larger guardband resulting in lower resource efficiency, larger filter order increases the power consumption linearly but only a smaller guardband is needed.

Therefore, the IF BPF and BB LPF filter can be studied by considering the bandwidth of low-power WUS, receiver requirements on adjacent channel selectivity or adjacent subcarrier selectivity, minimum guardband between channel carrying low-power WUS and adjacent channel, minimum guardband between low-power WUS subcarriers and adjacenet subcarriers, as well as power consumption budget.

Proposal 5    Study the followings related to the analog IF/BB filter, i.e., filter bandwidth by considering the bandwidth of low-power WUS, receiver requirements on adjacent channel selectivity or adjacent subcarrier selectivity, minimum guardband between channel carrying low-power WUS and adjacent channel, minimum guardband between low-power WUS subcarriers and adjacenet subcarriers, as well as power consumption budget.

  - ADC

ADC facilitates removing interference and noise effect further via digital processing, the power consumption of ADC depends on the resolution and sampling rate. As summarized in[20], the energy, i.e., power divided by sampling rate, exponentially increases when resolution increases. For example, in walden FoM(figure of merit), it takes 2 times enery per bit added in resolution, while in Schreier FoM, it takes 4 times enery per bit added in resolution.

On the other hand, the detection performance of low-power WUS by low-power WUR also relies on the resolution and sampling rate. The following is an example of the miss detection / false alarm performance of the multiple-bit ADC performance of the low-power wake up signal. It is oberserved that with sampling rate fixed at 3.84MHz, the performance of comparator (1-bit ADC) is quite poor in both scenarios with adjacent subcarrier interference (ASCI) and without ASCI. While the detection performance of  4-bit ADC approximately approach to that of 8-bit ADC. Therefore, in ADC design, the resolution and sampling rate should be determined based on both detection performance requirements and power consumption budget. The relevant simulation assumptions are shown in Appendix A.

Figure 8 Detection performance of low-power WUR with different ADC resolution: 1, 2 , 4, 8

*Note:ASCI in above figures means adjacent subcarrier interference. ADC bitwidth with X bits are maked as “X ADC”; ADC with 1 bit bitwidth is marked as ‘comparator’; ‘ideal’ ADC means no quantization.

Proposal 6    Study multi-bit ADC, i.e., resolution and sampling rate by considering both power consumption budget and detection performance.

- Conclusion

In this contribution, we provide our views on the performance metrics as well as the potential LP-WUR receiver architectures with detailed components and the achieved performance from existing literatures. The observations and  proposals are listed as below:

Observation 1  Design on low-power WUR architecture is a trade-off of power consumption, sensitivity and data rate.

Observation 2 Achievable sensitivity of low-power WUR should be investigated along with the supported data rate.

Observation 3  Due to demanding a band specific high-Q RF BPF, the receiver architecture with amplitude detection at RF is more suitable for devices supporting single band.

Observation 4  The reported sensitivity for receiver architecture with amplitude detection at RF in the literatures[5][6]is -56.5dBm~-75dBm with data rate serval kbps to hundred kbps under power consumption less than 1uw to tens of uw.

Observation 5  For heterodyne architecture with IF envelope detection, the power consumption can be reduced by replacing  a high accuracy LO with a medium accuracy LO, and the frequency offset caused by the frequency error of the LO can be further studied.

Observation 6  The reported sensitivity for heterodyne architecture with IF envelope detection in the literatures[7][8] is -83dBm~-97dBm with data rate tens of kbps to several Mbps under power consumption hundreds of uw.

Observation 7    For homodyne/zero-IF architecture with BB envelope detection,  low-power solution on flicker noise and DC offset issue should be studied.

Observation 8 The reported sensitivity for homodyne/zero-IF architecture with baseband envelope detection in the literature[9] is −92.6dBm with data rate tens of kbps to hundreds of kbps under power consumption hundreds of uw.

Observation 9  The relative levels of active power consumtion of receive architectures for OOK detection are 0.01x~0.1x unit for RF envelope detection,  0.1x~1x unit for heterodyne architecture with IF envelope detection, and 0.1x~1x unit for homodyne/zero-IF architecture with BB envelope detection.

Observation 10   Compared to single-subcarrier FSK, multi-subcarrier FSK is more robust to channel fluctuation.

Observation 11  2M frequencies or frequency sets are required to carry M bits for single-subcarrier FSK and multi-subcarrier FSK, respectively.

Observation 12  Frequency gaps are required among multiple frequencies or frequency sets for frequency differentiation .

Observation 13 For FSK receiver based on parallel OOK receivers, 2M analog bandpass filters and envelope detectors are needed for reception of FSK carrying M bits.

Observation 14 For FSK receiver based on parallel OOK receivers with a heterodyne or zero-IF architecture, the frequency gap between two adjacent frequencies or two adjacent frequency sets should be not smaller than two times of the max frequency offsets.

Observation 15 For FSK receiver based on parallel OOK receivers with a heterodyne or zero-IF architecture, at least two times of the max frequency offsets within the frequency gap shall not be used by other DL signals/channels or other WUS signals.

Observation 16 For FSK receiver based on parallel OOK receivers, interference rejection performance highly depends on  whether the interferences across 2M frequencies or frequency sets are coherent or not.

Observation 17  A high precise phase shifting network is necessary to discriminate frequency deviation.

Observation 18 The DC offset in analog quadrature FM discriminator deteriorates the detection performance.

Observation 19  A sensitivity level of around -70dBm with data rate several kbps under power consumption several milli watts is achieved by analog quadrature FM discriminator.

Observation 20  For FSK receiver based on frequency to amplitude conversion, it does not work for multi-subcarrier FSK detection.

Observation 21  For FSK receiver, it is difficult to utilize soft information in correlation-based sequence detection for FSK carrying M bits.

Observation 22  The relative levels of active power consumtion of receive architectures for FSK detection are 0.01x~0.1x unit for parallel RF/IF/BB envelop detection based receiver and 1x unit for frequency to amplitude conversion based receiver.

Observation 23  The hardware components of OFDM receiver with relaxed hardware requirements are typically different from those of OFDM receiver for main radio.

Observation 24  The reported values of OFDM receiver with low power consumption mainly fall into level of  tens of milliwatts.

Proposal 1: The main radio and low-power WUR exchange information between each other, such as

- Low-power WUR gets initial configurations from the main radio (received from gNB configuration)

- Low-power WUR can indicate ‘wake-up’ to the main radio

- Low-power WUR can pass additional decoded messages to the main radio, these messages are processed and parsed in the main radio but agnostics to the low-power WUR

Proposal 2:  Study the metric for representing the sensitivity at certain data rate for low-power WUR, e.g., the sensitivity normalized to data rate.

Proposal 3: OFDM receiver architecture for main radio is not considered for LP WUR due to high power consumption.

Proposal 4: When considering the reduced power consumption for a OFDM receiver, the receiver architecture with detailed functional blocks as well as the relaxed hardware requirements, e.g.,  frequency error, phase noise,  noise figure, I/Q mismatch, and etc.,  should be provided.

Proposal 5    Study the followings related to the analog IF/BB filter, i.e., filter bandwidth by considering the bandwidth of low-power WUS, receiver requirements on adjacent channel selectivity or adjacent subcarrier selectivity, minimum guardband between channel carrying low-power WUS and adjacent channel, minimum guardband between low-power WUS subcarriers and adjacenet subcarriers, as well as power consumption budget.

Proposal 6    Study multi-bit ADC, i.e., resolution and sampling rate by considering both power consumption budget and detection performance.

# References

- RP-222644	Revised SID: Study on low-power Wake-up Signal and Receiver for NR, vivo, RAN #94 e-meeting.

- RAN1 Chair’s Notes, RAN1 #111 e-meeting.

- Low power radio survey, https://wics.engin.umich.edu/ultra-low-power-radio-survey

- P.H.Wang, H.W. Jiang, L. Gao, " A Near-Zero-Power Wake-Up Receiver Achieving −69-dBm Sensitivity,“IEEE JOURNAL OF SOLID-STATE CIRCUITS, VOL. 53, NO. 6, JUNE 2018

- N.E.Roberts, et.al, “A 236nw -56.5dBm sensitivity Bluetooth low-energy wake up receiver with energy harvesting in 65nm CMOS,” IEEE ISSCC, 2016.

- X. Huang, S. Rampu, X. Wang, G. Dolmans and H. de Groot, “A 2.4GHz/915MHz 51µW wake-up receiver with offset and noise suppression,” 2010 IEEE International Solid-State Circuits Conference - (ISSCC), 2010.

- L. Jae-Seung, K. Joo-Myoung, L. Jae-Sup, et.al, “13.1 A 227pJ/b −83dBm 2.4GHz multi-channel OOK receiver adopting receiver-based FLL,” 2015 IEEE International Solid-State Circuits Conference - (ISSCC) Digest of Technical Papers, 2015.

- C. Salazar, A. Cathelin, A. Kaiser and J. Rabaey, “A 2.4 GHz Interferer-Resilient Wake-Up Receiver Using A Dual-IF Multi-Stage N-Path Architecture,”in IEEE Journal of Solid-State Circuits, vol. 51, no. 9, pp. 2091-2105, Sept. 2016.

- R. Liu et al., “An 802.11ba-based wake-up radio receiver with Wi-Fi transceiver integration”, IEEE J. Solid-State Circuits, vol. 55, no. 5, pp. 1151-1164, May 2020.

- Po-Chiun Huang, Yi-Huei Chen, et.al, “A 2-V CMOS 455 kHz FM/FSK demodulator using feedforward offset cancellation limiting amplifier,” Proceedings of Second IEEE Asia Pacific Conference on ASICs. AP-ASIC 2000.

- J. -H. C. Zhan et al., "A 55nm CMOS 4-in-1 (11b/g/n, BT, FM, and GPS) radio-in-a-package with IPD front-end components directly connected to antenna," 2014 IEEE Radio Frequency Integrated Circuits Symposium, Tampa, FL, USA, 2014, pp. 209-212.

- X. Yu et al., "A Sub-GHz low-power transceiver with PAPR-tolerant power amplifier for 802.11ah applications," 2015 IEEE Radio Frequency Integrated Circuits Symposium (RFIC), Phoenix, AZ, USA, 2015, pp. 231-234.

- Z. Song, X. Liu, X. Zhao, Q. Liu, Z. Jin and B. Chi, "A Low-Power NB-IoT Transceiver With Digital-Polar Transmitter in 180-nm CMOS," in IEEE Transactions on Circuits and Systems I: Regular Papers, vol. 64, no. 9, pp. 2569-2581, Sept. 2017.

- P. S. Tseng et al., "A 55nm SAW-Less NB-IoT CMOS Transceiver in an RF-SoC with Phase Coherent RX and Polar Modulation TX," 2019 IEEE Radio Frequency Integrated Circuits Symposium (RFIC), Boston, MA, USA, 2019, pp. 267-270.

- H. Guo, T. F. Chan, Y. T. Lai, K. C. Wan, L. Chen and W. P. Wong, "A SAW-Less NB-IoT RF Transceiver with Hybrid Polar and On-Chip Switching PA Supporting Power Class 3 Multi-Tone Transmission," 2020 IEEE International Solid- State Circuits Conference - (ISSCC), San Francisco, CA, USA, 2020, pp. 464-466.

- Lee et al., "NB-IoT and GNSS All-in-One System-on-Chip Integrating RF Transceiver, 23dBm CMOS Power Amplifier, Power Management Unit and Clock Management System for Low-Cost Solution," 2020 IEEE International Solid- State Circuits Conference - (ISSCC), San Francisco, CA, USA, 2020, pp. 462-464.

- S. Rostami, K. Heiska, O. Puchko, K. Leppanen and M. Valkama, "Novel Wake-up Scheme for Energy-Efficient Low-Latency Mobile Devices in 5G Networks," in IEEE Transactions on Mobile Computing, vol. 20, no. 4, pp. 1511-1528, 1 April 2021.

- T. J. Odelberg et al., "A 2.1mW −109dBm NB-IoT Wake-Up Receiver", IEEE RFIC, pp. 235-238, 2021.

- https://devzone.nordicsemi.com/power/w/opp/3/online-power-profiler-for-lte

- B. Murmann, "ADC Performance Survey 1997-2022," [Online]. Available: http://web.stanford.edu/~murmann/adcsurvey.html.

# Appendix A –Simulation assomptions

Table 1. Assumptions for link level simulation for LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2GHz |
| Channel structure | Preamble +data +CRC: 32 chips+ 32 bits +8 CRC bits |
| Coding | 1/2 rate Manchester coding (For information bits and CRC bits) |
| Data rate/ Raw Data rate | 224kbps (128kbps) |
| SCS | 30kHz |
| gNB Channel BW | 20MHz (50 RB) |
| WUS BW | 12RB ~4.32MHz |
| Guard band | 1RB on each side of LP-WUS bandwidth |
| Filter | 5th Order Butterworth with 4.32MHz bandwidth |
| ASCI | PDSCH mapped on RBs not used for LP-WUS and guard band; EPRE of LP-WUS vs EPRE of PDSCH = 1:1. |
| Sampling Rate | 4MHz |
| ADC | 1bit (comparator) 2bits, 4bits, 8bits, ideal ADC |
| Channel Model | AWGN |
| Performance metric | {FAR, MDR}: {0.1%, 1%} |