3GPP TSG RAN WG1 #112				                            R1-2300536

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.12.1

Source:	LG Electronics

Title:	Enhancements on inter-cell beam management for mobility

Document for:	Discussion and Decision

Introduction

In RAN#94 meeting, the objectives for further NR mobility enhancements were approved [1], which includes the following objectives related to L1 enhancements for inter-cell beam management and timing advance management as:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

In this contribution, we discuss potential enhancements on inter-cell beam management for mobility based on L1/L2 measurement/reporting.

- Discussion

In Rel-17 feMIMO WI, inter-cell beam management (BM) including beam measurement/reporting and beam indication was introduced based on unified TCI framework where the inter-cell BM allows UE to utilize SSBs from a serving cell with different PCID as QCL source for beam indication/update. However, in this case, the serving cell does not change and it has a limitation on UE mobility support and it leads that an RRC-based handover (HO) is inevitable once the UE moves out of serving cell coverage. In current NR system, gNB can determine whether to operate handover for UE to change the serving cell based on the measurement reporting from the UE and the HO procedure is shown in Figure 1 below.

Figure 1. Legacy signaling procedure for HO between gNB and UE

RRC reconfiguration (measurement) is for a new measurement target and related settings for HO to the UE while the UE is in the ‘connected’ mode. In this configuration, ReportConfigNR can be included where the IE specifies criteria for triggering of an NR measurement reporting events as below.

#### ReportConfigNR (TS38.331, clause 6.3.2) The IE ReportConfigNR specifies criteria for triggering of an NR measurement reporting event or of a CHO or CPC event. For events labelled AN with N equal to 1, 2 and so on, measurement reporting events and CHO or CPC events are based on cell measurement results, which can either be derived based on SS/PBCH block or CSI-RS. Event A1: Serving becomes better than absolute threshold; Event A2: Serving becomes worse than absolute threshold; Event A3: Neighbour becomes amount of offset better than PCell/PSCell; Event A4: Neighbour becomes better than absolute threshold; Event A5: PCell/PSCell becomes worse than absolute threshold1 AND Neighbour/SCell becomes better than another absolute threshold2; Event A6: Neighbour becomes amount of offset better than SCell; CondEvent A3: Conditional reconfiguration candidate becomes amount of offset better than PCell/PSCell; CondEvent A5: PCell/PSCell becomes worse than absolute threshold1 AND Conditional reconfiguration candidate becomes better than another absolute threshold2; For event I1, measurement reporting event is based on CLI measurement results, which can either be derived based on SRS-RSRP or CLI-RSSI. Event I1: Interference becomes higher than absolute threshold.

According to the setting and reporting criterion, the UE may perform an operation of reporting a RRM measurement value which is managed by L3, as shown in measurement reporting in Figure 1. Based on this, gNB gives the UE a HO command.

In RAN#111, the agreement was made on UE/event triggered report for L1 measurement as follows.

Agreement For L1 measurement report for Rel-18 L1/L2 mobility, if UE event triggered report for L1 measurement is supported based on further study - At least the following aspects may be considered - How to define UE event and exact definition of events, - Report container - Resource allocation/assignment for UE event triggered report - Necessity of indication to gNB when the condition UE event is met, and how - Necessity to define the condition to start/stop the reporting, - Contents of the report/reporting format, PCI, RS ID, measurement result etc. - The interaction with filtered L1 measurement results (if supported) - Support of simultaneous configuration of both UE event triggered and any of periodic/semi-persistence/aperiodic reporting, and solutions when both of them are configured. - Report destination, whether the report is sent to serving cell only or can be sent to one or more candidate cell(s). - Benefit when L3 measurement is involved

Regarding L1 reporting for inter-cell operation, event-based reporting analogous to L3 measurement and reporting procedure would be efficient since beam report for candidate cell would not always needed. Moreover, compared to the measurement reporting for RRM, it can be beneficial in terms of signalling overhead/latency and UE power consumption by considering high mobility UE in a dense cell environment. For this L1 event-based reporting, measurement RS group for serving/candidate cell can be configured and the defined triggering condition(s) based on the cell quality for measurement reporting can be replaced with the condition(s) based on L1 beam quality, e.g. for modified (beam quality based) Event A3 triggering in ReportConfigNR based on the corresponding RS groups for serving/candidate cell. When the event is occurred such as the best quality of the measurement RS in the measurement RS group for the candidate cell is better than that of the measurement RS group for the serving cell, serving cell beam report could be replaced by candidate cell beam report or candidate cell beam report can be added into the serving cell report. To this end, PUCCH and/or DG-/CG-PUSCH can be utilized as a report container. To be specific on PUCCH, PUCCH format 0/1 with a small number of bits can be used to indicate whether an event has occurred and the detailed information related to the event is sent by the subsequent PUSCH.

Proposal #1: Consider UE event-triggered L1-based beam reporting for candidate cells as follows

- Resource allocation/assignment

  - Measurement RS group for serving/candidate cell can be configured (including SSB and/or CSI-RS)

- Definition of the event, e.g.,

  - The best quality of the measurement RS in the measurement RS group for the candidate cell is better than that of the measurement RS group for the serving cell (via Modified Event A3 triggering)

- Report container and event indication (via PUCCH)

  - PUCCH and/or DG-/CG-PUSCH, e.g., using PUCCH format 0/1 with a small number of bits to indicate whether an event has occurred and the detailed information related to the event is sent by the subsequent PUSCH

Along with the above, the necessity of filtering L1 measurement for mobility robustness, i.e., ping-pong issue was also discussed in the last meeting by considering the aspects on UE-based filtering to the L1 measurement results and applicability to L1-RSRP and so on. When the HO event trigger condition is replaced from the L3 to the L1 perspective based on L1 measurement reporting only, the HO is performed more frequently due to the short term information on the beam quality value. To handle this issue, it can be considered to add/modify the threshold and offset for the beam quality as event triggering conditions in ReportConfigNR since the threshold and offset is based only on the cell quality from the L3 perspective in legacy. Also, it would be possible to check the event triggering condition according to a specific time period or a number of measurement samples. Regarding the specific time period, it can be based on ‘time to trigger’ operation in the existing HO operation where it checks whether the same triggering condition is satisfied in N times (i.e. reportAmount) consecutively in terms of beam quality value at a specific duration interval (i.e. reportInterval) for measurement reporting.

Proposal #2: Consider filtering L1 measurement results to mitigate ‘ping-pong’ issue by taking into account the followings.

  - Add or modify event triggering condition in ReportConfigNR with L1-based threshold/offset

  - Check event triggering condition in terms of beam quality based on ‘time to trigger’ operation

Regarding L1 measurement RS, the following agreement was made in RAN1#110bis-e where SSB is supported for L1 intra-/inter-frequency measurement.

Agreement For Rel-18 L1/L2 mobility, - SSB is supported for L1 intra-frequency measurement - SSB is supported for L1 inter-frequency measurement if inter-frequency L1 measurements are supported Further study the following L1 measurement RS for candidate cell - CSI-RS for tracking, beam management, CSI and mobility, CSI-IM, which is for L1 intra-frequency and L1 inter-frequency (if supported)

In RAN1#111, the introduction of CSI-RS was also discussed but did not make a progress due to the lack of time. Based on SSB to facilitate measurement and reporting of candidate cell RS, configuring mobility CSI-RS as a reference RS in QCL information can be beneficial because the mobility CSI-RS can have a relatively narrower beam and a more flexible configuration compared to those of SSB.

Proposal #3. Support configuring mobility CSI-RS as a reference RS in QCL information for LTM.

Conclusion

In this contribution, we have discussed the potential enhancements on inter-cell beam management for mobility based on L1/L2 measurement/reporting. Based on the discussions above, following proposal is given as:

Proposal #1: Consider UE event-triggered L1-based beam reporting for candidate cells as follows

- Resource allocation/assignment

  - Measurement RS group for serving/candidate cell can be configured (including SSB and/or CSI-RS)

- Definition of the event, e.g.,

  - The best quality of the measurement RS in the measurement RS group for the candidate cell is better than that of the measurement RS group for the serving cell (via Modified Event A3 triggering)

- Report container and event indication (via PUCCH)

  - PUCCH and/or DG-/CG-PUSCH, e.g., using PUCCH format 0/1 with a small number of bits to indicate whether an event has occurred and the detailed information related to the event is sent by the subsequent PUSCH

Proposal #2: Consider filtering L1 measurement results to mitigate ‘ping-pong’ issue by taking into account the followings.

  - Add or modify event triggering condition in ReportConfigNR with L1-based threshold/offset

  - Check event triggering condition in terms of beam quality based on ‘time to trigger’ operation

- Proposal #3. Support configuring mobility CSI-RS as a reference RS in QCL information for LTM.

- References

[1] RP-213565	New WID on Further NR mobility enhancements			MediaTek