3GPP TSG RAN WG1 #112			R1-2300573

Athens, Greece, February 27th – March 3rd, 2023

Source:             Xiaomi

Title:			   Discussion on evaluation on NR duplex

Agenda item:     9.3.1

Document for:   Discussion

Introduction

In RAN1#111 meeting, we further discussed issues related to evaluation on NR duplex evolution [1]. The detailed agreements can be found in the appendix.

In this contribution, we share views on deployment scenarios and methodology. Furthermore, we provide preliminary SLS evaluation results based on the current assumptions.

Discussion

Remaining issues on SLS evaluation

Channel model and penetration loss

SBFD Deployment Case 1: Urban Macro and Dense Urban

Penetration loss for UE-to-UE link

In previous meetings, the channel model for UE-to-UE link has been agreed for SBFD Deployment Case 1. Besides, the UE distribution has also been discussed. In this contribution, we further discuss the penetration loss for UE-to-UE link when UE clustering distribution is applied. The relevant agreements for UE-to-UE channel modeling are shown as below.

Working Assumption in 110 For UE-UE channel model, reuse the UE-UE channel model for flexible duplex evaluation in TR 38.802 for both FR1 and FR2 as baseline, and other models are not precluded. UE-UE channel model Dense urban, Urban macro; Indoor hotspot. Large-scale channel parameters; FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843(*), penetration loss between UEs follows Table A.2.1-13 in TR38.802 - Option 2: UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-13 in TR38.802 FR2-1: - UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 in TR38.802; FR1: - Option1 : UE-to-UE: A.2.1.2 in TR36.843 (*) - Option 2: UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) FR2-1: - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m). Fast fading parameters; FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843 (ITU InH) for indoor to indoor, and 3D UMi for other cases. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Optioin 2: UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA. FR2-1: - UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA.; FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843 (ITU InH), ASD statistics updated to be the same as ASA. - Option2: UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA FR2-1: - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA. (*): For outdoor to indoor case, and indoor to indoor case, use “Remaining Layout Options” in A.2.1.2 of TR36.843 for pathloss calculation, and “ITU-R IMT UMi” for LOS Probability derivation. For outdoor to indoor case, the penetration loss term “20.0+0.5* din” is excluded in pathloss formula given in A.2.1.2 of TR36.843, and the penetration loss is derived according to Table A.2.1-13 in TR38.802.. Agreement in 110 Update the previous agreement as below: For UE distribution of Urban Macro and Dense Urban Macro layer, - Baseline: (UE clustering at least for FR1) - M users per macro TRP - Step 1: Randomly drop X UE cluster centers within one macro cell geographical area considering the minimum distance between macro TRP to UE cluster center as Dmacro-to-cluster and the minimum distance between two UE cluster centers as Dinter-cluster - Step 2: Y% UEs are randomly and uniformly dropped within the UE clusters with the radius of R, (1-Y%) users randomly and uniformly dropped in the macro geographical area outside the clusters - Note: UEs dropped within the UE cluster(s) are indoor with 3km/h; UEs dropped outside the UE cluster(s) are outdoor in car with 30km/h - UE outdoor/indoor proportion: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - Outdoor UEs: 1.5 m; - FFS: Indoor UEs height - Y%=80% - FFS the values of M, X, Dmacro-to-cluster, Dinter-cluster, R - Optional: - 10 users per macro TRP (per direction), and all users are randomly and uniformly dropped within the macro cell - At least for FR1: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - Outdoor UEs: 1.5 m; - Indoor UEs: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) [refer to TR 36.873 Table 6-1] - FFS: FR2 details Agreement in 110b Confirm the working assumption for UE-UE channel model made in RAN1#110. Agreement in 111 For UE-UE channel model for FR1, the penetration losses between UEs are updated to follow Table A.2.1-12 in TR38.802 Agreement in 110b When UE clustering distribution is used, - consider the UEs in the same cluster are in the same building - For Alt-2 (M=20, X=2), consider the UEs in different clusters are in different buildings Agreement in 111 To support UE clustering distribution for Dense Urban Macro layer for FR2-1 - Option 1: UE clustering distribution, 100% outdoor UEs without car penetration loss (3km/h) - the same UE clustering method as for FR1 is applied except that all the UEs (including UEs in the clusters and out of the clusters) are outdoor UEs without car penetration loss (3km/h). - R=20m, M=10, X=1 - X=2 and M=20 can also be evaluated. - Other values of R can be evaluated. - UE height is 1.5m for all UEs - For any other aspects, reuse what was agreed for FR1 UE clustering is new baseline for Dense Urban Macro layer for FR2-1.

It has been agreed that for UE-UE channel model for FR1, the penetration losses between UEs are updated to follow Table A.2.1-12 in TR38.802. In Table A.2.1-12 in TR38.802, the distance between two indoor UEs is used to determine whether two UEs are in the same building or not. If inter-user 2D distance is larger than 50m, two indoor UEs are in different building, and vice versa.

However, inter-user 2D distance is not suitable to determine whether two indoor UEs are in the same building or not in the case of UE clustering distribution Alt 2 (M=20, X=2) in FR1. On the other hand, UE clustering distribution Alt 2 (M=20, X=2) is considered as baseline for Urban Macro and Dense Urban Macro layer in FR1 and as optional for Dense Urban Macro layer in FR2-1. For Alt-2 (M=20, X=2), it has been agreed that the UEs in different clusters are in different buildings. Considering that the minimum distance between two clusters is 50m in FR1 and 40m in FR2-1, it is possible that the UE#A and UE#B are in cluster#1 and cluster#2, respectively (in different buildings), and inter-user 2D distance between UE#A and UE#B is less than 50m (UE#A and UE#B are in same building according to Table A.2.1-12 in TR38.802).

One example of UE clustering distribution for Urban Macro in FR1 is shown in Figure 1. It can be observed that inter-user 2D distance between UE#A and UE#B is less than 50m but UE#A and UE#B are in different clusters. Correspondingly, different criteria of determining whether different UEs locates in the same building or not will certainly impacts the calculation of penetration loss. To better understand the influence coming with different criteria, we provides the CDF curves for the following two cases in Figure 2:

- Case 1) UEs are in same building if inter-user 2D distance is less than 50m;

- Case 2) UEs are in same building if they are in the same cluster.

Simulation results shows that about 6% reduction of 0 penetration loss is observed in case 2). Hence it will diverge the final results among companies.

Figure 1 UE clustering distribution for Urban Macro in FR1.

Figure 2 Inter-UE penetration in two cases: 1) UEs are in same building if inter-user 2D distance is less than 50m; 2) UEs are in same building if they are in the same cluster.

Thus, the penetration losses between two indoor UEs should be updated when UE clustering distribution Alt 2 (M=20, X=2) is considered for Urban Macro and Dense Urban Macro layer in FR1. Two indoor UEs are in the same building only when they are in the same cluster.

When UE clustering distribution Alt 2 (M=20, X=2) is considered for Urban Macro and Dense Urban Macro layer in FR1, the penetration losses between two indoor UEs follow Table A.2.1-12 in TR38.802 with the following updates:

- Two indoor UEs are in the same building when they are in the same cluster.

- Two indoor UEs are in different buildings when they are in different clusters.

SBFD Deployment Case 3-2: 2-layer scenario B

gNB-to-UE link

The agreements related to UE distribution and gNB-UE channel modelling for 2-layer Scenario B are summarized as below:

Agreement in RAN1#111 Regarding the UE distribution of 2-layer Scenario B (HetNet with Urban Macro and Indoor), - Layer 1: Urban Macro - 10 users per macro TRP per direction, and all users are randomly and uniformly dropped within the macro cell outside the Indoor office / Indoor factory - Indoor/outdoor proportion: - Option 1 (baseline): 100% outdoor without car penetration loss, 3km/h, UE height is 1.5m - Option 2 (optional): - 20% outdoor in cars: 30km/h, UE height is 1.5m - 80% indoor in houses: 3km/h, UE height is 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) - Layer 2: Indoor office (baseline) - 10 users per indoor TRP per direction, and all users are randomly and uniformly dropped within the building. - UE speed is 3km/h, UE height is 1.5m - Minimum UE-UE (2D) distance: 1m - Minimum BS-UE (2D) distance: 0m - UE selected macro TRP or indoor TRP is determined based on the RSRP, i.e., the UE in the indoor office/indoor factory can select the Macro cell as serving cell, and the UE outside the indoor office/indoor factory can select the indoor TRP as serving cell Agreement in RAN1#110b Adopt the following table for gNB-UE channel model for 2-layer Scenario B (HetNet with Urban Macro and Indoor). gNB-UE channel model for 2-layer Scenario B. Large-scale channel parameters; FR1: - Macro TRP to Outdoor UE: UMa in TR 38.901 - Car penetration loss is modelled - Indoor TRP to Indoor UE: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - Penetration loss is not modelled. - Macro TRP to Indoor UE: UMa in TR 38.901 - O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - Indoor TRP to Outdoor UE: - Option 1: - A.2.1.2 in TR36.843 - Penetration loss between UEs follows Table A.2.1-13 in TR38.802 - Option 2: - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1] - For Indoor factory layer: InF in TR 38.901 - Both Car penetration (for outdoor UE) and O2I penetration loss are modelled, wherein, O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered.. Fast fading parameters; FR1: - Macro TRP to Outdoor UE: UMa in TR 38.901 - Indoor TRP to Indoor UE: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - Macro TRP to Indoor UE: UMa in TR 38.901 - Indoor TRP to Outdoor UE: - Option 1: - 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1] - For Indoor factory layer: InF in TR 38.901.

In RAN1#111 meeting, it was agreed that the serving macro TRP or indoor TRP is determined based on the RSRP, i.e., the UE in the indoor office/indoor factory can select the Macro cell as serving cell, and the UE outside the indoor office/indoor factory can select the indoor TRP as serving cell. If UE#A locating in the house, i.e. assuming Indoor/outdoor proportion option 2 is used and it is an indoor UE, is close to the indoor TRP and is far away from the macro TRP, it is possible that UE#A will select indoor TRP as serving cell based on the RSRP. In this situation, indoor TRP and UE#A are in different buildings. However, channel model for indoor TRP to indoor UE is considered only when the indoor TRP and indoor UE are in the same building. Thus, channel model for indoor TRP to indoor UE in different buildings should also be included.

Figure 3 Indoor UE outside the office belongs to the indoor TRP based on RSRP.

Channel model for indoor TRP to indoor UE in different buildings should also be considered in 2-layer Scenario B when option 2 is assumed for Indoor/outdoor UE proportion on Urban Macro layer.

As channel model for indoor TRP to outdoor UE has been approved in RAN1#110b-e for 2-layer Scenario B, it could be reused for indoor TRP to indoor UE in different buildings with modifying penetration loss accordingly.

In TR 38.901, it is mentioned that only the high-loss model is applicable to InF. If indoor factory is considered in 2-layer scenario B, the penetration loss for indoor TRP or indoor UE in the factory should use 100% high-loss model. Thus, the Large-scale channel of indoor TRP to indoor UE in different buildings can be formulated as following.

- Indoor TRP to indoor UE in different buildings:

  - Option 1:

    - A.2.1.2 in TR36.843

    - Penetration loss between indoor TRP and indoor UE in different buildings follows Table A.2.1-12 in TR38.802

  - Option 2:

    - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1]

    - For Indoor factory layer: InF in TR 38.901

    - Two O2I penetration losses are modelled, wherein, O2I penetration loss follows TR 38.901

    - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered for each O2I penetration loss for indoor office layer.

    - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered for O2I penetration loss of indoor UE and 100% high-loss model is considered for O2I penetration loss of indoor factory TRP for indoor factory layer.

Channel model from serving indoor TRP to its facilitated UE in different buildings could reuse the channel model for indoor TRP to outdoor UE with modification of penetration loss.

- Indoor TRP to indoor UE in different buildings:

  - Option 1:

    - A.2.1.2 in TR36.843

    - Penetration loss between indoor TRP and indoor UE in different buildings follows Table A.2.1-12 in TR38.802

  - Option 2:

    - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1]

    - For Indoor factory layer: InF in TR 38.901

    - Two O2I penetration losses are modelled, wherein, O2I penetration loss follows TR 38.901

    - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered for each O2I penetration loss for indoor office layer.

    - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered for O2I penetration loss of indoor UE and 100% high-loss model is considered for O2I penetration loss of indoor factory TRP for indoor factory layer.

For Large-scale channel model of InH-Office in TR 38.901, d2D-in is used in the LOS probability calculation. Considering that indoor TRP and indoor UE are in the same building, LOS probability should be related to the 2D distance between indoor TRP and indoor UE, i.e., d2D. However, d2D-in is defined as the distance between indoor UE and the wall of building, which is not suitable for LOS probability calculation of InH-Office. Hence, d2D-in should be interpreted as 2D distance between Indoor TRP and indoor UE in the same building.

Indoor-Mixed office: PrLOS=1,d2D-in≤1.2mexp-d2D-in-1.24.71.2m<d2D-in<6.5mexp-d2D-in-6.532.6∙0.326.5m≤d2D-in.

Indoor-Open office:  PrLOS=1,d2D-in≤5mexp-d2D-in-570.85m<d2D-in<49mexp-d2D-in-49211.7∙0.5449m≤d2D-in.

Channel model of InH-Office in TR 38.901 could also be used for Indoor TRP to Outdoor UE and Indoor TRP to indoor UE in different buildings. It is complex to figure out new LOS probability models for these two new links. For simplicity, same LOS probability model could be used for indoor TRP to indoor UE in the same building, indoor TRP to outdoor UE and indoor TRP to indoor UE in different buildings.

Figure 4 d2D-in for Indoor TRP to Outdoor UE, Indoor TRP to Indoor UE in the same building and Indoor TRP and Indoor UE in different buildings.

For Indoor TRP to indoor UE in the same building, indoor TRP to outdoor UE and indoor TRP to indoor UE in different buildings, the formula of calculating LOS probability is updated as below:

Indoor-Mixed office: PrLOS=1,d2D≤1.2mexp-d2D-1.24.71.2m<d2D<6.5mexp-d2D-6.532.6∙0.326.5m≤d2D.

Indoor-Open office:  PrLOS=1,d2D≤5mexp-d2D-570.85m<d2D<49mexp-d2D-49211.7∙0.5449m≤d2D

Where d2D is 2D distance between indoor TRP and its facilitated UE.

UE-to-UE link

Based on the agreements as mentioned in section 2.1.1.2.1, only the channel between Indoor UEs within the same building needs to be modelled. Considering the UE distribution option 2 of Layer 1 (Macro Layer), there are 80% indoor UEs in the house. In this situation, it is possible that one indoor UE in the house and another indoor UE in the office. They are both indoor UEs but certainly locates in different buildings. The rules of determining whether two indoor UEs are in the same building should be clarified.

- Two indoor UEs are in the same building:

  - Two indoor UEs are both in the indoor office/factory.

  - Two indoor UEs are both in the houses (not in the indoor office) and distance is less than 50m (according to Table A.2.1-12 in TR38.802).

- Two indoor UEs are in different buildings:

  - One indoor UE is in the indoor office/factory, another indoor UE is in the house (not in the indoor office).

  - Two indoor UEs are both in the houses (not in the indoor office) and distance is larger than 50m (according to Table A.2.1-12 in TR38.802).

Figure 5 The determination of whether two indoor UEs are in the same building or not.

It should be clarified on whether two indoor UEs are in the same building if UE distribution option 2 is adopted for 2-layer scenario B.

- Two indoor UEs are in the same building:

  - Two indoor UEs are both in the indoor office/factory.

  - Two indoor UEs are both in the houses (not in the indoor office) and distance is less than 50m (according to Table A.2.1-12 in TR38.802).

- Two indoor UEs are in different buildings:

  - One indoor UE is in the indoor office/factory, another indoor UE is in the house (not in the indoor office).

  - Two indoor UEs are both in the houses (not in the indoor office) and distance is larger than 50m (according to Table A.2.1-12 in TR38.802).

Penetration loss and other issues

Some typos are observed in the gNB-UE channel model for 2-layer Scenario B.

- Large-scale channel parameters of Option 1 for Indoor TRP to Outdoor UE, the model of penetration should be updated as: Penetration loss between indoor TRP and outdoor UE follows Table A.2.1-12 in TR38.802.

- Small-scale channel parameters of Option 1 for Indoor TRP to Outdoor UE, ASD and ZSD statistics updated to be the same as ASA and ZSA are not needed since the heights of Indoor TRP and Outdoor UE are different.

Besides, consider the 2-layer scenario B with indoor factory, the penetration loss for indoor TRP or indoor UE in the factory should use 100% high-loss model. Previous agreements for gNB-UE, gNB-gNB and UE-UE channel model for 2-layer Scenario B could be updated accordingly.

Update the previous agreements for gNB-UE, gNB-gNB and UE-UE channel model for 2-layer Scenario B as below:

- Adopt the following table for gNB-UE channel model for 2-layer Scenario B (HetNet with Urban Macro and Indoor).

|  | gNB-UE channel model for 2-layer Scenario B |
| --- | --- |
| Large-scale channel parameters | FR1: - Macro TRP to Outdoor UE: UMa in TR 38.901 - Car penetration loss is modelled - Indoor TRP to Indoor UE in the same building: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - Penetration loss is not modelled. - Macro TRP to Indoor UE: UMa in TR 38.901 - O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - 100% high-loss model is considered if UE is in the factory. - Indoor TRP to Outdoor UE: - Option 1: - A.2.1.2 in TR36.843 - Penetration loss between indoor TRP and outdoor UE UEs follows Table A.2.1-12 A.2.1-13 in TR38.802 - Option 2: - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1] - For Indoor factory layer: InF in TR 38.901 - Both Car penetration (for outdoor UE) and O2I penetration loss are modelled, wherein, O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - 100% high-loss model is considered if Indoor TRP is in the factory. - Indoor TRP to indoor UE in different buildings: - Option 1: - A.2.1.2 in TR36.843 - Penetration loss between indoor TRP and indoor UE in different buildings follows Table A.2.1-12 in TR38.802 - Option 2: - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1] - For Indoor factory layer: InF in TR 38.901 - Two O2I penetration losses are modelled, wherein, O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered for each O2I penetration loss for indoor office layer. - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered for O2I penetration loss of indoor UE and 100% high-loss model is considered for O2I penetration loss of indoor factory TRP for indoor factory layer. |
| Fast fading parameters | FR1: - Macro TRP to Outdoor UE: UMa in TR 38.901 - Indoor TRP to Indoor UE in the same building: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - Macro TRP to Indoor UE: UMa in TR 38.901 - Indoor TRP to Outdoor UE and Indoor TRP to Indoor UE in different buildings: - Option 1: - 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1] - For Indoor factory layer: InF in TR 38.901 |

Adopt the following table for gNB-gNB channel model for 2-layer Scenario B (HetNet with Urban Macro and Indoor).

|  | gNB-gNB channel model for 2-layer Scenario B |
| --- | --- |
| Large-scale channel parameters | FR1: - Macro TRP to Macro TRP: not needed. - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered - For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). - For Indoor factory layer: InF in TR 38.901 (hUE =3m). - Penetration loss is not modelled. - Macro TRP to Indoor TRP: UMa in TR 38.901 (hUE =3m) - O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - 100% high-loss model is considered if Indoor TRP is in factory. - Indoor TRP to Macro TRP: same as Macro TRP to Indoor TRP |
| Fast fading parameters | FR1: - Macro TRP to Macro TRP: not needed. - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered. - For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). ASA and ZSA statistics updated to be the same as ASD and ZSD. - For Indoor factory layer: InF in TR 38.901 (hUE =3m). ASA and ZSA statistics updated to be the same as ASD and ZSD - Macro TRP to Indoor TRP: UMa O2I in TR 38.901 - Indoor TRP to Macro TRP: same as Macro TRP to Indoor TRP |

Adopt the following table for UE-UE channel model for 2-layer Scenario B (HetNet with Urban Macro and Indoor).

|  | UE-UE channel model for 2-layer Scenario B |
| --- | --- |
| Large-scale channel parameters | FR1: - Outdoor UE to Outdoor UE: - Option 1: A.2.1.2 in TR36.843 (*) - Option 2: UMi-Street canyon in TR 38.901 (hBS =1.5m) - Penetration loss between UEs follows Table A.2.1-13 A.2.1-12 in TR38.802 - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 1: A.2.1.2 in TR36.843 (*). - Option 2: - For Indoor office layer: InH-Office in TR 38.901 (hBS =1.5m). - For Indoor factory layer: InF in TR 38.901 (hBS =1.5m). - Penetration loss is not modelled. - Outdoor UE to Indoor UE: - Option 1: A.2.1.2 in TR36.843 (*). - Option 2: UMi-Street canyon in TR 38.901 (hBS =1.5m). - Penetration loss between UEs follows Table A.2.1-13 A.2.1-12 in TR38.802 - 100% high-loss model is considered if Indoor UE is in factory |
| Fast fading parameters | FR1: - Outdoor UE to Outdoor UE: - Option 1: 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: UMi-Street canyon in TR 38.901, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 1: A.2.1.2 in TR36.843 (ITU InH), ASD statistics updated to be the same as ASA. - Option 2: - For Indoor office layer: InH-Office in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. - For Indoor factory layer: InF in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Outdoor UE to Indoor UE: - Option 1: 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: UMi-Street canyon in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. |
| (*): For outdoor to indoor case, and indoor to indoor case, use “Remaining Layout Options” in A.2.1.2 of TR36.843 for pathloss calculation, and “ITU-R IMT UMi” for LOS Probability derivation. For outdoor to indoor case, the penetration loss term “20.0+0.5* din” is excluded in pathloss formula given in A.2.1.2 of TR36.843, and the penetration loss is derived according to Table A.2.1-13 A.2.1-12 in TR38.802. |  |

Penetration loss for UE-UE channel model

Determination of whether two indoor UEs are in the same building

The determination of whether two indoor UEs are in the same building for UE-UE penetration loss calculation is different for UE clustering distribution and uniform UE distribution in Urban Macro and Dense Urban Macro layer in FR1. Besides, the determination differs in InH, Dense Urban with 2-layer scenario in FR1 and 2-layer scenario B. The following criterion could be considered.

- For Urban Macro and Dense Urban Macro layer in FR1,

  - If UE clustering distribution is used, two indoor UEs are in the same building when they are in the same cluster and two indoor UEs are in different buildings when they are in different clusters.

  - If uniform UE distribution is used, two indoor UEs are in the same building when inter-UE d2D≤50m and two indoor UEs are in different buildings when inter-UE d2D>50m.

- For Dense Urban Macro layer in FR2-1,

  - If UE clustering distribution is used, all UEs are outdoor UE.

  - If uniform UE distribution is used, two indoor UEs are in the same building when inter-UE d2D≤50m and two indoor UEs are in different buildings when inter-UE d2D>50m.

- For InH in FR1/FR2-1, all UEs are in the same building.

- For 2-layer scenario B in FR1,

  - For baseline UE distribution, all users are outdoor UEs without car penetration for Urban Macro Layer. All users are randomly and uniformly dropped within the building for Indoor office layer. All indoor UEs are in the same building.

  - For optional UE distribution, 80% indoor in houses and 20% outdoor in cars are considered for Urban Macro Layer. All users are randomly and uniformly dropped within the building for Indoor office layer.

    - Two indoor UEs are in the same building:

      - Two indoor UEs are both in the indoor office/factory.

      - Two indoor UEs are both in the houses (not in the indoor office) and inter-UE d2D≤50m.

    - Two indoor UEs are in different buildings:

      - One indoor UE is in the indoor office/factory, another indoor UE is in the house (not in the indoor office).

      - Two indoor UEs are both in the houses (not in the indoor office) and inter-UE d2D>50m.

- For Dense Urban with 2-layer in FR1, two indoor UEs are in the same building when inter-UE d2D≤50m and two indoor UEs are in different buildings when inter-UE d2D>50m.

The following criterion could be used to determine whether two indoor UEs are in the same building or not for UE-UE penetration loss calculation:

- For Urban Macro and Dense Urban Macro layer in FR1,

  - If UE clustering distribution is used, two indoor UEs are in the same building when they are in the same cluster and two indoor UEs are in different buildings when they are in different clusters.

  - If uniform UE distribution is used, two indoor UEs are in the same building when inter-UE d2D≤50m and two indoor UEs are in different buildings when inter-UE d2D>50m.

- For Dense Urban Macro layer in FR2-1,

  - If UE clustering distribution is used, all UEs are outdoor UE.

  - If uniform UE distribution is used, two indoor UEs are in the same building when inter-UE d2D≤50m and two indoor UEs are in different buildings when inter-UE d2D>50m.

- For InH in FR1/FR2-1, all UEs are in the same building.

- For 2-layer scenario B in FR1,

  - For baseline UE distribution, all users are outdoor UEs without car penetration for Urban Macro Layer. All users are randomly and uniformly dropped within the building for Indoor office layer. All indoor UEs are in the same building.

  - For optional UE distribution, 80% indoor in houses and 20% outdoor in cars are considered for Urban Macro Layer. All users are randomly and uniformly dropped within the building for Indoor office layer.

    - Two indoor UEs are in the same building:

      - Two indoor UEs are both in the indoor office/factory.

      - Two indoor UEs are both in the houses (not in the indoor office) and inter-UE d2D≤50m.

    - Two indoor UEs are in different buildings:

      - One indoor UE is in the indoor office/factory, another indoor UE is in the house (not in the indoor office).

      - Two indoor UEs are both in the houses (not in the indoor office) and inter-UE d2D>50m.

- For Dense Urban with 2-layer in FR1, two indoor UEs are in the same building when inter-UE d2D≤50m and two indoor UEs are in different buildings when inter-UE d2D>50m.

Value of d2D-in in penetration loss calculation

O2I building penetration loss contains Path loss through external wall, Indoor loss PLin and Standard deviation in TR 38.901. Indoor loss is determined by d2D-in, i.e. PLin=0.5d2D-in, where d2D-in is the 2D distance between TRP/UE and the wall. d2D-in is minimum value of two independently variables generated via uniform distribution between 0 and 25 m for UMa and UMi-Street Canyon, i.e., d2D-in=min{U1(0, 25m), U2(0, 25m)}.

The above determination of d2D-in is suitable for gNB-UE channel model since Minimum BS-UE (2D) distance for Macro layer is 35m. However, for UE-UE channel model, d2D≥1m is assumed and d2D-in might be larger than d2D, which doesn’t make sense. Considering the case that two indoor UEs are in different building, the distance between indoor UE#A and wall is d2D-in#1 and distance between indoor UE#B and wall is d2D-in#2. To ensure d2D>d2D-in#1+d2D-in#2, one solution is to generate d2D-in based on the minimum of 25m and d2D/2, i.e, d2D-in=min{U1(0, min(d2D2,25m)), U2(0, min(d2D2,25m))}.

For O2I building penetration loss of UE-UE channel model, PLin=0.5d2D-in, where d2D-in is updated as below:

- d2D-in=min{U1(0, min(d2D2,25m)), U2(0, min(d2D2,25m))} and d2D is inter-UE 2D distance.

LOS probably for UE-UE channel model

Working Assumption in RAN1#110 For UE-UE channel model, reuse the UE-UE channel model for flexible duplex evaluation in TR 38.802 for both FR1 and FR2 as baseline, and other models are not precluded. UE-UE channel model Dense urban, Urban macro; Indoor hotspot. Large-scale channel parameters; FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843(*), penetration loss between UEs follows Table A.2.1-13 in TR38.802 - Option 2: UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-13 in TR38.802 FR2-1: - UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 in TR38.802; FR1: - Option1 : UE-to-UE: A.2.1.2 in TR36.843 (*) - Option 2: UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) FR2-1: - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m). Fast fading parameters; FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843 (ITU InH) for indoor to indoor, and 3D UMi for other cases. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Optioin 2: UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA. FR2-1: - UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA.; FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843 (ITU InH), ASD statistics updated to be the same as ASA. - Option2: UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA FR2-1: - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA. (*): For outdoor to indoor case, and indoor to indoor case, use “Remaining Layout Options” in A.2.1.2 of TR36.843 for pathloss calculation, and “ITU-R IMT UMi” for LOS Probability derivation. For outdoor to indoor case, the penetration loss term “20.0+0.5* din” is excluded in pathloss formula given in A.2.1.2 of TR36.843, and the penetration loss is derived according to Table A.2.1-13 in TR38.802.. Agreement in RAN1#110b Adopt the following table for UE-UE channel model for 2-layer Scenario B (HetNet with Urban Macro and Indoor). UE-UE channel model for 2-layer Scenario B. Large-scale channel parameters; FR1: - Outdoor UE to Outdoor UE: - Option 1: A.2.1.2 in TR36.843 (*) - Option 2: UMi-Street canyon in TR 38.901 (hBS =1.5m) - Penetration loss between UEs follows Table A.2.1-13 in TR38.802 - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 1: A.2.1.2 in TR36.843 (*). - Option 2: - For Indoor office layer: InH-Office in TR 38.901 (hBS =1.5m). - For Indoor factory layer: InF in TR 38.901 (hBS =1.5m). - Penetration loss is not modelled. - Outdoor UE to Indoor UE: - Option 1: A.2.1.2 in TR36.843 (*). - Option 2: UMi-Street canyon in TR 38.901 (hBS =1.5m). - Penetration loss between UEs follows Table A.2.1-13 in TR38.802. Fast fading parameters; FR1: - Outdoor UE to Outdoor UE: - Option 1: 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: UMi-Street canyon in TR 38.901, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 1: A.2.1.2 in TR36.843 (ITU InH), ASD statistics updated to be the same as ASA. - Option 2: - For Indoor office layer: InH-Office in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. - For Indoor factory layer: InF in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Outdoor UE to Indoor UE: - Option 1: 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: UMi-Street canyon in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA.. (*): For outdoor to indoor case, and indoor to indoor case, use “Remaining Layout Options” in A.2.1.2 of TR36.843 for pathloss calculation, and “ITU-R IMT UMi” for LOS Probability derivation. For outdoor to indoor case, the penetration loss term “20.0+0.5* din” is excluded in pathloss formula given in A.2.1.2 of TR36.843, and the penetration loss is derived according to Table A.2.1-13 in TR38.802..

The channel model of UE-to-UE link is modeled as UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m) in SBFD Deployment Case 1 for Dense urban and Urban macro in FR1 and FR2-1 and in SBFD Deployment Case 3-2: 2-layer scenario B. In UMi-Street canyon in TR 38.901, the LOS probability is calculated as follows.

PrLOS=1,d2D-out≤18m18d2D-out+exp-d2D-out361-18d2D-out,18m<d2D-out.

d2D-out can be interpreted as follows:

- For outdoor UE to outdoor UE and indoor UE to indoor UE in the same building, d2D-out= d2D.

- For indoor UE to outdoor UE and outdoor UE to indoor UE, d2D-out= d2D-d2D-in.

- For indoor UE to indoor UE in different building, d2D-out= d2D-d2D-in#1-d2D-in#2, where d2D-in#1 is distance between indoor UE#A and the wall of building#1 and d2D-in#2 is distance between indoor UE#B and the wall of building#2.

When channel model of UMi-Street canyon in TR 38.901 is used for UE-to-UE link, d2D-out can be interpreted as follow in the LOS probability calculation:

- For outdoor UE to outdoor UE and indoor UE to indoor UE in the same building, d2D-out= d2D.

- For indoor UE to outdoor UE and outdoor UE to indoor UE, d2D-out= d2D-d2D-in.

- For indoor UE to indoor UE in different building, d2D-out= d2D-d2D-in#1-d2D-in#2, where where d2D-in#1 is distance between indoor UE#A and the wall of building#1 and d2D-in#2 is distance between indoor UE#B and the wall of building#2.

The channel model of UE-to-UE channel model is modeled as InH-Office in TR 38.901 (hBS =1.5m) in SBFD Deployment Case 1 for Indoor hotspot in FR1 and FR2-1 and in SBFD Deployment Case 3-2: 2-layer scenario B (indoor UE to indoor UE in the same building). Similar to the LOS probability calculation for Indoor TRP to indoor UE in the same building,  the LOS probability can be calculated as follows.

Indoor-Mixed office: PrLOS=1,d2D≤1.2mexp-d2D-1.24.71.2m<d2D<6.5mexp-d2D-6.532.6∙0.326.5m≤d2D.

Indoor-Open office:  PrLOS=1,d2D≤5mexp-d2D-570.85m<d2D<49mexp-d2D-49211.7∙0.5449m≤d2D

Where d2D is inter-UE 2D distance.

When channel model of InH-Office in TR 38.901 is used for UE-to-UE link, the LOS probability is calculated as follows.

Indoor-Mixed office: PrLOS=1,d2D≤1.2mexp-d2D-1.24.71.2m<d2D<6.5mexp-d2D-6.532.6∙0.326.5m≤d2D.

Indoor-Open office:  PrLOS=1,d2D≤5mexp-d2D-570.85m<d2D<49mexp-d2D-49211.7∙0.5449m≤d2D

Common Issue for penetration loss

Agreement in 111 To support UE clustering distribution for Dense Urban Macro layer for FR2-1 - Option 1: UE clustering distribution, 100% outdoor UEs without car penetration loss (3km/h) - the same UE clustering method as for FR1 is applied except that all the UEs (including UEs in the clusters and out of the clusters) are outdoor UEs without car penetration loss (3km/h). - R=20m, M=10, X=1 - X=2 and M=20 can also be evaluated. - Other values of R can be evaluated. - UE height is 1.5m for all UEs - For any other aspects, reuse what was agreed for FR1 UE clustering is new baseline for Dense Urban Macro layer for FR2-1. Agreement in 111 Regarding the UE distribution of 2-layer Scenario B (HetNet with Urban Macro and Indoor), - Layer 1: Urban Macro - 10 users per macro TRP per direction, and all users are randomly and uniformly dropped within the macro cell outside the Indoor office / Indoor factory - Indoor/outdoor proportion: - Option 1 (baseline): 100% outdoor without car penetration loss, 3km/h, UE height is 1.5m - Option 2 (optional): - 20% outdoor in cars: 30km/h, UE height is 1.5m - 80% indoor in houses: 3km/h, UE height is 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) - Layer 2: Indoor office (baseline) - 10 users per indoor TRP per direction, and all users are randomly and uniformly dropped within the building. - UE speed is 3km/h, UE height is 1.5m - Minimum UE-UE (2D) distance: 1m - Minimum BS-UE (2D) distance: 0m - UE selected macro TRP or indoor TRP is determined based on the RSRP, i.e., the UE in the indoor office/indoor factory can select the Macro cell as serving cell, and the UE outside the indoor office/indoor factory can select the indoor TRP as serving cell

100% outdoor UE distribution without car penetration loss is considered in several cases, i.e., UE clustering distribution for Dense Urban Macro layer for FR2-1, UE distributed on Urban Macro layer for 2-layer Scenario B (HetNet with Urban Macro and Indoor). However, the penetration losses between UEs in Dense Urban Macro layer for FR2-1 and penetration losses between outdoor UE to outdoor UE, outdoor UE to indoor UE in 2-layer Scenario B follow Table A.2.1-12 in TR38.802 in previous agreements, where car penetration is considered for outdoor UE. Besides, car penetration is also considered for Macro to outdoor UE and Indoor TRP to outdoor UE in 2-layer Scenario B. Thus, it should be clarified that car penetration related to outdoor UE should not be modelled when UE distribution of 100% outdoor UE without car penetration loss is assumed.

Car penetration related to outdoor UE should not be modelled when UE distribution of 100% outdoor UE without car penetration loss is assumed.

- The penetration losses between UEs in Dense Urban Macro layer for FR2-1 and penetration losses between outdoor UE to outdoor UE, outdoor UE to indoor UE in 2-layer Scenario B follow Table A.2.1-12 in TR38.802 with modification, where car penetration loss should not be modeled for outdoor UE.

- For Macro to outdoor UE and Indoor TRP to outdoor UE in 2-layer Scenario B, car penetration loss should not be modeled for outdoor UE if Indoor/outdoor proportion option 1 is assumed.

Layout and UE distribution

SBFD Deployment case 3-2: 2-Layer scenario B

Agreement in RAN1#111 Regarding layout of 2-layer Scenario B (HetNet with Urban Macro and Indoor), - Layer 1: Urban Macro - Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around, ISD=500m - Layer 2: Indoor office (baseline) - Only one building randomly dropped in the whole network as in the figure below. The building has to be confined within one macro cell area. - 12 (baseline) or 3 (optional) TRPs per 120m x 50m x 3m - the distance between two indoor TRPs: 20m for 12 TRPs, 40m for 3 TRPs - The orientation of the building is fixed as in the figure below (i.e., the long side of the rectangular is along the x-axis) - The minimum 2D distance between macro TRP and indoor office center is 100m - The minimum 2D distance between macro TRP and indoor/outdoor UE is 35m

The layout of 2-layer Scenario B (HetNet with Urban Macro and Indoor) has been agreed as shown in the above agreement. For the HetNet with Urban Macro and Indoor factory, the layout should be further determined. It is captured in TR 38.901 that candidate room size of indoor factory is different for sub-cases, which is shown as below:

- InF-SL: 120m x 60 m

- InF-DL: 300m x 150 m

- InF-SH: 300m x 150 m

- InF-DH: 120m x 60 m

Considering that ISD of Macro cell is 500m, only the small hall size (120x60 m) should be considered for indoor factory layer. In Table 7.8-8 in TR 38.901, simulation assumptions for large scale calibration for the indoor factory scenario are provided. To reused the simulation assumptions, the size of indoor factory, BS deployment, BS height, Clutter density, Clutter height, Clutter size should follow the assumptions in TR 38.901.

Regarding layout of 2-layer Scenario B (HetNet with Urban Macro and Indoor factory), small hall size 120m x60 m (sub-cases InF-SL and InF-DH) should be considered. Besides, the simulation assumptions, e.g. BS deployment, BS height, Clutter density, clutter height and clutter size, in TR 38.901 should be reused.

- Layer 1: Urban Macro

  - Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around, ISD=500m

- Layer 2: Indoor factory (optional)

  - Only one building randomly dropped in the whole network as in the figure below. The building has to be confined within one macro cell area.

    - 18 (baseline) or 3 (optional) TRPs are considered.

    - The hall size is 120m x 60m for InF-SL and InF-DH.

    - The distance between two indoor TRPs: 20m for 18 TRPs, 40m for 3 TRPs

    - The height of indoor TRP is 1.5m for InF-SL and 8m for InF-DH.

  - The orientation of the building is fixed as in the figure below (i.e., the long side of the rectangular is along the x-axis).

  - Clutter density, clutter height and clutter size follow Table 7.8-8 in TR 38.901.

- The minimum 2D distance between macro TRP and indoor factory center is 100m

- The minimum 2D distance between macro TRP and indoor/outdoor UE is 35m

Figure 6 Layout of 2-layer Scenario B (HetNet with Urban Macro and Indoor factory).

SBFD Deployment Case 4

In RAN1#111 meeting, two options were raised for dropping clusters in two network. The agreement is shown as below:

Agreement in RAN1#111 UE clustering distribution is also applied for SBFD Deployment Case 4 as baseline. Down-select from the following two options in RAN1#112: - Option 1. Cluster centers for each operator are independently dropped. - Option 2. Cluster centers for operator A are dropped. The cluster centers are used for operator B. - FFS: grid shift case

UE clustering distribution is used to evaluate the effects of UE-to-UE CLI. In option 1, the cluster centers are independently dropped, the distance between UEs of operator A and UEs of operator B is large with high probability. Besides, UEs in different clusters are in different buildings, the penetration loss between UEs of operator A and UEs of operator B is high. As a result, UE-to-UE CLI is weak across different operators may be negligible. Thus, option 1 is not preferred in the SBFD Deployment Case 4.

For option 2, the cluster centers for operator A and operator B are same. In this situation, the distance between UEs of operator A and UEs of operator B is small with high probability. There is no penetration loss among UEs in the same cluster, despite of which operator is the serving operator. Thus, it is friendly to assess the influence on network performance comes from UE-to-UE CLI. As for the 100% grid shift case, cluster should locate within the common area between operator A and operator B.

Figure 7 The position restriction of cluster when option 2 is used with 100% grid shift.

For UE clustering distribution in SBFD Deployment Case 4, option 2 is preferred.

- Grid shift of two operator can be 0% and 100%.

- For 100% grid shift case, cluster should locate within the common area between operator A and operator B.

Interference modeling for SBFD

ACLR and ACS for gNB in FR1

In previous meetings, the following agreements were achieved on how to model the inter-site gNB-gNB co-channel inter-subband CLI:

Agreement in RAN1#110b For SLS in RAN1, if both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the inter-site gNB-gNB co-channel inter-subband CLI signal across all Rx chains at UL RB n at victim gNB can be modeled as IInter-Site-CLIn=Ileakagen+Iselectivity, where, - Ileakagen=HCLInWyn is the first part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at UL RB n, caused by power leakage at aggressor gNB, - HCLIn is the NR×NT channel matrix between aggressor gNB and victim gNB at UL RB n, the beamforming of the aggressor gNB and the victim gNB can be taken into account by HCLIn, - yn=y0n,y1n,…,yNT-1nT is the unwanted emission across all Tx chains at UL RB n at aggressor gNB, - NT is the number of Tx chains at aggressor gNB, - ykn~N0,σy,n2, k=0,1,…,NT-1, is modelled as white Gaussian noise, - σy,n2=Ptxper-RB*Nused-DL-RBACLRBS* 1NDLRB is the total leakage power at UL RB n at aggressor gNB, - Ptxper-RB is the DL power transmitted across all Tx chains at one DL RB at aggressor gNB, Ptxper-RB=Ptxmax/NDLRB, - Nused-DL-RB is the number of DL RBs scheduled for DL transmission by aggressor gNB, - NDLRB is the total number of DL RBs in the DL subbands - W is the NT×NT normalized identity matrix with unit norm, WF=1, - FFS whether W can be other values and corresponding conditions - FFS for Iselectivity - Note: ACLRBS and ACSBS are in linear scale. In RAN4 reply LS, gNB ACLR (i.e., ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e., ACSBS) is provided as the candidate for Receiver impairment. - Note: the model is based on the assumption that the same transmission power across different DL RBs are used in SLS. This does not prevent companies to use other DL power allocation schemes in SLS. - Note: This model is not applicable to the RBs in the guardband. - Send LS to RAN4 to confirm RAN1’s understanding. Agreement in RAN1#111 Regarding the modelling of inter-site gNB-gNB co-channel inter-subband CLI agreed in RAN1#110bis for the case that both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, - For Ileakagen=HCLInWyn, it is up to companies to report other values of W and the corresponding applicable conditions. Agreement in RAN1#111 Regarding the modelling of inter-site gNB-gNB co-channel inter-subband CLI agreed in RAN1#110bis for the case that both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the second part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at one UL RB, caused by receiver selectivity at victim gNB, can be modelled as Iselectivity=z0n,z1n,…,zNR-1nT - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise - σz,n2=PblockerICSBS×NDLRB - Pblocker=1NR*m∈Used DL RBsHCLImWmsm2 - HCLIm is the NR×NT channel matrix between aggressor gNB and victim gNB at DL RB m, the analog beams of the aggressor gNB and the victim gNB can be taken into account by HCLIm, - Wm is the digital precoder at DL RB m at aggressor gNB, WmF=1, - sm is the symbol transmitted at DL RB m at aggressor gNB with transmission power for each layer as Ptxper-RB=Ptxmax/NDLRB. - NDLRB is the total number of DL RBs in the DL subbands, - RAN1 can assume ICSBS (in channel selectivity) is given by gNB ACS unless further RAN4 guidance is received. - Send LS to RAN4 to confirm RAN1 understanding and check whether ICSBS can be modelled depending on the value of the blocker interference, e.g., ICSBS= ICS1 Pblocker<P1 ICS2, P1< Pblcoker<P2ICS3 Pblocker>P2 - Note: Pblocker can be reported by companies

When both large fading and small fading are considered for the inter-site gNB-gNB co-channel inter-subband CLI modelling, the value of ACLRBS and ACSBS are used to calculate Ileakagen and  Iselectivity, respectively. In RAN4 reply LS, gNB ACLR (i.e., ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e., ACSBS) is provided as the candidate for Receiver impairment. Besides, RAN4 considers to reuse the BS ACLR/ACS values captured in TR38.828. [3] It is proposed that ACLR/ACS for FR1 BS: 45/46 dBc, FR2 BS: 28/23.5 dBc can be used. [4]

In the inter-site gNB-gNB co-channel inter-subband CLI modelling when both large fading and small fading are considered, the following values of ACLRBS and ACSBS can be assumed for simulation purpose.

- In FR1, ACLR and ACS of gNB are 45dB and 46dB, respectively.

- In FR2-1, ACLR and ACS of gNB are 28dB and 23.5dB, respectively.

UE-UE co-channel inter-subband CLI modelling

Tx leakage model

Agreement in RAN1#111 For SLS in RAN1, regarding Tx leakage model of UE-UE co-channel inter-subband CLI modelling, Option 1 is used as starting point. - Option 1: RAN1 to take in-band emission (IBE) defined in TS38.101-1 and TS38.101-2 as starting point. - Send LS to RAN4 to ask them whether it can be modelled as an equivalent frequency flat model (e.g., IBEUE,ave) based on RAN4 IBE requirement, and if possible, what is the value of IBEUE,ave

If IBE model defined in TS38.101-1 and TS38.101-2 is used in the simulation (Option 1), an in-band emissions (IBE) combined limit is applied on each non-allocated RB. For each RB, the minimum requirement is calculated as the maximum value between (PRB - 30 dB for FR1) or (PRB - 25 dB for FR2-1) and the power sum of all limit values (General, IQ Image or Carrier leakage). With the model, TX leakage power on each DL RB could be obtained and can be used to model Tx leakage when only large-scale fading is modelled for UE-UE co-channel inter-subband CLI.

For SLS in RAN1, regarding Tx leakage model of UE-UE co-channel inter-subband CLI modelling, Option 1 can be used to Tx leakage when only large-scale fading is modelled for UE-UE co-channel inter-subband CLI.

In channel selectivity model

Initial question 2-1-7: For SLS in RAN1, regarding Rx impairment model of UE-UE co-channel inter-subband CLI modelling, what’s your views on the following options or other options? - Option 1: Reuse UE ACS in absence of further RAN4 inputs. - Option 2: RAN1 assumes no UE in-band selectivity in absence of further RAN4 inputs, i.e., 0 dB without any rejection/attenuation on interference in Rx model.

Considering that in-channel selectivity (ICS) requirements for the UE are not defined, option 1 is straightforward. However, it is too strict for UE implementation if UE ICS requirements are same as UE ACS. On the other hand, when UE ACS is used in the SLS, the UE-UE co-channel inter-subband CLI is much weaker than the realistic interference. Thus, ICSUE smaller than UE ACS could be used in UE-UE co-channel inter-subband CLI.

In option 1, it is too strict for UE implementation if UE ICS requirements are same as UE ACS, and reuse UE ACS in the SLS could not effectively evaluate the effects of UE-to-UE CLI at victim UE.

- ICSUE smaller than UE ACS could be used and could be reported by companies.

With given ICSUE, the inter-site gNB-gNB co-channel inter-subband CLI modelling could be reused for UE-UE co-channel inter-subband CLI.

For SLS in RAN1, if only large scale fading is modelled for UE-UE co-channel channel model, the power of UE-UE co-channel inter-subband CLI experienced by the victim UE on each receiver chain at DL RB n can be modelled as IUE-UE-CLIA'→A, n=IleakageA'→A, n+Iselectivity, where

- Iselectivity=PtxUE A',per-RB*CLUE A'→UE A*1ICSUE* Nused-UL-RBUE A'NULRB

  - Iselectivity is the second part of UE-UE co-channel inter-subband CLI from UE A' to UE A on each receiver chain at DL RB n (linear value), caused by in-channel selectivity at the victim UE

  - PtxUE A',per-RB is UL transmission power of UE A' across all transmit chains per RB (linear value). PtxUE A',per-RB=PtxUE A'/Nused-UL-RBUE A'.

  - Nused-UL-RBUE A' is the number of UL RBs allocated for UL transmission of UE A'

  - NULRB is the total number of UL RBs in the UL subbands

  - CLUE A'→UE A is the coupling loss between UE A' and UE A (linear value), accounting for beamforming at the aggressor UE and victim UE.

  - Note: ICSUE is in linear scale, is the in-channel selectivity of victim UE A.

For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for UE-UE co-channel model, the power of UE-UE co-channel inter-subband CLI experienced by the victim UE on each receiver chain at DL RB n can be modelled as IUE-UE-CLIA'→A, n=IleakageA'→A, n+Iselectivity, where

- Iselectivity=PtxUE A',per-RB*CLUE A'→UE A*1ICSUE* Nused-UL-RBUE A'NULRB

  - Iselectivity is the second part of UE-UE co-channel inter-subband CLI from UE A' to UE A on each receiver chain at DL RB n (linear value), caused by in-channel selectivity at the victim UE

  - PtxUE A',per-RB is UL transmission power of UE A' across all transmit chains per RB (linear value). PtxUE A',per-RB=PtxUE A'/Nused-UL-RBUE A'.

  - Nused-UL-RBUE A' is the number of UL RBs allocated for UL transmission of UE A'

  - NULRB is the total number of UL RBs in the UL subbands

  - CLUE A'→UE A is the coupling loss between UE A' and UE A (linear value), accounting for beamforming at the aggressor UE and victim UE.

  - Note: ICSUE is in linear scale, is the in-channel selectivity of victim UE A.

For SLS in RAN1, if both large scale fading and small scale fading are modelled for UE-UE co-channel channel model, the UE-UE co-channel inter-subband CLI signal across all Rx chains at DL RB n at victim UE can be modeled as IUE-UE-CLIn=Ileakagen+Iselectivity, where,

Iselectivity=z0n,z1n,…,zNR-1nT

- zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise

- σz,n2=PblockerICSUE×NULRB

- Pblocker=1NR*m∈Used UL RBsHCLImWmsm2

- HCLIm is the NR×NT channel matrix between aggressor UE and victim UE at UL RB m, the analog beams of the aggressor UE and the victim UE can be taken into account by HCLIm,

- Wm is the digital precoder at UL RB m at aggressor gNB, WmF=1,

- sm is the symbol transmitted at UL RB m at aggressor UE with transmission power for each layer as PtxUE A',per-RB=PtxUE A'/Nused-UL-RBUE A'.

- Nused-UL-RBUE A' is the number of UL RBs allocated for UL transmission of UE A'

For SLS in RAN1, if both large scale fading and small scale fading are modelled for UE-UE co-channel channel model, the UE-UE co-channel inter-subband CLI signal across all Rx chains at DL RB n at victim UE can be modeled as IUE-UE-CLIn=Ileakagen+Iselectivity, where,

Iselectivity=z0n,z1n,…,zNR-1nT

- zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise

- σz,n2=PblockerICSUE×NULRB

- Pblocker=1NR*m∈Used UL RBsHCLImWmsm2

- HCLIm is the NR×NT channel matrix between aggressor UE and victim UE at UL RB m, the analog beams of the aggressor UE and the victim UE can be taken into account by HCLIm,

- Wm is the digital precoder at UL RB m at aggressor gNB, WmF=1,

- sm is the symbol transmitted at UL RB m at aggressor UE with transmission power for each layer as PtxUE A',per-RB=PtxUE A'/Nused-UL-RBUE A'.

- Nused-UL-RBUE A' is the number of UL RBs allocated for UL transmission of UE A'

In Option 2, RAN1 assumes no UE in-channel selectivity in absence of further RAN4 inputs, i.e., 0 dB without any rejection/attenuation on interference in Rx model. In this case, the interference is not attenuated, and there is no receiver selectivity at the receiver. Iselectivity model of inter-site gNB-gNB co-channel inter-subband CLI could not be reused as it is used to model receiver selectivity/ Rx impairment.

In option 2, Iselectivity model of inter-site gNB-gNB co-channel inter-subband CLI for receiver selectivity at victim gNB could be not reused for Rx impairment of victim UE.

Study the Rx impairment model if RAN1 assumes no UE in-channel selectivity.

In RAN1#111 meeting, model for the UE-UE co-channel inter-subband CLI when option 2 is applied has been proposed in the summary, which is shown as follows.

- For inter-UE inter-subband CLI modeling, the blocker interference at the UL subband of the victim UE can be modelled as increase of quantization noise which affects the DL signal-to-interference- quantization-and-noise ratio (SIQNR) when blocker power is higher than RSSI of the DL signal.

  - ΔQndB ≈Ix-RSSI ,

  - SIQNR= SPNp+Iz+ Qn+ ΔQn(Ix),

where

- Ix is the strength of blocking interference from aggressor UE in dB,

- RSSI is the power of received DL signal in dB,

- Iz is the strength of Tx leakage of UE-UE inter-subband CLI from aggressor UE in linear scale,

- Qn is the quantization noise in linear scale,

- ΔQndB and  ΔQn(Ix) are the increased quantization noise in dB and linear scale, respectively,

- Np is the thermal noise in linear scale,

- SP is the power of signal in linear scale.

In this model, the process to obtain value of Qn should be further determined. As MMSE-IRC receiver is assumed in the evaluation assumptions, the effect of quantization noise to the receiver performance can be assumed to be same with thermal noise in the SLS.

For inter-UE inter-subband CLI modeling when option 2 is applied, the blocker interference at the UL subband of the victim UE can be modelled as increase of quantization noise which affects the DL SIQNR when blocker power is higher than RSSI of the DL signal.

  - ΔQndB ≈Ix-RSSI

  - SIQNR= SPNp+Iz+ Qn+ ΔQn(Ix),

where

- Ix is the strength of blocking interference from aggressor UE in dB,

- RSSI is the power of received DL signal in dB,

- ΔQndB is the increased quantization noise in dB,

- Iz is the strength of Tx leakage of UE-UE inter-subband CLI from aggressor UE in linear scale,

- SP is the power of signal in linear scale,

- Np is the thermal noise in linear scale,

- Qn is the quantization noise in linear scale,

- ΔQn(Ix) is the increased quantization noise in linear scale, respectively

The effect of quantization noise to the receiver performance can be assumed to be same with thermal noise in the SLS.

FFS: The process to obtain quatization noise Qn should be determined.

Issues in the calibration

Simulation parameters in the calibration

The simulation assumptions for SLS calibration are summarized in RAN1#111. Some updates related to UE numbers per cluster, boresight direction of TRxP.

Update the simulation assumptions for SLS calibration as below:

|  | Urban Macro (FR1) |  | Dense Urban Macro Layer (FR2-1) | Indoor office (FR1) | Indoor office (FR2-1) |
| --- | --- | --- | --- | --- | --- |
| Carrier frequency | 4 GHz |  | 30GHz | 4 GHz | 30GHz |
| System bandwidth | 100MHz |  | 100MHz | 100MHz | 100MHz |
| Numerology | 14 OFDM symbol slot SCS = 30kHz |  | 14 OFDM symbol slot SCS = 120kHz | 14 OFDM symbol slot SCS = 30kHz | 14 OFDM symbol slot SCS = 120kHz |
| BS transmit power for SBFD | 53 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD |  | 40 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | 24 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | 23 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD |
|  | Assume the BS transmit power spectrum density is kept the same for SBFD operation and legacy TDD operation. BS transmit power is proportional to the RBs used for DL transmission. |  |  |  |  |
| UE Tx power | 23dBm |  | 23 dBm. EIRP should not exceed 43 dBm | 23dBm | 23 dBm. EIRP should not exceed 43 dBm |
| Layout | Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around |  |  | 12 TRPs per 120m x 50m x 3m |  |
| Inter-BS (2D) distance | 500m |  | 200m | 20m |  |
| Minimum BS-UE (2D) distance | 35m |  | 35m | 0m |  |
| Minimum UE-UE (2D) distance | 1m |  | 1m | 1m |  |
| BS antenna height | 25m |  | 25m | 3m |  |
| UE distribution | UE Clustering |  | UE Clustering | Uniform |  |
| UE number per macro/indoor TRP (per direction) (M) | 20 |  | 10 | 10 |  |
| UE cluster number per macro cell (X) | 2 |  | 1 | - |  |
| UE outdoor/indoor proportion | 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h |  | 100% outdoor without car penetration loss: 3km/h | 100% indoor in houses: 3km/h |  |
| UE number per cluster | 8 |  | 8 | - |  |
| Indoor UE height (m) | 1.5m |  | 1.5m | 1.5m |  |
| Outdoor UE height (m) | 1.5m |  | 1.5m | - |  |
| Radius of cluster (R) | 25m |  | 20m | - |  |
| Minimum distance between macro TRP to UE cluster center (Dmacro-to-cluster) | 60m |  | 55m | - |  |
| Minimum distance between two UE cluster centers (Dinter-cluster) | 50m |  | - | - |  |
| gNB-UE Channel model (large-scale) | Macro-to-UE: UMa in TR 38.901 For FR1, gNB-UE O2I penetration loss: 80% low-loss model, 20% high-loss model |  |  | TRP-to-UE: InH-Office in TR 38.901 Penetration loss is not modelled. |  |
| gNB-gNB Channel model (large-scale) | Macro-to-Macro: UMa in TR 38.901 (hUE =25m) LOS probability: If the 2D distance between two Macro gNBs are less than or equal to the ISD, set the LOS probability to 0.75; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. |  |  | TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) Penetration loss is not modelled. |  |
| UE-UE Channel model (large-scale) | UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m). For FR1, penetration loss between UEs follows Table A.2.1-12 in TR38.802 |  |  | UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) |  |
| BS antenna array configuration for Legacy TDD | M,N,P,Mg,Ng;Mp,Np= (8,8,2,1,1;2,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization | M,N,P,Mg,Ng;Mp,Np= (4,16,2,2,2; 1,1) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization |  | M,N,P,Mg,Ng;Mp,Np = (4,4,2,1,1; 4,4) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization | M,N,P,Mg,Ng;Mp,Np=(16,8,2,1,1; 1,1) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization |
| BS antenna array configuration for SBFD | - SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (8,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ |  | - SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (4,16,2,2,2). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ | - SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (4,4,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ | - SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (16,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ |
| BS antenna radiation pattern | Reuse Table 9 in Report ITU-R M.2412 |  |  | Reuse Table 10 in Report ITU-R M.2412 |  |
| UE antenna configuration | - 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization - 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |  | 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1); (dH,dV) = (0.5,0.5)λ,(dg,V,dg,H) = (0, 0)λ, 0°/90° polarization; Θmg,ng=90°; Ω0,1=Ω0,0+180° | - 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization - 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization | 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1); (dH,dV) = (0.5,0.5)λ,(dg,V,dg,H) = (0, 0)λ, 0°/90° polarization; Θmg,ng=90°; Ω0,1=Ω0,0+180° |
| UE antenna radiation pattern | Omni-directional with 0 dBi element gain |  | reuse Table 11 in Report ITU-R M.2412 | Omni-directional with 0 dBi element gain | reuse Table 11 in Report ITU-R M.2412 |
| BS receiver noise figure | 5dB |  | 7 dB | 5dB | 7 dB |
| UE receiver noise figure | 9 dB |  | 13 dB | 9 dB | 13 dB |
| Open loop power control parameters | P0= -80 dBm, alpha = 0.8 |  | P0= -86 dBm, alpha = 0.9 | P0= -60 dBm, alpha = 0.6 |  |
| Handover margin (dB) | 3 dB |  | 3 dB | 3 dB | 3 dB |
| UE attachment | Based on RSRP from port 0 |  | Based on RSRP from port 0. - Out of the two UE panels, the UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. - Single gNB panel is used for UE attachment | Based on RSRP from port 0 | Based on RSRP from port 0. - Out of the two UE panels, the UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. - Single gNB panel is used for UE attachment |
| Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 |  |  |  |  |
| Boresight direction | 0°, 120° and 240° for three sectors |  |  | 90° |  |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |  | 90° in GCS (pointing to horizontal direction) | 180° in GCS (pointing to the ground) | 180° in GCS (pointing to the ground) |
| Electronic tilt | (According to Zenith angle in "Beam set at TRxP") |  | (According to Zenith angle in "Beam set at TRxP") | 90° in LCS | (According to Zenith angle in "Beam set at TRxP") |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = 0 Zenith angle θj = pi*102/180 NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |  | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = {-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16} Zenith angle θj = {5*pi/8, 7*pi/8} NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | - | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = {-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16} Zenith angle θj = {pi/4, 3*pi/4} NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | - |  | For direction of UE analog beam steering (in LCS): Azimuth angle φi = {-3*pi/8, -pi/8, pi/8, 3*pi/8}; Zenith angle θj = {pi/4, 3*pi/4}; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | - | For direction of UE analog beam steering (in LCS): Azimuth angle φi = {-3*pi/8, -pi/8, pi/8, 3*pi/8}; Zenith angle θj = {pi/4, 3*pi/4}; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |

Other issues on SLS evaluation

Another issue is whether to use dynamic TDD for legacy TDD for comparison. The methodology and feasibility of dynamic TDD is under discussion. It may lead to duplicated work if dynamic TDD is also considered in the SBFD evaluation. Furthermore, we are still not clear on how to handle the CLI and which scenario(s) is/are recommended for dynamic TDD. It is premature to use dynamic TDD for legacy TDD for comparison.

Dynamic TDD is not used for legacy TDD for comparison.

Evaluation cases

In previous RAN1 meetings, very good progress has been achieved on evaluation methodology and deployment scenarios [1]. On the other hand, there is a lot of freedom for companies to select the detail parameters of SLS simulation. We list the key parameters for system level evaluations with different options as below:

- UL/DL traffic generation: there are two options for UL/DL traffic generation, i.e. 1) Each UE is either assigned UL traffic or DL traffic. 2) Each UE is assigned both UL traffic and DL traffic.

- FTP packet size: symmetric packet size or asymmetric packet size.

- Channel estimation: ideal channel estimation or realistic channel estimation.

- BS transmit power: two options for Macro scenario.

- UE-UE channel model: two options for FR1.

- gNB antenna architecture: three options were identified in RAN1#110 meeting.

- Slot configuration for legacy TDD: DDDSU in Alt 2, Alt 4 and Alt 1, DDSUU in Alt 3

- Slot configuration for SBFD: Frame structure#1 (DXXXU), Frame structure#2 (XXXXU) and Frame structure#3 (XXXXX)

- UE receiver: MMSE-IRC as the baseline receiver and advanced receiver is not precluded.

- Transmission scheme: SU-MIMO, MU-MIMO, maximum layers for SU-MIMO/MU-MIMO,

We want to emphasize the options for each listed key parameter are equal and which option is adopted in the simulation is fully up to company. Without further guidance, the optional parameters would lead to divergent parameter combinations. Considering the above parameters are fundamental for SLS evaluation, different parameter would lead to quite different results. It makes comparison among companies very difficult if not impossible. Accordingly, it is important to identify a baseline combination of the above parameters.

A baseline combination is needed for the following key parameters for easy comparison among companies:

- UL/DL traffic generation

- FTP packet size

- Channel estimation

- BS transmit power

- UE-UE channel model

- gNB antenna architecture

- Slot configuration for legacy TDD

- Slot configuration for SBFD

- UE receiver

- Transmission scheme

Considering that simulation assumptions for SLS calibration are processed to determine baseline parameters for calibration, the parameters determined in the simulation assumptions for SLS calibration can be also used in the baseline combination.

The parameters considered in simulation assumptions for SLS calibration can be used performance evaluation:

- BS transmit power

- UE-UE channel model

- gNB antenna architecture

Except for the parameters mentioned in simulation assumptions for SLS calibration, other parameters with multiple options should also be down-selected. Besides, the parameters with single option are also listed in this paper.

Except for the parameters considered in simulation assumptions for SLS calibration, other parameters can be used for the baseline combination is shown in table 1 and table 2 in appendix.

Interference definition

In RAN1#109 e-meeting, a bunch of interference type were defined for facilitating further discussion. Regarding to gNB-UE co-channel interference and UE-gNB co-channel interference, the following two types were identified:

- gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration.

- UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration.

However, the above interference cannot cover all cases in deployment case 2, wherein inter-subband interference is introduced by unaligned UL subband configuration across gNB. One example is shown in Figue 3, wherein the aggressor gNB#1 and victim gNB#2 have different UL subband configuration in a DL slot. Accordingly, the following two interference should be taken into consideration in the simualtion if needed.

- gNB-UE co-channel inter-subband interference: Interfernece caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to DL reception of the victim UE on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

- UE-gNB co-channel inter-subband interference: Interfernece caused by UL transmission of the aggressor UE on a first set of RBs in a carrier to UL reception of the victim gNB on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

Figure 8 Illustration of gNB-UE/UE-gNB co-channel inter-subband interference

For deployment case 2, the following two interference type should be take into account:

- gNB-UE co-channel inter-subband interference: Interfernece caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to DL reception of the victim UE on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

- UE-gNB co-channel inter-subband interference: Interfernece caused by UL transmission of the aggressor UE on a first set of RBs in a carrier to UL reception of the victim gNB on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

LLS evaluation

Agreement RAN1 agrees link-level simulations (LLS) may be performed for various purposes related to SBFD performance and feasibility in both FR1 and FR2, interested companies may perform LLS at least for the following purpose: - To evaluate coverage performance - Option 1: Take link level evaluation methodology in TR 38.830 (i.e., LLS + Link budget analysis) as starting point to evaluate the coverage performance (e.g., MPL, MCL, MIL) for SBFD considering inter-gNB/sector interference and self interference. - Other options (e.g. SLS as a tool to obtain the coverage metric) are not precluded - Details on LLS including but not limited to impact of different BS antennas to channel reciprocity / BF - FFS: - To evaluate advanced receivers and realistic demodulation performance - To evaluate UE-UE CLI mitigation performance - To evaluate gNB-gNB CLI mitigation performance - To evaluate feasibility and performance of self-IC accounting for realistic non-linearities in the gNB transmit and receive chains - Details on LLS including but not limited to impact of different BS antennas to channel reciprocity / BF

To evaluate coverage performance of SBFD, the effects of gNB-gNB CLI and self-interference at gNB should be captured. The co-site inter-sector co-channel inter-subband CLI modelling, inter-site gNB-gNB co-channel inter-subband CLI modelling, and self-interference modelling could be considered in the LLS, and the modelling in SLS could be reused.

For co-site inter-sector co-channel inter-subband CLI modelling, inter-site gNB-gNB co-channel inter-subband CLI modelling, and self-interference modelling, the modelling used in SLS could be reused.

The self-interference, co-site inter-sector co-channel inter-subband CLI and inter-site gNB-gNB co-channel inter-subband CLI (only large-scale fading is considered) can be modelled as white Gaussian noise with given variance.

The self-interference co-site inter-sector co-channel inter-subband CLI and co-site gNB-gNB co-channel inter-subband CLI (only large-scale fading is considered) can be modelled as white Gaussian noise with given variance.

If both large scale fading and small scale fading are considered for inter-site gNB-gNB co-channel inter-subband CLI, the gNB-gNB channel model should be considered. In the LLS, gNB-gNB channel model could reused gNB-UE channel model (e.g., CDL-C) with some modifications.

In the LLS, gNB-gNB channel model could reuse gNB-UE channel model (e.g., CDL-C) with some modifications.

Preliminary SLS calibration results

Based on the discussion in section 2.1.5, we provide preliminary SLS calibration results for FR1 and FR 2-1. Urban Macro and InH for FR1, Dense Urban and InH for FR2-1 are simulated. Coupling loss for gNB-gNB, gNB-UE and UE-UE are shown in the following figures. The detail simulation assumptions can be found in section 2.1.4.

Figure 9 SLS calibration results for Urban Macro in FR1.

Figure 10 SLS calibration results for InH in FR1.

Figure 11 SLS calibration results for Dense Urban in FR2-1.

Figure 12 SLS calibration results for InH in FR2-1.

SLS evaluation results for SBFD

In this section, we provide preliminary SLS evaluation results for SBFD deployment case 1 to investigate the performance gain offered by SBFD.[1] At the same time, we provide results for DL transmission when SBFD is introduced in order to see the impacts on DL direction. Furthermore, we also provide the results with different assumptions on self-interference suppression capability, which comes from the reply LS from RAN4.  Dense urban with single layer scenario is assumed wherein 20 UEs are dropped per sector. In each sector, one UE cluster is dropped and 80% UEs are uniformly distributed within the cluster. More detail simulation assumptions can be found in the appendix. Regarding the evaluation metrics, UL/DL user perceived thoughput, UL/DL latency and UL/DL RU are used in the simulation so as to comprehensively evaluate SBFD technique.  The aforementioned metrics are defined in previous meeting.[1]

Basically, the following cases are simulated:

- Baseline-DDDSU: Legacy TDD with TDD UL-DL configuration DDDSU.

- SBFD-XXXXU-UL20%-145dB: On top of DDDSU frame structure, the first four slots are SBFD slot, on which UL subband occupies 20% portion of system bandwidth. Self-interference capability is assumed to be 145dB, i.e. the isolation between Tx and Rx of a SBFD gNB is 145dB.

- SBFD-XXXXU-UL20%-185dB: On top of DDDSU frame structure, the first four slots are SBFD slot, on which UL subband occupies 20% portion of system bandwidth. Self-interference capability is assumed to be 145dB, i.e. the isolation between Tx and Rx of a SBFD gNB is 185dB.

- SBFD-XXXXU-UL50%-145dB: On top of DDDSU frame structure, the first four slots are SBFD slot, on which UL subband occupies 50% portion of system bandwidth. Self-interference capability is assumed to be 145dB, i.e. the isolation between Tx and Rx of a SBFD gNB is 145dB.

- SBFD-XXXXU-UL50%-185dB: On top of DDDSU frame structure, the first four slots are SBFD slot, on which UL subband occupies 50% portion of system bandwidth. Self-interference capability is assumed to be 145dB, i.e. the isolation between Tx and Rx of a SBFD gNB is 185dB.

- SBFD- DXXXU -UL20%-145dB: On top of DDDSU frame structure, the first four slots are SBFD slot, on which UL subband occupies 20% portion of system bandwidth. Self-interference capability is assumed to be 145dB, i.e. the isolation between Tx and Rx of a SBFD gNB is 145dB.

- SBFD- DXXXU -UL20%-185dB: On top of DDDSU frame structure, the first four slots are SBFD slot, on which UL subband occupies 20% portion of system bandwidth. Self-interference capability is assumed to be 145dB, i.e. the isolation between Tx and Rx of a SBFD gNB is 185dB.

- SBFD- DXXXU -UL50%-145dB: On top of DDDSU frame structure, the first four slots are SBFD slot, on which UL subband occupies 50% portion of system bandwidth. Self-interference capability is assumed to be 145dB, i.e. the isolation between Tx and Rx of a SBFD gNB is 145dB.

- SBFD- DXXXU -UL50%-185dB: On top of DDDSU frame structure, the first four slots are SBFD slot, on which UL subband occupies 50% portion of system bandwidth. Self-interference capability is assumed to be 145dB, i.e. the isolation between Tx and Rx of a SBFD gNB is 185dB.

- 2.4.1 System simulation results for dense urban scenario

In this section, the evaluation results for dense urban scenarios are provided which take different assumptions into account.

- 2.4.1.1 UL and DL User Perceived Throughput

In this section, we provide results derived from following user perceived througput definition for DL and UL respectively.

  - Average-UPT of a user: defined as the average from all UPTs for all FTP packets intended for this user [Refer to TR36.814].

  - Tail-UPT of a user: defined as the worst 5% UPT among all FTP packets intended for this user [Refer to TR36.814].

  - Median-UPT of a user: defined as the 50% UPT among all FTP packets intended for this user.

In order to reflect the traffic load in real world, we also provide results corresponding to different RU. The RU is determined by the baseline legacy TDD.

Figure 13 UL Average-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD.

Figure 14 DL Average-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 15 UL Medium-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 16 DL Medium-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 17 UL Tail-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 18 DL Tail-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Based on the simualtion results, we can have the following observations:

- DL user perceived thoughput is degraded significantly once UL subband is configured in DL slots, no matter which kind of UPT is used.

  - The degradation of performance is caused by less available DL resources in each SBFD slot

  - The larger UL subband in a SBFD slot, the more serious impacts on DL user thoughput

- UL user perceived thoughput is improved with introduction of UL subband in SBFD slots.

  - Significant performance gain can be observed when UL subband is introduced. The gain increases with the UL subband size, i.e. a larger UL subband brings better UL user throughput

  - The gain obtained from SBFD is impacted by the capability of self-interference suppression. Self-interference is one of the major obstacles in-between to harvest benefits from SBFD.

- In ratio, improvement of UL performance is much more than the degradation of DL performance.

- The maximum UL user throughput depends on how much additional UL resources can be provided by UL subband. For example, the UL user perceived throughput of a UE is better when XXXXU is configured comparing with DXXXU.

- Self-interference isolation impacts the benefits we can harvest from SBFD. For 185 dB SI suppression capability and 145 dB SI suppression capability, the former assumption slightly outperforms the later one. In the other word, 145 dB SI suppression capability already provides enough isolation for self-interference

When UL subband is introduced in DL slot, UL user throughput is improved significantly:

- Degradation of DL user thoughput is also observed, which depends on the UL subband configuration.

- In ratio, improvement of UL performance is much more than the degradation of DL performance.

- Better self-interference isolation achieves higher UL user throughput. 145 dB SI suppression can provide sufficient isolation between Tx and Rx at gNB side.

- 2.4.1.2 UL and DL latency

In this section, we provide results derived from following latency definition for DL and UL respectively.

- Option 1: Calculate the latency for each packet for each UE, and then generate CDF of latency for all these packets from all the UEs.

  - Packet-Latency CDF: The CDF of the packet latencies of all the packets from all the UEs.

  - Mean/5%/50%/95% Packet-Latency: The mean/5%/50%/95% value of Packet-Latency of all the packets from all the UEs.

- Option 2: Calculate the latency for each packet for each UE, and then calculate the average latency for each UE, then generate the CDF for these average latency for each UE

  - UE-Average-Latency: defined as the average packet latency for a UE

  - UE-Average-Latency CDF: The CDF of the UE-Average-Latency for all users.

  - Mean/5%/50%/95% UE-Average-Latency: The mean/5%/50%/95% value of UE-Average-Latency for all users.

Figure 19 UL Packet-latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 20 DL Packet-latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 21 UL UE-latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 22 DL UE-latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 23 UL UE-average(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 24 DL UE-average(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Based on the simualtion results, we can have the following observations:

- DL latecy is increased once UL subband is configured in DL slots no matter which kind of packet latency is adopted.

  - The degradation of performance comes from less available DL resources in each SBFD slot

  - The larger UL subband in a SBFD slot, the more serious impacts on DL latency

- UL latency is reduced with introduction of UL subband in SBFD slots no matter which kind of packet latency is adopted.

  - Significant performance gain can be observed when UL subband is introduced. The gain increases with the UL subband size, i.e. a larger UL subband brings better performance on latency.

  - The gain obtained from SBFD is impacted by the capability of self-interference suppression. Self-interference is one of the major obstacles in-between to harvest benefits from SBFD.

- In ratio, the magnitudes of improvement for UL performance is larger than degradation for DL performance.

- Self-interference isolation impacts the benefits we can harvest from SBFD. Lower UL latency can be achieved when larger self-interference isolation is assumed.

When UL subband is introduced in DL slot, UL lateny can be reduced significantly:

- Increasement of DL latency is also observed, which depends on the UL subband configuration.

- Better self-interference isolation brings lower UL latency.

- 2.4.1.3 UL and DL resource utilization

Baseed on the simulation results, the DL and UL resource utilization at 5%, 50% and percentile are summarized in Figure 16 and Figure 17 respectively.

Figure 25 DL resource utilization for Dense urban scenario

Figure 26 UL resource utilization for Dense urban scenario

Based on the simualtion results, we can have the following observations:

- DL resource utilization is increased once UL subband is configured in DL slots across light traffic load, medium traffic load and high traffic load.

  - The increasement of DL resource ultilization comes from less available DL resources in each SBFD slot

- UL resource utilization is reduced with introduction of UL subband in SBFD slots across light traffic load, medium traffic load and high traffic load.

- Better self-interference isolation can further reduce UL resource ultilization.

When UL subband is introduced in DL slot, UL resource utilization can be reduced.

- Increasement of DL RU is also observed, which depends on the UL subband configuration.

- In ratio, reduction of UL performance is close to the increasement of DL performance.

- Better self-interference isolation achieves lower UL resource utilization.

- 2.4.2 System simulation results for Indoor/Hotspot scenario

In this section, the evaluation results for Indoor/Hotspot scenarios are provided which take different assumptions into account.

- 2.4.2.1 UL and DL User Perceived Throughput

In this section, we provide results derived from following user perceived througput definition for DL and UL respectively.

  - Average-UPT of a user: defined as the average from all UPTs for all FTP packets intended for this user [Refer to TR36.814].

  - Tail-UPT of a user: defined as the worst 5% UPT among all FTP packets intended for this user [Refer to TR36.814].

  - Median-UPT of a user: defined as the 50% UPT among all FTP packets intended for this user.

In order to reflect the traffic load in real world, we also provide results corresponding to different RU. The RU is determined by the baseline legacy TDD.

Figure 27 UL Average-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 28 DL Average-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 29 UL Average-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 30 DL Average-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 31 UL Tail-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 32 DL Tail-UPT(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Based on the simualtion results, we can have the following observations:

- DL user perceived thoughput is degraded significantly once UL subband is configured in DL slots, no matter which kind of UPT is used.

  - The degradation of performance is caused by less available DL resources in each SBFD slot

  - The larger UL subband in a SBFD slot, the more serious impacts on DL user thoughput

- UL user perceived thoughput is improved with introduction of UL subband in SBFD slots.

  - Significant performance gain can be observed when UL subband is introduced. The gain increases with the UL subband size, i.e. a larger UL subband brings better UL user throughput

  - The gain obtained from SBFD is impacted by the capability of self-interference suppression. Self-interference is one of the major obstacles in-between to harvest benefits from SBFD.

- In ratio, improvement of UL performance is much more than the degradation of DL performance.

- The maximum UL user throughput depends on how much additional UL resources can be provided by UL subband. For example, the UL user perceived throughput of a UE is better when XXXXU is configured comparing with DXXXU.

- Self-interference isolation impacts the benefits we can harvest from SBFD. For 185 dB SI suppression capability and 145 dB SI suppression capability, the former assumption slightly outperforms the later one. In the other word, 145 dB SI suppression capability already provides enough isolation for self-interference

Hence similar observation for indoor/hotspot can be observed as dense urban scenario.

- 2.4.1.2 UL and DL latency

In this section, we provide results derived from following latency definition for DL and UL respectively.

- Option 1: Calculate the latency for each packet for each UE, and then generate CDF of latency for all these packets from all the UEs.

  - Packet-Latency CDF: The CDF of the packet latencies of all the packets from all the UEs.

  - Mean/5%/50%/95% Packet-Latency: The mean/5%/50%/95% value of Packet-Latency of all the packets from all the UEs.

- Option 2: Calculate the latency for each packet for each UE, and then calculate the average latency for each UE, then generate the CDF for these average latency for each UE

  - UE-Average-Latency: defined as the average packet latency for a UE

  - UE-Average-Latency CDF: The CDF of the UE-Average-Latency for all users.

  - Mean/5%/50%/95% UE-Average-Latency: The mean/5%/50%/95% value of UE-Average-Latency for all users.

Figure 33 UL Packet-latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 34 DL Packet-latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 35 UL UE-Average-Latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Figure 36 DL UE-Average-Latency(mean/5%/50%/95%) assuming different Target RU for legacy TDD

Based on the simualtion results, we can have the following observations:

- DL latecy is increased once UL subband is configured in DL slots no matter which kind of packet latency is adopted.

  - The degradation of performance comes from less available DL resources in each SBFD slot

  - The larger UL subband in a SBFD slot, the more serious impacts on DL latency

- UL latency is reduced with introduction of UL subband in SBFD slots no matter which kind of packet latency is adopted.

  - Significant performance gain can be observed when UL subband is introduced. The gain increases with the UL subband size, i.e. a larger UL subband brings better performance on latency.

  - The gain obtained from SBFD is impacted by the capability of self-interference suppression. Self-interference is one of the major obstacles in-between to harvest benefits from SBFD.

- In ratio, the magnitudes of improvement for UL performance is larger than degradation for DL performance.

- Self-interference isolation impacts the benefits we can harvest from SBFD. Lower UL latency can be achieved when larger self-interference isolation is assumed.

Hence similar observation for indoor/hotspot can be observed as dense urban scenario.

- 2.4.1.3 UL and DL resource utilization

Baseed on the simulation results, the DL and UL resource utilization at 5%, 50% and percentile are summarized in Figure 37 and Figure 38, respectively.

Figure 37 DL resource utilization for Indoor/Hotspot

Figure 38 UL resource utilization for Indoor/Hotspot

Based on the simualtion results, we can have the following observations:

- DL resource utilization is increased once UL subband is configured in DL slots across light traffic load, medium traffic load and high traffic load.

  - The increasement of DL resource ultilization comes from less available DL resources in each SBFD slot

- UL resource utilization is reduced with introduction of UL subband in SBFD slots across light traffic load, medium traffic load and high traffic load.

- Better self-interference isolation can further reduce UL resource ultilization.

Hence similar observation for indoor/hotspot can be observed as dense urban scenario.

SBFD technology can harvest more benefits in Indoor/Hotspot scenario compared with Dense Urban scenaro, thanks to low gNB power and good isolation.

Conclusion

In this contribution, we provide our views on evaluation on NR duplex evolution. We have the following observation:

- Channel model for indoor TRP to indoor UE in different buildings should also be considered in 2-layer Scenario B when option 2 is assumed for Indoor/outdoor UE proportion on Urban Macro layer.

- In option 1, it is too strict for UE implementation if UE ICS requirements are same as UE ACS, and reuse UE ACS in the SLS could not effectively evaluate the effects of UE-to-UE CLI at victim UE.

- ICSUE smaller than UE ACS could be used and could be reported by companies.

In option 2, Iselectivity model of inter-site gNB-gNB co-channel inter-subband CLI for receiver selectivity at victim gNB could be not reused for Rx impairment of victim UE.

A baseline combination is needed for the following key parameters for easy comparison among companies:

- UL/DL traffic generation

- FTP packet size

- Channel estimation

- BS transmit power

- UE-UE channel model

- gNB antenna architecture

- Slot configuration for legacy TDD

- Slot configuration for SBFD

- UE receiver

- Transmission scheme

For deployment case 2, the following two interference type should be take into account:

- gNB-UE co-channel inter-subband interference: Interfernece caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to DL reception of the victim UE on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

- UE-gNB co-channel inter-subband interference: Interfernece caused by UL transmission of the aggressor UE on a first set of RBs in a carrier to UL reception of the victim gNB on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

For co-site inter-sector co-channel inter-subband CLI modelling, inter-site gNB-gNB co-channel inter-subband CLI modelling, and self-interference modelling, the modelling used in SLS could be reused.

The self-interference co-site inter-sector co-channel inter-subband CLI and co-site gNB-gNB co-channel inter-subband CLI (only large-scale fading is considered) can be modelled as white Gaussian noise with given variance.

In the LLS, gNB-gNB channel model could reuse gNB-UE channel model (e.g., CDL-C) with some modifications.

When UL subband is introduced in DL slot, UL user throughput is improved significantly:

- Degradation of DL user thoughput is also observed, which depends on the UL subband configuration.

- In ratio, improvement of UL performance is much more than the degradation of DL performance.

- Better self-interference isolation achieves higher UL user throughput. 145 dB SI suppression can provide sufficient isolation between Tx and Rx at gNB side.

When UL subband is introduced in DL slot, UL lateny can be reduced significantly:

- Increasement of DL latency is also observed, which depends on the UL subband configuration.

- Better self-interference isolation brings lower UL latency.

When UL subband is introduced in DL slot, UL resource utilization can be reduced.

- Increasement of DL RU is also observed, which depends on the UL subband configuration.

- In ratio, reduction of UL performance is close to the increasement of DL performance.

- Better self-interference isolation achieves lower UL resource utilization.

SBFD technology can harvest more benefits in Indoor/Hotspot scenario compared with Dense Urban scenaro, thanks to low gNB power and good isolation.

Based on the aforementioned discussion and observation, we have the following proposals:

- When UE clustering distribution Alt 2 (M=20, X=2) is considered for Urban Macro and Dense Urban Macro layer in FR1, the penetration losses between two indoor UEs follow Table A.2.1-12 in TR38.802 with the following updates:

- Two indoor UEs are in the same building when they are in the same cluster.

- Two indoor UEs are in different buildings when they are in different clusters.

Channel model from serving indoor TRP to its facilitated UE in different buildings could reuse the channel model for indoor TRP to outdoor UE with modification of penetration loss.

- Indoor TRP to indoor UE in different buildings:

  - Option 1:

    - A.2.1.2 in TR36.843

    - Penetration loss between indoor TRP and indoor UE in different buildings follows Table A.2.1-12 in TR38.802

  - Option 2:

    - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1]

    - For Indoor factory layer: InF in TR 38.901

    - Two O2I penetration losses are modelled, wherein, O2I penetration loss follows TR 38.901

    - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered for each O2I penetration loss for indoor office layer.

    - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered for O2I penetration loss of indoor UE and 100% high-loss model is considered for O2I penetration loss of indoor factory TRP for indoor factory layer.

For Indoor TRP to indoor UE in the same building, indoor TRP to outdoor UE and indoor TRP to indoor UE in different buildings, the formula of calculating LOS probability is updated as below:

Indoor-Mixed office: PrLOS=1,d2D≤1.2mexp-d2D-1.24.71.2m<d2D<6.5mexp-d2D-6.532.6∙0.326.5m≤d2D.

Indoor-Open office:  PrLOS=1,d2D≤5mexp-d2D-570.85m<d2D<49mexp-d2D-49211.7∙0.5449m≤d2D

Where d2D is 2D distance between indoor TRP and its facilitated UE.

It should be clarified on whether two indoor UEs are in the same building if UE distribution option 2 is adopted for 2-layer scenario B.

- Two indoor UEs are in the same building:

  - Two indoor UEs are both in the indoor office/factory.

  - Two indoor UEs are both in the houses (not in the indoor office) and distance is less than 50m (according to Table A.2.1-12 in TR38.802).

- Two indoor UEs are in different buildings:

  - One indoor UE is in the indoor office/factory, another indoor UE is in the house (not in the indoor office).

  - Two indoor UEs are both in the houses (not in the indoor office) and distance is larger than 50m (according to Table A.2.1-12 in TR38.802).

Update the previous agreements for gNB-UE, gNB-gNB and UE-UE channel model for 2-layer Scenario B as below:

- Adopt the following table for gNB-UE channel model for 2-layer Scenario B (HetNet with Urban Macro and Indoor).

|  | gNB-UE channel model for 2-layer Scenario B |
| --- | --- |
| Large-scale channel parameters | FR1: - Macro TRP to Outdoor UE: UMa in TR 38.901 - Car penetration loss is modelled - Indoor TRP to Indoor UE in the same building: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - Penetration loss is not modelled. - Macro TRP to Indoor UE: UMa in TR 38.901 - O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - 100% high-loss model is considered if UE is in the factory. - Indoor TRP to Outdoor UE: - Option 1: - A.2.1.2 in TR36.843 - Penetration loss between indoor TRP and outdoor UE UEs follows Table A.2.1-12 A.2.1-13 in TR38.802 - Option 2: - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1] - For Indoor factory layer: InF in TR 38.901 - Both Car penetration (for outdoor UE) and O2I penetration loss are modelled, wherein, O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - 100% high-loss model is considered if Indoor TRP is in the factory. - Indoor TRP to indoor UE in different buildings: - Option 1: - A.2.1.2 in TR36.843 - Penetration loss between indoor TRP and indoor UE in different buildings follows Table A.2.1-12 in TR38.802 - Option 2: - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1] - For Indoor factory layer: InF in TR 38.901 - Two O2I penetration losses are modelled, wherein, O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered for each O2I penetration loss for indoor office layer. - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered for O2I penetration loss of indoor UE and 100% high-loss model is considered for O2I penetration loss of indoor factory TRP for indoor factory layer. |
| Fast fading parameters | FR1: - Macro TRP to Outdoor UE: UMa in TR 38.901 - Indoor TRP to Indoor UE in the same building: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - Macro TRP to Indoor UE: UMa in TR 38.901 - Indoor TRP to Outdoor UE and Indoor TRP to Indoor UE in different buildings: - Option 1: - 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1] - For Indoor factory layer: InF in TR 38.901 |

Adopt the following table for gNB-gNB channel model for 2-layer Scenario B (HetNet with Urban Macro and Indoor).

|  | gNB-gNB channel model for 2-layer Scenario B |
| --- | --- |
| Large-scale channel parameters | FR1: - Macro TRP to Macro TRP: not needed. - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered - For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). - For Indoor factory layer: InF in TR 38.901 (hUE =3m). - Penetration loss is not modelled. - Macro TRP to Indoor TRP: UMa in TR 38.901 (hUE =3m) - O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - 100% high-loss model is considered if Indoor TRP is in factory. - Indoor TRP to Macro TRP: same as Macro TRP to Indoor TRP |
| Fast fading parameters | FR1: - Macro TRP to Macro TRP: not needed. - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered. - For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). ASA and ZSA statistics updated to be the same as ASD and ZSD. - For Indoor factory layer: InF in TR 38.901 (hUE =3m). ASA and ZSA statistics updated to be the same as ASD and ZSD - Macro TRP to Indoor TRP: UMa O2I in TR 38.901 - Indoor TRP to Macro TRP: same as Macro TRP to Indoor TRP |

Adopt the following table for UE-UE channel model for 2-layer Scenario B (HetNet with Urban Macro and Indoor).

|  | UE-UE channel model for 2-layer Scenario B |
| --- | --- |
| Large-scale channel parameters | FR1: - Outdoor UE to Outdoor UE: - Option 1: A.2.1.2 in TR36.843 (*) - Option 2: UMi-Street canyon in TR 38.901 (hBS =1.5m) - Penetration loss between UEs follows Table A.2.1-13 A.2.1-12 in TR38.802 - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 1: A.2.1.2 in TR36.843 (*). - Option 2: - For Indoor office layer: InH-Office in TR 38.901 (hBS =1.5m). - For Indoor factory layer: InF in TR 38.901 (hBS =1.5m). - Penetration loss is not modelled. - Outdoor UE to Indoor UE: - Option 1: A.2.1.2 in TR36.843 (*). - Option 2: UMi-Street canyon in TR 38.901 (hBS =1.5m). - Penetration loss between UEs follows Table A.2.1-13 A.2.1-12 in TR38.802 - 100% high-loss model is considered if Indoor UE is in factory |
| Fast fading parameters | FR1: - Outdoor UE to Outdoor UE: - Option 1: 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: UMi-Street canyon in TR 38.901, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 1: A.2.1.2 in TR36.843 (ITU InH), ASD statistics updated to be the same as ASA. - Option 2: - For Indoor office layer: InH-Office in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. - For Indoor factory layer: InF in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Outdoor UE to Indoor UE: - Option 1: 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: UMi-Street canyon in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. |
| (*): For outdoor to indoor case, and indoor to indoor case, use “Remaining Layout Options” in A.2.1.2 of TR36.843 for pathloss calculation, and “ITU-R IMT UMi” for LOS Probability derivation. For outdoor to indoor case, the penetration loss term “20.0+0.5* din” is excluded in pathloss formula given in A.2.1.2 of TR36.843, and the penetration loss is derived according to Table A.2.1-13 A.2.1-12 in TR38.802. |  |

The following criterion could be used to determine whether two indoor UEs are in the same building or not for UE-UE penetration loss calculation:

- For Urban Macro and Dense Urban Macro layer in FR1,

  - If UE clustering distribution is used, two indoor UEs are in the same building when they are in the same cluster and two indoor UEs are in different buildings when they are in different clusters.

  - If uniform UE distribution is used, two indoor UEs are in the same building when inter-UE d2D≤50m and two indoor UEs are in different buildings when inter-UE d2D>50m.

- For Dense Urban Macro layer in FR2-1,

  - If UE clustering distribution is used, all UEs are outdoor UE.

  - If uniform UE distribution is used, two indoor UEs are in the same building when inter-UE d2D≤50m and two indoor UEs are in different buildings when inter-UE d2D>50m.

- For InH in FR1/FR2-1, all UEs are in the same building.

- For 2-layer scenario B in FR1,

  - For baseline UE distribution, all users are outdoor UEs without car penetration for Urban Macro Layer. All users are randomly and uniformly dropped within the building for Indoor office layer. All indoor UEs are in the same building.

  - For optional UE distribution, 80% indoor in houses and 20% outdoor in cars are considered for Urban Macro Layer. All users are randomly and uniformly dropped within the building for Indoor office layer.

    - Two indoor UEs are in the same building:

      - Two indoor UEs are both in the indoor office/factory.

      - Two indoor UEs are both in the houses (not in the indoor office) and inter-UE d2D≤50m.

    - Two indoor UEs are in different buildings:

      - One indoor UE is in the indoor office/factory, another indoor UE is in the house (not in the indoor office).

      - Two indoor UEs are both in the houses (not in the indoor office) and inter-UE d2D>50m.

- For Dense Urban with 2-layer in FR1, two indoor UEs are in the same building when inter-UE d2D≤50m and two indoor UEs are in different buildings when inter-UE d2D>50m.

For O2I building penetration loss of UE-UE channel model, PLin=0.5d2D-in, where d2D-in is updated as below:

- d2D-in=min{U1(0, min(d2D2,25m)), U2(0, min(d2D2,25m))} and d2D is inter-UE 2D distance.

When channel model of UMi-Street canyon in TR 38.901 is used for UE-to-UE link, d2D-out can be interpreted as follow in the LOS probability calculation:

- For outdoor UE to outdoor UE and indoor UE to indoor UE in the same building, d2D-out= d2D.

- For indoor UE to outdoor UE and outdoor UE to indoor UE, d2D-out= d2D-d2D-in.

- For indoor UE to indoor UE in different building, d2D-out= d2D-d2D-in#1-d2D-in#2, where where d2D-in#1 is distance between indoor UE#A and the wall of building#1 and d2D-in#2 is distance between indoor UE#B and the wall of building#2.

When channel model of InH-Office in TR 38.901 is used for UE-to-UE link, the LOS probability is calculated as follows.

Indoor-Mixed office: PrLOS=1,d2D≤1.2mexp-d2D-1.24.71.2m<d2D<6.5mexp-d2D-6.532.6∙0.326.5m≤d2D.

Indoor-Open office:  PrLOS=1,d2D≤5mexp-d2D-570.85m<d2D<49mexp-d2D-49211.7∙0.5449m≤d2D

Car penetration related to outdoor UE should not be modelled when UE distribution of 100% outdoor UE without car penetration loss is assumed.

- The penetration losses between UEs in Dense Urban Macro layer for FR2-1 and penetration losses between outdoor UE to outdoor UE, outdoor UE to indoor UE in 2-layer Scenario B follow Table A.2.1-12 in TR38.802 with modification, where car penetration loss should not be modeled for outdoor UE.

- For Macro to outdoor UE and Indoor TRP to outdoor UE in 2-layer Scenario B, car penetration loss should not be modeled for outdoor UE if Indoor/outdoor proportion option 1 is assumed.

Regarding layout of 2-layer Scenario B (HetNet with Urban Macro and Indoor factory), small hall size 120m x60 m (sub-cases InF-SL and InF-DH) should be considered. Besides, the simulation assumptions, e.g. BS deployment, BS height, Clutter density, clutter height and clutter size, in TR 38.901 should be reused.

- Layer 1: Urban Macro

  - Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around, ISD=500m

- Layer 2: Indoor factory (optional)

  - Only one building randomly dropped in the whole network as in the figure below. The building has to be confined within one macro cell area.

    - 18 (baseline) or 3 (optional) TRPs are considered.

    - The hall size is 120m x 60m for InF-SL and InF-DH.

    - The distance between two indoor TRPs: 20m for 18 TRPs, 40m for 3 TRPs

    - The height of indoor TRP is 1.5m for InF-SL and 8m for InF-DH.

  - The orientation of the building is fixed as in the figure below (i.e., the long side of the rectangular is along the x-axis).

  - Clutter density, clutter height and clutter size follow Table 7.8-8 in TR 38.901.

- The minimum 2D distance between macro TRP and indoor factory center is 100m

- The minimum 2D distance between macro TRP and indoor/outdoor UE is 35m

For UE clustering distribution in SBFD Deployment Case 4, option 2 is preferred.

- Grid shift of two operator can be 0% and 100%.

- For 100% grid shift case, cluster should locate within the common area between operator A and operator B.

In the inter-site gNB-gNB co-channel inter-subband CLI modelling when both large fading and small fading are considered, the following values of ACLRBS and ACSBS can be assumed for simulation purpose.

- In FR1, ACLR and ACS of gNB are 45dB and46dB, respectively.

- In FR2-1, ACLR and ACS of gNB are 28dB and 23.5dB, respectively.

For SLS in RAN1, regarding Tx leakage model of UE-UE co-channel inter-subband CLI modelling, Option 1 can be used to Tx leakage when only large-scale fading is modelled for UE-UE co-channel inter-subband CLI.

For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for UE-UE co-channel model, the power of UE-UE co-channel inter-subband CLI experienced by the victim UE on each receiver chain at DL RB n can be modelled as IUE-UE-CLIA'→A, n=IleakageA'→A, n+Iselectivity, where

- Iselectivity=PtxUE A',per-RB*CLUE A'→UE A*1ICSUE* Nused-UL-RBUE A'NULRB

  - Iselectivity is the second part of UE-UE co-channel inter-subband CLI from UE A' to UE A on each receiver chain at DL RB n (linear value), caused by in-channel selectivity at the victim UE

  - PtxUE A',per-RB is UL transmission power of UE A' across all transmit chains per RB (linear value). PtxUE A',per-RB=PtxUE A'/Nused-UL-RBUE A'.

  - Nused-UL-RBUE A' is the number of UL RBs allocated for UL transmission of UE A'

  - NULRB is the total number of UL RBs in the UL subbands

  - CLUE A'→UE A is the coupling loss between UE A' and UE A (linear value), accounting for beamforming at the aggressor UE and victim UE.

  - Note: ICSUE is in linear scale, is the in-channel selectivity of victim UE A.

For SLS in RAN1, if both large scale fading and small scale fading are modelled for UE-UE co-channel channel model, the UE-UE co-channel inter-subband CLI signal across all Rx chains at DL RB n at victim UE can be modeled as IUE-UE-CLIn=Ileakagen+Iselectivity, where,

Iselectivity=z0n,z1n,…,zNR-1nT

- zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise

- σz,n2=PblockerICSUE×NULRB

- Pblocker=1NR*m∈Used UL RBsHCLImWmsm2

- HCLIm is the NR×NT channel matrix between aggressor UE and victim UE at UL RB m, the analog beams of the aggressor UE and the victim UE can be taken into account by HCLIm,

- Wm is the digital precoder at UL RB m at aggressor gNB, WmF=1,

- sm is the symbol transmitted at UL RB m at aggressor UE with transmission power for each layer as PtxUE A',per-RB=PtxUE A'/Nused-UL-RBUE A'.

- Nused-UL-RBUE A' is the number of UL RBs allocated for UL transmission of UE A'

Study the Rx impairment model if RAN1 assumes no UE in-channel selectivity.

For inter-UE inter-subband CLI modeling when option 2 is applied, the blocker interference at the UL subband of the victim UE can be modelled as increase of quantization noise which affects the DL SIQNR when blocker power is higher than RSSI of the DL signal.

  - ΔQndB ≈Ix-RSSI

  - SIQNR= SPNp+Iz+ Qn+ ΔQn(Ix),

where

- Ix is the strength of blocking interference from aggressor UE in dB,

- RSSI is the power of received DL signal in dB,

- ΔQndB is the increased quantization noise in dB,

- Iz is the strength of Tx leakage of UE-UE inter-subband CLI from aggressor UE in linear scale,

- SP is the power of signal in linear scale,

- Np is the thermal noise in linear scale,

- Qn is the quantization noise in linear scale,

- ΔQn(Ix) is the increased quantization noise in linear scale, respectively

The effect of quantization noise to the receiver performance can be assumed to be same with thermal noise in the SLS.

FFS: The process to obtain quatization noise Qn should be determined.

Update the simulation assumptions for SLS calibration as below:

|  | Urban Macro (FR1) |  | Dense Urban Macro Layer (FR2-1) | Indoor office (FR1) | Indoor office (FR2-1) |
| --- | --- | --- | --- | --- | --- |
| Carrier frequency | 4 GHz |  | 30GHz | 4 GHz | 30GHz |
| System bandwidth | 100MHz |  | 100MHz | 100MHz | 100MHz |
| Numerology | 14 OFDM symbol slot SCS = 30kHz |  | 14 OFDM symbol slot SCS = 120kHz | 14 OFDM symbol slot SCS = 30kHz | 14 OFDM symbol slot SCS = 120kHz |
| BS transmit power for SBFD | 53 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD |  | 40 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | 24 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | 23 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD |
|  | Assume the BS transmit power spectrum density is kept the same for SBFD operation and legacy TDD operation. BS transmit power is proportional to the RBs used for DL transmission. |  |  |  |  |
| UE Tx power | 23dBm |  | 23 dBm. EIRP should not exceed 43 dBm | 23dBm | 23 dBm. EIRP should not exceed 43 dBm |
| Layout | Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around |  |  | 12 TRPs per 120m x 50m x 3m |  |
| Inter-BS (2D) distance | 500m |  | 200m | 20m |  |
| Minimum BS-UE (2D) distance | 35m |  | 35m | 0m |  |
| Minimum UE-UE (2D) distance | 1m |  | 1m | 1m |  |
| BS antenna height | 25m |  | 25m | 3m |  |
| UE distribution | UE Clustering |  | UE Clustering | Uniform |  |
| UE number per macro/indoor TRP (per direction) (M) | 20 |  | 10 | 10 |  |
| UE cluster number per macro cell (X) | 2 |  | 1 | - |  |
| UE outdoor/indoor proportion | 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h |  | 100% outdoor without car penetration loss: 3km/h | 100% indoor in houses: 3km/h |  |
| UE number per cluster | 8 |  | 8 | - |  |
| Indoor UE height (m) | 1.5m |  | 1.5m | 1.5m |  |
| Outdoor UE height (m) | 1.5m |  | 1.5m | - |  |
| Radius of cluster (R) | 25m |  | 20m | - |  |
| Minimum distance between macro TRP to UE cluster center (Dmacro-to-cluster) | 60m |  | 55m | - |  |
| Minimum distance between two UE cluster centers (Dinter-cluster) | 50m |  | - | - |  |
| gNB-UE Channel model (large-scale) | Macro-to-UE: UMa in TR 38.901 For FR1, gNB-UE O2I penetration loss: 80% low-loss model, 20% high-loss model |  |  | TRP-to-UE: InH-Office in TR 38.901 Penetration loss is not modelled. |  |
| gNB-gNB Channel model (large-scale) | Macro-to-Macro: UMa in TR 38.901 (hUE =25m) LOS probability: If the 2D distance between two Macro gNBs are less than or equal to the ISD, set the LOS probability to 0.75; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. |  |  | TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) Penetration loss is not modelled. |  |
| UE-UE Channel model (large-scale) | UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m). For FR1, penetration loss between UEs follows Table A.2.1-12 in TR38.802 |  |  | UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) |  |
| BS antenna array configuration for Legacy TDD | M,N,P,Mg,Ng;Mp,Np= (8,8,2,1,1;2,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization | M,N,P,Mg,Ng;Mp,Np= (4,16,2,2,2; 1,1) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization |  | M,N,P,Mg,Ng;Mp,Np = (4,4,2,1,1; 4,4) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization | M,N,P,Mg,Ng;Mp,Np=(16,8,2,1,1; 1,1) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization |
| BS antenna array configuration for SBFD | - SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (8,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ |  | - SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (4,16,2,2,2). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ | - SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (4,4,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ | - SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (16,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ |
| BS antenna radiation pattern | Reuse Table 9 in Report ITU-R M.2412 |  |  | Reuse Table 10 in Report ITU-R M.2412 |  |
| UE antenna configuration | - 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization - 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |  | 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1); (dH,dV) = (0.5,0.5)λ,(dg,V,dg,H) = (0, 0)λ, 0°/90° polarization; Θmg,ng=90°; Ω0,1=Ω0,0+180° | - 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization - 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization | 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1); (dH,dV) = (0.5,0.5)λ,(dg,V,dg,H) = (0, 0)λ, 0°/90° polarization; Θmg,ng=90°; Ω0,1=Ω0,0+180° |
| UE antenna radiation pattern | Omni-directional with 0 dBi element gain |  | reuse Table 11 in Report ITU-R M.2412 | Omni-directional with 0 dBi element gain | reuse Table 11 in Report ITU-R M.2412 |
| BS receiver noise figure | 5dB |  | 7 dB | 5dB | 7 dB |
| UE receiver noise figure | 9 dB |  | 13 dB | 9 dB | 13 dB |
| Open loop power control parameters | P0= -80 dBm, alpha = 0.8 |  | P0= -86 dBm, alpha = 0.9 | P0= -60 dBm, alpha = 0.6 |  |
| Handover margin (dB) | 3 dB |  | 3 dB | 3 dB | 3 dB |
| UE attachment | Based on RSRP from port 0 |  | Based on RSRP from port 0. - Out of the two UE panels, the UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. - Single gNB panel is used for UE attachment | Based on RSRP from port 0 | Based on RSRP from port 0. - Out of the two UE panels, the UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. - Single gNB panel is used for UE attachment |
| Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 |  |  |  |  |
| Boresight direction | 0°, 120° and 240° for three sectors |  |  | 90° |  |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |  | 90° in GCS (pointing to horizontal direction) | 180° in GCS (pointing to the ground) | 180° in GCS (pointing to the ground) |
| Electronic tilt | (According to Zenith angle in "Beam set at TRxP") |  | (According to Zenith angle in "Beam set at TRxP") | 90° in LCS | (According to Zenith angle in "Beam set at TRxP") |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = 0 Zenith angle θj = pi*102/180 NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |  | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = {-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16} Zenith angle θj = {5*pi/8, 7*pi/8} NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | - | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = {-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16} Zenith angle θj = {pi/4, 3*pi/4} NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | - |  | For direction of UE analog beam steering (in LCS): Azimuth angle φi = {-3*pi/8, -pi/8, pi/8, 3*pi/8}; Zenith angle θj = {pi/4, 3*pi/4}; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | - | For direction of UE analog beam steering (in LCS): Azimuth angle φi = {-3*pi/8, -pi/8, pi/8, 3*pi/8}; Zenith angle θj = {pi/4, 3*pi/4}; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |

Dynamic TDD is not used for legacy TDD for comparison.

The parameters considered in simulation assumptions for SLS calibration can be used performance evaluation:

- BS transmit power

- UE-UE channel model

- gNB antenna architecture

Except for the parameters considered in simulation assumptions for SLS calibration, other parameters can be used for the baseline combination is shown in table 1 and table 2 in appendix.

Reference

  - RAN1 Chairman notes, RAN1#111 meeting

  - R1-2210779, Final summary on evaluation on NR duplex evolution, Moderator (CMCC)

  - R4-2214379, WF for co-existence study (CMCC)

  - R4-2214379, WF on Simulation assumption for adjacent co-existence study WF for co-existence study (CMCC)

Appendix

Other parameters used in the baseline combination

Table 1 Other parameters used in the baseline combination for Urban Macro (FR1), Dense Urban Macro Layer (FR2-1) and indoor office (FR1 &FR2-1)

|  | Urban Macro (FR1) | Dense Urban Macro Layer (FR2-1) | Indoor office (FR1) | Indoor office (FR2-1) |
| --- | --- | --- | --- | --- |
| Slot configuration for legacy TDD | Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] |  |  |  |
| SBFD and slot configuration | 273 PRB: < ND, NU, NG > = <104, 55, 5> Frame structure#2 (XXXXU), where X denotes a SBFD slot. | 66 PRB: < ND, NU, NG > = <25, 14, 1> Frame structure#2 (XXXXU), where X denotes a SBFD slot. | 273 PRB: < ND, NU, NG > = <104, 55, 5> Frame structure#2 (XXXXU), where X denotes a SBFD slot. | 66 PRB: < ND, NU, NG > = <25, 14, 1> Frame structure#2 (XXXXU), where X denotes a SBFD slot. |
| gNB-UE Channel model (small-scale) | Macro-to-UE: UMa in TR 38.901 |  | TRP-to-UE: InH-Office in TR 38.901 |  |
| gNB-gNB Channel model (small-scale) | Macro-to-Macro: UMa O2O in TR 38.901 (hUE =25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 |  | TRP-to-TRP: InH-Office in TR 38.901 (hUE=3m), ASA and ZSA statistics updated to be the same as ASD and ZSD |  |
| UE-UE Channel model (small-scale) | UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA. |  | UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA |  |
| Traffic model | General: UL and DL are simulated simultaneously. Option 2: Each UE is assigned both UL traffic and DL traffic. FTP packet size: - Option 1: Symmetric packet size: - 1Kbyte for DL/UL, 0.1Mbytes for DL/UL, 0.5Mbytes for DL/UL, 2Mbytes for DL/UL UL arrival rate for legacy TDD: - The UL arrival rate is selected to reach a target UL traffic load (RU). - UL Traffic load: low UL RU ([<10%]), medium UL RU ([20%-30%]), and high UL RU ([~50%]). - Note: Type-2 RU definition (calculated per link direction) is used DL arrival rate for legacy TDD: - The DL arrival rate is selected to reach a target DL traffic load (RU). - DL Traffic load: low DL RU ([<10%]), medium DL RU ([20%-30%]), and high DL RU ([~50%]). - Note: Type-2 RU definition (calculated per link direction) is used Arrival rate for SBFD: The UL and DL FTP packet arrival rate for SBFD are the same as legacy TDD. |  |  |  |
| BS receiver | MMSE-IRC |  |  |  |
| UE receiver | MMSE-IRC |  |  |  |
| Feedback assumption | Ideal |  |  |  |
| Channel estimation | Ideal |  |  |  |
| UE processing capability | UE processing capability 1 |  |  |  |
| DL/UL modulation | Up to 256QAM |  |  |  |
| Transmission scheme | Consider SU-MIMO with single layer for both DL and UL |  |  |  |
| Scheduling | PF |  |  |  |

Table 2 Other parameters used in the baseline combination for 2-layer Scenario B

|  | 2-layer Scenario B |
| --- | --- |
| Carrier frequency | 4GHz |
| System bandwidth | 100Mhz |
| Numerology | 14 OFDM symbol slot SCS = 30kHz |
| BS transmit power for SBFD | Layer 1: Urban Macro 53 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD Layer 2: Indoor office 24 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD |
|  | Assume the BS transmit power spectrum density is kept the same for SBFD operation and legacy TDD operation. BS transmit power is proportional to the RBs used for DL transmission. |
| UE Tx power | 23dBm |
| Layout | Layer 1: Urban Macro - Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around Layer 2: Indoor office - 12 TRPs per 120m x 50m x 3m - Only one building randomly dropped in the whole network as in the figure below. The building has to be confined within one macro cell area. - The orientation of the building is fixed as in the figure below (i.e., the long side of the rectangular is along the x-axis) |
| Inter-BS (2D) distance | Macro-to-macro: 500m Minimum macro TRP-to-indoor-office centre: 100m Indoor TRP-Indoor -TRP: 20m for 12 TRPs |
| Minimum BS-UE (2D) distance | Macro-to-UE: 35m Indoor TRP-to-UE:0m |
| Minimum UE-UE (2D) distance | 1m |
| BS antenna height | 25m for macro cells and 3m for indoor office |
| UE distribution | Layer1: Urban Macro - Uniform distribution outside the Indoor office Layer2: Indoor office - Uniform distribution in the Indoor office |
| UE number per macro/indoor TRP (per direction) (M) | 10 |
| UE cluster number per macro cell (X) | - |
| UE outdoor/indoor proportion | Layer 1: Urban Macro - 100% outdoor without car penetration loss, 3km/h Layer 2: Indoor office - 100% indoor in the building, 3km/h |
| Indoor UE height (m) | 1.5m |
| Outdoor UE height (m) | 1.5m |
| Radius of cluster (R) | - |
| Minimum distance between macro TRP to UE cluster center (Dmacro-to-cluster) | - |
| Minimum distance between two UE cluster centers (Dinter-cluster) | - |
| gNB-UE Channel model (large-scale) | - Macro TRP to Outdoor UE: UMa in TR 38.901 - Penetration loss is not modelled - Indoor TRP to Indoor UE in the same building: - InH-Office in TR 38.901 - Penetration loss is not modelled. - Macro TRP to Indoor UE: UMa in TR 38.901 - O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - Indoor TRP to Outdoor UE: - InH-Office in TR 38.901 - O2I penetration loss are modelled, wherein, O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - Indoor TRP to Indoor UE in different buildings: - InH-Office in TR 38.901 - Two O2I penetration losses are modelled, wherein, O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. |
| gNB-UE Channel model (small-scale) | - Macro TRP to Outdoor UE: UMa in TR 38.901 - Indoor TRP to Indoor UE in the same building: - InH-Office in TR 38.901 - Macro TRP to Indoor UE: UMa in TR 38.901 - Indoor TRP to Outdoor UE: - Indoor TRP to Indoor UE in different buildings: - InH-Office in TR 38.901 |
| gNB-gNB Channel model (large-scale) | - Macro TRP to Macro TRP: not needed. - Indoor TRP to Indoor TRP in the same building: - InH-Office in TR 38.901 - Penetration loss is not modelled. - Macro TRP to Indoor TRP: UMa in TR 38.901 (hUE =3m) - O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - Indoor TRP to Macro TRP: same as Macro TRP to Indoor TRP |
| gNB-gNB Channel model (small-scale) | - Macro TRP to Macro TRP: not needed. - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered. - For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). ASA and ZSA statistics updated to be the same as ASD and ZSD. - Macro TRP to Indoor TRP: UMa O2I in TR 38.901 - Indoor TRP to Macro TRP: same as Macro TRP to Indoor TRP |
| UE-UE Channel model (large-scale) | - Outdoor UE to Outdoor UE: - UMi-Street canyon in TR 38.901 (hBS =1.5m) - Penetration loss is not modeled - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 2: - InH-Office in TR 38.901 (hBS =1.5m). - Penetration loss is not modelled. - Outdoor UE to Indoor UE: - Option 2: UMi-Street canyon in TR 38.901 (hBS =1.5m). - Penetration loss between UEs follows Table A.2.1-12 in TR38.802 - Car penetration loss is not modeled for outdoor UE |
| UE-UE Channel model (small-scale) | - Outdoor UE to Outdoor UE: - UMi-Street canyon in TR 38.901, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 2: - InH-Office in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Outdoor UE to Indoor UE: - Option 2: UMi-Street canyon in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. |
| Slot configuration for legacy TDD | Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] |
| SBFD and slot configuration | 100MHz channel bandwidth and 30kHz SCS (273 PRB): < ND, NU, NG > = <104, 55, 5> Frame structure#2 (XXXXU), where X denotes a SBFD slot. |
| Traffic model | General: UL and DL are simulated simultaneously. Option 2: Each UE is assigned both UL traffic and DL traffic. FTP packet size: - Option 1: Symmetric packet size: - 1Kbyte for DL/UL, 0.1Mbytes for DL/UL, 0.5Mbytes for DL/UL, 2Mbytes for DL/UL UL arrival rate for legacy TDD: - The UL arrival rate#1 of Macro cell and UL arrival rate#2 of Indoor TRP are selected to reach target UL traffic load (RU)#1 of Macro cell and target UL traffic load (RU)#2 of Indoor TRP, respectively - UL Traffic load: low UL RU ([<10%]), medium UL RU ([20%-30%]), and high UL RU ([~50%]). - Note: Type-2 RU definition (calculated per link direction) is used DL arrival rate for legacy TDD: - The DL arrival rate#1 of Macro cell and DL arrival rate#2 of Indoor TRP are selected to reach target DL traffic load (RU)#1 of Macro cell and target DL traffic load (RU)#2 of Indoor TRP, respectively - DL Traffic load: low DL RU ([<10%]), medium DL RU ([20%-30%]), and high DL RU ([~50%]). - Note: Type-2 RU definition (calculated per link direction) is used Arrival rate for SBFD of Indoor TRP: The UL and DL FTP packet arrival rate for SBFD are the same as legacy TDD. |
| BS antenna array configuration for Legacy TDD | Layer 1: Urban Macro M,N,P,Mg,Ng;Mp,Np=(8,8,2,1,1;2,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization Layer 2: Indoor office M,N,P,Mg,Ng;Mp,Np = (4,4,2,1,1; 4,4) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization |
| BS antenna array configuration for SBFD | Layer 2: Indoor office SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (4,4,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ |
| BS antenna radiation pattern | Layer 1: Urban Macro - Reuse Table 9 in Report ITU-R M.2412 layer 2: Indoor office - Reuse Table 10 in Report ITU-R M.2412 |
| UE antenna configuration | - 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization - 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |
| UE antenna radiation pattern | Omni-directional with 0 dBi element gain |
| BS receiver noise figure | 5dB |
| UE receiver noise figure | 9 dB |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Feedback assumption | Ideal |
| Channel estimation | Ideal |
| UE processing capability | UE processing capability 1 |
| Open loop power control parameters | Layer 1: Urban Macro - P0= -80 dBm, alpha = 0.8 Layer 2: Indoor office - P0= -60 dBm, alpha = 0.6 |
| Handover margin (dB) | 3 dB |
| UE attachment | Based on RSRP from port 0 |
| Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 |
| DL/UL modulation | Up to 256QAM |
| Transmission scheme | Consider SU-MIMO with single layer for both DL and UL |
| Scheduling | PF |
| Mechanic tilt | Layer 1: Urban Macro - 90° in GCS (pointing to horizontal direction) Layer 2: Indoor office - 180° in GCS (pointing to the ground) |
| Electronic tilt | Layer 1: Urban Macro - According to Zenith angle in "Beam set at TRxP" Layer 2: Indoor office - 90° in LCS |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | Layer 1: Urban Macro For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = 0 Zenith angle θj = pi*102/180 NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 Layer 2: Indoor office - |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | - |

Simulation assumptions for SBFD SLS

Table 3 Simulation parameters for dense urban

| Parameters | Values |
| --- | --- |
| Layout | Single layer with 7 hexagonal cell with wrap around |
| Minimum BS-UE (2D) distance | 35 m: TR38.828 |
| Minimum UE-UE (2D) distance | 3 m: TR38.828 |
| Inter-BS distance | 200 m |
| Carrier frequency | 4 GHz |
| BS Tx power | 44 dBm for 100MHz [refer to TR 38.802 Table A.2.1-1] |
| UE Tx power | 23 dBm for 100MHz TR38.802 |
| Path-loss model | -Macro(Aggressor) → Macro(Victim) - BS-to-BS: TR38.901 - BS-to-UE: TR38.901 - UE-to-UE: Option 1: A.2.1.2 in TR36.843(*), penetration loss between UEs follows Table A.2.1-13 in TR38.802 |
| BS antenna configurations | For legacy TDD gNB: M,N,P,Mg,Ng;Mp,Np= (8,8,2,1,1;2,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization For SBFD gNB: - SBFD antenna configuration option-2 (Method 2-1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (8,8,2,1,1). - Number of TxRUs: same as legacy TDD dH,dV = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ |
| BS antenna height | 25 m: TR38.802 |
| BS antenna element gain + connector loss | 5 dBi : TR38.828 |
| BS receiver noise figure | 5 dB: TR38.802 |
| UE antenna configuration | Omni: TR38.802 |
| UE antenna height | hUT=3(nfl-1)+1.5 nfl for outdoor UEs: 1 nfl for indoor UEs: nfl~uniform(1,Nfl) where Nfl ~uniform(4,8) TR36.873 |
| UE antenna gain | 0 dBi: TR38.828 |
| UE receiver noise figure | 9 dB: TR38.802 |
| UE power control | Power control as defined in TR 37.910, (p0=-86dBm, alpha=0.9) |
| Cell selection criteria | Cell selection is based on coupling loss |
| Traffic model | FTP model 3 Packet size: 0.5Mbytes Arrivial rate(DL/UL): 0.2/0.1, 1.5/0.25, 3/0.5 |
| Output | UL/DL UPT (5%,50%, 95%), UL/DL Latency, Resource utilization |
| TDD UL-DL configuration | DDDSU |
| Subband configuration | XXXXU, DXXXU; Structure of flexible slot: 12D2G0U UL subband portion in system bandwidth: 50% |
| SI suppression capability | Advanced capability#1: 145 dB Advanced capability#2: 185 dB Note: legacy gNB-gNB ACIR is incorporated in the above capability |
| Other assumption | Reuse SI suppression capability for inter-sector CLI |
| Antenna architecture | The number of antenna element for SBFD gNB is two times of that for legacy TDD gNB. The number of antenna element for each direction is same as that of legacy TDD |
| Simulaltion bandwidth | 100MHz Agreement Note: Tx power needs to be scaled accordingly |
| UE distribution | Clustering based, refer to chairman notes. Number of UE cluster=1, Dmacro-to-cluster=35+Micro Radius, Dinter-cluster,=2* Micro Radius Note: Micro Radius =28.9 |
| gNB-to-gNB LOS probability | If the 2D distance between two Macro gNBs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to X; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. - X = 0.75 |
| Number of UEs per Macro TRP | 20 Note: 20% UEs are distributed in Macro area 80% UEs are distributed in UE clusters |

Table 4 BS antenna element pattern for dense urban

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=65°,SLAV=25dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=25dB |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi (assuming 1.8dB loss) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 3: A 65 degree horizontal element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation results. |  |

Table 5 Simulation parameters for indoor

| Parameters | Indoor |
| --- | --- |
| Layout | Single layer with 12 BSs per 120m x 50m |
| Minimum BS-UE (2D) distance | 0 m: TR38.802 |
| Minimum UE-UE (2D) distance | 1 m: TR38.828 |
| Inter-BS distance | 20 m TR38.802 |
| Carrier frequency | 4 GHz |
| BS Tx power | 24 dBm for 100MHz |
| UE Tx power | 23 dBm: TR38.828 |
| Path-loss model | - BS-to-BS: TR38.901 - BS-to-UE: TR38.901 - UE-to-UE: Option 1: A.2.1.2 in TR36.843(*), penetration loss between UEs follows Table A.2.1-13 in TR38.802 |
| BS antenna configurations | For legacy TDD gNB: M,N,P,Mg,Ng;Mp,Np= (4,4,2,1,1;4,4) dH,dV = (0.5, 0.5)λ, +45°/-45° polarization For SBFD gNB: - SBFD antenna configuration option-2 (Method 2-1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (4,4,2,1,1). - Number of TxRUs: same as legacy TDD dH,dV = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ |
| BS antenna height | 3 m: TR38.802 |
| BS antenna element gain + connector loss | 3.5 dBi : TR38.828 |
| BS receiver noise figure | 5 dB: TR38.802 |
| UE antenna configuration | Omni: TR38.802 |
| UE antenna height | hUT=1.5 m TR38.802 |
| UE antenna gain | 0 dBi: TR38.828 |
| UE receiver noise figure | 9 dB: TR38.802 |
| UE power control | Power control as defined in TR37.910, (p0=-60dBm, alpha=0.6) |
| Cell selection criteria | Cell selection is based on coupling loss |
| Traffic model | FTP model 3 Packet size: 0.5Mbytes Arrivial rate(DL/UL): 0.8/0.2, 2.5/0.6, 4.5/ 1.1 |
| Output | UL/DL UPT (5%,50%, 95%), UL/DL Latency, Resource utilization |
| TDD UL-DL configuration | DDDSU |
| Subband configuration | XXXXU, DXXXU; Structure of flexible slot: 12D2G0U UL subband portion in system bandwidth: 50% |
| SI suppression capability | Advanced capability#1: 145 dB Advanced capability#2: 185 dB Note: legacy gNB-gNB ACIR is incorporated in the above capability |
| Other assumption | Reuse SI supporession capability for inter-sector CLI |
| Antenna architecture | The number of antenna element for SBFD gNB is two times of that for legacy TDD gNB. The number of antenna element for each direction is same as that of legacy TDD |
| Simulaltion bandwidth | 100MHz Note: Tx power needs to be scaled accordingly |
| UE distribution | 100% Indoor |
| gNB-to-gNB LOS probability | TR38.901 |
| gNB-to-UE LOS probability | TR38.901 |
| Number of UEs per Macro TRP | 10 UEs per BS TR38.802 |