18573GPP TSG RAN WG1 #112	R1-2300700

Athens, Greece, 27th February – 3rd March, 2023

Agenda item:		9.13.3

Title:	L1 signal design and procedures for low power WUS

Source:	Nokia, Nokia Shanghai Bell

Document for:		Discussion and Decision

# Introduction

At the RAN#94-e meeting, it was agreed to study Wake Up Signal and Receivers designs.  These designs are to be primarily targeted at delay and power-sensitive, small form-factor devices, such as industrial sensors, controllers and wearables.  Unlike previous power saving study items, the objectives (see [1]) for this study encompasses new signals and new receiver architectures.

The study item includes the following objectives: - Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

In RAN1#111 meeting, following agreements were made, which are provided below. The highlighted sections are the ones addressed in this technical document.

Agreement - Study generation and link performance of multi-carrier (MC)-ASK (including OOK) waveform - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS. - Note that above does not preclude DFT-S-OFDMA - Study generation and link performance of multi-carrier (MC)-FSK waveforms - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS. - Study link performance of OFDMA-based signals/channels considering at least the existing signal/channel structure (e.g. CSI-RS, SSS) - Other signal/channel structures are not precluded - For next meeting, companies to provide input on aspects to consider that might impact link performance Agreement For the purpose of study, the BW of one LP-WUS is not greater than X (FFS X is 5 or 20) MHz for FR1, study further - whether BW of LP-WUS is configurable (implicitly or explicitly) - size of guard band [FFS: within or outside of BW X], if any - whether there is different X for Idle, Connected, Inactive modes FFS: Whether FR2 is included in the scope of LP-WUS SI Agreement For a UE support LP-WUR in IDLE/INACTIVE mode, - Study how to reduce UE power consumption due to existing RRM measurement requirements at least for mobility support, - study feasibility of RRM measurements performed by LP-WUR, at least for serving/camping cell, based on signals detected by LP-WUR - FFS: measurement metric - FFS: whether and how to identify cell/ tracking area - FFS: need for neighbouring cells - FFS: need for relaxation of existing RRM measurement requirements (for UE)

In this contribution, we provide our thoughts on the highlighted topics regarding the wake-up signal design and procedure and protocol changes, specifically the:

- Wake Up signal design considerations

- L1 Procedures for low power WUS

- UE Measurements in idle/inactive mode (with and without mobility)

- UE Measurements in connected mode

- Other Procedures for low power WUS

# Discussion

## Wake Up Signal Design Considerations

The wake-up signal design should consider different aspects to ensure the feasibility of LP-WUS use. The primary driver of LP-WUS design is to enable use of power efficient wake-up receiver (LR) architectures. The overall power consumption/saving is affected by multiple factors (such as mobility as discussed in Section 2.2), but the WUR contribution is an important factor to consider. Also, to ensure feasible deployment of the wake-up signal, the design should be such that it can be easily introduced in the existing NR deployments, e.g. without any hardware changes in network deployments. In following sub-sections, we consider different aspects related to the wake-up signal design.

To achieve low power consumption of the WUR, different modulation schemes with low spectrum efficiency may be considered for the WUS signal, e.g. OOK (or MC-OOK). The WUS overhead in terms of radio resources shall be investigated.

### The Signal Bandwidth

One elementary parameter for the LP-WUS design is the bandwidth of the signal as it will affect many aspects of the LP-WUS operation and LR design. Assuming higher bandwidth would allow higher signal power, but also result more noise power, so the receiver SNR may remain unchanged, thus the typically considered receivers may not benefit from this in terms of performance. However increased bandwidth will increase robustness against narrowband interference and fading. Also higher bandwidth per (e.g. OOK) symbol may result in lower spectrum efficiency, unless we use DFT-s-OFDM, and also offer fewer multiplexing opportunities and make coexistence with pre-existing signals more problematic.

Observation 1:  Wider LP-WUS BW can offer robustness against fading, but impacts the spectrum efficiency and feasibility of multiplexing with other signals

On other hand Rel-17 RedCap devices are restricted to a RF/BB bandwidth of 20MHz and introduction of RedCap devices with further restricted BW, down to 5MHz, is being specified for Rel-18, thus limited LP-WUS BW could also enable wider adoption. Limiting the BW of LP-WUS transmission could enable wider use of LP-WUS in different type of deployments, with different transmission bandwidths, down to 5MHz and even below, which are supported in NR. Therefore, the trade-off between LP-WUS BW and e.g. data rate should be also considered.

Observation 2:  Having limited LP-WUS BW could allow adoption in different device types and enable use in different type of deployments.

Higher bandwidth (more sub-carriers) could give more flexibility for the gNB when creating the WUS signal, which may e.g. allow to reduce PAPR (averaging the signal power across the symbols, rather than having steep peaks) and/or allow using a frequency domain sequence with good cross correlation properties that could be benefitted by more complex LR to e.g. increase sensitivity. Increased bandwidth also increases the tolerance to frequency offset in the LR. This can help simplify the design of the WUR (e.g. simpler/no LO) and thereby also reduce power consumption.

Observation 3:  Wider LP-WUS BW could enable support of more flexibility in signal generation and different signal characteristics and improve the tolerance to implementation impairments.

It is also expected that the bandwidth of LP-WUS would scale with the SCS, i.e. a fixed number of subcarriers are used independent of the SCS. At least to facilitate the LP-WUS generation, it would seem preferable to assume same SCS in LP-WUS generation as used in the carrier for other NR signals, and scale the LP-WUS symbol duration based on the applied NR SCS on the carrier.

Observation 4:  The effects of assumed SCS on the LP-WUS symbol duration should be studied.

Proposal 1: 	The SI evaluates the LP-WUS BW accounting performance/robustness and applicability to different device types and deployments.

### The structure of WUS signal.

When considering the LP-WUS structure, different elements can be seen beneficial depending on the purpose and targeted procedure/behaviour. Following the example of 802.11ba, the LP-WUS could be envisioned to incorporate different fields/elements to support different purposes. In Figure 1, we illustrate one possible generic structure of LP-WUS.

Figure 1: LP-WUS message with preamble, sync, payload and FCS/CRC fields.

The preamble and sync field may help to improve both detectability and reliability of WUS signal irrespective of whether the LR is continuously or periodically listesting for an on-demand wakeup. The benefit of having each field in the LP-WUS signal shown in Figure 1 are as follows.

- Settling of comparator threshold voltage: In case of OOK and comparator-based receivers, the initial part of the preamble may be used for settling of comparator threshold voltage. If the preamble is a toggling sequence of 0’s and 1’s or if the preamble is a Manchester encoded sync word, the threshold voltage may be found by LP filtering the received preamble signal. In this case, the length of the preamble should be long enough to ensure reliable settling of the LP filter at the LR. On the other hand, the LP filter settling time shall be much longer than the ON/OFF duration to ensure that the threshold voltage is maintained across the ON and OFF symbols.

- Detecting the alignment of the WUS message: To find the start of wake-up message payload, the preamble may be followed by a sync pattern, which could assist devices to establish timing alignment with the gNB. Furthermore, different sync patterns could be used to differentiate LP-WUS messages e.g. in terms of content or intended group etc.

The payload field may contain different type of (encoded) information:

- Purpose/type of wakeup signal, to enable multi-purpose design, with varying payload size e.g. a wakeup signal could be configured as a 802.11ba style cell beacon

- Indentifiers such as cell ID or UE/group ID

- Other control information such as access type or short message etc. The exact content would be need to reflect the intended procedure and use case.

Finally the check sum field at end can be added to minimize the false alarm rate (FAR) in the LR, since in each wake-up cycle of eMBB, whole system bring-up from ultra-deep sleep state together with cell re-sync is performed before monitoring the PO. Unlike waking from deep sleep, the wake-up from ultra-deep sleep state consumes 100x more power than the former, which undermines the benefit of LP-WUS framework.

Observation 5:  It may be beneficial for some HW implementations to have a preamble in order for comparator threshold settling.

Observation 6:  A sync field may be added to help the LR to find the beginning of the WUS payload. The sync field pattern can be also used to distinguish content and target group.

Observation 7:  Payload field can be used to carry relevant information and have different content and/or sizes based on the use case/procedure.

Observation 8:  A FCS/CRC check field calculated on the wake-up message payload will lower the FAR and ensure the integrity of the payload.

Evaluations are recommended to determine the need and the benefits of different fields for LP-WUS structure illustrated in Figure 1. These could be used to determine whether benefits in terms of LP-WUS detection performance, the feasibility to maintain synchornization to the gNB, coverage level estimation/identification, outweigh the disadvantages of added inter-cell interference and RE overhead and LR power consumption. It is worth noting that the periodic transmission of LP-WUS SYNC signal, could also help to relax how often the MR needs to waken up e.g. to make cell quality measurements, thus reducing the power consumption of the devices.

Proposal 2: 	The SI evaluates the benefits and the cost of supporting different LP-WUS fields.

State of the art wakeup receivers tend to trade sensitivity for lower power consumption, e.g. by avoiding a power consuming LNA. To achieve a wide cell coverage of the WUS signal, we believe that some of these components may still be required, and therefore alternative means may be needed to achieve the low power consumption. Supporting DRX for the wakeup receiver should lower the overall average power consumption, but the amount of power saving will be highly dependent on the clock accuracy/drift.

Observation 9: 	To achieve a good trade-off between sensitivity and power consumption, it may be necessary to introduce DRX on the wakeup receiver (as opposed to an always on receiver).

Observation 10: 	The LP-WUS signal reception can suffer from intercell interference and other serving cell signals, which may degrade the false detection performance thereby reducing the power consumption savings from the introduction of the LP-WUS.

Since, the primary focus on the design requirements of LR is the power consumption, therefore, much of the receiver components used in the LR may not be in par with the MR components. This may increase the probability of false detections, which will eventually wakes up the MR to perform paging reception at the respective PO. Since every wake-up of MR includes complete or partial boot-up of the hardware sub systems followed by cell resynchronization and PO monitoring, the overall power consumption is much higher than the traditional DRX based wake-up of MR. This is mainly due to the reason that in DRX based wake-up of MR without the presence of LR, MR is in deep sleep state, and therefore it requires to perform only timing and frequency corrections followed by SSB reception to measure the channel quality. Therefore, always ON monitoring for LR increases the probability of false detection, thereby undermining the original purpose of this SI.

Proposal 3:	The SI evaluates the potential benefits and drawbacks of supporting fixed wakeup occasions allowing the wakeup receiver to enter a lower power DRX mode in between such occasions.

Proposal 4:	The SI considers techniques to improve the robustness of the LP-WUS to inter-cell and intra-cell interference.

### The Modulation and Coding Schemes.

Modulation plays a vital role in the overall behavior of LP-WUS framework. It has to be robust against inter-symbol, inter-carrier, and inter-cell interference by providing better coverage for users experiencing different signal conditions. Since the main objective is to design a LP-WUS waveform such that a low-power receiver can decode and interpret the message transmitted by gNB. Depending on the type of modulation scheme used, it can be categorized as follows.

#### OOK based transmission

One of the candidate waveform for LP-WUS is the OOK modulation scheme, which is also used in 802.11ba systems. The main benefit is that it does not require power hungry ADC at the receiver. It can be detected using simple comparator to few-bit ADCs with non-coherent detection. Furthermore, the multi-carrier OOK (MC-OOK) modulation can be easily created to co-exist with the legacy OFDM modulation, thereby allowing reuse of existing gNB hardware.

OOK with Manchester encoding is often used in legacy systems. The advantages of this type of signal include:

- The signal incorporates a self-clocking characteristic

- After the envelope detector, the Manchester encoded signal has a fixed DC offset when averaged. This can be used to derive the comparator threshold e.g. by LP filtering the signal.

- A relatively simple and low power comparator based LR architecture can be used for decoding.

- A disadvantage of the Manchester encoding is that it will increase the information symbol duration and thereby reduce the throughput/channel capacity.

- To increase coverage, the LP-WUS signal may also have deterministic sequence applied in the ON duration, which receiver could benefit. LP-WUS receiver could support an ADC with dynamic sampling rate enabling it to sample the received signal with higher rate during poor signal conditions. This will allow the receiver to switch between two modes, i.e. the most power optimum mode where the signal is received as an OOK signal (ADC switched OFF/operated with low rate) and a mode with better sensitivity where the WUS signal is sampled (ADC switched ON/to higher rate) and digital processed by e.g. a correlator.

##### OFDM symbol OOK

- The ON/OFF is at CP-OFDM symbol, i.e., using entire BW to transmit a portion of the Manchester encoded bit.

- Main disadvantage is the under utilization of the useful spectrum as it transmits one bit per CP-OFDM symbol

- On the contrary, it is robust against frequency selective fading

##### DFT-s-OFDM based OOK

- It involves DFT spreading using M point DFT before the final M≪N point IFFT

- Since multiple OOK signal can be transmitted in an OFDM symbol depending on the LP-WUS BW, it utilizes the available spectrum more efficiently, i.e., transmitting more bits in an OFDM symbol

- On the contrary, it suffers from increased complexity and the BW determines the smearing of ON/OFF signaling, and performance suffers from ISI in a rich scattering environment

- Some of the above mentioned issues can be solved by using sufficiently long ON/OFF durations i.e. limiting the number of bits per OFDM symbol

- Since the OOK signal lies within OFDM symbol, CP can alter the synchronization across OFDM symbol.

Before proceeding with the performance of DFT-s-OFDM, we will briefly touch upon the issue with the generation of multi-bit OOK signal within CP-OFDM framework.

###### Effect of CP on the transmitted DFT-s-OFDM signal in time domain

Figure 2 Manchester encoded DFT-s-OFDM over 2 symbols 10bits/symbol @15KHz SCS in 10MHz BW

As discussed in [2], the effect of appending CP to the DFT-s-OFDM signal causes unwanted glitch in the time domain waveform as highlighted in Figure 2. It depends on the samples present in the tail of the respective CP-OFDM symbol occupying the LP-WUS BW. Even though, the size of the CP glitch varies due to two different CP lengths, the effect is not noticeable if the receiver operates at the LP-WUS BW.

Figure 3 Adapting OOK pulse width to accomodate for CP inclusion

Thus, to overcome the effect of CP addition on the MC-OOK transmission, following approaches can be considered.

- Insert padding symbol at the end of the OFDM symbol to ensure correct CP behaviour. The receiver will have to discard the padding symbol upon reception in Figure 3

- Zeroing out tail samples of each MC-OOK symbols before DFT spreading

Observation 11:     If DFT-s-OFDM type of modulation is considered, effect of CP in the transmission should be studied, if LP-WUS spans multiple OFDMA symbols.

###### A Study on the sequence usage and the preamble structure

Before proceeding further, we will briefly discuss the structure of LP-WUS transmission used in the evaluation. The LP-WUS transmission is assumed to span 5MHz BW and 14 symbols or 1 slot in time duration. The first two symbols are assumed to be preamble symbols, carrying “1” and “0” in symbol 0 and symbol 1 respectively. The width of the pulse used for preamble is same as the data bit width. The samples are generated as follows.

xn=bk⨂WM

where WM is a window sequence of length M, i.e., if rectangular window is used, then WM=[1,1,…, 1M]. Finally, bk is the bit sequence transmitted in a CP-OFDM symbol and xn is the equivalent time domain OOK sequence. Figure 4 illustrates the behavior of time domain signal before and after low-pass filtering along with the transmitted spectrum of gNB, while using all-ones and ZC sequence in the ON duration of each bit in the symbol. It can be seen that by using ZC sequence in ON duration ensures constant spectrum in Figure 4b as compared to all-ones in Figure 4a.

| (a) All-Ones used in ON duration | (b) ZC sequence in ON duration |
| --- | --- |

Figure 4 Effect of sequence used in ON duration

Similar to PAPR or cubic metric, which are measured on the time domain signal, variations on the samples inserted on the CP-OFDM tones impact the fidelity of the IFFT process performed at the gNB for transmission. If all-ones are used in the ON duration of OOK signal prior to DFT spreading, the resulting samples populated on the tones of CP-OFDM symbol has a sinc waveform pattern. This increases the dynamic range of the samples populated in the CP-OFDM symbol prior to IFFT process. Since the IFFT/FFT process performed at the gNB has a finite resolution, the fidelity of reproducing the time domain signal may show the sign of quantization effects.

The preamble always carries “1” and “0” in the first and second symbol respectively, using the same pulse shape as that of data bits transmitted in symbols 2-13. Therefore, preamble bits are used to determine the threshold for each LP-WUS transmission slot. Thus the threshold, which is obtained from the preamble is used to decode “1”s and “0”s transmitted in the symbols 2-13. The threshold detection is simply calculated using either “1” and “0” region of preamble and by using the following formula κ=E0+0.5×E1-E0, where Eb denotes the energy of either “0” bit or “1” bit from preamble. Thus if the data bit energy satisfies κ≥Eb, then the bit b is declared as “0” otherwise as “1”. The three types of detectors used in our evaluation are listed in Table 1.

| Coherent detector | Non-coherent detector | Max detector or one-shot estimate |
| --- | --- | --- |
| Eb=1Mn=0Mxn2 | Eb=1Mn=0Mxn2 | Eb=max∀n∈{0,…, M-1}¦xn​2 |

Table 1 Detector Types

Finally, to avoid the impact of CP in the LR detection, the relevant samples corresponding to the CP duration, i.e., the tail samples, are excluded from the mapping of OOK bits prior to DFT spreading. Since the samples are zeroed out in the time domain LP-WUS waveform before DFT spreading, after IFFT operation of NR transmission BW, the CP inclusion will add zeros in the prefix. Therefore, the effect of CP glitch is avoided, however, it reduces the system resource utilization. Alternatively, we can choose the tail bits carefully in such a way to utilize the circular property of CP inclusion between tail and head samples as discussed in 2.1.3.1.2.1 without compromising on the system resources.

###### Performance of Multi-Bit DFT-s-OFDM Transmission

In the following section, we will study the performance of multi-bit MC-OOK transmission, wherein we begin by comparing various detectors under AWGN channel using all-ones as the transmitted sequence in the ON duration. The benefit of having all-ones as the transmitted sequence enables us to use coherent detection at the receiver, which can be used as an upper bound on the LR performance. In order to avoid the Gibbs phenomenon, i.e., oscillations around the transmitted bits, pulse shaping is performed to facilitate smooth transitions around the OOK symbol edges when the signal is bandlimited by LP-WUS BW. The simulation parameters used for evaluation are listed in Table 2.

| Simulation Parameter | Design Value |
| --- | --- |
| LP-WUS BW | 4.68MHz (26 PRB in 15KHz or 13 PRB in 30KHz) |
| CP-OFDM BW | 10MHz (1024 tones, 52 PRBs) |
| LP-WUS Guard BW | 180KHz on either side (1 PRB in 15KHz and ½ PRB in 30KHz) |
| SCS | 15KHz, 30KHz |
| Sequence used in ON duration | All-ones, ZC seq (with tapered cosine pulse shape with 0.5 roll-off) |
| Number of TX and RX antennas | 1x1 |
| Detectors used for all-ones as sequence | Coherent, non-coherent sampled at 5MHz, max dector sampled at data rate. |
| Detectors used for ZC sequence | Sequence detector sampled at 5MHz, non-coherent and data rate max detection |
| NR transmission in the adjacent carriers | Active with QAM256 modulation. |
| LPF filter used | 4th order Butterworth filter with downsampling by a factor of 3@15.36MHz |
| LP-WUS duration (in terms of CP-OFDM symbols) | 14 symbols or 1 slot is used for transmission |
| Time synchronization | Assumed perfect synchronization in time |
| Frequency offset and number of ADC bits and impairments | ±200 ppm, no-ADC bit width or impairments are considered. |
| Preamble symbols | Symbol 0 (of slot) is used for “ONE” pulse and symbol 1 for noise, i.e., “ZERO” |
| Data symbols | Symbols 2-13 of slot are used for data transmission |
| CP handling | In order to avoid interference to adjacent NR transmissions, the samples corresponding to CP duration are zeroed out to avoid CP discontinuities |

Table 2 Simulation Parameter Assumptions

Figure 5 compares the performance of coherent, non-coherent, and max estimate, all sampled at 5MHz LP-WUS BW with two SCS values, namely, 15KHz and 30KHz. Since all-ones sequence are used, coherent detection is possible with simple matched filter, i.e., LPF with 5MHz BW. The comparison is made by varying the number of bits transmitted within a CP-OFDM symbol, which takes the values as L={1,2,4,6,8,10}. As the number of bits per CP-OFDM symbol increases, the number of oversampling gain per OOK bit reduces for a fixed sampling rate, as shown in Figure 5. Furthermore, the coherent detection provides ≈1.5dB compared to that of non-coherent detection, which is in line with the non-coherent gain of ∝√M compared to coherent gain ∝M. Finally, the one-shot detection using the maximum energy over the samples corresponding to the OOK bit is way too inferior with respect to the other detectors. On the otherhand, the false alarm probability is higher for the detection using single sample, which is sampled at data rate. Thus, having the oversampled version is always beneficial to improve the receive SNR in order to ensure efficient detection performance.

| (a) SCS 15KHz | (b) SCS 30KHz |
| --- | --- |

Figure 5 Performance all-ones DFT-s-OFDM detectors in AWGN

Figure 5 compares various detectors using two SCS values, namely, 15 and 30KHz. Doubling the SCS by two reduces the number of samples available per symbol for a fixed BW of operation. Thus, we can see the degradation in SCS 30KHz compared to that of SCS 15. However, for a fixed BW of operation, the overall data rate achieved by two SCS values will be the same, the number of CP-OFDM symbols will be doubled for SCS 30KHz compared to SCS 15KHz for a same duration. As shown in b, the performance of 10 bits per CP-OFDM symbol flattens out as compared to Figure 5a using SCS 15KHz due to the ISI experienced by the transmitted bits in shorter duration of SCS 30KHz.

| (a) SCS 15KHz | (b) SCS 30KHz |
| --- | --- |

Figure 6 Performance all-ones DFT-s-OFDM detectors under fading channel TDL-C 30ns

Figure 5 compares the performance of different detectors under TDL-C τmax≈30ns fading channel, which corresponds to non line-of-sight (NLOS) chanel model. In order to bring out the performance difference, AWGN behavior is also provided for reference in Figure 5. It can be seen that the fading channel eventually degrades the performance of multi-bit OOK signaling due to the presence of multi-path, which causes symbol smearing, thereby introducing severe ISI degradation. Due to the presence of multi-path effect, coherent detection is not used for detection. However, as the delay spread increases with more number of clusters and multi-paths, such as CDL-B τmax≈300ns, the performance can degrade significantly as compared to that of Figure 6.

Observation 12:     The performance of multi-bit OOK degrades significantly in comparison with that of AWGN channel due to multi-path effect.

Figure 7 Performance of ZC based OOK transmission in AWGN

Before proceeding with the performance of OOK detectors under LR impairments, let us discuss the performance of Zadoff-Chu (ZC) sequence on the transmitted waveform. In the ON duration of OOK sequence, if a ZC sequence is used, what benefit it provides at the LR is studied and presented in . It can be seen that as long as the number of bits per CP-OFDM is capable of providing enough length for ZC encoding, the performance of having sequence detection is beneficial as it can provide improved coverage. However, on the downside, the performance degrades significantly when the number of bits increases due to diminishing ZC sequence length.

One benefit of having ZC sequence in the ON duration is that it can handle the inter-cell interference if the sequences are carefully chosen across the cells. It aids the LR detection by decorrelating the interference while correlating well with the desired serving cell sequence. The draw back of having sequence detection is the increase in the LR power consumption. Thus, there is a trade-off between the power consumption and the performance.

Observation 13:     Oversampling at the LR is essential to improve the performance of OOK reception and to reduce the probability of false alarm.

Observation 14:     By using properly designed sequence in the ON duration of LP-WUS transmission, LRs can improve the coverage and can also overcome the inter-cell interference

The effect of system impairments on the LR detection performance is presented in Figure 8 and Figure 9. In Figure 8, we demonstrate the effect of varying frequency offset experienced by the LR upon reception. It is worth noting that the presence of adjacent NR transmission does not affect the OOK detection performance until the frequency offset is ≈100KHz≤180KHz, i.e., the guard band added around the LP-WUS transmission. However, increasing the frequency offset beyond the guard band, i.e., ≈240MHz≫1PRB BW, the leakage from the NR transmission degrades the receiver performance as can be seen in Figure 8. This effect is seen only when the bits/symbol is higher, i.e., 8 and 10 bits, since the number of samples per bit reduces for a fixed sampling rate.

Figure 8 Effect of varying frequency offset on LR performance

Similarly, the effect of varying the number of filter taps used at the LR for LPF is studied in Figure 8. The number of filter taps used at the LR is varied from [1-4], with “1” exhibiting poor roll-off characteristics and “4” produces sharper roll-off, i.e., having better adjacent carrier rejection performance. As expected, using steeper roll-off with “4” taps provides better performance compared to that of the single tap LPF filter.

| (a) Using SCS 15KHz | (b) Using SCS 30KHz |
| --- | --- |

Figure 9 Effect of varying LPF roll-off on the LR performance with Ntaps={1,2,3,4}

Using lower number of filter taps for LPF degrades the performance of OOK transmission when the number of bits packed within a CP-OFDM symbol is more, i.e., when L≥4, as can be seen in Figure 9a and b. This is mainly attributed due the presence of fewer samples per bit as the number of bits per CP-OFDM symbol increases. Secondly, it is also due to the spread of energy over entire LP-WUS BW, thus the roll-off degrades the high frequency transitions, which is more likely with more bits. In Figure 9b, which uses SCS 30KHz, the performance of degrades rapidly with increasing number of bits per CP-OFDM symbol. This is due to the reason that for the same sampling rate, SCS 15KHz will have double the number of samples compared to that of SCS 30KHz, thereby, performing worse than the SCS 15KHz scenario.

Observation 15:     The LP-WUS modulation should be resource efficient accounting need for possible guard bands, device BW restrictions and efficient multiplexing with other LP-WUS and other legacy signals.

Observation 16:     The LP-WUS modulation scheme selected should provide robust performance (sensitivity and selectivity) using low power receiver architectures.

Observation 17:     A LP-WUS designed to support a flexibly sized payload would allow the LP-WUS to be easily adopted for different use cases.

#### FSK based transmission

FSK modulation is another waveform proposed for LP-WUS in RAN1. This has been studied extensively in the literature and also used prevalently in many earlier technologies. The generation of FSK modulation follows the same approach as OOK transmission, while using twice the BW as compared to the OOK transmission. Unlike the traditional FSK modulation wherein two discrete tones are used for communication, LP-WUS waveform based on FSK must be designed to have a set of tones. This is to avoid the effect of frequency selective fading and to ensure robust detection at the receiver with huge PPM offset.

It is worth noting that the Manchester encoding can be considered to be inherently present in the waveform modelling in frequency domain. Thus, if we compare the Manchester encoded OOK signaling with that of FSK transmission, the resource utilization is the same, since only half the duration is used for Manchester encoded OOK. The benefit of having Manchester encoding can be seen at the receiver as “1” and “0” are present in each transmitted bits, which aids in detection performance.

On the contrary, if it is coupled with the DFT-s-OFDM, then the FSK scheme is comparable to that of OOK signaling discussed in the previous section. Few advantages of FSK within DFT-s-OFDM are as follows.

- Due to constant envelope, FSK reception is more noise tolerant, thereby, reducing the cost of LR components

- The presence of two discrete frequencies, which toggles at the DFT-s-OFDM symbol level, can be used for synchronizing the local oscillator of LR

- It has similar overhead posed by the OOK signaling employed with DFT-s-OFDM

- Detection is robust against the Doppler shift caused by the mobility, as the tones are affected similarly

Having discussed the above mentioned advantages, it has few shortcommings as well, which are listed below.

- Effect of cyclic prefix in the OFDM symbol may interfere with the detection performance of the receiver

- Adjacent carrier interference and inter-cell interference affect the detection performance

- To avoid spectral leakage, tones used for FSK must have integer cycles within DFT-s-OFDM symbol

Figure 10 DFT-s-OFDM for FSK modulated transmission @10bits/OFDM symbol, 2 OFDM symbols are used

In the following discussion, an example of generation and the detection of DFT-s-OFDM using FSK transmission is briefly discussed in Figure 10. outlines the behavior of the signal in time domain with two discrete tones f0 and f1, which are separated by an integer factor in order to ensure orthogonality. A stream of 10 bits, wherein each bit is represented by a complex frequency of length 24, is provided to M point DFT. Then the DFT spreaded waveform is placed within OFDM carriers contiguously so as to ensure single carrier properties. Once the LP-WUS waveform is multiplexed within an OFDM symbol, the final IFFT operation of size N≫M is performed to obtain the time domain waveform. This process is shown in Figure 10a and Figure 10b.

Figure 10c, subplot shows the received waveform after translated and down sampled to allow only 4MHz BW. The received signal can be decoded in the time domain and the effect of cyclic prefix (CP) is visible at the symbol boundaries, though not pronounced much. This shows only the concept of generating and decoding of FSK signal under DFT-s-OFDM framework. The effect of adjacent NR signal multiplexing and the inter-cell interference should be studied to ensure the reliability of the candidate LP-WUS waveform. Finally, in Figure 10d, the received spectrum after frequency translation, i.e., prior to LPF, is presented, where we can see the signal is contained within 4MHz transmission BW.

Observation 18:     The LP-WUS modulation scheme selected should be easy to generate using the existing gNB architecture.

Observation 19:     Possibility of multiplexing a spreading sequence in the ON duration of DFT-s-OFDM should be studied to enhance coverage and to improve detection performance.

#### CP-OFDM based transmission

The third waveform under consideration is CP-OFDM scheme. This is prevalently used waveforms in the existing standards and has proven advantages over other waveforms on the improved throughput, coverage, mobility and spectral utililzation. The main advantages of having CP-OFDM waveform are listed below

- Ability to reuse the existing SSB signals for synchronization and measurements

- Generation of CP-OFDM is simpler as it is aligned with the NR signal generation

- Sequence base WUS is used in LTE.

- It is robust against both fading and inter-cell interference scenarios

- It can handle higher data rate for a given LP-WUS BW compared to that of OOK and FSK schemes

- Guard band requirements can be relaxed, since LP-WUS CP-OFDM signal is orthogonal to NR transmission

Having said the above benefits of using CP-OFDM based LP-WUS, it has few shortcomings as well as listed below

- The cost of LR is higher, since the minimum requirements on the receiver components are higher, i.e., CP-OFDM receivers require higher clock stability and expensive hardwares to perform baseband processing, such as, FFTs and correlations

- Due to the usage of complex baseband for processing, both in-phase and quadrature components are to be extracted at the LR, thereby increasing the cost significantly.

- The power consumption of such receiver is significantly higher compared to that of simpler receivers used for OOK and FSK.

If the LR is designed to operate in always ON mode, the CP-OFDM may not be a meaningful candidate waveform for LP-WUS. However, if the LRs are assumed to operate in a duty cycled manner, i.e., aligning the LR monitoring instants with the LP-WUS transmission occasions, then the power consumed by the LR designed for CP-OFDM based LP-WUS reception is comparable to that of other modulation schemes.

Observation 20:     LP-WUS using sequence based CP-OFDM must be studied in this SI, since it has been used in previous generation cellular standard

## Design Metrics (KPIs)

In order to identify a suitable candidate modulation scheme for LP-WUS, there is no one single criterion or question to be addressed. As such, the considered waveforms for LP-WUS, i.e., OOK, FSK, and CP-OFDM has their own respective merits and shortfalls. In this section, we will make an attempt to list out the possible metrics used to evaluate the candidate waveform and the optimal waveform for the chosen metric.

### Power Consumption at LR

The primary topic of LP-WUS study item is concerned with the idle mode power consumption of the UE. It is well known that the CP-OFDM receivers are quite resource hungry and consumes more power. This is the main reason behind the inclusion of addition LP-WUS waveforms like OOK and FSK schemes. If the choice is based on the power consumption metric, then the wiser choice will be OOK/FSK modulation schemes. However, it is evident from our discussions in Section 2.1.3 that the performance of such schemes are limited by the propagation channels and the available BW. Furthermore the spectral resources are severely under utilized.

The choice of OOK based wake-up signal used in Wi-fi 80.11ad is wiser as it primarily targets the indoor scenarios wherein the multi-path effects are not of bigger problems. On the contrary, the cellular models are wide area network, which has no shortage of multi-paths and inter-cell interference. Thus the quality of reception with OOK based LP-WUS waveforms need to be studied for these scenarios.

### Data rate

Since LP-WUS does not envisage higher data rate like eMBB devices, the choice of modulation scheme based on the data rate is not meaningful. However, if the underlying assumption for LP-WUS is to wake-up a single UE or a smaller subset of UEs to monitor the paging occasion, then the LP-WUS must aim for few hundreds of bits per second. In such cases, the OOK/FSK based schemes must aim at pushing multiple bits in each CP-OFDM symbol for a given BW. If the bits per CP-OFDM symbol increases, then the OOK symbols are squeezed within a single CP-OFDM symbol, thereby causing ISI.

However, if we increase the BW of LP-WUS transmission to accommodate more number of bits per CP-OFDM symbols, the the performance degrades in the presence of multi-path fading conditions, wherein the LR might require need an equalizer to mitigate the multi-path effect. This is the very same problem addressed by the CP-OFDM framework. Thus, the number of bits per CP-OFDM symbol for OOK based transmission is upper bounded by the length of CP, which ensures ISI free transmission in the presence of multi-path. Thus the choice of a modulation scheme is not possible as it depends on the target data rate requirement.

### Coverage and Mobility

The unanimous choice of a modulation scheme, i.e., within OOK, FSK, and CP-OFDM, that provides better coverage and increased robustness against mobility will be CP-OFDM. It is well known that CP-OFDM can improve the coverage by either with repetition, power boosting. Even though other modulation schemes can do the same to improve the coverage but the gains may not be significant as compared to that of CP-OFDM scheme.

Mobility handling with CP-OFDM is simple due to the presence of periodic SSB transmissions, which also uses CP-OFDM scheme. It can be used by LR to measure the reception quality and can be used to synchronize their internal clocks for proper time and frequency synchronization. On the contrary, if OOK or FSK based schemes are used, then a periodic synchronization signal is required to ensure the detection capability of LRs as there is no periodic transmissions based on OOK to be used for synchronization. This will reduce the spectral utilization from the gNB perspective.

### Spectrum and Resource utilization

The choice will be simpler, i.e., CP-OFDM. This is due to the following reasons.

- Ability to reuse the existing SSBs for synchronization and for measurements to determine the signal quality

- Since the subcarriers of CP-OFDM are orthogonal, the size of LP-WUS guard band can be relaxed unlike the OOK and FSK based schemes

- Since the CP-OFDM is designed to handle ISI with a simple FFT based receiver, the spectral utilization is higher as it can push more bits for a given duration.

Observation 21:     The modulation scheme used for LP-WUS must be chosen to reduce both power and latency of majority of UEs in the cell rather targeting a subset of UEs whose channel conditions are good.

## Procedures for low power WUS

In this section, we consider different procedures related to the LP-WUS operation. In section 2.2.1 and 2.2.2 we look at existing the mobility related procedures and the measurements required to be performed. For cellular network operation maintaining a good mobility performance is a key element to ensure high quality connectivity and thereby good user experience. The measurements and related procedures assume that the UE is able to read the system information in the case of reselection, to perform measurements such as RSRP (SSB/CSI-RS), and provide CSI reports. In addition, we discuss different options for paging procedure with LP-WUS in Section 2.2.3.

### UE Measurements in idle/inactive mode

In IDLE and Inactive mode, after PLMN search and successful registration to network, the UE continues to carry out measurements of identified/detected cells and continues searches to detect new candidate cells to ensure that the UE is camped on the best ranked cell and highest priority cell, as described in TS38.304. In RAN1#111 it was agreed to further consider the impact of mobility measurements:

Agreement For a UE support LP-WUR in IDLE/INACTIVE mode, - Study how to reduce UE power consumption due to existing RRM measurement requirements at least for mobility support, - study feasibility of RRM measurements performed by LP-WUR, at least for serving/camping cell, based on signals detected by LP-WUR - FFS: measurement metric - FFS: whether and how to identify cell/ tracking area - FFS: need for neighbouring cells - FFS: need for relaxation of existing RRM measurement requirements (for UE)

In the companion document [3], we evaluated the impact of the MR based mobility measurements to the power saving benefits of LP-WUS. We compared the attainable power saving gain (i) when no MR based mobility measurements are done, (ii) full and reduced mobility measurements based on MR, i.e., fewer measurements within PTW, measurements every 2nd eDRX cycle). From those evaluations, it could be observed that relaxation of existing requirements could be considered to reduce the power saving loss due to MR based mobility measurements over the LR-only operation.

In [2] we discussed the frequency of the mobility measurements to be carried out during IDLE and INACTIVE mode. Evidently, the frequency of mobility measurements affects the length of the evaluation period in order to have sufficient number of samples (for accuracy and other averaging), which results in the delay to observe the changes in strongest cell in order to trigger related mobility procedures. Thus increased delay can lead to reduced service quality e.g. due to paging misses. Hence, the serving cell evaluations form the basis of IDLE/Inactive state mobility measurement, is imperative to ensure that UE is constantly in the coverage area of a suitable cell. For the case when eDRX is not configured, the measurement activity due to the serving cell evaluation is likely to limit the attainable gains from LP-WUS, unless relaxed operation is considered for the serving cell evaluation in normal DRX. Thus, when LP-WUS procedures are considered for IDLE/Inactive state operation, care should be taken that UE can be maintained in normal service. If the measurement activity (of e.g. serving cell) is to be based on assumption that eDRX is configured, it would need to be understood whether the use cases are restricted to semi-static devices or whether they can tolerate long interruptions/latency in service. If the devices are assumed to move more freely, the delayed serving cell evaluations could result in prolonged interruption of service due to the delay in re-selections. Thus, when LP-WUS procedures are considered for IDLE/Inactive state operation, care should be taken that the assumptions related to mobility are aligned with other assumptions, such as service latency.

Observation 22: 			If eDRX based evaluation rate is assumed, the mobility of the devices should be assumed to be restricted to semi-stationary to avoid service interruptions due to delayed mobility evaluations. Alternatively, if higher mobility (than semi-stationary) is assumed, the service should be able to tolerate longer latency.

Proposal 5: 	Clarify the mobility assumption for the purpose of LP-WUS evaluations and design and ensure that assumptions are aligned with other assumptions.

Another aspect that affects the mobility performance is the accuracy of the measurements. This is related to the afore noted delay also, as the amount of available physical layer samples is affected by the measurement evaluation period. The RAN4 specification TS38.133 determines the measurement accuracy requirements e.g. for SSB-RSRP in CONNECTED mode. For most cases, IDLE/INACTIVE mode requirements are determined indirectly based on the IDLE/Inactive mode re-selection requirements. As an example, if it is determined that UE has to identify the neigbouring cell that has met the re-selection criteria if the cell is 3dB better ranked. As noted above accuracy can to a certain extent improved with increased number of samples, but it results latency, which may degrade mobility performance in changing conditions (e.g. due to UE movement).

Thus LP-WUS can be considered to be used for mobility measurents and related evaluations. It is then necessary to consider the attainable accuracy and latency (evaluation period) from the LP-WUS design. This relates to the aspects like availability and frequency/periodicity of LP-WUS (e.g. beacon) and design of the signal to enable good metrics. Optimizing both from mobility measurement perspective, may results increased overhead.

Observation 23: 			If LP-WUS is considered for mobility measurements, the LP-WUS design would need to account the requirement to enable good accuracy within reasonable evaluation period. This may imply increased overhead.

It is also good to note that the accuracy for absolute measurements is not solely determined by the baseband processing, but also on the RX chain imperfections. The accuracy of the gain level of RX chain and possible AGC steps has a direct impact on to the absolute accuracy. Other aspects like ADC and frequency error can also affect the attainable accuracy of measureements, and thereby the feasibility of using LP-WUS/WUR for mobility measurements. Also, the baseline receiver architecture will affect the feasibility of the metrics that can be considered, and thereby the feasibility of the mobility measurements.

Observation 24: 			RX chain imperfections of LP-WUR, as well the baseline reference architecture affects the feasibility of LP-WUS based mobility measurements.

As can be seen from the afore discussion, serving cell evaluations form the basis of IDLE/Inactive state mobility measurement activity to ensure that UE is constantly in the coverage area of a suitable cell. Carrying out afore described RRM measurements require enabling MR e.g. every eDRX cycle. Due to the (power) cost of activating the MR, it could be considered whether the LP-WUS design could be used at least partially alleviate the need for MR based RRM measurements or whether measurements could be further relaxed for eDRX, similarly as was done for DRX in Rel-16 based on the UE mobility assumptions. Evidently, the mobility performance should be maintained in an acceptable level

Proposal 6: 	Evaluate the possible alternatives and feasibility to reduce the need of MR based RRM measurements with limited mobility performance impact.

Investigation into other LR and LP-WUS performance measurements/alarms is also recommended to assist with:

- Load balancing of wake-up signal by optimizing the paging group sizes per LP-WUS ocassion

- Ability to adapt the transmission ocassion of LP-WUS transmission so as to minimize the inter-cell interference so as to ensure good quality reception at LR

- In order to improve the coverage, LP-WUS resources must be adapted

- WUS signal resource configuration optimisation to improve the coverage via increasing the resources

Proposal 7: 	Evaluate new LR/LP-WUS performance measurements/alarms to assist the network to optimise the LP-WUS configuration.

### UE Measurements in connected mode

In Connected mode, the UE typically carries out measurements for different purposes; mobility measurements, monitoring and measurements for beam management (e.g. BFD and link recovery), radio link monitoring and CSI measurements (e.g. L1-RSRP). Furthermore, the mobility measurements may be intra-frequency or inter-frequency.

For mobility related measurements, measurement activity (or applicability) for intra-frequency and inter-frequency mobility measurements are determined by a measurement configuration provided by network (e.g. SMTC periodicity, measurement gaps etc.). As discussed, in case of IDLE/Inactive mode measurements, also in the CONNECTED mode, UEs maybe provided a threshold (s-MeasureConfig), which the UE can choose to stop the non-serving cell measurements if serving cell quality exceeds the threshold. Also, the applied C-DRX cycle allows UE to apply relaxed measurement activity for (new) neighbour cell identification/measurements.

The RLM, beam failure detection and CSI measurement periodicity are also dependent on the network configuration via the periodicity of associated RS resources. These can be also relaxed using the C-DRX cycle to limit the required activity. The CSI measurements based on CSI-RS are only carried during the active time or when drx-onDurationTimer is running (if configured so by ps-TransmitOtherPeriodicCSI or ps-TransmitPeriodicL1-RSRP). In Rel-17, there is the option to allow further relaxed RLM and BFD evaluations when C-DRX cycle ≤ 80ms.

For the LP-WUS related evaluations in Connected mode, it is important to ensure that the risk of losing service is minimized so that beam management related measurements, such as L1-RSRP and BFD, are not significantly delayed. Limiting or reducing these measurements related to link quality extensively is likely to result in service interruptions. Also delayed/older CSI measurement occasions can degrade the system capacity and/or user throughput. Thus, the assumptions for power saving should also consider the impact of necessary measurements for serving cell quality tracking in connected mode.

Observation 25: 		Reducing the measurement activity in CONNECTED mode can have negative impact on service quality.

Proposal 8: 	If CONNECTED mode operation with LP-WUS is considered, the link quality measurements and reporting need to be accounted for in evaluations.

### Other Procedures for low power WUS

In IDLE/Inactive mode UE can use Discontinuous Reception (DRX), to reduce power consumption. The UE is expected to monitor one paging occasion (PO) per DRX cycle or set of POs in PTW per eDRX cycle. A UE can also be configured with the option to monitor the Paging Early indication (PEI) (except for the UEs expecting multicast session activation notification) to determine whether it should read the actual paging message in PO.

Given these alternative use cases for the LP-WUS, it should be discussed as a part of the LP-WUS/WUR study, the procedures and different configurations supported by LP-WUS. For example, whether or not the LP-WUS triggers UE to monitor PEI and/or PO, or whether the LP-WUS indicates to UEs to go directly to connection establishment and transmit RACH (in RACH occasion). The key difference in the afore options is the requirement on the LP-WUS payload (e.g. sequence ID space) and probability of false (paging) wake-ups of main radio.

If for example, LP-WUS is considered to be PO specific, then the LP-WUS could be cell specific, i.e., payload would cover cell ID and possible PO specific ID. This would require low payload, but would increase the likelihood of unnecessary wake-ups depending on the number of POs and paging load (and UE paging probability). With a slightly larger payload, a similar approach as applied for Rel-17 PEI could be considered, where the LP-WUS payload would in addition to cell ID (and PO ID) carry a sub-grouping indication so as to reduce the false alarm paging probability. With these approaches, the UE would be required to monitor PO (PEI) to confirm that the paging message is intended to sub-group the UE belongs to in question before initiating establishing connection and moving to Connected mode. With a larger payload, the LP-WUS could be considered also to have UE-specific information that could be used to directly indicate to the UE to start RACH procedure and initiate the connection establishment. This solution could offer improved latency and power saving opportunities by eliminating PO/PEI monitoring, but would come at the cost of a more complex LP-WUS and LP-WUR design.

The feasibility of these alternative options should also be considered from the system perspective. For example, with UE-specific LP-WUS, if the indications can only be multiplexed in time and/or frequency domain, the resource reservation of LP-WUS will increase directly as a function of paging load, increasing the cost of deploying LP-WUS.

Observation 26:  		LP-WUS can be sent as an indication of Paging and can enable UE to start RRC state transition to RRC-Connected in different ways depending on the payload –

- LP-WUS as indication of paging for legacy paging group will trigger main radio to monitor paging PDCCH or early paging indication.

- Sub grouping of UE will reduce the unnecessary paging monitoring but increases LP-WUS payload

- LP-WUS with full UE ID may enable skipping paging information monitoring, and minimize power consumption due to false alarm paging monitoring but will increase LP-WUS payload.

Proposal 9:   Consider different alternatives for LP-WUS payloads to support/replace Paging PDCCH monitoring.

# Conclusion

In this contribution we discussed different aspects related to the LP-WUS signal design and procedures. In section 2.1 we considered the signal characteristics such as bandwidth, payload size, modulation and coding and came to the following observations and proposals.

Observation 1:  Wider LP-WUS BW can offer robustness against fading, but impacts the spectrum efficiency and feasibility of multiplexing with other signals

Observation 2:  Having limited LP-WUS BW could allow adoption in different device types and enable use in different type of deployments.

Observation 3:  Wider LP-WUS BW could enable support of more flexibility in signal generation and different signal characteristics and improve the tolerance to implementation impairments.

Observation 4:  The effects of assumed SCS on the LP-WUS symbol duration should be studied.

Observation 5:  It may be beneficial for some HW implementations to have a preamble in order for comparator threshold settling.

Observation 6:  A sync field may be added to help the LR to find the beginning of the WUS payload. The sync field pattern can be also used to distinguish content and target group.

Observation 7:  Payload field can be used to carry relevant information and have different content and/or sizes based on the use case/procedure.

Observation 8:  A FCS/CRC check field calculated on the wake-up message payload will lower the FAR and ensure the integrity of the payload.

Observation 9: 	To achieve a good trade-off between sensitivity and power consumption, it may be necessary to introduce DRX on the wakeup receiver (as opposed to an always on receiver).

Observation 10: 	The LP-WUS signal reception can suffer from intercell interference and other serving cell signals, which may degrade the false detection performance thereby reducing the power consumption savings from the introduction of the LP-WUS.

Observation 11:  If DFT-s-OFDM type of modulation is considered, effect of CP in the transmission should be studied, if LP-WUS spans multiple OFDMA symbols.

Observation 12:  The performance of multi-bit OOK degrades significantly in comparison with that of AWGN channel due to multi-path effect.

Observation 13:  Oversampling at the LR is essential to improve the performance of OOK reception and to reduce the probability of false alarm.

Observation 14:  By using properly designed sequence in the ON duration of LP-WUS transmission, LRs can improve the coverage and can also overcome the inter-cell interference

Observation 15:  The LP-WUS modulation should be resource efficient accounting need for possible guard bands, device BW restrictions and efficient multiplexing with other LP-WUS and other legacy signals.

Observation 16:  The LP-WUS modulation scheme selected should provide robust performance (sensitivity and selectivity) using low power receiver architectures.

Observation 17:  A LP-WUS designed to support a flexibly sized payload would allow the LP-WUS to be easily adopted for different use cases.

Observation 18:  The LP-WUS modulation scheme selected should be easy to generate using the existing gNB architecture.

Observation 19:  Possibility of multiplexing a spreading sequence in the ON duration of DFT-s-OFDM should be studied to enhance coverage and to improve detection performance.

Observation 20:  LP-WUS using sequence based CP-OFDM must be studied in this SI, since it has been used in previous generation cellular standard

Observation 21:  The modulation scheme used for LP-WUS must be chosen to reduce both power and latency of majority of UEs in the cell rather targeting a subset of UEs whose channel conditions are good.

Observation 22: 	If eDRX based evaluation rate is assumed, the mobility of the devices should be assumed to be restricted to semi-stationary to avoid service interruptions due to delayed mobility evaluations. Alternatively, if higher mobility (than semi-stationary) is assumed, the service should be able to tolerate longer latency.

Observation 23: 	If LP-WUS is considered for mobility measurements, the LP-WUS design would need to  account the requirement to enable good accuracy within reasonable evaluation period. This may imply increased overhead.

Observation 24:  RX chain imperfections of LP-WUR, as well the baseline reference architecture affects the feasibility of LP-WUS based mobility measurements.

Observation 25:  Reducing the measurement activity in CONNECTED mode can have negative impact on service quality.

Observation 26:  	LP-WUS can be sent as an indication of Paging and can enable UE to start RRC state transition to RRC-Connected in different ways depending on the payload –

- LP-WUS as indication of paging for legacy paging group will trigger main radio to monitor paging PDCCH or early paging indication.

- Sub grouping of UE will reduce the unnecessary paging monitoring but increases LP-WUS payload

- LP-WUS with full UE ID may enable skipping paging information monitoring, and minimize power consumption due to false alarm paging monitoring but will increase LP-WUS payload.

Proposal 1: 	The SI evaluates the LP-WUS BW accounting performance/robustness and applicability to different device types and deployments.

Proposal 2: 	The SI evaluates the benefits and the cost of supporting different LP-WUS fields.

Proposal 3:	The SI evaluates the potential benefits and drawbacks of supporting fixed wakeup occasions allowing the wakeup receiver to enter a lower power DRX mode in between such occasions.

Proposal 4:	The SI considers techniques to improve the robustness of the LP-WUS to inter-cell and intra-cell interference.

Proposal 5: 	Clarify the mobility assumption for the purpose of LP-WUS evaluations and design and ensure that assumptions are aligned with other assumptions.

Proposal 6: 	Evaluate the possible alternatives and feasibility to reduce the need of MR based RRM measurements with limited mobility performance impact.

Proposal 7: 	Evaluate new LR/LP-WUS performance measurements/alarms to assist the network to optimise the LP-WUS configuration.

Proposal 8: 	If CONNECTED mode operation with LP-WUS is considered, the link quality measurements and reporting need to be accounted for in evaluations.

Proposal 9:   Consider different alternatives for LP-WUS payloads to support/replace Paging PDCCH monitoring.

# References

[1]   RP-222644   Revised SID: Study on low-power Wake-up Signal and Receiver for NR       Vivo

[2]   R1-2211271  L1 signal design and procedures for low power WUS								Nokia

[3]   R1-2300698  Low power WUS Evaluation Methodology								Nokia, Nokia Shangai Bell