3GPP TSG RAN WG1 #112		R1-2300747

Athens, Greece, Feb. 27th – Mar.3rd, 2023

Agenda Item:	9.2.3.2

Source:	Fujitsu

Title:	Sub use cases and specification impact on AI/ML for beam management

Document for:	Discussion/Decision

Introduction

In RAN1#111 meeting, the following agreements [1] have been achieved about sub use cases and specification impacts on AI/ML for beam management.

Agreement For the sub use case BM-Case1 and BM-Case2, at least support Alt.1 and Alt.2 for AI/ML model training and inference for further study: - Alt.1. AI/ML model training and inference at NW side - Alt.2. AI/ML model training and inference at UE side - The discussion on Alt.3 for BM-Case1 and BM-Case2 is dependent on the conclusion/agreement of Agenda item 9.2.1 of RAN1 and/or RAN2 on whether to support model transfer for UE-side AI/ML model or not - Alt.3. AI/ML model training at NW side, AI/ML model inference at UE side Agreement For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on the following L1 reporting enhancement for AI/ML model inference - UE to report the measurement results of more than 4 beams in one reporting instance - Other L1 reporting enhancements can be considered Agreement Regarding the data collection for AI/ML model training at UE side, study the potential specification impact considering the following additional aspects. - Whether and how to initiate data collection - Configurations, e.g., configuration related to set A and/or Set B, information on association/mapping of Set A and Set B - Assistance information from Network to UE (If supported) - Other aspect(s) is not precluded Agreement Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the necessity and the potential specification impacts from the following aspects: - UE reporting of beam measurement(s) based on a set of beams indicated by gNB. - Signalling, e.g., RRC-based, L1-based. - Note: Performance and UE complexity, power consumption should be considered.

This contribution presents our views on AI/ML for DL beam prediction and the potential specification impacts.

Discussion on the sub use case

Discussion on the types of beam prediction

In RAN1-111 meeting, the following proposal is discussed in [2] for the type of beam prediction.

Proposal 3.2: For the sub use case BM-Case1 and BM-Case2, focus on Alt.1 and Alt.3 for the predicted beams for further study - Note: Alt.1 and Alt.3 were agreed in RAN1#110 meeting as below - Alt.1: DL Tx beam prediction - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam)

From our understanding, the study for the types of beam prediction needs to consider its feasibility. The feasibility on the types of beam prediction for NW-side model and UE-side model may be different. For Tx beam prediction with UE-side model, the Tx beam information is necessary to be informed from gNB to UE. In this case, the feasibility on how and what information to be informed needs to be studied. However, for NW-side model, the gNB intrinsically knows the Tx beam information. For the Tx-Rx beam pair prediction with NW-side, the Rx beam information is necessary to be informed from UE to gNB. The feasibility on how and what information to be informed needs to be studied. But if the AL/ML model is deployed in UE side, the Rx beam information is intrinsically known by UE.

The discussion on such feasibility is time consuming and not easy to achieve the agreement. For the sake of progress, it is suggested to have high priority on the study of DL Tx beam prediction with NW-side model and DL Tx-Rx beam pair prediction with UE-side model.

Proposal 1: For the sake of progress, it’s suggested to prioritize the following types of beam prediction for further study

- Alt.1: DL Tx beam prediction with NW-side model

- Alt.3: DL Tx-Rx beam pair prediction with UE-side model

Potential specification impacts

Discussion on the data collection

In RAN1-111 meeting, the following proposal about the AI/ML model training at NW side is discussed in [2].

Proposal 5.2.1: Regarding the data collection mechanism for AI/ML model training at NW side, study the following alternatives options as a starting point. - Opt.1Alt.1: UE measures the beams of Set A and report M1 L1-RSRPs optionally with the corresponding RS indicators, where M1 can be larger than 4 - FFS: the range of M1 - Opt.2Alt.2: UE measures the beams of Set B and report M2 L1-RSRPs optionally with the corresponding RS indicators, where M2 can be larger than 4, measures the beams of Set A and report M3 L1-RSRPs optionally with the corresponding RS indicator, where M3 can be larger than 4, - FFS: the range of M2, M3 - Note1: the measurement and reporting related to Set A may be separate from/transparent to the operations related to Set B - Opt.3Alt.3: UE measures the beams of Set B and report M4 L1-RSRPs optionally with the corresponding RS indicator, where M4 can be larger than 4, measures the beams of Set A and report M5 RS indicators corresponding to the best beam(s) - FFS: the range of M4, M5 - Note2: the measurement and reporting related to Set A may be separate from/transparent to the operations related to Set B - Other option(s) alternative(s) is not precluded - Note3: Data collection for model training may be implemented by gNB in a transparent way - Note4: Potential down-selection/prioritization will be discussed later - Note5: UE complexity and power consumption should be considered for the above options

According to the above proposal, from our understanding, the opt.1 is for the case that Set B is subset of Set A and opt.2 is for the case that the Set B is different from Set A. However, the opt.3 may be for both cases with less overhead since only RS indicators of the best beams are reported for the measured beams of Set A. Generally, we agree on this proposal.

Form this proposal, UE should report the measurement results of beam configured by gNB according to the requirement of AI/ML model. For example, for opt.2, the gNB configures the RSs of Set B and Set A, and UE measures and reports the results of Set B and Set A. It is observed that the UE behaviour on the measurement reporting is different from the legacy method. In legacy method, which measurement results of beams are reported is an implementation issue of UE. However, for NW-side model, which beams will be reported depends on the configuration of gNB.

Proposal 2: Regarding the data collection for AI/ML model training at NW-side, study the potential specification impacts on the UE behavior of measurement reporting.

Regarding the data collection for AI/ML model training at NW side, all the options require the number of measurement results in one report to be larger than 4. When the measurement results of Set A are required to be reported, e.g., opt.1, the number of measurement results increase a lot compared with the legacy beam reporting. For example, it’s assumed that the measured L1-RSRPs of 64 beam pairs (16 Tx beams and 4 Rx beams) are used to predict the L1-RSRPs of total 256 beam pairs (32 Tx beams and 8 Rx beams). The measurement results about 256 beam pairs of Set A will be reported to NW for data collection. In this case, the overhead increases 64 times compared with at most four measured results reported to NW.

Proposal 3: Regarding the data collection for AI/ML model training at NW side, the mechanism of reporting overhead reduction is suggested to be studied.

From our understanding, the data collection happens infrequently. The overhead within one report can be balanced with multiple reporting occasions. The enhanced signaling/procedure on the reporting configuration is necessary to be studied.

Proposal 4: Study the potential specification impacts on the enhanced signaling/procedure of reporting configuration for data collection.

Discussion on the model inference

Regarding the inference of NW-side model, it has the similar specification impacts on the UE behavior of measurement reporting. The measurement results of Set B are required to be reported from UE and which beams will be reported for model inference depends on the configuration of gNB. However, in legacy beam reporting, which beams are reported depends on the implementation of UE. In this case, the UE behavior on the measurement reporting is different for the NW-side model compared with the legacy method.

Proposal 5: Regarding the inference of NW-side model, study the potential specification impacts on the UE behavior of measurement reporting.

For NW-side model, if DL Tx-Rx beam pair prediction is supported, the Rx beam information should be included in the measurement report especially for the variable set B since the Rx beam information is transparent to gNB. In terms of how to define the Rx beam information, one candidate is the physical beam information such as beam angle. The other is the logical beam information such as beam ID, but how to map the logical beam into physical beam needs to be further studied.

Proposal 6: For the DL beam pair prediction with a NW-side model, study the potential specification impacts on the Rx beam information included in report instance.

Proposal 7: Regarding the Rx beam information included in report instance for the DL beam pair prediction with a NW-side model, it is suggested to study

- Physical beam information (e.g., beam angle)

- Logical beam information (e.g., beam ID)

  - FFS: How to map the logical beam into physical beam

For the UE-side model, if the AI/ML model information is transparent to gNB, the gNB does not know which RSs corresponding to the Tx beams are necessary to be transmitted for model inference. A signaling about the required Tx beams of Set B from UE to gNB is necessary for model inference. And how to map the Set B to Set A needs to be studied for the required Tx beams.

Proposal 8: For DL beam (pair) prediction with a UE-side model, study the potential specification impacts of model inference on

- The interactions with NW about the required Tx beams of Set B

- The mechanism of the mapping Set B to Set A

Discussion on the model monitoring

In RAN1-110b meeting, the following proposal on the model monitoring is agreed.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection: - Atl1. UE-side Model monitoring - UE monitors the performance metric(s) - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Atl2. NW-side Model monitoring - NW monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation - Alt3. Hybrid model monitoring - UE monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

For the model monitoring with a UE-side AI/ML model, from our understanding, the NW should be aware of the model to make related configuration. For example, the NW only configures part of Tx beams for the inference of AI/ML model when it is activated in UE side. When the AI/ML model is deactivated, the NW may be fallback to the legacy configuration for beam sweeping procedure. In this case, it’s suggested that NW makes decisions of model selection/activation/deactivation /switching/fallback operation.

Proposal 9: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, the Alt.2 and Alt.3 are suggested to be prioritized for further study of model monitoring.

For the hybrid model monitoring, when performance metrics are calculated in UE side, it’s necessary to consider the robust calculation results with the fluctuation of wireless channel conditions. In this case, the filtering on the instant results can be considered. And gNB can configure the filtering parameters for the metrics calculation. When the performance metrics are calculated, it’s not necessary to feedback them to NW for each monitoring instance. To reduce the overhead of signaling, it is suggested to configure the trigger-event for the monitoring results feedback.

Proposal 10: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the potential specification impacts of hybrid model monitoring on

- The filtering on the performance metric calculation

- The tigger-event for reporting of monitoring results

- Conclusion

In this contribution, we present our views on sub use cases and potential specification impacts of AI/ML beam management. For the discussion, we have the following proposals.

Proposal 1: For the sake of progress, it’s suggested to prioritize the following types of beam prediction for further study

- Alt.1: DL Tx beam prediction with NW-side model

- Alt.3: DL Tx-Rx beam pair prediction with UE-side model

Proposal 2: Regarding the data collection for AI/ML model training at NW-side, study the potential specification impacts on the UE behavior of measurement reporting.

Proposal 3: Regarding the data collection for AI/ML model training at NW side, the mechanism of reporting overhead reduction is suggested to be studied.

Proposal 4: Study the potential specification impacts on the enhanced signaling/procedure of reporting configuration for data collection.

Proposal 5: Regarding the inference of NW-side model, study the potential specification impacts on the UE behavior of measurement reporting.

Proposal 6: For the DL beam pair prediction with a NW-side model, study the potential specification impacts on the Rx beam information included in report instance.

Proposal 7: Regarding the Rx beam information included in report instance for the DL beam pair prediction with a NW-side model, it is suggested to study

- Physical beam information (e.g., beam angle)

- Logical beam information (e.g., beam ID)

  - FFS: How to map the logical beam into physical beam

Proposal 8: For DL beam (pair) prediction with a UE-side model, study the potential specification impacts of model inference on

- The interactions with NW about the required Tx beams of Set B

- The mechanism of the mapping Set B to Set A

Proposal 9: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, the Alt.2 and Alt.3 are suggested to be prioritized for further study of model monitoring.

Proposal 10: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the potential specification impacts of hybrid model monitoring on

- The filtering on the performance metric calculation

- The tigger-event for reporting of monitoring results

- Reference

[1] Draft_Minutes_report_RAN1#110b-e_v010, online 10th-19th 2022

[2] R1-2210357 Summary#5 for other aspects on AI/ML for beam management