3GPP TSG RAN WG1 #112	R1-2300771

Meeting, Fen 11th – 18th, 2022

Source:	CableLabs

Title:	Clarifications of SL-U Channel Access Mechanism

Agenda Item:	9.4.1.1

Document for: 	Discussion and Decision

# Introduction

1.1 Forwarding COT

During RAN1-110bis-e, the following agreement has been reached [1], [2]:

Agreement

- For UE-to-UE COT sharing, continue considering the following alternatives:

  - Alt. 1: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the at least COT initiating UE’s PSSCH data transmission in the COT.

--------------------------

- gNB relaying/forwarding a UE initiated COT to another UE is not supported in Rel-18

----------------------------

The following are agreements/proposals made during the last meeting (RAN1#111) We would like to discuss some of the FFS listed below.

Agreement

For UE-to-UE COT sharing,

- When performing S-SSB transmission(s), a responding UE can utilize a COT shared by a COT initiating UE (using type 1 channel access) when the responding UE is intended to transmit S-SSB within RB set(s) corresponding to the shared COT.When performing PSFCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE.

  - FFS: whether a responding UE can transmit PSFCH(s) to UE(s) other than the initiator

- When performing PSSCH/PSCCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when the responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to the shared COT is intended for the COT initiating UE

  - FFS whether to support the case if a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information.

    - FFS: how to determine / what are the restrictions to the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) to utilize the COT shared by the initiating UE.

    - FFS whether the responding UE can utilize the COT when at least the responding UE’s PSCCH transmission in the reserved resources within the shared COT or MCSt is intended for the COT initiating UE and what are the restrictions (e.g., priority, etc.) and indication to the responding UE.

- FFS: UE forwarding/relaying information about a COT initiated by another UE.

Related to the same topic, during RAN1-111, the following proposal has been discussed [5]:

For UE-to-UE COT sharing,

- When performing S-SSB transmission(s), a responding UE can utilize a COT shared by a COT initiating UE (using type 1 channel access) when the responding UE is intended to transmit S-SSB within RB set(s) corresponding to the shared COT. When performing PSFCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE.

  - FFS: whether a responding UE can transmit PSFCH(s) to UE(s) other than the initiator

----------------------------

- FFS: UE forwarding/relaying information about a COT initiated by another UE.

1.2 Type 2A channel access applicability [5]

During RAN1 #111, the following agreement has been reached:

- Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met:

  - Time duration is at most 1ms per transmission

  - The duty cycle of the S-SSB transmissions is at most 1/20

  - FFS: details of EDT

------------------------------

1.3 Type A or B multi-channel support for SL-U S-SSB/PSFCh transmissions [5]

During RAN1 #111, the following agreement has been reached:

- For dynamic channel access mode with multi-channel case in SL-U, use NR-U DL (Type A or Type B) multi-channel access procedure as the baseline for multiple PSFCH transmissions on multiple channels, where each PSFCH transmission is confined within one LBT channel

  - FFS: the case for S-SSB if agreed to transmit S-SSB (or S-SSB can be (pre-)configured) in more than one RB set

  - FFS: whether type A or type B or both will be supported for this case for PSFCH

  - FFS: whether multiple PSFCH transmissions on multiple channels after performing the multi-channel access procedure is limited to contiguous RB sets

# Discussion

2.1 COT forwarding

TS 37.213 Section #4.1.2 [4] specifies the conditions under gNB could execute transmissions, re-using a COT initiated by a UE. Also [4] doesn’t support gNB relaying/forwarding transmissions within a COT initiated by another UE. Hence [2] prohibited a gNB to relay/forward a UE COT to another UE.

RAN1 #111 agreed on supporting PSFCh from a responding UE back to the initiator UE, sharing the COT originated by the initiating UE. [need to add some sentence to include FFS point, is FFS point about another UE share COTS not destined to COTS initiator??, seems very sudden switch]

- The coverage of a cluster of SL-U UE may partly/fully overlap with a Wi-Fi network coverage.

- Based on the example below (Figure 1), the SL-U cluster of {UE01, UE02, UE03} overlaps partially with the coverage provided by STA01 (both coverages shaped up by the local geography). A propagation blockage exists between {UE01, UE02} and {AP01, STA01}. Dependent on the propagation channel between UE03 and STA01, either STA01 or UE03 acts as a hidden node for the other node, not allowing the other node to execute a clear LBT

- Assuming UE02 originates a shared COT transmission dedicated to UE01 and the latter node forwards the COT to UE03, since i) UE 03 may transmit over STA01 (no proper LBT executed by UE03) or ii) STA01 may transmit over UE03, both cases may be resulting into interference between STA01 and UE03.

Proposal 1

A responding UE shall not transmit PSFCh to UEs, other than the initiating UE

Figure 1. Overlapping coverage between a Mode 2 SL-U cluster of cells and a Wi-Fi AP/STA

2.2 Type 2A channel access clarification

As indicated by section #1.2, the EDT used for S-SSB transmitted by a UE without shared channel occupancy, remained open for further clarifications (concerning Type 2A access). TS 37.213 Section #4.1.5 [4], specified the Energy Detection Threshold (EDT). According with the formulas defined in this section, the Bandwidth employed in the EDT is ‘the single channel bandwidth in MHz’. The NR-U specifications [4] do not specify EDT calculated for a fractional bandwidth (selected number of PRBs).

Proposal 2

The EDT procedure, defined by NR-U specs [4] (e.g., the signle channel bandwidth in MHz) applies to the Type 2A channel access procedure when considering the S-SSB transmission from a UE (without shared channel occupancy).

[4] didn’t cover specifically the SL-U case (e.g. PSFCh). However, PSFCh transmissions could be covered by Type 2A transmissions as defined by [4], since PSFCh could be considered as ‘non-unicast transmissions” per section #4.1.2 [4].

Proposal 3

If Type 2A is applicable to PSFCh (without a shared channel occupancy), apply the same restriction to S-SSB i.e., PSFCh could be multiplexed with S-SSB subject to a duration of at most 1ms and the discovery burst duty cycle is at most 120.

Proposal 4

Changing the duration limit and duty cycle factor of discovery burst multiplexed with non-unicast data or PSFCh, as provided by section 4.1.2 [4] is not supported.

2.3 Multi-channel Type A or B applicability to S-SSB/PSFCh transmisssions

Concerning the opinions sought on the agreement #1.3, we have the following suggestions.

In order to maintain the compatibility with NR-U specs, we support the S-SSB transmission over one RB set.

Proposal 5

S-SSB shall be transmitted over one RB set.

The PSFCh transmissions may be differing between each other, content and time wise. They could be potentially transmitted over multiple channels, and each one of them subject clearing their LBT procedure. These indicate the possibility that one PSFCh transmission may end up later or earlier than other PSFCh transmissions. If so, then all related counters over the rest of the channel should be re-initialized.

Proposal 6

Dynamic multi-channel PSFCh transmissions shall be supported by Type A2.

We do not see the benefits of occupying more than one channel with PSFCh traffic.

Proposal 7

Multi-channel PSFCh transmissions should be limited to a contiguous RB set.

# Conclusions

WE discussed and made the following proposals related to SL-U channel access.

Proposal 1

A responding UE shall not transmit PSFCh to UEs, other than the originator.

Proposal 2

The EDT procedure, defined by NR-U specs [4] applies to the Type 2A channel access procedure when applied to S-SSB transmission from a UE (without shared channel occupancy).

Proposal 3

If Type 2A is applicable to PSFCh (without a shared channel occupancy), then PSFCh could be multiplexed with S-SSB subject to a duration of at most 1ms and the discovery burst duty cycle is at most 120.

Proposal 4

Changing the duration limit and duty cycle factor of discovery burst multiplexed with non-unicast data or PSFCh, as provided by section 4.1.2 [4] is not supported.

Proposal 5

S-SSB shall be transmitted over one RB set.

Proposal 6

Dynamic multi-channel PSFCh transmissions shall be supported by Type A2.

Proposal 7

Multi-channel PSFCh transmissions should be limited to a contiguous RB set.

# References

[1] R1-2207792, “FL summary #4 for AI 9.4.1.1: SL-U channel access mechanism”, RAN1 #110, August 2022

[2] R1-2208321, “3GPP TSG RAN WG1 Meeting #110. Final report’, August 2022

[3] R1-2212802, “FL summary #7 for AI 9.4.1.1: SL-U channel access mechanism”, RAN1- #111

[4] “3GPP TS 37.213. Physical layer procedures for shared spectrum channel access”

[5] 3GPP RAN-1 #111, ‘Draft minutes report v020’, RAN1 #111, November 2022