3GPP TSG RAN WG1 #112	R1-2300849

Athens, GR, February 27th – March 3rd, 2023

Source:	Sharp

Title:	SRS enhancement targeting TDD CJT and 8 TX operation

Agenda Item:	9.1.3.2

Document for:	Discussion and Decision

Background

In RAN#94e, the working item to enhance both downlink and uplink MIMO operations in Rel-18 was agreed [1].

According to the WID, the following item needs to be studied, and if justified, specified:

- enhancement of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints the 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

- SRS enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle Industrial devices

Potential enhancements for 8Tx SRS

In up to RAN1#111 meeting, the following agreements were made in AI 9.1.3.2 for SRS enhancement [2].

Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), at least support: - For comb 2, support 1 and 2 comb offsets - For comb 4, support 2 and [4] comb offset - For comb 8, support 4 comb offsets Agreement For single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH or ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with 8 ports and m OFDM symbols (m > 1), support the case of 8 ports mapped onto the m OFDM symbols - Option 1: Different SRS ports are mapped onto different OFDM symbols (i.e., TDM) - FFS: m can be legacy values, i.e., 2,4,[8,10,12,14]. Agreement For an 8-port SRS resource in a SRS resource set ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with m OFDM symbols (m >= 1), at least support the 8 ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof). - m takes the legacy values, i.e., 1,2,4,8,10,12,14. Agreement For one single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH, when the SRS resource is configured with n ports (n <= 8) and m OFDM symbols (m >= 1), at least support the n ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof). - n can be 8 - m takes the legacy values, i.e., 1,2,4,8,10,12,14.

  - Mapping of 8 SRS ports onto one OFDM symbol

For an 8-port SRS resource in one OFDM symbol, legacy mechanisms (i.e., FDM + Cyclic shift) can be reused as the followings:

|  | Comb 2 Max cyclic shift: 8 | Comb 4 Max cyclic shift: 12 | Comb 8 Max cyclic shift: 6 |
| --- | --- | --- | --- |
| 1 comb offset | 8 cyclic shift values |  |  |
| 2 comb offsets | 4 cyclic shift values | 4 cyclic shift values |  |
| 4 comb offsets |  | [2 cyclic shift values] | 2 cyclic shift values |

The cyclic shift value nSRSCS,i for each comb can be calculated by legacy equation except for comb 8. Since the maximum cyclic shift is 6 for comb 8 and it cannot be divided into 4, the enhancement on the cyclic shift value nSRSCS,i determination for comb 8 is needed. The cyclic shift value nSRSCS,i for each antenna port pi can be calculated as the following:

When comb = 2 with 1 comb offset (8 cyclic shift values),

nSRSCS,i=nSRSCS+nSRSCS,max(pi-1000)NapSRS mod nSRSCS,max.

When comb = 2 with 2 comb offsets, or when comb = 4 with 2 comb offsets (4 cyclic shift values),

nSRSCS,i=nSRSCS+nSRSCS,max(pi-1000)/2NapSRS/2 mod nSRSCS,max.

When comb = 8 with 4 comb offsets (2 cyclic shift values),

nSRSCS,i=nSRSCS+nSRSCS,max(pi-1000)/4NapSRS/4 mod nSRSCS,max.

Proposal 1: For mapping of 8 SRS ports onto one OFDM symbol, the cyclic shift value nSRSCS,i for comb 8 should be calculated as the following:

- nSRSCS,i=nSRSCS+nSRSCS,max(pi-1000)/4NapSRS/4 mod nSRSCS,max

Regarding FFS point for 4 comb offsets for comb 4, 2 cyclic shift values have higher orthogonality than 4 cyclic shift values in the code domain. Furthermore, since the equation enhancement on the cyclic shift value nSRSCS,i in Proposal 1 can be reused, 4 comb offsets have small spec impacts. For these reasons, we can support 4 comb offsets for comb 4.

Proposal 2: For mapping of 8 SRS ports onto one OFDM symbol, we can support 4 comb offsets for comb 4.

  - Mapping of 8 SRS ports onto multiple OFDM symbol

Although it was agreed that 8 SRS ports are mapped onto m OFDM symbols, m is FFS point. In terms of the cyclic shift determination, when m is 2, 4, or 8, the determination of cyclic shift nSRSCS,i for 4, 2, or 1 SRS port(s) can be reused, respectively. For this reason, we can support m is 2, 4, and 8.

Proposal 3: For mapping of 8 SRS ports onto m OFDM symbols, we can support m = 2, 4, and 8.

For channel sounding, the CSI averaged across m OFDM symbols and Msc,bSRS subcarriers is calculated at the gNB side. For this reason, frequency hopping (e.g., partial frequency hopping), power control, and change of UL Tx spatial filter for SRS should be stopped during m OFDM symbols to achieve accurate channel sounding.

Proposal 4: For SRS transmission with 8 SRS ports on m OFDM symbols, frequency hopping, power control, and change of UL Tx spatial filter SRS should be stopped during m OFDM symbols.

  - SRS resource set with usage ‘nonCodebook’

For an SRS resource set with usage ‘nonCodebook’, it was agreed to support 8 1-port SRS resources in the SRS resource set. For the current spec, up to N 1-port SRS resources in the SRS resource set occupy the same RBs in the same symbol, where N is UE capability (i.e., maxNumberSimulaneousSRS-ResourceTx). For accurate channel estimation, a time gap between SRS resources within the same SRS resource set should be as small as possible. Therefore, the maximum number of simultaneous SRS resources should be 8.

Proposal 5: For an SRS resource set with usage ‘nonCodebook’, the maximum number of simultaneously transmitted SRS resources should be 8.

SRI field

Agreement For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, Alt1 is supported, that is - Alt1: A single SRS resource set configured with up to 8 single-port SRS resources - FFS: Configuration of up to two, or four SRS resource sets, each configured with up to 4, or 2 single-port SRS resources, respectively.

For the maximum number of layers is 8, the enhancement on SRI field for nonCodebook is needed. For SRI for nonCodebook, parts or all of SRS resources within the SRS resource set are indicated, and the indicated SRS resources are used as SRS ports and the number of layers. Note that the number of SRS ports is the same as the number of DMRS ports for nonCodebook because precoder matrix W is identity matrix. When NSRS=8 and Lmax=8, the number of all combinations of the SRS resources is calculated as

C18+C28+C38+C48+C58+C68+C78+C88=8+28+56+70+56+28+8+1=255,

and 8 bits (additional 4 bits) are needed for SRI field. In our view, the SRS resource(s) need to be indicated among all combinations to maximize capacity and optimize rank because the optimal combination of the SRS resources depends on channel condition. Therefore, SRI field for nonCodebook should include all combinations of the SRS resources.

Besides, we think the second SRI field cannot be used for indication of part or all of 8 SRS resources within the SRS resource set with usage set to nonCodebook. This is because the second SRI field indicates the same number of layers as the SRI field, and it has lower flexibility for rank indication. In our view, two or four SRS resource sets for 8 SRS resources should be used for MTRP mode. For 8Tx in STRP mode, it is enough that single SRS resource set is used for 8 SRS resources.

Proposal 6: SRI field for nonCodebook should include all combinations of the SRS resources.

SRS capacity enhancement and/or interference randomization for SRS targeting TDD CJT

At RAN1#111 meeting, the following agreements were made in AI 9.1.3.2 for SRS enhancement [2].

Agreement For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, - FFS: Hopping pattern - Support at least hopping based on slot index, OFDM symbol index - FFS: Use of symbol group based on repetition factor - FFS: Additional details on intra-slot hopping based on OFDM symbol index, inter-slot hopping based on slot index, per occasion of SRS resource - FFS: Re-initialization periodicity - Applicable to at least periodic/semi-persistent SRS with usage antennaSwitching - FFS: Other types of SRS - FFS: Configuring a subset of comb offsets / cyclic shifts for comb offset hopping / cyclic shift hopping, respectively - FFS: Combined comb offset hopping and cyclic shift hopping, supporting both, or down selecting one Agreement For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with a network-configured ID. - FFS: The ID could be cell ID nIDcell, SRS sequence identity nIDSRS, C-RNTI, or a new ID - FFS: The relation between the legacy group / sequence hopping and the new hopping Agreement For SRS interference randomization, support one from the following options (to be decided in RAN1#112): - Opt. 1: Cyclic shift hopping - Opt. 2: Comb offset hopping - Opt. 3: Both cyclic shift hopping and comb offset hopping - FFS: details including whether to support separate and/or combined hopping - FFS: details on UE capability and signaling Conclusion No consensus on enhanced signaling for flexible SRS transmission in Rel-18 Agreement For SRS TD OCC for SRS enhancements for TDD CJT, study: - Comparison against SRS on 1 OFDM symbol - Comparison against SRS repeated on multiple OFDM symbols - Study the following aspects: evaluation performance, SRS overhead, per-symbol per-port transmission power, impact of channel delay, dropping rules of collision with other uplink resource, etc. Conclusion The discussion of resource mapping for SRS transmission based on network-provided parameters or system parameters is merged into the discussions of other SRS enhancements for TDD CJT. Agreement For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for an SRS resource set: - Option 1: - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha - Each transmission occasion of the SRS resource is towards multiple TRPs - Option 2: - More than 1 power control processes each for a subset of SRS resource of an SRS resource set where each of the power control process is based on a different UL power control parameter set (Po, alpha, and closed loop state) associated with a different DL pathloss RS - Different transmission occasions of the SRS resource can be towards different TRPs Conclusion - No further discussion of increasing the maximum number of cyclic shifts for CJT SRS. - No further discussion of partial frequency sounding extensions for CJT SRS.

In our view, the remaining SRS enhancements are:

- Cyclic shift hopping

- Comb offset hopping

- TD-OCC

- Precoded SRS for DL CSI acquisition

- Power control

TD-OCC

For the current spec, if SRSs on multiple OFDM symbols are partially overlapped with HARQ-ACK, a part of SRSs is partially dropped and other part of SRSs is transmitted because all the SRS ports are mapped to one OFDM symbol and lack of information does not occur. That is, for legacy procedure, overlapping and dropping in the time domain does not cause lack of information. However, if TD-OCC is used, this partial dropping causes lack of information. Furthermore, the sequence hopping, frequency hopping, and repetition are related to OFDM symbol index. If TD-OCC is used to transmit SRS onto multiple OFDM symbols, the sequence hopping and frequency hopping cannot be used or need to be modified. For these reasons, we don’t support TD-OCC for SRS enhancements for TDD CJT.

Proposal 7: We don’t support TD-OCC for SRS enhancements for TDD CJT.

Precoded SRS for DL CSI acquisition

For precoded SRS for DL CSI acquisition, a CJT DL channel estimated based on a CSI-RS for all TRPs is used to make a SRS precoder. Although the UE can report DL CSI to each TRP based on the CSI-RS, the UE can also transmit precoded SRS towards all TRP. For the latter, the number of used SRS ports can be reduced by SRS port-to-antenna mapping of precoding. However, for precoded SRS, there are some overheads such as reception of a CSI-RS from all TRPs, CJT DL channel estimation, and precoding.

Observation 1: For precoded SRS, there are some overheads such as reception of a CSI-RS from all TRPs, CJT DL channel estimation, and precoding.

Power control

We support Option 2 (i.e., Different transmission occasions of the SRS resource can be towards different TRPs).

In Option 1, the SRS transmission power P is expressed as [3]:

P=PO+10 log10M+1NTRPn=1NTRPαn*PLn+h

where PLn is a downlink pathloss for each TRP in dB. For this reason, αnPLn [dB] is averaged across multiple TRPs whose number is NTRP. In Figure 2, we show an example of Option 1 in a simple case that NTRP=2 and α=1. Since the averaged pathloss PL is 50 dB, a gap between PL1 that is pathloss from TRP#1 and PL is 30 dB. Therefore, to reach TRP#1, 210 repetitions are necessary to improve 30=10log10210 dB. Consequently, Option 1 potentially consumes a lot of UL resources and causes a lot of interference.

Furthermore, α should not be averaged across multiple TRPs. This is because it is beneficial that α is determined for each TRP. For example, in Figure 2, the UE is in the edge of Area#1 but is around the center of Area#2. Therefore, a case that α1<α2 is better than other case that (α1+α2)/2  to mitigate interference from TRP#1 to TRP#2. In other words, since α is beneficial to mitigate near-far problem, averaging α is unnecessary.

Figure 2: An example of Option 1.

On the other hand, in Option 2, the number of transmission occasions is always same as the number of TRPs. That is, 2 repetitions are enough in Figure 2. Additionally, α can be determined for each TRP. For this reason, we support Option 2.

Proposal 8: For per-TRP power control, different transmission occasions of the SRS resource should be towards different TRPs.

Comb offset hopping and cyclic shift hopping

In our view, comb offset hopping is beneficial for SRS interference randomization because comb offset hopping is currently not supported except for SRS for positioning. However, we think comb offset hopping has less performance than cyclic shift hopping. When comb = 2, 4, and 8, the number of cyclic shift values are 8, 12, and 6, respectively. For this reason, when comb = 2 or 4, cyclic shift is better. Particularly, comb offset hopping has marginal improvement when comb value is 2. Therefore, our 1st preference is to support cyclic shift hopping and our 2nd preference is to support both cyclic shift hopping and comb offset hopping.

Proposal 9: For SRS targeting TDD CJT, cyclic shift hopping should be supported.

- Second preference is to support both cyclic shift hopping and comb offset hopping

  - Cyclic shift hopping

Hopping granularity

In Figure 3, we show cyclic shift value (i.e., αj2πnnSRSCS,inSRSCS,max) when sequence length is 8, the number of SRS symbols is 4 symbols, repetition factor R is 2. At the left side of Figure 2 (legacy mapping), each cyclic shift value is constant across 4 symbols. On the other hand, at the right side (proposal mapping), cyclic shift value is hopped for each pair of adjacent 2 symbols.

In proposal mapping, initial value of cyclic shift nSRSCS which is determined by higher layer parameter in the current spec can be dynamically updated by nSRS,l''CS,i as the following:

nSRSCS,i=nSRSCS+nSRS,l''CS'+nSRSCS,max(pi-1000)/2NapSRS/2 mod nSRSCS,max, if NapSRS=4 and nSRSCS,max=6 nSRSCS+nSRS,l''CS'+nSRSCS,max(pi-1000)NapSRS mod nSRSCS,max, otherwise,

where l'' is symbol group index based on repetition factor R (e.g., l''=0,…,NsymbSRSR-1) to has flexibility of hopping granularity.

In Figure 2/3/4/5, we show proposed cyclic shift hopping at the right side. Initial value of cyclic shift is dynamically and implicitly/explicitly updated based on pair of adjacent R symbols.

Proposal 10: For cyclic shift hopping, a value offset of cyclic shift nSRS,l''CS' should be added into nSRSCS,i.

Proposal 11: For cyclic shift hopping, a value offset of cyclic shift nSRS,l''CS' should be determined for each pair of adjacent R symbols (e.g., l''=0,…,NsymbSRSR-1).

Hopping pattern

It was agreed that the value offset of cyclic shift nSRS,l''CS' is determined based on the pseudo-random sequence c(i), initialized with a network-configured ID. In our view, the network-configured ID for SRS sequence ID can be reused for initialization of hopping. Since the cell ID is cell-common ID, it is not suitable for intra-cell MTRP case.

Proposal 12: For cyclic shift hopping pattern, the SRS sequence ID should be used for the initialization of the pseudo-random sequence c(i).

Figure 3: Example of cyclic shift hopping for antenna port 1000/1001 when NsymbSRS is 4 and R is 2 adjacent symbols.

Figure 4: Example of cyclic shift hopping for antenna port 1002/1003 when NsymbSRS is 4 and R is 2 adjacent symbols.

Figure 5: Example of cyclic shift hopping for antenna port 1000/1001 when NsymbSRS is 4 and R is 1 symbol.

Figure 6: Example of cyclic shift hopping for antenna port 1002/1003 when NsymbSRS is 4 and R is 1 symbol.

Conclusion

In this contribution, we have the following proposal:

Proposal 1: For mapping of 8 SRS ports onto one OFDM symbol, the cyclic shift value nSRSCS,i for comb 8 should be calculated as the following:

- nSRSCS,i=nSRSCS+nSRSCS,max(pi-1000)/4NapSRS/4 mod nSRSCS,max

Proposal 2: For mapping of 8 SRS ports onto one OFDM symbol, we can support 4 comb offsets for comb 4.

Proposal 3: For mapping of 8 SRS ports onto m OFDM symbols, we can support m = 2, 4, and 8.

Proposal 4: For SRS transmission with 8 SRS ports on m OFDM symbols, frequency hopping, power control, and change of UL Tx spatial filter SRS should be stopped during m OFDM symbols.

Proposal 5: For an SRS resource set with usage ‘nonCodebook’, the maximum number of simultaneously transmitted SRS resources should be 8.

Proposal 6: SRI field for nonCodebook should include all combinations of the SRS resources.

Proposal 7: We don’t support TD-OCC for SRS enhancements for TDD CJT.

Proposal 8: For per-TRP power control, different transmission occasions of the SRS resource should be towards different TRPs.

Proposal 9: For SRS targeting TDD CJT, cyclic shift hopping should be supported.

- Second preference is to support both cyclic shift hopping and comb offset hopping

Proposal 10: For cyclic shift hopping, a value offset of cyclic shift nSRS,l''CS' should be added into nSRSCS,i.

Proposal 11: For cyclic shift hopping, a value offset of cyclic shift nSRS,l''CS' should be determined for each pair of adjacent R symbols (e.g., l''=0,…,NsymbSRSR-1).

Proposal 12: For cyclic shift hopping pattern, the SRS sequence ID should be used for the initialization of the pseudo-random sequence c(i).

Observation 1: For precoded SRS, there are some overheads such as reception of a CSI-RS from all TRPs, CJT DL channel estimation, and precoding.

References

- [1]	RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Dec., 2021.

- [2]	RAN1 chair’s note, RAN1#111 meeting, Nov. 2022.

- [3]	R1-2210848, FUTUREWEI, “SRS enhancements for TDD CJT and 8TX operation”, Nov., 2022.