3GPP TSG RAN WG1 #112	R1-2300855

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.6.1

Source:	NEC

Title: 	Discussion on Rel-18 RedCap UE

Document for:	Discussion and Decision

Introduction

In RAN1#111, maximum number of PRBs for Rel-18 RedCap UE was narrowed-down as follows:

Agreement

For UE BB bandwidth reduction, for PUSCH, down-select between the following options for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable:

- Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS

- Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS

For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), down-select between the following options for the maximum number of PRBs that the UE can process per slot:

- Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS

- Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS

Same option will be selected for both PDSCH and PUSCH.

In RAN#98, the following agreements were made and WID has been updated accordingly [1]:

For Issue – 1 UE peak rate reduction as a standalone feature

Conclusion

- Revisit in RAN#99 (no additional discussion in RAN1 in 1Q’23)

For Issue – 2 Additional separate early indication

Agreement

- Update the WID by adding objective:

  - Support additional separate early indication(s) (RAN1, RAN2)

  - How to support is decided by WGs

In this contribution, we provide our view on Rel-18 RedCap UE.

Discussion

Maximum number of PRBs

In RAN1#111, options for maximum number of PRBs for PUSCH/PDSCH was narrowed down to option 3 or 4.

The maximum number of PRBs of option 4 is identical to “maximum transmission bandwidth configuration” of 5 MHz channel bandwidth for both 15 kHz and 30 kHz SCS [2]. There might be a merit of adopting the same value as existing value of “maximum transmission bandwidth configuration” from standardization effort point of view. At least, there is no need to introduce a new number of PRBs for maximum transmission bandwidth configuration in RAN4 specification(s).

Option 3 is the same as RAN4 specification for 15 kHz SCS only, but different for 30 kHz SCS. For 30 kHz SCS, it was said that option 3 is more flexible than option 4 because 12 PRB of option 3 can fully be utilized for uplink resource allocation with DFT-s-OFDM but 11 PRB of option 4 cannot. On the other hand, a new number of PRBs for maximum transmission bandwidth configuration may need to be introduced into RAN4 specification(s).

Proposal:

- Adopt option 4 for the maximum number of PRBs for PUSCH/PDSCH, which will introduce no new number of PRBs for “maximum transmission bandwidth configuration” of RAN4 specifications.

Additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs

In RAN1#111, no support for configuration of an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs was proposed by FL as follows, and the majority of companies supported the proposal:

FL1/FL2/FL3 Medium Priority Proposal 2.4-1a: For a cell supporting both Rel-17 and Rel-18 RedCap UEs,

- Configuration of an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is not supported.

We also do not see the need for such optimization. It would rather increase the overhead of the carrier and may introduce more segmentation into uplink carrier.

Proposal:

- Agree with FL proposal in RAN1#111 that configuration of an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is not supported.

Additional separate early indication(s)

Early identification of Rel-18 RedCap UE would be needed for the network and feasible with indication in Msg3. From RAN1 point of view, there would be no technical issue to support additional separate early indication in Msg3. As early indication of Msg3 was decided and introduced by RAN2 in Rel-17 RedCap WI, decision should also be up to RAN2 in Rel-18.

Proposal:

- From RAN1 point of view, there would be no technical issue to support additional separate early indication in Msg3. Support of additional separate early indication of Rel-18 RedCap UE in Msg3 can be up to RAN2. RAN1 does not need to discuss this further.

In RAN1#111, it has been agreed that Msg2 can be larger than 5MHz for Rel-18 RedCap UE. Therefore, early identification between Rel-17 and Rel-18 RedCap UE is not needed for the purpose of limiting bandwidth of Msg2. On the other hand, it has been agreed that bandwidth of Msg3 for Rel-18 RedCap UE is limited up to the maximum number of PRBs. Though it could be handled by gNB implementation, if gNB would like to differentiate the bandwidth of Msg3 between Rel-17 and Rel-18 RedCap, early indication by Msg1 would be needed. For this purpose, Rel-17 framework can be reused, and no enhancement is needed from RAN1 point of view.

Proposal:

- Rel-17 framework of early indication of RedCap UE can be reused for additional separate indication of Rel-18 RedCap UE if gNB would like to differentiate bandwidth of Msg3 between Rel-17 and Rel-18 RedCap UE.

Summary

In this contribution, we discussed the maximum number of PRBs for Rel-18 RedCap UE and additional separate early indication of Rel-18 RedCap UE and made proposals as follows:

Proposal:

- Adopt option 4 for the maximum number of PRBs for PUSCH/PDSCH, which will introduce no new number of PRBs for “maximum transmission bandwidth configuration” of RAN4 specifications.

Proposal:

- Agree with FL proposal in RAN1#111 that configuration of an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is not supported.

Proposal:

- From RAN1 point of view, there would be no technical issue to support additional separate early indication in Msg3. Support of additional separate early indication of Rel-18 RedCap UE in Msg3 can be up to RAN2. RAN1 does not need to discuss this further.

Proposal:

- Rel-17 framework of early indication of RedCap UE can be reused for additional separate indication of Rel-18 RedCap UE if gNB would like to differentiate bandwidth of Msg3 between Rel-17 and Rel-18 RedCap UE.

Reference

[1] RP-223544	Enhanced support of reduced capability NR devices	Ericsson

[2] TS 38.101-1