3GPP TSG RAN WG1 Meeting #112	R1-2300860

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.14.1

Source: 	Lenovo

Title: 	PRACH coverage enhancements

Document for:	Discussion

Introduction

The Rel-18 WI further NR coverage enhancements was agreed during RAN#94-e meeting [1], where one of the objectives is targeted to specify PRACH coverage enhancements, and the detailed objectives in the WID are listed below:

- Specify following PRACH coverage enhancements (RAN1, RAN2) - Multiple PRACH transmissions with same beams for 4-step RACH procedure - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable. - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable.

In RAN1#110b-e and RAN1#111 meeting, following agreements have been reached for PRACH coverage enhancement.

Agreement - For multiple PRACH transmissions with same beam, at least support to use same PRACH preamble during the multiple PRACH transmissions in one RACH attempt. - FFS: whether different preambles can be utilized in different PRACH transmissions during the multiple PRACH transmissions in one RACH attempt. Agreement - For multiple PRACH transmissions with same beam, at least ROs located at different time instances can be utilized for the transmissions. - FFS: whether/how the starting RB of ROs can be different at different time instances for multiple PRACH transmissions. - FFS: whether/how multiple PRACH transmissions located in the same time instance, e.g., for UEs with multiple Tx chains. Agreement For multiple PRACH transmissions with same beam, for RAR monitoring, consider the following options. - Option 1: One RAR window per each PRACH transmission, the RAR window follows the legacy design. - FFS: RA-RNTI. - Option 2: Only one RAR window for all of the multiple PRACH transmissions. - FFS: the start position of the RAR window. - FFS: RA-RNTI. Agreement For multiple PRACH transmissions with same Tx beam, support to differentiate at least between multiple PRACH transmissions and single PRACH transmissions. Agreement For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, consider one or multiple of the following options. - Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs. - Option 2: Multiple PRACH are transmitted on separate ROs. - Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs. - Other options are not precluded. - Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. Agreement - Study at least the following case for multiple PRACH transmissions with different Tx beams. - UE uses different TX beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS - FFS: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs /CSI-RSs, where the different SSBs/CSI-RSs are not associated with the same RO. - Note: not related to decision on CFRA Note: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs/CSI-RSs, where the different SSBs/CSI-RSs are associated with the same RO is not considered. - Working Assumption - Simulation results for multiple PRACH transmissions with different beam(s) and same beam(s) (baseline) to be discussed in the next meeting. - Simulation assumptions in TR 38.830 are used as the starting point for the simulation. - Focus on FR2. - UE antenna configuration 2-2-2(baseline), 1-4-1(optional) - Performance metric: 0.1% false alarm, 1% miss-detection - Companies report the number of beams, the beam widths, beam correspondence assumption, and the boresights. - Channel model for link-level simulation: CDL-A defined in table 7.7.1-1 in TR 38.901. - Both that UE fulfills beamCorrespondence requirements Without UL-BeamSweeping and UE fulfils beamCorrespondence requirements With UL-BeamSweeping can be considered in the simulation are used as starting point for simulation. Agreement For multiple PRACH transmissions with same Tx beam, down-select one option from the following options. - Option 1: gNB can only configure one value for the number of multiple PRACH transmissions. - Option 2: gNB can configure one or multiple values for the number of multiple PRACH transmissions. - FFS: details Agreement - For multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt. - Note: whether to support multiple numbers of PRACH transmissions is separately discussed.

In this contribution, we present our views on detailed design to support multiple PRACH transmissions with same beams and discussions on multiple PRACH transmissions with different beams.

Discussion

Currently 5G NR supports preamble transmission without repetition, and preamble transmission takes place in a set of PRACH slots that are configured in each PRACH configuration period. Furthermore, within each PRACH slot, there may be one or multiple frequency domain RACH occasions (ROs), each covering multiple consecutive resource blocks. The ROs are associated with SSBs that may be transmitted with different beams. The association of SSBs (beams) and ROs could be 1-to-1, 1-to-N, or N-to-1 depending on network configuration. And the association is performed periodically in each SSB to RO association period.

## 2.1 Multiple PRACH transmissions with same beams

In Rel-18, to realize multiple PRACH transmission with same beams, some issues should be discussed, which may include:

Issue 1: How does UE determine the RO and preamble to be used for each transmission?

Issue 2: How to determine the number of PRACH transmissions?

Issue 3: How to differentiate the multiple PRACH transmissions with single PRACH transmission?

Issue 4: How to reduce the PRACH transmission latency?

Issue 5: How to determine the PDCCH monitoring window and RNTI for RAR after the PRACH transmissions?

And in the following we will discuss these issues one by one.

Issue 1: How does UE determine the RO and preamble to be used for each transmission?

The UE needs to determine the ROs and preamble to be used for each transmission. As one solution, based on the determined transmission beam (or SSB), the UE could determine a starting RO for PRACH repetition, and then repeat the PRACH transmission in the following ROs that are association with the same beam with the starting RO.  the available starting ROs could be determined implicitly from e.g., the configured/determined PRACH repetition number or through an explicit signaling. Besides, it is expected that the available starting RO positions are aligned between the gNB and the UEs, such that the gNB could know from where to start the reception for a PRACH transmission.

Figure1 shows an example, the UE selects SSB#0 and transmits PRACH repetition in the ROs that are associated with SSB#0. The PRACH repetition number is 4, and the starting RO periodicity is as 4, same with the PRACH repetition number. When an event triggers a PRACH procedure, the UE selects the nearest starting RO to transmit the PRACH repetition.

Figure 1 Starting ROs for PRACH repetition

Alternatively, RO bundle is configured to the UE and PRACH repetition is transmitted in the ROs in an RO bundle. The RO bundle contains a set of ROs that are associated with a same SSB.

Figure 2 shows an example, the PRACH repetition number is 4 and correspondingly an RO bundle contains 4 ROs that are associated with a same SSB. The UE selects SSB#0 and transmits PRACH repetition in the ROs that are associated with SSB#0. When an event triggers a PRACH repetition, as shown in the figure, the UE starts the PRACH transmission in a nearest RO bundle.

Figure 2 RO bundles for PRACH repetition

Proposal 1: Two options are proposed for RO determination for PRACH repetition

- Opt.1: Starting ROs are configured or implicitly determined, and PRACH repetition starts from a starting RO and repeats in the following ROs that are associated with a same SSB.

- Opt.2: RO bundles are configured and PRACH repetition is transmitted in the ROs of an RO bundle.

It has been agreed in last RAN1 meeting that for multiple PRACH transmissions with same beam, at least ROs located at different time instances can be utilized for the transmissions. We don't think the repetition shall be performed in the FDMed ROs in same time instance, otherwise there will be little or even no performance improvement on PRACH detection due to transmit power division in the FDMed ROs.

Proposal 2: The PRACH repetition should not be performed in FDMed ROs.

When FDMed ROs are configured, the UE needs to determine one RO out of the FDMed ROs for PRACH repetition, and the RO is associated with the target (best) beam. Due to the flexibility of RO configuration and association between ROs and SSBs, there might be one or multiple FDMed ROs associated with the same target SSB. And these ROs might be in same/different position in frequency domain in different PRACH slots.

Figure 3 shows an example, where the configured SSBs for PRACH transmission is {0,1,2,3,4,5,6,7}. Assuming SSB-PerRACH-Occasion = 1/2, meaning that 2 consecutive ROs are mapped to a same SSB, Msg1-FDM = 4, meaning that there are 4 FDMed ROs in frequency domain. As a result, two FDMed ROs are mapped to a same SSB, as shown in the figure. For a UE that chooses SSB#0 as the target beam, the UE needs to determine one RO out of the two FDMed ROs mapped to SSB#0 for PRACH repetition.

Figure 3 An example for PRACH transmission in the FDMed ROs

Figure 4 shows another example, where the configured SSBs for PRACH transmission is {0,1,2,3,4,5}. Assuming SSB-PerRACH-Occasion = 2, meaning that 2 SSBs are mapped to a same RO, Msg1-FDM = 4, meaning that there are 4 FDMed ROs in frequency domain. The resulted pattern for SSB to RO association is shown in the figure. The ROs associated with the same set of SSBs are in different frequency position in different time instance, also the number of targets ROs out of the FDMed ROs is different. For a UE that chooses SSB#0 as the target beam, the UE needs to determine one RO out of the two FDMed ROs mapped to SSB#0 for PRACH repetition, as shown in the figure.

Figure 4 An example for PRACH transmission in the FDMed ROs

Proposal 3: Study RO determination for PRACH transmission when FDMed ROs are configured and are associated with same SSB.

As for the preamble determination, it has been agreed that at least support to use same PRACH preamble during the multiple PRACH transmissions in one RACH attempt. This allows gNB to perform combining on multiple detection statistics for better performance. It is not preferred to use different preambles in different PRACH repetitions of an PRACH attempt, since this means higher standards impact and complex PRACH preamble allocation considering for some of the UEs, a same preamble is allocated for each UE, while others need to be allocated with a different set of preambles for PRACH repetition. Besides, we don’t think there is clear performance gain when combining the different preambles in a PRACH attempt over that when combing the same preamble. Based on these, we don’t think different preambles can be utilized in different PRACH transmissions during the multiple PRACH transmissions in one RACH attempt.

Proposal 4: Different preambles cannot be utilized in different PRACH transmissions during the multiple PRACH transmissions in one RACH attempt.

Issue 2: How to determine the number of PRACH transmissions?

Before PRACH transmission, UE should determine the number of transmissions. For the number of transmissions, it has been agreed to down-select one form the following options:

- Option 1: gNB can only configure one value for the number of multiple PRACH transmissions.

- Option 2: gNB can configure one or multiple values for the number of multiple PRACH transmissions.

  - FFS: details

We think option 2 is better than option 1, because multiple values could be configured to adapt to different channel condition and it’s up to gNB to determine one or multiple value are configured. As for how to determine one from the configured multiple values, it has been agreed that at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt. So maybe the most straight forward method is to configure multiple SSB-RSRP thresholds range which is corresponding to multiple values for the number of multiple PRACH transmissions and a specific number of transmissions is used for PRACH transmission if the measured RSRP is in a configured RSRP threshold range.

Proposal 5: gNB can configure one or multiple values for the number of multiple PRACH transmissions (Option 2).

- Multiple SSB-RSRP thresholds range could be configured, and which are corresponding to the multiple values for the number of multiple PRACH transmissions and a specific number of transmissions is used for PRACH transmission if the measured RSRP is in a configured RSRP threshold range.

Issue 3: How to differentiate the multiple PRACH transmissions with single PRACH transmission?

To facilitate preamble reception, the gNB should know whether the PRACH is repeated and the transmission number. Considering that in initial access process, there is no UL signalling could be used by UE to indicate the transmission number, the UE can indicate PRACH repetition via separate PRACH occasion or separate PRACH preambles.

To differentiate the multiple PRACH transmissions with single PRACH transmission, consider one or multiple of the following options.

- Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs.

- Option 2: Multiple PRACH are transmitted on separate ROs.

- Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs.

- Other options are not precluded.

- Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

We think both option 1 and option 2 could be considered, because this is similar with the mechanism utilized for the indication of Msg3 repetition transmission in Rel.17, which have been discussed carefully in Rel-17 and it is better to reuse the result.

Proposal 6: UE can indicate multiple RRACH transmissions via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions (option 1 and option 2).

Issue 4: How to reduce the PRACH transmission latency?

With PRACH repetition, the PRACH transmission latency might be significant. As shown in Figure 1 or Figure 2, when an event triggers a PRACH repetition, the UE needs to wait and then starts the PRACH transmission from a next starting RO or in the next RO bundle. The waiting time might be long, especially in the TDD system with limited number of UL slots in each DL/UL configuration pattern.

To reduce the PRACH repetition latency, one solution is to introduce denser starting ROs for PRACH repetition (based on the structure of opt.1 in proposal 1). Figure 5 shows an example, where the PRACH repetition number is 4. The UE selects SSB#0 and transmits PRACH repetition in the ROs that are associated with SSB#0. A denser starting RO is configured with the periodicity being indicated as 2. When an event triggers a PRACH procedure, the UE selects the nearest start RO to transmit the PRACH repetition. The waiting time is reduced with such denser starting ROs, compared with the waiting time in Figure 1.

Figure 5 Denser starting ROs for PRACH repetition

Alternatively, an RO bundle can be configured to include smaller number of ROs, which is less than the PRACH repetition number. The PRACH repetition could start from a RO bundle and transmitted in more than 1 RO bundles. One can observed that this actually also corresponds to denser starting ROs for PRACH repetition.

Figure 6 shows an example, the UE selects SSB#0 and transmits PRACH repetition in the ROs that are associated with SSB#0. The PRACH repetition number is 4. An RO bundle contains 2 ROs associated with a same SSB. The PRACH repetition can start from a nearest RO bundle and is transmitted in two RO bundles. The UE selects the nearest RO bundle to transmit the PRACH repetition. The waiting time is reduced with this structured, compared with the waiting time in Figure 2.

Figure 6 RO bundles with No. of ROs less than the PRACH repetition number

Proposal 7: The PRACH transmission latency can be reduced by introducing denser starting ROs for PRACH repetition.

Issue 5: How to determine the PDCCH monitoring window and RNTI for RAR after the multiple PRACH transmissions?

For initial access, after transmitting Msg1 in the determined ROs, the UE starts a RAR response window, within which the UE monitors downlink control information (DCI) for RAR. The DCI is identified by a RA-RNTI, which is determined by the time/frequency position of the RO that preamble is transmitted in. That is, for different ROs, the corresponding RA-RNTI is different. When multiple PRACH transmissions are performed, the timing of the UE starts the RAR response window and the RNTI for RAR should be determined.

For this issue, it has been agreed to consider the following options.

- Option 1: One RAR window per each PRACH transmission, the RAR window follows the legacy design.

  - FFS: RA-RNTI.

- Option 2: Only one RAR window for all of the multiple PRACH transmissions.

  - FFS: the start position of the RAR window.

For option 1, if one RAR window is determined per each PRACH transmission, there is an issue that the determined multiple RAR windows would be overlapped in time domain if the length of PDCCH monitoring window is not configured carefully. For example, in Figure 7, assuming there are 4 PRACH repetitions, then 4 RAR windows would be determined according to option 1, and they are overlapped in the time domain. This might increase blind detection complexity considering that there should be max 4 RA-RNTIs being monitored in the overlapped region. In order to decrease the monitoring complexity, how to determine the RA-RNTI in the overlapped region should be further decided.

Figure 7 One RAR window per each PRACH transmission

Considering the monitoring complexity and the specification effect, at least option 2 is preferred. For this opition, the UE could start the RAR response window after a time gap relative to the first transmission, so that the UE could early terminate the PRACH transmission if RAR is received before the transmission finished. Alternatively, the UE could also start the RAR response window after a time gap relative to the last transmission simply.

As for RNTI determination, considering the limited monitoring capability, one RO should be chosen, for example, the first RO could be used for RNTI determination.

Proposal 8: For multiple PRACH transmissions with same beam, for RAR monitoring, option 2 should be supported.

- The start position of the RAR window is referenced to the first PRACH transmission or last PRACH transmission.

- The RA-RNTI is determined based on the first RO.

For BFR, for PRACH transmission in slot n and according to antenna port quasi co-location parameters associated with periodic CSI-RS resource configuration or with SS/PBCH block associated with index qnew provided by higher layers, the UE monitors PDCCH in a search space set for detection of a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI starting from slot n+4+2μ∙kmac, where μ is the SCS configuration for the PRACH transmission and kmac is a number of slots provided by K-Mac or kmac=0 if K-Mac is not provided, within a window configured by BeamFailureRecoveryConfig. When there are multiple ROs used for PRACH transmissions, the timing of the UE starts the monitoring PDCCH for BFR should also be decided. Same method as RAR could be considered.

## 2.2 Multiple PRACH transmissions with different beams

It is not clear how well the network can rely on the beam correspondence implementations during initial access. Besides, the UE capability is transparent in random access procedure in initial access, and beam correspondence may not be guaranteed due to lack of reciprocity. Moreover, transmission with different beams increases robustness even for UE with beam correspondence. The gain of multiple PRACH transmission with different beams is provided in TR 38.830. So, supporting different beams could also be beneficial.

Proposal 9: PRACH transmission with different beam for 4 step RACH should be supported in Rel-18.

Conclusion

In summary, we propose the followings for Rel-18 NR PRACH coverage enhancements:

Proposal 1: Two options are proposed for RO determination for PRACH repetition

- Opt.1: Starting ROs are configured or implicitly determined, and PRACH repetition starts from a starting RO and repeats in the following ROs that are associated with a same SSB.

- Opt.2: RO bundles are configured and PRACH repetition is transmitted in the ROs of an RO bundle.

Proposal 2: The PRACH repetition should not be performed in FDMed ROs.

Proposal 3: Study RO determination for PRACH transmission when FDMed ROs are configured and are associated with same SSB.

Proposal 4: Different preambles cannot be utilized in different PRACH transmissions during the multiple PRACH transmissions in one RACH attempt.

Proposal 5: gNB can configure one or multiple values for the number of multiple PRACH transmissions (Option 2).

- Multiple SSB-RSRP thresholds range could be configured, and which are corresponding to the multiple values for the number of multiple PRACH transmissions and a specific number of transmissions is used for PRACH transmission if the measured RSRP is in a configured RSRP threshold range.

Proposal 6: UE can indicate multiple RRACH transmissions via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions (option 1 and option 2).

Proposal 7: The PRACH transmission latency can be reduced by introducing denser starting ROs for PRACH repetition.

Proposal 8: For multiple PRACH transmissions with same beam, for RAR monitoring, option 2 should be supported.

- The start position of the RAR window is referenced to the first PRACH transmission or last PRACH transmission.

- The RA-RNTI is determined based on the first RO.

Proposal 9: PRACH transmission with different beam for 4 step RACH should be supported in Rel-18.

References

- RP-213579, “New WI: Further NR coverage enhancements”.