3GPP TSG RAN WG1 #112  		 				                                                 				R1-2300889

Athens, Greece, 27 February – 3 March 2023

Agenda Item:					    9.11.2

Source:	Sony

Title:	Network verified UE location for NR NTN

Document for:	Discussion

# Introduction

At RAN#98e, the NR NTN work item description [3] was updated with the following objective related to NTN location verification.

Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied. Note 1: Enhancements assume reuse of the RAT dependent positioning framework Note 2: The specification of DL-TDOA enhancements will be subject to the study of the impact of realistic UE clock drift onto DL-TDOA performance Note 3: The target accuracy for position verification purposes is as documented in clause « recommendations » of the 3GPP TR 38.882 (i.e. 10 km granularity) Note 4 : Multiple satellite in view by the UE may be considered if time allows Note 5 : The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved Note 6 : The enhancements should take into account the mirror-image ambiguity Note 7 : Network verified UE location is an optional UE feature

In this contribution, we discuss our views on reliable UE location verification for NTNs for the high priority case of multi-RTT positioning.

# Multi-RTT with a single satellite

The updated WID states that NTN location verification operation should work with a single satellite in view and the work should assume reuse of the RAT dependent positioning framework.

In multi-RTT positioning for terrestrial networks, the UE is assumed to be in coverage of more than one gNB/eNB at the time when the UE carries out its positioning-related measurements. In contrast, simultaneous coverage from more than one NTN cell is likely to be relatively rare. Indeed, for non-continuous coverage NTNs, single satellite cell coverage per UE is likely to be the norm. Therefore, the multi-RTT positioning method needs adaptation in NTN so that it can function with single NTN cell coverage. The prevailing idea is to take measurements from one NGSO satellite at different times as the satellite navigates its orbit.

The conclusion of the study phase was documented in the chairman’s notes for RAN1#111 Toulouse and for multi-RTT the following observation was made:

For network verified UE location based on multi-RTT positioning method using Rx-Tx time difference measurements with single satellite, assuming the ambiguity of the mirror image position is resolved, if the UE reports needed to perform multi-RTT can be assumed to be trusted: - Five sources observed that multi-RTT positioning method can meet the NTN UE location verification accuracy requirement for LEO 600km: - Four sources observed that the positioning horizontal accuracy of less than 10km can be achieved with few seconds over-the-air latency (less or equal to 10s) with 95-percentile confidence level. - … - One source observed that the positioning horizontal accuracy of less than 10km can be achieved with 180 seconds latency for earth fixed beam with 90-percentile confidence level - … - Two sources observed that multi-RTT positioning method require latency larger than 60 seconds for UE located nearby the orbital plane of a satellite during a certain time duration.

While some companies observe that location can be verified within 10 seconds, some other companies observed that location verification would take significantly longer. Within the window of time to perform location verification, multiple RTT measurements may be required, especially given the challenging coverage conditions for the NTN uplink. It is hence apparent that the timing of the multi-RTT measurements should be configurable, as already supported in the existing RAT dependent positioning framework.

Observation 1: The existing RAT dependent positioning framework supports collection of multiple RTT measurements over the [up to] 10 second period required for location verification.

## Ambiguity of RX-TX time difference measurement in NTN

The multi-RTT technique is based on measurements of the time difference between PRS and SRS signals. Multiple measurements may be required for UE positioning estimation purpose (e.g., for multi-lateration computation of UE positioning estimation). Multiple measurements may also be required for NTN location verification due to the low SNR conditions that are characteristic of the NTN link budget. The PRS transmissions may be periodic. The round trip time in NTN may be large (25ms for LEO-600). In order to avoid ambiguity between which SRS and which PRS the RX-TX time difference measurement refers to, the SRS and PRS might have to be spaced far apart in order to avoid the ambiguous situation illustrated in Figure 1. Should the RX-TX time difference measurement refer to the time between PRS#1 and SRS#1, TRX_TX1, or should it refer to the RX-TX time difference between PRS#1 and SRS#2, TRX_TX2 ? In order to avoid this ambiguity, it is proposed that the association between PRS and SRS is explicitly configured for the purposes of RX-TX time difference measurements.

Figure 1 – Illustration of possible association of SRS and PRS for RX-TX time difference measurements

Proposal 1: The association between PRS and SRS should be configured for the purposes of RX-TX time difference measurements.

Note that for the scenario shown in Figure 1, it may be preferable for the network to configure an association between SRS#1 and PRS#1 (where the SRS is transmitted before the PRS is received), rather than between PRS#1 and SRS#2, as this minimises the time gap between SRS and PRS at the satellite. Minimisation of this time gap is desirable as, due to the high velocity of the LEO satellite, the distance that the satellite moves during the gNB’s TX-RX time difference measurement (i.e. during the time gap) increases the error in calculated location.

Observation 2: The time gap between PRS and SRS should be minimized for the gNB’s TX-RX time difference measurement.

## Location verification for moving UEs

The multi-RTT technique in terrestrial networks assumes that the UE takes measurements from the different gNBs/eNBs at the same time. In NTN, depending on the mobility of the UE, it may be necessary to consider the UE speed both in the intervals between which the measurements are taken and in the accuracy of the procedure that uses the resulting measurements for UE location verification. The degree to which UE mobility is considered may be influenced by the expected accuracy of the measurements. If UE speed is taken into account, then the UE may need to report its current GNSS location in each measurement report. In this manner, the network can verify the set of reported GNSS locations against the associated set of RTT measurements and check for consistency.

Proposal 2: RAN1 should consider whether or not the UE speed should be taken into account during the location verification process.

## Applicability of UE-specific TA

At RAN1#110bis-e and RAN1#111, there was a lot of discussion about whether or not measurements dependent on or parameters derived from the GNSS-derived UE location can be trusted. One parameter that was discussed a lot was the UE specific TA which is itself derived from the reported GNSS location of the UE.

Concerns with use of the UE specific TA include its accuracy and whether it can be trusted, given that it is derived from GNSS measurements include:

- Trustworthiness. The UE specific TA may not be trustworthy, given that it is derived from a GNSS measurement. It is not clear what added information the UE specific TA provides over and above the GNSS measurement itself.

- Accuracy. The granularity of UE specific TA reporting may make it unsuitable for location verification with a multi-RTT technique. Although the accuracy requirements for NR location verification are not stringent (approximately 10km), the geometry of the measurements and dilution of precision mean that accurate propagation time measurements are required even for the 10km level of accuracy required.

We feel that UE specific TA cannot by itself be used to validate the location of the UE. Nevertheless, it can either be assessed over multiple instances for consistency or combined with other trusted parameters such as RTT and tested for consistency. A test of consistency may suffice for UE location verification.

Proposal 3: RAN1 should use a combination of parameters both trusted and untrusted in UE location verification.

# Ensuring resilience against malicious UE

At RAN#96, the mini study item on network verification of UE location incorporated in the Rel18 work item on NTN enhancements [1] was concluded and TR38.882[2] approved. In the TR it is acknowledged that UE location verification in NTN is mainly to meet regulatory requirements for certain regulated services such as:

- Public Warning System (PWS)

- Lawful interception (LI)

- Emergency services (EMS)

- Charging and Tariff notifications

Nevertheless, it is also acknowledged as indicated in the following excerpt that a malicious UE may intentionally report a false GNSS location so as to connect to a different PLMN other than the one in its actual location.

As reported in TR 38.882[2],

Furthermore, a malicious UE might "fake" its selected PLMN in order to attempt connecting to a different core network. Upon such an attempt the AMF will disconnect the UE and inform the RAN node via an appropriate NGAP cause value, so the RAN can take appropriate action on subsequent attempts by the same UE.

A malicious UE intent on faking its selected PLMN would likely report a fake location that is commensurate to the actual PLMN it wants to use to access the network. Furthermore, such a fake UE knowing that the network would verify its location using a RAT-dependent positioning method (such as multi-RTT) and, being able to derive the locations of the satellite(s) from the broadcast ephemeris information, can also report fake positioning measurements that are commensurate with the fake position it reported. Network UE location verification methods therefore have to be deployed in a manner that such a malicious UE would be thwarted in this endeavour.

Observation 3: A malicious UE intent on reporting a fake location can also report fake location verification measurements commensurate with its fake location.

Proposal 4: RAN1 shall define network location verification methods that are immune to spoofing by malicious UEs intent on reporting a fake location.

# Conclusions

This document has considered location verification for NTN. The following observations and proposals are made:

Observation 1: The existing RAT dependent positioning framework supports collection of multiple RTT measurements over the [up to] 10 second period required for location verification.

Observation 2: The time gap between PRS and SRS should be minimized for the gNB’s TX-RX time difference measurement.

Observation 3: A malicious UE intent on reporting a fake location can also report fake location verification measurements commensurate with its fake location.

Proposal 1: The association between PRS and SRS should be configured for the purposes of RX-TX time difference measurements.

Proposal 2: RAN1 should consider whether or not the UE speed should be taken into account during the location verification process.

Proposal 3: RAN1 should use a combination of parameters both trusted and untrusted in UE location verification.

Proposal 4: RAN1 shall define network location verification methods that are immune to spoofing by malicious UEs intent on reporting a fake location.

# References

- RP-220953. “NR NTN (Non-Terrestrial Networks) enhancements”. RANP#95e. Feb 2022.

- TR38.882, “Study on requirements and use cases for network verified UE location for Non-Terrestrial-Networks (NTN) in NR (Release 18, TSG RAN

- RP-223534. “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements”. RANP#98e. December 2022.

- R1-2211627. “Network verified UE location for NR NTN”. Sony. RAN1#111, Toulouse, France. November 2022.