3GPP TSG RAN WG1 #112	R1-2300905

Athens, Greece, February 27th – March 3rd, 2023

Source:	Panasonic

Title: 	Discussion on coverage enhancement for NR-NTN

Agenda Item:		9.11.1

Document for:	Discussion and decision

Introduction

In the new WID on coverage enhancement for NR NTN [1] agreed in RAN#98e, the following are described.

The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port). Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO. The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps. The detailed objectives are for NTN: - To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4] - To specify if necessary, enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) [RAN1]

In previous RAN1 meetings, PUCCH enhancement and the necessity of enhancement related to DMRS bundling for PUSCH were discussed. The agreements are copied in Annex B. In this document, we discuss issues on PUCCH enhancement for Msg.4 HARQ-ACK and PUSCH DMRS bundling for NR-NTN coverage enahncement.

PUCCH enhancement for Msg.4 HARQ-ACK

Enhancement on frequency hopping and DMRS bundling

In RAN1#110b-e, support of up to 8 repetitions were agreed. In addition, enhancement on frequency hopping and DMRS bundling were listed as FFS.

In Rel.17 PUCCH repetition, intra-slot hopping and inter-slot frequency hopping can be UE-specifically configured by intraSlotFrequencyHopping and interslotFrequencyHopping in PUCCH-config. Because PUCCH for Msg4 HARQ-ACK is transmitted before UE specific RRC configuration, it is transmitted with intra-slot frequency hopping as the default behaviour. On the other hand, inter-slot frequency hopping and DMRS bundling allow a better channel estimation accuracy.

We evaluated the performance of intra-slot frequency hopping, inter-slot frequency hopping with/without DMRS bundling (i.e. joint channel estimation: JCE). Simulation assumptions are shown in Table 4 in Annex A. ACK miss detection curve and NACK-to-ACK error rate curve are shown in Figure 2 and Figure 3 in Annex A, respectively. SNR at 1% of miss detection rate and SNR at 0.1% of NACK-to-ACK error are listed in Table 1 (1T1R) and Table 2 (1T2R) below. In the tables, the higher SNR between miss-detection and NACK-to-ACK error is chosen as the required SNR.

As seen in Table 1 and Table 2, large gain of inter-slot FH and JCE are obtained in NACK-to-ACK (N-to-A) error performance thanks to the better channel estimation accuracy. On the other hand, only marginal gain is obtained from inter-slot FH and JCE for ACK miss-detection performance because channel estimation accuracy does not much contribute to the ACK detection performance. For 4 or less repetitions, because miss-detection rate is more critical (i.e. required SNR of miss-detection rate is higher than that of NACK-to-ACK error rate), benefit of inter-slot FH and JCE is small. For 8 repetitions, NACK-to-ACK error is more critical in case of intra-slot FH due to poor channel estimation under low SNR region. Then, inter-slot FH and JCE can reduce the required SNR by around 1 dB. Because miss-detection rate becomes more critical when inter-slot FH is used, additional gain of JCE is not obtained although gain of JCE is seen in NACK-to-ACK error rate. Therefore, inter-slot FH without joint channel estimation would be sufficient.

Observation 1: Inter-slot FH and DMRS bundling of PUCCH for Msg.4 HARQ-ACK has large gain in NACK-to-ACK error performance but only marginal gain in ACK miss-detection performance. ACK miss-detection is dominant in 4 or less repetitions case and NACK-to-ACK error is dominant in 8 repetitions case.

Proposal 1: Support inter-slot frequency hopping together with repetitions of PUCCH for Msg.4 HARQ-ACK.

Proposal 2: It is not required to support DMRS bundling of PUCCH for Msg.4 HARQ-ACK.

Table 1 required SNR and gain of inter-slot FH and joint channel estimation (1T1R)

Table 2 required SNR and gain of inter-slot FH and joint channel estimation (1T2R)

Procedure and signaling of PUCCH repetition for Msg.4 HARQ-ACK

Because PUCCH for Msg.4 HARQ-ACK is transmitted by UE before RRC reconfiguration takes effect, procedure and signaling of PUCCH repetition for Msg.4 HARQ-ACK needs to be enhanced. We discuss based on the following agreement in RAN1#110b-e and working assumption in RAN1#111-e.

Agreement For PUCCH repetition for Msg4 HARQ-ACK, - Discuss the following options of procedure to perform repetitions - Option 1: UE always performs repetition if configured in cell-specific manner - FFS: details of cell-specific configuration - FFS: behavior of UE being incapable of repetition - Option 2: UE requests repetition and is dynamically indicated to perform repetition - FFS: details of repetition request - FFS: details of configuration and dynamic repetition indication - Option 3: UE indicates repetition capability and is dynamically indicated to perform repetition - How UE indicates repetition capability before Msg4 Working Assumption For PUCCH repetition for Msg4 HARQ-ACK, - One or more repetition factors may be configured via SIB - If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor - FFS: whether UE requests repetition or indicates repetition capability - If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB - FFS: whether UE requests repetition or indicates repetition capability - FFS: whether repetition factor is indicated by UE - FFS: UE behavior when repetition factor is not configured via SIB - FFS: whether one or more UE capabilities are needed for the above is for further discussion

Msg4 PUCCH repetition procedure

Only cell-specific configuration may be a simple solution, but a drawback is the same configuration of the repetition is applied to all UEs in a cell. There would be a scenario where UE specific repetition factor is beneficial. According to the link budget calculation for LEO 1200km, the SNR difference is 4.4 dB between UEs with elevation angle 30 degree and UE with elevation agnle 90 degree. In addition, at least 3dB difference may be seen considering the difference of the beam gain (i.e. 3dB difference between beam edge and beam center). For GEO and MEO, much larger SNR difference in a cell is expected because of the larger distance between cell center and cell edge. Therefore, required number of repetitions is different for different UE locations. To apply the same number of PUCCH repetitions for all UEs is inefficient. The required number of repetitions for Msg 4 PUCCH could be estimated at gNB from PRACH and Msg 3 PUSCH received signal and/or TA report in Msg 3. PRACH resource may be also used depending on the discussion on how to use PRACH resource indication. Then, dynamic indication of the number of repetitions for Msg 4 PUCCH via DCI would be feasible.

In our view, only supporting dynamic indication is sufficient because dynamic indication can support an operation with cell specific repetition factor by indicating cell specific value in the DCI. But, on the other hand, there are no critical issue on the working assumption that supports both cell specific configuration and dynamic indication of repetition factor. Our understanding is that this working assumption allows a flexibility of network operation. We assume the following usage. For a scenario where one repetition factor per cell is sufficient (e.g. small cell coverage area), the network would indicate one repetition factor via SIB. In this case, network operation can be simple. For a scenario where multiple repetition factors per cell is beneficial (e.g. large cell coverage area with multiple SSB beams), the network would configure multiple repetition factors via SIB and dynamically indicate among the configured repetition factors. In this case, efficient UL resource usage would be possible by assigning repetition factor for each UE e.g. depending on propagation loss.

Proposal 3: Confirm the working assumption on the indication method of PUCCH repetition factor for Msg4.

Regarding FFS point on UE capability, just one capability is essential. If there is a possibility that UE only supporting cell specific repetition factor (i.e. SIB indication of only one repetition factor) and UE supporting dynamic indication exist in a same cell, gNB would need to configure only one repetition factor and therefore dynamic indication cannot be used as the capability is not known by gNB. Hence, one capability should cover both cell specific and dynamic indication of PUCCH repetition for msg4.

Proposal 4: The UE supporting cell specific indication of PUCCH repetition for msg4 should support dynamic indication of PUCCH repetition for msg4.

UE capability should be further discussed including the relation with Msg.3 PUSCH repetition and PRACH repetition.

Regarding FFS point on UE bahavior when repetition factor is not configured via SIB, we see the folowing two behaviors. One is to fall back to Rel.17 behavior, i.e. no repetition for msg4 PUCCH. The other is to use default set of repetition factors for dynamic indication. In the former case, whether repetition factor is indicated via SIB implicitly indicates whether msg4 PUCCH repetition is supported by the network. This operation is backward compatible with earlier NTN UEs. In the latter case, additional indication on whether msg4 PUCCH repetition is supported by the network is necessary. The latter case would be less attractive.

Proposal 5: When repetition factor is not conigured via SIB, Rel.17 behavior should be used, i.e. no repetition for msg4 PUCCH.

Indication for UE capability/request of Msg4 PUCCH repetition

Whether UE requests repetition or indicates repetition capability has been discussed in the previous RAN1 meetings, but it has not been concluded yet. We think the request of the repetition from its capable UE based on DL channel measurement would make more sense than just indicate repetition capability because UE can estimate more precisely than gNB whether repetitions would be necessary based on DL channel measurement.

Proposal 6: support UE request of repetition for msg4 PUCCH from UE supporting Msg4 PUCCH repetition.

On the indication method, two schemes are considered in RAN1 meetings,, indication via PRACH resource selection and indication via Msg.3 PUSCH.

- Indication via PRACH resource selection

Similar to Msg.3 PUSCH repetition, PRACH resource selection can be used for the indication of the repetition request. If the UE supports and requests the PUCCH repetition for Msg.4 based on UE measurement, e.g. RSRP or location, a specific group of PRACH resources, e.g. RO, preamble sequence, that is configured by SIB are used. Then, UE incapable with PUCCH repetition (or capable but not requesting PUCCH repetition) transmits PUCCH without repetition, and UE capable with PUCCH repetition (or capable and requesting PUCCH repetition) transmits PUCCH with repetition as configured by SIB.

Because PRACH resource selection could be used for Msg.3 PUSCH repetition, PRACH repetition and Msg.4 PUCCH repetition, PRACH resource partitioning (e.g. RSRP threshould setting) should take into account the repetition of these three channels. There was a concern on too many PRACH resource partitioning. Therefore, the number of resource partitions (i.e. group of resources) can be reduced by restricting the combinations, e.g. UE requesting Msg.4 PUCCH repetition should always request Msg.3 PUSCH repetition and PRACH repetition. Based on the past SINR evaluation including coverage enhancement, our understanding is when Msg.4 PUCCH repetition is required, Msg.3 PUSCH repetition would be also required. The relation to the number of PRACH repetition depends on what PRACH format is chosen. Therefore, for example, if PRACH repetition is used, it can imply Msg3 PUSCH repetition and Msg4 PUCCH repetition are also requested. If PRACH repetition is not used, it can imply no Msg4 PUCCH repetition is requested. In this case, additional PRACH resource partinioning specific to Msg4 PUCCH repetition is not needed. The similar design can be also applicable when Msg.3 PUSCH and Msg.4 PUCCH repetition but without PRACH repetition are used.

- Indication via Msg.3 PUSCH

Another method is indication via Msg3 PUSCH. Two methods were proposed in RAN1#111 meeting. One is to use different scrambling sequences on CRC or DMRS of the PUSCH in case of UE requesting Msg.4 PUCCH repetition. The other is to use reserved field of MAC CE. The former case would have a large impact on gNB complexity. The latter case gNB complexity impact would be small, but it should be determined in RAN2 as it is their expertise. As there can be the network operation not to configure above PRACH resource selection, if the reserved codepoints is sufficient, it would be useful to have the indication via Msg.3 PUSCH.

Proposal 7: PRACH resource selection should be used for request of Msg4 PUCCH repetition. The restriction of combinations among PRACH repetition, Msg3 PUSCH repetition and Msg4 PUCCH repetition should be applied in order to avoid too many PRACH resource partitioning.

Proposal 8: Indication via Msg.3 should be concluded in RAN2.

Dynamic indication of Msg4 PUCCH repetition

For dynamic indication of PUCCH repetition for Msg.4, DCI 1_0 used for scheduling Msg.4 PDSCH can be used. In order to keep the size of DCI 1_0 same as Rel.17, existing field should be reused for the indication of PUCCH repetition. The following would be the cadidate fields.

- PUCCH resource indicator (3 bits)

This field indicates PUCCH resource for HARQ-ACK. If 1 or 2 bits are used for indication of Msg4 PUCCH repetition, flexibility of PUCCH resource selection is restricted. But, PUCCH collision between UEs would not be a big concern for NTN because all slots are available for FDD which is assumed in NTN. Therefore, restriction of PUCCH resource indicator field would not be so critical.

- PDSCH-to-HARQ timing indicator (3 bits)

This field indicates PUCCH timing for HARQ-ACK. If 1 or 2 bits are used for indication of Msg4 PUCCH repetition, flexibility of PUCCH timing is restricted. But, because HARQ-ACK can be assigned in any slot for FDD, flexibility of HARQ-ACK timing is not so important. Therefore, use of this field for Msg4 PUCCH repetition is reasonable.

- TPC for PUCCH (2 bits)

This field indicates TPC command for PUCCH. For NTN, UE would be likely to transmit with full power in most scenarios. Therefore, this field may not be so useful. But, in some scenarios, TPC field may be beneficial. This field would be less attractive compared to above PUCCH resource indiator and PDSCH-to-HARQ timing indicator.

- MCS (5 bits)

This field indicates MCS of PDSCH. Because higher MCS is not likely to be used for NTN, this field can be partly used for indication of Msg4 PUCCH repetition. But, in some scenarios, higher MCS may be used (e.g. with high satellite power). This field would be less attractive compared to above PUCCH resource indiator and PDSCH-to-HARQ timing indicator.

- HARQ process number (4 bits)

By restricting the available HARQ process number for PDSCH transmission, part of this field can be used for the indication of PUCCH repetition. Generally, large number of HARQ processes would be used in NTN due to the long RTT as discussed in study item phase in Rel.16. If only msg4 PDSCH is transmitted, it is not harmful to restrict the number of HARQ processes. On the other hand, if PUCCH repetition is supported for PUCCH transmission before dedicated RRC configuration in addition to msg4 HARQ-ACK, use of this field would not be preferable.

- Downlink assignment index (2 bits)

For DCI scrambled with TC-RNTI, this field is reserved. Therefore, if PUCCH repetition is supported for PUCCH transmission before dedicated RRC configuration in addition to msg4 HARQ-ACK, this field can not be used for the indication of the repetition factor. Furthermore, in general, to use reserved fields should be avoided as much as possible. Use of this field would not be preferable.

As discussed above, preferred candidates are PUCCH resource indicator and PDSCH-to-HARQ timing indicator.

Proposal 9: PUCCH resource indicator or DSCH-to-HARQ timing indicator in DCI 1_0 should be re-purposed for indication of PUCCH repetition factor.

DMRS bundling for PUSCH with VoIP

The following conclusions were made in RAN1#111.

Conclusion For the study of NTN-specific PUSCH DMRS bundling, RAN1’s understanding is that Phase variation due to constant frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1 does not have impact on the phase continuity requirement for two adjacent slots specified as Table 6.4.2.5-1 in 38.101-1, according to annex F.9 and F.4 of 38.101-1. Conclusion RAN1 concluded that PUSCH DMRS bundling with sufficient TDW size should be applicable in NTN to meet the performance requirement for VoIP - FFS: How to determine TDW size, including UE capability. - Note: The above does not mean the performance requirements will be satisfied with DMRS bundling

In NTN, the propagation delay and the Doppler shift are dependent on satellite location and velocity. Timing advance (TA) and transmission frequency are autonomously adjusted by UE using satellite ephemeris and common TA parameters broadcasted via SIB19 and GNSS location information obtained by UE. Time and frequency accuracy requirements and maximum allowable phase difference for DMRS bundling need to be satisfied. Below, we give a consideration on whether these requirements can be satisfifed.

Time and frequency accuracy requirements

The accuracy requirement of the timing specified in TS38.133 is shown in Table 3. Note that Tc is 0.509ns according to TS38.211.

Table 3 Timing Error Limit Te_NTN (Table 7.1C.2-1 in TS38.133)

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 29*64*Tc |
|  |  | 30 | 24*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 24*64*Tc |
|  |  | 30 | 22*64*Tc |
|  |  | 60 | N/A |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

The frequency accuracy requirement is defined in TS38.101-5 as follows

- 6.4.1 Frequency error The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift. For the test case, the location of the UE is explicitly provided to the UE from the test equipment.] Requirement will be verified for at least two cases of which one has zero Doppler conditions.

Figure 1 shows time and frequency change rate for LEO 600 km. It is assumed that the satellite orbit is along with the equator and UE is located on latitude 0 degree. Curves with various gateway latitude (0-15 degrees) are shown in the figures. The maximum timing change rate is around 42 us/s and the maximum Doppler frequency change rate is 0.57ppm/s (i.e. 0.57 x 10-6 x 2 GHz / s). It means that propagation delay is changed at most 42 ns per slot for SCS 15kHz and the Doppler frequency is changed at most 0.57 x 10-3 ppm per slot.

According to our simulation ersults for PUSCH VoIP shown in Annex A, DMRS bundling with 10 slots (10 ms for 15kHz SCS) for DMRS bundling has sufficient gain. Timing change over 10 slots is 420 ns and Doppler frequency change over 10 slots is 0.0057 ppm. These are lower than the timing error requirement, i.e. 945 ns (=29 x 64 x Tc) for 15kHz SCS and 781 ns (=24 x 64 x Tc) for 30kHz SCS, and frequency accuracy requirement of 0.1 ppm, respectively. Therefore, time and frequency accuracy requirement can be satisfied without adjusting them during the TDW.  In addition, how often the timing components, NTA,adjcommon+NTA,adjUE, needs to be updated to satisfy the accuracy requirement depends on the latitude of the satellite.

Observation 1: DMRS bundling with 10 slots or less is sufficient for PUSCH VoIP.

Observation 2: Time and frequency accuracy requirement can be satisfied without adjusting them during the TDW of 10 slots.

Observation 3: How often the timing components, NTA,adjcommon+NTA,adjUE, needs to be updated to satisfy the accuracy requirement depends on the latitude of the satellite.

Figure 1 time (left figure) and frequency (right figure) change rate for LEO 600km

Maximum allowable phase difference for DMRS bundling

The requirement of the maximum allowable phase difference for DMRS bundling is specified in Table 6.4.2.5-1 of TS 38.101-1 (copied below).

According to the conclusion in RAN1#111, phase variation due to constant frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1 does not have impact on the phase continuity requirement. Therefore, the Doppler frequency fluctuation due to satellite movement (i.e. up to 0.57 ppm/s) only is taken into account below.

Time and frequency drift of 42 us/s and 0.57 ppm/s causes the following phase rotation over slot (1ms), respectively.

Phase rotation due to timing drift (2RB case): 2π×1 ms×42μs/s×360kHz=0.095 rad=5.4 degree

Phase rotation due to frequency drift: 2π×0.57 ppm/s ×1 ms=0.00716 rad=0.4 degree

In total, phase rotation of 5.8 degree per slot is observed as NTN specific factor. The dominant factor is the phase rotation due to timing drift. During 10 slots TDW of DMRS bundling, roughly 60 degrees phase rotation is observed in addition to the phase rotation due to the UE transmitter impairment as in TN (terrestrial network). Considering other (non-NTN specific) factors of phase error, updating especially time pre-compensation during TDW might be necessary to satisfy the requirement of maximum allowable phase difference.

As discussed above, how often the timing components, NTA,adjcommon+NTA,adjUE, needs to be updated to satisfy the accuracy requirement depends on the latitude of the satellite. Based on the UE implementation, the timing is updated according to the satellite movement. For phase drift caused by satellite movement, there are various ways of the realization, e.g. Time/frequency shift constantly changed on every symbol without abrupt phase changes, segmented pre-compensation without carrier phase jump [3][4].

For DMRS bundling, UE report its TDW capability, i.e. the maximum duration during which UE is able to maintain power consistency and phase continuity for supporting DM-RS bundling at gNB, as UE capability, e.g. considering the UE transmitter impairment such as PA and oscillator. Nominal TDW and actual TDW are defined in Rel.17. Nominal TDW is semi-statically configured by gNB e.g. based on reported UE capability. A nominal TDW consists of one or multiple actual TDWs. Events, which cause power consistency and phase continuity not to be maintained within the nominal TDW, are defined, e.g. frequency hopping, timing adjustment in response to TA command.

Based on the above discussions, the reporting TDW capability length depends on whether UE transmitter inpairments, timing drift due to the satellite movement and phase drift due to the satellite movement are taken into account in a TDW. UE transmitter impairments always need to be taken into account but the other two are taken into account or not in a TDW are the specification design choices. The possibility of reporting TDW capability is summarized in Table 4.

Table 4 The reporting TDW capability options for DMRS bundling in NTN

|  | UE transmitter impairments | Timing drift due to satellite movement | Phase drift due to satellite movement |
| --- | --- | --- | --- |
| Option 1 | Considered in a TDW | Not considered | Not considered |
| Option 2 | Considered in a TDW | Considered in a TDW | Not considered |
| Option 3 | Considered in a TDW | Considered in a TDW | Considered in a TDW |

Option 1: TDW capability considering UE transmitter impairments only.

In this option, TDW capability does not take into account the satellite location. UE would update the timing and phase pre-compensation at the boundary between adjustment two TDWs only. Instead of using full length of TDW, the length of joint estimation at gNB is adjusted depending on the satellite elevation angle from the UE. This would require the knowledge of the UE location at gNB, but the length of joint channel estimation may be estimated to some extent based on rough location. Such adjustment of the length of joint estimation is similar to the handling of high doppler in TN operation.

Option 2: TDW capability considering UE transmitter impairments and timing drift due to satellite movement only.

In this option, one direction is the TDW capability is based on the assumed satellite location. First design is UE would report the shortest TDW over the satellite orbit which would correspond to the lowest elevation angle where timing change rate is the maximum. In this case, gNB can use only a small number of slots for joint channel estimation because the reported TDW from UE itself is short and the benefit from the DMRS bundling can not be fully obtained. In addition, because phase rotation caused by the timing drift due to satellite movement is remained in the gNB received signal, gNB would need to post-compensate the phase rotation depending on the TDW. The other design would be UE report the longest TDW which would correspond to the highest elevation angle where timing change rate is the minimum. In this case, the gNB operation would be similar to Option 1.

The second direction is the TDW capability report based on the real satellite location. In this case, gNB’s estimation of the length of joint channel estimation based on satellite and UE location would not be required. On the other hand, an extension to more dynamic TDW report is necessary.

Option 3: TDW capability considering UE transmitter impairments, timing drift due to satellite movement and phase drift due to satellite movement.

In this option, gNB does not need to post-compensate the phase drift due to satellite movement because it is taken into account as TDW capability report. Similar to Option 2, two directions with assumed satellite location and with real satellite location are possible for TDW capability report. For the assumed satellite location, two designs of TDW report are to report the shortest TDW based on the satellite location with the lowerst elevatition angle and to report the longest TDW based on the satellite location with the highest elevatition angle. The diretion of TDW capability report based on the real satellite location would require more dynamic TDW report depending on satellite movement.

As discussed above, appropriate gNB behavior for joint channel estimation would be different depending on whether time and phase drift due to satellite movement is taken into account for TDW report. Therefore, condition of the TDW reported as UE capability needs to be specified.

Furthermore, if Optio 2 or 3 with TDW report based on the real satellite location are taken, because appropriate TDW depends on the satellite elevation angle, more dynamic TDW report and/or indication would be necessary, which would ease gNB implemtation to estimate the window legnth for the joint channel estimation and/or post-compensate the phase rotation due to satellite movement.

Proposal 10: Whether time and phase drift due to satellite movement is taken into account or not in the TDW reported should be specified.

Proposal 11: Consider more dynamic TDW report and/or indication.

Conclusion

We discussed issues on coverage enhancement for NR-NTN. The following proposals on PUCCH for Msg.4 HARQ-ACK and on DMRS bundling for PUSCH are made.

Proposal 1: Support inter-slot frequency hopping together with repetitions of PUCCH for Msg.4 HARQ-ACK.

Proposal 2: It is not required to support DMRS bundling of PUCCH for Msg.4 HARQ-ACK.

Proposal 3: Confirm the working assumption on the indication method of PUCCH repetition factor for Msg4.

Proposal 4: The UE supporting cell specific indication of PUCCH repetition for msg4 should support dynamic indication of PUCCH repetition for msg4.

Proposal 5: When repetition factor is not conigured via SIB, Rel.17 behavior should be used, i.e. no repetition for msg4 PUCCH.

Proposal 6: support UE request of repetition for msg4 PUCCH from UE supporting Msg4 PUCCH repetition.

Proposal 7: PRACH resource selection should be used for request of Msg4 PUCCH repetition. The restriction of combinations among PRACH repetition, Msg3 PUSCH repetition and Msg4 PUCCH repetition should be applied in order to avoid too many PRACH resource partitioning.

Proposal 8: Indication via Msg.3 should be concluded in RAN2.

Proposal 9: PUCCH resource indicator or DSCH-to-HARQ timing indicator in DCI 1_0 should be re-purposed for indication of PUCCH repetition factor.

Proposal 10: Whether time and phase drift due to satellite movement is taken into account or not in the TDW reported should be specified.

Proposal 11: Consider more dynamic TDW report and/or indication.

- Reference

- RP-223534, “WID: NR NTN (Non-Terrestrial Networks) enhancements”, Thales

- TR38.821 V16.0.0

- R1-2209071, “On coverage enhancement for NR NTN”, Intel Corporation

- R1-2210004, “Coverage enhancements for NR NTN”, Qualcomm

- Annex A: LLS assumptions and results

Performance of PUCCH repetition and DMRS bundling for Msg.4

Table 5 evaluation assumptions for PUCCH format 1

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 1 bit UCI |
| Frequency hopping | w/ frequency hopping |
| BLER | For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. |
| Antenna configuration | 1T1R, 1T2R |
| Repetitions | 1, 2, 4, 8 |
| PUCCH duration | 14 OS |
| Number of PRBs | 1 PRB |

(a) ACK miss detection probability                            (b) NACK to ACK error probability

Figure 2 Simulation results for PUCCH format 1 repetition (1T1R)

(a) ACK miss detection probability                            (b) NACK to ACK error probability

Figure 3 evaluation results for PUCCH format 1 repetition (1T2R)

Perfomance of PUSCH DMRS bundling

Table 6 evaluation assumptions for PUSCH VoIP

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| UE speed | 3 km/h |
| Subcarrier spacing | 15 kHz |
| Waveform | DFT-s-OFDM |
| PUSCH duration | 14 OS |
| Frequency hopping | w/ inter-slot frequency hopping (hopping BW 20MHz) |
| Antenna configuration | 1T1R, 1T2R |
| DMRS | Type I, 2 DMRS symbol, no multiplexing with data. DMRS bundling window: 1, 2, 5, 10 |
| HARQ | HARQ is not modelled |
| MCS, PRBs, repetitions | MCS9, 1PRB, 20 repetitions |

Figure 4 shows BLER performance for PUSCH VoIP using DMRS bundling with various TDW length (denoted by n) for 1T1R and 1T2R. The agreed scenario and parameters are assumed, i.e. LEO1200, parameter set 1, NTN TDL-C (NLOS) with evevation angle 30 degrees. 20 repetitions with inter-slot frequency hopping period of 10 slots is assumed. In this case, DMRS bundling up to 10 slots are possible. TDW length (denoted by n) 1, 2, 5 and 10 slots are evaluated. Other simulation assumptions are aligned with the agreement in RAN1#109e as listed in Table 5.

According to the simulation results, for both 1T1R and 1T2R, difference between n=5 and 10 is already merginal. Therefore, TDW length of larger than 10 slots would not be needed.

Figure 4 LLS results for joint channel estimation (DMRS bundling) of PUSCH VoIP

- Annex B: Agreements

< PUCCH enhancement for Msg4 HARQ-ACK >

RAN1#110b-e

Agreement

For PUCCH for Msg4 HARQ-ACK,

- Support PUCCH repetition

  - Further discuss the specification impact for at least the following

    - Procedure and signaling (e.g., cell-specific configuration, request to gNB and dynamic indication from gNB, UE capability indication before Msg4, etc.)

    - Repetition factor

    - Repetition slot counting for FDD

  - Further study whether to enhance or support the following

    - Frequency hopping

    - DMRS bundling

Agreement

For PUCCH repetition for Msg4 HARQ-ACK,

- Discuss the following options of procedure to perform repetitions

  - Option 1: UE always performs repetition if configured in cell-specific manner

    - FFS: details of cell-specific configuration

    - FFS: behavior of UE being incapable of repetition

  - Option 2: UE requests repetition and is dynamically indicated to perform repetition

    - FFS: details of repetition request

    - FFS: details of configuration and dynamic repetition indication

  - Option 3: UE indicates repetition capability and is dynamically indicated to perform repetition

  - How UE indicates repetition capability before Msg4

Agreement

For PUCCH transmission for Msg4 HARQ-ACK, supported number of transmissions are 1, 2, 4, 8.

- Note: single PUCCH transmission is performed as in the existing specification, and/or (if supported for single PUCCH transmission) according to configuration/indication e.g., in signaling with respect to number of transmissions.

- FFS: whether larger number of transmissions is supported

- FFS: whether/how single PUCCH transmission can be configured and/or indicated

Conclusion

For PUCCH repetition for Msg4 HARQ-ACK,

- The existing mechanism on repetition slot counting (as in section 9.2.6 of TS 38.213) can be applied.

  - FFS: whether specification update to apply the existing mechanism to PUCCH repetition for Msg4 HARQ-ACK is needed.

RAN1#111

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK,

- One or more repetition factors may be configured via SIB

  - If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor

    - FFS: whether UE requests repetition or indicates repetition capability

  - If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB

    - FFS: whether UE requests repetition or indicates repetition capability

    - FFS: whether repetition factor is indicated by UE

  - FFS: UE behavior when repetition factor is not configured via SIB

  - FFS: whether one or more UE capabilities are needed for the above is for further discussion

< NTN-specific PUSCH DMRS bundling >

RAN1#110-b

Agreement

For NTN-specific PUSCH DMRS bundling,

- Discuss further the need of enhancement in consideration of at least the following:

  - Phase difference due to timing drift and/or doppler shift.

    - e.g., whether/how long a UE can meet phase continuity requirement specified as Table 6.4.2.5-1 in 38.101-1 in consideration of frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-5 and timing error specified in Table 7.1C.2-1 of 38.133, whether RAN1 should introduce enhancement to meet the requirement and/or recommend RAN4 to update the requirement or UE should pre-compensate phase difference by UE implementation, etc.

  - An event which causes power consistency and phase continuity not to be maintained.

    - e.g., whether the new event is necessary to determine actual TDW(s) from each nominal TDW or the existing specification can work without any specification change or whether such event may not occur depending on implementations, etc.

  - Note: baseline performance for legacy UEs can include antenna switching

RAN1#111

Conclusion

For the study of NTN-specific PUSCH DMRS bundling, RAN1’s understanding is that Phase variation due to constant frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1 does not have impact on the phase continuity requirement for two adjacent slots specified as Table 6.4.2.5-1 in 38.101-1, according to annex F.9 and F.4 of 38.101-1.

Conclusion

RAN1 concluded that PUSCH DMRS bundling with sufficient TDW size should be applicable in NTN to meet the performance requirement for VoIP

- FFS: How to determine TDW size, including UE capability.

- Note: The above does not mean the performance requirements will be satisfied with DMRS bundling