3GPP TSG RAN WG1 #112                                                                 R1-2300944

Athens, Greece, February 27th – March 3rd, 2023

Source:	Intel Corporation

Title:	Other Aspects on AI-ML Beam Management

Agenda item:	9.2.3.2

Document for:	Discussion and Decision

# Introduction

In this paper, we present initial views on the study of AI/ML applications to physical layer for beam management based on the objectives in the Rel-18 NR study item on AI/ML [1]:

Use cases to focus on: - Initial set of use cases includes: - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

The different sub-use-cases that can be considered for AI/ML aided beam management framework are discussed along with initial assessment of possible specification impact to enable these sub-use-cases.

# AI/ML for Beam Management - Use Cases

The beam management use case for AI/ML study can be broadly divided into two sub-cases i.e., spatial domain beam management and temporal domain beam management. Regardless for the specific use case, a common AI/ML model training and deployment framework can be considered.

Figure 1: AI/ML Model Training and Deployment

An example is shown in Figure 1, where a supervised machine learning model with offline training/validation is considered. The model is offline in the sense that dataset for training and testing are pre-generated, and the model is pre-trained in a non-real time manner. Once the model is trained, it can be deployed in a real-time environment to aid in specific beam management tasks based on the following sub-use cases:

- Spatial Domain Beam Management (BM-Case1): Predict the optimal beam at UE or BS without exhaustive search with an aim to reduce measurement and reporting latency

  - Beam Prediction at the UE: For this use-case, consider a 2D planar array at UE with multiple analog Rx beams and fixed Tx beam at the gNB. The ML model is provided with channel or RSRP values for a subset of the beams and the best beam index or top K beam indices can be predicted from a subset of measurements thereby reducing the latency of beam acquisition or tracking

  - Beam prediction at the gNB: Similar to the previous use case but for Tx/Rx beam selection at the gNB for a fixed directional or omni beam at the UE. Only a subset of gNB are measured and the ML model predicts the beam index or top K beam indices from these subset of measurement leading to latency reduction for optimal beam selection

  - Joint UE-gNB beam pair link prediction: This use case would be similar to a joint P2+P3 procedure where the best beam at the gNB and the best beam at the UE are simultaneously identified by the ML model from a subset of measurements made on select beam pairs.

- Temporal Domain Beam Management (BM-Case2): This use case predicts the best beam at gNB or UE for the next time instant given beams from a window of preceding time instants. Time series data can be collected from UEs moving across an environment and LSTM based models can be used to predict the best beam.

In the following sections, we address some open issues from previous meetings.

## Predicted Beams for Case 1 and 2

Agreement For the sub use case BM-Case1 and BM-Case2, further study the following alternatives for the predicted beams: - Alt.1: DL Tx beam prediction - Alt.2: DL Rx beam prediction - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam) Note1: DL Rx beam prediction may or may not have spec impact

For cases when UE and BS have a large number of beams, performing P2 and P3 procedures sequentially results in large latency. Implementing a joint P2/P3 procedure has been discussed in RAN1 before and can be shown to greatly reduce latency. In our understanding a joint P2/P3 based training methodology can be studied under Alt-3 of the above agreement based on L1 measurement and reporting framework. If ML based solutions are considered, then it may be possible to further reduce the latency by measuring only on certain combination of UE and gNB beams to predict the best beam pair link. This can potentially save measurement overhead as well provide large latency gains. Performance results in our paper [2] show that fraction of measurements still provides good prediction accuracy for this joint P2/P3 approach.

- Beam Pair prediction (Alt-3) should be supported, at least for BM-Case 1 since it can provide large latency and measurement gains for joint P2/P3 procedure

## Data Collection

Agreement For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact: - Signaling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), Reference signals - Content/type of the collected data - Other aspect(s) is not precluded Agreement In order to facilitate the AI/ML model inference, study the following aspects as a starting point: - Enhanced or new configurations/UE reporting/UE measurement, e.g., Enhanced or new beam measurement and/or beam reporting - Enhanced or new signaling for measurement configuration/triggering - Signaling of assistance information (if applicable) - Other aspect(s) is not precluded

For the agreements above, an important factor is where the model resides and how training and inference are linked i.e., if the model inference is performed where the model is trained. If the training and inference has access to the same type of data, then the measurement/reporting and collection of the data can be standardized. It also needs to be clarified where the model is expected to be trained using data from the 3GPP air interface design. If yes, then it would be possible to further design signalling, configuration and measurement mechanisms to aid in training and model inferencing.

- The impact of 3GPP specification related procedures for data collection for training as well as inference depends on where the model resides and if training and inferencing is being performed at the same node.

Furthermore, even for models that may not directly impact specification e.g., model at the UE for UE Rx beam prediction, there may need to be some measurement triggering enhancements wherein a UE can request L1-RSRP report from the gNB in order to refine or update its model. The format of such reporting and the frequency may depend as well on how the model was trained. Therefore, it is important to first clarify if companies expect training data to be collected using 3GPP air interface which would then dictate the format of reporting as well as some basic methodology for how the dataset for model training is constructed. While models themselves may be non-standardized, the dataset being constructed based on standardized measurement and reporting schemes may have some advantages to harmonize AI/ML integration into the 3GPP air interface.

- Training dataset construction using 3GPP specified measurement and reporting framework may be advantageous for harmonizing deployment of proprietary AI/ML models.

To this end, the following agreement was made in RAN1#111:

Agreement Regarding the data collection for AI/ML model training at UE side, study the potential specification impact considering the following additional aspects. - Whether and how to initiate data collection - Configurations, e.g., configuration related to set A and/or Set B, information on association/mapping of Set A and Set B - Assistance information from Network to UE (If supported) - Other aspect(s) is not precluded

For data collection purposes, especially for model training at UE side, it would be beneficial if the UE could trigger RS transmissions from the gNB enabling the UE to measure one different gNB beams in Set B for BM-Case1 and BM-Case2. This method of UE initiated RS transmissions and measurement may also be useful for model performance monitoring for UE side model where it may be up to the UE to fine-tune or re-train the model based on measurement results.

- For data collection with AI/ML model training at UE side, support UE triggered reference signal transmission from the gNB to enable the UE to perform L1 measurements at least on Set B for both BM-Case 1 and 2.

On the issue of how to initiate data collection, it could be part of the same UE trigger where the UE can potentially transmit a data collection request along with the request for transmission of reference signals for measurement.

## AI/ML Model Output

Agreement Regarding the sub use case BM-Case1 and BM-Case2, study the following alternatives for AI/ML output: - Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - E.g., N predicted beams can be the top-N predicted beams - Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information - FFS: other information (e.g., probability for the beam to be the best beam, the associated confidence, beam application time/dwelling time, Predicted Beam failure) - E.g., N predicted beams can be the top-N predicted beams - Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - E.g., N predicted beams can be the top-N predicted beams - FFS: details of Beam angle(s) - FFS: how to select the N DL Tx and/or Rx beams (e.g., L1-RSRP higher than a threshold, a sum probability of being the best beams higher than a threshold, RSRP corresponding to the expected Tx and/or Rx beam direction(s)) - Note1: It is up to companies to provide other alternative(s) - Note2: Beam ID is only used for discussion purpose - Note3: All the outputs are “nominal” and only for discussion purpose - Note4: Values of N is up to each company. - Note5: All of the outputs in the above alternatives may vary based on whether the AI/ML model inference is at UE side or gNB side. - Note 6: The Top-N beam IDs might have been derived via post-processing of the ML-model output

In RAN1#110, a major point of contention in the above agreement was based on how input to the AI/ML model is defined. In our view, the AI/ML model input should be considered prior to the pre-processing of data especially if 3GPP air interface is used for such data collection. Companies should disclose their pre-processing and post-processing methods in conjunction with details on how the data is collected for pre-processing and how it is used after post-processing. Additionally, as part of specification impact of AI/ML, it may be possible to standardize how beam IDs are mapped in the spatial domain. Since neither “beams” nor “beam angles” are defined in specification, it is difficult to understand how Alt-3 may work unless additional pre-processing is used. Therefore, Alt-1 should be considered as the baseline use-case with Alt-2 providing additional side information if necessary.

- For BM-Case1 and 2, Alt-1 (Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams) should be considered as the baseline use case, with potential specification impact on how beam IDs are mapped in the spatial domain.

## AI/ML Model Training and Inference

Agreement For BM-Case1 with a UE-side AI/ML model, study the potential specification impact of L1 signaling to report the following information of AI/ML model inference to NW - The beam(s) that is based on the output of AI/ML model inference - FFS: Predicted L1-RSRP corresponding to the beam(s) - FFS: other information Agreement For BM-Case2 with a UE-side AI/ML model, study the potential specification impact of L1 signaling to report the following information of AI/ML model inference to NW - The beam(s) of N future time instance(s) that is based on the output of AI/ML model inference - FFS: value of N - FFS: Predicted L1-RSRP corresponding to the beam(s) - Information about the timestamp corresponding the reported beam(s) - FFS: explicit or implicit - FFS: other information

For BM-Case-1 and BM-Case-2, the predicted RSRP from the AI/ML model may be a normalized version or a linear or non-linear function of the RSRP and may not correspond directly to the RSRP value. It is also dependent on input normalization and corresponding model output and architecture. Therefore, mandating reporting of such values should be avoided. Additionally, the definition of predicted RSRP should be clarified. If the predicted RSRP is the actual measured RSRP on predicted beam, then it may be ok to report it back to the network.

- If the predicted RSRP is directly obtained from the output of the AI/ML model, such metrics should not be reported back to the network. If the predicted RSRP is defined as the actual measured RSRP on the best beam predicted by the model, such metric can be reported back to the network assuming that the overhead due to the additional measurement, if needed, is accounted for in the KPI calculation.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection: - Atl1. UE-side Model monitoring - UE monitors the performance metric(s) - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Atl2. NW-side Model monitoring - NW monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation - Alt3. Hybrid model monitoring - UE monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

For AI/ML model monitoring, the above agreement was reached in RAN1#110bis-e for the case when the model resides at the UE side. Alt-1 and Alt-3 are good starting points for monitoring of UE side models. Alt-1 may not have a lot of specification impact since it may totally up to UE on whether and how to update model related parameters. Furthermore, Hybrid monitoring may be useful for tight coupling of network and UE leading to improvement in overall performance. Both these scenarios also need to consider whether the UE side model is visible or invisible to network and if the model is trained and downloaded from the network to the UE. For UE side model monitoring letting the network have complete autonomy over model decisions may not be a good starting point.

- For UE-side AI/ML model, support UE-side and Hybrid model monitoring. NW-side monitoring can be further studied and used in specific cases if the model is transferred from the NW to the UE.

Agreement For BM-Case1 and BM-Case2 with a network-side AI/ML model, study potential specification impact on the following L1 reporting enhancement for AI/ML model inference - UE to report the measurement results of more than 4 beams in one reporting instance - Other L1 reporting enhancements can be considered

For AI/ML model training and inference at network side, potential specification impact would include potential enhancements to beam reporting from the UE. Current only up to 4 beams and related L1-measurement can be reported in one reporting instance. This can be updated to higher number of beams to enable BM-Case1 and 2 but the requirements for each case may be different. Additionally, the beam report may also be transmitted over a MAC-CE if a large number of beams are reported in a single reporting instance.

- For AI/ML model training and inference at NW side, support larger than 4 beams to be reported in one beam reporting instance with potential beam reporting over MAC-CE.

# Discussion on Specification Impact of AI/ML for Beam Management

AI/ML model implementation in most cases may be dependent on implementation and specific models will be difficult to specify. However, from a standards perspective, the functionalities required to enable AI/ML models, irrespective of implementation details to be integrated into 5G-NR networks should be studied. Current networks offer a myriad of beam measurement and reporting functionalities which may be further augmented to enable AI/ML use cases. Some examples for AI/ML model integration into current 5G networks are provided below

## Spatial Domain Beam Prediction

Spatial domain beam prediction can be applied towards initial beam acquisition using SSB beams or beam tracking using narrower CSI-RS beams in conjunction with current beam reporting framework for faster and more accurate beam selection. The ML model may reside either at UE or gNB and the major specification impact will be based on triggering of beam measurement reports and reference signal transmissions.

### Spatial Domain Beam Prediction with UE-side Model

Figure 2: ML Aided Beam Prediction with Model residing at the UE

For this case, the ML model may reside at the UE to enable ML Aided UE Rx beam tracking. It can work as follows:

- UE measures Rx beam quality for fixed gNB beam

- If Rx beam drops below threshold, UE triggers CSI-RS with repetition from gNB

- gNB transmits CSI-RS with repetition (BM)

- UE can measure subset of Rx beams to find the best Rx beam

The specification impact may be from UE triggered CSI-RS transmissions for beam measurements. Initial performance evaluation of such models has been provided in [2].

### Spatial Domain Beam Prediction with gNB side model

Figure 3: ML Aided Beam Prediction at gNB with Model residing at the gNB

For this use-case, the ML model may reside at the network to enable UE and ML-aided gNB beam tracking using DL measurements. The process can function as follows:

- gNB triggers CSI-RS for CSI based on periodic/aperiodic beam report from UE if Tx beam drops below threshold

- gNB transmits M CSI-RS where M<< total number of CSI-RS beams

- UE measures L1-RSRP and reports to gNB

- gNB can use M measurement to predict best Tx (Rx) beam

In this case, the UE does not need to know which CSI-RS beams are transmitted i.e., gNB can sample spatial domain based on its own implementation. The scheme relies on the L1 beam report from the UE for best beam prediction at the gNB. Potential gain is from reduced CSI-RS transmissions for measurement.

### Joint Beam Pair Link Prediction

Figure 4: ML Aided Joint Beam-Pair Link Prediction with Model residing at the gNB

This use case is similar to joint P2+P3 procedure and can work with either DL or UL measurements as shown in the figure. The following setups can be considered with the assumption that the AI/ML model resides at the gNB:

- Setup 1 – DL measurement and reporting

  - CSI-RS transmission from gNB on subset of beams

  - UE measures on sub-set of Rx beams and reports back to gNB

  - gNB predicts best beam pair link using AI/ML model

- Setup 2 – UL Measurement

  - SRS transmission using a subset of UE beams

  - gNB measures using a sub-set of Rx beams and predicts the best beam pair link

To enable this use case, new L1 reporting formats may need to be specified. For example, under setup 1, there may be a need for the gNB to signal to the UE that it should measure each Tx beam from gNB on a specific Rx beam and then the UE should be able to report the measurement corresponding to each of these beam pair links to the gNB using a new L1 reporting format. The major gains in this use case are envisioned to be from the latency gains due to reduction in the total number of measurements needed.

## Temporal Domain Beam Prediction

A temporal domain beam prediction method is used to predict the base station (BS) and/or user equipment (UE) beams used for future transmission and reception. In general, the beam prediction process includes two phases, the observation phase and the prediction phase, as shown in Figure 5. The beam prediction can be implemented at the BS side, or the UE side, or both sides. In observation phase, measurements are made, e.g., L1-RSRP is collected. In prediction phase, the measurements are fed into the beam prediction model that generates the predicted beams, and the BS and/or UE uses the predicted beams to transmit and receive data in the prediction phase.

Figure 5. Temporal Domain Beam Prediction with L1-RSRP

An AI/ML based implementation of the temporal domain beam prediction function would allow for measurements over the observation window to be fed into a model which will then predict the measurements for the prediction window and these predictions can be used infer the best beam or set of beams to be used as shown in the figure below.

Figure 6. AIML-based Temporal Domain Beam Prediction

For the temporal domain beam prediction problem, in addition to the L1 measurement and reporting updates discussed in the previous section, another major area of specification impact would be to enable configuration of the observation window size where and the prediction window size. Depending on where the model resides, one or both may need to be configured to the UE. Furthermore, depending on model implementation and if the model is transferred one node to another, it may also be required to configure model selection at the inference node based on different prediction and/or observation window lengths. Finally, as mentioned in Section 2.2, if the model resides at the UE side, in order to perform measurements during the observation phase, the UE may need to trigger aperiodic reference signal transmissions from the gNB.

# Conclusion

In this paper, beam management use cases and related specification impact has been discussed. The main proposals from this paper are outlined here:

- Beam Pair prediction (Alt-3) should be supported, at least for BM-Case 1 since it can provide large latency and measurement gains for joint P2/P3 procedure

- For data collection with AI/ML model training at UE side, support UE triggered reference signal transmission from the gNB to enable the UE to perform L1 measurements at least on Set B for both BM-Case 1 and 2.

- For BM-Case1 and 2, Alt-1 (Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams) should be considered as the baseline use case, with potential specification impact on how beam IDs are mapped in the spatial domain.

- If the predicted RSRP is directly obtained from the output of the AI/ML model, such metrics should not be reported back to the network. If the predicted RSRP is defined as the actual measured RSRP on the best beam predicted by the model, such metric can be reported back to the network assuming that the overhead due to the additional measurement, if needed, is accounted for in the KPI calculation.

- For UE-side AI/ML model, support UE-side and Hybrid model monitoring. NW-side monitoring can be further studied and used in specific cases if the model is transferred from the NW to the UE.

- For AI/ML model training and inference at NW side, support larger than 4 beams to be reported in one beam reporting instance with potential beam reporting over MAC-CE.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” Qualcomm, 3GPP TSG RAN#94e, December 6 – 17, 2021.

- R1-2300943, “Evaluation for AI/ML Beam Management”, Intel Corporation, 3GPP RAN1#112, Greece, February 2022.