3GPP TSG RAN WG1 #112	R1-2300975

Athens, Greece, February 27th – March 3rd, 2023

Source:	Intel Corporation

Title:	Discussion on NR operation with less than 5MHz Bandwidth

Agenda item:	9.16.1

Document for:	Discussion and Decision

# Introduction

In RAN1 #111 meeting, the following agreements were reached on supporting NR operation with less than 5MHz bandwidth.

Agreement In an LS to RAN4, in addition to reuse 5 MHz channel bandwidth, RAN1 suppose only 3 MHz channel bandwidth is supported, and would like to get RAN4 responses on the maximum transmission bandwidth (the number of PRBs) for this channel BW. Agreement RAN1 would like to ask RAN4 if finer sync. raster for the 3MHz and/or 5MHz channel bandwidth is feasible, as well as any input from RAN1 for RAN4’s answer to this question. Agreement Before getting RAN4 responses, RAN1 assume maximum transmission bandwidth, 15RBs or 16RBs for 3 MHz channel BW for evaluation and analysis. Note: include agreement into the LS Agreement Before getting RAN4 responses, RAN1 assume that the UE could know which RBs are used for SSB transmission after PSS/SSS is detected for evaluation and analysis. Note: it does not mean indication signaling is needed. Note: include this agreement into the LS Agreement Including following 2 questions into the LS Question 1: RAN1’s understanding is that in addition to reuse 5 MHz channel bandwidth, RAN1 suppose only 3 MHz channel bandwidth is supported, and would like to get RAN4 responses on the maximum transmission bandwidth (the number of PRBs) for this channel BW Question 2: RAN1 have discussed aspects related to synch raster in the spectrum of interest. RAN1 would like to ask RAN4 if finer sync. raster for the 3MHz and/or 5MHz channel bandwidth is feasible, as well as if RAN4 needs any input from RAN1. Agreement The Draft LS to RAN4 R1-2212898 is endorsed in principle with modified question as agreed above and all agreements and conclusions made in RAN1#111. Agreement Final LS to RAN4 R1-2212919 is endorsed. Agreement For transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth, a subset of PRBs of 20-PRB PBCH are used for PBCH transmission if the transmission BW of a channel is less than 20PRBs. - FFS which PRBs are used and how to use the PRBs - Note: PRBs for PSS/SSS are not punctured. Agreement For CORESET#0 configuration for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth, following options are for study, - Opt.1: Existing configuration table for 15kHz SCS, 5MHz minimum channel BW (i.e., table 13-1 in TS38.213) is reused for configuration - Opt.2: A new CORESET#0 configuration table is to be introduced for the configuration. Conclusion No enhancements are required for PRACH to operate NR on transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. - Note: PRACH formats and configurations not fitting into the transmission BW are not applicable Agreement Short PRACH formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS are supported for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth. Conclusion No enhancements are needed for PUCCH to support transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth, - FFS: the necessity for PUCCH FH disabling. Agreement Study whether and how to recover PDCCH detection performance of CORESET#0 for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered, - Opt.1: Power boosting - Opt.2: Non-interleaved CCE-to-REG mapping - Opt.3: A new interleaver to ensure PDCCH is fully mapped in the spectrum - Opt.4: New aggregation level(s) for fit in the spectrum - Opt.5: PDCCH rate matching - Opt.6.: no enhancement specified Agreement Study whether and how to recover PBCH detection performance for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered, - Opt.1: Power boosting - Opt.2: Multiple PBCH receptions - Opt.3: PBCH remapping - Opt.4: PBCH payload reduction - Opt.5: PBCH rate matching around the punctured PRBs - Opt.6: no enhancement specified

In this contribution, we provide our views on the enhancements to support NR operation with less than 5MHz bandwidth in FR1.

# Discussion on enhancement for NR with less than 5MHz

## 2.1 PBCH

In NR, the existing SSB structure is shown in Figure 1. It can be seen that PSS/SSS occupies 12 RBs, corresponding to bandwidth 12*12*15KHz=2.16 MHz with SCS of 15kHz. Thus, the PSS/SSS can be reused for the case of operation with less than 5MHz.

For PBCH, the number of RBs is 20, corresponding to bandwidth 20*12*15KHz = 3.6MHz with SCS of 15kHz.

In the WID, bandwidth from around 3 MHz up to 5 MHz should be supported. Therefore, some enhancement is required for PBCH to support operation with less than 5MHz, e.g., 3MHz. With 3MHz bandwidth, 16 RBs are required for SCS of 15 kHz.

Figure 1 Existing SSB structure in NR

In RAN1 #111 meeting, it was agreed that a subset of the 20 PRBs will be used for PBCH transmission for operation with less than 5MHz bandwidth.

Therefore, the PBCH puncturing pattern should be defined, such as:

- Several RBs, e.g., 4 RBs, at the lower frequency part of the PBCH is punctured

- Several RBs, e.g., 4 RBs, at the higher frequency part of the PBCH is punctured

- Several RBs, e.g., 2 RBs at both the lower frequency part and the higher frequency part of the PBCH is punctured.

Proposal 1:

- For NR operation with less than 5MHz, the PBCH puncturing pattern should be defined, i.e., which part of RBs in frequency is punctured.

With PBCH puncturing, there would be performance loss. In RAN1 #111 meeting, several options were identified to recover the PBCH detection performance.

-	Opt.1: Power boosting

-	Opt.2: Multiple PBCH receptions

-	Opt.3: PBCH remapping

-	Opt.4: PBCH payload reduction

-	Opt.5: PBCH rate matching around the punctured PRBs

-	Opt.6: no enhancement specified

In contrast, we think Option 1 (power boosting) and Option 5 (rate matching) can provide better performance and relatively less spec change. With Option 3 or Option 4, it changes the PBCH design and will need more discussion.

With power boosting, the EPRE of the punctured PBCH should be defined.

Proposal 2:

- For NR operation with less than 5MHz, PBCH rate matching and/or power boosting could be considered to compensate the performance loss by PBCH puncturing.

## 2.2 PDCCH

In NR, for PDCCH, the resource configuration for CORESET #0 is shown in Figure 2 which is copied from TS 38.213. It can be observed that for SCS of 15 kHz, the minimum number of RBs for CORESET #0 is 24, corresponding to bandwidth of 24*12*15KHz = 4.32 MHz.

Therefore, in order to support operation with less than 5MHz, smaller number of RBs should be supported for CORESET #0, e.g., 16 RBs.

Figure 2 Existing resource configuration for CORESET #0 with 15KHz SCS

In RAN1 #111 meeting, it was discussed on CORESET #0 configuration to support operation with less than 5MHz bandwidth. Two options were identified, one is to reuse the legacy configuration table for CORESET #0. The other option is to introduce new configuration table for CORESET #0.

In our view, introducing new configuration table with smaller bandwidth configuration is slightly preferred.

Proposal 3:

- For NR operation with less than 5MHz, it is slightly preferred to introduce a new configuration table for CORESET #0.

To recover the PDCCH detection performance of CORESET #0, we think the power boosting and rate matching could be applied for CORESET #0 with smaller bandwidth.

Proposal 4:

- For NR operation with less than 5MHz, rate matching and/or power boosting could be considered to recover the PDCCH detection performance of CORESET #0.

One issue for CORESET #0 is the offset between CORESET #0 and SSB. Since PBCH is punctured, it should be clarified whether the offset between CORESET #0 and SSB is based on the punctured PBCH or the original PBCH.

Proposal 5:

- For NR operation with less than 5MHz, it should be clarified whether the offset between CORESET #0 and SSB is based on the punctured PBCH or the original PBCH.

## 2.3 Initial DL BWP

In current NR spec, the initial DL BWP bandwidth is the same as CORESET #0. To support operation with less than 5MHz bandwidth, there might be some impact on the initial DL BWP size determination.

If the new configuration table is introduced for CORESET #0, then the existing rules for initial DL BWP size could be used, i.e., the same as CORESET #0.

If the legacy configuration table for CORESET #0 is used, and the PDCCH candidates for CORESET #0 is punctured, then the initial DL BWP size should be clarified. For example, the initial DL BWP size should be the same as CORESET #0 after puncturing.

Proposal 6:

- For NR operation with less than 5MHz, RAN1 to further discuss the initial DL BWP size determination, especially if puncturing is applied for CORESET #0.

## 2.4 CSI-RS/TRS

In order to support NR operation with smaller than 5 MHz bandwidth, another aspect to be considered is CSI-RS and TRS.

In legacy spec, for CSI-RS, the minimum bandwidth is min{24 PRBs, bandwidth of BWP}. For TRS, the minimum bandwidth is min{52 PRBs, bandwidth of BWP}.

Therefore, the minimum bandwidth of CSI-RS/TRS should be extended to smaller number of RBs, e.g., 16 RBs.

Regarding the sequence generation for CSI-RS/TRS, we think the existing processing could be reused.

Proposal 7:

- For NR operation with less than 5MHz, RAN1 to discuss the potential enhancement on CSI-RS/TRS, i.e., extending the minimum bandwidth of CSI-RS/TRS to smaller bandwidth.

# Conclusion

In conclusion, we have the following proposals on supporting NR operation with less than 5 MHz bandwidth in Rel-18.

Proposal 1:

- For NR operation with less than 5MHz, the PBCH puncturing pattern should be defined, i.e., which part of RBs in frequency is punctured.

Proposal 2:

- For NR operation with less than 5MHz, PBCH rate matching and/or power boosting could be considered to compensate the performance loss by PBCH puncturing.

Proposal 3:

- For NR operation with less than 5MHz, it is slightly preferred to introduce a new configuration table for CORESET #0.

Proposal 4:

- For NR operation with less than 5MHz, rate matching and/or power boosting could be considered to recover the PDCCH detection performance of CORESET #0.

Proposal 5:

- For NR operation with less than 5MHz, it should be clarified whether the offset between CORESET #0 and SSB is based on the punctured PBCH or the original PBCH.

Proposal 6:

- For NR operation with less than 5MHz, RAN1 to further discuss the initial DL BWP size determination, especially if puncturing is applied for CORESET #0.

Proposal 7:

- For NR operation with less than 5MHz, RAN1 to discuss the potential enhancement on CSI-RS/TRS, i.e., extending the minimum bandwidth of CSI-RS/TRS to smaller bandwidth.

# References

- 3GPP TS 38.211 v17.4.0, Physical channels and modulation

- 3GPP TS 38.212 v17.4.0, Multiplexing and channel coding

- 3GPP TS 38.213 v17.4.0, Physical layer procedures for control

- 3GPP TS 38.214 v17.4.0, Physical layer procedures for data

- 3GPP TS 38.331 v17.3.0, Radio Resource Control (RRC) protocol specification

- RP-213603, 3GPP TSG RAN Meeting #94-e, Dec 6 – Dec 17, 2021

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #111, November 14th – 18th, 2022