3GPP TSG RAN WG1 #112		                                         R1-2301025

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.12.2

Source: 	CMCC

Title: 	Discussion on timing advance management to reduce latency

Document for:	Discussion and Decision

Introduction

In the RAN1#111 meeting, a few agreements had been achieved for the TA acquisition before cell switching command. The agreements are listed in the related sections.

In this contribution, we provide our views on the TA management in the mobility scenarios.

Discussion

Shorten the interruption time during the handover can reduce the latency of the RACH procedure for UL synchronization. If the uplink synchronization can be achieved before the handover procedure or before the interruption time, this part of latency for the mobility can be reduced.

Agreement On mechanism to acquire TA of the candidate cells, the following solutions can be further studied: - RACH-based solutions - e.g., PDCCH ordered RACH, UE-triggered RACH, higher layer triggered RACH from NW other than L3 HO cmd - RACH-less solutions - e.g., SRS based TA acquisition, Rx timing difference based, RACH-less mechanism as in LTE, UE based TA measurement (including UE based TA measurement with one TAC from serving cell)

Two category solutions are agreed for further study. RACH-based solutions are mainly based on the RACH procedure or PRACH to acquire the uplink timing. PDCCH ordered RACH depends on serving cell’s decision for the acquisition of TA of the candidate cells. And RACH-less solutions were subject to the scenarios that zero TA or the candidate cell and the serving cell have the same TA. To acquire the TA of candidate cells, UE based TA estimation could be considered. Based on the TA estimation from UE, the RACH-less solutions can work.

For the PDCCH order RACH, the agreements are listed below.

Agreement On mechanism to acquire TA of the candidate cell(s) in Rel-18 LTM, at least support PDCCH ordered RACH. - The PDCCH order is only triggered by source cell - FFS: the details including content of DCI, RACH resource configuration, RAR transmission mechanism, etc. - Note: any other RACH-based solutions are for discussion separately Agreement For PDCCH ordered RACH in LTM, at least the following enhancements are supported - Introduce indication of candidate cell and/or RO of candidate cell in DCI - configuration of RACH resource for candidate cell(s) is provided prior to the PDCCH order - FFS: whether/how to transmit RAR Agreement On whether RAR is needed for PDCCH ordered RACH for a candidate cell in LTM, the following alternatives are considered for further study - Alt 1: RAR is needed - Alt 2: RAR is not needed - Note: If Alt 2 is supported, TA value of candidate cell is indicated in cell switch command - Alt 3: whether RAR is needed can be configured Agreement - TA updating (i.e. re-acquisition of TA) for candidate cell can be triggered by NW. - same triggering mechanism reuse the initial TA acquisition, i.e., PDCCH order triggered RACH in a candidate cell

For the RACH based solution, the PDCCH ordered RACH is triggered by the source cell. And since the UE is still connected to the serving cell, and there is no connection between the UE and the target cell, the UE can get the RAR which is mainly used for TA indication from the serving cell. And if the TA value is still valid, the UE can transmit the uplink using the TA value after the switching command is received. In this case, the switching command does not need to carry the TA value for the target cell. Another way is that, the initial TA is indicated in the cell switch command, as Alt 2. For the Alt 1, multiple TAs could be indicated through the RAR. But the number of TAs that could be maintained depends on UE’s capability. And if the UE cannot maintain multiple TAs, the Alt 2 is more proper. Since finally the UE can use only one TA for the uplink transmission to the target cell. The TA value(s) could be carried in one MAC CE. And the MAC CE can be attached in the either a RAR or in a cell switching command. From our understanding both Alt 1 and Alt 2 can work. Although whether RAR is supported for PDCCH ordered RACH for a candidate cell in LTM is still under discussion. It should be clarified whether the RAR in the agreement is the legacy one. And if a new RAR would be defined, the contents of the RAR for L1/L2 mobility should be defined. At least the TA value(s) of the candidate cells should be included in the RAR for mobility. And if the multiple TAs could be conveyed in one RAR or MAC CE, the number of TAs should depend on UE’s capability.

Proposal 1:

The contents of the RAR for L1/L2 mobility need further discussion. At least the TA value(s) of the candidate cell should be included.

Proposal 2:

The number of TA value of candidate cells which UE can maintain can be a UE capability.

In the last meeting, the TA updates for candidate cell is agreed to be triggered by NW. And the same triggering mechanism of initial TA acquisitions is used. A timer or some event could be set to evaluate whether the TA values are still valid. If the TA of candidate cell expires, then an PDCCH ordered RACH could be triggered by gNB to update or just for TA acquistions.

Proposal 3:

It should be further discussed how to differentiate the PRACH is for initial access or it is for TA acquisition for L1/L2 mobility.

The RACH-less solutions were only used under certain scenarios. One is when the TA of the target cell is the same with the current cell. The 2nd is that the TA value equals zero which usually happens in the small cell. In those scenarios, the initial TA could be acquired based on either the TA of current cell or the DL reception timing. If the DL reception timing difference could be required by UE, UE could calculate the uplink TA to the candidate cells. The initial TA derived be UE based on the timing difference could be used for the first transmission to the target cell. And the target cell can adjust the TA values based on the first uplink transmission. The Rx timing difference based solutions can expand the using scenarios compared to the LTE scenarios.

Proposal 4:

Rx timing difference based solution should be supported among the RACH-less solutions.

Agreement For TA acquisition of a candidate cell before cell switch command is received, study at least the following alternatives of associating TA/TAG to candidate cell: - Alt1: Associate TA/TAG and candidate cell implicitly, e.g., - the association between TA/TAG and TCI states can be configured - Alt2: Associate TA/TAG and candidate cell explicitly, e.g., - the association is provided as a part of candidate cell(s) configuration - the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration

In the two TAs under the multiple TRP scenario, the configuration of the two TRPs could be delivered to the UE with two TAs. All the configurations are from the serving cell and configured to the UE. The additional TA/TAG could associate with a configured TCI state. But in the mobility scenario, the UE only has the configuration of its current serving cell. All the configurations of the candidate cell are still unknown or not activated. The UE cannot directly associate the TA or TAG with some deactivated parameters and event implicitly. If the configuration of the candidate cell could be provided directly to the UE and serving cell. Then the UE could connect the TA or TAG directly to the parameters of the candidate cell.

Proposal 5:

The Alt 2, associating TA/TAG and the candidate cell explicitly, is slightly preferred.

Conclusions

In this contribution, we provide our views on the TA management in the mobility scenarios. The proposals are listed as below.

Proposal 1:

The contents of the RAR for L1/L2 mobility need further discussion. At least the TA value(s) of the candidate cell should be included.

Proposal 2:

The number of TA value of candidate cells which UE can maintain can be a UE capability.

Proposal 3:

It should be further discussed how to differentiate the PRACH is for initial access or it is for TA acquisition for L1/L2 mobility.

Proposal 4:

Rx timing difference based solution should be supported among the RACH-less solutions.

Proposal 5:

The Alt 2, associating TA/TAG and the candidate cell explicitly, is slightly preferred.

References

[1] Chair’s notes RAN1_111, 3GPP TSG RAN WG1 #111, Toulouse, France, November 14th – 18th, 2022