3GPP TSG RAN WG1 #112 R1-2301087 Athens, Greece, February 27th – March 3rd, 2023 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; rev; -; Current version:; 17.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on configuration of G-RNTI / G-CS-RNTI per serving cell. Source to WG:; Lenovo. Source to TSG:; R1. Work item code:; NR_MBS-Core; Date:; 2023-02-15. Category:; F; Release:; Rel-17. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; According to RAN1#108-e agreements listed as below, multicast reception for UE is not supported on more than one cell. ## RAN1#108-e Agreement If UE supports carrier aggregation for unicast, multicast reception on an activated SCell with self-scheduling is supported subject to UE capability in Rel-17. - UE is not expected to be configured simultaneously with more than one component carrier for multicast reception. - Cross-carrier scheduling for multicast reception is not supported in Rel-17. - The capability of supporting MBS multicast on SCell is a separate capability from the CA capability for unicast. - The granularity of UE reporting the capability of supporting MBS multicast reception is per FSPC As mentioned in above agreements, “UE is not expected to be configured simultaneously with more than one component carrier for multicast reception.”, so when UE is configured with one or multiple G-RNTIs, these G-RNTIs should be configured only on one serving cell of the UE. The current spec, “A UE can be provided one or more G-RNTIs for multicast per serving cell for scrambling the CRC of multicast DCI formats for scheduling PDSCH receptions. The UE can be provided one or more G-CS-RNTI per serving cell for scrambling the CRC of multicast DCI formats providing activation/release/scheduling retransmission for SPS PDSCH receptions.”, may imply that UE can be provided multiple G-RNTIs on more than one cell. In that sense, it does not clearly cover the intention of RAN1 agreement that UE receives multicast only from a single serving cell. It is necessary to cover the intention to make spec clear.. -. Summary of change:; Replace “per serving cell” with “for a serving cell.”. Consequences if not approved:; G-RNTI configuration for multicast is not aligned with RAN1 agreements.. Clauses affected:; 18. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:; This is the first version for this CR.. # 18 Multicast Broadcast Services This clause is applicable only for PDCCH receptions, PDSCH receptions, and PUCCH transmissions for MBS on a serving cell. DCI formats with CRC scrambled by G-RNTI for multicast or G-CS-RNTI scheduling PDSCH receptions are referred to as multicast DCI formats and the PDSCH receptions are referred to as multicast PDSCH receptions. DCI formats with CRC scrambled by MCCH-RNTI or G-RNTI for broadcast scheduling PDSCH receptions are referred to as broadcast DCI formats and the PDSCH receptions are referred to as broadcast PDSCH receptions. HARQ-ACK information associated with multicast DCI formats or multicast PDSCH receptions is referred to as multicast HARQ-ACK information. A UE can be provided one or more G-RNTIs for multicast for a serving cell for scrambling the CRC of multicast DCI formats for scheduling PDSCH receptions. The UE can be provided one or more G-CS-RNTI for the serving cell for scrambling the CRC of multicast DCI formats providing activation/release/scheduling retransmission for SPS PDSCH receptions. <Unchanged part omitted>