3GPP TSG RAN WG1 #112			R1-2301111

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.10.1

Source: 	LG Electronics

Title: 	Discussion on XR-specific capacity enhancements

Document for:	Discussion and decision

- Introduction

In the last meeting, following agreements have been made for XR SI [1][2].

From RAN1#111 chairman notes: Agreement Support dynamic indication of the unused CG PUSCH occasion(s) based on UCI (e.g., CG-UCI or a new UCI) by the UE Agreement Support multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration From WID: Specify the enhancements related to capacity: - Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE (RAN1);

- Discussion

In this contribution, we present our views on the XR-specific capacity enhancements. Specifically, we provide potential issues and possible solutions for XR capacity enhancement techniques.

  - Multiple CG occasions in a period of a single CG PUSCH configuration

Considering XR traffic characteristic, like AR/VR video, frames are periodically generated but the size of each frames are not predictable. Also, due to different frame size, small jitter can occur at each frame generations. To support those traffic efficiently, it has been discussed to use multiple CG occasions per frame. UE can freely use a number of PUSCHs per frame according to the video frame size, as well as to mitigate jitter impact.

Until Rel-17 NR, it is not possible to allocate multiple CG occasion for different TBs in a period with single configuration. Therefore, gNB have to configure multiple CG configurations for UE, in order to allocate multiple PUSCHs as a burst. Multiple configurations could be easiest way to support multiple CG occasions, but it could be hassle, since gNB needs to activate CG configurations one by one.

In the last meeting, it has been agreed to support multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration. Accordingly, we provide potential issues to support multiple CG occasions in period and possible solutions for those issues.

Some approaches to support multiple CG occasions had been discussed. Those approaches can be distinguished by following two types.

- Schedule a TDRA row configured with multiple SLIVs

- Configure a resource with repetition pattern in a period

The first type of solutions is to use TDRA table to schedule multiple resources at once. By either DCI or RRC configuration, gNB can indicate UE with a TDRA row configured with multiple time-domain resources for a period. Then, UE would get recurring resources that differ only in time domain resources.

The second type of solution may use different RRC parameter to create desired pattern. For example, we can consider existing configured grant design for NR-U, which determines CG occasion by two parameters: cg-nrofPUSCH-InSlot-r16 and cg-nrofSlots-r16. With this two parameters, multiple PUSCH occasions can be configured in a slot, and such slots can be configured at the beginning of each period.

The main difference between two types is flexibility. With TDRA row consisting of multiple different SLIVs, resource size can vary and it is easier to adjust resource position in a slot. In our view, though the second type of solution may have less specification impact, we slightly prefer the first approach. In Rel-17 NR, the TDRA table where a row include multiple SLIVs has been introduced for dynamic scheduling purpose, so we can re-use it as enhanced TDRA table for CG PUSCH.

Proposal 1: Support a TDRA table where each rows includes multiple SLIV values for CG.

- FFS : DMRS mapping type, repetition type, numberOfRepetitions

In the current specification, it is specified as an error case that a TDRA row has multiple SLIVs with repetition factors larger than 1. Considering XR requirement, it could be beneficial to support repetition and multiple CG occasions in a period at the same time.

Proposal 2: It is necessary to investigate how to support the repetition for each of multiple SLIVs in a same TDRA row.

With enhanced TDRA table, UE can be indicated with multiple SLIVs via the CG activation DCI, in case of type 2 CG. For type 1 CG, it is a bit complicated since there is no activation DCI so there is no DCI format. Instead, type 1 CG uses virtual DCI format consisted of RRC parameters. It is not possible to assign specific TDRA table for type 1 CG. Therefore, it is necessary to discuss how Type 1 CG can be supported with enhanced TDRA table.

Also, TDRA table may have different K2 values for different SLIVs to distribute multiple CG occasions in a period. However, type 1 CG configuration ignores K2 value of given TDRA row since there is already slot level offset for type 1 CG. Therefore, related UE behavior should be changed to allocate multiple CG occasions in a period of type 1 CG.

Proposal 3: Discuss how to apply the enhanced TDRA for type 1 CG.

In terms of video, 60 frames can be generated per one second in general. To support those kind of traffic, it may be necessary to allocate multiple CG occasions in a period, spaced at 16 or 17 ms intervals. However, According to current specification, PUSCH is scheduled with K2 offset referring to the slot where DCI is received.

Figure 1. How K2 is applied for type 2 CG

Figure 1 shows how K2 offset can be applied for type 2 CG activated with multiple SLIVs. In this example, different K2 are given for each SLIV. In figure 1, the 2nd and the 3rd K2 offsets should be larger than 16 or 32, respectively, in order to have 16 ms interval between adjacent CG occasions. According to current specification, the value range is quite limited.

To solve this kind of issue, it can be considered to use different reference point for K2. For example, as shown in figure 2, each K2 offset can be applied with referring to previous PUSCH occasion.

Figure 2. Alternative way to apply K2 for type 2 CG

Proposal 4: Support separated K2 offset per SLIV for flexibility.

- Each K2 starts from the slot where previous SLIV ends.

  - Dynamic indication of the unused CG PUSCH by UE

In the existing specification, UE can skip CG PUSCH occasion according to its buffer status so that transmit power can be saved. However, at the gNB side, it is highly limited to re-use the CG PUSCH occasion skipped by UE, since there is no explicit indication from UE that wouldn’t use the CG PUSCH. In the last meeting, it has been agreed to support dynamic indication of unused CG PUSCH occasion from UE, in short, unused resource indication; hereinafter referred as “URI”.

To support URI, following aspects need to be decided:

- Where URI is transmitted

- Where URI indicates

- How to construct and apply URI information

    - Where URI is transmitted

URI is basically uplink control information generated by UE. Thus, it should be considered to use existing container for UCIs; like PUSCH and/or PUCCH. In NR-U, CG-UCI has been introduced to indicate transmission parameters which are determined by UE, on CG PUSCH. In our view, we can simply re-use the design principle of CG-UCI and/or use a new separated PUCCH for flexibility.

Proposal 5: Support CG UCI or separated PUCCH in order to transmit URI.

- FFS: How to multiplex the URI with other existing UCIs

URI may have an information for resource usage on a number of PUSCHs. Also, the information in URI is not so critical. Even if UE fails to transmit URI, gNB would just lose some chance to re-scheduling PUSCH. However, since every CG PUSCH can be skipped by UE, it is difficult to predict where UE requires to transmit URI. In our view, we think the baseline is to transmit URI on every CG occasion where UE sends PUSCH.

Proposal 6: As a baseline, URI is transmitted on every CG occasion where UE sends PUSCH.

    - Where URI indicates

Considering XR traffic characteristic, the purpose of CG usage indication is adaptive resource utilization for XR traffic. For example, based on encoding result of XR video traffic, UE can determine how many PUSCHs are required for the encoded video frames. If gNB already schedules a plenty of CG resources for the latency, UE may indicate how many CG occasions would be used (or unused) via CG usage indication, and gNB may utilize remaining unused UL resources for other purposes. Given that, it is important how to indicate CG usage with the guarantee of the time to re-schedule.

Observation: the re-scheduling time (in gNB side) should be guaranteed in between where UE transmit URI and the beginning of unused resources indicated by the URI.

To make URI information useful, it should be based on real buffer status and proper prediction. If UE has data to send in its buffer, UE may know next CG occasion need to be used. However, these kind of information is not useful to enhance XR capacity. In order for UE to determine unused resources, the UE needs to be able to predict the buffer status at a future. Without optimistic assumptions about XR awareness, the unused resource indicated by URI might be limited to the CG occasions that are close to where URI transmitted.

Observation: the range of unused resource that can be indicated by URI might be limited without XR-awareness information.

Figure 3. An example of the range of URI

Based on above observation, we think the range of URI should be based on where URI is transmitted. For example, the range of URI starts X symbol after where URI transmission ends, and its length is given by gNB configuration in symbols based on UE capability, where X is the re-scheduling time required in gNB side. Otherwise, for simplicity and to keep fixed information size, the range can ends at the last of CG PUSCH in current or next period.

Proposal 7: the range of unused resource that can be indicated by URI is determined based on where URI is transmitted.

- The first CG PUSCH indicated by URI starts no earlier than X symbol after where URI transmission ends, where X is re-scheduling time required by gNB.

- The last CG PUSCH starts no later than Y symbol after the beginning of the first CG PUSCH, where Y is provided by gNB configuration based on UE capability

- FFS: How to define X and Y

    - How to construct and apply URI information.

Given the range of URI, URI information can be simply generated as a bitmap where each bit corresponds to each CG PUSCH in the range, and the first CG occasion mapped to the first bit.

Proposal 8: URI can be constructed as bitmap, where each bit corresponds to each CG PUSCH.

It would be beneficial to assume that different CG configurations have separated CG usage indication since the necessity of the indication is up to the traffic characteristic. For an example, for deterministic traffic having static size, like pose data, CG usage indication wouldn’t be beneficial.

Proposal 9: URI transmission is configured per CG configuration.

If UE is configured with multiple CG configurations, it may be necessary to multiple URI configuration to indicate unused resource in different CG configuration. It could be hassle especially when URI is transmitted on CG-UCI, since only MAC layer knows which PUSCH would be transmitted. Some PUSCHs might be dropped, so it would be difficult for gNB to get URI information. To solve this issue, it can be considered to apply the URI transmitted via a CG configuration to the other CG configuration. A parameter indicating CG configuration to which URI applies could be necessary. By doing so, gNB only need to get single URI information to manage multiple CG configuration. This approach is beneficial when CG configuration (e.g. CG1) for pose data, which have short periodicity, handles other CG configuration (e.g. CG2) for like video (for example, URI is transmitted via CG1 and the URI indicates unused resource on CG2).

Proposal 10: Support to apply URI transmitted via a CG configuration to the other CG configuration.

- A parameter indicating CG configuration to which URI applies can be provided by gNB.

Once UE generated an information of URI, UE has to avoid using indicated (as unused) CG PUSCH occasion. In addition to this, to make empty uplink resource, UE need to drop other PUSCH overlapped with the indicated CG PUSCH.

Proposal 11: a PUSCH indicated by URI and other PUSCH overlapped with the PUSCH are assumed to be dropped.

- No MAC PDU is generated for those CG PUSCHs.

Conclusion

In this contribution, we shared our views on the potential enhancements of capacity enhancement techniques for XR. Following are proposed for XR-specific capacity improvement:

Proposal 1: Support a TDRA table where each rows includes multiple SLIV values for CG.

- FFS : DMRS mapping type, repetition type, numberOfRepetitions

Proposal 2: It is necessary to investigate how to support the repetition for each of multiple SLIVs in a same TDRA row.

Proposal 3: Discuss how to apply the enhanced TDRA for type 1 CG.

Proposal 4: Support separated K2 offset per SLIV for flexibility.

- Each K2 starts from the slot where previous SLIV ends.

Proposal 5: Support CG UCI or separated PUCCH in order to transmit URI.

- FFS: How to multiplex the URI with other existing UCIs

Proposal 6: As a baseline, URI is transmitted on every CG occasion where UE sends PUSCH.

Observation: the re-scheduling time (in gNB side) should be guaranteed in between where UE transmit URI and the beginning of unused resources indicated by the URI.

Observation: the range of unused resource that can be indicated by URI might be limited without XR-awareness information.

Proposal 7: the range of unused resource that can be indicated by URI is determined based on where URI is transmitted.

- The first CG PUSCH indicated by URI starts no earlier than X symbol after where URI transmission ends, where X is re-scheduling time required by gNB.

- The last CG PUSCH starts no later than Y symbol after the beginning of the first CG PUSCH, where Y is provided by gNB configuration based on UE capability

- FFS: How to define X and Y

Proposal 8: URI can be constructed as bitmap, where each bit corresponds to each CG PUSCH.

Proposal 9: URI transmission is configured per CG configuration.

Proposal 10: Support to apply URI transmitted via a CG configuration to the other CG configuration.

- A parameter indicating CG configuration to which URI applies can be provided by gNB.

Proposal 11: a PUSCH indicated by URI and other PUSCH overlapped with the PUSCH are assumed to be dropped.

- No MAC PDU is generated for those CG PUSCHs.

- References

RAN1#111 Chairman’s notes

RP-223502, New WID on XR Enhancement for NR