3GPP TSG RAN WG1 #112			R1-2301147

Athens, Greece, February 27th – March 3rd, 2023

Source:	Panasonic

Title: 	Discussion on general aspects of AI/ML framework

Agenda Item:		9.2.1

Document for:	Discussion

- Introduction

A study item “Study on Artificial Intelligence and Machine Learning for NR air-interface” has been approved for Rel.18 [1].

After the end of RAN1#111, the final FL summary [2] was provided by FL. FL shares the recommendation of the topics to be discussed in this RAN1#112. Based on FL recommendation, this document discusses general aspects of AI/ML framework.

The agreement in the past meeting is described in Annex.

- Discussion

The approaches on functionality/model identification

Our understanding is ultimately the exact function of AI/ML model can be only defined by the tests AI/ML model, although natural language based description of the function like use case, radio channel environment and so on is helpful to identify AI/ML model. Therefore, the model identification should be based on the defined set of tests. The test set can be 3gpp or bi-lateral based. The functionality identification is rather just grouping of the models. Therefore, LCM discussion can be focused on model ID based approach.

Proposal 1: The exact function of AI/ML model can be defined only by the tests. The exact AI/ML model identification is based on the defined set of tests. LCM discussion can be focused on model ID based.

LCM including functionality identification, model identification, and UE capability

Based on the above discussion, the function of the AI/ML model is defined by the test set of AI/ML inference. In addition, the other information like use case, radio channel, the used input/output for the training are useful additional information.

- The test set number of AI/ML inference

- The use case

- The radio channel and/or environment where AI/ML model is trained or certain scenarios/configurations/sites information. Note that the usage is not limited to situation where it is trained but the usage is up to the network decision.

- Input and output for AI/ML inference

With above definition of AI/ML function, model ID could contain following information.

- Index to identify the function

- Index to distinguish inter-vendor operable case and bi-lateral case

- Index to identify the vendor

- Index to identify the version number to be identified by the network.

Proposal 2: The function is defined by test set number of AI/ML inference. In addition, the use case, radio channel and/or environment are useful as the additional information.

Proposal 3: The model ID could contain the functional ID, the distinction of inter-vender and bi-lateral, vender identification, the version number to be identified by the network.

For level y, UE capability reports the model ID which UE can use. The extension of current UE capability report or new report is up to RAN2 discussion including where the UE capability report is sent. Based on model ID, the network can identify the function, vendor and version number. Based on the network decision, the network indicates the model ID to be applicable by UE.

For level z, before UE capability report, which indicates the AI/ML model availability, the model transfer procedure is carried out. The model transfer procedure may include the step to indicate whether UE can accept the model. Therefore, the procedure that UE capability report and the latter are no difference.

If/when UE has new AI/ML model available for the deployment, the model ID with corresponding version number is reported by UE capability report. Only the AI/ML models that UE can be enabled are reported as UE capability. Therefore, if certain model is not saved within UE because of HW or memory limitation, such AI/ML model is not reported.

In addition to the version number to be identified by the network, UE vendor may utilize multiple versions of AI/ML models. These models may be different on the complexity, performance and/or power consumption. If the inference performance is identical among them, it may not be required to be identified by the network. On the other hand, if some performance can be different, the difference on AI/ML model needs to be identified by the network.

For inter-operable case, test to check inference performance of AI/ML may be specified in RAN4. For bi-lateral case, test to check inference performance may be carried out as the part of inter-operator test.

Following figure shows the LCM of inter-vender case of UE side model.

3gpp specifies AI/ML model function and related test case

UE vendor prepare AI/ML model

UE vendor test the AI/ML model

Using UE capability report, UE reports availability of the AI/ML model

Network indicates to enable the AI/ML model

UE enables the AI/ML model for inference

UE vendor updates

the AI/ML model

Figure 1. LCM of inter-vendor case of UE side model in level y

Proposal 4: UE capability reports the model ID which UE can use to the network. The network indicates the model ID to be applicable by UE.

Proposal 5: If/when UE has new AI/ML model available for the deployment, the model ID with corresponding version number is reported to the network.

The views on the input for the LS to RAN2 and SA2

RAN2 discussed the model transfer/delivery aspect in [3]. We think the LS outlined in RAN1#111 can be sent to RAN2 and SA2.

The scenario and pros/cons of the Model delivery/transfer

There are views that no need of the discussion on AI/ML model delivery/transfer with open-format model. Our view is at least high level framework and corresponding pros/cons should be identified as the outcome of the study. It is not required to have detailed realization in the study item.

Proposal 6: The high level framework and corresponding pros/cons of AI/ML model delivery/transfer with open-format model should be concluded in the study. It is not required to have detailed realization in the study item.

The merit of model delivery/transfer would be following.

- The network can provide AI/ML model specific to certain site deployment is possible. Without disclosing the detail of antenna deployment or other network proprietary information from the network, the beam management, positioning or CSI reporting of specific site situation is possible.

- The update of AI/ML model can be carried out at the network and test of AI/ML inference at NW	 side as well. In the case of open-format models, the improvement/knowledge based on the other vendor's AI/ML model can be also incorporated, which further reduce the update cycle.

- The number of AI/ML model recognized by the network may be limited by the number of the test as the test ultimately determine the function of AI/ML model. The open-format model with model delivery/transfer may allow the limitation of the number of tests as the responsibility of AI/ML model can be located at the network.

- The usage of open-format model may require compilation from open-format to specific implementation format. It can be within UE or within OTT server. This involves more steps. The open-format model may not be optimized for power consumption and efficiency for AI/ML model Inference.

We propose to take above as the observations.

Proposal 7: To take the observation of pros/cons of AI/ML model delivery/transfer with open-format model as following.

- The merit of AI/ML model delivery/transfer with open-format model is to allow certain site specific model without disclosing the network proprietary information, shorter update cycle with multi-vendor improvement/knowledge, and management of large AI/ML models without the restriction on the number of specified test.

- The demerit of AI/ML model delivery/transfer with open-format model is more specification effort, the need of the compilation to the specific implementation, possible more power consumption and more expensive HW cost.

Online training at the network

FL said online training at the network is a matter of implementation and no strong reason to have explicit discussion. We share such view. Online training at the network may require specific measurement by UE but it should be discussed in use case specific and not required to be discussed in the general agenda.

Proposal 8: Online training at the network is not required to have specific discussion. If specific measurement is required, it should be discussed in use case specific.

Online training at the UE

FL said to prioritize LCM framework not involving online training and online training is instead as additional discussion. We support such view.

Proposal 9: LCM framework not involving online training should be prioritized discussion.

After online training, AI/ML model is updated. One option is the updated AI/ML model can be deployed without test. The other option is the updated AI/ML model needs some of the test before the deployment. The deployment without test can reduce the cycle of the update but the deployment without test has the concern especially UE side model. Therefore, we propose following.

Proposal 10: The need of test after online training before the deployment needs some discussion.

- Conclusion

We propose following.

Proposal 1: The exact function of AI/ML model can be defined only by the tests. The exact AI/ML model identification is based on the defined set of tests. LCM discussion can be focused on model ID based.

Proposal 2: The function is defined by test set number of AI/ML inference. In addition, the use case, radio channel and/or environment are useful as the additional information.

Proposal 3: The model ID could contain the functional ID, the distinction of inter-vender and bi-lateral, vender identification, the version number to be identified by the network.

Proposal 4: UE capability reports the model ID which UE can use to the network. The network indicates the model ID to be applicable by UE.

Proposal 5: If/when UE has new AI/ML model available for the deployment, the model ID with corresponding version number is reported to the network.

Proposal 6: The high level framework and corresponding pros/cons of AI/ML model delivery/transfer with open-format model should be concluded in the study. It is not required to have detailed realization in the study item.

Proposal 7: To take the observation of pros/cons of AI/ML model delivery/transfer with open-format model as following.

- The merit of AI/ML model delivery/transfer with open-format model is to allow certain site specific model without disclosing the network proprietary information, shorter update cycle with multi-vendor improvement/knowledge, and management of large AI/ML models without the restriction on the number of specified test.

- The demerit of AI/ML model delivery/transfer with open-format model is more specification effort, the need of the compilation to the specific implementation, possible more power consumption and more expensive HW cost.

Proposal 8: Online training at the network is not required to have specific discussion. If specific measurement is required, it should be discussed in use case specific.

Proposal 9: LCM framework not involving online training should be prioritized discussion.

Proposal 10: The need of test after online training before the deployment needs some discussion.

- Reference

[1]	RP-213599, “New SI: Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR Air Interface,”	Qualcomm (Moderator), RAN#94e.

[2] R1-2213003, "Summary#1 of General Aspects of AI/ML Framework", Moderator (Qualcomm), RAN1#111

[3] "Report of [Post120][053][AIML18] model transfer delivery (Huawei)", Huawei (email rapporteur),  https://www.3gpp.org/ftp/Email_Discussions/RAN2/%5BRAN2%23120%5D/%5BPost120%5D%5B053%5D%5BAIML18%5D%20model%20transfer%20delivery%20(Huawei)/Phase%202

- Past agreements

Agreements in RAN1#109:

Agreement

Use 3gpp channel models (TR 38.901) as the baseline for evaluations.

Note: Companies may submit additional results based on other dataset than generated by 3GPP channel models

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

The description of the terminologies may be further refined as the study progresses.

New terminologies may be added as the study progresses.

It is FFS which subset of terminologies to capture into the TR.

Table: Working list of terminologies

| Terminology | Description |
| --- | --- |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| Model download | Model transfer from the network to UE |
| Model upload | Model transfer from UE to the network |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Offline field data | The data collected from field and used for offline training of the AI/ML model |
| Online field data | The data collected from field and used for online training of the AI/ML model |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Unsupervised learning | A process of training a model without labelled data. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Model activation | enable an AI/ML model for a specific function |
| Model deactivation | disable an AI/ML model for a specific function |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function |

Conclusion

As indicated in SID, although specific AI/ML algorithms and models may be studied for evaluation purposes, AI/ML algorithms and models are implementation specific and are not expected to be specified.

Observation

Where AI/ML functionality resides depends on specific use cases and sub-use cases.

Conclusion

- RAN1 discussion should focus on network-UE interaction.

  - AI/ML functionality mapping within the network (such as gNB, LMF, or OAM) is up to RAN2/3 discussion.

Agreement

Take the following network-UE collaboration levels as one aspect for defining collaboration levels

- Level x: No collaboration

- Level y: Signaling-based collaboration without model transfer

- Level z: Signaling-based collaboration with model transfer

Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings

FFS: Clarification is needed for Level x-y boundary

Agreements in RAN1#110:

Agreement

Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management

- Data collection

  - Note: This also includes associated assistance information, if applicable.

- Model training

- [Model registration]

- Model deployment

  - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device.

- [Model configuration]

- Model inference operation

- Model selection, activation, deactivation, switching, and fallback operation

  - Note: some of them to be refined

- Model monitoring

- Model update

  - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training.

- Model transfer

- UE capability

Note: Some aspects in the list may not have specification impact.

Note: Aspects with square brackets are tentative and pending terminology definition.

Note: More aspects may be added as study progresses.

Agreement

The following is an initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML

- Performance

  - Intermediate KPIs

  - Link and system level performance

  - Generalization performance

- Over-the-air Overhead

  - Overhead of assistance information

  - Overhead of data collection

  - Overhead of model delivery/transfer

  - Overhead of other AI/ML-related signaling

- Inference complexity

  - Computational complexity of model inference: FLOPs

  - Computational complexity for pre- and post-processing

  - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte)

- Training complexity

- LCM related complexity and storage overhead

  - FFS: specific aspects

- FFS: Latency, e.g., Inference latency

Note: Other aspects may be added in the future, e.g. training related KPIsNote: Use-case specific KPIs may be additionally considered for the given use-case.

Working Assumption

| Terminology | Description |
| --- | --- |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |

Note: It is encouraged for the 3gpp discussion to proceed without waiting for online/offline training terminologies.

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

| Terminology | Description |
| --- | --- |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |

Note:

Companies are encouraged to bring discussions on various options and their views on how to define Level y/z boundary in the next RAN1 meeting.

Agreements in RAN1#110bis:

Working Assumption

- Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not.

- Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z

- Clarifying note: Level y includes cases without model delivery.

Agreement

Clarify Level x/y boundary as:

- Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE.
(Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.)

Agreement

Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models

FFS: Detailed discussion of model ID with associated information and/or model functionality.

FFS: usage of model ID with associated information and/or model functionality based LCM procedure

FFS: whether support of model ID

FFS: the detailed applicable AI/ML operations

Agreement

For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE

  - Event-triggered as configured by the network, UE’s decision is reported to network

  - UE-autonomous, UE’s decision is reported to the network

  - UE-autonomous, UE’s decision is not reported to the network

FFS: for network sided models

FFS: other mechanisms

Conclusion

Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact.

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models.

Note: User data privacy needs to be preserved. The provision of assistance information may need to consider feasibility of disclosing proprietary information to the other side.

Agreement

Study the specification impact to support multiple AI models for the same functionality, at least including the following aspects:

-	Procedure and assistance signaling for the AI model switching and/or selection

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training).

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case:

    - Monitoring based on inference accuracy, including metrics related to intermediate KPIs

    - Monitoring based on system performance, including metrics related to system peformance KPIs

    - Other monitoring solutions, at least following 2 options.

      - Monitoring based on data distribution

        - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc.

        - Output-based: e.g., drift detection of output data

      - Monitoring based on applicable condition

Note: Model monitoring metric calculation may be done at NW or UE

Agreement

Study performance monitoring approaches, considering the following model monitoring KPIs as general guidance

    - Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance)

    - Overhead (e.g., signaling overhead associated with model monitoring)

    - Complexity (e.g., computation and memory cost for model monitoring)

    - Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring)

    - FFS: Power consumption

    - Other KPIs are not precluded.

Note: Relevant KPIs may vary across different model monitoring approaches.

FFS: Discussion of KPIs for other LCM procedures

Agreement

Study various approaches for achieving good performance across different scenarios/configurations/sites, including

    - Model generalization, i.e., using one model that is generalizable to different scenarios/configurations/sites

    - Model switching, i.e., switching among a group of models where each model is for a particular scenario/configuration/site

      - [Models in a group of models may have varying model structures, share a common model structure, or partially share a common sub-structure. Models in a group of models may have different input/output format and/or different pre-/post-processing.]

    - Model update, i.e., using one model whose parameters are flexibly updated as the scenario/configuration/site that the device experiences changes over time. Fine-tuning is one example.

Agreement

The following are additionally considered for the initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML

- Clarification on inference complexity

  - Note: Inference complexity includes complexity for pre- and post-processing.

- LCM related complexity and storage overhead

  - Storage/computation for training data collection.

  - Storage/computation for training and model update

  - Storage/computation for model monitoring.

  - Storage/computation for other LCM procedures, e.g., model activation, deactivation, selection, switching, fallback operation.

- FFS: Power consumption, latency (e.g., Inference latency)

Conclusion

This RAN1 study considers ML TOP/FLOP/MACs as KPIs for computational complexity for inference. However, there may be a disconnection between actual complexity and the complexity evaluated using these KPIs due to the platform- dependency and implementation (hardware and software) optimization solutions, which are out of the scope of 3GPP.

Agreements in RAN1#111:

Agreement

For UE-part/UE-side models, study the following mechanisms for LCM procedures:

- For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality

  - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality.

  - FFS: Whether or how to indicate Funtionality

- For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs

Working Assumption

Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion,

| Proprietary-format models | ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format |
| --- | --- |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspecive |

From RAN1 discussion viewpoint, RAN1 may assume that:

- Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared.

- Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared

Working Assumption

| Terminology | Description |
| --- | --- |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |

| Terminology | Description |
| --- | --- |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |

Note: whether and how to indicate Functionality will be discussed separately.

Working Assumption

| Terminology | Description |
| --- | --- |
| Model update | Process of updating the model parameters and/or model structure of a model |
| Model parameter update | Process of updating the model parameters of a model |