3GPP TSG RAN WG1 #112			R1-2301159

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.11.4

Source:	InterDigital, Inc.

Title:	Improved GNSS operations for IoT NTN

Document for:	Discussion and Decision

# Introduction

Non-terrestrial networks are characterized by large moving cells and reduced effectiveness of measurement-based procedures due to reduced signal-strength variation between cell center and cell edge. Rel-17 has partly addressed these issues by incorporating UE location into various procedures such as time pre-compensation, CHO, and measurement reporting. Since other RAT-dependent positioning methods are not currently supported, maintaining updated and accurate GNSS information is essential in NTN. It is assumed that all Rel-17/18 NTN capable devices are GNSS capable.

An objective for Rel-18 IoT NTN is to continue performance enhancements to address remaining issues from Rel-17. One of these remaining issues is improved GNSS operation [1]:

4.1.1 IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17 This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2] - Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1]

RAN1 has made the following relevant agreements in RAN1#110 regarding enhanced GNSS operation for IoT NTN devices [2]:

Agreement GNSS assistance information that UE reports to eNB at least consists of: - GNSS position fix time duration for measurement - GNSS validity duration Agreement When eNB triggers UE to make GNSS measurements, UE re-acquires GNSS position fix - FFS details of signalling - FFS how UE reports GNSS assistance information after eNB trigger and the detailed content - Note: further discuss whether a UE is expected to handle all eNB triggers

In RAN1#110bis-e the following relevant agreements were made [3]:

Agreement Support eNB to at least aperiodically trigger UE to make GNSS measurement. Agreement If eNB aperiodically triggers UE to make GNSS measurement, a MAC CE is used. Agreement UE reports GNSS position fix time duration for measurement at least during the initial access stage - which message carries this information is up to RAN2 Agreement In connected mode, UE may report GNSS validation duration with MAC CE.

And in RAN1#111 the following relevant agreements were made [4]:

Agreement For GNSS measurement in RRC connected, if eNB aperiodically triggers connected UE to make GNSS measurement, UE can re-acquire GNSS position fix with a gap - FFS details of gap configuration The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement - FFS based on configured timing

This document continues the discussion on improved GNSS operations. Specifically, signaling details for GNSS assistance information to support measurement gap configuration for GNSS acquisition, as well as a method to prohibit excessive GNSS acquisition and reporting.

# GNSS acquisition and reporting in IoT NTN

Acquisition of GNSS is a time and power intensive process, which is a particular issue for devices where low power consumption is a key concern. Furthermore, a subset of these devices (e.g. NB-IoT) may not capable of simultaneously supporting GNSS acquisition and data transmission/reception, so additional consideration must be given to trade off throughput vs. updated GNSS information.

In Release 17 it was assumed that NTN IoT operation would be limited to short and sporadic traffic. Few enhancements were therefore needed/adopted to reduce GNSS acquisition time since it was unlikely that a scheduled transmission/reception would overlap with GNSS acquisition. Release 18 IoT NTN removes the assumption of short-sporadic traffic, and although throughput is increased, so is the probability of collision between the GNSS acquisition window and UL transmission/DL receptions as shown below:

Figure 1: Rel-18 IoT NTN devices supporting longer & more frequent traffic can impact GNSS acquisition.

## GNSS assistance information

To support higher traffic environments, RAN1 has agreed to introduce assistance information to facilitate measurement gap configuration for GNSS acquisition. Signalling details, how information is reported, and the detailed content of the report are currently listed as for further study, however GNSS assistance information includes at least the following:

- GNSS position fix time duration for measurement

- GNSS validity duration

In RAN1#110bis-e it was agreed that UE reports GNSS position fix time duration for measurement at least in initial access stage. The reporting framework may be like TA report during random access, where system information enables/disables reporting and indicates which message the GNSS position fix is included in (e.g. Msg3 or Msg5). However, as agreed in previous RAN1 meeting, signalling details can be confirmed/decided in RAN2.

In connected mode, a UE may report GNSS validation duration via MAC CE. However, details of whether this re-uses/re-purposes/extends an existing MAC CE or a new MAC CE is defined is unclear. Since the GNSS assistance information may eventually support additional information other than the validity duration, and the information contained within may change independently requiring a unique set of report triggering conditions, the simplest and most future proof-way way may be to define a new GNSS acquisition assistance information MAC CE and corresponding reporting framework.

Proposal 1:	Introduce a new ‘GNSS acquisition assistance information MAC CE’ to report GNSS validity duration in connected mode.

Currently only GNSS validity duration may be reported in connected mode, meaning that if the GNSS position fix changes at any time after initial access there is no way to update the value until a subsequent initial access attempt. However, the GNSS position fix time duration may also change dynamically based on UE characteristics like UE surroundings (e.g., an indoor UE or a UE surrounded by tall trees/buildings may take longer to acquire GNSS than a UE in open space).

Observation 1:	Both validity duration and position fix time duration can vary dynamically based on UE characteristics (e.g. surroundings, UE speed).

If the position fix time duration becomes outdated, then the currently configured measurement gap may become too short to acquire GNSS and the UE will lose synchronization and connection. Alternatively, if the measurement gap is too long then resources may be wasted. Either way, it is important to maintain an updated position fix time duration.

To support connected mode reporting, the UE may optionally include GNSS position fix duration within the GNSS acquisition assistance information MAC CE along with validity duration. To avoid unnecessary signaling overhead, the UE may only include the GNSS position fix time duration, for example, if it has changed since a previous report or has been explicitly requested by the network.

Proposal 2:	In connected mode, UE may also optionally report GNSS position fix time duration via the new GNSS acquisition assistance information MAC CE (e.g. if the current value has changed since the previous report during initial access).

Referring to RAN1 agreements, the following is captured regarding GNSS assistance information reporting:

- FFS how UE reports GNSS assistance information after eNB trigger and the detailed content

Based on the underlined portion of the agreement, it seems that GNSS assistance information is at least reported based on network trigger (e.g., upon reception of NW request). Although, as previously described, the GNSS assistance information may vary dynamically based on the UE circumstances, and the network may not be aware of when the UE circumstances changes. It may therefore be necessary to support additional triggering conditions for GNSS assistance information reporting which are evaluated by the UE. For example, the UE may trigger GNSS assistance information if the UE detects that the GNSS validity duration changes by a configured threshold (e.g., due to a change in UE speed).

Proposal 3:	In connected mode, GNSS assistance information MAC CE is at least sent upon NW request. FFS other reporting trigger conditions.

## UE autonomous GNSS acquisition

In RAN1#111, the following agreement was made regarding UE re-acquiring GNSS autonomously:

Agreement

The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement

- FFS based on configured timing

Configuring UE-autonomous GNSS re-acquisition allows UE to maintain synchronization in situations where the network hasn’t or is unable to send an aperiodic trigger. When based on configured timing, the UE and gNB are aligned on when the GNSS measurement occurs, and the timing can be configured to consider aspects like validity duration.

However, it is well known the vast majority of NB-IoT UEs commercialized are based on CP solution only and RRC reconfiguration is NOT applicable for those UEs. This means that the configured timing must be decided at the initial RRC connection setup only and not adaptable over time for most NB-IoT UEs. If the timing for autonomous GNSS acquisition is configured via RRC, it cannot be updated to accommodate a change in validity duration during connection which can jeopardize UE synchronization if validity duration becomes less than the configured timing.

Observation 2:	NB-IoT UEs with CP solution only do not support RRC reconfiguration. If UE is configured via RRC with timing to autonomously re-acquire GNSS, the timing cannot be reconfigured to accommodate changes in e.g., validity duration.

Although this is not an issue for eMTC devices, RAN1 should discuss whether autonomous GNSS acquisition based on configured timing can also be supported for NB-IoT UEs which support CP solution (i.e., whether it is acceptable that a configured timing can be configured without subsequent adjustment).

Proposal 4:	A UE may re-acquire GNSS autonomously (when configured by the network) based on configured timing for at least eMTC UEs. FFS for NB-IoT UEs.

# Conclusion

In this contribution the following observations and proposals were made concerning enhanced GNSS operation for IoT NTN devices:

Observation 1:	Both validity duration and position fix time duration can vary dynamically based on UE characteristics (e.g. surroundings, UE speed).

Observation 2:	NB-IoT UEs with CP solution only do not support RRC reconfiguration. If UE is configured via RRC with timing to autonomously re-acquire GNSS, the timing cannot be reconfigured to accommodate changes in e.g., validity duration.

Proposal 1:	Introduce a new ‘GNSS acquisition assistance information MAC CE’ to report GNSS validity duration in connected mode.

Proposal 2:	In connected mode, UE may also optionally report GNSS position fix time duration via the new GNSS acquisition assistance information MAC CE (e.g. if the current value has changed since the previous report during initial access).

Proposal 3:	In connected mode, GNSS assistance information MAC CE is at least sent upon NW request. FFS other reporting trigger conditions.

Proposal 4:	A UE may re-acquire GNSS autonomously (when configured by the network) based on configured timing for at least eMTC UEs. FFS for NB-IoT UEs.

# References

R2-221806 – WID on IoT NTN enhancements – MediaTek Inc.

RAN1#110 chairman’s notes

RAN1#110bis-e chairman’s notes

RAN1#111 chairman’s notes