3GPP TSG RAN WG1 #112	R1-2301196

Athens, Greece, Feb 27 - Mar 3, 2023

Agenda item:		9.13.3

Source:	Nordic Semiconductor ASA

Title:					On LP-WUS signal design

Document for:		Discussion and Decision

# Introduction

In RAN#97, study item in RP-222644 has been approved. In this contribution we focus on

Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1]

# Discussion

## Signal design

We have had the following agreement in Toulouse RAN1#111

Agreement - Study generation and link performance of multi-carrier (MC)-ASK (including OOK) waveform - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS. - Note that above does not preclude DFT-S-OFDMA - Study generation and link performance of multi-carrier (MC)-FSK waveforms - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS. - Study link performance of OFDMA-based signals/channels considering at least the existing signal/channel structure (e.g. CSI-RS, SSS) - Other signal/channel structures are not precluded - For next meeting, companies to provide input on aspects to consider that might impact link performance

M increases the bit rate, but at the same time reduces the amount of energy per bit. For DFT-S-OFDMA, with large M the OFDM CP starts to impact reception. The CP in NR is given by the following equation

Symbols 0 and 7 have longer CP than the other symbols. When M=14, symbol length becomes comparable with normal CP. This means that 14th symbol is repeated in front of the 1st symbol as CP. With large M, special handling is required at the receiver to handle CP. The issue with CP can be avoided by increasing SCS of LP-WUS, such CP is added to each LP-WUS symbol. On the other hand, transmitter must be capable of operation with mixed numerology. Given that objective is low power WUS, our preference would be to increase M by increasing SCS for LP-WUS or if transform precoding is deemed as better solution, keeping M low. This being beneficial for LP-WUS receiver.

Proposal-1: M can be increased by increasing SCS of LP-WUS to be higher than SCS of hosting BWP, as the first preference. Alternatively, transform precoding can be used, but M should be kept low, such M<=4.

In the last meeting a need for synch signal for LP-WUS (LP-SS) has been discussed, and the majority of companies seen a clear benefit form introducing such a signal. In our opinion, the main motivation for synch signal is to enable detection of serving cell presence, as further discussed in RRM section. Another motivation of LP-SS would be to track and correct timing drift due to ring oscillator. Otherwise, UE would need to wake up much prior to LP-WUS MO because timing may have drifted a lot.

LP-SS would be a signal which is known, cell-specific and always present. Whether it contains information (analogically to PBCH) or not, is to be further studied.

Observation-1: Synchronization signal for LP-WUS is beneficial to ensure that LP-WUS can hear the camping cell and to correct timing drift. Such signal should be known to UE, cell-specific and always present.

Synchronization signal if transmitted with sufficient frequency, could be then used for AGC as well as to indicate parameters of LP-WUS signal (similarly to PBCH), such as whether only sub-group indication is transmitted or also UE-IDs are included in the LP-WUS. In addition, frame/subframe number could be transmitted such that LP-WUR can keep track of time. Alternatively, a preamble could take these functionalities, which would be sent on demand, and before the LP-WUS.

Proposal-2: Study a need for an additional on-demand signal (e.g. preamble) and what would be its functionality.

In the last meeting companies discussed also what should be content of LP-WUS. The majority view has been that UE ID (39 bits per ID in Idle and 24bits in Inactive mode) would be infeasible to support with broadcast due to low sensitivity of LP-WUS. And, majority seemed to prefer subgroup indication, used e.g. in PEI. On the other hand, it could be feasible to transmit small number of UE IDs to users closer to cell as a separate information.

Observation-2: When determining baseline information size for LP-WUS, for the purpose of simulations, candidate values could be 1,2,4,8,16 bits, these corresponding to number of sub-groups indicated in the LP-WUS.

Observation-3: When determining additional information size for LP-WUS, for the purpose of simulations, candidate values could be N*39 or N*24, where 39 corresponds to bit-size of NR UE ID in Idle, 24 corresponds to bit-size of NR UE ID in Inactive and N is max number of UE IDs indicated by the LP-WUS.

## RRM measurement relaxations

Agreement For a UE support LP-WUR in IDLE/INACTIVE mode, - Study how to reduce UE power consumption due to existing RRM measurement requirements at least for mobility support, - study feasibility of RRM measurements performed by LP-WUR, at least for serving/camping cell, based on signals detected by LP-WUR - FFS: measurement metric - FFS: whether and how to identify cell/ tracking area - FFS: need for neighbouring cells - FFS: need for relaxation of existing RRM measurement requirements (for UE)

R17 supports RRM measurement relaxations of up to 6-fold. Normally, a UE shall perform neighbor cell-search every DRX, or eDRX cycle, but with RRM measurements relaxations defined in R17, if e.g. static criterion is met, UE can perform such only every 6th DRX/eDRX cycle instead. However, in LTE LPWA, RRM measurements are relaxed even further, up to once a 24h. In (industrial) IoT or measuring type of IoT, sensors are often placed on static object. There is almost 100% probability that after wake-up, UE finds the same cell.

Obsevation-4: For LP-WUS and MR overall power consumption reduction, there is a need to further relax RRM measurements.

Regarding measurement metric, it would not be feasible for LR to perform RSRP/RSRQ serving cell measurements, which are now used as criteria for relaxing RRM measurements. However, it will be feasible for LR to determine whether serving cell signal is present or not. In other words, if LR can detect presence of e.g. LP-SS of the cell, it will hear also the corresponding LP-WUS. If UE would not hear LP-SS it would wake-up MR instead. MR then can perform RRM and re-selection.  To summarize, the detection of presence of LP-SS, would serve as a criteria for neighbor cell relaxation.

Observation-5: It would be feasible for LR to detect presence of LP-SS, which could be a criteria for RRM measurement relaxation.

The remaining open question is how to identify cell/tracking area. In our opinion, LR should not try to identify any cell or tracking area, per se. It would be MR which performs re-selection when needed. Upon selecting a camping cell, MR would check whether cell supports LP-WUS. If cell supports LP-WUS, MR would provide necessary configurations to LR, and MR would go to MICO-like sleep.

Proposal-3: Identification of cell and tracking areas is left to MR. If MR decides to camp on cell supporting LP-WUS, it provides configuration information to LR, and go to MICO-like sleep.

## LP-WUS bandwidth

Agreement For the purpose of study, the BW of one LP-WUS is not greater than X (FFS X is 5 or 20) MHz for FR1, study further - whether BW of LP-WUS is configurable (implicitly or explicitly) - size of guard band [FFS: within or outside of BW X], if any - whether there is different X for Idle, Connected, Inactive modes FFS: Whether FR2 is included in the scope of LP-WUS SI

In our opinion, BW of LP-WUS should not be configurable. It would increase complexity of LP-WUR. However, if FR2 is supported, BW for FR1 and FR2 could be different, since separate HW is needed anyway. And, we clearly do not see need to differentiate BW based on RRC modes.

Observation-6: Configurability of LP-WUS BW would increase LP-WUR complexity. Target a single BW size.  Different BW for FR2, if supported, could be considered.

When it comes to zero-IF architectures using ring-oscillator, its precision could be somewhere at 200-300ppm. This creates uncertainty of LP-WUS location in baseband. At 4GHz this uncertainty already translates to 0.8MHz, while at 1GHz this is 0.2MHz. This being one another factor determining the LP-WUS BW. Design must take into account that LP-WUS receiver may receive only part of LP-WUS due to frequency shift.

Observation-7: LP-WUS BW requirement should take into account a frequency error of the receiver.

## Higher layer aspects

Some UEs in the network will NOT be able to benefit from LP-WUS, for example users on the cell edge (LP-WUS poor sensitivity), users with moderate or high mobility. In addition, LP-WUS will not be supported by all R18 UEs. Therefore, we think that a UE in favorable conditions for LP-WUS. e.g. smartwatch sitting on the table overnight, would sub-scribe for LP-WUS monitoring in a particular cell.

Observation-8: A UE should subscribe for reception of LP-WUS in a camping cell.

After receiving LP-WUS trigger, there are 3 possible procedures for waking up MR

Direct PRACH transmission: This makes sense only if a UE is wakened up with UE ID in LP-WUS and FAR is kept low. This would decrease the latency of wake-up time.

Paging monitoring: procedures may be similar to those of PEI to PO, except that gap between PO and LP-WUS must include large wake-up time of MR from MICO deep sleep. PO and LP-WUS would be often in different DRX cycle.

PEI monitoring: we hardly see any benefit from using PEI in between LP-WUS and PO. Adding additional monitoring  in-between to procedure will increases MDR, latency and degree of synchronization needed for PEI is similar to that of PO.

Therefore, RAN1 should study transition to paging monitoring as baseline. And if UE ID signaling is supported in LP-WUS, at least as optional feature of LP-WUS, RAN1 should study transition to directly sending PRACH.

Proposal-4: After MR is waken-up by LR, study the transition of MR from MICO deep sleep to

- PO monitoring

- transmitting directly PRACH, if including an UE ID in LP-WUS is deemed feasible.

# Conclusions

In this contribution we discussed issues related to LP-WUS signal design and had observations and proposals:

Proposal-1: M can be increased by increasing SCS of LP-WUS to be higher than SCS of hosting BWP, as the first preference. Alternatively, transform precoding can be used, but M should be kept low, such M<=4.

Observation-1: Synchronization signal for LP-WUS is beneficial to ensure that LP-WUS can hear the camping cell and to correct timing drift. Such signal should be known to UE, cell-specific and always present.

Proposal-2: Study a need for an additional on-demand signal (e.g. preamble) and what would be its functionality.

Observation-2: When determining baseline information size for LP-WUS, for the purpose of simulations, candidate values could be 1,2,4,8,16 bits, these corresponding to number of sub-groups indicated in the LP-WUS.

Observation-3: When determining additional information size for LP-WUS, for the purpose of simulations, candidate values could be N*39 or N*24, where 39 corresponds to bit-size of NR UE ID in Idle, 24 corresponds to bit-size of NR UE ID in Inactive and N is max number of UE IDs indicated by the LP-WUS.

Obsevation-4: For LP-WUS and MR overall power consumption reduction, there is a need to further relax RRM measurements.

Observation-5: It would be feasible for LR to detect presence of LP-SS, which could be a criteria for RRM measurement relaxation.

Proposal-3: Identification of cell and tracking areas is left to MR. If MR decides to camp on cell supporting LP-WUS, it provides configuration information to LR, and go to MICO-like sleep.

Observation-6: Configurability of LP-WUS BW would increase LP-WUR complexity. Target a single BW size.  Different BW for FR2, if supported, could be considered.

Observation-7: LP-WUS BW requirement should take into account a frequency error of the receiver.

Observation-8: A UE should subscribe for reception of LP-WUS in a camping cell.

Proposal-4: After MR is waken-up by LR, study the transition of MR from MICO deep sleep to

- PO monitoring

- transmitting directly PRACH, if including an UE ID in LP-WUS is deemed feasible.