3GPP TSG RAN WG1 #112	    			            R1-2301267

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.4.3

Source: 	Samsung

Title: 	On Enhanced SL Operation in FR2

Document for:	Discussion and Decision

# Introduction

In RAN#98-e it was agreed for RAN1 to continue studying enhancements for SL FR2 in Rel-18. The WID has been updated to reflect that [1].

- Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2]

- Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1]

- Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2]

  - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

In this contribution we present Samsung’s views on beam management design for SL operation on FR2 licensed spectrum.

# Discussion

## Initial beam acquisition during link establishment

On the Uu interface, cell selection/identification is the first process a UE performs. Additionally, in FR 2, beam pairing is done before communication between a UE and a gNB can start. Hence, in FR2 on the Uu interface, initial beam acquisition happens with cell selection/identification. The identity of the node (e.g., Physical Cell Identity (PCI)) is included in the beam sweeping RS, i.e., the PSS/SSS includes the PCI. In side link, the UE identity is included in the second stage SCI (source ID).

Now we will see how this process can be applied to the SL (PC5) interface during or before link establishment. Figure 1 illustrates the Layer-2 link establishment procedure for unicast mode of V2X communication over PC5 reference point (figure 6.3.3.1-1 of TS 23.387). Part of the link establishment procedure is to send a direct communication request from the source UE to the destination UE. A first scenario for initial beam pairing is when the source UE (UE-1) knows the beam pair to use for communicating with the destination UE (UE-2) before link establishment, the direct communication request can be sent directly using that beam pair. A second scenario for initial beam pairing is when the beam pair for communication between the source UE (UE-1) and the destination UE (UE-2) has not been previously identified, it is then identified during the link establishment procedure.

Figure 1: Layer-2 link establishment procedure.

For scenario 1, as illustrated in Figure 2, to established a link , the destination UE can send a beam identification reference signal on different beams, this would help a source UE to identify the beam to use when establishing the link (or sending the direct communication request (DCR)) to the destination UE. This procedure is analogous, on the Uu interface, to the gNB transmitting SSBs that identify beams and include the PCI, and the UE measuring the SSBs to determine a beam pair for initial communication. However, in SL (PC5) interface, the S-SSB doesn’t included the UE identity, it only includes an index that can identify a beam. Therefore, we can consider the case where the beam identifier (BI) includes both the UE identity and a beam identity, and the where BI only includes the beam identity.

Observation 1: For enhanced SL operation on FR2 licensed spectrum, an initial beam acquisition/identification signal can include:

- a UE identity and beam identity to identify the UE and the beam, or

- a beam identity to identify the beam.

Figure 2: Beam Identification before link establishment.

For scenario 2, as illustrated in Figure 3, the beam pair is not yet identified at the time of link establishment. In this case, the direct communication request (DCR) can be sent on multiple beams from UE-1 to UE-2. UE-2 receives the DCRs from the multiple beams and responds to a detected DCR indicating the preferred beam for further communication between the two UEs. In this scenario, the DCR can include a UE identity as well as a beam identity.

Figure 3: Beam Identification during link establishment.

Observation 2: On Enhanced SL operation on FR2 licensed spectrum, the following options are possible for initial beam pairing

- Beam pairing during link establishment procedure

- Beam pairing before link establishment procedure

Proposal 1: For enhanced SL operation on FR2 licensed spectrum, study beam pairing before sidelink establishment procedure.

Proposal 2: For enhanced SL operation on FR2 licensed spectrum, study beam pairing during sidelink establishment procedure.

One aspect of beam identification during initial beam-pairing is how the UEs indicate to each other the best beam-pair to use. This can be based on implicit or explicit signalling. In the Uu interface, both implicit and explicit signalling of beams are used. An example of implicit signalling is the association between SSBs and ROs, the UE transmits the preamble in the RO associated with a preferred SSB (which represents a beam). Explicit signalling is when a beam ID (e.g., TCI state or reference signal ID) is used to signal a preferred beam. For beam indication during link establishment between a pair of UEs, explicit and implicit beam indication can be further studied.

Proposal 3: On Enhanced SL operation on FR2 licensed spectrum, study beam indication mechanisms during initial beam pairing.

## Beam maintenance

After link establishment and initial beam acquisition, the UE needs to maintain the beam-pair used for communication between two UEs. As the UEs of a unicat link move around or rotate a different beam pair can be used. The UEs may also perform beam refinement, where a wide beam is found during the initial acquisition, this beam is subsequently refined to have a narrow beam.

On the Uu interface there are 3 types of beam management procedures between two devices as described in TR 38.802 clause 6.1.6.1. In each procedure one or both devices perform beam sweeping to identify or refine a beam pair. Procedure P-1 is when both devices perform beam sweeping, generally used for initial beam acquisition, to acquire a coarse beam, Procedures P-2 and P-3 are used for beam refinement and beam maintenance of the transmit and receive beams.

Proposal 4: For enhanced SL operation on FR2 licensed spectrum, consider legacy P-2 or P-3 procedures for sidelink beam maintenance.

P-2 and P-3 beam management procedure on the Uu interface involve the following steps:

- Reference signals are transmitted either on different beams (for P-2) or the same beam (for P-3) from device-A (gNB or UE).

- The reference signals are measured by Device-B (UE or gNB). In P-2, the same the receive beam is used, in P-3 a different receive beam is used.

- Depending on P-2 or P-3

  - If P-2, depending on the device:

    - If Device-A is a gNB and Device-B is a UE, device-B reports to device-A the beam measurement results.

    - If Device-A is a UE and Device-B is a gNB, device-B does not share measurement results to Device-A

  - If P-3, a refined receive beam is determined by Device-B.

- If P-2, a beam-pair is determined by the gNB and signalled to the UE.

A simplified example of the P-2 procedure, is illustrated in Figure 4.

Figure 4: Example of beam management procedure on the Uu Interface.

A similar procedure can be designed for the SL (PC5) interface. Here Device-A and Device-B are UE-1 and UE-2 of a unicast link. The first aspect to consider is the reference signals to be used for beam measurement including aspects such as how and when these reference signals are triggered, reference signal structure for example, whether the reference signals are transmitted in the same slot or in different slots when used for different beam, as well as how to support RS repetition. We also have to consider timing aspects, whether these reference signals are periodic or semi-persistent or aperiodic.

The next aspect to consider is the measurement report including the configuration, and triggering of the measurement report, the container of the measurement report, and the timing of the measurement report.

The next aspect to consider is beam indication, one UE indicates the beam to other UE. A beam indicator (e.g., similar to a TCI state on the Uu interface) can be defined to indicate a beam from one UE to the other. The beam indicator can refer to one separate beam pair (from example from UE-1 to UE-2 or UE-2 to UE-1), or to a joint beam pair for example in case of beam correspondence, one beam-pair can be indicated for communication from UE-1 to UE-2 and from UE-2 to UE-1. One other aspect to consider is which UE (or both UEs) makes the decision on beam pair selection. We also have to consider the channel used to convey the beam indicator, this can be based on MAC CE signalling or L1 control signalling.

Finally, the receiving UE would also need to update its receive beam by doing receive beam sweeping (similar to P-3 procedure). Since beam measurement and update is done at the same device, no reporting or update command is needed. However, a UE may request the other UE to transmit a reference signal with repetition to refine its receive beam.

Proposal 5: For enhanced SL operation on FR2 licensed spectrum, study the design of the reference signal used for beam measurement and reporting including aspects such as:

- Configuration, triggering and signaling

- Signal structure with and without repetition

- Timing

Proposal 6: For enhanced SL operation on FR2 licensed spectrum, study the design, content and timing of beam measurement reports.

Proposal 7: For enhanced SL operation on FR2 licensed spectrum, study beam indication mechanisms including aspects such:

- Design of beam indicators

- Joint vs separate beam indication

- Configuration and signal of beam indicators

- Which UE can perform as a controller to determine a beam pair to use and associated signalling.

Proposal 8: For enhanced SL operation on FR2 licensed spectrum, study signaling and procedures for receive beam refinement including:

- Requesting transmission of reference signal with beam repetition

Another important aspect of beam-based operation is power control. On the Uu interface, power control parameters and pathloss (PL) RS are determined based on the beam used. The power control parameters and PL-RS are associated with the UL or Joint TCI state. In SL only open-loop power control is supported. Open loop power control can be DL-based or SL-based. Beam-based operation can be introduced to both DL-based and SL-based power control for PC5 interface.

Proposal 9: For enhanced SL operation on FR2 licensed spectrum, study beam-based open-loop power control operation.

SL has two modes of operation, mode 1, where resource allocation is performed by the network, and mode 2 where resource allocation can be performed by the UE. For mode 1, beam management can be performed by the network or by the UE, or a combination of both. For example, which entity decides when to transmit reference signals for beam measurements, and which entity decides on the beams to use. These operations can be done by the gNB or by a SL UE. There are pro-cons associated with each. For example, having beam decisions taken by the network, can allow the network to schedule overlapping UEs that have non-interfering spatial beams, however it increases beam indication and application latency. This topic can be further studied

Proposal 10: For enhanced SL operation on FR2 licensed spectrum for mode 1 study network-based beam management.

Another area of study mentioned in the WID is “beam failure recovery”. Beam recovery depends on the overall beam management framework including, measurement reference signals and beam indication design. We suggest to defer this topic until more progress is made on SL beam management framework.

Proposal 11: For enhanced SL operation on FR2 licensed spectrum, defer work on beam failure recovery until more progress is made on the SL beam management framework.

# Conclusions

The following observations and proposals have been made regarding SL operation on FR2 licensed spectrum

Observation 1: For enhanced SL operation on FR2 licensed spectrum, an initial beam acquisition/identification signal can include:

- a UE identity and beam identity to identify the UE and the beam, or

- a beam identity to identify the beam.

Observation 2: On Enhanced SL operation on FR2 licensed spectrum, the following options are possible for initial beam pairing

- Beam pairing during link establishment procedure

- Beam pairing before link establishment procedure

Proposal 1: For enhanced SL operation on FR2 licensed spectrum, study beam pairing before sidelink establishment procedure.

Proposal 2: For enhanced SL operation on FR2 licensed spectrum, study beam pairing during sidelink establishment procedure.

Proposal 3: On Enhanced SL operation on FR2 licensed spectrum, study beam indication mechanisms during initial beam pairing.

Proposal 4: For enhanced SL operation on FR2 licensed spectrum, consider legacy P-2 or P-3 procedures for sidelink beam maintenance.

Proposal 5: For enhanced SL operation on FR2 licensed spectrum, study the design of the reference signal used for beam measurement and reporting including aspects such as:

- Configuration, triggering and signaling

- Signal structure with and without repetition

- Timing

Proposal 6: For enhanced SL operation on FR2 licensed spectrum, study the design, content and timing of beam measurement reports.

Proposal 7: For enhanced SL operation on FR2 licensed spectrum, study beam indication mechanisms including aspects such:

- Design of beam indicators

- Joint vs separate beam indication

- Configuration and signal of beam indicators

- Which UE can perform as a controller to determine a beam pair to use and associated signalling.

Proposal 8: For enhanced SL operation on FR2 licensed spectrum, study signaling and procedures for receive beam refinement including:

- Requesting transmission of reference signal with beam repetition

Proposal 9: For enhanced SL operation on FR2 licensed spectrum, study beam-based open-loop power control operation.

Proposal 10: For enhanced SL operation on FR2 licensed spectrum for mode 1 study network-based beam management.

Proposal 11: For enhanced SL operation on FR2 licensed spectrum, defer work on beam failure recovery until more progress is made on the SL beam management framework.

# References

- RP-222806, “WID revision: NR Sidelink Evolution”, 3GPP RAN#98e, e-Meeting, December, 2022.