3GPP TSG RAN WG1 #112	R1- 2301285

# Athens, Greece, February 27th - March 3rd, 2023

Agenda Item:		9.11.3

Source:				Samsung

Title:					Disabling of HARQ feedback for IoT NTN

Document for:	Discussion

# Introduction

A WID on further enhancements for NB-IoT/eMTC in non-terrestrial networks (NTN) was approved in RAN#92-e [1]. One objective is to specify disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1, RAN2].

In RAN1#111 [2], the following was agreed:

Working assumption For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission: - Support Option 1 by default, and support Option 3 to override default configuration for corresponding transmission - Additional RRC signaling to enable Option 3 - If the bitmap for option 1 is not present and if option 3 is configured then the DCI directly indicates HARQ enable/disable. Option 3 can also be configured when the bitmap for option 1 is configured. - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ enabled and disabled processes or only applied to semi-statically HARQ disabled processes or only applied to semi-statically HARQ enabled processes. - FFS #2: whether/how to support Option 3 overriding default configuration for corresponding transmission for multiple TBs scheduled by single DCI For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A.

This contribution further discusses aspects of disabling HARQ feedback.

# Discussion

WA on signaling from RAN1#111

The WA on signaling to enable/disable HARQ feedback for downlink transmission refers to Option 1 and Option 3 that were previously defined as follows:

- Option 1: per HARQ process via UE specific RRC signaling

- Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field)

The WA aims at introducing a DCI-based signaling to enable/disable HARQ feedback for a downlink transmission in addition to a default RRC configuration.

According to the WA, the RRC configuration is supported by default, which means that the UE is provided an RRC parameter for enabling/disabling HARQ feedback that can be set to ‘enable’ or ‘disable’ (e.g., bitmap with entries for all HARQ processes). Also according to the WA, the dynamic indication, if present, overrides the setting provided by the RRC parameter. Thus, the bitmap is present because the WA assumes that the RRC configuration (Option 1) is supported by default.

The WA assumes that there is a configuration for the DCI-based indication, which means that whether the DCI format includes the indication of enabling/disabling HARQ feedback is subject to the configuration. The need for this configuration is not clear. A DCI-based indication of enabling/disabling HARQ feedback is a simple feature and an explicit indication by 1-bit is straightforward. Then, if the dynamic indication is present, the DCI format has an additional bit.

Proposal 1: Modify the WA as follows:

Working assumption

For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission:

Option 1: per HARQ process via UE specific RRC signaling

Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field)

- Support Option 1 by default, and support Option 3 to override default configuration for corresponding transmission

  - Additional RRC signaling to enable Option 3

  - If the bitmap for option 1 is not present and if option 3 is configured then the DCI directly indicates HARQ enable/disable. Option 3 can also be configured when the bitmap for option 1 is configured.

  - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ enabled and disabled processes or only applied to semi-statically HARQ disabled processes or only applied to semi-statically HARQ enabled processes.

  - FFS #2: whether/how to support Option 3 overriding default configuration for corresponding transmission for multiple TBs scheduled by single DCI

For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A.

Proposal 2: If a DCI-based indication of enabling/disabling HARQ feedback for downlink transmission is agreed, support new 1-bit field in the DCI for the indication.

Number of HARQ feedbacks to disable

The number of HARQ processes for NB-IoT can be 1 or 2, hence the benefits of disabling HARQ-ACK reports for NB-IoT might be smaller than for eMTC or NR. Whether to allow disabling the HARQ-ACK reports for the case of single HARQ process can be further discussed. Also whether to allow disabling HARQ-ACK reports for all processes when more than one HARQ process is configured can be further discussed. A disadvantage of disabling HARQ-ACK reporting is an inability of the gNB to perform open-loop link adaptation and also having to rely on costly higher layer ARQ for retransmissions and/or higher PDSCH reception reliability while an advantage can be some UE power savings. That can be left to gNB implementation and enabling/disabling all/some of the HARQ-ACK reports for PDSCH receptions would be a gNB choice.

Proposal 3: For IoT NTN, enabling/disabling HARQ feedback for downlink transmission is supported for single HARQ process and for multiple HARQ processes.

Enhancements to the HARQ procedure for SPS PDSCH

One proposed enhancement for disabling of HARQ feedback for downlink transmission is about HARQ-ACK reporting for SPS PDSCH, Rel-17 NR NTN introduced that for each SPS PDSCH, a UE follows the per HARQ process configuration of HARQ-ACK enabled/disabled except for the first SPS PDSCH after activation if it happens to be associated with a HARQ process with disable HARQ-ACK report. For IoT the considered scenarios are not latency sensitive. The gNB can activate at a time where the first SPS PDSCH has a HARQ process with enabled HARQ-ACK report and, even if the gNB does not do that, as long as some HARQ processes have enabled HARQ-ACK reports, there will be HARQ-ACK associated with the activated SPS PDSCH configuration (and hence for the activation itself). Thus, the need to introduce this solution for NTN IoT is unclear.

Proposal 4: The need to introduce enhancements to the HARQ procedure for SPS PDSCH when disabling HARQ feedback in NTN IoT is unclear.

Potential enhancements to assist the network for enabling/disabling HARQ feedback

Another aspect to consider is to report additional information by a UE in order to assist the network in configuring enabling/disabling HARQ-ACK reporting. As discussed during the Rel-17 SI, the additional information can be signaling that a sufficient number of repetitions has already been transmitted, requesting a number of repetition, reporting buffer status for HARQ operation, explicit indication to request enabling/disabling HARQ reports, etc.

Proposal 5: Further discuss the reporting of additional information by the UE.

# Conclusion

This contribution discusses remaining aspects related to timing relationship enhancements. Proposals made in this contribution are summarized below.

Proposal 1: Modify the WA as follows:

Working assumption

For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission:

Option 1: per HARQ process via UE specific RRC signaling

Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field)

- Support Option 1 by default, and support Option 3 to override default configuration for corresponding transmission

  - Additional RRC signaling to enable Option 3

  - If the bitmap for option 1 is not present and if option 3 is configured then the DCI directly indicates HARQ enable/disable. Option 3 can also be configured when the bitmap for option 1 is configured.

  - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ enabled and disabled processes or only applied to semi-statically HARQ disabled processes or only applied to semi-statically HARQ enabled processes.

  - FFS #2: whether/how to support Option 3 overriding default configuration for corresponding transmission for multiple TBs scheduled by single DCI

For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A.

Proposal 2: If a DCI-based indication of enabling/disabling HARQ feedback for downlink transmission is agreed, support new 1-bit field in the DCI for the indication.

Proposal 3: For IoT NTN, enabling/disabling HARQ feedback for downlink transmission is supported for single HARQ process and multiple HARQ processes.

Proposal 4: The need to introduce enhancements to the HARQ procedure for SPS PDSCH when disabling HARQ feedback in NTN IoT is unclear.

Proposal 5: Further discuss the reporting of additional information by the UE.

# References

- RP-220979, WID on IoT NTN enhancements, RAN#95-e, March 2022.

- Chairman’s notes, RAN1#111, November 2022.