3GPP TSG RAN WG1 #112		R1-2301328

Athens, Greece, February 27th – March 3rd, 2023

Source: 	Apple Inc.

Title:                     On Small Data Transmission for Redcap UEs

Agenda item:	8.6

Document for:	Discussion and Decision

# 1 Introduction

This contribution presents our views on the remaining issues related to SDT support for Redcap UE.

# Discussions

In RAN1 111 meeting, the following was agreed for Redcap UE to support SDT [1]:

Agreement Discuss the necessary UE behavior of the following cases in this meeting: - Issue 5.1: RA-SDT without subsequent transmission in BWP without CD-SSB - Issue 5.2: RA-SDT with subsequent transmission in BWP without CD-SSB - Issue 5.3: CG-SDT in BWP without CD-SSB - Issue 5.4: NCD-SSB can be used for CG-SDT Conclusion - No issue is identified for RedCap UEs supporting RA-SDT to support initial (non-subsequent) RA-SDT transmission in a RedCap-specific separate initial BWP without CD-SSB.

In Rel-17 Redcap design, Redcap-specific sperate initial BWP was introduced mainly to provide flexibility to NW to avoid resource fragmentation and impact on eMBB UEs. Depending on UE capability, basic Redcap UE (i.e., UE feature 28-1) always assumes there is NCD-SSB or CD-SSB within Redcap-specific initial DL BWP such that the RF retuning to measure SSB for maintaining TO/FO tracking loop is mitigated and power consumption is minimized. It should be noted that battery life is significantly  important for Redcap devices as clearly captured in the approved WID as one of design goals.

Rel-17 SDT feature enables UE to stay in RRC Inactive state for data communication without requiring an RRC state transmission for small packet, which is a very attractive and useful feature for Redcap devices. Support SDT for Redcap UE should follow the same design guideline as what is used for RRC_CONNECTED mode UE to achieve the same goal of battery life.

For CG-SDT, after the initial transmission of CG-PUSCH, a UE is required to monitor C-RNTI/CS-RNTI in a dedicated search space on the Redcap-specific initial DL BWP for a potential retransmission until the CG timer or CG-SDT timer expires. The maximum duration of CG-SDT timer is 40.96 sec assumption CG-SDT periodicity is 640ms and CG Timer is set to be ‘64’. To meet the transmission timing requirement defined in [2], UE expects to perform SSB-based measurement for TO/FO and compensate the time/frequency drift, which is essentially same as TO/FO measurement performed in RRC_CONNECTED mode for the PDCCH monitoring. To avoid RF retuning for the measurement, NCD-SSB should be provided in the initial DL BWP if network wants to configure basic Redcap UE to operate SDT on it. Of course, network can have a full freedom to restrict Redcap UE with SDT in the initial BWP containing the CD-SSB together with non-Redcap UE, which clearly avoids the NCD-SSB overhead. In the RAN1 111 meeting, the concern on NCD-SSB overhead was raised. It should be noted that additional SSB is caused due to the configuration of SDT on initial DL BWP, which offers scheduling flexibility for network for offloading. If the overhead is indeed concern, gNB always have the full freedom to share CD-SSB for Redcap UEs with SDT as elaborated earlier.

Proposal: For CG-SDT support,

- A Redcap UE supporting mandatory FG 28-1 (but not optional FG 28-1a) expects that the BWP with CG-SDT contains either CD-SSB or NCD-SSB.

- A Redcap UE supporting optional FG 28-1a can be configured with CG-SDT without any SSB.

# 3. Conclusion

In the previous sections, we have discussed how to support SDT for Redcap UEs. Based on the discussions above, the following proposals were made:

Proposal: For CG-SDT support,

- A Redcap UE supporting mandatory FG 28-1 (but not optional FG 28-1a) expects that the BWP with CG-SDT contains either CD-SSB or NCD-SSB.

- A Redcap UE supporting optional FG 28-1a can be configured with CG-SDT without any SSB.

# References

- RAN1 111 e-meeting Chairman notes

- TS 38.133, Requirements for support of radio resource management