3GPP TSG RAN WG1 Meeting #112	                                                       R1-2301332

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.1.3.1

Source:	Apple Inc.

Title:	Views on supporting increased number of orthogonal DMRS ports

Document for:	Discussion/Decision

# Introduction

In RAN1#111, following agreements related to DMRS enhancements in MIMO have been made:

Agreement

For FD-OCC length 4 for PDSCH/PUSCH, select the following:

- Opt.1-1 (Walsh matrix) for PDSCH

- Opt.1-2 (Cyclic shift) for PUSCH

Agreement

- For the antenna ports indication in Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, all of the following port combinations can be indicated:

  - Cat. 1) Legacy port indexes (eType 1: p=0~7, eType 2: p=0~11)

  - Cat. 2) New port indexes (eType 1: p=8~15, eType 2: p=12~23)

  - Cat. 3) Legacy port indexes and New port indexes at least within a CDM group at least for maxLength=1 (eType 1: up to 4 ports from {0, 1, 8, 9} and/or up to 4 ports from {2, 3, 10, 11}, eType 2: up to 4 ports from {0, 1, 12, 13} and/or up to 4 ports from {2, 3, 14, 15} and/or up to 4 ports from {4, 5, 16, 17}) at least for S-TRP case,

    - For up to 4 ranks, only one CDM group is used per UE. For larger than 4 ranks, more than one CDM groups can be used per UE.

- FFS: Whether to increase the size of antenna ports field in DCI format 1_1/1_2, or introduce new DCI field for antenna ports indication, or not.

- FFS: Whether the new antenna port(s) table is specified or not.

- FFS: MU restrictions for certain entries. e.g., DMRS ports = {0,2}, or {8,10}, etc.

- FFS: Cat.3 for M-TRP case.

- Note: DMRS port index for PDSCH is determined by p +1000

Agreement

For > 4 layers PUSCH, support new antenna ports tables for rank = 5,6,7,8 for both single-symbol/double-symbol DMRS.

- For Type 1/Type 2 Rel.15 DMRS ports, new antenna ports tables are the following:

  - The same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused at least for full or non-coherent UL codebook.

- For Rel.18 eType1/eType2 DMRS ports,

  - New antenna ports tables with new DMRS port combinations are used for rank = 5,6,7,8 (FFS: details).

  - Note: Whether the DMRS port combination allows to use single symbol DMRS for rank = 5,6,7,8 should be checked.

- FFS: For partial coherent UL codebook, support layers to DMRS port mapping that layers associated to the same antenna port group are multiplexed into the same DMRS CDM group.

- FFS: One or more than one DMRS port combination(s) for each rank and TPMI

- Note: New DMRS port combinations above does not preclude the new antenna ports tables including the current DMRS port combination(s) for PDSCH for rank = 5,6,7,8 in Rel.15-17.

- FFS: Whether the antenna ports combinations for rank = 5,6,7,8 can be indicated by the reserved entries of existing antenna ports tables for rank =1,2,3,4, if the rank is indicated together with DMRS antenna ports.

Agreement

For length 2 TD-OCC (across consecutive DMRS symbols, if any) for DMRS of PDSCH/PUSCH for Rel.18 eType 1/2 DMRS, support

| TD-OCC index | Wt(0) | Wt(1) |
| --- | --- | --- |
| 0 | +1 | +1 |
| 1 | +1 | -1 |

Agreement

- For full-coherent PUSCH with rank 5-8, UE shall expect only one port PTRS to be configured.

- Down select from the following in RAN1#112:

  - Alt.1: the size of PTRS-DMRS association field is 2bit in DCI format 0_1/0_2.

    - FFS: Association with the CW with the higher MCS.

Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port with the CW with the higher MCS |
| 1 | 2nd scheduled DMRS port the CW with the higher MCS |
| 2 | 3rd scheduled DMRS port the CW with the higher MCS |
| 3 | 4th scheduled DMRS port the CW with the higher MCS |

  - Alt.2: The size of PTRS-DMRS association field is 3bit in DCI format 0_1/0_2, and the following PTRS-DMRS association for UL PTRS port 0 is specified in TS38.212.

Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port |
| 1 | 2nd scheduled DMRS port |
| 2 | 3rd scheduled DMRS port |
| 3 | 4th scheduled DMRS port |
| 4 | 5th scheduled DMRS port |
| 5 | 6th scheduled DMRS port |
| 6 | 7th scheduled DMRS port |
| 7 | 8th scheduled DMRS port |

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- FFS: Antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for M-TRP case.

Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes |
| 0 | [1] | [0] | Cat. 1 | [0] | [2] | [0,1,2,3,8] | [Rank 5-8 with one DMRS symbol] |
| 1 | [1] | [1] |  | [1] | [2] | [0,1,2,3,8,10] |  |
| 2 | [1] | [0,1] |  | [2] | [2] | [0,1,2,3,8,9,10] |  |
| 3 | 2 | 0 |  | [3] | [2] | [0,1,2,3,8,9,10,11] |  |
| 4 | 2 | 1 |  |  |  |  |  |
| 5 | 2 | 2 |  |  |  |  |  |
| 6 | 2 | 3 |  |  |  |  |  |
| 7 | 2 | 0,1 |  |  |  |  |  |
| 8 | 2 | 2,3 |  |  |  |  |  |
| 9 | [2] | [0-2] |  |  |  |  |  |
| 10 | [2] | [0-3] |  |  |  |  |  |
| 11 | [2] | [0,2] |  |  |  |  |  |
| 12 | [1] | [8] | Cat.2 |  |  |  |  |
| 13 | [1] | [9] |  |  |  |  |  |
| 14 | [1] | [8,9] |  |  |  |  |  |
| 15 | 2 | 8 |  |  |  |  |  |
| 16 | 2 | 9 |  |  |  |  |  |
| 17 | 2 | 10 |  |  |  |  |  |
| 18 | 2 | 11 |  |  |  |  |  |
| 19 | 2 | 8,9 |  |  |  |  |  |
| 20 | 2 | 10,11 |  |  |  |  |  |
| 21 | [2] | [8-10] |  |  |  |  |  |
| 22 | [2] | [8-11] |  |  |  |  |  |
| 23 | [2] | [8, 10], [9, 11] |  |  |  |  |  |
| 24 | [1] | [0,1,8] | Cat.3 |  |  |  |  |
| 25 | [1] | [0,1,8,9] |  |  |  |  |  |
| 26 | 2 | 0,1,8 |  |  |  |  |  |
| 27 | 2 | 0,1,8,9 |  |  |  |  |  |
| 28 | 2 | 2,3,10 |  |  |  |  |  |
| 29 | 2 | 2,3,10,11 |  |  |  |  |  |

In this contribution, we provide our views on following aspects for DMRS enhancements:

- MU-MIMO restrictions/limitations between legacy and new DMRS antenna ports

- DMRS antenna port indication

- PTRS to DMRS enhancements

# MU-MIMO between legacy and new DMRS ports

In RAN#110 [2], following agreement related to MU-MIMO has been made:

Agreement

Support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports.

- For MU-MIMO by different CDM groups, no MU-MIMO scheduling restriction of PUSCH/PDSCH (i.e. MU-MIMO between Rel.15 UE and Rel.18 UE is allowed).

- For MU-MIMO within a CDM group, study whether and how to support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH.

  - Note: the study includes MU-MIMO between Rel.15 UE and Rel.18 UE, and between Rel.18 UEs.

- Note: PUSCH above is CP-OFDM waveform.

Furthermore, in RAN1#111, following proposal has been considered (but not agreed) in the moderator’s summary for DMRS enhancements [3]:

FL Proposal 2-6A

- For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

  - 1) For PUSCH, there is no restriction.

  - 2) For PDSCH, there is no additional restriction between UE1 indicated with Rel-18 Legacy ports (eType1: ports 1000-1007, eType2: ports 1000-1011) and UE2 indicated with Rel.15 DMRS ports in a CDM group.

  - 3) For PDSCH, between UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and UE2 indicated with Rel.15 DMRS ports in a CDM group,

    - Alt.1: UE does not expect such MU-MIMO in a CDM group.

    - Alt.2: Introduce RRC signaling and UE capability signaling for UE configured with Rel.15 DMRS ports, to indicate that there may be another UE with Rel.18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) in the same CDM group, so that the UE can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports.

    - Alt.3: It is up to gNB implementation whether to indicate such MU-MIMO in a CDM group (no need to specify).

    - Note: Alt.2 can be applied to only Rel.18 UE configured with Rel.15 DMRS ports (e.g. to save DCI overhead of antenna ports field). For Rel.15-17 UE, only Alt.1 or 3 can be applied.

In our view, for PDSCH, for legacy UEs, it should be clear that no multiplexing of other UEs with the new DMRS antenna ports within the same CDM group is supported. As there is no possibility to indicate to the legacy UEs, therefore, it should not be considered. For the case of Rel-18 UE, there could be two possibilities. One possibility is that the UE is only configured with legacy DMRS antenna ports and the other possibility is that the UE is configured with legacy as well as new DMRS antenna ports. For the first possibility, the default/baseline capability should be also that there is no multiplexing of other UEs with the new DMRS antenna ports within the same CDM group. Additionally, it could be an optional UE capability to indicate the support of multiplexing between legacy and new DMRS antenna ports within the same CDM group for PDSCH. For the second possibility, it can be a default capability that multiplexing between legacy and new DMRS antenna ports within the same CDM group for PDSCH is supported.

Proposal 1: For a legacy UE, multiplexing with a Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH is not supported

Proposal 2: For a Rel-18 UE that is only configured with legacy DMRS antenna ports, multiplexing with another Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH is not supported as a default capability

Proposal 3: For a Rel-18 UE that is only configured with legacy DMRS antenna ports, multiplexing with another Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH can be supported as an optional UE capability

Proposal 4: For a Rel-18 UE that is configured with new DMRS antenna ports, multiplexing with another Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH is supported

Furthermore, similar to legacy behavior, specific entries in the new DMRS antenna port indication table can be specified for which MU-MIMO within the same CDM group is not supported. For example, for the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, if one UE is scheduled with one codeword and assigned with one of the row indices {9,10,11,21,22,23,24,28} from its configured table, then UE may assume that all the remaining DMRS antenna ports within the same CDM group are not associated with transmission of PDSCH to another UE.

Proposal 5: Specific entries in the new DMRS antenna port indication table can be specified for which MU-MIMO within the same CDM group is not supported:

- For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, if one UE is scheduled with one codeword and assigned with one of the row indices {9,10,11,21,22,23,24,28} from its configured table, then UE may assume that all the remaining DMRS antenna ports within the same CDM group are not associated with transmission of PDSCH to another UE

# DMRS antenna port indication

For DMRS antenna port indication, it is quite straightforward to add new entries to the existing tables and consequently increase the bitfield size, wherever the current bitfield size is not sufficient. Based on this, no new field is needed in the DCI.

Proposal 6: For DMRS antenna port indication, add new entries to the existing tables and increase the bitfield size, wherever required

- No new field related to DMRS is needed

Another remaining key issue is related to dynamic switching between legacy DMRS antenna port indication table and new DMRS antenna port indication table. In our view, similar procedure as in legacy cases can be followed. Currently, the DMRS configuration type and maximum length is semi-statically configured. No dynamic switching between these tables is supported. Similarly, new DMRS antenna port indication tables should also be considered as new additional configurations and similar semi-static switching should be sufficient. Already the flexibility provided by new DMRS antenna port indication table is sufficient.

Proposal 7: DCI based switching between DMRS port(s) associated with length 2 FD-OCC and DMRS port(s) associated with length M FD-OCC (where M > 2) is not supported

# PTRS to DMRS enhancements

In the current specification, we only support up to 2 port PTRS, the 2 bits “PTRS-DMRS association” field is supported in DCI. The current design supports up to 2 independent phase noise and up to 4-layer PUSCH transmission, for example. 4 Tx partial coherent codebook operation with two panels (port groups). In Rel-18, it was agreed to support 8 Tx UL operation, we need to consider the following agreed antenna architecture

Table 1: Illustration of UE antenna layout

If we assume each antenna port group has its own independent phase noise in the worst case, and if we support up to 8 Tx UL partial-coherent operation over 4 port groups (panels) with up to 8 layers, we need the following

- 4 port PTRS

- 4 bit “PTRS-DMRS association” field in DCI

Proposal 8: To support 8 Tx UL operation, support 4 port PTRS and 4 bit “PTRS-DMRS association” field in DCI

# Conclusion

In this contribution, we have discussed our views on DMRS enhancements and made following proposals:

Proposal 1: For a legacy UE, multiplexing with a Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH is not supported

Proposal 2: For a Rel-18 UE that is only configured with legacy DMRS antenna ports, multiplexing with another Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH is not supported as a default capability

Proposal 3: For a Rel-18 UE that is only configured with legacy DMRS antenna ports, multiplexing with another Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH can be supported as an optional UE capability

Proposal 4: For a Rel-18 UE that is configured with new DMRS antenna ports, multiplexing with another Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH is supported

Proposal 5: Specific entries in the new DMRS antenna port indication table can be specified for which MU-MIMO within the same CDM group is not supported:

- For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, if one UE is scheduled with one codeword and assigned with one of the row indices {9,10,11,21,22,23,24,28} from its configured table, then UE may assume that all the remaining DMRS antenna ports within the same CDM group are not associated with transmission of PDSCH to another UE

Proposal 6: For DMRS antenna port indication, add new entries to the existing tables and increase the bitfield size, wherever required

- No new field related to DMRS is needed

Proposal 7: DCI based switching between DMRS port(s) associated with length 2 FD-OCC and DMRS port(s) associated with length M FD-OCC (where M > 2) is not supported

Proposal 8: To support 8 Tx UL operation, support 4 port PTRS and 4 bit “PTRS-DMRS association” field in DCI

# Reference

- 3GPP RAN1#111, “Chairman Notes”

- 3GPP RAN1#110, “Chairman Notes”

- R1-2212760, “ FL summary on DMRS#3”, Moderator (NTT DOCOMO)