3GPP TSG RAN WG1 Meeting #112	R1- 2301380

Athens, Feb. 27th – March 3rd, 2023

Title:	[Draft] Reply to RAN2 LS on default CBR configuration

Response to:	R2-2213168

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	Qualcomm [RAN1]

To:	RAN2

Cc:

Contact person:

Name:	Gabi Sarkis

E-mail Address:	gsarkis@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall Description

RAN1 thanks RAN2 for their Reply LS to RAN1 on default CBR configuration and would like to provide the following reply.

Q1: When UE performs full sensing in R16/17 normal resource pool and has full sensing result available, is it a valid case that UE has no CBR measurement results and makes use of R16 default CBR for congestion control?

In a normal resource pool, i.e. not in the exceptional pool, the minimum full sensing window size is 100ms.The CBR measurement window is either 100ms or 100 slots which is less than or equal to 100ms. Hence, if a UE has full sensing results available, it will have the requisite CBR measurement results.

# Actions

To RAN2:

Action: RAN1 kindly asks RAN2 to take the above reply into account for further work.

# Dates of Next TSG-RAN1 Meetings:

RAN1 #112-bis			17 April – 26 April 2023				            	            Electronic

RAN1 #113			22 May – 26 May 2023					    Incheon, Korea