3GPP TSG-RAN WG1 #112	R1-2301416

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.4.3

Source: 	Qualcomm Incorporated

Title: 	Enhanced sidelink operation on FR2 licensed spectrum

Document for:	Discussion/Decision

# Introduction

At the RAN#98 meeting, it was agreed that enhanced sidelink operation on FR2 licensed spectrum will continue in Rel-18. The objective is following [1]:

- Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1] - Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2] - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

In this contribution, we provide our initial views on beam management aspects of enhanced sidelink operation on FR2 licensed spectrum.

# Initial beam-pairing

For SL unicast communication, UEs first conduct L2 unicast-link establishment procedure as specified in [2]. As illustrated in Fig. 1 below, a UE initiating L2 unicast-link establishment procedure sends a Direct Communication Request Message that contains Source User Info, Target User Info (if any), and ProSe Service Info. To this message, UE(s) who are targeted/interested-in would respond. After the first round of the message exchange, the initiating UE and a responding UE identifies a pair of source-ID and destination-ID for their communication. Then, after some further messages exchange for security related message, UE-capability/assisting information and PC5-RRC configuration, the pair of UEs can start actual SL unicast communication over the unicast-link.

Fig. 1		Layer-2 link establishment procedure

For FR2-SL, it is necessary to enable establishment/maintaining appropriate beam-pair for a pair of UEs for the unicast communication. At the beginning, a UE who initiates L2 unicast-link establishment procedure would not know in which direction(s) the UE(s) that is/are targeted/interested-in are located and hence may need to sweep transmissions over multiple Tx beams. Another UE who would monitor messages also no idea from which Rx beam it receives a message. The message transmission/reception would be successful when the pair of Tx/Rx beams are (occasionally) matched.

In Uu, initial beam-pairing is done by using SSB and PRACH, where multiple SSBs are transmitted over time using different beams from a BS, and a UE that detects an SSB from the BS would transmit a PRACH on the resource associated with the detected/selected SSB. Based on the predefined rules between a SSB index and a PRACH resource/timing, the BS and the UE can establish the beam-pair with the SSB transmission/reception from the BS to the UE and PRACH transmission/reception from the UE to the BS. In other words, the initial beam-pairing is done based on the SSB beam – PRACH resource association in Uu. For FR2-SL, similar association can be considered. In particular, an initiating UE transmits a signal/channel multiple times using different Tx beams. A UE that detects the signal/channel transmitted by the initiating UE may respond to the initiating UE using a particular resource/timing associated with the signal/channel detected from the initiating UE. The initiating UE monitors the response by using the Rx beam that corresponds to the Tx beam it has used for the transmission of the signal/channel. This is pretty much same as the Uu SSB – PRACH association.

Fig. 2	Tx beam-sweep and response resources associated with initiating UE’s Tx beams

Observation 1-1:

- For FR2-SL, message exchanges in L2 unicast-link establishment procedure between a pair of UEs would need to be based on beam-forming

  - In this case, the use of appropriate Tx/Rx beams between a pair of UEs would be essential

- For FR2-Uu, initial-access is based on SSB and PRACH with the beam-resource association to enable beam-pairing

  - Such beam-resource association would also be useful for FR2-SL

For the Tx beam-sweeping at the very first step of unicast-link establishment where paired UE(s) is/are not yet identified each other, a simple approach is to transmit the initiating message (e.g., Direct Communication Request Message) over the multiple Tx beams. As long as a UE that is targeted or interested-in the unicast-link establishment is within the coverage of a Tx beam, the target/interested UE can receive it using a Rx beam and can respond to the initiating UE using the Tx beam corresponding to the Rx beam that was used to receive the initiating message. However, this would require a number of slots for Tx beam-sweeping and a number of slots for monitoring potential responses. Efficient approach, e.g., enabling Tx beam-sweep in a shorter period of time (e.g., within a slot), is desirable.

Observation 1-2:

- Transmitting initiating message (e.g., Direct Communication Request Message) over multiple Tx beams and monitoring response(s) over multiple Rx beams for initial beam-sweep/search causes high overhead and latency

  - Efficient way of enabling first step beam-pairing is desirable

After the initial message exchange between an initiating UE and a responding UE using matched Tx/Rx beams, the UEs identify the initial beam-pair for the UE pair. The identified beam-pair should be maintained and used for subsequent message exchanges for the UE pair to complete L2 unicast-link establishment and PC5-RRC establishment. Note that a UE may have multiple unicast-links with multiple different UEs using the same or different beams. Also, a UE having an already established unicast-link with another UE may initiate the other unicast-link establishment with the other UE, or may respond to the other UE’s direct communication request message. All these cases should be taken into account for further study of initial beam-paring for FR2-SL.

Based on the discussion above, we propose following.

Proposal 1:

- Study methods to enable initial beam-pairing for FR2-SL, including following:

  - Procedure to enable initial beam-pairing, as well as UE-pairing (e.g., L2 unicast-link establishment), for a SL unicast communication

  - Efficient beam-search method before a pair of UEs find a beam-pair

    - Including potential solution for faster beam sweeping (e.g., multiple beams within a slot)

  - Beam – resource association, similar to Uu SSB – PRACH association

# Beam-maintenance

## Beam measurement

In the current NR sidelink, after unicast-link/PC5-RRC establishment, UEs can transmit/measure SL CSI-RS for CSI measurement. For this, SL CSI-RS is always associated with the PSCCH/PSSCH in the same slot and that can be triggered by the CSI request field in the corresponding SCI format 2-A/2-C. A UE receiving the PSCCH/PSSCH checks source-ID and destination-ID and then measure the associated SL CSI-RS, if triggered for the given unicast-link. Then, the UE reports RI/CQI via Sidelink CSI Reporting MAC CE.

For FR2 beam-maintenance, beam measurement and its report for beam refinement are essential. In Uu, both SSB-based and CSI-RS-based beam measurement have been supported. NW can configure SSBs or CSI-RSs for beam measurements and let UE to report the measurement results back to the NW. Various combinations of (periodic, semi-persistent, and aperiodic) measurement and (periodic, semi-persistent, and aperiodic) report are supported.

For FR2-SL beam measurement, SL RSs are transmitted using different beams by a UE and are measured by a paired UE. To make the measurement efficient, it is desirable to enable transmission/measurement of multiple SL RSs for beam measurement within a short period of time. For example, multiple SL RSs associated with different beams in a slot can be considered. Following the legacy SL CSI-RS transmission, these multiple SL RSs for beam measurement can be associated with a PSCCH/PSSCH. As for legacy SL CSI-RS trigger, the associated SCI format 2 in the PSSCH can trigger the SL RSs to measure different beams. In the slot, PSCCH/PSSCH and one or multiple SL RS resources for beam measurement should be TDMed, so that they can be associated with different Tx beams.

Fig. 3	SL RS for beam measurement associated with PSCCH/PSSCH

Considering that beam measurement using SL RSs does not necessarily require associated data payload, periodic transmission of SL RS for beam measurement would also be useful. Such ‘stand-alone’ SL RSs for beam measurement may enable further efficient beam measurement with manner (i.e., not associated with PSCCH/PSSCH) would also be useful.

Fig. 4	SL RS for beam measurement not associated with PSCCH/PSSCH (‘stand-alone’)

Observation 2-1:

- It is necessary to enable beam measurement using SL RS

- For efficient beam measurement, multiple SL RSs in a short period of time (e.g., within as lot) is desirable

- SL RS for beam measurement associated with a PSCCH/PSSCH could be an extension from SL CSI-RS

  - Note: The SL RSs and associated PSCCH/PSSCH should be TDMed when different beams apply

- SL RS for beam measurement not associated with PSCCH/PSSCH may improve the efficiency further

For mode 1 (or in-coverage scenario), gNB has knowledge of SL unicast-link communications within the cell (from the SidelinkUEInformationNR reported by each SL UE). With a proper gNB’s centralized management, it would be possible to let UEs to transmit SL RS for beam measurement in non-stand-alone or stan-alone manner.

For mode 2 (or out-of-coverage scenario), potential impact from SL RS for beam measurement for distributed resource/beam management should be further studied. For SL RSs for beam measurement associated with PSCCH/PSSCH, UEs can select available resources for SL RS transmission based on resource sensing using associated PSCCH/PSSCH, taking into account the information available from the detected SCI format as well as RSRP measured by DMRS. However, if the beam for PSCCH/PSSCH and the beams for the associated SL RSs are quite different (e.g., from different panels), sensing information achieve from the associated PSCCH/PSSCH may not be useful. For SL RSs for beam measurement not associated with PSCCH/PSSCH (stand-alone case), legacy resource sensing/selection would not work and hence RS collision/confusion among transmitting UEs may occur.

Observation 2-2:

- For mode 1 (or in-coverage scenario), it would be possible for gNB to manage SL RS for beam management for SL UEs in the coverage based on proper information of the SL UEs in the coverage

- For mode 2 (or out-of-coverage scenario), impact of SL RS for beam measurement on resource sensing/selection needs to be studied

Based on the above discussion, we propose the following.

Proposal 2:

- Study SL RS for beam measurement, including the feasibility of the following potential options

  - Opt.1: SL RS for beam measurement is associated with a PSCCH/PSSCH

  - Opt.2: SL RS for beam measurement is not associated with a PSCCH/PSSCH (‘stand-alone’)

- Consider to enable SL RSs with multiple beams in a short period of time (e.g., within a slot)

- Study potential impact of SL RS for beam measurement on mode 2 or out-of-coverage scenario

## Beam measurement report

Once beam measurement using SL RSs is carried out, the UE should send the report back to the paired UE of the unicast-link that has transmitted the SL RSs. Currently, SL CSI feedback is based on a SL MAC-CE. It would be straightforward to consider using SL MAC-CE for the measurement report back to the paired UE. As for reporting quantity, L1-RSRP and L1-SINR of the measured SL RSs can be considered.

In addition to the measurement report back to the paired UE fot the unicast-link, beam measurement report to the gNB can also be considered for in-coverage scenario, which would help gNB’s scheduling and/or resource management for SL UEs in mode 1. The report to gNB can be from the measuring UE or can be forwarded from the paired UE that has received the report. The SL beam measurement report to the serving cell can be UCI (similar to legacy SL HARQ-ACK or Uu BM report for serving cell/non-serving cell) or can be based on a Uu MAC-CE (similar to legacy SL CSI feedback).

Observation 3-1:

- Reporting L1-RSRP/L1-SINR based on SL RS for beam measurement is essential for beam management

- For beam measurement report to the paired UE, reporting using SL MAC-CE would be a straightforward extension from the existing SL CSI feedback

- For beam measurement report to the serving cell, following two cases are possible:

  - The measuring UE reports to the serving cell

  - The measuring UE reports to the paired UE that has transmitted SL RS for beam measurement, and the paired UE forward the report to the serving cell

Fig. 5	Beam measurement report (1) back to the paired UE, (2) to the serving cell from measuring UE, and (3) to the paired UE from measuring UE and the paired UE forward it to the serving cell

We consider all the three cases are valid options for further study. With this understanding, we propose the following:

Proposal 3:

- Study report of beam measurement using SL RS

  - Consider L1-RSRP and L1-SINR as the reporting quantities

  - Case 1: Measuring UE reports to the paired UE that has transmitted SL RS for beam measurement

    - SL MAC-CE is a starting point of the reporting format

  - Case 2: Measuring UE reports to the serving cell

    - FFS: reporting methodology (e.g., based on UCI or MAC-CE)

  - Case 3: Measuring UE reports to the paired UE that has transmitted SL RS for beam measurement and then the paired UE forwards the report to the serving cell

## Beam determination for SL transmission/reception

Based on the beam measurement, each UE should be able to determine which beam to use for actual sidelink data communication (e.g., for PSCCH/PSSCH/PSFCH). S-SSB transmission/reception for synchronization and MIB delivery should also be taken into account. As a first step, it is necessary to consider spatial-QCL relations between all SL channels/signals for transmission and/or reception over sidelink.

For beam determination for SL transmission/reception, following two cases can be considered:

- Case 1: Each UE determines its own Tx beam and informs it to the paired UE

- Case 2: One UE determines its own Tx beam and the paired UE’s Tx beam, and informs it to the paired UE

For Case 1, both UEs are supposed to perform beam management. When a UE (UE1) determines its own Tx beam and informs to the paired UE (UE2), UE2 determines, based on the provided information, the corresponding Rx beam. UE2 can use a Tx beam corresponding to the Rx beam used for the reception from UE1 for its own transmission. Or alternatively, UE2 can use another Tx beam for its own transmission, based on the measurement of SL RS from UE1, in which case UE2 should inform UE1 of the Tx beam used for its own transmission.

For Case 2, one of the UEs performs beam management. When UE1 determines its own Tx beam and informs to UE2, UE2 determines, based on the provided information, the corresponding Rx beam. In addition, UE1 further determines UE2’s Tx beam based on its own measurement of SL RSs from UE2, and then informs UE2 to use it for UE2’s transmission. This case could be suitable for a topology such as cluster/star topology, where one UE works as ‘hub’ for multiple UEs using multiple unicast-links.

In addition, for the case where SL beam management is done by gNB as described in Section 2.2.2, SL beam indication by gNB can also be considered (Case 3).

Observation 4-1:

- For beam indication, there are following three cases:

  - Case 1: Each UE determines its own Tx beam and informs it to the paired UE

  - Case 2: One UE determines its own and the paired UE’s Tx beams and inform them to the paired UE

  - Case 3: gNB determines Tx beams of the UEs

- To further discuss how to determine beam(s) for SL transmission/reception, spatial-QCL relations across SL RSs and SL channels should be taken into account

Fig. 6 Three cases of beam determination/indication for SL transmission/reception

Signalling for beam indication should also be studied. For legacy Uu BM, beams are characterized by TCI-states (and spatial relation information). Multiple TCI-states (or spatial relation information) can be configured by RRC, and a subset of the configured TCI-states can be activated by MAC-CE, and one of the active TCI-states can be indicated by a field in a DCI format. This multi-level signalling method helps to reduce the signalling overhead for both higher-layer and physical layer. Further, in Rel-17, overall signalling method for DL/UL TCI/beam has been simplified by introducing unified-TCI framework. Similar signalling methodology can be considered for FR2-SL.

It should be noted that the beam indication cannot be effective immediately. Similar to Uu, a certain timeline has to be guaranteed between a beam indication signalling and its effective timing. For Uu, the timeline is informed by a UE capability signalling, timeDurationForQCL, where each UE reports {7, 14, or 28} symbols for SCS 60kHz and {14 or 28} symbols for SCS 120kHz. Required timeline and its impact on overall operation/performance for FR2-SL should also be studied.

Observation 4-2:

- For Uu, beam indication utilizes RRC, MAC-CE, and DCI for configuring/activating/indicating TCI-state(s) to reduce signalling overhead for both higher-layer and lower-layer

  - Similar ‘multi-level’ signalling would also be useful for FR2-SL

- There is a timeline from the time that signalling for beam indication is provided to the timing that the indication is effective

Based on the above discussion, we propose following.

Proposal 4:

- Clarify spatial-QCL relations b/w SL RSs for beam measurement and SL transmission/reception

- Following three cases are considered in the study:

  - Case 1: Each UE determines its own Tx beam and informs it to the paired UE

  - Case 2: One UE determines its own and the paired UE’s Tx beam and inform them to the paired UE

  - Case 3: gNB determines Tx beams of the UEs

- Study signalling for beam indication/determination

  - Taking into account the trade-off between signalling overhead and indication flexibility

    - Consider multi-level signalling using RRC, MAC-CE, and SCI/DCI

    - Consider Unified-TCI framework as the starting point

  - Including necessary timeline between a signalling and the timing that the indicated beam is effective

# Beam failure recovery

For FR2-SL, it is possible that one or both UEs in a unicast-link rotates or moves and then all the beams do not work for the link. Beam failure recovery is expected to enable recovery from such outage scenario, as already supported for Uu.

For beam failure recovery for FR2-SL, similar to Uu, essential components would be following:

- BFD-RS: The RS monitored by a UE and check whether the beam failure condition is met.

- CBD-RS: The RS(s) monitored by the UE as the candidate beams for BFR. In case beam failure is declared, the UE select one of the monitored CBD-RSs that satisfies a quality so that the recovery is triggered by using the selected CBD-RS.

- Beam failure indication resource: The UE who declares beam failure should be able to inform the beam failure and the selected CBD-RS to the paired UE.

For Uu BFR, either SSB or CSI-RS can be configured as BFD-RS and CBD-RS. As for beam failure indication, PRACH is used for SpCell BFR while SR resource is used for SCell BFR.

For FR2-SL, since the UEs have the common unicast-link, either or both of UEs in a pair can monitor BFD-RS and CBD-RS and declares beam failure. Regarding beam failure declaration, two options can be considered: (1) declaration to the paired UE, and (2) declaration to the serving cell.

For (1) declaration to the paired UE, straightforward approach would be to configure beam failure indication resources associated to the CBD-RSs and then the UE sends beam failure indication based on the selected CBD-RS for recovery. The paired UE monitors beam failure indication resources associated to the CBD-RSs, and once it detects the indication associated to a CBD-RS, the paired UE notices beam failure and consider the beam for the CBD-RS is the one that should be used for recovery. This is pretty much the same as Uu SpCell BFR where contention-free PRACH is used for beam failure indication. However, this requires periodic RS transmission/monitoring and reservation of beam failure indication resources for BFR.

For (2), the UE in beam failure situation uses Uu link over the serving cell for BFR. Therefore, it is not necessary to reserve a number of resources for beam failure indication associated with CBD-RSs for this case. This is similar to Uu SCell BFR in which case beam failure occurs on SCell but Uu link over the PCell is still valid/active. For Uu SCell BFR, only a SR resource is reserved for beam failure indication. If the UE declares beam failure of a SCell and indicates it using the SR resource, the network can further request UE to report the details of BFR request such as serving cell index, CBD-RS ID, etc. Similar procedure can be considered in this case.

Observation 5-1:

- For FR2-SL, two options for BFR can be considered: between the pair of UEs, and through the serving cell

- SL BFR between the pair of UEs would be the same as Uu SpCell BFR in the sense that beam failure indication resources need to be reserved for potential beam failure

- SL BFR through the serving cell can utilize the valid/active Uu link for SL recovery and hence can be similar to Uu SCell BFR

SL RSs for BFD and CBD and channel/signal for beam failure indication need to be identified. Same as for SL RS for beam measurement, SL RS transmission over multiple beams should be efficient. Since CBD-RS need to be transmitted per candidate beam, efficiency/overhead is an important design factor.  Beam failure indication over SL should also be a low overhead signal/channel since the beam indication resources are reserved per CBD-RS beam per UE.

Observation 5-2:

- It is desirable to enable multiple SL RSs for candidate beams in a short period of time (e.g., within as lot)

- For BFR between UEs, it would be necessary to reserve beam failure indication resources for candidate beams for each UE

  - Low overhead format is desirable

Based on the above discussion, we propose to study further SL BFR procedures between a pair of UEs and via the serving cell. Further discussion is necessary on efficient methodology for SL RSs for multiple candidate beams for recovery, as well as reserving resources for potential beam failure indication.

Proposal 5:

- Study methods to enable beam failure recovery for FR2-SL, including following:

  - Procedure to enable beam failure recovery between the pair of UEs

  - Procedure to enable beam failure recovery through the serving cell

  - Efficient SL RS transmissions for candidate beams for recovery

  - Efficient signal/channel structure for beam failure indication

# Mode 2 aspects

## Resource sensing/selection

In mode 2, each UE performs resource sensing and selection. For legacy SL, the mode 2 resource sensing/selection is based on SCI format 1 and PSCCH/PSSCH DMRSs. For FR2-SL, all PSCCH/PSSCH transmissions are supposed to be beam-formed. In addition, resource sensing/selection may also be based on a particular beam at the sensing UE. This may impact on reliability of mode 2 resource sensing/selection procedure.

An example is illustrated in Fig. 2. Here, UE1 transmits PSCCH/PSSCH to UE2, and UE3 perform sensing it. All the three UEs use particular beams for Tx/Rx/sensing. In Case 1, UE3 senses UE1’s transmission using a sensing beam that direct to UE1 and therefore, the UE3 takes this sensing result into account for resource selection. On the other hand, in Case 2, UE3 uses sensing beam which is not directed to UE1. In this case, UE3 may consider the resource is available as a result of sensing and may transmit on the resource using the beam corresponding to the sensing beam – causing resource collision for UE2’s reception.

Fig. 7 Example of mode 2 resource sensing when beam-forming is used

In this situation, it is important to make sure that UE3 is able to sense it, and exclude the resource/beam(s) that causes resource collision at the receiving UE. Firstly, RAN1 should study whether the current mode 2 resource sensing/selection properly works when beam-forming are performed for FR2-SL and discuss if any enhancements are necessary.

Proposal 6:

- Study the impact of beam-forming operation in mode 2 resource sensing/selection

## Inter-UE coordination

Inter-UE coordination enables UEs to exchange preferred/non-preferred resources. However, similar issue as for resource sensing/selection explained in 5.1 exists. For example, legacy IUC request cannot indicate for which beam the IUC info is requested. Also, legacy IUC info cannot indicate which beam the preferred/non-preferred resources in the IUC info is associated. It should be studied whether/how to take into account beam(s) for preferred/non-preferred resources for FR2-SL inter-UE coordination.

An example is illustrated in Fig. 8. UE1 and UE2 has a unicast-link and there are two candidate beam-pairs to use for the unicast communication. Suppose UE2 receives ICU request from UE1 and then UE2 informs preferred/non-preferred resource to UE1. If UE2 may sense transmission from UE3 by using the beam in the beam-pair 2 but not by using the beam in the beam-pair 1, UE2’s preferred/non-preferred resources for beam-pair 1 and beam-pair 2 can be different.

Fig. 8	Example of inter-UE coordination with the presence of beam-forming

Proposal 7:

- Study inter-UE coordination in the presence of beam-forming at the UEs.

# Other aspects

## SL Tx/Rx beam correspondence

According to TR 38.912, Uu Tx/Rx beam correspondence was captured as follows for the study:

Tx/Rx beam correspondence at TRP and UE are considered as follows: - Tx/Rx beam correspondence at TRP holds if at least one of the following is satisfied: - TRP is able to determine a TRP Rx beam for the UL reception based on UE’s DL measurement on TRP’s one or more Tx beams - TRP is able to determine a TRP Tx beam for the DL transmission based on TRP’s UL measurement on TRP’s one or more Rx beams - Tx/Rx beam correspondence at UE holds if at least one of the following is satisfied: - UE is able to determine a Tx beam for the UL transmission based on the UE’s measurement on the UE’s one or more Rx beams. - UE is able to determine a Rx beam for the DL reception based on TRP’s indication based on UL measurement on UE’s one or more Tx beams.

Later on, the UE capabilies for Uu Tx/Rx beam correspondence were defined in TS 38.306 as beamCorrespondenceWithoutUL-BeamSweeping, beamCorrespondenceSSB-based-r16, and beamCorrespondenceCSI-RS-based-r16. Necessary requirements corresponding to these capabilities were specified in RAN4 as well.

For FR2-SL, it is necessary to consider SL Tx/Rx beam correspondence. Following the descriptions in TR 38.912, SL Tx/Rx beam correspondence for UE-A that communicates with UE-B can be characterized by one or multiple of the following:

- UE-A has the ability to determine:

  - its Tx beam for SL transmission based on UE-A’s SL measurement on one or more UE-A’s Rx beams or

  - its Rx beam for SL reception based on UE-B’s SL measurement on one or more UE-A’s Tx beams

Details should be for further study.

We consider that the Rel-18 FR2-SL study should prioritize the case where SL Tx/Rx beam correspondence applies at each UE.

Proposal 8:

- Consider SL Tx/Rx beam correspondence with the following as the starting point of the condition(s) that SL Tx/Rx beam correspondence applies:

  - UE-A has the ability to determine:

    - its Tx beam for SL transmission based on UE-A’s SL measurement on one or more UE-A’s Rx beams

    - its Rx beam for SL reception based on UE-B’s SL measurement on one or more UE-A’s Tx beams

- For Rel-18 FR2-SL study, prioritize the case where the SL Tx/Rx beam correspondence applies at each UE

## Overall framework for FR2 SL beam management

It can be observed in the previous sections of this paper that beam management requires multiple RS transmissions/receptions associated with different beams for various purposes such as beam measurement and candidate beam monitoring for BFR. As presented already, for both purposes, SL RS transmissions in a short period of time (e.g., within a slot) is desirable.

For initial beam-pairing, similar issue would exist; Tx beam-sweeping and response resource monitoring are inevitable for initial beam-pairing and hence, it is highly desirable that these can be done in an efficient manner.

We consider that it would be beneficial if there is a unified framework to enable overall FR2 SL beam management. For example, SL RS associated or not associated with PSCCH/PSSCH (illustrated in Fig. 3 and Fig. 4) can be common tools for initial beam-pairing, beam measurement, and BFR.

Proposal 9:

- The FR2-SL study should consider possibility of enabling efficient multi-beam transmissions/receptions within a short period of time for initial beam-pairing, beam measurement, and BFR based on a unified framework

# Conclusions

In this contribution, we provided our initial views on beam management aspects of enhanced sidelink operation on FR2 licensed spectrum.

On initial beam-pairing, we have following observations and proposal.

Observation 1-1:

- For FR2-SL, message exchanges in L2 unicast-link establishment procedure between a pair of UEs would need to be based on beam-forming

  - In this case, the use of appropriate Tx/Rx beams between a pair of UEs would be essential

- For FR2-Uu, initial-access is based on SSB and PRACH with the beam-resource association to enable beam-pairing

  - Such beam-resource association would also be useful for FR2-SL

Observation 1-2:

- Transmitting initiating message (e.g., Direct Communication Request Message) over multiple Tx beams and monitoring response(s) over multiple Rx beams for initial beam-sweep/search causes high overhead and latency

  - Efficient way of enabling first step beam-pairing is desirable

Proposal 1:

- Study methods to enable initial beam-pairing for FR2-SL, including following:

  - Procedure to enable initial beam-pairing, as well as UE-pairing (e.g., L2 unicast-link establishment), for a SL unicast communication

  - Efficient beam-search method before a pair of UEs find a beam-pair

    - Including potential solution for faster beam sweeping (e.g., multiple beams within a slot)

  - Beam – resource association, similar to Uu SSB – PRACH association

On beam measurement, we have following observations and proposal.

Observation 2-1:

- It is necessary to enable beam measurement using SL RS

- For efficient beam measurement, multiple SL RSs in a short period of time (e.g., within as lot) is desirable

- SL RS for beam measurement associated with a PSCCH/PSSCH could be an extension from SL CSI-RS

  - Note: The SL RSs and associated PSCCH/PSSCH should be TDMed when different beams apply

- SL RS for beam measurement not associated with PSCCH/PSSCH may improve the efficiency further

Observation 2-2:

- For mode 1 (or in-coverage scenario), it would be possible for gNB to manage SL RS for beam management for SL UEs in the coverage based on proper information of the SL UEs in the coverage

- For mode 2 (or out-of-coverage scenario), impact of SL RS for beam measurement on resource sensing/selection needs to be studied

Proposal 2:

- Study SL RS for beam measurement, including the feasibility of the following potential options

  - Opt.1: SL RS for beam measurement is associated with a PSCCH/PSSCH

  - Opt.2: SL RS for beam measurement is not associated with a PSCCH/PSSCH (‘stand-alone’)

- Consider to enable SL RSs with multiple beams in a short period of time (e.g., within a slot)

- Study potential impact of SL RS for beam measurement on mode 2 or out-of-coverage scenario

On beam measurement report, we have following observation and proposal.

Observation 3-1:

- Reporting L1-RSRP/L1-SINR based on SL RS for beam measurement is essential for beam management

- For beam measurement report to the paired UE, reporting using SL MAC-CE would be a straightforward extension from the existing SL CSI feedback

- For beam measurement report to the serving cell, following two cases are possible:

  - The measuring UE reports to the serving cell

  - The measuring UE reports to the paired UE that has transmitted SL RS for beam measurement, and the paired UE forward the report to the serving cell

Proposal 3:

- Study report of beam measurement using SL RS

  - Consider L1-RSRP and L1-SINR as the reporting quantities

  - Case 1: Measuring UE reports to the paired UE that has transmitted SL RS for beam measurement

    - SL MAC-CE is a starting point of the reporting format

  - Case 2: Measuring UE reports to the serving cell

    - FFS: reporting methodology (e.g., based on UCI or MAC-CE)

  - Case 3: Measuring UE reports to the paired UE that has transmitted SL RS for beam measurement and then the paired UE forwards the report to the serving cell

On beam determination for SL Tx/Rx, we have following observations and proposal.

Observation 4-1:

- For beam indication, there are following three cases:

  - Case 1: Each UE determines its own Tx beam and informs it to the paired UE

  - Case 2: One UE determines its own and the paired UE’s Tx beams and inform them to the paired UE

  - Case 3: gNB determines Tx beams of the UEs

- To further discuss how to determine beam(s) for SL transmission/reception, spatial-QCL relations across SL RSs and SL channels should be taken into account

Observation 4-2:

- For Uu, beam indication utilizes RRC, MAC-CE, and DCI for configuring/activating/indicating TCI-state(s) to reduce signalling overhead for both higher-layer and lower-layer

  - Similar ‘multi-level’ signalling would also be useful for FR2-SL

- There is a timeline from the time that signalling for beam indication is provided to the timing that the indication is effective

Proposal 4:

- Clarify spatial-QCL relations b/w SL RSs for beam measurement and SL transmission/reception

- Following three cases are considered in the study:

  - Case 1: Each UE determines its own Tx beam and informs it to the paired UE

  - Case 2: One UE determines its own and the paired UE’s Tx beam and inform them to the paired UE

  - Case 3: gNB determines Tx beams of the UEs

- Study signalling for beam indication/determination

  - Taking into account the trade-off between signalling overhead and indication flexibility

    - Consider multi-level signalling using RRC, MAC-CE, and SCI/DCI

    - Consider Unified-TCI framework as the starting point

  - Including necessary timeline between a signalling and the timing that the indicated beam is effective

On beam failure recovery, we have following observations and proposal.

Observation 5-1:

- For FR2-SL, two options for BFR can be considered: between the pair of UEs, and through the serving cell

- SL BFR between the pair of UEs would be the same as Uu SpCell BFR in the sense that beam failure indication resources need to be reserved for potential beam failure

- SL BFR through the serving cell can utilize the valid/active Uu link for SL recovery and hence can be similar to Uu SCell BFR

Observation 5-2:

- It is desirable to enable multiple SL RSs for candidate beams in a short period of time (e.g., within as lot)

- For BFR between UEs, it would be necessary to reserve beam failure indication resources for candidate beams for each UE

  - Low overhead format is desirable

Proposal 5:

- Study methods to enable beam failure recovery for FR2-SL, including following:

  - Procedure to enable beam failure recovery between the pair of UEs

  - Procedure to enable beam failure recovery through the serving cell

  - Efficient SL RS transmissions for candidate beams for recovery

  - Efficient signal/channel structure for beam failure indication

On mode 2 aspects, we have following proposals.

Proposal 6:

- Study the impact of beam-forming operation in mode 2 resource sensing/selection

Proposal 7:

- Study inter-UE coordination in the presence of beam-forming at the UEs.

In addition, we have following proposal on SL Tx/Rx beam correspondence.

Proposal 8:

- Consider SL Tx/Rx beam correspondence with the following as the starting point of the condition(s) that SL Tx/Rx beam correspondence applies:

  - UE-A has the ability to determine:

    - its Tx beam for SL transmission based on UE-A’s SL measurement on one or more UE-A’s Rx beams

    - its Rx beam for SL reception based on UE-B’s SL measurement on one or more UE-A’s Tx beams

- For Rel-18 FR2-SL study, prioritize the case where the SL Tx/Rx beam correspondence applies at each UE

Lastly, we have following proposal for overall SL beam management.

Proposal 9:

- The FR2-SL study should consider possibility of enabling efficient multi-beam transmissions/receptions within a short period of time for initial beam-pairing, beam measurement, and BFR based on a unified framework

# References

- RP-222806, “WID revision: NR sidelink evolution”, RAN #98-e.

- 3GPP TS23.304