3GPP TSG RAN WG1 #112		     R1-2301430

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.9.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on Rel-18 UL Tx switching

Document for:	Discussion/Decision

# Introduction

In the RAN1#111 meeting, Rel-18 UL Tx switching made good progress and concluded most issues. In this paper, we present our views and preference on the leftover issues.

# Switching period location

RAN4 sent the LS to ask RAN1 to discuss the location of switching period.

Issue 4: Location of switching period - For single-TAG case, RAN4 agreed to reuse the Rel-16/17 approach (i.e., semi-static configuration of switching period on one of the band for each switching band pair) and discuss further details for Rel-18 Tx switching scenario in RAN1. - Meanwhile, RAN4 has not concluded on the switching period location for 2-TAG case, with further discussions ongoing.

In Rel-16/17, one of the two bands is configured to take the switching period by RRC IE - uplinkTxSwitchingPeriodLocation. This switching period location could be configured on one of carriers of two CA bands or one of NUL & SUL band. The configured band would be impacted, and no scheduling is expected during the switching period.

For Rel-18 UL Tx switching, the dynamic switching would involve more than two bands. The current RRC configured switching period location needs to be revisited. Take three bands switching as an example, if the RRC configuration indicates band A to take the switching period but the dynamic switching is scheduled between band B and C, there would be ambiguity as none of them is configured with switching period location.

In RAN1#111 meeting, this issue was discussed but without agreement due to limited time. The moderator summarized the following alternatives and suggested to down select in next WG meeting.

### Updated proposed agreement 6 Down-select a solution for the ambiguity issue on switching period location from following alternatives. - Alt.1: switching period location is configured per band pair - If there are multiple bands configured with switching period location as TRUE in the bands involved in a switching, the switching period location is determined to highest carrier frequency among the bands configured with switching period location as TRUE - Alt.2: switching period location is configured per band pair, and the priority list of bands is also configured - If there are multiple bands configured with switching period location as TRUE in the bands involved in a switching, the switching period location is determined to the band with lowest priority among bands configured with switching period location as TRUE - Alt.3: gNB configures “switching-from band” or “switching-to band” - If gNB configures “switching-from band” as switching period location, switching period is located on band(s) where preceding transmission is performed - Alt.4: gNB configures switching period location per switching case - In 3 bands case, gNB configures switching period location for each of switching case pair such as {A - B}, {A - C}, {B - C}, {A+B - C}, {A+C - B}, {B+C - A} - In 4 bands case, gNB configures switching period location for each of switching case pair such as {A - B}, {A - C}, {A - D}, {B - C}, {B - D}, {C - D}, {A+B - C}, {A+B - D}, {A+C - B}, {A+C - D}, {A+D - B}, {A+D - C}, {B+C - A}, {B+C - D}, {B+D - A}, {B+D - C}, {C+D - A}, {C+D - B}, {A+B – C+D}, {A+C – B+D}, {A+D – B+C} - Alt.4-rev: gNB configures switching period location per configured 3 or 4 bands - In 3 bands case, gNB configures switching period location for each of switching case pair such as {A, B}, {A, C}, {B, C}, {A, B, C} - In 4 bands case, gNB configures switching period location for each of switching case pair such as {A, B}, {A, C}, {A, D}, {B, C}, {B, D}, {C, D}, {A, B, C}, {A, B, D}, {B, C, D}, {A, B, C, D} - Alt.5: gNB configures priorities to each carrier/band. - The UE determines the switching period location on the band that is not with the highest priority.

Among these five alternatives, both Alt.1 and Alt. 2 are per band pair configuration which could lead to location ambiguity for 3 or 4 bands switching if more than 1 band is configured for source & target switching bands. For example, if configuration is {band A for band pair A, B} and {band C for band pair C, D}, there would be two bands (A, C) for switching case A+B -> C+D. The additional rules or configuration are needed to resolve the ambiguity issue. Alt. 1 requires the highest carrier/band among the bands configured with switching period location to take the switching period. Alt. 2 further introduces the priority rules once more than 1 bands configured with switching period location.

Alt. 3 is the one with least configuration signalling bits as only single or couple bits is needed to indicate either “switching-from” or “switching-to” band to take the switching period. Alt. 4 allows the detailed configuration for each switch case pair from network and no ambiguity would happen thereafter. The major drawback of Alt. 4 is the signalling bits consumption. Alt. 5 is to define the priority for each band/carrier and lower priority band takes the switching period.

Table 1 is the exemplary signalling consumption for each alternative. Alt. 1 needs indication of the band for each band pair, which is roughly 3 bits for 3 bands and 6 bits for 4 bands. Compared with Alt. 1, Alt. 2 needs additional configuration of band priority list and this additional part consumes same signalling bits as Alt. 5. Alt. 3 only needs 1 or couple bits which is the least signalling consumption. Alt. 4 needs more than 20 bits and Alt.4 revision reduces the signalling cost compared with Alt. 4, and the signalling is 4 bits for 3 bands, 10 bits for 4 bands.

Alt. 4 provides the clearest configuration without any ambiguity, but companies show concerns on the signaling load in RAN1#111. For sake of the progress, if companies have strong concern on signaling load, we could support Alt. 3 or 5 as they consume much less signalling compared with other alternatives.

Table 1 Exemplary signalling consumption

| Alt. | Switching period location | Exemplary signalling consumption |
| --- | --- | --- |
| 1 | Per band pair + highest carrier | If using 1 bit to indicate true or false for each band, 1 bit * band pair numbers. 3 bits for 3 bands and 6 bits for 4 bands. |
| 2 | Per band pair + priority rule | Same number of bits for per band pair as above + signalling for priority list |
| 3 | Switching-to or switching-from band | 1 or 2 bits |
| 4 | Per switching case pair | Alt. 4 needs >20bits. Alt.4 revision needs less bits. If using 1 bit to indicate true or false for each band in each band combo, 4 bits for 3 bands, 10 bits for 4 bands. |
| 5 | Band priority | signalling for priority list |

Proposal 1: For switching period location, if companies have strong concern on signaling overhead, choose Alt. 3 or 5 as they require less signaling compared with other alternatives.

# Minimum Separation time between two consecutive switches

From system performance perspective, frequent switching within a short time period would unnecessarily degrade the throughput as the switching period could be 70us or 210us. RAN1#111 agreed below Working Assumption to resolve the frequent switching issue. However, we found the WA could not serve this purpose for some cases.

Agreement Following restrictions are applied for Rel-18 UL Tx switching across 3 or 4 bands. - The UE does not expect to perform more than one uplink switching within a reference slot based on µUL = max(µUL, 1, µUL, 2, µUL, 3) in case of 3 bands, µUL = max(µUL, 1, µUL, 2, µUL, 3, µUL, 4) in case of 4 bands, where µUL, 1, µUL, 2, µUL, 3, µUL, 4 are SCSs of active UL bandwidth parts of the bands in the band combination - If there are two consecutive intra-band carriers in one band, µUL, 1 = max(µUL, 1-1, µUL, 1-2), where µUL, 1-1 and µUL, 1-2 are SCSs of active UL bandwidth parts of the carriers in the band - (working assumption) If two uplink switching are triggered and result in UL transmissions on more than 2 bands within any two consecutive reference slots, then the time duration between the end of all transmission(s) prior to the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time - The minimum separation time is a sum of X us and the switching gap required for the second uplink switching. - X us is subject to UE capability with a value set of {0us, 500us}

Figure 2 is the Working Assumption illustrative figure from RAN1 #111. Two back-to-back switches are scheduled on slot #4 and #5, the above WA requires the network to guarantee the minimum separation time between end of transmission in #1 and start of transmission in slot #5. Even UE doesn’t slot #2 and #3 transmit, the back-to-back switches issue still remains as the switching scheduling could arrive at slot #2 or #3. It should be noted that this issue doesn’t exist in the illustrative figure (Figure 3) of RAN1#111 as the UL Tx before 1st UL Tx switching is close to the switching period.

To fix this issue, we propose to revise minimum separation time to between start of first switching period (in slot #4) and start of all transmission(s) after the second uplink switching within the two reference slots. We provide an illustrative figure of the proposal in Figure 3.

Band A

Band B

Band C

1Tx

2Tx

gap

2Tx

gap

Dash line slot boundary

Scheduled UL Tx

1Tx

Min separation time, Xus

Slot

1

2

3

4

5

Figure 2 Illustrative examples of issue of current WA for minimum separation between two consecutive switches

Figure 3 Illustrative figure in RAN1 #111

Slot

1

2

3

4

5

Band A

Band B

Band C

1Tx

2Tx

gap

2Tx

gap

Dash line slot boundary

Scheduled UL Tx

1Tx

proposal

Figure 4 Illustrative examples of issue of proposal for minimum separation between two consecutive switches

Proposal 2: Agree on the following revised WA

- If two uplink switching are triggered and result in UL transmissions on more than 2 bands within any two consecutive reference slots, then the time duration between the end of all transmission(s) prior to start of the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time

  - The minimum separation time is a sum of X us and the switching gap required for the second uplink switching.

  - X us is subject to UE capability with a value set of {0us, 500us}

# Conclusions

In this paper, we have provided our views on Rel-18 UL Tx switching leftover issues and made the following proposals.

Proposal 1: For switching period location, if companies have strong concern on signaling overhead, choose Alt. 3 or 5 as they require less signaling compared with other alternatives.

Proposal 2: Agree on the following revised WA

- If two uplink switching are triggered and result in UL transmissions on more than 2 bands within any two consecutive reference slots, then the time duration between the end of all transmission(s) prior to start of the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time

  - The minimum separation time is a sum of X us and the switching gap required for the second uplink switching.

  - X us is subject to UE capability with a value set of {0us, 500us}