3GPP TSG-RAN WG1 #112                                                                                                                                	               R1-2301441

Athens, Greece, February 27 – March 3, 2023

Agenda item:	9.14.1

Source: 	Qualcomm Incorporated

Title: 	PRACH Coverage Enhancement

Document for:	Discussion and Decision

- Introduction

In RAN1 meeting #111, the following was agreed for PRACH repetition:

Agreement

For multiple PRACH transmissions with same Tx beam, support to differentiate at least between multiple PRACH transmissions and single PRACH transmissions.

Agreement

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, consider one or multiple of the following options.

- Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs.

- Option 2: Multiple PRACH are transmitted on separate ROs.

- Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs.

- Other options are not precluded.

- Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

Agreement

- Study at least the following case for multiple PRACH transmissions with different Tx beams.

  - UE uses different TX beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS

  - FFS: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs /CSI-RSs, where the different SSBs/CSI-RSs are not associated with the same RO.

  - Note: not related to decision on CFRA

Note: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs/CSI-RSs, where the different SSBs/CSI-RSs are associated with the same RO is not considered.

Working Assumption

Simulation results for multiple PRACH transmissions with different beam(s) and same beam(s) (baseline) to be discussed in the next meeting.

- Simulation assumptions in TR 38.830 are used as the starting point for the simulation.

    - Focus on FR2.

      - UE antenna configuration 2-2-2(baseline), 1-4-1(optional)

    - Performance metric: 0.1% false alarm, 1% miss-detection

    - Companies report the number of beams, the beam widths, beam correspondence assumption, and the boresights.

  - Channel model for link-level simulation: CDL-A defined in table 7.7.1-1 in TR 38.901.

  - Both that UE fulfills beamCorrespondence requirements Without UL-BeamSweeping and UE fulfils beamCorrespondence requirements With UL-BeamSweeping can be considered in the simulation are used as starting point for simulation.

Agreement

For multiple PRACH transmissions with same Tx beam, down-select one option from the following options.

- Option 1: gNB can only configure one value for the number of multiple PRACH transmissions.

- Option 2: gNB can configure one or multiple values for the number of multiple PRACH transmissions.

  - FFS: details

Agreement

- For multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt.

  - Note: whether to support multiple numbers of PRACH transmissions is separately discussed.

In this document, we present our views related to PRACH repetition and specification impacts.

- PRACH repetition with the same or different beams

One important question is whether PRACH repetition should be transmitted with the same beam or multiple UL Tx beams (or UL spatial filtering). Using multiple UL Tx beams can provide UL beam diversity. It can also provide opportunity for UL beam refinement for Msg3 and future UL transmission, provided that the best beam (among multiple UL beams) is indicated to the UE (after reception of PRACH). However, this indication needs to be provided explicitly or implicitly by Msg2 or Msg2 PDCCH, and the specification impacts and potential methods for this indication need to be studied.

Proposal 1: Study of PRACH repetition with multiple UL beams should focus on its benefit for UL beam refinement in RACH and the potential necessary specification impact, including the selection of UL Tx beam for Msg3.

- FFS interference impact of UL beam sweeping

Another benefit of PRACH repetition (other than increasing the coverage of PRACH) is to provide opportunity for gNB to perform gNB-side beam refinement. If gNB knows which PRACH repetition are transmitted with the same spatial filtering, it can check different refinement of gNB-side Rx beam for reception of different copies among those repetitions; and use the refined gNB-side beam for transmission and reception of future RACH messages. However, for this purpose, it is important that gNB and UE are on the same page about the set of PRACH repetitions that are supposed to be transmitted with the same beam.

Proposal 2: Support mechanism for gNB to know the subset of PRACH repetitions that are expected to be transmitted with the same UL Tx beam.

- FFS: required configuration by system information

- Criteria for using PRACH repetition and interaction with Msg3 repetition request

In this section, we consider criteria for using PRACH repetition by UE, the associated PRACH preambles, and the interaction of PRACH repetition with Msg3 repetition request (which was introduced in R17).

Criteria for using PRACH repetition

Similar to Msg3 repetition request, it is important to have criteria for using PRACH repetition, to avoid excessive usage of this option by UEs (which can result in excessive congestion of resources). Because the need for PRACH repetition is linked to the coverage situation of the UE, some criteria similar to what was used for Msg3 repetition request (i.e. based on a configured threshold on SS-RSRP) can be useful for triggering PRACH repetition, as agreed in RAN1 meeting #111. The criteria can be also dependent on UE power class because the final reception power of PRACH is dependent both on the path loss (which is related to SS-RSRP) and the UL Tx power. Also, it is better to have more relaxed criteria in case of previous failed attempt of PRACH.

Proposal 3: UE should use PRACH repetition based on criteria that depends on the SSB-based RSRP and the following parameters:

- UE power class ​

- Prior failed attempts of PRACH

- FFS: linkage to the criteria for Msg3 repetition request

Another issue is whether UE can select among multiple PRACH repetition numbers, or only one repetition number is used. Using multiple options for PRACH repetition number allows UE to better adapt PRACH repetition to its conditions and avoid excessive usage of PRACH resources (by unnecessarily using a large repetition number because of having only one option).

Proposal 4: Support selection of PRACH repetition number by UE (among a set of preconfigured or predefined repetition numbers) based on at least SBB-based RSRP

- FFS: dependence on UE power class ​

- FFS: dependence on prior failed attempts of PRACH

Interaction of PRACH repetition with Msg3 repetition request

Because both Msg3 repetition request and PRACH repetition are expected to be for UEs with similar condition (i.e. needing coverage enhancement for UL RACH messages), usage of PRACH repetition by UE may be considered as an indication of Msg3 repetition request too. This approach can make PRACH resource usage more efficient (by avoiding excessive partitioning).

Proposal 5: Support combination of PRACH resources (and/or RACH occasions) and PRACH repetition to indicate the Msg3 repetition request by UE​.

- e.g. Support the option that PRACH repetition can automatically indicate Msg3 repetition request (irrespective of PRACH resource)

Differentiation between UEs w/ and w/o multiple PRACHs

Under a cell, some UEs (e.g., at the cell edge) may need to support multiple PRACH transmission while the other UEs (e.g., legacy UEs and R18 UEs in good locations) may not need multiple PRACH transmission. If the gNB performs combining across the multiple PRACH transmissions, it should know whether UE performs multiple PRACH transmission or not. Hence, it is beneficial to discuss how the UEs with multiple PRACH transmission and the UEs without multiple PRACH transmission should be differentiated. This differentiation can be through sharing ROs with different preamble subsets (for PRACH repetition and legacy) or through different set of ROs for repetition.

Proposal 6: Support differentiating multiple PRACH repetition and single PRACH transmission via different subsets of preambles on shared ROs.

- FFS: relation to the preamble subset for requesting Msg3 repetition.

- Multiple PRACH transmission configuration and resource mapping

For multiple PRACH transmission, the gNB is expected to configure multiple PRACH transmissions e.g., the number of repetitions. Depending on UE RRC state, such configuration can be conveyed to the UE in cell-specific signalling (e.g., in idle/inactive states) or in UE-specific signalling (e.g., in connected state).

Proposal 7: Discuss how to configure multiple PRACH transmissions (e.g., the number of repetitions).

# For preambles that are used during PRACH repetition, it is already agreed that using the same preamble for all repetitions should be supported. The remaining question is whether to also support using different preambles for different repetitions of the same RACH attempt.

# Proposal 8: In case different preambles are utilized in different PRACH transmissions in one RACH attempt, they should be linked with a one-to-one mapping, and all should be determined based on the preamble used for the first repetition.

PRACH repetition for CFRA

PRACH repetition can be useful for both CBRA and CFRA. For CFRA, having RRC connection can provide more flexibility to the gNB for providing UE-dedicated configuration about the details of PRACH repetition, including the triggering criteria.

Proposal 9: In addition to support PRACH repetition for CBRA, support PRACH enhancement in CFRA, via RRC configuration of rules for selection of CFRA PRACH transmission method based on SSB measurements and/or past measurements.

Resource mapping for PRACH repetition

When mapping the PRACH transmission to the valid ROs, there could be two alternatives: one is to map the transmissions in frequency domain first and then time; the other is to map the transmission in time only. The power gain may not be achieved in the former approach. Hence, we prefer to map the PRACH transmissions in time only as being proposed in the following proposal:

Proposal 10: PRACH repetitions are only transmitted in the valid ROs associated with the same SSB at different time with the following order:

- First, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot.​

- Second, in increasing order of indexes for PRACH slots.​

- Third, in increasing order of indexes for PRACH association period.

Figure 1: N=1/2, msg1-FDM=2, NTxSSB=2, repetition =4

In a time instance, there are RACH configurations that allow multiple ROs associated with the same SSB that are frequency multiplexed e.g., when msg1-FDM > 1 and one SSB is associated with multiple ROs. Such configurations would allow the opportunities for UE to send PRACH in different frequencies at different times for frequency diversity. Hence, it is beneficial to define some rule to enable frequency hopping for multiple PRACH transmission. Furthermore, when defining the rule, we should take into account the PRACH transmission latency. In particular, if the frequency hopping rule should considers whether there is valid RO in the time instance if the RO that would have been used is invalid. An example provided in Figure 2 illustrates that if the RO#1 would have been used for the 4th PRACH repetition is invalid, the UE could use RO#0 in the same time instance for transmitting the 4th PRACH repetition instead of postponing it to the next time instance.

Figure 2: Frequency hopping for PRACH transmission handling with invalid RO

Proposal 11: For multiple PRACH transmissions with same beam, the starting RB of ROs can be different at different time instances for multiple PRACH transmissions.

- FFS: determine starting RB of ROs at different times

- FFS: handle transmission at time having multiple ROs associated with the same SSB but the RO that would have been use for PRACH transmission is invalid.

PRACH repetition counting

It is critical to understand how the number of PRACH repetition is counted. Similar to R17 PUSCH repetition counting based on available slot, the question here is that whether we should count the number of repetitions based on the valid ROs.

Proposal 12: Discuss how to determine the first PRACH repetition transmission.

Proposal 13: The counting of PRACH repetitions is based on the valid ROs.

- Impact on RAR monitoring and RA-RNTI

In the existing specification, after PRACH transmission, the UE attempts to detect a DCI format 1_0 with CRC scrambled by RA-RNTI during a window that starts at least one symbol after the last symbol of PRACH occasion corresponding to the PRACH transmission. Below is a related snapshot of TS 38.213:

However, since there will be multiple PRACH occasions corresponding to multiple PRACH transmissions, the question here is that which RACH occasion shall the UE use to determine the starting of the monitoring window? In RAN1 meeting #110-bis-e, it was agreed to downselect between one RAR window per each repetition (Option 1) or one RAR window after all PRACH repetitions (Option 2)

Proposal 14: Support only one RAR window for all the PRACH repetitions in one PRACH attempt (option 2).

- FFS: the start of RAR window.

RA-RNTI consideration

The UE needs to compute RA-RNTI in detecting the DCI format 1_0 with CRC scrambled by RA-RNTI. The RA-RNTI computation depends on the index of the first OFDM symbol of the PRACH occasion corresponding to PRACH transmission. Below is a related snapshot of TS 38.321:

However, since there will be multiple PRACH occasions corresponding to multiple PRACH transmissions, the question here is that which RACH occasion shall the UE use to compute RA-RNTI? From our perspectives, the PRACH occasion for RA-RNTI computation is the occasion that the UE uses to determine the start of RAR window.

Proposal 15: The PRACH occasion for RA-RNTI computation is the occasion that the UE uses to determine the start of RAR window.

- Conclusions

In this paper, we discussed different aspects related to PRACH repetition, and made the following proposals:

Proposal 1: Study of PRACH repetition with multiple UL beams should focus on its benefit for UL beam refinement in RACH and the potential necessary specification impact, including the selection of UL Tx beam for Msg3.

- FFS interference impact of UL beam sweeping

Proposal 2: Support mechanism for gNB to know the subset of PRACH repetitions that are expected to be transmitted with the same UL Tx beam.

- FFS: required configuration by system information

Proposal 3: UE should use PRACH repetition based on criteria that depends on the SSB-based RSRP and the following parameters:

- UE power class ​

- Prior failed attempts of PRACH

- FFS: linkage to the criteria for Msg3 repetition request

Proposal 4: Support selection of PRACH repetition number by UE (among a set of preconfigured or predefined repetition numbers) based on at least SBB-based RSRP

- FFS: dependence on UE power class ​

- FFS: dependence on prior failed attempts of PRACH

Proposal 5: Support combination of PRACH resources (and/or RACH occasions) and PRACH repetition to indicate the Msg3 repetition request by UE​.

- e.g. Support the option that PRACH repetition can automatically indicate Msg3 repetition request (irrespective of PRACH resource)

Proposal 6: Support differentiating multiple PRACH repetition and single PRACH transmission via different subsets of preambles on shared ROs.

- FFS: relation to the preamble subset for requesting Msg3 repetition.

Proposal 7: Discuss how to configure multiple PRACH transmissions (e.g., the number of repetitions).

# Proposal 8: In case different preambles are utilized in different PRACH transmissions in one RACH attempt, they should be linked with a one-to-one mapping, and all should be determined based on the preamble used for the first repetition.

Proposal 9: In addition to support PRACH repetition for CBRA, support PRACH enhancement in CFRA, via RRC configuration of rules for selection of CFRA PRACH transmission method based on SSB measurements and/or past measurements.

Proposal 10: PRACH repetitions are only transmitted in the valid ROs associated with the same SSB at different time with the following order:

- First, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot.​

- Second, in increasing order of indexes for PRACH slots.​

- Third, in increasing order of indexes for PRACH association period.

Proposal 11: For multiple PRACH transmissions with same beam, the starting RB of ROs can be different at different time instances for multiple PRACH transmissions.

- FFS: determine starting RB of ROs at different times

- FFS: handle transmission at time having multiple ROs associated with the same SSB but the RO that would have been use for PRACH transmission is invalid.

Proposal 12: Discuss how to determine the first PRACH repetition transmission.

Proposal 13: The counting of PRACH repetitions is based on the valid ROs.

Proposal 14: Support only one RAR window for all the PRACH repetitions in one PRACH attempt (option 2).

- FFS: the start of RAR window.

Proposal 15: The PRACH occasion for RA-RNTI computation is the occasion that the UE uses to determine the start of RAR window.

- References

- RP-220937, Revised WID on Further NR coverage enhancements, 3GPP TSG RAN Meeting #95-e

- Chairman’s notes, RAN1 meeting #111