3GPP TSG RAN WG1 #112                                   R1-2301544

Athens, Greece, February 27th – March 3rd, 2023

Source:	Sharp

Title:	Discussion on physical channel design framework for NR sidelink evolution on unlicensed spectrum

Agenda Item:	9.4.1.2

Document for:	Discussion and Decision

Introduction

In RAN#97e meeting, Rel-18 work item on support of [1] has the following objectives to study and specify support of sidelink on unlicensed spectrum.

2. Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98. - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

In this contribution, we share some views on modifications of physical channel design for adapting sidelink communication on unlicensed spectrum.

Discussions

Physical layer structure

Sub-channel definition and resource allocation

Sub-channel for 15KHz and 30kHz

It has been agreed in previous RAN1 meeting that 1 sub-channel equals K interlace(s). Moreover, several options regarding mapping between sub-channel and interlace were agreed for further study as below.

Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding 1 sub-channel equals K interlace(s) - At least K=1 and K=2 is supported for 15 kHz SCS - At least K=1 is supported for 30 kHz SCS - FFS: details related to multiple RB sets Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, RAN1 further study the followings: - Option 1: 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool - Option 2: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 3: 1 sub-channel is defined across all RB sets within the resource pool, i.e., 1 sub-channel includes K interlace(s) across all RB sets within the resource pool - Option 4: 1 sub-channel is defined within 1 RB set or 2 adjacent RB sets, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 5: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly across different RB sets within the resource pool, then across different interlaces in the RB set - FFS: whether/how to use intra-cell guardband PRBs

As specified in the above agreement, 5 options can be generally split into two cases, i.e., 1 sub-channel is defined within 1 RB set or 1 sub-channel is defined across all RB sets within a resource pool. Regarding the mapping between sub-channel and interlace, whether 1 sub-channel is defined within 1 RB set or across all the RB set(s) is also related to whether to reuse NR-SL frequency domain resource indication or NR-U frequency domain resource indication. NR-SL frequency domain resource indication seems to be suitable for the case that 1 sub-channel is defined within 1 RB set, while NR-U frequency domain resource indication seems to be suitable for the case that 1 sub-channel is defined across all the RB set(s) within a resource pool.

For option 1 and option 3, in our views, there is not too much difference between them in terms of resource allocation, although option 1 literally defines 1 sub-channel to be within 1 RB set. For both option 1 and option 3, when referring to a sub-channel in a resource pool but without explicitly indicating RB set index, specific resource blocks for the sub-channel would be of ambiguity. A sub-channel with a same sub-channel index in both option 1 and option 3 still spans across RB sets in the resource pool. Therefore, explicit RB set index is needed for indicating resource blocks in which RB set(s) for frequency domain resource indication. The existing NR-U frequency domain resource indication is suitable for both option 1 and option 3.

For option 2, 1 sub-channel is defined within 1 RB set and sub-channels in different RB sets have different sub-channel indexes. For option 2, when referring to a sub-channel in resource pool, corresponding RB set index is implicitly indicated by the sub-channel index. NR-SL frequency domain resource indication is suitable for the option 2.

Option 4 has similarities to option 2 in sub-channel RB mapping and indexing. Compared to option 2, option 4 is more flexible to allow one sub-channel to be defined across two adjacent RB sets. However, according to currently agreed values of K for 15kHz and 30kHz, one sub-channel is basically defined within 1 RB set. Option 4 aims to solve sub-channel mapping if K=2 interlaces is agreed for 30kHz. However, there was a long and extensive debate as to whether to support K=2 for 30kHz in previous RAN1 meeting. And no consensus was reached to support K = 2 interlaces for 30kHz. Option 4 should not be taken into consideration in sub-channel mapping discussion unless K=2 interlaces for 30kHz is agreed.

For option 5, option 5 differs from option 2 in how to index sub-channel. The most striking difference is that, sub-channels in option 2 are incrementally indexed in order of interlaces within an RB set firstly, while sub-channels in option 5 are incrementally indexed in order of RB sets firstly. In consequence of mapping sub-channel by option 5, one PSSCH transmission over multiple sub-channels always requires one PSSCH transmission on multi channels according to NR-SL frequency domain resource indication. It has to be noticed that NR-U UL channel access procedure was agreed as baseline for transmission on multiple channels in SL-U. As long as one channel with allocated sub-channel(s) is sensed to be busy, the whole PSSCH transmission needs to drop. In this respect, option 5 increases risk of transmission blocking and has less transmission opportunities.

In NR-SL, sub-channel concept is treated as smallest granularity involved in several SL fundamental procedures, such as, resource sensing/selection procedure, CBR-based congestion control. Therefore, to keep the existing NR-SL procedures as much as possible, we prefer to define 1 sub-channel to be within 1 RB set.

Regarding frequency resource scheduling, the NR-SL frequency domain resource indication is able to schedule different numbers and different indexes of interlaces in different RB sets while the indicated interlace indexes should be contiguous. The NR-U frequency domain resource indication only indicates same number(s) and same index(s) of interlaces in different RB sets. In addition, the NR-SL frequency domain resource indication is designed to be able to indicate same or different frequency domain resources for up to 3 slots. The NR-U frequency domain resource indication is designed for one slot. From this point of view, it seems NR-SL frequency domain resource indication slightly has scheduling flexibility.

Above all, among option 1 and option 2, each option can work and we do not see one option has overwhelming advantage against the other option. From perspective of specification effort, option 2 is preferred for mapping between sub-channel and interlace for interlace RB-based PSCCH/PSSCH transmission in SL-U. And frequency domain resource indication in NR SL Rel-16 is reused.

In addition, the options including option 2 in above agreement seem not explicitly specify how to index sub-channel across interlaces within an RB set. From options themselves, it is not clear that sub-channel is indexed in increasing order of interlace indexes or in decreasing order of interlace indexes within an RB set. According to RAN1#111 discussion and FL summary [2], the intention of sub-channel indexing within an RB set is such that 1 sub-channel is incrementally indexed in increasing order of interlace indexes starting from interlace index #0 within an RB set. For example, option 2 is taken as an illustration of sub-channel indexing within an RB set, as shown in Figure 1. Sub-channel #0 starts from an RB of interlace #0 with lowest PRB index within a resource pool, which is different from NR-SL where an RB with lowest index of sub-channel#0 indicates start position of the resource pool. As in Figure 1, an RB corresponding to start position of the resource pool belongs to sub-channel#3. To avoid ambiguity on how to index sub-channel within an RB set and consolidate understanding of sub-channel indexing within an RB set agreed in last RAN1 meeting, a modification of option 2 is proposed.

Figure 1: Illustration of sub-channel indexing within an RB set for option 2

Proposal 1: For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and interlace, the following option 2 with modification is agreed.

  - Option 2: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly in increasing order of interlace indexes starting from interlace index#0 within an RB set, then in increasing order of across different RB sets indexes within the resource pool.

Proposal 2: Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, FRIV in NR SL Rel-16 is reused.

Sub-channel for 60KHz

To support SL transmission on unlicensed band where OCB and PSD is required, interlace RB-based transmission has been agreed to meet OCB and PSD requirement for PSSCH transmission under 15kHz and 30kHz SCS, while the solution on how to meet OCB and PSD requirement for PSSCH transmission under 60kHz SCS has not been discussed well. In view of the fact that the concept of interlace has not been introduced for 60kHz in NR-U, it seems not suitable to introduce interlace for 60kHz SCS, which would cause additional specification effort. One straightforward solution is to define only 1 sub-channel in 1 RB set, that is, all RBs in each RB set of a resource pool are mapped to 1 sub-channel. According to Table 5.3.2-1 in the TS38.101, maximum transmission bandwidth configuration NRB is 24 for 20MHz under 60kHz. 1 sub-channel with 24 RBs with respect to 60kHz occupies 17.28MHz channel bandwidth which can meet the requirement of more than 80% of 20MHz.

Table 5.3.2-1(in TS38.101): Maximum transmission bandwidth configuration NRB

On the other hand, even if a much finer scheduling granularity is required, 1 RB set with 24 PRBs can possibly include at most 2 sub-channels. That is because channel bandwidth only corresponding to at least 23 PRB with respect to 60kHz can be larger than 80% of 20MHz. There are possibly two ways to map 24 PRBs in an RB set to 2 sub-channel where each sub-channel can occupy channel bandwidth corresponding to bandwidth of 23 PRBs. One way is to simply assign RBs with even indexes to a sub-channel and to assign RBs with odd indexes to the other sub-channel for each RB set. The other way is to use a similar principle that is under discussion for PSFCH transmission under 60kHz. As has been discussed for PSFCH transmission under 60kHz SCS, RB set-edge PRBs can be used as common PRB such that one PSFCH transmission can occupy channel bandwidth larger than 80% of nominal channel bandwidth. That is, a single PPB on one side/edge of RB set is mapped to a sub-channel #0, while a single PRB on the other side is mapped to a sub-channel #1. Remaining PRBs in an RB set can be divided into two consecutive RB groups for two sub-channel. The consecutive RB group can be assigned to a different sub-channel from the sub-channel where RB on the edge near to the consecutive RB group is mapped. Figure 2 shows an example of sub-channel mapping in an RB set under 60kHz. As shown in the Figure 2 (A) and (B), one sub-channel can span channel bandwidth with 23 PRBs such that the corresponding NCB (i.e., 16.56MHz) can meet OCB requirement.

- (B)

Figure 2: sub-channel mapping in an RB set for 60kHz SCS

Proposal 3: Regarding PSCCH/PSSCH transmission under 60kHz SCS, study following options to meet OCB and PSD requirement,

- Option 1: One RB set includes only 1 sub-channel.

  - In each RB set, all resource blocks are mapped to one sub-channel.

- Option 2: One RB set includes 2 sub-channels where each includes 12 PRBs.

  - FFS details of how to map RBs in one RB set to 2 sub-channels

Time resource allocation for MCSt

In RAN1#110 meeting, the following was agreed,

Agreement For PSCCH and PSSCH resource indication in time/frequency domain: - For time domain: R16 NR SL TRIV is reused as baseline - For frequency domain: - further study sub-channel indexing and resource indication - FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission

One FFS in above agreement is whether to enhance the R16 NR SL TRIV/FRIV if new feature is introduced in SL-U. As known, it was agreed in AI 9.4.1.1 that multi-consecutive slots transmission is supported for Mode 1 and Mode 2 resource allocation in SL-U.

Agreement Multi-consecutive slots transmission (MCSt) is supported for Mode 1 and Mode 2 resource allocation in SL-U. - FFS details

However, the existing time resource assignment in Rel-16 only supports 2 or 3 consecutive or non-consecutive slots transmission. Up to 3 consecutive slots seems restrictive and is not suitable to adapt to multi-consecutive slots transmission.

Proposal 4: Enhance on R16 NR-SL time resource assignment to support more than 3 slot consecutive transmission for multi-consecutive slots transmission in SL-U.

TBS determination

In SL-U, a number of aspects impact upon TBS determination, e.g.

- With the support of two candidate starting symbols within a slot, the total number of REs used for two (re)transmissions of a TB would be remarkably different if one uses the first candidate starting symbol and the other uses the second candidate starting symbol.

- In interlace RB-based PSSCH transmissions, since an interlace may contain 10 or 11 RBs within one RB Set, there may be more than one sub-channel sizes in a resource pool. Therefore, (re)transmissions of a given TB, even with a same number of sub-channels, may correspond to different total number of RBs.

- Use of intra-cell guard band (i.e. when the two adjacent RB Sets are both used for transmission) may add additional RBs for some of the (re)transmissions of a TB.

Reference symbol length for TBS determination

Regarding how to determine reference symbol length for TBS determination in a resource pool with 2 candidate starting symbols, four options were agreed for down-selection in last RAN1#111 meeting.

Agreement If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - TBS is determined based on a reference symbol length, down-select one of the followings: - Option 1: The reference symbol length is dynamically indicated by Tx UE - Option 2: The reference symbol length is determined based on 1st starting symbol - Option 3: The reference symbol length is determined based on 2nd starting symbol - Option 4: The reference symbol length is (pre-)configured

Option 1 with dynamic indication seems not much helpful given the UE is not aware of the channel status when the UE generates a TB. TBS determination assuming the second candidate starting symbol would cause quite a few changes to the legacy TBS determination procedure, therefore, we propose to always determine TBS based on the first candidate starting symbol, i.e., the option 2 in above agreement.

Proposal 5: If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- TBS is determined based on a reference symbol length, the following option 2 is supported:

  - Option 2: The reference symbol length is determined based on 1st starting symbol

Reference number of PRBs for TBS determination

On the issue of interlaced-RB based PSSCH transmissions and use of intra-cell guard band, this can be addressed by using a reference total number of RBs, for example, by calculating the total number of RBs in TBS determination by using the size of the lowest sub-channel in the resource pool. This simple approach also has the benefit of being backward compatible with legacy NR SL.

Proposal 6: When determining TBS in SL-U, the total number of RBs are calculated by multiplying the size of the lowest sub-channel in the resource pool and the total number of allocated sub-channels.

PSFCH transmission for SL-U

PSFCH transmission under 15 kHz and 30kHz

The following related to PSFCH transmission under 15kHz and 30kHz was agreed in RAN1#111-e meeting.

Agreement Regarding PSFCH transmission under 15 kHz and 30 kHz SCS, RAN1 continues studying the following updated alternatives: - Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - FFS: value of K3 - Alt 2-1a: each PSFCH transmission occupies 1 interlace, and further apply frequency-domain OCC - FFS: details of FD-OCC, e.g., OCC length, RB-level, RE-level, etc. - Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace - FFS: value of K1 - Alt 2-3a: each PSFCH transmission occupies 1 interlace - Alt 2-4a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE uses different cyclic shifts on different PRBs in the interlace - Alt 3-1a: each PSFCH transmission occupies 1 dedicated PRB and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - The above dedicated PRB and common PRBs are within 1 interlace - FFS: value of K2 - Alt 3-2a: each PSFCH transmission occupies 1 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of a RB set - FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth - FFS: whether IBE issue exists and whether/how to address it - Note: in the above descriptions - The dedicated PRB/cyclic shift conveys ACK/NACK information - Note: as previously agreed: to meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS. Agreement Slots with PSFCH symbols only have 1 candidate starting symbol for PSCCH/PSSCH. Agreement To address PSFCH transmission dropping due to LBT failure, RAN1 down-select one of followings, or support the combination of followings: - Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - FFS other details, e.g., HARQ-ACK timeline - Alt 2: PSFCH occasions are dynamically indicated - FFS: Whether/how to handle the case where some TB’s corresponding PSFCH cannot be transmitted within the same or different COT - FFS other details, e.g., dynamically indicate one or more PSFCH transmission(s), container of the indication, etc. - FFS: Whether such PSFCH occasions are within the same or different COT of corresponding PSSCH - FFS: Whether/how to address PSFCH collision if any - FFS: Whether/how to handle the linearly decreased PSFCH capacity

It was agreed as below that the interlaced transmission is supported for PSFCH in SL-U and at least there is 1 PSFCH occasion per PSCCH/PSSCH transmission. Interlace is a concept across all RB sets in a SL BWP. Regarding one PSFCH resource, each PSFCH should be confined in an interlace in an RB set to reduce the probability of LBT failure.

Proposal 7: For interlaced transmission for PSFCH in SL-U,

- Frequency resource of a PSFCH is configured in an interlace in an RB set.

One controversial discussion point is how to support RB-based interlace for PSFCH transmission to meet OCB and PSD requirement. Three groups of alternatives regarding PSFCH transmission as above were listed for further study.

Alt.1-1a is intended to solve the PSFCH capacity issue given that the whole PRBs of an interlace needs to be used as one PSFCH resource. Alt.1 utilizes a common interlace and K3 dedicated PRB(s) where the common interlace is only used to meet the OCB requirement and does not carry any HARQ-ACK information. With Alt.1, K3 dedicated PRBs are used to carry HARQ-ACK information, while most of the transmission power would be wasted in the common interlace which carry no valid HARQ-ACK information. Moreover, it is inevitable that a dedicated PRB coexists with an RB of a common interlace within a same 1MHz bandwidth. If K3>1 dedicated PRBs are agreed, how to distribute K3 dedicated PRBs should be further discussed to alleviate the power limitation issue. Otherwise, contiguous dedicated PRBs would aggravate the power limitation issue.

Observation 1: Alt.1-1a with using common interlace may have power limitation issue, and K3>1 dedicated PRBs may aggravate the power limitation issue if not equally distribute K3>1 dedicated PRBs to different frequency bandwidth.

The group of Alt.2 (i.e., Alt 2-1a, Alt 2-2a, Alt 2-3a, Alt 2-4a) is to use dedicated interlace to carry valid HARQ-ACK information. For an interlace with 15kHz or 30kHz, frequency interval between two consecutive resource blocks of one interlace exceeds 1MHz bandwidth. The alternatives in the group of Alt.2 do not have power limitation issue which may exists in Alt.1-1a.

For the group of Alt.3 (i.e., Alt 3-1a and Alt 3-2a), a constraint is imposed for Alt 3-1a that dedicated PRB and common PRBs should be within 1 interlace, while there is no such constraint for Alt 3-2a. As has been agreed in previous RAN 1 meeting, RB-based interlace is agreed at least for PSFCH transmission under 15 kHz and 30 kHz SCS. Therefore, Alt 3-2a seems to beyond what has been agreed and should be removed. Likewise, the alternative 3-1a does not have power limitation issue which may exists in Alt.1-1a.

Overall, PSFCH performance by using the group of Alt.2 and alt.3-1a would be better than that by using Alt.1-1a.

The group of Alt.2 is divided into sub-alternatives to take PSFCH capacity issue into consideration. And alternative 3-1a aims to solve the PSFCH capacity issue as well. Therefore, it is important to firstly conclude whether the PSFCH capacity issue exists or not. Regarding the PSFCH capacity issue, according to the Rel-16/17 PSFCH design principle, the required quantity of the PSFCH resources in a resource pool depends on the number of sub-channels in the resource pool, the configured period, the number of cyclic shift pair indexes. As in the WID of Rel-18 SL, the primary motivation of supporting sidelink over unlicensed spectrum is to increase sidelink data rate. Therefore, it is foreseeable that a sub-channel would occupy several interlaces or a PSSCH would occupy several sub-channels. In this regard, the PSFCH capacity issue can be alleviated in SL-U.

Observation 2: Supporting increased SL data rate over unlicensed spectrum may, in a sense, alleviate the PSFCH capacity issue from interlaced PSFCH transmission.

In addition, TS38.321 in NR SL specifies that, for groupcast, if a group size of recipient of a PSSCH is greater than the number of candidate PSFCH resources, the groupcast HARQ feedback option 1 would be selected. Therefore, same principle can be reused in SL-U if a group size of recipient of a PSSCH is larger than the number of candidate PSFCH resources.

Proposal 8: For groupcast HARQ feedback, if a group size of recipient of a PSSCH is larger than the number of candidate PSFCH resources, a same principle in NR Rel-16 SL is followed, i.e., HARQ-ACK feedback option 1 is used.

Proposal 9: Regarding PSFCH transmission under 15kHz and 30kHz, the following alternative 2-3a is supported:

- Alt 2-3a: each PSFCH transmission occupies 1 interlace.

PSFCH transmission under 60kHz

The following related to PSFCH transmission under 60kHz was agreed in RAN1#111-e meeting.

Agreement Regarding PSFCH transmission under 60 kHz SCS, further study the following alternatives: - Alt 1: Each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs - FFS details - Alt 2: Each PSFCH transmission occupies some dedicated PRBs - FFS details

To meet OCB and PSD requirement for PSFCH transmission under 60kHz SCS, PRBs used for PSFCH transmission should occupy channel bandwidth larger than 80% of nominal channel bandwidth. For an RB set, two RB set-edge PRBs should be used as common PRB to at least meet OCB requirement of 80% of 20MHz. Regarding Alt 1, it is unclear how different PSFCH transmissions in same symbols can be guaranteed to meet OCB requirement if different PSFCH transmissions occupy their own dedicated PRBs. Therefore, Alt 1 is a feasible solution for PSFCH transmission under 60kHz SCS to meet the OCB requirement.

Proposal 10: Regarding PSFCH transmission under 60kHz SCS, each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs.

S-SSB transmission

In RAN1#110bis-e, there is an FFS regarding the usage of PRBs within intra-cell guard-bands for S-SSB transmission as below.

Agreement Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets - Such PRBs are not used for PSCCH transmission - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission

Regarding the FFS on S-SSB, our view is that the same principle for other channels/signals (and the same principle in NR-U) should be followed, i.e. such PRBs should not be used for S-SSB transmission, or else there will be a problem in coexisting with NR-U.

Proposal 11: PRBs within intra-cell guard band of two adjacent RB sets are not used for S-SSB transmissions.

As has been debated in last several RAN1 meetings, it is difficult to conclude whether temporary exemption of OCB requirement is applicable for S-SSB transmission. The following was agreed in RAN1#111 to accommodates both solutions to meet OCB and PSD requirement and solution to apply OCB exemption to S-SSB transmission.

Agreement For S-SSB transmission, down-select one or more of the following for 15 kHz and 30 kHz SCS: - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - Option 1-2: Using interlaced RB transmission for PSBCH only, and apply OCB exemption to S-PSS and S-SSS - Option 3-1: Repeat S-PSS/S-SSS/PSBCH N times in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of N (e.g., N=2) - FFS gap of 0 - Option 3-2: Repeat only S-PSS/S-SSS K times in frequency domain, and PSBCH is rate matched. There is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of K - FFS gap of 0 - FFS PSBCH resource - Option 3-3: keep the legacy S-PSS/S-SSS/PSBCH while repeating PSBCH N times in frequency domain and rate-matching PSBCH to S-PSS/S-SSS symbols, and there is a gap between the PSBCH repetition(s) to meet OCB requirements - FFS details, e.g. the length of gap is (pre-)configured or pre-defined, value of N - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH - For Option 1-1 and 1-2 above - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set - FFS: whether transient period issue exists and whether/how to address it

Regarding down-selection among solutions to meet OCB and PSD requirements, we think the following design principles should be followed:

- Principle 1: An S-SSB transmission should be completely accommodated by one RB Set. Otherwise transmission of S-SSBs would only be possible in wideband operation, and the probability of dropping an S-SSB transmission due to LBT failure would be significantly increased.

- Principle 2: The time/frequency resources for S-SSBs should be configured within a SL BWP rather than within a resource pool. Otherwise UE behaviours for transmission/reception of S-SSBs would be quite uncertain, e.g. it may depend on which resource pool is selected for data transmission.

- Principle 3: Detection of S-SSBs should not be more complex than that in Rel-16. For example, the time/frequency locations of all candidate S-SSBs should be fully determined prior to detection of S-SSBs.

- Principle 4: A unified solution is used for all supported SCSs and all supported carrier bandwidths, regardless of the number of RB Sets configured in a SL BWP.

With Option 1-1 and option 1-2, the 1-slot/11-RB structure of S-SSB since Rel-16 can only be directly reused for some of the channel bandwidth and SCS combinations (e.g. 20 MHz with 15 kHz SCS where some interlaces have 11 RBs in one RB Set), and not for others (e.g. 20 MHz with 30 kHz SCS where all interlaces have 10 RBs in one RB Set); And another problem of Option 1-1 and option 1-2 is that if interlaced transmission is agreed for 15kHz and 30kHz, a different solution should be further studied given there is no definition of interlace for cover 60 kHz SCS. Different designs of S-SSB for different SCS require additional specification workload.

In addition, compared with option 3-2 and option 3-3, option 3-1 seems to be a more straightforward solution and require less specification workload with only some discussion to define repetition numbers and gap between each repetition in the frequency domain. For 60 kHz SCS, option 3-1 can be applied as well.

Proposal 12: For S-SSB transmission, the following option is supported to meet OCB and PSBCH requirement for 15 kHz and 30 kHz SCS:

- Option 3-1: Repeat S-PSS/S-SSS/PSBCH N times in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement.

  - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of N (e.g., N=2)

  - FFS gap of 0

On the other hand, if the temporary exemption of OCB requirement can be concluded to be applicable for S-SSB transmission, specification effort regarding S-SSB format would be minimized. According to ETSI EN 301.893, short transmissions during a COT can be temporarily allowed to not comply with the OCB requirement. In order to transmit the S-SSB, the UE might perform a LBT to initiate a COT. In our view, during the initiating COT, the S-SSB transmission can be considered as a short and temporary transmission and the temporary exemption of OCB requirement can be applicable for the S-SSB transmission.

In addition, if temporary exemption of OCB requirement is supported for S-SSB transmission, the case of S-SSB transmission with 15kHz needs to further study. The relevant FL proposal (i.e., Proposal 5-6 from [2]) for discussion was captured as following.

Proposal 5-6: Under temporary exemption of OCB requirement, regarding how to meet the minimum of 2 MHz requirement for 15 kHz SCS, RAN1 further study the followings: - Alt 1: repeat all or part of S-PSS/S-SSS/PSBCH - Alt 2: PSBCH spans over 12 PRBs, and is wrapped around S-PSS and S-SSS - [Alt 3: use higher SCS than 15kHz for S-PSS/S-SSS/PSBCH in a SL BWP with 15kHz] - FFS details

That is because S-SSB transmission with 11 RBs under 15kHz SCS occupies 1.98MHz channel bandwidth, which cannot meet the minimum of 2 MHz requirement. Three alternatives were listed in proposal 5-3d to solve the issue. We provide pros/cons on these three alternatives from our perspective.

- In terms of S-SSB format, both Alt.1 and Alt.2 require change to the legacy Rel-16 S-SSB format.

- In terms of coverage performance, Alt.3 provides wider bandwidth in the frequency domain than Alt.1 and Alt.2, which a better coverage is provided if considering the PSD limit with 10dBm/MHz.

- In terms of time duration, Alt.1 and Alt.2 span 1ms duration while Alt.3 spans 0.5ms or 0.25ms duration depending on the used SCS.

- In terms of UE complexity, Alt.3 would require a SCS switching between S-SSB and PSCCH/PSSCH. The total time of SCS switching and TX/RX switching can be accommodated in a gap symbol with 15kHz prior to S-SSB transmission and in remaining duration (e.g., 0.5ms or 0.75ms) within a S-SSB slot after S-SSB. It should be noted that a Rel-15 UE can be configured with different SCS of SSB from that of BWP. Moreover, SCS switching is not equivalent to BWP switching where most of the BWP switching delay are caused due to adapting different RRC configurations.

- In terms of specification impact, Alt.3 should have less impact on specification compared to Alt.1 and Alt.2.

Therefore, Alt. 3 is a feasible and straightforward solution.

Proposal 13: If temporary exemption of OCB requirement for S-SSB transmission is concluded to be supported, regarding how to meet the minimum of 2 MHz requirement under 15 kHz SCS, the following option is supported:

- Use higher SCS than 15kHz for S-PSS/S-SSS/PSBCH in a SL BWP with 15kHz.

Conclusion

In this contribution, we have discussed our views on physical channel design for adapting sidelink communication on unlicensed spectrum and have the following proposals and observation.

Proposal 1: For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and interlace, the following option 2 with modification is agreed.

  - Option 2: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly in increasing order of interlace indexes starting from interlace index#0 within an RB set, then in increasing order of across different RB sets indexes within the resource pool.

Proposal 2: Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, FRIV in NR SL Rel-16 is reused.

Proposal 3: Regarding PSCCH/PSSCH transmission under 60kHz SCS, study following options to meet OCB and PSD requirement,

- Option 1: One RB set includes only 1 sub-channel.

  - In each RB set, all resource blocks are mapped to one sub-channel.

- Option 2: One RB set includes 2 sub-channels where each includes 12 PRBs.

  - FFS details of how to map RBs in one RB set to 2 sub-channels

Proposal 4: Enhance on R16 NR-SL time resource assignment to support more than 3 slot consecutive transmission for multi-consecutive slots transmission in SL-U.

Proposal 5: If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- TBS is determined based on a reference symbol length, the following option 2 is supported:

  - Option 2: The reference symbol length is determined based on 1st starting symbol

Proposal 6: When determining TBS in SL-U, the total number of RBs are calculated by multiplying the size of the lowest sub-channel in the resource pool and the total number of allocated sub-channels.

Proposal 7: For interlaced transmission for PSFCH in SL-U,

- Frequency resource of a PSFCH is configured in an interlace in an RB set.

Observation 1: Alt.1-1a with using common interlace may have power limitation issue, and K3>1 dedicated PRBs may aggravate the power limitation issue if not equally distribute K3>1 dedicated PRBs to different frequency bandwidth.

Observation 2: Supporting increased SL data rate over unlicensed spectrum may, in a sense, alleviate the PSFCH capacity issue from interlaced PSFCH transmission.

Proposal 8: For groupcast HARQ feedback, if a group size of recipient of a PSSCH is larger than the number of candidate PSFCH resources, a same principle in NR Rel-16 SL is followed, i.e., HARQ-ACK feedback option 1 is used.

Proposal 9: Regarding PSFCH transmission under 15kHz and 30kHz, the following alternative 2-3a is supported:

- Alt 2-3a: each PSFCH transmission occupies 1 interlace.

Proposal 10: Regarding PSFCH transmission under 60kHz SCS, each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs.

Proposal 11: PRBs within intra-cell guard band of two adjacent RB sets are not used for S-SSB transmissions.

Proposal 12: For S-SSB transmission, the following option is supported to meet OCB and PSBCH requirement for 15 kHz and 30 kHz SCS:

- Option 3-1: Repeat S-PSS/S-SSS/PSBCH N times in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement.

  - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of N (e.g., N=2)

  - FFS gap of 0

Proposal 13: If temporary exemption of OCB requirement for S-SSB transmission is concluded to be supported, regarding how to meet the minimum of 2 MHz requirement under 15 kHz SCS, the following option is supported:

- Use higher SCS than 15kHz for S-PSS/S-SSS/PSBCH in a SL BWP with 15kHz.

References

RP-221938 “WID revision: NR sidelink evolution”, RAN#97e, OPPO.

R1-2212649 “FL summary#5 for AI 9.4.1.2 SL-U physical channel design framework”, RAN1#111, Moderator (Huawei).