3GPP TSG-RAN WG1 #112	R1-2301561

Athens, Greece, February 27th – March 3rd, 2023

Source:                    	DENSO CORPORATION

Title:  	Discussion on resource allocation for SL positioning reference signal

Document for:        	Discussion and decision

Agenda Item:         	9.5.1.3	-	Resource allocation for SL positioning reference signal

- Introduction

At RAN#94-e meeting, new study item expanded and improved NR positioning was approved [1]. At RAN1 #110bis-e and #110, the followings were agreed[2] [3].

Agreement At least for a dedicated resource pool for positioning, - With regards to the bandwidth of SL-PRS transmission, downselect from the following alternatives: - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool - Note: Companies are encouraged to provide their analysis and views on the above alternatives - FFS: Bandwidth of SL-PRS transmission for shared resource pool (if supported) Agreement For Scheme 2, with regards to Resource allocation mechanism for SL-PRS, pick one or both of the following options: - Option 1: A sensing based resource allocation should be introduced - Option 2: A random resource selection should be introduced - In either option 1 or 2, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed.

This paper discusses the resource allocation for SL-PRS.

- Discussion

According to the SID of Rel-18 positioning evolution, the sidelink-based positioning covers three coverage scenarios which are in-coverage, partial-coverage and out-of-coverage scenarios. Figure1 shows the UE’s operation under the all three scenarios. Under the control of the base station(gNB), the gNB allocates SL-PRS resources, on the other hand UEs could autonomously select SL-PRS resources in some cases. We think it is beneficial that one or more Anchor UEs exist, for example, the absolute position of the Target UE could be determined by sidelink ranging with one or more Anchor UEs as reference points in this architecture.

Figure 1. UE-based Sidelink Positioning Architecture

For out-of-coverage scenario, there is no support from the network for resource allocation to SL-PRS, configuration or SL-PRS start transmission trigger. Therefore, it is required that UEs autonomously respond to these difficulties and operate to obtain the required positioning accuracy.

2.1. Bandwidth of SL-PRS

According to the agreement at RAN1 #110bis-e, the SL-PRS allocation granularity for the dedicated resource pool shows the following options:

Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool

Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool

Alt.1 makes the bandwidth of the SL-PRS and other signaling in the dedicated resource pool flexible, but the mechanism for scheduling of the SL-PRS may cause complexity. Also, there would be a trade-off between the positioning accuracy and the bandwidth constraints of the SL-PRS. Because we think that the large bandwidth basically needs for the positioning accuracy of the SL-PRS, the bandwidth configurations of the SL-PRS could be as simple as possible for the dedicated resource pools and the flexible operation for the bandwidth configuration could be considered for the shared resource pools.

Proposal 1:	RAN1 to consider that the bandwidth of SL-PRS shall be the same as that of the resource pool at least for the dedicated resource pool for positioning (Alt.2).

2.2. Resource Allocation Mechanism for SL-PRS

According to the agreement at RAN1 #111, the SL-PRS allocation mechanism shows the following options:

Option 1: A sensing based resource allocation should be introduced

Option 2: A random resource selection should be introduced

Option 1 is expected to reduce the performance degradation for the positioning caused by resource collisions between the UEs even when the number of UEs is increased. However, for Option 1, there is a problem that resource collisions happen due to the UE could not sense the hidden UEs. Option 1 may cause increasing of power consumption and complexity for the UE. On the other hand, Option 2 is expected to achieve low latency, and power consumption would be reduced compared to the sensing based mechanism. However, for Option 2, the efficiency of the resource allocation for the SL-PRS may be decreased. We think that the configuration to switch Option 1 and Option 2 may cause additional overhead, and the complexity for the UE to sense the resource for SL-PRS would be increased. Accordingly, we prefer Option 1 to achieve efficiency for the resource allocation for the SL-PRS.

Proposal 2:	RAN1 to consider that a sensing based resource allocation should be introduced(Option 1 only).

- Summary and proposal

In summary, we make the following observations and proposals:

Proposal 1:	RAN1 to consider that the bandwidth of SL-PRS shall be the same as that of the resource pool at least for the dedicated resource pool for positioning (Alt.2).

Proposal 2:	RAN1 to consider that a sensing based resource allocation should be introduced(Option 1 only).

- References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RAN#94-e meeting.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #110bis-e.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #111.