3GPP TSG RAN WG1 Meeting #112		R1-2301593

Athens, Greece, February 17th – March 3rd, 2023

Agenda Item: 9.3.1

Source: MediaTek Inc.

Title: Deployment scenarios and evaluation methodology for NR duplex evolution

Document for: Discussion and Decision

# Introduction

A study item on evolution of NR duplex operation was approved in RAN#94e with the following objective ‎[1]:

- Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, we discuss the deployment scenarios and evaluations methodology/assumptions for SBFD and DTDD, and we provide a set of proposals on how to evaluate the performance and feasibility of these schemes.

# Evaluation assumptions

The performance for SBFD and DTDD schemes can be conducted via several approaches, including system-level simulations, link-level simulations, and analysis. In the following, we provide some of the assumptions that could be considered in evaluating the SBFD and DTDD schemes.

## UEs deployment

In the legacy TDD deployment, both UL and DL occurred during separate time slots hence there is no probability that a UE UL signal will cause interference with the DL signal of a neighbouring UE. However, with SBFD, both UL and DL occur during the same time slot at the gNB, which will introduce Inter-UE CLI as the UL transmission signal of one UE will negatively impact the reception of DL signal at another UE that is near it.

In conventional simulations, UEs are deployed in the simulation environment by being uniformly and randomly distributed. However, this results in a scenario whereby the separation distance between the UEs is relatively large. As an example, for a DU deployment scenario with 24 UEs per cell, as shown in Figure 1, most of the inter-UE distances (~91%) lie between 20m to 70m (we ignore the distances between far UEs). Using the IMT2020 ChannelB_UMi pathloss model, this would result in a pathloss of at least 71.7dB. Also, the probability of having UEs close to each other (e.g., < 2m distance) is negligible. Consequently, under these conventional simulation settings, the effect of inter-UE CLI will not be captured as the separation distance between the UEs is too large to account for this.

This uniform random distribution also does not depict a real-world scenario whereby users who may be in possession of one or more mobile devices congregate to form groups in settings such as in meeting rooms, restaurants and in public transportation. The separation distances in such settings are much smaller than those depicted in a uniformly and randomly distributed scenario.

The clustering method implemented in our simulation environment involved deploying the clustered UEs in X clusters within each sector. The position of each cluster’s centre within the sector was chosen randomly, ensuring the cluster boundary falls within the sector. The number of UEs in each cluster was also random on the condition that the cumulative total of the UEs in all X clusters is a percentage (e.g., 50%, 80%) of the total UEs specified per sector. The radius of each cluster is dependent on the number of UEs which will be dropped within the cluster, and we adopted the following method to define the radius of each cluster: R = (n+1)/2 meters, where n is the number of UEs in the cluster. The UEs uniformly and randomly distributed with the cluster.

In order to demonstrate the effect of clustering has on the overall system performance, the separation distance ranges from a uniformly and randomly distributed UEs were compared to a deployment with clustered UEs in Figure 1. As can be seen from the figure, with clustering, there is higher percentage for UEs with close proximity.

Figure 1: UEs’ separation distance ranges with: 1) Uniformly distributed UEs, 2) Clustered UEs.

Figure 2 depicts the CCI to inter-UE CLI ratio for both clustered and uniformly distributed deployments. From the two CDF plots, it is clear to see that the addition of the clusters causes the CLI to increase over the CCI. These results therefore underpin the need to have a clustered deployment scenario as it presents a better representation of the interference levels UEs will experience with SBFD being implemented.

Figure 2: CCI over CLI ratio CDFs for UEs’ deployment with and without clustering.

The idea of having UE clustering is to model the case of UEs with proximity. Having large cluster’s radius defeats the purpose of clustering. With large radius, the UEs’ separation will be similar to the uniform-distribution case. As it can be seen from Figure 3, there is negligible difference between the two methods (i.e., clustering R=25 and random-and-uniform) in terms of probability of UEs with proximity (e.g., UE-to-UE distance <5 meters). In the figure, in one method, 8 UEs randomly and uniformly distributed in a cluster with R = 25m, and in the other method, 8 UEs randomly and uniformly distributed in a sector (ISD=200m).

Figure 3: Inter-UE separation for uniform-distribution case and clustering with radius of 25m case.

To have UEs with proximity, the cluster radius need be small (relative to the number of UEs in a cluster). As shown in the figure below, with R=5, the probability of UEs with small separation increases.

Figure 4: Inter-UE separation for clustering case with different radius values.

Considering the two considered values for cluster’s radius (i.e., 5m and 25m),

Figure 5 shows the impact of CLI on the system performance for these two values. As it is clear from the figure, R=25 is not adequate to capture the impact of the inter UE CLI because of the distances between the UEs are very large.

Figure 5: Impact of cluster’s radius on the system performance.

- Uniform random distribution does not depict a real-world scenario whereby users congregate to form groups/clusters.

- Inter-UE CLI has significant impact to the DL performance.

- For the evaluations of SBFD and DTDD schemes, RAN1 should consider cluster radius value that results UEs with small inter-UE distance to accurately capture the impact of inter-UE CLI:

- For cluster with 8 UEs, the cluster size is 5m.

# Conclusion

In this contribution, we discussed the deployment scenarios and evaluations methodology/assumptions for SBFD/DTDD and presented preliminary SLS evaluation results. We have the following observations and proposals:

‎Observation 1: Uniform random distribution does not depict a real-world scenario whereby users congregate to form groups/clusters.

‎Observation 2: Inter-UE CLI has significant impact to the DL performance.

‎Proposal 1: For the evaluations of SBFD and DTDD schemes, RAN1 should consider cluster’s radius that results in UEs with small inter-UE distance to accurately capture the impact of inter-UE CLI:

- For cluster with 8 UEs, the cluster size is 5m.

# References

- RP-213591, “New SI: Study on evolution of NR duplex operation”, CMCC, RAN#94e, December 2021.

# Appendix: Simulation assumptions in SLS evaluation

Table 1: Simulation assumptions in SLS for SBFD evaluation.

| Scenario |  | FR1 UMa | FR1 Indoor HotSpot |
| --- | --- | --- | --- |
| Evaluation configuration |  | Configuration A | Configuration A |
| Channel model |  | UMa_A | InH_A |
| ISD |  | 500 m | 20 m (12TRxP) |
| Traffic Type |  | FTP model 3 with 0.5Mbyte and 0.1Mbyte as DL and UL packet size. Packet arrival rate is changed for different RUs | FTP model 3 with 0.5Mbyte and 0.1Mbyte as DL and UL packet size. Packet arrival rate is changed for different RUs. |
| Carrier Frequency |  | 4 GHz | 4 GHz |
| System bandwidth |  | 100MHz | 100MHz |
| Subcarrier spacing |  | 30 KHz | 30 KHz |
| Symbols number per slot |  | 14 | 14 |
| TDD Pattern |  | DDDSU | DDDSU |
| SBFD configuration |  | DXXXU | DXXXU |
| UL resources in SBFD slot |  | 20MHz in the center of channel bandwidth | 20MHz in the center of channel bandwidth |
| Number of antenna elements per TRxP |  | 128Tx cross-polarized antenna (M,N,P,Mg,Ng,Mp,Np) = (8,8,2,1,1,2,8) | 32Tx cross-polarized antenna (M,N,P,Mg,Ng,Mp,Np) = (4,4,2,1,1,4,4) |
| Number of TXRU per TRxP |  | 32TXRU | 32TXRU |
| Number of antenna elements per UE |  | 4Rx 0°,90° polarization antenna (M,N,P,Mg,Ng,Mp,Np) = (1,2,2,1,1,1,2) 2Tx 0°,90° polarization antenna (M,N,P,Mg,Ng,Mp,Np) = (1,1,2,1,1,1,1) | 4Rx 0°,90° polarization antenna (M,N,P,Mg,Ng,Mp,Np) = (1,2,2,1,1,1,2) 2Tx 0°,90° polarization antenna (M,N,P,Mg,Ng,Mp,Np) = (1,1,2,1,1,1,1) |
| Number of TXRU per UE |  | 2T4R | 2T4R |
| Transmit power per TRxP |  | 56dBm | 31dBm |
| UE max Tx power |  | 23dBm | 23dBm |
| TRxP number per site |  | 3 | 1 |
| Mechanic tilt |  | 90deg in GCS (pointing to the horizontal direction) | 180deg in GCS (pointing to the ground) |
| Electronic tilt |  | 105deg in LCS | 90deg in LCS |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 |
| Scheduling |  | MU-PF in DL and SU-PF in UL | MU-PF in DL and SU-PF in UL |
| ACK/NACK delay |  | Next available UL slot | Next available UL slot |
| MIMO mode |  | MU-MIMO with max rank 2 adaptation per user, Maximum MU layer = 12 SU-MIMO with max rank 2 adaptation per user | MU-MIMO with max rank 2 adaptation per user, Maximum MU layer = 12 SU-MIMO with max rank 2 adaptation per user |
| Receiver type |  | MMSE-IRC | MMSE-IRC |
| CSI feedback |  | 5 slots period based on CSI-RS with delay | 5 slots period based on CSI-RS with delay |
| Precoder derivation |  | NR Type II codebook based | NR Type II codebook based |
| Power control |  | α= 0.6, P0 = -60dBm | α= 0.9, P0 = -86dBm |
| Power backoff model |  | Continuous RB allocation: follow TS 38.101; Non-continuous RB allocation: additional 2 RB reduction | Continuous RB allocation: follow TS 38.101; Non-continuous RB allocation: additional 2 RB reduction |
| Overhead (DL) | PDCCH | 2 complete symbols | 2 complete symbols |
|  | DMRS | Type II, based on MU-layer (dynamic in simulation) | Type II, based on MU-layer (dynamic in simulation) |
|  | CSI-RS | 32 ports per 5 slots | 32 ports per 5 slots |
|  | SSB | 1 SSB per 20 ms | 1 SSB per 20 ms |
|  | TRS | 2 consecutive slots per 20ms, 1 port, 50PRB | 2 consecutive slots per 20ms, 1 port, 50PRB |
| Overhead (UL) | PUCCH | 2 RBs, 14 OFDM symbols | 2 RBs, 14 OFDM symbols |
|  | DMRS | Type II, 1 additional DMRS symbol, and no FDM with PUSCH | Type II, 1 additional DMRS symbol, and no FDM with PUSCH |
|  | SRS | 2 symbols per 5 slots, 8 RBs per symbol | 2 symbols per 5 slots, 8 RBs per symbol |
| Channel estimation |  | Ideal | Ideal |
| Waveform |  | OFDM | OFDM |