3GPP TSG RAN WG1 Meeting #112		R1- 2301646

meeting, Athens,  27th February – 3rd March, 2023

Agenda item:       9.1.3.2

Source:                 Nokia, Nokia Shanghai Bell

Title:                     SRS enhancement for TDD CJT and 8Tx operation

Document for:     Discussion and Decision

# 1	Introduction

In this contribution,   UL SRS enhancements for 8 TX antenna ports  and UL SRS enhancements for TDD based CJT operation are addressed for Rel-18 MIMO evolution for Downlink and Uplink.

# 2	SRS Enhancement for 8 Tx Antenna Ports

## 2.1	8-AP UL SRS Resource Set Configuration for Codebook

For codebook-based transmission in Rel-15, the UE determines its codebook subsets based on TPMI and upon the reception of higher layer parameter codebookSubset in pusch-Config for PUSCH associated with DCI format 0_1 which may be configured with 'fullyAndPartialAndNonCoherent', or 'partialAndNonCoherent', or 'nonCoherent' depending on the UE capability. It is worth noting that coherency assumptions for 8 TX codebook are defined in agenda item 9.1.4.2, where further details on 8-TX codebook coherency assumption can be found from our companion paper [2].

In Rel-15, when the SRS-resource set with usage set to "codebook" includes at least one SRS resource with 4 ports and one SRS resource with 2 ports, the codebookSubset associated with the 2-port SRS resource is 'nonCoherent'. For codebook-based transmission, only one SRS resource can be indicated based on the SRI from within the SRS resource set.

In Rel-17, when two SRS resource sets are configured with higher layer parameter usage in SRS-ResourceSet set to 'codebook', the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets.

For Rel-18 S-DCI and M-DCI based STxMP PUSCH, the UE is assumed to be configured with two UL SRS resource sets with usage ‘codebook’ with the assumption that only 4-APs can be simultaneously transmitted over two different antenna panels.

RAN1#111 Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), at least support: - For comb 2, support 1 and 2 comb offsets - For comb 4, support 2 and [4] comb offset - For comb 8, support 4 comb offsets

Figure 1 shows an example of UL SRS resource w/ usage codebook w/ one 8-AP resource.  As shown, one 8-AP UL SRS resource can be transmitted in one OFDM. From latency perspective, it is beneficial to support configuration where 8-TX antenna ports are transmitted in one OFDM symbol. However, in some scenarios, e.g. coverage/power limited scenarios, it is also beneficial to provide support for UL SRS resource set configuration where UL TX power boosting can be enabled by using multiple OFDM symbols where different APs are associated with different OFDM symbols.  Moreover, multi-symbol configuration enables to distribute resources across different SRS transmission which may be beneficial for a scheduler at the network side.

Figure 1 An example of  UL SRS resource set w/ usage codebook w/ one 8-AP resource.

## 2.1.1	8-AP with Single Resource

RAN1#111 Agreement For single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH or ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with 8 ports and m OFDM symbols (m > 1), support the case of 8 ports mapped onto the m OFDM symbols - Option 1: Different SRS ports are mapped onto different OFDM symbols (i.e., TDM) - FFS: m can be legacy values, i.e., 2,4,[8,10,12,14].

Regarding to previous meeting agreement, single SRS resource in a SRS resource set with usage ‘codebook’ for 8TX PUSCH when SRS resource is configured with 8 ports and m OFDM symbols (>1). Current RAN1 specification defines that single resource can be applied with multiple OFDM symbols with repetition and/or frequency hopping where antenna ports associated with the resource are not used in TDM:ed manner.  To define specification support for corresponding operation with TDM:ed antenna ports. It is important to clarify how the UE can differentiate between single resource SRS legacy  operation with repetition or frequency-hopping  and Rel-18 SRS with TDM:ed antenna ports with different SRS resource set usages. In other words, a new signaling needs to be defined,  e.g. new RRC parameter,  for enabling  TDM:ed antenna ports with single resource. Otherwise, there is ambiguity in the specification for the UE apply correct operation.

Observation 1: It is  important to clarify how the Rel-18 UE can differentiate between single resource SRS legacy  configuration with repetition or frequency-hopping  and Rel-18 single SRS resource with TDM:ed antenna ports resource configuration.

Regarding to FFS on legacy values, i.e. 2,4 [8,10,12,14], it would be reasonable to define first such that TDM:ed antenna ports associated with a single SRS resource are only allowed to be configured within the same time slot. In other words, only intra-slot operation supported and no support for inter-slot operation. Otherwise, extra specification and implementation efforts are needed resulting in minimalistic benefits, if any.

To define single SRS resource TDM:ed antenna ports for FFS legacy values , i.e. 2,4 [8,10,12,14], two different TDM operations should be supported: 1) consecutive OFDM symbols associated with values 2,4,8 and 2) non-consecutive symbols 4,8,10,12,14, where antenna ports  can be configured with equal or non-equal separation between OFDM symbols. In general, non-consecutive option can provide further flexibility for a scheduler which is from our perspective preferable.

Observation 2:  Single 8-AP resource SRS with TDM:ed antenna ports supported only with intra-slot operation.

Observation 3: Single 8-AP SRS resource with TDM antenna ports can be applied  with consecutive OFDM symbols with m=2,4 and 8.

Observation 4: Single 8-AP SRS resource with TDM antenna ports can applied with non-consecutive OFDM symbols with m= 4,8,10,12, and 14, where TDM:ed antenna ports  can be configured with equal or non-equal separation between OFDM symbols.

Figure 2 shows an example of single UL SRS 8-AP resource configuration with TDM:ed antenna ports without repetition and/or frequency hopping in two consecutive OFDM symbols.  UE is configured with one UL SRS resource set with single 8-AP resource (i.e. SRI#1) with TDM:ed antenna ports. Single resource is configured with starting symbol 1, number of symbols (Ns) set to 2 and with  comb-2 with comb-offset 0 and 1, where for each comb-offset value 4-APs configured. It is worth noting that current specification supports Ns >1 only with repetition and/or frequency hopping.

Figure 2 an example of single UL SRS 8-AP resource transmission with TDM:ed antenna ports with two consecutive symbols.

Figure 3 shows an example of single UL SRS 8-AP resource configuration with TDM:ed antenna ports without repetition and/or frequency hopping with non-consecutive symbols.

Figure 3 an example of single UL SRS 8-AP resource transmission with TDM:ed antenna ports with two non-consecutive symbols.

Proposal 1: Support only intra-slot configuration for single 8-AP SRS resource with TDM:ed antenna ports and multiple OFDM symbols.

Proposal 2: Define signaling/indication to differentiate the operation between legacy single resource SRS with repetition and/or frequency hopping and Rel-18 single 8-AP SRS resource with TDM:ed antenna ports with multiple OFDM symbols.

- FFS: new RRC parameter

Proposal 3: Support both consecutive and non-consecutive single 8-AP SRS resource transmission with TDM:ed antenna ports in Rel-18, where for consecutive transmission, m=2,4,8 and non-consecutive transmission, m =4,8,10, 12, 14

- FFS: how to signal offset between OFDM symbols for non-consecutive transmission

- FFS: for non-consecutive transmission equal or non-equal separation between symbols

## 2.1.2	8-AP with Multiple  Resources

As agreed in RAN1#110bis meeting, 8-TX codebook design supports different coherency assumptions with different coherency group sizes, i.e., Ng = 2 with two coherent groups, Ng=4 with four different coherency groups. It can be assumed that the UE indicates its coherency group and simultaneous 8TX antenna port capability for SRS transmission via capability signalling. As a result of this, the network may configure the UE with UL SRS resource set configuration with usage codebook for 8 TX SRS with one or more OFDM symbols.

Observation 5: Based on UE’s UL SRS capability information on coherency group and simultaneous 8TX antenna port capability, network may configure the UE with UL SRS resource set configuration with usage codebook for 8 TX SRS with one or more OFDM symbols.

Figure 4 shows an example of UL SRS resource set#1 configuration where two 4-AP resources, i.e., SRI#2 and SRI#4, are configured for the UE. Based on UE’s coherence group capability, Ng=2, the network can assume that UE supports two different sets of antenna ports where antenna ports within a group are having coherency with each other but non-coherency between groups. As a result of this, the network may configure the first set of four different coherent antenna ports to be within SRI#2 and another set of four different coherent antenna ports to be within SRI#4. Since the UE has indicated its capability that it supports simultaneous 8TX antenna port UL transmission, the network can configure resources, i.e. SRI#2 and SRI#4, to either in consecutive OFDM symbol or non-consecutive symbols without any gap (i.e. no any TX) between two resources. It is worth noting that if these resources would have been configured into two different resource sets also no gap between different resource sets would be needed. As a result of this, the UE shall transmit separately SRI#2 and SRI#4 resources with 4-antenna ports at any configured OFDM symbols within a slot.

Figure 4 an example of UL SRS resource set configuration w/ usage codebook w/ 4+4 resources for 8 TX with two OFDM symbols.

Observation 6: When the number of coherency groups, Ng, is 2, UL SRS resources w/ usage ‘codebook’ can be distributed across 2 different consecutive/non-consecutive OFDM symbols with 4-APs in each resource where different antenna ports used for the transmission in different OFDM symbols.

Figure 5 shows an example of UL SRS resource set configuration with usage codebook with four resources with 2-antenna port in each resource, when UE has indicated Ng=4 coherency groups via capability signaling.

Figure 5 an example of UL SRS resource set configuration w/ usage codebook w/ 2+2+2+2 resources for 8 TX with two OFDM symbols.

Observation 7: When the number of coherency groups, Ng, is 4, UL SRS resources w/ usage ‘codebook’ can be distributed across 4 different consecutive/non-consecutive OFDM symbols with 2-APs in each resource where different antenna ports used for the transmission in different OFDM symbols.

Proposal 4: Support 8-AP UL SRS resource set configurations with usage ‘codebook’ with one or more resources, where antenna ports of 8TX are transmitted in one or more OFDM symbols.

## 2.2 8-AP UL SRS Resource Set Configuration for Non-Codebook

For non-codebook-based transmission in Rel-15, the UE can calculate the precoder used for the transmission of SRS based on measurement of an associated NZP CSI-RS resource. In Rel-15, a UE can be configured with only one NZP CSI-RS resource for the SRS resource set with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook' if configured. The UE shall use one or multiple SRS resources for SRS transmission, where, in a SRS resource set, the maximum number of SRS resources which can be configured to the UE for simultaneous transmission in the same symbol and the maximum number of SRS resources are UE capabilities. Only one SRS antenna port for each SRS resource is configured.

For Rel-18 S-DCI and M-DCI based STxMP PUSCH, the UE is assumed to be configured with two UL SRS resource sets with usage ‘NoneCodebook’ with the assumption that only 4-APs can be simultaneously transmitted over two different antenna panels.

RAN1-110 Agreement For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols. - Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling.

As discussed in previous section, multi-symbol 8-TX antenna UL SRS resource configuration can enable TX power boosting and flexible distribution of different antenna ports over multiple OFDM symbols. Therefore, it is reasonable to provide specification support for such configuration in the context of SRS resource set with usage ‘NonCodebook’.

Figure 6 shows an example of UL SRS resource set configuration with usage non-codebook with 8 resources with 1-antenna port in each resource.  As can be seen, four different sub-resource groups with single antenna port resources are associated with four different OFDM symbols with different antenna ports, where each OFDM symbol consist of two single antenna port SRS resources. Since non-codebook-based precoding assumes full coherency between different antenna ports, the UE is assumed to maintain coherency over configured number of OFDM symbols.

Figure 6 an example of UL SRS resource set configuration w/ usage non-codebook w/ {1+1}+{1+1}+{1+1}+{1+1} resources for 8 TX with four OFDM symbols.

Observation 8: UL SRS resources w/ usage ‘NoneCodebook’ can be distributed across multiple different OFDM symbols with one or more sub-groups of  1-APs resources where each sub-group is associated  different OFDM symbols.

## 2.3 8-AP UL SRS Resource Set Configuration for Antenna Switching

RAN1-110 Agreement For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol. FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols.

In RAN1#110 meeting, it remained for further study whether UE can be configured with UL SRS resource set configuration with usage antenna-switching where different antenna ports are transmitted in different OFDM symbols. Figure 7 shows two example of UL sRS resource set configuration w/ usage antenna-switching for 8T8R with two OFDM symbols with 4-AP resources and with four OFDM symbols with 2-AP resources. Since the UE has indicated it’s capability to supports simultaneous 8TX antenna ports UL transmission, the network can configured resources to either in consecutive OFDM symbol or non-consecutive symbols without any gap (i.e. no any TX) between two resources.

- two resources w/ 4-Aps			b) four resources w/ 2-APs

Figure 7 an example of UL SRS resource set configuration w/ usage antenna-switching for 8T8R with two and four different OFDM symbols.

Observation 9: When resources are configured over multiple OFDM symbols with 8T8R, no gaps/guard periods are needed between different resources.

Proposal 5: Support 8-AP UL SRS resource set configurations with usage ‘antenna-switching’ with one or more resources, where antenna ports of 8TX are transmitted in one or more OFDM symbols.

# 3	UL SRS Enhancements for TDD CJT

In this section, the following Rel-18 UL SRS enhancements are consider for TDD CJT based operation in multi-TRP scenario:

- UL SRS Power Control Enhancements

- Precoded UL SRS Enhancements

- Randomized code and frequency domain resource mapping for SRS transmission

## 3.1 UL SRS Power Control Enhancements

In RAN1 #110bis-e, RAN1 agreed the following items to study for efficient per-TRP SRS power control.

RAN1-110b-e Agreement For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for an SRS resource set: - Option 1: - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha - Each transmission occasion of the SRS resource is towards multiple TRPs - Option 2: - More than 1 power control processes each for a subset of SRS resource of an SRS resource set where each of the power control process is based on a different UL power control parameter set (Po, alpha, and closed loop state) associated with a different DL pathloss RS - Different transmission occasions of the SRS resource can be towards different TRPs

In Rel-17 UE adjusts its UL SRS transmit power for all resources within UL SRS set based on a single pathloss reference (pathlossReferenceRS), i.e., DL reference signal/signal (e.g., NZP-CSI-RS or SSB) that is common for all resources within UL SRS set), set up by a single (serving) TRP. In a M-TRP scenario, the pathlosses to different TRPs are not necessarily represented by this single DL pathlossReferenceRS. Thus, since only a single pathloss reference can be configured for a UL SRS resource set, i.e., the UE uplink transmission power is adjusted based on a single TRP, the transmit power might not correspond to the feasible power level towards the other TRPs causing interference at other TRPs. On one hand, adjusting the UE transmit power based on a nearby TRP (i.e., low transmit power corresponding to low pathloss), the signal level at a far-off TRP is lowered further from the case when the UE transmits at maximum power. On the other hand, adjusting the UE transmit power based on a far-off TRP (i.e., high transmit power corresponding to high pathloss), the signal level at a nearby TRP may saturate reception of the UL SRS, causing near-far problems (i.e. interference) when multiple UEs need to be served.

One way to reduce SRS interference problems for UL SRS transmission in M-TRP scenario is to configure UE with multiple UL SRS resource sets and request the UE to transmit UL SRS resources sequentially in time TRP specifically, which is correspond to option 2 in the agreement in RAN1#1101bis-e. As a result of this, UL SRS resource overhead and latency can increase significantly. The amount of resources overhead and latency can be even further increased if there is need for antenna switching at the UE side.

Observation 10: TRP specific UL SRS power control(option 2) increases resource overhead and latency.

To reduce problems related to excessive latency and resource overhead with TRP-specific UL SRS transmission for TDD C-JT, a procedure can be defined where the UE determines single UL power control value based on multiple pathlossReferenceRS measurements associated with multiple with DL/joint DL and UL/UL TCI states from different TRPs, which is correspond to the option 1 in the above agreement. As a result of this, the UE can apply single power control value for UL SRS transmission in M-TRP scenario. By using a single value UL power PC for the UL SRS resource transmission introduces the pathloss information implicitly in the channel estimated at the TRP (that is, the relative pathloss differences between TRPs), which can then be more accurately accounted for when determining the downlink precoder for C-JT.

Observation 11:  By using multiple pathlossReferenceRS measurements for pathloss estimation associated with multiple with DL/joint DL and UL/UL TCI states from different TRPs, the UE can determine single UL PC value to applied with single resource/resource set transmission to multiple TPRs.

Proposal 6: Support option 1, where single value UL power control for single UL SRS resource transmission associated with multiple pathlossReferenceRS measurements with CJT.

## 3.2 Precoded UL SRS Enhancements

Rel-17 specification provides support only for precoded UL SRS transmission when usage is set to non-codebook to enable non-codebook based PUSCH transmission. However, there is no support for precoded UL SRS transmission when usage is set to antenna switching. UL SRS antenna switching with precoding can provide coverage and interference mitigation enhancements for Rel-18 TDD C-JT. As a result of this, the quality of DL CSI can be improved leading to enhanced DL throughput associated TDD C-JT. Moreover, via precoded UL SRS transmission better interference awareness can be provided for the network about interference at DL reception.

Observation 12: UL SRS antenna switching with precoding can provide coverage and interference mitigation enhancements for DL CSI acquisition for TDD C-JT.

Observation 13: Precoded UL SRS transmission can enable improved interference awareness for the network about interference at DL reception.

When the UE is configured with SRS with antenna switching configuration with precoding, where x=y, the UL SRS resource set configuration is similar with non-codebook based UL SRS.  Additionally, SRS resource set is associated with DL RS resource based on which the UE determines precoder for UL SRS transmission. Based on the configuration, the UE can transmit one or more single antenna port resources with precoding.

Figure 8 shows an example of UL SRS with antenna switching configuration with precoding, where x<y. As can be seen, the precoded UL SRS transmission needs to be divided into different transmission occasions that needs to re-constructed at the gNB to obtain effective channel (including channel and precoding).

Regarding to potential specification impacts, the specification should define that precoding is applied for UL SRS antenna-switching resources. At moment, specification provide support for antenna-switching without precoding. Furthermore, currently supported UL SRS xTyR antenna switching configurations define that SRS resources within each set are  multi-antenna port resources, except 1TyR cases with single antenna port resources. However, to support precoded antenna switching UL SRS transmission,  SRS resources within each UL SRS resource set  are defined as single antenna port resource.  Furthermore, for precoded antenna switching, the specification should define also for antenna switching configuration with precoding with x<y, that single antenna port SRS resources are transmitted in TDM:ed manner.

Observation 14: Precoded UL SRS with usage introduces following potential specification impacts:

- define how to indicate that precoding is applied for UL SRS antenna-switching resources

- UL SRS resource set configurations for antenna switching need to be specified to use single antenna port resources

- when antenna switching configuration with precoding with x<y is configured that single antenna port SRS resources need to be  transmitted in TDM:ed manner

Figure 8 an example of UL SRS resource configuration and non-codebook based precoded SRS transmission with 1T2R antenna switching configuration.

Proposal 7: Support precoded UL SRS transmission with antenna switching in Rel-18 with following specification impacts:

- FFS: define how to indicate that precoding is applied for UL SRS antenna-switching resources

- FFS: UL SRS resource set configurations with single antenna port resources

- FFS: TDM:ed single antenna port resource transmission for antenna switching configuration with x<y with precoding with x<y

## 3.3 Randomized code and frequency domain resource mapping for SRS transmission

In general, UL SRS (i.e. cross-SRS) interference may become a serious issue impacting channel state information (CSI) quality  that may limit potential merits (e.g.  interference reduction in downlink, throughput performance) of TDD based coherent joint transmission (CJT) for physical downlink data shared channel (PDSCH) transmission in multi-TRP scenarios. UL SRS interference or cross-SRS occurs when the different SRS sequences with corresponding SRS resources overlap in RE domain (i.e. sharing same symbol,  RBs, and same comb offset).  To reduce the impact of the UL cross-SRS interference to a system performance, interference randomization in a form of UL SRS comb-offset hopping and cyclic shift hopping can be seen as attractive approach for Rel-18.

RAN1#111 Agreement For SRS interference randomization, support one from the following options (to be decided in RAN1#112): - Opt. 1: Cyclic shift hopping - Opt. 2: Comb offset hopping - Opt. 3: Both cyclic shift hopping and comb offset hopping - FFS: details including whether to support separate and/or combined hopping - FFS: details on UE capability and signaling

In previous meting RAN1#11 meeting, it was agreed that one of the three options, i.e. cyclic-shift hopping, comb-offset hopping and combination of cyclic and comb-offset hopping will be selected for Rel-18.

When UE is configured with UL SRS Comb-2, the comb-offset hopping does not provide large possibility for interference randomization in frequency domain. Therefore, it can be beneficial to use cyclic-shift hopping having up to 12 different cyclic shifts for randomization purposes.

Observation 15: When UE is configured with UL SRS Comb-2,  it beneficial to use cyclic-shift hopping having up to 12 different cyclic shifts for randomization purposes.

When UE is configured with UL SRS with comb-offset hopping multi-user interference can be randomized by defining a comp-offset hopping pattern where a comb-type specific frequency offset is pseudo-randomized with network-controlled manner UE specifically. The number of comb-offsets depends on comb-type-N, where N=2, or 4 or 8, with corresponding offsets options 2,4 and 8, respectively.

When UE is configured with UL SRS Comb-4 or Comb-8, the comb-offset hopping enables possibility for interference randomization in frequency domain. Hence, it can be beneficial to use comb-offset hopping having up to 4 different frequency offsets (comb-4) and up to 8 different frequency offsets (comb-8) for interference randomization purposes.

Observation 16: When UE is configured with UL SRS Comb-4 or Comb-8,  it beneficial to use comb-offset hopping having up to 4 (Comb-4) or 8 (Comb-8) different frequency offsets  for randomization purposes.

Since both options 1 and 2 can enable benefits for the system in terms of  configuration flexibility, it is reasonable and beneficial to provide specification support for both of the options. Therefore, from our perspective,  Opt 3 (both cyclic shift hopping and comb-offset hopping) should be supported in Rel-18.

Observation 17: From system operation perspective, it is beneficial to provide specification support for Opt. 3.

Proposal 8: Support cyclic hopping and comb offset hopping (Opt. 3) for Rel-18.

As discussed above, the comb-offset hopping provides an attractive way to reduce cross-SRS interference by introducing randomness into the comb-offset associated with SRS resource. This, in turn, will randomize the interference characteristics when estimating the channel via any given received SRS transmission. However, up to now, the comb offset is assumed to be fixed for UL SRS bandwidth, causing potentially severe collision of SRS resource transmissions from two different sources leading to significant performance degradation of the system. It is worth noting that  the number of collisions may become even more serious when the number of UEs and TRPs are increased (i.e. typical scenario in C-JT deployment). Furthermore, as the randomness provided by this scheme is only pseudorandom, it can repeat cyclically, resulting in regular serious  collisions as shown in Figure 9.

Figure 9 An example of comb-offset hopping with fixed comb-offset over UL SRS bandwidth with  two UEs.

Observation 18: When comb offset is fixed over the UL SRS bandwidth,  severe collisions of UL SRS resource transmissions between different UEs may  occur.

To enhance further UL SRS resource interference randomization and reducingthe impact of collisions between different transmitted UL SRS resources associated with different UEs, further randomization can be introduced by defining comb-offset hopping pattern in frequency with multiple pseudo random comb-offsets which can be circulated over UL SRS bandwidth. Figure 10 shows an example of comb-offset hopping with non-fixed comb-offset values over UL SRS bandwidth with two UEs. Here, each small square with a different colour illustrates multiple  PRBs with different UL SRS comb-offset values. As can be seen, the collision probability of different UL SRS resources associated with different UEs becomes smaller with respect to fixed comb-offset case. Moreover, if a collision between different SRS resources occurs, the collision realizes  only for a part of SRS resource (i.e. a small part of UL SRS bandwidth) and not for whole bandwidth of the SRS resource as with the fixed comb-offset hopping.

Observation 19:  By introducing multiple comb-offset values for a SRS resource over UL SRS bandwidth, multi-user interference can be further randomized leading to reduced cross-SRS interference.

Figure 10 An example of comb-offset hopping with non-fixed comb-offsets over UL SRS bandwidth with two UEs.

Regarding to a hopping pattern for the comb-offset scheme, the comb-offset can be configured both in frequency and time domains.  Figure 11 shows an example of comb-offset pattern in frequency domain with length of 16 PRBs where each value of the comb-offset pattern corresponds to fixed   comb-offset value over 4 PRBs. The number of PRBs associated with fixed comb-offset value depends on comb-type and maximum number of cyclic shifts. The number of PRBs can be used as a processing window over which orthogonality between cyclic-shifts and sequences can be maintained at the gNB-side. In other words, block-by-block processing is required the gNB side. It is worth also noting that antenna ports within one SRS resource can be configured with a resource specific comb-offset value on top of which pseudo-random comb-offset pattern is applied.  Each UE can be configured with user-specific comb-offset pattern which applied over entire UL SRS bandwidth, where granularity of each comp-offset is large in terms of number of PRBs. Alternatively, to randomize interference even further, the UE can be configured to apply smaller  granularity (i.e. smaller number of PRBs) comb-offset pattern cyclically over entire UL SRS bandwidth, for example with three cyclic shifts of comb-offset pattern of length 16 resulting in total 48 PRBs,  as shown in Figure 12.

Figure 11 An example of UL SRS comb-4 with comb-offset pattern in frequency domain with length of 16 PRBs.

Figure 12 An example of three cyclically shifted comb-offset patterns with length of four in frequency domain.

Observation 20:  To further randomize multi-user interference,  UE can be configured to apply a comb-offset pattern cyclically over entire UL SRS bandwidth.

Proposal 9: Consider enhanced comb offset hopping with multiple comb-offset values for Rel-18.

# 4	Conclusion

Hereafter is a summary of proposals for SRS enhancement

Proposal 1: Support only intra-slot configuration for single 8-AP SRS resource with TDM:ed antenna ports and multiple OFDM symbols.

Proposal 2: Define signaling/indication to differentiate the operation between legacy single resource SRS with repetition and/or frequency hopping and Rel-18 single 8-AP SRS resource with TDM:ed antenna ports with multiple OFDM symbols.

- FFS: new RRC parameter

Proposal 3: Support both consecutive and non-consecutive single 8-AP SRS resource transmission with TDM:ed antenna ports in Rel-18, where for consecutive transmission, m=2,4,8 and non-consecutive transmission, m =4,8,10, 12, 14

- FFS: how to signal offset between OFDM symbols for non-consecutive transmission

- FFS: for non-consecutive transmission equal or non-equal separation between symbols

Proposal 4: Support 8-AP UL SRS resource set configurations with usage ‘codebook’ with one or more resources, where antenna ports of 8TX are transmitted in one or more OFDM symbols.

Proposal 5: Support 8-AP UL SRS resource set configurations with usage ‘antenna-switching’ with one or more resources, where antenna ports of 8TX are transmitted in one or more OFDM symbols.

Proposal 6: Support option 1, where single value UL power control for single UL SRS resource transmission associated with multiple pathlossReferenceRS measurements with CJT.

Proposal 7: Support precoded UL SRS transmission with antenna switching in Rel-18 with following specification impacts:

- FFS: define how to indicate that precoding is applied for UL SRS antenna-switching resources

- FFS: UL SRS resource set configurations with single antenna port resources

- FFS: TDM:ed single antenna port resource transmission for antenna switching configuration with x<y with precoding with x<y

Proposal 8: Support cyclic hopping and comb offset hopping (Opt. 3) for Rel-18.

Proposal 9: Consider enhanced comb offset hopping with multiple comb-offset values for Rel-18.

# References

- R1-2207550 Nokia, Nokia Shanghai Bell, “UL enhancements for enabling 8Tx UL transmission”, RAN1-110 Toulouse.