3GPP TSG RAN WG1 Meeting #112	R1-2301693

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.3.3

Source:	CEWiT, Reliance Jio, IIT Kanpur

Title:	Discussion on enhancements on dynamic/flexible TDD

Document for:	Discussion

# Introduction

A new study item (SI) on “Study on evolution of NR duplex operation” was approved in RAN#94-e [1]. According to the objectives of this SI, enhancements on dynamic/flexible TDD is to be studied. The major focus of the study is UE-to-UE and gNB-to-gNB CLI. The discussions started in RAN1#109-e. The contribution discusses the potential areas of enhancements to the CLI management and reporting procedures based on the agreements made in RAN1 #110bis-e meeting and RAN1 #111 meeting.

# Potential areas of enhancements in flexible TDD

## 2.1 Enhancements related to UE-to-UE CLI measurement and reporting

## Measurement resource enhancement

The following agreement was made for UE-to-UE co-channel CLI measurement in RAN1 #110bis-e meeting [3].

Agreement

For UE-to-UE co-channel CLI measurement, consider as baseline reusing existing channel(s)/signal(s)/measurement_resource(s)

- For example, SRS resources defined in Rel-16 for SRS-RSRP measurement, CLI-RSSI resources defined in Rel-16 for CLI-RSSI measurement

  - FFS potential enhancements

This section discusses the potential enhancements for SRS resources in Rel-16 for SRS-RSRP measurement.

According to Rel. 16 SRS-RSRP measurement, victim UE performs CLI measurement on sounding reference signal (SRS) transmitted from the aggressor UE. The victim UE shifts its receiving boundaries by a constant implementation specific offset while receiving the SRS.  As per 38.133 [4], the time difference between UE’s DL reference timing in the serving cell and SRS arrival time is no larger than Terror_SRS_RSRP, where

-	Terror_SRS_RSRP = TC × NTA_offset + 4.67us for FR1

-	Terror_SRS_RSRP = TC × NTA_offset + 3.67us for FR2

-	NTA_offset is defined in Table 7.1.2-2

-	TC is 0.509ns

As illustrated in Fig 1A, the SRS symbol is transmitted from UE 2 (aggressor UE) with some timing advance (TA). UE2 is served by gNB 2. The transmission boundary of gNB 2 is also shown in the figure. In Fig. 1B, the SRS is received at UE 1 (victim UE), which is served by gNB 1, after a propagation delay of ‘t’ in case of an ideal scenario. Here, UE 1 has adjusted its reception boundaries to match with the transmission boundaries of its serving gNB (gNB 1). This adjustment is possible since UE 1 knows its own TA and  NTA_offset. Also, it is assumed that gNB 1 and gNB 2 are perfectly synchronized and hence the transmission boundary of gNB1 and gNB2 are same. In this case, the full SRS sequence can be received by UE 1 since the misalignment in reception is within the CP duration as shown in Fig. 1B. However, in practical scenarios, there is a synchronization error (max 4.67us for FR1 and 3.67us for FR2) between gNB 1 and gNB 2 which is unknown at UE 1.  This is illustrated in Fig 1C. Hence, the reception boundary at UE 1 still remains misaligned by a factor of the synchronization error even after adjustment. Thus, the misalignment in SRS reception by UE1 goes beyond the CP duration resulting in loss of information, especially in cases of small CP duration.  This will degrade the RSRP measured by the UE.

Fig. 1 Transmission and reception boundaries of SRS at aggressor and victim UEs with and without gNB synchronization error

Observation 1: Factors like synchronization errors between gNB, smaller CP length in higher numerologies, higher propagation delay between the UEs and implementation specific adjustment of reception timing causes the misalignment to go beyond CP duration while measuring the CLI on Rel. 16 SRS as both the UEs are not time synchronized. This will degrade the CLI measurement accuracy.

The above mentioned issue can be solved by modifying the method of transmitting SRS as illustrated in Fig. 2. Here, S1 and S2 are two SRS sequences (same sequence) in frequency domain. S2 is phase rotated by a certain factor such that after taking IFFT and adding CP, time domain continuity is maintained over the 2 symbols. The design is similar to Rel. 16 RIM RS. The proposed method of transmitting the symbol will ensure that the SRS misalignment does not go beyond the CP duration. As a result, SRS information is not lost despite the synchronization error between the gNB.  DL rate matching around SRS can further improve the CLI measurement accuracy.

Fig. 2 Phase rotated SRS transmission

A simple simulation based comparison for RSRP measurement between Rel. 16 SRS transmission and the proposed method of SRS transmission is shown in Fig. 3. The simulation assumes FR2 and corresponding synchronization error (3.67 us) between gNB. The reception boundary is adjusted by TA + NTA_offset by the victim UE. The measured RSRP is compared with the RSRP of a sequence without noise to calculate the error. The CP duration decreases as the numerology is increased. As a result, for constant synchronization error, more amount of SRS information is lost in Rel. 16 transmission scheme as numerology increases which degrades the RSRP. However, in case of the proposed method, the error in RSRP is independent of the numerology.

Fig. 3 Comparison of error in SRS RSRP measurement for Rel. 16 SRS transmission and the proposed method

Further, using the proposed method, no TA adjustment is required at the aggressor UE side. TA adjustment at aggressor UE side will be an issue in case of multiple victim UEs receiving the SRS from one aggressor. Similarly, one victim UE can receive the SRS properly from multiple aggressor UEs since there is no UE specific reception timing adjustment required with the proposed method.

Observation 2: Timing adjustment for transmission or reception by aggressor and victim UE respectively will restrict SRS RSRP measurement in scenarios of multiple aggressor and victim UEs.

Observation 3: SRS RSRP measured on phase rotated SRS symbols (Enhanced Rel. 16 SRS) repeated in time domain (similar to RIM RS design principle) has the following advantages-

- SRS RSRP accuracy improves as compared to accuracy using Rel. 16 CLI RSRP measurement method based on Rel. 16 SRS.

- No need for TA adjustment at the aggressor UE. Thus, SRS RSRP measurement can be done by multiple victim UEs.

- SRS RSRP measurement can be done by a single victim UE from multiple  aggressor UEs.

Moreover, RIM RS design has already been proven to work in case of gNB-to-gNB interference measurement where there is timing synchronisation misalignment between the gNBs. The same design principle can be applied to enhance the Rel. 16 SRS.

Observation 4: RIM RS design has already been proven to work in case of gNB-to-gNB interference measurement where there is timing synchronisation misalignment between the gNBs. The same design principle can be applied to enhance the Rel. 16 SRS by phase rotating the symbols and repeating them in time domain.

Based on the above observations, the following proposals are made.

Proposal 1: Phase rotated SRS symbols repeated in time domain (based on RIM RS design principle) is supported for measurement of SRS RSRP.

Proposal 2: Support DL rate matching around SRS to further improve the CLI measurement accuracy.

## Reporting of UE-to-UE CLI

In Rel. 16 CLI management, the receiving victim UE is semi-statically configured by the gNB to perform L3 reporting of SRS-RSRP when the measured RSRP goes beyond a particular threshold. The agreement in RAN1 Meeting Ad-Hoc Meeting 1901 [2] was as follows:

Agreement

For SRS-RSRP measurement report, L3 measurement reporting is applied.

Dynamic reporting and handling of CLI is not possible with L3 reporting. Moreover, L3 report provides the time averaged CLI reflecting the long-term fading of the channel. However, CLI varies with dynamic scheduling and hence dynamic measurement and reporting should be supported.

The following agreement was made for UE-to-UE co-channel CLI measurement in RAN1 #110bis-e meeting [3].

Agreement

For UE-to-UE co-channel CLI handling, study L1/L2 based UE-to-UE CLI measurement and reporting

- Note: Accounting for UE processing/reporting delay – companies to share their assumptions

- Note: Proponents are encouraged to provide the mechanism of L1/L2 based CLI measurement and reporting, and to provide the benefits of L1/L2 based CLI measurement and reporting compared with existing L3 CLI/CSI measurement and report with evaluation result

- Note: Accounting for information exchange delay between gNBs (if applicable)

The following proposal was made for UE-to-UE co-channel CLI measurement in RAN1 #111 meeting [5].

Agreement

For the purpose of UE-to-UE CLI mitigation, consider the following potential enhancements:

- For L1/L2 UE-to-UE CLI reporting, periodic, semi-persistent, aperiodic reporting.

  - FFS: Event triggered reporting.

- For L1/L2 UE-to-UE CLI measurement, periodic, semi-persistent, or aperiodic measurement resource.

Companies are encouraged to bring additional details and evaluation results to determine the benefit of the above potential enhancements.

The advantages of  L1/L2 CLI measurement and report can be summarized as follows.

- The gNB can get an on-demand CLI report similar to CSI report. CSI cannot capture the CLI explicitly. The L1/L2 CLI captures the CLI explicitly since the measurement is performed on dedicated CLI resources, e.g., SRS.

- The UE can also use the measured CLI for its own receiver processing to suppress it, e.g., the CLI measured on CSI-IM resources. In this case, the CLI report need not be even shared with the gNB.

- The gNB can use the  L1/L2 CLI report to dynamically update the scheduling resources, active beams etc. for the UE. In this case, the CLI report need not be even shared with the adjacent gNB.

Hence, the following observation is made.

Observation 5: The L1/L2 CLI measurement and report has following advantages

- The gNB can get an on-demand CLI report similar to CSI report. CSI cannot capture the CLI explicitly. The L1/L2 CLI captures the CLI explicitly since the measurement is performed on dedicated CLI resources, e.g., SRS.

- The UE can also use the measured CLI for its own receiver processing to suppress it, e.g., the CLI measured on CSI-IM resources. In this case, the CLI report need not be even shared with the gNB.

- The gNB can use the  L1/L2 CLI report to dynamically update the scheduling resources, active beams etc. for the UE. In this case, the CLI report need not be even shared with the adjacent gNB.

The L1/L2 CLI measurement/ reporting can be done in the following way.

- The gNB configures the victim UE with aperiodic (A)/semi-persistent (SP) CLI measurement resources. This is similar to the existing CSI-RS resource configuration.

- The victim UE measures L1/L2 CLI on the resources. The measurement can be based on the Rel. 16 CLI metrics like RSRP/RSSI. The UE processing delay can be similar to CSI processing delay for UE. The L1/L2 CLI can capture the current CLI experienced by the UE as compared to the L3 CLI which requires averaging. CSI-IM resources can be enhanced to have a comb pattern that matches with the SRS comb pattern. This can ensure more accurate measurement of CLI.

- The reporting can also be A/SP. The reporting of  L1/L2 CLI can be of on-demand based: In this case, the gNB explicitly asks the UE to report the measured CLI. E.g., the gNB sends a trigger in DCI for the UE to report the CLI. Periodic measurement and reporting of CLI is unnecessary since CLI is expected to be taken care of when reported.

Observation 6: Periodic measurement and reporting of CLI is unnecessary since CLI is expected to be taken care of when reported.

Based on the above discussion, the following proposals are made.

Proposal 3: Study semi-persistent and aperiodic measurement and on-demand reporting of L1/L2 CLI.

Proposal 4: Support enhanced  CSI-IM resources with comb pattern that matches with the SRS comb pattern to measure  L1/L2 CLI.

Proposal 5: Existing CSI processing delay for UE is the baseline for CLI measurement processing delay.

Further, in our view, the information exchange delay between gNBs is not relevant here. L1/L2 CLI captures the current CLI at the UE. This information can be dynamically used by the UE for receiver processing/ beam selection etc. The report from the UE to its serving gNB can also be used by the gNB  to dynamically update the scheduling resources, beams etc.

Observation 7:  L1/L2 CLI can be directly used by UE/serving gNB for dynamic handling of CLI and not necessarily shared with the adjacent gNB.

Proposal 6:  Information exchange delay between gNBs is necessarily not applicable for comparison between L1/L2 CLI and L3 CLI measurement and reporting.

## Relevant information exchange across gNBs for UE-to-UE co-channel CLI management

Rel. 16 CLI management requires the aggressor to transmit SRS and the victim to measure CLI on the transmitted SRS. Hence, the configuration of the SRS transmitted by the aggressor should be known by the serving gNB of the victim UE to ensure proper measurement configuration is given to the victim. This requires the SRS configuration parameters to be shared between the aggressor and victim gNBs. However, Rel. 16 does not specify the parameters to be shared.

Observation 8: Rel. 16 CLI management does not specify required SRS configuration parameters for CLI measurement to be shared across gNBs.

Fig. 4 Partially overlapping between BWPs of victim and aggressor UEs

Further, consider a scenario where the aggressor UE and the victim UE uses partially overlapping bandwidth parts (BWPs) as shown in Fig. 4. In this case, the aggressor UE will transmit SRS within its own allocated BWP as shown in Fig. 4 while victim UE can measure SRS RSRP only in the overlapping resources. Therefore, only a part of the SRS will be received by the victim UE for RSRP measurement. The aggressor UE generates and fills the SRS sequence from P1 as the reference point up to the end of its BWP. The victim UE should generate the same SRS sequence for correlating and obtaining the RSRP. However, the victim UE will assume that the SRS sequence is filled at the transmitter starting from P2 as the reference point.  As a result, in the overlapping portion of the 2 BWPs, there will be a mismatch between the sequence actually transmitted by the aggressor UE and the sequence assumed by the victim UE. Thus, there will be a mismatch in how the SRS sequence is filled by the aggressor and how SRS sequence is interpreted by the victim. This discrepancy will affect the accuracy of the measured RSRP. E.g., based on simulation analysis, a difference of 1RB between the 2 BWPs will result in an error of around 25 dB.

Observation 9:  In case of partial overlap of BWPs, the victim UE receives only a part of the SRS transmitted by the aggressor UE for measurement of CLI RSRP leading to mismatch in how the SRS sequence is filled by the aggressor and how SRS sequence is interpreted by the victim. E.g., based on simulation analysis, a difference of 1RB between the 2 BWPs will result in an error of around 25 dB.

A knowledge of a common reference point across the gNB will help to tackle the problem. For example, wrt Fig. 4, point P1 is the common reference point that can be shared across the gNBs. As a result, the victim UE can generate the SRS sequence from point P1 instead of P2. This will ensure that the interpretation of the filled SRS sequence is same for both victim and aggressor UEs. Based on  simulation analysis, the error value reduces to an average of 0.2dB by considering a common reference point.

Further, different UEs across cells in a network might be operating at different numerologies. The following agreement made with respect to SRS RSRP in RAN1 Ad-Hoc Meeting 1901 [2],

Agreement

UE is not required to measure SRS using different SCS compared to the downlink active BWP SCS of the same carrier.

Thus, if the numerology of the downlink active BWP of the victim UE is different from the numerology of the transmitted SRS by aggressor UE, the victim UE will not measure the RSRP using the numerology of the transmitted SRS. E.g., the aggressor UE 2 operates at 30 kHz while victim UE 1 operates at 15 kHz. The SRS for CLI measurement will be transmitted at 30 kHz by UE 2. However, UE 1  measures the received SRS using 15 kHz numerology. This discrepancy in the transmitted and received SRS numerologies will affect the accuracy of CLI RSRP measurement.

Observation 10:  When aggressor and victim UE are operating at different numerology, discrepancy arises in the transmitted and received SRS numerologies  that will affect the accuracy of CLI RSRP measurement.

The above mentioned issue can be handled in implementation specific ways if the gNB serving the victim UE is aware of the numerology of SRS transmission by aggressor UE and it informs the victim UE to process SRS using the numerology of the aggressor.

The following proposal is made wrt the previous observations.

Proposal 7: The following information exchange between gNBs should be supported for efficient UE-to-UE CLI measurement.

- Rel. 16 CLI management related SRS configuration parameters

  - Numerology of transmission of SRS

- A common reference point for CLI RSRP measurement

## Spatial domain related enhancements

In this section, spatial domain related enhancements for  UE-to-UE co-channel CLI is explained. The following Moderator proposal was discussed in RAN1 #110bis-e meeting.

Moderator Proposal

For details of spatial domain coordination method for UE-to-UE co-channel CLI, followings can be studied.

- Tx-Rx beam coordination/avoidance between UEs

.

Refering to Fig. 5, if UE 2 can form a null in the direction of UE 1 while performing its UL transmission to gNB 2, the CLI from UE 2 to UE 1 can be minimized. Rel. 16 CLI management does not define any procedure for CLI to be handled at the aggressor UE side.

However, enabling CLI management at the aggressor side will have some advantages, especially in case where the victim UE is a legacy Release 15 UE which is not capable of measurement and reporting of CLI.

Observation 11: Transmit beam null-forming by aggressor UE towards the direction of victim UE can help to minimize CLI at the victim UE, even in cases where the aggressor/victim UE is a legacy Rel. 15 UE.

The aggressor UE needs some information regarding the approximate direction/location of the victim UE. This information will help the victim UE to null-form in that particular direction. Further, if multiple victim UEs are in the same approximate location, null-forming will help to minimize CLI for all the victim UEs. There are various ways in which the  approximate direction/location related information of the victim UE can be obtained and used. The ways are listed below.

1. The serving gNB of the aggressor UE can acquire information regarding the location/direction of the victim UE from its serving gNB and configure aggressor UE to null form in the direction of victim UE.

2.  The serving gNB of the aggressor UE can acquire information regarding the location/direction of the victim UE from its serving gNB and provide this information to the aggressor UE to null form in the direction of victim UE.

3. Potential aggressor UEs can be configured to receive SRS from the victim UE. This will be useful specifically in the case where the victim UE is Rel. 15 UE. It can transmit SRS but cannot receive it. Here, the serving gNB of the victim UE can identify a UE as victim UE based on conditions like whether it is a cell edge UE experiencing a high amount of interference. The aggressor UEs can directly measure the received SRS and find out the approximate direction of the victim UE. This can be done in an implementation specific way eg., by using receive beam sweeping.

4. Similar scenario can be considered where the aggressor UE is a legacy Release 15 UE. Its serving gNB can obtain the information about the victim UE and inform an optimum beamforming precoder to the aggressor. Since, the legacy UE is already capable of receiving information on precoders, the optimum beamforming precoder can be informed to it without any new signaling.

In all the above cases, it is observed that

- the null-forming at the aggressor UE is done by the aggressor UE itself or

- an optimum precoder is provided by the serving gNB to the aggressor UE.

In case the aggressor UE does the null-forming, it is basically implementation specific but needs to be aided. In the other case, the BS obtains an optimum precoder and provides it to the aggressor UE. Thus, if the approximate direction/location of the victim UE is known at the aggressor UE or its serving BS, the optimum precoder can be always updated even when the UE moves/rotates. Moreover, its only an approximate direction and not the exact location. Thus, even if the UE moves the approximate direction does not get updated very fast.

Apart from the scenarios involving legacy UEs, Rel. 18 and beyond UEs can also benefit from it. Thus, any aggressor UE in the network can be configured to perform an optimum beamforming to minimize CLI.

Fig. 5 Transmit side beamforming to suppress CLI

Based on the above discussion, the following proposal is made.

Proposal 8: Mechanism to enable transmit beam null-forming by aggressor UE towards the direction of victim UE for UE-to-UE CLI management is supported.

## 2.2 Enhancements related to gNB-to-gNB CLI measurement and reporting

## 2.2.1 gNB-to-gNB CLI measurement resources

The following agreements were made in RAN1 #110bis-e meeting and RAN1 #111 meeting respectively.

Agreement

For gNB-to-gNB co-channel CLI measurement, the potential benefit of uplink resources muting can be studied further.

Note: Proponents of uplink resource muting are encouraged to provide evaluation result for comparison of performance between two cases when uplink resource muting based gNB-gNB CLI handling schemes including both UE transparent and non-UE transparent schemes is applied or not.

Agreement

For gNB-to-gNB co-channel CLI measurement and/or channel measurement, at least periodic NZP CSI-RS/SSB is the baseline in RAN1 study.

- FFS: Whether SSB is CD-SSB or NCD-SSB

In the study RAN1 assumes that exchange of configuration for NZP CSI-RS /SSB can be an enabler for gNB-to-gNB CLI measurement and/or channel measurement.

For  gNB-to-gNB CLI measurement, RIM-RS can be enhanced as shown in Fig. 6 and used as a measurement resource. The main advantage of using the RIM RS is that the CLI measurement accuracy will not be affected due to the timing synchronisation error that exists between two gNBs. Further, UL muting can be done over a duration of 1 symbol to measure the RS which can be managed by the gNB in a UE transparent way.

Observation 12: Enhanced RIM-RS for gNB-to-gNB CLI measurement can handle the timing synchronisation error that exists between two gNBs.

Fig.  6 Phase rotated RS transmission (Enhanced RIM RS)

Another option is to reuse the DL NZP CSI-RS for  gNB-to-gNB CLI measurement. The aggressor gNB transmits NZP CSI-RS and shares its configuration with the victim gNB. The victim gNB measures the NZP CSI-RS in muted UL resources. For this purpose, the PUSCH at the victim gNB has to be muted around the NZP CSI-RS.

Proposal 9: Support one of the following for gNB-to-gNB co-channel CLI measurement

a. Enhanced RIM RS with UL muting in UE transparent way

b. NZP CSI-RS with UL muting around the  NZP CSI-RS

# Conclusion

In this contribution, the following observations and proposals are made:

Observation 1: Factors like synchronization errors between gNB, smaller CP length in higher numerologies, higher propagation delay between the UEs and implementation specific adjustment of reception timing causes the misalignment to go beyond CP duration while measuring the CLI on Rel. 16 SRS as both the UEs are not time synchronized. This will degrade the CLI measurement accuracy.

Observation 2: Timing adjustment for transmission or reception by aggressor and victim UE respectively will restrict SRS RSRP measurement in scenarios of multiple aggressor and victim UEs.

Observation 3: SRS RSRP measured on phase rotated SRS symbols (Enhanced Rel. 16 SRS) repeated in time domain (similar to RIM RS design principle) has the following advantages-

a. SRS RSRP accuracy improves as compared to accuracy using Rel. 16 CLI RSRP 	measurement method based on Rel. 16 SRS.

b. No need for TA adjustment at the aggressor UE. Thus, SRS RSRP measurement  	can be 	done by multiple victim UEs.

c. SRS RSRP measurement can be done by a single victim UE from multiple  	aggressor UEs.

Observation 4: RIM RS design has already been proven to work in case of gNB-to-gNB interference measurement where there is timing synchronisation misalignment between the gNBs. The same design principle can be applied to enhance the Rel. 16 SRS by phase rotating the symbols and repeating them in time domain.

Proposal 1: Phase rotated SRS symbols repeated in time domain (based on RIM RS design principle) is supported for measurement of SRS RSRP.

Proposal 2: Support DL rate matching around SRS to further improve the CLI measurement accuracy.

Observation 5: The L1/L2 CLI measurement and report has following advantages

- The gNB can get an on-demand CLI report similar to CSI report. CSI cannot capture the CLI explicitly. The L1/L2 CLI captures the CLI explicitly since the measurement is performed on dedicated CLI resources, e.g., SRS.

- The UE can also use the measured CLI for its own receiver processing to suppress it, e.g., the CLI measured on CSI-IM resources. In this case, the CLI report need not be even shared with the gNB.

- The gNB can use the  L1/L2 CLI report to dynamically update the scheduling resources, active beams etc. for the UE. In this case, the CLI report need not be even shared with the adjacent gNB.

Observation 6: Periodic measurement and reporting of CLI is unnecessary since CLI is expected to be taken care of when reported.

Proposal 3: Study semi-persistent and aperiodic measurement and on-demand reporting of  L1/L2 CLI.

Proposal 4: Support enhanced CSI-IM resources with comb pattern that matches with the SRS comb pattern to measure L1/L2 CLI.

Proposal 5: Existing CSI processing delay for UE is the baseline for CLI measurement processing delay.

Observation 7:  L1/L2 CLI can be directly used by UE/serving gNB for dynamic handling of CLI and not necessarily shared with the adjacent gNB.

Proposal 6:  Information exchange delay between gNBs is necessarily not applicable for comparison between L1/L2 CLI and L3 CLI measurement and reporting.

Observation 8: Rel. 16 CLI management does not specify required SRS configuration parameters for CLI measurement to be shared across gNBs.

Observation 9:  In case of partial overlap of BWPs, the victim UE receives only a part of the SRS transmitted by the aggressor UE for measurement of CLI RSRP leading to mismatch in how the SRS sequence is filled by the aggressor and how SRS sequence is interpreted by the victim. E.g., based on simulation analysis, a difference of 1RB between the 2 BWPs will result in an error of around 25 dB.

Observation 10:  When aggressor and victim UE are operating at different numerology, discrepancy arises in the transmitted and received SRS that will affect the accuracy of CLI RSRP measurement.

Proposal 7: The following information exchange between gNBs should be supported for efficient UE-to-UE CLI measurement.

- Rel. 16 CLI management related SRS configuration parameters

    - Numerology of transmission of SRS

- A common reference point for CLI RSRP measurement

Observation 11: Transmit beam null-forming by aggressor UE towards the direction of victim UE can help to minimize CLI at the victim UE, even in cases where the aggressor/victim UE is a legacy Rel. 15 UE.

Proposal 8: Mechanism to enable transmit beam null-forming by aggressor UE towards the direction of victim UE for UE-to-UE CLI management is supported.

Observation 12: Enhanced RIM-RS for gNB-to-gNB CLI measurement can handle the timing synchronisation error that exists between two gNBs.

Proposal 9: Support one of the following for gNB-to-gNB co-channel CLI measurement

a. Enhanced RIM RS with UL muting in UE transparent way

b. NZP CSI-RS with UL muting around the  NZP CSI-RS

# References

- RP-213591, New SI: Study on evolution of NR duplex operation, RAN#94-e, Dec. 2021

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting Ad-Hoc Meeting 1901, Jan. 2019

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #110bis-e, October 2022

- 3GPP TS 38.133, 5G ; NR; Requirements for support of radio resource management.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #111, November 2022